3GPP TSG RAN WG1 #109-e			R1-2204351

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Maintenance on Enhancements to Integrated Access and Backhaul

Agenda Item:	8.10

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss remaining issues eIAB.

- Resource management in frequency domain

In Rel-17, for frequency domain H/S/NA resource configuration, following agreement was made for cell-specific/semi-static signals.

Agreement Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17.

Based on this agreement, when frequency domain H/S/NA is provided, RB sets of these cell-specific/semi-static signals should be regarded as hard RB sets, which is not captured in the spec. Thus, we propose to adopt the following TP, otherwise, how to treat cell-specific/semi-static signals when frequency domain H/S/NA is provided is not clear.

Proposal 1: Adopt the following TP for TS 38.213 section 14.

## <Unchanged text is omitted> If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, a RB set in a symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS on the RB set in the symbol of the slot, or would receive a PRACH or a SR on the RB set in the symbol of the slot. <Unchanged text is omitted>

For IAB node behavior on soft RB sets, when DC is not configured, IAB-DU can transmit or receive on a soft RB set if the IAB-MT’s transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to the use of the RB set. Similarly, when DC is configured, to determine availability of a soft RB set, not only IAB-MT’s transmission or reception on the RB set should be considered, IAB-MT’s transmission or reception on other RB sets also should be considered, which is not captured in the spec. Thus, we propose to adopt the following TP, otherwise, when DC is configured, IAB-MT’s transmission or reception on other RB sets is not taken into account in IAB-DU soft RB set availability determination.

Proposal 2: Adopt the following TP for TS 38.213 section 14.

## <Unchanged text is omitted> When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - with respect to all serving cells, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available if the IAB-MT is not configured with an SCG, or - the IAB-MT detects two DCI formats 2_5 with an AI index field value indicating the soft RB set as available from the MCG and SCG, respectively, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available from one cell group and with respect to all serving cells of the other cell group, the IAB-MT would transmit or receive on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell does not change due to a use of the RB set in the symbol by the IAB-DU. When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the RB set in the symbol. ## <Unchanged text is omitted>

- Conclusion

In this contribution, we discussed remaining issues for eIAB. Based on the discussion, we made following proposals.

Proposal 1: Adopt the following TP for TS 38.213 section 14.

## <Unchanged text is omitted> If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, a RB set in a symbol of a slot is equivalent to being configured as hard if an IAB-DU would transmit a SS/PBCH block, PDCCH for Type0-PDCCH CSS sets configured by pdcchConfigSIB1, or a periodic CSI-RS on the RB set in the symbol of the slot, or would receive a PRACH or a SR on the RB set in the symbol of the slot. <Unchanged text is omitted>

Proposal 2: Adopt the following TP for TS 38.213 section 14.

## <Unchanged text is omitted> When an RB set of a downlink, uplink, or flexible symbol is configured as hard, the IAB-DU cell can respectively transmit, receive, or either transmit or receive on the RB set in the symbol. When an RB set of a downlink, uplink, or flexible symbol is configured as soft, the IAB-DU cell can respectively transmit, receive or either transmit or receive on the RB set in the symbol only if - the IAB-MT does not transmit or receive on the RB set during the symbol of the IAB-DU cell, or - with respect to all serving cells, the IAB-MT would transmit or receive on the RB set during the symbol of the IAB-DU cell, and the transmission or reception on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell is not changed due to a use of the RB set in the symbol by the IAB-DU, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available if the IAB-MT is not configured with an SCG, or - the IAB-MT detects two DCI formats 2_5 with an AI index field value indicating the soft RB set as available from the MCG and SCG, respectively, or - the IAB-MT detects a DCI format 2_5 with an AI index field value indicating the soft RB set as available from one cell group and with respect to all serving cells of the other cell group, the IAB-MT would transmit or receive on the RB set or any RB set that is configured as unavailable or configured as soft and not indicated as available during the symbol of the IAB-DU cell, and the transmission or reception on the RB set during the symbol of the IAB-DU cell does not change due to a use of the RB set in the symbol by the IAB-DU. When an RB set of a downlink, uplink, or flexible symbol is configured as unavailable, the IAB-DU neither transmits nor receives in the RB set in the symbol. ## <Unchanged text is omitted>