3GPP TSG RAN WG1 #109-e			     R1-2204352

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Maintenance of sidelink resource allocation for power saving

Agenda Item:	8.11.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1#108-e meeting [1], there were discussions on resource allocation for power saving and this topic was concluded. In this contribution, we share our views for maintenance.

- Discussions

  - Conditions to perform PBPS and to perform CPS

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - If the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min, - How UE includes other candidate slots is up to UE implementation - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results starting at M consecutive logical slots before ty0 and ending at Tproc,0 + Tproc,1 slots earlier than ty0. - FFS: By default, M is 31 unless (pre-)configured with another value, or where M is (pre-)configured based on transmission priority - FFS: The range of (pre-)configured M from a TBD lowest value up to 30 - When the minimum M slots for CPS cannot be guaranteed, support both - Option A, the UE ensures the Y’min criterion is fulfilled - Option B: UE performs random resource selection When the UE performs Option A or Option B is up to UE implementation

At the previous meeting, the above agreement was reached to define partial sensing for aperiodic transmission. Here, Y’ candidate slots are used instead of Y candidate slots. However, it seems that step 2 in the current specification still uses only Y candidate slots. In addition, we believe that intention of the agreement is, Y’ slots are selected from slots corresponding to PBPS slots that were already monitored. The spec text is not aligned with the above agreement and the agreement is imperfect to achieve the intention, therefore appropriate TP should be discussed and agreed. From our side, the following update including other small updates is proposed.

Proposal 1:

- UE selects a set of Y’ candidate slots from slots corresponding to PBPS slots that were already monitored. If the total number of Y’ slots is less than threshold, it is up to UE implementation to include other slots.

  - Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. A set of Y' candidate slots are selected such that the UE monitored slots corresponding to the set of Y' candidate slots in periodic-based partial sensing. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList. The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y or Y’ candidate slots of PBPS. When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window of CPS is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y or Y’ candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0, the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. ...

  - SL DRX inactive time vs. full sensing

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

At the last meeting, UE behavior when SL DRX is (pre-)configured and partial sensing is performed was concluded as above captured. One more issue is still raised – UE behavior when SL DRX is (pre-)configured and full sensing is performed. On this issue, in short, it is unclear for us whether this combination is really feasible.

Full sensing is intended for detecting all reservations from other UEs at the sacrifices of power saving aspect; i.e. aiming to reliability. Meanwhile, SL DRX is intended not to do receptions within some time duration. If monitoring is skipped due to SL DRX inactive time, the above direction of full sensing is completely broken. Alternatively, if monitoring is performed according to full sensing during SL DRX inactive time, no power saving gain is assumed since the UE shall monitor anytime so that the UE can transmit aperiodic traffic, which is suddenly delivered from higher layer. Rather, UE requiring power saving should use partial sensing or random selection with SL DRX. Why such a UE uses full sensing is quite unclear. Without valid motivation, additional rule to cover the strange combinations of (pre-)configurations should be avoided.

Proposal 2 (for conclusion):

- SL DRX is not (pre-)configured with full sensing.

  - Partial sensing within SL DRX inactive time

At the previous meeting, it was agreed that sensing is performed within SL DRX inactive time when partialSensingInactiveTime is enabled (see the agreement in the last section of this paper), and it is mentioned in the latest specification. Meanwhile, UE behavior when disabled is not specified in the spec. Whether sensing is allowed or prohibited might be misunderstood.

Proposal 3:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. When it is disabled, UE is not required to perform partial sensing on the slots in SL DRX inactive time. ...

  - CPS disabling

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value - The range of (pre-)configured M is from 0 (working assumption) to 30

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE may perform contiguous partial sensing. ...

Until the last meeting, CPS was discussed as a main part of NR-SL partial sensing and almost concluded. However, it seems that companies on whether CPS is always performed are divergent. Our understanding is that CPS shall be performed as agreed. According to the agreements so far, CPS is performed under the following conditions: Partial sensing is (pre-)configured for the resource pool; Higher layer determines to use partial sensing for TX; Higher layer requests resource candidates to PHY layer. No other rule is necessary and CPS shall be performed based on the (pre-)configured M value since aperiodic reservation by other UE, which is monitored by CPS, cannot be disabled. The current specification text seems not to be aligned with the previous agreements on CPS. The following TP should be agreed.

Proposal 4:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE may performs contiguous partial sensing. ...

  - Issues on re-evaluation/pre-emption check with partial sensing

    - Half-duplex consideration

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, ... - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. ...

Detailed UE behaviors of re-evaluation/pre-emption check with partial sensing were agreed and specified as above captured. One issue here is whether UE needs to monitor any slots corresponding to PBPS/CPS. In other words, whether “except for those in which its own transmission occur” in Step 2 of full sensing case with cyan-highlight is applied to partial sensing for re-evaluation/pre-emption check or not. One note is that this exception is not applied in resource (re)selection with partial sensing, according to the previous agreements and the current specification (e.g. the text for PBPS with yellow-highlight).

For this issue, it would be difficult/impossible to avoid any transmissions at slots corresponding to PBPS/CPS for re-evaluation/pre-emption check. For example, when a UE reserved a resource at slot n+20 from a resource slot n, the UE will perform pre-emption check before slot n+20. In the procedure, slot n will be included in the CPS window, but the UE cannot monitor the slot due to half-duplex. If “The UE shall monitor slots ... except for those in which its own transmission occur” is not applied, reservation - re-evaluation - pre-emption check concept does not work.

Proposal 5:

- For re-evaluation/pre-emption check with partial sensing, monitoring slots determined based on PBPS/CPS is skipped due to its own transmission.

  - Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, ... - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL except for those in which its own transmission occur, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL except for those in which its own transmission occur. - By default, M is 31 unless (pre-)configured with another value. ...

    - Insufficient candidate resources

In the current specification text for re-evaluation/pre-emption check by using partial sensing, how to perform PBPS and CPS are mentioned clearly. Meanwhile, it seems that the relationship between the PBPS/CPS behavior and the steps to identify SA is unclear. Without appropriate spec update, the UE just performs the PBPS/CPS behavior but there is no impact on the SA identification.

Proposal 6:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, candidate resource set (SA) is determined by the above steps with the following exceptions: - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, candidate resource set (SA) is determined by the above steps with the following exceptions: - Candidate resource set (SA) is initialized to the remaining Y’ candidate slots starting from slot tyiSL and ending at the last slot of the Y’ candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

    - Insufficient candidate resources

At the agreed re-evaluation/pre-emption check mechanism with partial sensing, SA is initialized to resources from the remaining Y/Y’ candidate slots selected at the initial resource selection behavior. Here we believe that there is an issue on this procedure, which is how to handle the case where the number of the resources in the initialized SA is smaller than X∙Mtotal. An example of this issue is illustrated below. In the illustration, initial resource selection is triggered at slot n. Y candidate slots are determined and a few resources are selected. Then before the selected resource, re-evaluation / pre-emption check is performed. As an example, the illustration shows pre-emption check behaviour. If the target resource of re-evaluation / pre-emption check is in quite later part among the initial Y candidate slots, the remaining Y candidate slots will be only a few slots. Without any additional rule, the UE cannot break out of the infinite loop to meet X∙Mtotal. One important note would be that this situation is not a rare case.

Fig. 1: Only a few slots in the initialized SA of re-evaluation/pre-emption.

Re-evaluation/pre-emption check is mainly motivated only to check reservation transmitted after the initial selection procedure. Thus, there would be no need to determine new SA for re-evaluation/pre-emption check. Alternatively, we believe that partial sensing only for the pre-selected resources (r0,r1,r2,…) and/or the reserved resources (r0',r1',r2',…) without SA identification is a reasonable option for the issue on insufficient number of resources in the initialized SA, which is illustrated below. What the UE needs to do is to check whether there is other reservation to indicate the pre-selected resources and/or reserved resources and whether RSRP of the reservation from other UE is over RSRP threshold or not.

For this procedure, appropriate RSRP threshold should be used, but the conventional loop of step 4/5/6 is not applied. Instead, the RSRP threshold value can be same as the last value in the initial resource selection procedure. The value was determined the loop of step 4/5/6 in the initial resource selection procedure; valid value can be assumed for the re-evaluation/pre-emption check as well.

Fig. 2: Pre-emption check without S_A identification.

Proposal 7:

- For re-evaluation/pre-emption check with partial sensing, when the number of resources in the initialized set SA in Step 4 is smaller than X∙Mtotal,

  - UE performs partial sensing only slots corresponding to the pre-selected resources (r0,r1,r2,…) and/or reserved resources (r0',r1',r2',…).

  - UE does not identify new S_A for re-evaluation/pre-emption check.

  - UE detects re-evaluation/pre-emption by using RSRP threshold value that is same as the last value in the initial resource (re)selection procedure.

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is equal to or larger than X⋅Mtotal, [candidate resource set (SA) is determined by the above steps with the following exceptions:] - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation or pre-emption checking, and if Prsvp_TX≠0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is smaller than X⋅Mtotal, steps 1)-7) are not performed. - The UE performs PBPS for the slots including the pre-selected resources (r0,r1,r2,…) and/or reserved resources (r0',r1',r2',…) according to ty''-k×PreserveSL, where ty''SL is a slot belonging to the slots including the pre-selected and/or reserved resources, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyjSL to Tproc,0SL+Tproc,1SL slots earlier than tyjSL, where tyjSL is the first slot among the slots including the pre-selected and/or reserved resources. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. - The UE shall report re-evaluation if ri meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection. - The UE shall report pre-emption if ri' meets the conditions below. - ri' meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection, and - ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and - the associated priority prioRX, satisfies one of the following conditions: - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is equal to or larger than X⋅Mtotal, [candidate resource set (SA) is determined by the above steps with the following exceptions:] - Candidate resource set (SA) is initialized to the remaining Y’ candidate slots starting from slot tyiSL and ending at the last slot of the Y’ candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0. When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is smaller than X⋅Mtotal, steps 1)-7) are not performed. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyjSL to Tproc,0SL+Tproc,1SL slots earlier than tyjSL, where tyjSL is the first slot among the slots including the pre-selected and/or reserved resources. - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0. - The UE shall report re-evaluation if ri meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection. - The UE shall report pre-emption if ri' meets the conditions below. - ri' meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection, and - ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and - the associated priority prioRX, satisfies one of the following conditions: - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX ...

- Conclusion

In this contribution, we discussed maintenance of SL resource allocation enhancement for power saving. Observations/Proposals are summarized as following:

Proposal 1:

- UE selects a set of Y’ candidate slots from slots corresponding to PBPS slots that were already monitored. If the total number of Y’ slots is less than threshold, it is up to UE implementation to include other slots.

  - Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 for UE performing periodic-based partial sensing correspond to one candidate single-slot resource, or in a set of Y' candidate slots within the time interval [n+T1,n+T2] for UE performing contiguous partial sensing if Prsvp_TX=0, correspond to one candidate single-slot resource, where - selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP; - if T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to T2min ≤ T2 ≤ remaining packet delay budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots). - Y is selected by UE where Y≥Ymin. - Y' is selected by UE where Y'≥Ymin'. A set of Y' candidate slots are selected such that the UE monitored slots corresponding to the set of Y' candidate slots in periodic-based partial sensing. When the UE performs contiguous partial sensing and if Prsvp_TX=0, if the number of candidate single-slot resources Y' is smaller than Ymin', it is up to UE implementation to include other candidate slots. 2) The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL), when the UE performs full sensing, where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which belongs to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. When the UE performs periodic-based partial sensing, the UE shall monitor slots at ty-k×PreserveSL, where tySL is a slot of the selected candidate slots. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots. The value of Preserve corresponds to periodicSensingOccasionReservePeriodList if configured, otherwise, the values correspond to all periodicity from sl-ResourceReservePeriodList. The UE monitors k sensing occasions determined by additionalPeriodicSensingOccasion, as previously described, and not earlier than n –T0. For a given periodicity Preserve, the values of k correspond to the most recent sensing occasion earlier than ty0SL-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. ty0SL is the first slot of the selected Y or Y’ candidate slots of PBPS. When the UE performs periodic-based partial sensing and contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) enabled, the sensing window of CPS is defined by the range of slots [n+TA, n+TB]. n+TA is M consecutive logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y or Y’ candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. If Prsvp_TX≠0, the value of M is (pre-)configured with the contiguousSensingWindowPeriodic. If contiguousSensingWindowPeriodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. ...

Proposal 2 (for conclusion):

- SL DRX is not (pre-)configured with full sensing.

Proposal 3:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... Whether the UE is required to performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by higher layer parameter partialSensingInactiveTime. When it is enabled, if UE performs periodic-based partial sensing on the slots in SL DRX inactive time for a given Prsvp_TX, UE monitors only the default periodic sensing occasions (most recent sensing occasion) from the slots; if UE performs contiguous partial sensing on the slots in SL DRX inactive time, UE monitors a minimum of M slots from the slots. When it is disabled, UE is not required to perform partial sensing on the slots in SL DRX inactive time. ...

Proposal 4:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When a UE is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool, the resource pool is (pre-)configured with allowedResourceSelectionConfig including partial sensing, and partial sensing is configured by higher layer, the UE may performs contiguous partial sensing. ...

Proposal 5:

- For re-evaluation/pre-emption check with partial sensing, monitoring slots determined based on PBPS/CPS is skipped due to its own transmission.

  - Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, ... - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL except for those in which its own transmission occur, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL except for those in which its own transmission occur. - By default, M is 31 unless (pre-)configured with another value. ...

Proposal 6:

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, candidate resource set (SA) is determined by the above steps with the following exceptions: - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, candidate resource set (SA) is determined by the above steps with the following exceptions: - Candidate resource set (SA) is initialized to the remaining Y’ candidate slots starting from slot tyiSL and ending at the last slot of the Y’ candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

Proposal 7:

- For re-evaluation/pre-emption check with partial sensing, when the number of resources in the initialized set SA in Step 4 is smaller than X∙Mtotal,

  - UE performs partial sensing only slots corresponding to the pre-selected resources (r0,r1,r2,…) and/or reserved resources (r0',r1',r2',…).

  - UE does not identify new S_A for re-evaluation/pre-emption check.

  - UE detects re-evaluation/pre-emption by using RSRP threshold value that is same as the last value in the initial resource (re)selection procedure.

- Apply the following TP.

38.214 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ... When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX≠0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is equal to or larger than X⋅Mtotal, [candidate resource set (SA) is determined by the above steps with the following exceptions:] - During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot tyiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to [q x Prsvp_Tx + tySL], where tySL is a slot index of Y candidate slots used in the initial resource (re)selection. - tyiSL is the first candidate slot after slot n+T3. - The UE performs PBPS for the remaining Y candidate slots according to ty'-k×PreserveSL, where ty'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation or pre-emption checking, and if Prsvp_TX≠0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is smaller than X⋅Mtotal, steps 1)-7) are not performed. - The UE performs PBPS for the slots including the pre-selected resources (r0,r1,r2,…) and/or reserved resources (r0',r1',r2',…) according to ty''-k×PreserveSL, where ty''SL is a slot belonging to the slots including the pre-selected and/or reserved resources, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than [tyiSL]-(Tproc,0SL+Tproc,1SL ) if additionalPeriodicSensingOccasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if additionalPeriodicSensingOccasion is (pre-)configured. - The UE performs CPS starting from M logical slots earlier than tyjSL to Tproc,0SL+Tproc,1SL slots earlier than tyjSL, where tyjSL is the first slot among the slots including the pre-selected and/or reserved resources. - By default, M is 31 unless (pre-)configured with another value by contiguousSensingWindowPeriodic. - The UE shall report re-evaluation if ri meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection. - The UE shall report pre-emption if ri' meets the conditions below. - ri' meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection, and - ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and - the associated priority prioRX, satisfies one of the following conditions: - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is equal to or larger than X⋅Mtotal, [candidate resource set (SA) is determined by the above steps with the following exceptions:] - Candidate resource set (SA) is initialized to the remaining Y’ candidate slots starting from slot tyiSL and ending at the last slot of the Y’ candidate slots, where tyiSL is the first candidate slot after slot n+T3. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyiSL to Tproc,0SL+Tproc,1SL slots earlier than tyiSL - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0. When the UE is triggered to perform re-evaluation and pre-emption checking, and if Prsvp_TX=0, and if the number of candidate single-slot resources Rx,y in the set SA at step 4 is smaller than X⋅Mtotal, steps 1)-7) are not performed. - It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled - UE performs CPS starting from at least M consecutive logical slots earlier than tyjSL to Tproc,0SL+Tproc,1SL slots earlier than tyjSL, where tyjSL is the first slot among the slots including the pre-selected and/or reserved resources. - For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by contiguousSensingWindowAperiodic. When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than tyiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0. - The UE shall report re-evaluation if ri meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection. - The UE shall report pre-emption if ri' meets the conditions below. - ri' meets the conditions for exclusion in step 6 with the final threshold after executing steps 1)-7) in the corresponding resource (re-)selection, and - ri' meets the conditions for exclusion in step 6, with ThprioRX,prioTX set to the final threshold after executing steps 1)-7), i.e. including all necessary increments for reaching X⋅Mtotal, and - the associated priority prioRX, satisfies one of the following conditions: - sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX - sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX ...

References

- 3GPP RAN1#108-e meeting,	chair’s notes