3GPP TSG RAN WG1 #109-e			     R1-2204353

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Maintenance of sidelink resource allocation for reliability and latency

Agenda Item:	8.11.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1#108-e meeting [1], there were discussions on resource allocation for reliability/latency improvements and this topic was concluded. In this contribution, we share our further views for maintenance.

- Discussions

  - Inter-UE coordination - scheme 1

    - Cast type for IUC scheme 1

Agreement For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported Conclusion For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

Regarding cast type for scheme 1, the above agreements/conclusion were reached. However, it seems that there is no corresponding text in RAN1/RAN2 spec. Basically cast type determination should be specified in RAN2 spec, thus an LS to inform these agreements/conclusion should be sent to RAN2. Meanwhile SCI format 2-C is a PHY feature. Corresponding text should be added to RAN1 spec.

Proposal 1:

- Send an LS to RAN2 to inform agreements/conclusion on cast type related to IUC scheme 1 and ask to update their specification accordingly.

- Apply the following TP.

38.212 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: ...

    - Priority value for ICU scheme 1

Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data Agreement For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B. - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data Agreement For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. - FFS: Otherwise, the priority value is determined by UE-A’s implementation. - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

Regarding priority values for scheme 1 transmissions, the above agreements were reached. However, it seems that there is no corresponding text in RAN1/RAN2 spec. Here, we believe that there is a question of whether the priority-related rules agreed above are for higher layer priority or PHY layer priority. Although PHY layer priority might have been assumed, the agreements are using terminology of ‘priority value of data’. From PHY layer perspective, IUC message is included in data (i.e. TB) since multiplexing is done at MAC layer; hence it seems that the agreements should be applied to higher layer priority.

Proposal 2:

- For scheme 1, priority values determinations agreed for transmissions of preferred/non-preferred set or explicit request are applied to higher layer priorities.

- Send an LS to RAN2 to inform agreements on the priority values determinations and ask to update their specification accordingly.

    - UE-B’s behavior after receiving IUC message

Agreement In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any)

38.321 5.22.1.1 SL Grant reception and SCI transmission ... 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 6> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> use the randomly selected resource to select a set of periodic resources spaced by the resource reservation interval for transmissions of PSCCH and PSSCH corresponding to the number of transmission opportunities of MAC PDUs determined in TS 38.214 [7];

UE-B’s behavior after receiving coordination message was discussed above. Regarding Option B, the agreement is saying that this option is supported at least when UE-B does not support sensing/resource exclusion. However, the current specification is not aligned with the previous agreement, which is the texts with yellow/cyan highlight.

One important observation would be that Option B is an absolute issue for other UE’s transmission performance since UE-B does not consider the surrounding environment, including any other UEs’ reservations, which is a kind of random selection from perspective of the surrounding UEs. Even if UE-B’s performance is good in Option B, Option A should be prioritized from system performance perspective when applicable.

Fig. 1: Issue of Option B.

There were discussions on this issue in the previous meeting, but companies’ preferences were divergent. One alternative is not to allow Option B if the UE supports full sensing/partial sensing as stated above. Another alternative is to allow Option B anytime. Then, we suggest the following middle-ground alternative as compromise.

Proposal 3:

- For scheme 1, Option B of preferred resource set is available at least one of the following condition is met:

  - UE-B does not have a capability of performing sensing/resource exclusion.

  - UE-B performs random resource selection.

    - It is allowed only in resource pool (pre-)configured with random resource selection.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

    - UE-B’s behavior when UE-B receives multiple IUC messages from a single UE-A

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

38.214 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. 38.321 5.22.1.1 SL Grant reception and SCI transmission ... 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> if configured by RRC, interUECoordinationScheme1Explicit or interUECoordinationScheme1Condition enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 6> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> use the randomly selected resource to select a set of periodic resources spaced by the resource reservation interval for transmissions of PSCCH and PSSCH corresponding to the number of transmission opportunities of MAC PDUs determined in TS 38.214 [7];

At the last meeting, there were discussions on UE behavior when a UE-B received multiple resource sets from a single UE-A. For multiple preferred resources, the UE-B’s behavior is up to UE implementation and it is specified as yellow highlight. For multiple non-preferred resources, no spec update from the text with cyan highlight was agreed. There is no limitation to use non-preferred resources here, which means that all non-preferred resources are used.

Meanwhile, UE-B’s behavior when the UE-B received a single preferred resource set and a single non-preferred resource set from a single UE-A is still under discussion. In our view, a non-preferred resource set are not the complementary information of a preferred resource set; therefore, using both information would be beneficial for UE-B. Then regarding how to use both information, Option A with preferred resources is already applied with non-preferred resources in the current specification since Option A uses intersection of preferred resources and SA where non-preferred resources are excluded. However, Option B does not utilize SA, which means that non-preferred resources are not considered in the current spec. Additional agreement for this option is necessary.

Proposal 4:

- For scheme 1, when UE-B receives a single preferred resource set and a single non-preferred resource set from a single UE-A,

  - UE-B uses both sets in a resource selection behavior. If UE-B selects resource(s) without sensing/resource exclusion, the UE-B selects resource(s) from preferred resources except for resources belonging to non-preferred resources.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

    - UE-B’s behavior when UE-B receives multiple IUC messages from multiple UE-As

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

Similarly to the last section, UE behavior when UE-B receives multiple IUC messages from different UE-As were discussed in the last meeting and UE-B’s behavior for multiple preferred resource sets from different UE-As and multiple non-preferred resource sets from different UE-As was agreed as the texts with yellow/cyan highlight at the last section. For the former case, UE-B uses only the preferred set(s) from the destination UE. For the latter case, no spec update was agreed, which means that all non-preferred resource sets will be used at UE-B.

Meanwhile, UE-B’s behavior when the UE-B received a single preferred resource set and a single non-preferred resource set from different UE-As is still under discussion. In our view, the agreed directions can be applied to the FFS case as they are. In other words, the UE-B uses preferred resource set from the destination UE and non-preferred resource set from any UE. In addition, further rule as discussed in the last section is necessary for this case as well.

Proposal 5:

- For scheme 1, when UE-B receives a single preferred resource set and a single non-preferred resource set from different UE-As,

  - UE-B uses the preferred resource set if the UE-A is the destination UE of the corresponding transmission from the UE-B.

  - UE-B uses the non-preferred resource set received from different UE-A in the same resource selection. If UE-B selects resource(s) without sensing/resource exclusion, the UE-B selects resource(s) from preferred resources except for resources belonging to non-preferred resources.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

  - Inter-UE coordination - scheme 2

    - UE-A’s behavior by using ‘Conflict information receiver flag’ field in SCI 1-A

Agreement Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not. - Working Assumption: - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation. - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

At the last meeting, the above agreement was reached, where a UE with higher priority value is set as UE-B when the two UEs scheduling the conflicting TBs support UE-B behavior of scheme 2 (and with further conditions). Here we believe that this agreement is imperfect since there is another case where only either of the two UEs supports UE-B behavior of scheme 2 and the other UE does not support. Rather, this situation is the motivation to introduce ‘Conflict information receiver flag’ field; therefore the correction is an essential feature.

At UE-A side, there is an (above) agreement on UE-B determination based on priorities attached to TBs. Then regarding relationship with the agreed priority-based UE-B determination, two directions are considerable for UE-B determination.

- Alt 1: Firstly capability-based selection, then priority-based selection. After this rule, certainly one UE is selected as UE-B if at least one UE supports scheme 2.

- Alt 2: Firstly priority-based selection, then capability-based selection. After this rule, none of UE might be selected as UE-B.

In our view, Alt 2 is not preferable since collision indication is not sent and collision is not avoided in some cases. There was an argument that collision indication might be sent to UE transmitting a TB with lower priority, which is not a good way. However, collision indication to UE transmitting a TB with lower priority was already agreed as a case where sl-TypeUE-A is disabled and destination UE of either of the conflicting TBs is not UE-A. Thus the argument is not valid comment. Appropriate (pre-)configuration of collision detection, e.g. SIR-based conflict detection, can handle such a situation.

Proposal 6:

- For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A,

  - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, that UE is UE-B.

- Apply the following TP.

38.213 16.3.0 UE procedure for transmitting PSFCH with control information ... A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - Conflict information receiver flag field in SCI Format 1-A from the second UE is set to 1 and Conflict information receiver flag field in SCI Format 1-A from the third UE is set to 0, or the indicationUEB flag Conflict information receiver flag field in SCI Format 1-A from the second UE and the third UE is set to 1 and p2<p1, if indicationUEBScheme2 sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 ...

    - Details of the “next” reserved resource

Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources. - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B Agreement For Scheme 2, - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer. - When UE-B receives a conflict indicator for resource(s) indicated by its SCI, - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer. - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources Conclusion When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted, - if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”, - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

38.213 16.3.0 UE procedure for transmitting PSFCH with control information ... A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by optionForCondition2A1Scheme2 to determine conflict of reserved resources in a resource pool - if optionForCondition2A1Scheme2 = 'RSRP-ThresPerPriorities', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if optionForCondition2A1Scheme2 = 'RSRP-ThresWithRsrpMeasurement', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 - If PSFCHOccasionScheme2 = 'followSCI', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If PSFCHOccasionScheme2 = 'followReservedResource', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. 16.3.0 UE procedure for transmitting PSFCH with control information ... A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter-UECoordinationScheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if slotLevelResourceExclusionScheme2 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if slotLevelResourceExclusionScheme2 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A ...

At the previous meetings, ‘next reserved resource’ concept was agreed as above, and the corresponding UE behavior is specified as above texts with yellow/cyan/gray/green highlight. From the text with yellow highlight, UE-A detects resource overlap. Then, the UE-A checks the received RSRPs and determines resource conflict as cyan part. After that, a PSFCH resource to transmit the collision indication is selected as gray part and the UE-A transmits a PSFCH on the resource. The UE-B receives the PSFCH and reports resources corresponding to ‘the next reserved resource’ based on the text with green highlight.

It seems from the above spec description that this mechanism is applied even if UE-B transmits a SCI (R0) with reservations of two resources and the resource conflict occurs at the 2nd resource (R2) of the two reserved resources (R1, R2). Here, we have two options of PSFCH resource determination for collision indication. Option 1 determines a PSFCH corresponding to PSSCH (R0) with the SCI reserving resources. Option 2 determines a PSFCH corresponding to the reserved resource with resource conflict (R2). They are illustrated as below.

Fig. 2: Collision indication based on ‘the next reserved resource’ concept.

In Option 2, there is no problem since the collision indication is transmitted for the reserved resource with resource conflict (R2) as shown in the illustration and the 1st reserved resource (R1) is not involved with the behavior. However, we believe that there is a big issue in Option 1. In Option 1, ‘the next reserved resource’ corresponding to the PSFCH is R1. Although R1 is not involved with the resource overlap at R2, the UE-B will report resources corresponding to R1 to its higher layer while the UE-B does not know the resource overlap at R2. R2 is still available by the UE-B. The collision indication becomes meaningless; therefore correction for this issue is essential One note would be that even in this situation, it seems that UE-A will not skip transmission of the PSFCH according to the spec description above.

To solve this issue with keeping ‘the next reserved resource’ concept, it would be better that UE-A skips transmission of the PSFCH in the above situation in Option 1. By this correction, the meaningless PSFCH transmission and reselection are avoided, which is the clear benefit of this solution. Of course, this solution does not solve resource conflict at R2. If which resource a conflict occurs at can be indicated, ‘the next reserved resource’ concept should not be applied for this case and another mechanism should be introduced instead of the skipping solution. However, it was implicitly agreed that ‘which resource’ cannot be indicated. In addition, we are now in maintenance phase. A significant modification would not be allowed.

Proposal 7:

- For Condition 2-A-1 in Scheme 2, when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

  - UE-A does not transmit a conflict indicator if there is no resource conflict at the next reserved resource indicated by the corresponding UE-B’s SCI but there is resource conflict at the subsequent reserved resource.

- Apply the following TP.

38.213 16.3.0 UE procedure for transmitting PSFCH with control information ... If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 - If PSFCHOccasionScheme2 = 'followSCI', and if the UE determined a resource conflict at a next in time reserved resource indicated by the SCI format 1-A, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If PSFCHOccasionScheme2 = 'followReservedResource', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. ...

- Conclusion

In this contribution, we discussed maintenance of resource allocation for reliability and latency enhancements. Observations/Proposals are summarized as following:

Proposal 1:

- Send an LS to RAN2 to inform agreements/conclusion on cast type related to IUC scheme 1 and ask to update their specification accordingly.

- Apply the following TP.

38.212 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: ...

Proposal 2:

- For scheme 1, priority values determinations agreed for transmissions of preferred/non-preferred set or explicit request are applied to higher layer priorities.

- Send an LS to RAN2 to inform agreements on the priority values determinations and ask to update their specification accordingly.

Proposal 3:

- For scheme 1, Option B of preferred resource set is available at least one of the following condition is met:

  - UE-B does not have a capability of performing sensing/resource exclusion.

  - UE-B performs random resource selection.

    - It is allowed only in resource pool (pre-)configured with random resource selection.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

Proposal 4:

- For scheme 1, when UE-B receives a single preferred resource set and a single non-preferred resource set from a single UE-A,

  - UE-B uses both sets in a resource selection behavior. If UE-B selects resource(s) without sensing/resource exclusion, the UE-B selects resource(s) from preferred resources except for resources belonging to non-preferred resources.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

Proposal 5:

- For scheme 1, when UE-B receives a single preferred resource set and a single non-preferred resource set from different UE-As,

  - UE-B uses the preferred resource set if the UE-A is the destination UE of the corresponding transmission from the UE-B.

  - UE-B uses the non-preferred resource set received from different UE-A in the same resource selection. If UE-B selects resource(s) without sensing/resource exclusion, the UE-B selects resource(s) from preferred resources except for resources belonging to non-preferred resources.

- Send an LS to RAN2 to inform this UE-B’s behavior and ask to update their specification accordingly.

Proposal 6:

- For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A,

  - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, that UE is UE-B.

- Apply the following TP.

38.213 16.3.0 UE procedure for transmitting PSFCH with control information ... A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - Conflict information receiver flag field in SCI Format 1-A from the second UE is set to 1 and Conflict information receiver flag field in SCI Format 1-A from the third UE is set to 0, or the indicationUEB flag Conflict information receiver flag field in SCI Format 1-A from the second UE and the third UE is set to 1 and p2<p1, if indicationUEBScheme2 sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 ...

Proposal 7:

- For Condition 2-A-1 in Scheme 2, when PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

  - UE-A does not transmit a conflict indicator if there is no resource conflict at the next reserved resource indicated by the corresponding UE-B’s SCI but there is resource conflict at the subsequent reserved resource.

- Apply the following TP.

38.213 16.3.0 UE procedure for transmitting PSFCH with control information ... If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 - If PSFCHOccasionScheme2 = 'followSCI', and if the UE determined a resource conflict at a next in time reserved resource indicated by the SCI format 1-A, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If PSFCHOccasionScheme2 = 'followReservedResource', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. ...

References

- 3GPP RAN1#108-e meeting,	chair’s notes