3GPP TSG-RAN WG1 #109-e			R1-2204354

e-Meeting, May 9 – 20, 2022

Agenda item:	8.12.1

Title:	Remaining issues on HARQ-ACK feedback procedure for multicast

Source:	NTT DOCOMO, INC.

Document for:	Discussion/Decision

- Introduction

In this contribution, the remaining issues related to HARQ-ACK feedback procedure for multicast are discussed.

- Discussion

  - ACK/NACK based HARQ-ACK feedback for multicast

    - Type-1 HARQ-ACK codebook for multicast

In RAN1#108-e the following agreements were made [5].

Agreement For Type-1 codebook generation, if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, FFS between the 3 alternatives below: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI - Other alternatives are not precluded Agreement If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

Enabling/disabling HARQ-ACK feedback should be based on the reliability required for multicast service. When multiple G-RNTIs/G-CS-RNTIs are configured, HARQ-ACK feedback may be enabled for some G-RNTIs/G-CS-RNTIs and disabled for other G-RNTIs/G-CS-RNTIs. In such a case, if the HARQ-ACK information for a TB with disabled HARQ-ACK feedback is fixed to NACK, gNB can decode the HARQ-ACK feedback using the known information. As a result, it may be possible to improve HARQ-ACK transmission performance compared to the case where it is not fixed to NACK. Therefore, we prefer Alt 1.

Proposal 1: For Type-1 HARQ-ACK codebook generation for multicast

- When multiple G-RNTIs are configured, support Alt 1.

- When HARQ-ACK feedback for dynamic multicast scheduling is enabled and HARQ-ACK feedback for multicast SPS is disabled, support Alt 1.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = semi-static, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. The UE reports a NACK value for a HARQ-ACK information bit corresponding to a transport block with HARQ-ACK feedback disabled in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

    - Type-2 HARQ-ACK codebook for multicast

In RAN1#108-e the following agreement was made.

Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded

Alt 1-2 is not appropriate because it would impose significant scheduling constraints. Alt 1-1 minimizes the codebook size but is less tolerant of DCI miss detection. Alt 1-3 is more tolerant of DCI miss detection than Alt 1-1, but the codebook size may be more than necessary. Figure 1 shows an example of HARQ-ACK information mismatch in Alt 1-1. As shown in the figure, a HARQ-ACK information mismatch will occur if a UE miss detects the last DCI of the G-RNTI which is not the last one in the order of concatenating sub-codebooks. Even in that case, Alt 1-3 can prevent HARQ-ACK information mismatch. Considering the purpose of UL DAI as a countermeasure against DCI miss detection, we think Alt 1-3 is more appropriate.

Figure 1 An example of HARQ-ACK information mismatch due to DCI miss detection

Proposal 2 For how to count the total number of HARQ-ACK bits for all configured G-RNTIs, support Alt 1-3.

- No spec change is necessary.

    - Different codebook types for unicast and multicast

In RAN1#107-e the following agreement was made [4].

Agreement When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot, - UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast. - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI. - FFS: when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast - Define a UE capability

Type-3 HARQ-ACK codebook will naturally include ACK/NACK bits for multicast since HARQ-ACK feedback for all HARQ PIDs will be included in Type-3 HARQ-ACK codebook. So there will be no need to generate a separate codebook for multicast for Type-3 HARQ-ACK codebook. gNB can avoid sending a Type-3 HARQ-ACK codebook and a multicast HARQ-ACK codebook in the same slot by scheduling.

Enhanced Type-3 HARQ-ACK codebook contains HARQ-ACK for a subset of HARQ PIDs for a subset of configured serving cells. Since gNB can control whether to include multicast HARQ-ACK in enhanced Type-3 HARQ-ACK codebook, there would be no need to generate a separate multicast HARQ-ACK codebook for enhanced Type-3 HARQ-ACK codebook transmission. As in the case of Type-3 HARQ-ACK codebook, transmission of enhanced Type-3 HARQ-ACK codebook and multicast HARQ-ACK codebook in the same slot can be avoided by scheduling.

Proposal 3: UE does not expect to be scheduled to report HARQ-ACK for unicast with Type-3 HARQ-ACK codebook or enhanced Type-3 HARQ-ACK codebook and HARQ-ACK for multicast with different codebook type with the same priority in the same PUCCH slot.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If, for unicast and multicast HARQ-ACK information of same priority value, a UE - is provided - either pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16 and pdsch-HARQ-ACK-Codebook-Multicast = semi-static, - or pdsch-HARQ-ACK-Codebook = semi-static and pdsch-HARQ-ACK-Codebook-Multicast = dynamic, and - would multiplex the unicast and multicast HARQ-ACK information in a same PUCCH or PUSCH the UE - appends the HARQ-ACK codebooks for the multicast HARQ-ACK information to the HARQ-ACK codebooks for the unicast HARQ-ACK information - if OACK+OSR+OCSI≤11, the UE determines nHARQ-ACK for obtaining a power of a PUCCH transmission with the HARQ-ACK information, as described in clause 7.2.1, as a sum of the nHARQ-ACK value from clause 9.1.2.1 or clause 9.1.3.3 and the nHARQ-ACK value from clause 9.1.3.1. A UE does not expect to be scheduled to transmit a Type-3 HARQ-ACK codebook for unicast HARQ-ACK information and a Type-1 HARQ-ACK codebook or a Type-2 HARQ-ACK codebook for multicast HARQ-ACK information with the same priority value in the same slot. A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

    - Multiplexing multicast HARQ-ACK with unicast HARQ-ACK

In previous meetings the following agreements were made [1].

Agreement: For the cases of HARQ-ACK feedback (at least for ACK/NACK based feedback) is available for multicast and unicast for a given UE receiving multicast, for determining the PUCCH resource, - Support multiplexing for the same priority and prioritizing for different priorities at least when the corresponding PUCCH resources overlap in time in a slot. - FFS whether it is subject to UE capability. - FFS the case of non-overlapping PUCCHs resources for HARQ-ACK in the same slot. - FFS whether sub-slot based PUCCH transmission for HARQ-ACK is supported. - FFS the case of HARQ-ACK feedback for multicast and other UCI for unicast. Agreement When UE is configured with different codebook types for unicast and multicast and when UE is scheduled to multiplex HARQ-ACK for unicast and HARQ-ACK for multicast with the same priority in the same PUCCH slot, - UE generates two separate sub-codebooks for unicast and multicast respectively and then concatenates them by appending sub-codebook for multicast to the sub-codebook for unicast. - Note: The PUCCH resource for transmitting the codebook is based on the last unicast DCI. - FFS: when Type-3 HARQ-ACK codebook or enhanced Type-2 codebook is used for unicast - Define a UE capability

Conditions for multiplexing unicast HARQ-ACK and multicast HARQ-ACK are not yet completely clear. There are two possible alternatives.

- Alt 1: Multiplexing is applied when two PUCCHs with HARQ-ACK feedback overlap

- Alt 2: Multiplexing is applied when two HARQ-ACK feedbacks are reported in the same slot

For Alt 1, one important note of the current specification is that HARQ-ACK codebook generation is done before PUCCH resource determination. At the time of codebook generation, the UE will generate separate codebooks for unicast and multicast and then select respective PUCCH resources. After that, if the PUCCH resources overlap, multiplexing is applied. When multiplexing is applied, the codebook must be regenerated. PUCCH resource reselection is also necessary. UE behavior becomes quite complicated. In addition, it is necessary to discuss what to do when there is no overlap. Supporting two PUCCHs with HARQ-ACK in the same slot violates existing principles.

In Alt 2, a UE first checks to see if it is scheduled to report unicast HARQ-ACK feedback and multicast HARQ-ACK feedback in the same slot. If both HARQ-ACK feedbacks are reported in the same slot, the UE generates a HARQ-ACK codebook containing both unicast HARQ-ACK and multicast HARQ-ACK; otherwise the UE generates a codebook containing only either unicast HARQ-ACK or multicast HARQ-ACK. No codebook regeneration is performed afterwards. It also does not violate existing principles. Based on these analysis, we support Alt 2.

Proposal 4: Multiplexing is applied when unicast HARQ-ACK feedback and multicast HARQ-ACK feedback are reported in the same slot.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = semi-static, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = dynamic, the UE generates a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. If a UE is scheduled to report unicast HARQ-ACK information and multicast HARQ-ACK information in a same slot, the UE multiplexes unicast HARQ-ACK information and multicast HARQ-ACK information in a HARQ-ACK codebook. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

    - HARQ-ACK feedback for SPS GC-PDSCH

In previous meetings the following agreements were made [2][3].

Agreement: Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling. - FFS for SPS activation/deactivation. Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported. Agreement: For multicast SPS PDSCH without PDCCH scheduling, HARQ-ACK feedback option is configured by UE RRC signalling. - FFS: Whether the configuration is per SPS configuration index or per G-CS-RNTI. - Note: Whether there is a UE capability for support of NACK-only based HARQ-ACK or not will be discussed as part of UE features discussion

Based on these agreements, it seems ambiguous how UEs configured with NACK-only based feedback as a feedback option for multicast SPS provide HARQ-ACK feedback for multicast SPS activation/deactivation. UEs configured with NACK-only based feedback as a feedback option for multicast SPS should provide ACK/NACK based feedback only for SPS activation/deactivation and NACK-only based feedback for multicast SPS PDSCH without PDCCH scheduling.

Proposal 5: A UE configured with NACK-only based feedback as a feedback option for multicast SPS provides ACK/NACK based feedback only for SPS activation/deactivation and NACK-only based feedback for multicast SPS PDSCH without PDCCH scheduling.

- Adopt the following text proposal in clause 18 of TS 38.213 to clarify the HARQ-ACK reporting mode for SPS GC-PDSCH activation/deactivation.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable when a number of HARQ-ACK information bits is more than four, or for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. If a UE is configured with the second HARQ-ACK reporting mode for a G-CS-RNTI, the UE provides HARQ-ACK information for the first SPS PDSCH reception after activation of SPS PDSCH receptions or an SPS PDSCH release using the G-CS-RNTI according to the first HARQ-ACK reporting mode and provides HARQ-ACK information for the other SPS PDSCH according to the second HARQ-ACK reporting mode. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

  - NACK-only based HARQ-ACK feedback for multicast

    - PUCCH resource for NACK-only based feedback

In RAN1#108-e the following agreement was made.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability

For the sake of commonality, it would be better to reuse the rules for ACK/NACK based feedback for NACK-only based feedback as much as possible. When Alt 1 (i.e., moreThanOneNackOnly mode 1) is applied, the PUCCH resource should be determined by the same method as ACK/NACK based feedback.

Proposal 6: In moreThanOneNackOnly mode 1, the PUCCH resource is determined as described in clause 9 and 9.2 through 9.2.3.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. If the UE is provided moreThanOneNackOnly-Mode = 'mode1', the PUCCH resource is determined as described in clause 9 and 9.2 through 9.2.3. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

In Alt 4 (i.e., moreThanOneNackOnly mode 2), the PUCCH slot to report HARQ-ACK feedback should be determined by k1 in the DCI scheduling multicast PDSCH. It is noted that “last DCI” is used not to determine PUCCH slot but to determine PUCCH resource in the indicated PUCCH slot. For the mapping between the HARQ-ACK bit sequence and PUCCH resources, a mapping that requires fewer PUCCH resources in the PUCCH resource list would be better when the number of HARQ-ACK bits is two or three. Therefore, we propose the following mapping.

Proposal 7: In moreThanOneNackOnly mode 2, the PUCCH transmission slot is determined by k1 in the DCI.

Proposal 8: For NACK-only based HARQ-ACK feedback for multicast, use the following table to determine the PUCCH resource.

| HARQ-ACK information |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits | 4 bits |  |
| ‘0’ | '00' | '000' | ‘0000’ | 1st PUCCH resource in the PUCCH resource set |
| ‘1’ | '10' | '100' | ‘1000’ | 2nd PUCCH resource in the PUCCH resource set |
|  | '01' | '010' | ‘0100’ | 3rd PUCCH resource in the PUCCH resource set |
|  | '11' | '110' | ‘1100’ | 4th PUCCH resource in the PUCCH resource set |
|  |  | '001' | ‘0010’ | 5th PUCCH resource in the PUCCH resource set |
|  |  | '101' | ‘1010’ | 6th PUCCH resource in the PUCCH resource set |
|  |  | '011' | ‘0110’ | 7th PUCCH resource in the PUCCH resource set |
|  |  | '111' | ‘1110’ | 8th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0001’ | 9th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1001’ | 10th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0101’ | 11th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1101’ | 12th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0011’ | 13th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1011’ | 14th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0111’ | 15th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1111’ | 16th PUCCH resource in the PUCCH resource set |

Whether to apply moreThanOneNackOnly mode 2 to a single G-RNTI or multiple G-RNTIs is one of the remaining issues. When applied to a single G-RNTI, the PUCCH resource is determined based on the HARQ-ACK information of a single G-RNTI. HARQ-ACK for different G-RNTIs are transmitted in different slots. It was pointed out at the last meeting that it would be difficult to make this scheme work in a typical TDD configuration (DDDSU) because of the small number of UL slots.

When applied to multiple G-RNTIs, the HARQ-ACK bits of multiple G-RNTIs are concatenated and the PUCCH resource is determined based on the concatenated HARQ-ACK bits. Concerns were raised at the last meeting that it may be difficult to configure PUCCH resources to be shared among UEs in a group when different UEs receive different G-RNTIs.

Concerns about multiple G-RNTIs can be addressed by gNB implementation. Applying to a single G-RNTI seems a bit restrictive. Therefore, we prefer to apply to multiple G-RNTIs. When multiple G-RNTIs are configured, a sub-codebook should be generated for each G-RNTI, and they should be concatenated.

Proposal 9: moreThanOneNackOnly mode 2 applies to multiple G-RNTIs.

- When a UE reports NACK-only based feedback for multiple G-RNTIs, the UE generates a sub-codebook for each G-RNTI and concatenates them in the order of G-RNTIs.

When moreThanOneNackOnly mode 2 is used, gNB should schedule PDSCH so that the number of HARQ-ACK bits is not more than 4. UE would not expect the number of HARQ-ACK bits to be larger than 4. In addition, when the number of PUCCH resources included in the configured PUCCH resource set is RPUCCH (e.g. 4 or 8), it would be valid that UE does not expect the number of HARQ-ACK bits to be larger than log2RPUCCH.

Proposal 10: When moreThanOneNackOnly mode 2 is used, a UE does not expect to report more than 4 bits or log2RPUCCH bits of HARQ-ACK, where RPUCCH is the number of PUCCH resources in the configured PUCCH resource set.

- Adopt the following text proposal in clause 18 of 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. If the UE is provided moreThanOneNackOnly-Mode = 'mode2', the UE does not expect to report more than 4 bits or log2RPUCCH bits of HARQ-ACK information. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

    - HARQ-ACK codebook type for NACK-only based feedback

Dynamic codebook can be used for NACK-only based feedback. However, it would be difficult to use semi-static codebook for NACK-only based feedback because NACK-only based feedback can only transmit up to 4 bits. Also, if semi-static codebook is used for NACK-only based feedback, a UE will transmit HARQ-ACK feedback except when the UE correctly receives PDSCHs in all PDSCH occasions. That is, if the UE receives PDSCHs only in part of PDSCH occasions, the UE need to transmit NACK-only based feedback regardless of the reception results. It leads to useless HARQ-ACK feedback transmission. Only dynamic codebook should be supported for NACK-only based feedback.

Proposal 11: For NACK-only based HARQ-ACK feedback for multicast, support only dynamic HARQ-ACK codebook.

    - Multiplexing NACK-only based feedback with ACK/NACK based feedback

In RAN1#108-e the following agreements were made.

Agreement When PUCCH transmission for the NACK-only based feedback for one G-RNTI collides with PUCCH transmission for ACK/NACK feedback for another G-RNTI with the same priority, support UE multiplexing the NACK-only based feedback with the ACK/NACK feedback onto the same PUCCH by transforming NACK-only into the ACK/NACK based HARQ-ACK bit. - Note: When the TB configured with NACK-only feedback is correctly decoded, the ACK will be transmitted and multiplexed with others. Agreement For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the following Alt2 (from the previous agreement) is adopted: - Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback. - Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

It was agreed that multiplexing is applied when a PUCCH transmission for NACK-only based feedback collides with another PUCCH transmission for ACK/NACK based feedback with the same priority. However, the definition of “collide” is not clear. There are two possible definitions.

- Alt 1: Multiplexing is applied when two PUCCHs with HARQ-ACK feedback overlap

- Alt 2: Multiplexing is applied when two HARQ-ACK feedbacks are reported in the same slot

As discussed in section 2.1.3, Alt 1 has significant problems. Alt 2 should be supported.

Proposal 12: Multiplexing is applied when NACK-only based HARQ-ACK feedback and ACK/NACK based HARQ-ACK feedback are reported in the same slot

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE would report HARQ-ACK information according to the first HARQ-ACK reporting mode and HARQ-ACK information of same priority value according to the second HARQ-ACK reporting mode in a same slot, the UE multiplexes their HARQ-ACK information. If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

    - Multiplexing NACK-only based feedback with SR

In RAN1#106bis-e the following agreement was made.

Agreement: When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority, support UE multiplexing the NACK-only based feedback with the HARQ-ACK feedback/CSI on PUCCH or on to PUSCH by transforming NACK-only into the ACK/NACK HARQ bit. - This applies to at least the case of the feedback addressing one TB. NACK-only based feedback for more than one TBs is to be handled separately. - Note: When the TB is correctly decoded, the ACK will be transmitted and multiplexed with others. - FFS: The case of PUCCH for SR.

In the current specification, when a PUCCH format 0 with HARQ-ACK overlaps with a PUCCH with positive SR, the SR and the HARQ-ACK are multiplexed and transmitted in the PUCCH format 0 using a different cyclic shift value. If the rule is directly applied to a PUCCH resource for NACK-only based feedback, gNB will not be able to identify which UE sent the SR since PUCCH resources for NACK-only based feedback will be shared among UEs in a group.

In such a case, it would be better to transform the NACK-only based feedback to ACK/NACK based HARQ-ACK bit(s), as in the case of multiplexing NACK-only based feedback and unicast HARQ-ACK/CSI. Then the HARQ-ACK bit(s) and the SR should be multiplexed and transmitted in another PUCCH resource for ACK/NACK based feedback for unicast, which is different from the PUCCH resource for NACK-only feedback and would be different among UEs. Then SR will be transmitted in the UE-specific PUCCH resource, so gNB can identify which UE sent the SR.

Proposal 13: When a PUCCH format 0 with NACK-only based feedback for multicast overlaps with a PUCCH with positive SR, the NACK-only based feedback is transformed into ACK/NACK based HARQ-ACK bits, multiplexed with SR, and transmitted in another PUCCH resource for ACK/NACK based feedback for unicast.

- Conclusion

In this contribution, we have following proposals:

Proposal 1: For Type-1 HARQ-ACK codebook generation for multicast

- When multiple G-RNTIs are configured, support Alt 1.

- When HARQ-ACK feedback for dynamic multicast scheduling is enabled and HARQ-ACK feedback for multicast SPS is disabled, support Alt 1.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = semi-static, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. The UE reports a NACK value for a HARQ-ACK information bit corresponding to a transport block with HARQ-ACK feedback disabled in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 2 For how to count the total number of HARQ-ACK bits for all configured G-RNTIs, support Alt 1-3.

- No spec change is necessary.

Proposal 3: UE does not expect to be scheduled to report HARQ-ACK for unicast with Type-3 HARQ-ACK codebook or enhanced Type-3 HARQ-ACK codebook and HARQ-ACK for multicast with different codebook type with the same priority in the same PUCCH slot.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If, for unicast and multicast HARQ-ACK information of same priority value, a UE - is provided - either pdsch-HARQ-ACK-Codebook = dynamic or pdsch-HARQ-ACK-Codebook-r16 and pdsch-HARQ-ACK-Codebook-Multicast = semi-static, - or pdsch-HARQ-ACK-Codebook = semi-static and pdsch-HARQ-ACK-Codebook-Multicast = dynamic, and - would multiplex the unicast and multicast HARQ-ACK information in a same PUCCH or PUSCH the UE - appends the HARQ-ACK codebooks for the multicast HARQ-ACK information to the HARQ-ACK codebooks for the unicast HARQ-ACK information - if OACK+OSR+OCSI≤11, the UE determines nHARQ-ACK for obtaining a power of a PUCCH transmission with the HARQ-ACK information, as described in clause 7.2.1, as a sum of the nHARQ-ACK value from clause 9.1.2.1 or clause 9.1.3.3 and the nHARQ-ACK value from clause 9.1.3.1. A UE does not expect to be scheduled to transmit a Type-3 HARQ-ACK codebook for unicast HARQ-ACK information and a Type-1 HARQ-ACK codebook or a Type-2 HARQ-ACK codebook for multicast HARQ-ACK information with the same priority value in the same slot. A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 4: Multiplexing is applied when unicast HARQ-ACK feedback and multicast HARQ-ACK feedback are reported in the same slot.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = semi-static, the UE generates a Type-1 HARQ-ACK codebook as described in clauses 9.1.2, 9.1.2.1, and 9.1.2.2. If a UE is provided pdsch-HARQ-ACK-Codebook-Multicast = dynamic, the UE generates a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. If a UE is scheduled to report unicast HARQ-ACK information and multicast HARQ-ACK information in a same slot, the UE multiplexes unicast HARQ-ACK information and multicast HARQ-ACK information in a HARQ-ACK codebook. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 5: A UE configured with NACK-only based feedback as a feedback option for multicast SPS provides ACK/NACK based feedback only for SPS activation/deactivation and NACK-only based feedback for multicast SPS PDSCH without PDCCH scheduling.

- Adopt the following text proposal in clause 18 of TS 38.213 to clarify the HARQ-ACK reporting mode for SPS GC-PDSCH activation/deactivation.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, the UE does not transmit a PUCCH that would include only HARQ-ACK information with ACK values. The second HARQ-ACK reporting mode is not applicable when a number of HARQ-ACK information bits is more than four, or for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. If a UE is configured with the second HARQ-ACK reporting mode for a G-CS-RNTI, the UE provides HARQ-ACK information for the first SPS PDSCH reception after activation of SPS PDSCH receptions or an SPS PDSCH release using the G-CS-RNTI according to the first HARQ-ACK reporting mode and provides HARQ-ACK information for the other SPS PDSCH according to the second HARQ-ACK reporting mode. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 6: In moreThanOneNackOnly mode 1, the PUCCH resource is determined as described in clause 9 and 9.2 through 9.2.3.

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. If the UE is provided moreThanOneNackOnly-Mode = 'mode1', the PUCCH resource is determined as described in clause 9 and 9.2 through 9.2.3. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 7: In moreThanOneNackOnly mode 2, the PUCCH transmission slot is determined by k1 in the DCI.

Proposal 8: For NACK-only based HARQ-ACK feedback for multicast, use the following table to determine the PUCCH resource.

| HARQ-ACK information |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| 1 bit | 2 bits | 3 bits | 4 bits |  |
| ‘0’ | '00' | '000' | ‘0000’ | 1st PUCCH resource in the PUCCH resource set |
| ‘1’ | '10' | '100' | ‘1000’ | 2nd PUCCH resource in the PUCCH resource set |
|  | '01' | '010' | ‘0100’ | 3rd PUCCH resource in the PUCCH resource set |
|  | '11' | '110' | ‘1100’ | 4th PUCCH resource in the PUCCH resource set |
|  |  | '001' | ‘0010’ | 5th PUCCH resource in the PUCCH resource set |
|  |  | '101' | ‘1010’ | 6th PUCCH resource in the PUCCH resource set |
|  |  | '011' | ‘0110’ | 7th PUCCH resource in the PUCCH resource set |
|  |  | '111' | ‘1110’ | 8th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0001’ | 9th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1001’ | 10th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0101’ | 11th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1101’ | 12th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0011’ | 13th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1011’ | 14th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘0111’ | 15th PUCCH resource in the PUCCH resource set |
|  |  |  | ‘1111’ | 16th PUCCH resource in the PUCCH resource set |

Proposal 9: moreThanOneNackOnly mode 2 applies to multiple G-RNTIs.

- When a UE reports NACK-only based feedback for multiple G-RNTIs, the UE generates a sub-codebook for each G-RNTI and concatenates them in the order of G-RNTIs.

Proposal 10: When moreThanOneNackOnly mode 2 is used, a UE does not expect to report more than 4 bits or log2RPUCCH bits of HARQ-ACK, where RPUCCH is the number of PUCCH resources in the configured PUCCH resource set.

- Adopt the following text proposal in clause 18 of 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> For the second HARQ-ACK reporting mode, when a number of HARQ-ACK information bits is 2, 3, or 4, the UE can be indicated by moreThanOneNackOnly-Mode to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode or by selecting a resource from a set of resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1. If the UE is provided moreThanOneNackOnly-Mode = 'mode2', the UE does not expect to report more than 4 bits or log2RPUCCH bits of HARQ-ACK information. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 11: For NACK-only based HARQ-ACK feedback for multicast, support only dynamic HARQ-ACK codebook.

Proposal 12: Multiplexing is applied when NACK-only based HARQ-ACK feedback and ACK/NACK based HARQ-ACK feedback are reported in the same slot

- Adopt the following text proposal in clause 18 of TS 38.213.

----------------------------------- Start of Text proposal to 18 of 38.213 ------------------------------------------------ <Unchanged text is omitted> If a UE would report HARQ-ACK information according to the first HARQ-ACK reporting mode and HARQ-ACK information of same priority value according to the second HARQ-ACK reporting mode in a same slot, the UE multiplexes their HARQ-ACK information. If a UE would multiplex HARQ-ACK information that the UE would provide according to the second HARQ-ACK reporting mode with other UCI in a first PUCCH, or in a PUSCH, as described in clauses 9 and 9.2.5, the UE provides the HARQ-ACK information according to the first HARQ-ACK reporting mode. For resolving an overlapping among a second PUCCH with HARQ-ACK information according to the second HARQ-ACK reporting mode and other PUCCHs or PUSCHs prior to multiplexing the HARQ-ACK information in a PUCCH or PUSCH, the UE considers that the UE would transmit the second PUCCH when all values of the HARQ-ACK information are ‘ACK’. <Unchanged text is omitted> ----------------------------------- End of Text proposal to 18 of 38.213 ------------------------------------------------

Proposal 13: When a PUCCH format 0 with NACK-only based feedback for multicast overlaps with a PUCCH with positive SR, the NACK-only based feedback is transformed into ACK/NACK based HARQ-ACK bits, multiplexed with SR, and transmitted in another PUCCH resource for ACK/NACK based feedback for unicast.

- References

- Draft Report of 3GPP TSG RAN WG1 #104-e v0.3.0

- Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0

- Draft Report of 3GPP TSG RAN WG1 #106b-e v0.1.0

- Draft Report of 3GPP TSG RAN WG1 #107-e v0.1.0

- Draft Report of 3GPP TSG RAN WG1 #108-e v0.1.0