3GPP TSG RAN WG1 #109-e   	           R1-2204356

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel-17 FeMIMO UE features

Agenda Item:	8.16.1

Document for: 	Discussion and Decision

Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e is captured in R1-2202929. In this contribution, we provide our views on Rel.17 FeMIMO UE features.

Discussions

  - Multi-beam Operation

For FG23-1-1, we have the following suggestion:

- FG23-1-1 should include both intra-and inter cell. It is not necessary to report the same set of capabilities for intra cell and inter cell separately.

- Component 2: We support to confirm the WA with “per BWP per CC”, considering that TCI state is configured per BWP. We don’t believe reporting “per band combination” is necessary.

- Component 3: We believe reporting per CC in a band is enough. Or, it is also fine to remove all of “[in a band] [in a band combination]. We don’t believe reporting “per band combination” is necessary.

- Component 3: We don’t have strong view on “[mode]”, but we can simply say as “TCI state indication for update and activation”. We think it is good to clarify a) means supporting only one active TCI state, but we are also fine to remove it, because it is obvious.

For FG23-1-1b, we have the following suggestion:

- FG23-1-1b should include both intra-and inter cell. It is not necessary to report the same set of capabilities for intra cell and inter cell separately.

- Component 1: We don’t have strong view on “[mode]”, but we can simply say as “TCI state indication for update and activation”.

- Component 2: We agree to have component 2. Beam application timing is counted from HARQ-ACK, which is already specified as higher layer parameter BeamAppTime_r17, and its value range is agreed as {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336} symbol in RAN1#108e. We can use the agreed value range for value range of component 2. We can clarify “ 84, 98, 112, 224, 336” are only for FR2/FR2-2.

Agreement (RAN1#108e) The value range of beamAppTime-r17 is (1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336) symbols. - Discuss the applicability of 84, 98, 112, 224, 336 for FR2/FR2-2 in UE features session - These values are not applicable for FR1

- Component 3: We don’t believe reporting “per band combination” is necessary.

- Component 4: We believe we should remove component 4. The main intention of the agreement in RAN1#105e is to agree time gap between HARQ-ACK and beam application timing. It also mentions the time gap between PDCCH and beam application timing, however, it is just a note, and our understanding of the note is the Rel.15 UE capability of timeDurationForQCL. The Rel.15 UE capability of timeDurationForQCL is only reported in FR2, hence we don’t need to care about the time gap between PDCCH and beam application timing, because Rel.15-16 does not have such a time gap in FR1. Moreover, if UE has concern of the short time gap, UE can simply report the large value by component 2. Hence, there is no need to have component 4.

Agreement (RAN1#105e) On Rel-17 DCI-based beam indication, regarding application time of the beam indication, the first slot that is at least X ms or Y symbols after the last symbol of the acknowledgment of the joint or separate DL/UL beam indication. - Note: The gap between the last symbol of the beam indication DCI and that first slot shall satisfy the UE capability - FFS: Application time and whether additional offset is needed for the application time in case of cross carrier beam indication and common TCI state ID update across a set of configured CCs if CCs have different SCSs - FFS: Whether inter-cell beam switching needs higher X/Y values than intra-cell - FFS: Whether application time can be indicated/determined dynamically for different scenarios, e.g. cross CC, inter-cell, inter-panel without reverting previous RAN1 agreements

For FG23-1-1c, we have the following suggestion:

- Component 2: We don’t think the component 2 is needed, because we don’t need to repeat UE capabilities of all Rel.16 features by Rel.17 UE capability. Meanwhile, we can accept to have component 2. For“ [in a band with SpCell BFR]”, we understand Rel.16 issue if UE reports the number of CCs for SCell BFR as X, it means UE needs to support X+1 BFR in a band with SpCell, because gNB may configure both SCell BFR and SpCell BFR in the same band. However, if we agree with “in a band with SpCell BFR”, it is not clear the value of “in a band without SpCell BFR”. Considering that the issue already exist in Rel.16, we can simply remove [in a band with SpCell BFR]. If this is controversial, we simply suggest to remove component 2.

For FG23-1-1d/23-1-1e, we have the following suggestion:

- As we commented in RAN1#108e, we believe it is important to make basic function that all UE support. Considering that we have the following agreement that UE capability of the max number of TCI state pools, we believe we should make FG23-1-1e as basic function.

- Component 2 of FG23-1-1e, we don’t believe multiple TCI state pools can be configured for FG23-1-1e, and UE only need to report “1”. Hence, we believe component 2 can be removed.

Agreement (RAN1#106e) On Rel.17 unified TCI framework, confirm the following working assumption as an agreement with a minor refinement highlighted in red For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs: - RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16 - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported - RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC - When the BWP/CC ID (i.e. bwp-Id or cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies - Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1 - FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band - FFS: How to define reference BWP/CC

For FG23-1-1h/23-1-1i/23-1-1j, we are fine to remove [ ].

For FG23-1-2, we have the following suggestion:

- Component 5/6/8/9: gNB cell-specifically transmit SSBs and the SSB location is determined to operate network for Rel.15 UEs. Even if Rel.17 UE has preference of SSB location of different cells, it is very difficult for gNB to change SSB location only for the Rel.17 UE. Hence, these components should be removed.

- Pre-requisite as FG2-24/2-29: We don’t understand why we need the pre-requisite features only for FG23-1-2.

For FG23-1-3, we have the following suggestion:

- Component 1: we support to keep the text “PHR”

- Component 3: we support current candidate values.

For FG23-1-4, we think the text “sets” can be removed. Candidate value ‘0’ can be removed since we had a conclusion that there is no consensus to support DL only panel. “This FG is a working assumption” can be removed since we had reached an agreement in the last meeting.

Proposal 1: Adopt the following for Rel-17 Multi-beam Operation.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - The maximum number of configured joint TCI states per BWP per CC in a band - One MAC-CE activated joint TCI state per CC in a band - TCI state indication for update and activation a) MAC CE based TCI state indication for one active TCI state - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band |  | Yes |  | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management is not supported | Per band | n/a | n/a | n/a | FFS: how to count the MAC-CE activated joint TCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication for update and activation b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1 | Yes |  | Unified TCI with joint DL/UL TCI update for intra- and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336} Note: values larger than 70 is FR2/FR2-2 only. Component 3 candidate values: [{2, …}] Note: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in 23-1-1, component 5 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1c | SCell BFR with unified TCI framework | 1. Support of SCell BFR with unified TCI framework 2. Maximum number of CCs configured with SCell BFR with unified TCI framework |  | Yes |  | SCell BFR with unified TCI framework is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1d | Per BWP TCI state pool configuration for CA mode | 1. Support of TCI state pool configuration per BWP for CA mode | 23-1-1 | Yes |  | Per BWP TCI state pool configuration for CA mode is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1e | TCI state pool configuration with TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC | 23-1-1 | Yes |  | TCI state pool configuration with TCI pool sharing for CA mode is not supported | Per band | n/a | n/a | n/a | A UE that supports 23-1-1 together with CA must support this FG] | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1f | Common multi-CC TCI state ID update and activation | Common multi-CC TCI state ID update and activation | 23-1-1 | Yes |  | Common multi-CC TCI state ID update and activation is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1g | Beam misalignment between the DL source RS in the TCI state | Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS | 23-1-1 | Yes |  | Beam misalignment between the DL source RS in the TCI state is not supported | Per band | n/a | n/a | n/a | FR2 only | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1h | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS | 23-1-1 | Yes |  | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS is not supported | Per band | n/a | n/a | n/a | Note: A UE that supports FG 23-1-1 must indicate this FG is supported | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1i | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH , and SRS | Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH , and SRS (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH , and SRS reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band | n/a | n/a | n/a | Note: This has no impact on detail signaling design for SRS TCI indication A UE that supports 23-1-1 must indicate this FG is supported | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band | n/a | n/a | n/a | A UE that supports 23-1-1 must indicate this FG is supported | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting (for inter-cell BM and mTRP) | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K=4 SSBRI-RSRP pairs in one report where at least one pair can be associated with a PCI different from serving cell PCI can be reported 3. The maximum number of RRC-configured PCI(s) different from serving cell PCI for L1-RSRP measurement 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI per CC 7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC} |  | Yes |  | Inter-cell beam measurement and reporting (for inter-cell BM and mTRP) is not supported | per band | n/a | n/a | n/a | [Note: Whether component 4 and/or 5 are also counted in FG16-1g/16-1g-1] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced PHR reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  | Yes |  | [MPE mitigation is not supported] | Per Band | n/a | n/a | n/a | 2. Candidate value of {1,2,3, 4} 3. Candidate value {8, 12, 16, 28, 32, 48, 64} Note: FR2 only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | 1. Supported UE capability value sets and corresponding max number of SRS ports for each UE capability value set |  | Yes |  | [MPUE support for UL is not supported] | per band | n/a | n/a | n/a | Component 1 candidate values: Up to 4 value sets each with one value of {1,2,4} Note: the reported list contains only unique value sets | Optional with capability signalling |

  - Multi-TRP for PDCCH

For FG23-2-1, we have following suggestion.

- Component 1: we think all the text in yellow can be removed. PDCCH repetition based on two linked SS sets already refers to TDM and FDM scheme (non-SFN scheme). “except FDM based PDCCH repetition for FR2”can be remove because we have FG23-2-2 for two QCL-TypeD for FDM repetition in FR2, so we do not need to make an exception of FDM in FR2 in this FG.

For FG23-2-1a, the prerequisite FG should be kept.

For FG23-2-2, we think the text “with non-SFN TDM and/or FDM scheme” can be kept since this FG is only for non-SFN.

Proposal 2: Adopt the following for Rel-17 M-TRP PDCCH

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | Yes |  | PDCCH repetition is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, FFS more} Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot Note: for component 3, each unique pair of overlaps is counted as one. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | 23-2-1 | Yes |  | Monitoring of individual candidates is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition with non-SFN TDM and/or FDM scheme | 23-2-1 | Yes |  | Two QCL TypeD for CORESET monitoring in PDCCH repetition is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-4 | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | Support of simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP | 23-2-1, 16-2a | Yes |  | Simultaneous configuration of PDCCH repetition and multi-DCI based multi-TRP is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |

  - Multi-TRP for PUSCH, PUCCH

For M-TRP PUSCH, for FG23-3-1, we have following suggestion.

- For prerequisite FG, we think FG2-14 (i.e., Codebook based PUSCH MIMO transmission) should be its prerequisite FG

- For component 4, we support ‘4’ as candidate value. 4 SRS resources can be configured for full power mode 2 and we think M-TRP PUSCH repetition can be supported together with full power mode 2.

Proposal 3: Adopt the following for Rel-17 M-TRP PUSCH

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) -codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 3. Support of two SRS resource sets with usage set to 'codebook' 4. Supported number of SRS resources in one SRS resource set | 2-14 | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for codebook based | per FS | n/a | n/a | n/a | Component 4 candidate values: {1,2 ,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-2 | Multi-TRP PUSCH repetition (type A) - non-codebook based | 1. Support of multi-TRP PUSCH repetition for non-codebook based PUSCH (based on PUSCH repetition type A) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'nonCodebook' 3. Supported number of SRS resources in one SRS resource set | 2-15 | Yes |  | Multi-TRP PUSCH repetition (type A) is not supported for non-codebook based | per FS | n/a | n/a | n/a | Component 3: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2a | Two associated CSI-RS resources | Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | 2-15a, 23-3-1-2 | Yes |  | Two associated CSI-RS resources are not supported | Per Band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-2b | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 4. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 5. UE can process up to X CSI-RS resources associated with SRS for non-codebook based transmission simultaneously | 23-3-1-2a | Yes |  | CSI-RS processing framework for SRS with two associated CSI-RS resources is not supported | Per Band | n/a | n/a | n/a | Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} Component 5: {1,2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping for Multi-TRP PUSCH repetition | Support of cyclic mapping when the number of repetitions is larger than 2 | 23-3-1 or 23-3-1-2 | Yes |  | Cyclic mapping for Multi-TRP PUSCH repetition is not supported | Per Band | n/a | n/a | n/a | Candidate component values: {for repetition Type A, for repetition Type B, both} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field for Multi-TRP PUSCH repetition | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | 23-3-1 or 23-3-1-2 | Yes |  | Second TPC field for Multi-TRP PUSCH repetition is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs.) | 23-3-1 or 23-3-1-2 | Yes |  | Two PHR reporting is not supported | Per Band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | Yes |  | A-CSI report on two PUSCH repetitions is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions | 23-3-1 or 23-3-1-2 | Yes |  | SP-CSI report on two PUSCH repetitions is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) | 23-3-1-1 or 23-3-1-2 | Yes |  | CG PUSCH transmission towards M-TRPs using a single CG configuration is not supported | Per Band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 -codebook based | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to ‘codebook’ 3. Supported number of SRS resources in one SRS resource set |  | Yes |  | Codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2[,4]} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-3 | Multi-TRP PUSCH repetition (type B) – non-codebook based | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. support of two SRS resource sets with usage set to ‘nonCodebook’ 3. supported number of SRS resources in one SRS resource set |  | Yes |  | Non-codebook based multi-TRP PUSCH repetition (type B) is not supported | Per FSPC | No | No | No | Component 3 candidate values: {1,2,3,4} | Optional with capability signalling |

For M-TRP PUCCH, for FG23-3-2, we have following suggestion.

- We support to keep the note “Note: power control parameter sets only apply to FR1”

Proposal 4: Adopt the following for Rel-17 M-TRP PUCCH

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition scheme 1 (inter-slot) | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition)- sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of up to two PUCCH power control parameter sets/spatial relation info per PUCCH resource | FFS | Yes |  | PUCCH repetition scheme 1 (inter-slot repetition) is not supported | Per band | n/a | n/a | n/a | Note: power control parameter sets only apply to FR1 Note: spatial relation info only applies to FR2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-2b | Cyclic mapping for multi-TRP PUCCH repetition | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | 23-3-2 | Yes |  | Cyclic mapping for multi-TRP PUCCH repetition is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c | Second TPC field for multi-TRP PUCCH repetition | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | 23-3-2 | Yes |  | Second TPC field for multi-TRP PUCCH repetition is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions | FFS | Yes |  | PUCCH repetition scheme 3 (intra-slot repetition) is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |

  - Inter-cell multi-TRP

For component 2, the candidate value of 0 is not needed because the deployment scenario in component 2 is important for commercial NW. Hence, case 1 in component 2 could be defined as a default case for inter-cell MTRP operation. We’re open for other candidate values.

Regarding the last note, it is true that case1 and case2 cannot be enabled simultaneously. However, it is related to NW configuration instead of UE feature, thus, the note can be removed.

Proposal 5: Adopt the following for Rel-17 inter-cell multi-TRP.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. The maximum number of configured additional PCIs per CC is X1 (case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1] | FFS | Yes |  | IntCell-mTRP is not supported | Per band | n/a | n/a | n/a | [Component 2 candidate values: {0,1,2,3,[ 4, 5, 6,] 7}] [Component 3 candidate values: {0,1,2,3, [ 4, 5, 6,]7}] Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Beam management for multi-TRP

For 23-5-2a, one PUCCH-SR for BFR should be baseline, like Rel-16 BFR. Hence, the note should be kept.

For 23-5-2b, the note is also supported as this is the main motivation to configure two PUCCH-SR resources.

Proposal 6: Adopt the following for Rel-17 beam management for M-TRP.

| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancement based on two BFD-RS sets | 1. Support of the Maximum number of supported BFD-RS resources per set per BWP 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] [3. Support one PUCCH-SR resource for MTRP BFRQ] 3. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP |  | Yes |  | MTRP BFR based on two BFD-RS sets is not supported | Per band | n/a | n/a | n/a | [Component 1 candidate values: {1, 2,…}] Component 3 candidate values: {2,3,4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Max number of PUCCH-SR resources for MTRP BFRQ per cell group [2. association between a BFD-RS resource set on SpCell and a PUCCH SR resource (if component candidate value equals 2)] |  | Yes |  | PUCCH-SR resources for MTRP BFRQ is not supported | Per UE | No | Yes | No | Component candidate values: {[0,1], 2} Note: A UE that supports FG 23-5-2 must indicate this FG is supported with at least component candidate value 1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2b | Association between a BFD-RS resource set on SpCell and a PUCCH SR resource | Support of association between a BFD-RS resource set on SpCell and a PUCCH SR resource | 23-5-2a | Yes |  | Association between a BFD-RS resource set on SpCell and a PUCCH SR resource is not supported | Per UE | No | Yes | No | Note: A UE that supports FG 23-5-2a with candidate value 2 must indicate this FG is supported with at least component candidate value 1 | Optional with capability signalling |

  - HST-SFN deployment

We have the following comments:

- For SFN scheme A (FG23-6-1/23-6-1-1/23-6-1b) and SFN scheme B (FG23-6-2/23-6-2b), at least one FG for each SFN scheme should be a basic function to avoid market fragmentation. During the RAN1 discussion for Rel.17 HST, the assumption of the basic operation is a combination of “ SFN-PDCCH + SFN-PDSCH” (i.e. FG23-6-1 and FG23-6-2), and some UE vendors claim difficulty of supporting other combinations. Hence, the basic capability of SFN scheme A should be FG23-6-1 and the basic capability of SFN scheme B should be FG23-6-2. We can make pre-requisite feature of FG23-6-1-1/23-6-1b as “FG23-6-1” and pre-requisite feature of FG23-6-2b as “FG23-6-2”. It means UE supporting FG23-6-1-1/23-6-1b must support FG23-6-1, or UE supporting FG23-6-2b must support FG23-6-2. We believe we should clarify that FG23-6-1 or FG23-6-2 is a basic function of each SFN scheme in the UE feature list.

- For FG 23-6-2a, we agree to remove [] of pre-requisite feature. Similar as FG23-6-1a, pre-requisite feature of 23-6-2a should be “23-6-2 or 23-6-2b”.

- For FG 23-6-4/23-6-4a, we agree to have pre-requisite feature as FG23-6-1 or FG23-6-2. As we discussed above, FG23-6-1 and FG23-6-2 are basic FGs of SFN schemes. All UE supports SFN schemes shall support either/both of FG23-6-1 or/and FG23-6-2.

Proposal 7: Adopt the following for HST-SFN deployment

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  | Yes |  | SFN scheme A (scheme 1) for PDSCH and PDCCH is not supported | per FS | n/a | n/a | n/a | Note: UE supporting SFN scheme(s) shall indicate either/both of FG23-6-1 or/and FG23-6-2. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1-1 | SFN scheme A (scheme 1) for PDCCH only | Support of SFN scheme A for PDCCH scheduling single TRP PDSCH | 23-6-1 | Yes |  | SFN scheme A (scheme 1) for PDCCH only is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and PDSCH SFN scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 or 23-6-1b | Yes |  | Dynamic switching - scheme A is not supported | per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH scheduled by single TRPPDCCH | 23-6-1 | Yes |  | SFN scheme A (scheme 1) for PDSCH only is not supported | per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH |  | Yes |  | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH is not supported | per FS | n/a | n/a | n/a | Note: UE supporting SFN scheme(s) shall indicate either/both of FG23-6-1 or/and FG23-6-2. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and PDSCH SFN scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 or 23-6-2b | Yes |  | Dynamic switching – scheme B is not supported | per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH scheduled by single TRP PDCCH | 23-6-2 | Yes |  | SFN scheme B (TRP based pre-compensation) for PDSCH only is not supported | per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs (HST/URLLC) | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE | 23-6-1 or 23-6-2 | Yes |  | Simultaneous activation of two TCI states for PDCCH across multiple CCs is not supported | Per UE | No | Yes | No |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup for SFN | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI when PDSCH is scheduled with offset equal or larger than the threshold, if applicable 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold | 23-6-1 or 23-6-2 | Yes |  | Default DL beam setup for SFN is not supported | Per band | n/a | n/a | n/a | Note: FR2 only for component 1 and 3 only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4a | Default UL beam setup for SFN | 1. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured | 23-6-1 or 23-6-2 | Yes |  | Default UL beam setup for SFN is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |

  - SRS

In addition, RAN2 has sent an LS [1], asking the following:

A) R1 23-8-3 RAN2 understanding is that this feature is an extension of srs-TxSwitch/srs-TxSwitch-v1610 to support SRS antenna switching xTyR with y>4. However, RAN2 is unclear on how this new capability is populated for a band in a band combination where at least one band in the band combination supports xTyR with y>4 and how it works with the existing srs-TxSwitch/srs-TxSwitch-v1610. There are 2 interpretations: (a) the new capabilities is populated for a band in the band combination only if at least one xTyR entry for the band supports y>4 (b) the new capabilities is populated for a band in the band combination regardless of whether the band supports y>4 If the interpretation is (a), the following note will not occur since the candidate values with xTyR with y>4 are {t2r6, t1r6, t4r8, t2r8, t1r8} where x and y are never equal. For b), the Note can be applicable to, but only in band of the band combination where xTyR supports y ≤ 4, while not applicable in band of the band combination where at least one xTyR entry supports y>4. In addition RAN2 understands component 2 and 3 are not reported if x=y as below, and wonders if there is any other case that component 2 and 3 are not reported. Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y. For both interpretations (a) and (b), should the component 1 be set consistently with the existing supportedSRS-TxPortSwitch/ supportedSRS-TxPortSwitch-v1610 in srs-TxSwitch/srs-TxSwitch-v1610 (i.e. the R1 23-8-3 bitmap in component 1 be aligned with the existing supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610 for the xTyR entries where y<=4). Also how are component 2 and 3 being set with respect to the Rel-15/16 capabilities if xTyR entries in component 1 contains the xTyR in the existing Rel-15/16 capabilities RAN2 would also like to inform that R1 23-8-3 will be implemented similar to Rel-15/16 srs-TxSwitch/srs-TxSwitch-v1610 in the BandParameters of a band combination rather than in FeatureSetUplink as components 2 and 3 require setting them to the band entry of a combination.

Our views for each of RAN2 questions above are as follows:

- How FG23-8-3 is populated for a band in a BC where at least one band in the BC supports xTyR with y>4, our understanding is interpretation (a), which we believe is aligned with the component description.

- For “Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y”, we think it can be removed since this FG will be used for a band only where the UE supports more than 4 Rx ports, i.e., there would be no case that the UE doesn’t report any combination with more than 4 Rx ports.

- For the consistency of component 1 with the existing supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610, we do not think it should be considered since the targets of this component and supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610 are different. FG23-8-3 is for Rel-17 UEs implementing more than 4 Rx ports, while supportedSRS-TxPortSwitch/supportedSRS-TxPortSwitch-v1610 are for Rel-15/16 UE implementing at most 4 Rx ports. Also, FG23-8-3 can report the corresponding capability with finer granularity, which was argued as important by some vendors in our understanding.

For FG23-8-6, the following agreement in the last e-meeting should be captured. In other words, this FG should be divided into two, one for FH case, and the others for non-FH case.

Agreement RPFS is applicable for both frequency hopping and non-frequency hopping cases, where support of RPFS for non-FH case is an optional UE feature for UEs supporting RPFS.

For FG23-8-9, whether to have a note “Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports” is FFS. We are fine with having this note as it is.

We also believe the following should be reflected by introducing a new FG, as we suggest as FG23-8-10 below. Note that since this FG is to be similar to FG23-8-9, we think it may be necessary to update FG23-8-9 as well, so that the difference between them is clarified enough, as suggested on the table below.

Agreement (#108-e) FL Proposal 3-1: Support N = 1 for aperiodic SRS configuration for 1T4R - This new configuration is UE optional.

Also, for FG23-8-6/7/8/9, some typo corrections can be reflected as follows.

Proposal 8: Adopt the following for Rel-17 SRS.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | Yes |  | SRS triggering offset enhancement is not supported | Per band | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Yes |  | Triggering SRS only in DCI 0_1/0_2 is not supported | per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | Yes |  | SRS Antenna switching for >4Rx is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported | Per FS | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | Yes |  | Increased repetition for SRS is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS with frequency hopping | 2-52 | Yes |  | Partial frequency sounding of SRS with frequency hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6a | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS without frequency hopping | 2-52 | Yes |  | Partial frequency sounding of SRS without frequency hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodic SRS | 23-8-6 | Yes |  | Start RB location hopping for partial frequency SRS is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  | comb-8 for SRS other than for positioning is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Support of 4 aperiodic SRS resource sets for 1T4R, and 2 aperiodic resource sets for 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | Yes |  | SRS antenna switching with 4 aperiodic SRS resource sets for 1T4R, and 2 aperiodic SRS resource sets for 1T2R and 2T4R are not supported | Per FS | N/A | N/A | N/A | Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Support of 1 aperiodic SRS resource set for 1T4R | Support of 1 aperiodic SRS resource set for 1T4R | 2-53, 2-55 | Yes |  | Extension of aperiodic SRS configuration with 1 aperiodic SRS resource set for 1T4R is not supported | Per FS | N/A | N/A | N/A |  | Optional with capability signalling |

Summay

Based on the discussions on Rel.17 FeMIMO UE features in Session 2, we propose to adopt the updated UE feature tables for each UE feature.

Reference

- R2-2204360, Reply LS on updated Rel-17 RAN1 UE features list for NR”, RAN WG2, April 2022