3GPP TSG RAN WG1 #109-e		     R1-2204359

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for NR NTN

Agenda Item:	8.16.4

Document for: 	Discussion and Decision

- Introduction

Updated Rel.17 UE feature list after RAN1#108 including NR sidelink enhancement has been agreed [1]. In this contribution, we present our views and updates regarding the UE features for NR NTN based on the list.

- Discussion

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 26. NR_NTN_solutions; 26-1; Uplink Time and Frequency pre-compensation and timing relationship enhancements; - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information; No; No; Release 17 NR UE cannot communicate via access [NTN/satellite/HAPS/ATG]; [Per UE/per band]; No; No; No; An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris; Optional with capability signalling For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported]. 26. NR_NTN_solutions; 26-3; Enhancement on the timing relationship; - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated; [26-1][26-2]; No; No; Release 17 UE cannot access [NTN/ satellite/HAPS/ATG]; [Per UE/per band]; No; No; No; FFS: whether this FG gets merged with FG 26-1; Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported]. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 26. NR_NTN_solutions; 26-6; [HARQ feedback disabling for] Type-2 HARQ codebook Enhancement; - [Support of HARQ feedback disabling] - Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] FFS: UE supports HARQ disabling; Yes; No; Type-2 HARQ codebook enhancement is not supported for NR communication via satellite; [Per UE/per band]; No; No; No; Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported]. 26. NR_NTN_solutions; [26-6a]; [[HARQ feedback disabling for] Type-1 HARQ codebook enhancement] with HARQ disabling; - [Support of HARQ feedback disabling] - Support of Enhancement on Type-1 HARQ codebook enhancements for feedback-disabled HARQ processes in NTN - FFS: HARQ disabling; [26-1, 26-2]; Yes; No; Type-1 HARQ codebook enhancement is not supported for NR communication via satellite; [Per UE/per band]; No; No; [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] No; FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive; Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported.] [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported]. 26. NR_NTN_solutions; 26-6b; [HARQ feedback disabling for] Type-3 HARQ codebook enhancement with HARQ disabling; - [Support of HARQ feedback disabling] - Support of Type-3 HARQ codebook enhancements for feedback-disabled HARQ processes; Yes; No; Type-3 HARQ codebook enhancement is not supported for NR communication via satellite; per band; No; No; No; Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 26. NR_NTN_solutions; 26-5; Increasing the number of HARQ processes; The maximal supported HARQ process number is X for UL and Y for DL; Yes; No; Increased number of HARQ processes is not supported for NR communication via satellite; [Per band or per FSPC or per UE]; No; No; No; Candidate component values for (X,Y): {(16,32),(32,16),(32,32)}; Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported]. Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown 26. NR_NTN_solutions; 26-6; [HARQ feedback disabling for] Type-2 HARQ codebook enhancement; - [Support of HARQ feedback disabling] - Support of type-2 HARQ codebook enhancements when there are for feedback-disabled HARQ processes; FFS; Yes; No; Type-2 HARQ codebook enhancement is not supported for NR communication via satellite; per band; No; No; No; Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. 26. NR_NTN_solutions; 26-6a; [HARQ feedback disabling for] Type-1 HARQ codebook enhancement; - [Support of HARQ feedback disabling] - Support of Type-1 HARQ codebook enhancements when there are for feedback-disabled HARQ processes; FFS; Yes; No; Type-1 HARQ codebook enhancement is not supported for NR communication via satellite; per band; No; No; No; Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported]. 26. NR_NTN_solutions; 26-6b; [HARQ feedback disabling for] Type-3 HARQ codebook enhancement; - [Support of HARQ feedback disabling] - Support of Type-3 HARQ codebook enhancements when there are for feedback-disabled HARQ processes; FFS; Yes; No; Type-3 HARQ codebook enhancement is not supported for NR communication via satellite; per band; No; No; No; Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported].

At the previous meeting, the above agreement and working assumption were reached. Details of UE capabilities for Rel-17 NTN need further discussions.

  - General aspects

One important discussion is on notes with yellow-highlight on applicability of each FG for TN. In our view, at least report of NTN-specific features that is not used in TN is unnecessary. For example, FG 26-1 has components to connect to NTN cell and do communication appropriately. Meanwhile, such a mechanism is unnecessary for any TN. For those FGs, such a note should be captured. The FGs would be FGs 26-1/26-4/26-8/26-9.

Then, there were discussions on whether such a note is captured for FGs that can potentially be used in TN as well as NTN. Basically, we believe that whether a mechanism is supported or not should be discussed in WI agenda having motivation to introduce the mechanism. Under this view, also FGs 26-5/26-6/26-6a/26-6b should have the same note as in FGs 26-1/26-4/26-8/26-9. However, it was decided in Rel-16 NR-U UE feature session that (normal) Type-3 HARQ-ACK CB is applicable for any cell including cells in licensed band. If majority companies really want the FGs for cell other than NTN cell, we do not object the direction to follow the precedent.

Regarding wording of the note, we think that the note should be separate into two sentences in order to avoid misunderstanding. In addition, we are not sure ‘ATG cell’ needs to be mentioned. RAN4 decided that Rel-17 does not support ATG cell and it is discussed in Rel-18.

Based on the above, the following is proposed.

Proposal 1:

- The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell.

  - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.

  - FGs 26-6/26-6a/26-6b

At the last meeting, ‘UE supports HARQ disabling’ was removed from these FGs since RAN2 has the corresponding FG. Then pre-requisite of these FGs is FFS to discuss whether the RAN2 FG should be added in these FGs as pre-requisite. In our understanding, HARQ-ACK CB enhancements introduced in Rel-17 NTN WI are only related to feedback-disabling mechanism. To add the RAN2 FG as pre-requisite would be straightforward without any problem. The following is proposed based on this discussion.

Proposal 2:

- harq-FeedbackDisabled is added as pre-requisite of FGs 26-6/26-6a/26-6b.

- Conclusion

Proposal 1:

- The following note is added to at least FGs 26-1/26-4/26-8/26-9, and also to FGs 26-5/26-6/26-6a/26-6b unless majority companies prefer to use the FG also for TN cell.

  - Note: This UE feature group is applicable only for NR NTN cell. This UE feature group is not supported for terrestrial cell.

Proposal 2:

- harq-FeedbackDisabled is added as pre-requisite of FGs 26-6/26-6a/26-6b.

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues, Mar. 2022.