3GPP TSG RAN WG1#109-e	 	 	R1-2204360

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel-17 UE features for NR positioning enhancements

Agenda Item:	8.16.5

Document for: 	Discussion and Decision

- Introduction

Updated Rel-17 UE features list after RAN1#108-e including NR positioning enhancements was made [1]. In this contribution, we present our view on UE features for Rel-17 NR positioning enhancements.

- Discussion

- FG 27-1-1: UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning

  - A consistent wording should be used as follows:

    - “UE assisted DL TDOA” can be changed to “UE-assisted DL TDOA”

- FG 27-2-1: DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD

  - Support the current FG 27-2-1 (i.e. the brackets on the note can be removed)

- FG 27-3-1: M-sample measurements

  - “[FFS others]” can be removed since only the value 1 is enough and is aligned with the motivation of this feature.

  - A consistent wording should be used as follows:

    - “UE based positioning” can be changed to “UE-based positioning”

- FG 27-3-3: DL PRS Processing Capability outside MG - buffering capability

  - Support the current FG 27-3-3 (i.e. the brackets on the component 2 can be removed)

- FG 27-4-1: LOS/NLOS Indicator for UE-assisted positioning

  - Regarding component 1 candidate values, considering the common understanding of the AI 8.5.5 Note: “Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp.”, “both” can be reported.

  - Regarding component 2 candidate values, we prefer to keep “both”.

  - “[Note: a single value is reported when both multi-RTT and DL-TDOA are supported]” can be removed.

  - “FFS: signalling per method” part can be removed.

- FG 27-10a: Low latency MG activation request for PRS measurements

  - The prerequisite FG 27-10 can be removed.

- FG 27-12: LOS/NLOS indicator for UE-based positioning assistance data

  - Regarding component 1 candidate values, considering the common understanding of the AI 8.5.5 Note: “Both does not imply that the UE can report both hard and soft values for the same association (e.g., PRS resource) in the same time stamp.”, “both” can be reported.

  - Regarding component 2 candidate values, we prefer to keep “both”.

- FG 27-15a: Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS

  - “[Need for location server to know if the feature is supported]” can be removed.

  - We need a clarification on prerequisite feature groups of FG 27-15b. When it has been clarified, “FFS: outside initial BWP” can be removed.

- FG 27-15b: Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP

  - We prefer to keep component 5.

  - We prefer to keep component 6.

  - “FFS: center frequency” can be removed.

  - Regarding semi-persistent SRS transmission for outside initial UL BWP, we need to discuss whether FG 27-15a is necessary in prerequisite feature groups in addition to FG 27-15.

  - Regarding component 1 candidate values, we need to consider SupportedBandwidth information elements, and we think all values can be included as candidate values for component 1.

  - We think component 5 candidate value is not needed.

  - We think component 6 candidate value is not needed.

  - Regarding component 7, the following candidate values can be considered.

    - Candidate values: {1, 2, 4, 8, 16, 32, 64}

  - Regarding component 8, the following candidate values can be considered.

    - Candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14}

  - “[Need for location server to know if the feature is supported]” can be removed.

- FG 27-16: OLPC for positioning SRS in RRC_INACTIVE state – gNB

  - We need to discuss the prerequisite FG since RAN1 received an LS requiring a clarification on FG 27-16 and 27-19 from RAN2 (R2-2204360). Considering the prerequisite of OLPC in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG.

- FG 27-16a: OLPC for positioning SRS in RRC_INACTIVE state – location server

  - Considering the prerequisite of OLPC in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG.

- FG 27-17: PRS processing in RRC_INACTIVE

  - The prerequisite FG 13-2, 13-3 and 13-4 can be removed since these prerequisites can be covered by the note, while FG13-1 can be kept as the prerequisite since it is not covered by the note. This is similar handling as for FG27-18a/b/c, and the consistent handling would be preferable.

- FG 27-19: Spatial relation for positioning SRS in RRC_INACTIVE state - gNB

  - We need to discuss the prerequisite FG since RAN1 received an LS requiring a clarification on FG 27-16 and 27-19 from RAN2 (R2-2204360). Considering the prerequisite of spatial relation in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG.

- FG 27-19a: Spatial relation for positioning SRS in RRC_INACTIVE state – location server

  - Considering the prerequisite of spatial relation in RRC_CONNECTED, we think that FG 27-15 (i.e. Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP) should be added to the prerequisite FG.

- FG 27-20: PRS subset association for UE assisted DL-AoD

  - Support the current FG 27-20 (i.e. the brackets on the component 3 can be removed)

  - A consistent wording should be used as follows:

    - “UE assisted DL-AoD” can be changed to “UE-assisted DL-AoD”

References

- R1-2202928, 3GPP RAN1#108-e, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2202853, 3GPP RAN1#108-e, “Summary of UE features for NR positioning enhancements”, Moderator (AT&T)