3GPP TSG RAN WG1 #109-e			R1-2204363

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for NR coverage enhancement

Agenda Item:	8.16.8

Document for: 	Discussion and Decision

- Introduction

Updated Rel.17 UE feature list after RAN1#108-e including NR coverage enhancements has been agreed [1]. In this contribution, we present our views and updates regarding the UE features for NR coverage enhancement based on the list.

- Discussion

  - FGs 30-1 to 30-2: Enhancements for PUSCH Type A repetitions and

The relationship between FG 30-1 and 30-2 is the remaining discussion point, and following points are FFS points.

1. Number of repetitions (K) for FG 30-2 : 8 or 16 or 32

2. Prerequisite for FG 30-2 : including 30-1 or not

Number of repetitions is independent from the FG 30-2, since it depends on whether UE supports repetitions of 16/32 or not, and FG 30-2 is for available slot counting. Therefore component of FG 30-2 does not need to include the number of repetition (K). And both available slot counting and increase number of repetitions can be configured simultaneously, FG 30-1 can be included in one of the prerequisite for FG 30-2.

Proposal 1 : Remove number of repetition (K) from component of FG 30-2.

Proposal 2 : FG 30-1 can be included in one of the prerequisite for FG 30-2.

  - FGs 30-3 to 30-3a: TB processing over multi-slot PUSCH

The prerequisite FGs of FG30-3 and 30-3a have not been decided yet [2]. As the repetition and TBoMS can be viewed as independent features, FG 11-6 does not need to be prerequisite FGs in our view. On the other hand, the repetition of TBoMS is the combination of TBoMS and repetition type A with available slot determination. Hence, the prerequisite FGs of FG 30-3a should be FG 11-6, FG 30-2, and FG 30-3.

Proposal 3: There is no prerequisite FG of FG 30-3.

Proposal 4: The prerequisite FGs of FG 30-3a should be FG 11-6, FG 30-2, and FG 30-3.

  - FGs 30-4 to 30-4h: DM-RS bundling

In LS from RAN4, RAN4 asks RAN1 to decide whether to add 16 slots in the set of values for TDD in Rel-17 in FG 30-4 [3]. For certain TDD patterns such as DDDSUDDSUU, the periodicity of TDD pattern is 10 slots. In those scenarios, it is beneficial to set 10 slots as the length of nominal time domain window (TDW) to avoid the boundary of nominal TDWs on consecutive UL slots. For example, one TDW could not be placed over UL consecutive slots, when the nominal TDW whose length is equal to 5 slots starts on UL slot as shown in Fig.1.  To enhance the coverage performance even in these TDD patterns, UE should be able to be configured with larger nominal TDW lengths if UE is capable of it.

Figure 1.  Nominal TDW placement when the length of nominal TDW is configured by 5 slots in TDD pattern DDDSUDDSUU.

In Rel-17 joint channel estimation, the length of nominal TDW that UE is configured with is up to the maximum duration. For those reasons, 16 slots should be included in the set of values for TDD in UE capability, to achieve the coverage enhancements in a variety of TDD patterns.

Proposal 5: 16 slots should be included in the set of maximum duration values for TDD.

Types of FGs 30-4a to 30-4h are still controversial [2]. As LS from RAN4 suggests the final decision on the granularity of UE capability to be made in RAN1, RAN1 should make the decision on it [3]. At the RAN1#108-e meeting, companies argued either per band or {per band and per BC}. In our view, the difference between per FS and {per band and per BC} is unclear. Also, we think per band is sufficient for FGs 30-4a to 30-4h, unless the explicit technical reasons why {per band and per BC} is necessary are clear.

Proposal 6: FGs 30-4a to 30-4h can be supported per band.

The prerequisite FGs for 30-4a, 30-4b, 30-4c, 30-4d, 30-4e, 30-4f, 30-4g, and 30-4h can be {FG 30-4 and FGs 5-14 or 5-16 or 5-17}, {FGs 30-4 and 11-5}, {FGs 30-4 and 30-3}, {FGs 30-4 and 4-23}, {FGs 30-4a or 30-4b or 30-4c}, {FG 30-4d}, {FG 30-4}, and {FGs 30-4a or 30-4b or 30-4c or 30-4d}, respectively.

Proposal 7: The prerequisite FGs for 30-4a, 30-4b, 30-4c, 30-4d, 30-4e, 30-4f, 30-4g, and 30-4h can be {FG 30-4 and FGs 5-14 or 5-16 or 5-17}, {FGs 30-4 and 11-5}, {FGs 30-4 and 30-3}, {FGs 30-4 and 4-23}, {FGs 30-4a or 30-4b or 30-4c}, {FG 30-4d}, {FG 30-4}, and {FGs 30-4a or 30-4b or 30-4c or 30-4d}, respectively.

  - FG 30-6: Msg3 repetition

It was discussed that the FG 30-6 is supported per UE or per band. Since Msg3 repetition is not the band specific feature, the FG can be per UE. Also, the differentiation between TDD and FDD is not necessary, following FGs of repetition type A.

Proposal 8: FG 30-6 can be supported per UE with no differentiation between TDD and FDD.

- Conclusion

In this contribution, we discussed on the UE features for NR coverage enhancement. Based on the discussion we made following proposal.

Proposal 1 : Remove number of repetition (K) from component of FG 30-2.

Proposal 2 : FG 30-1 can be included in one of the prerequisite for FG 30-2.

Proposal 3: There is no prerequisite FG of FG 30-3.

Proposal 4: The prerequisite FGs of FG 30-3a should be FG 11-6, FG 30-2, and FG 30-3.

Proposal 5: 16 slots should be included in the set of maximum duration values for TDD.

Proposal 6: FGs 30-4a to 30-4h can be supported per band.

Proposal 7: The prerequisite FGs for 30-4a, 30-4b, 30-4c, 30-4d, 30-4e, 30-4f, 30-4g, and 30-4h can be {FG 30-4 and FGs 5-14 or 5-16 or 5-17}, {FGs 30-4 and 11-5}, {FGs 30-4 and 30-3}, {FGs 30-4 and 4-23}, {FGs 30-4a or 30-4b or 30-4c}, {FG 30-4d}, {FG 30-4}, and {FGs 30-4a or 30-4b or 30-4c or 30-4d}, respectively.

Proposal 8: FG 30-6 can be supported per UE with no differentiation between TDD and FDD.

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Mar. 2022.

- Moderator (NTT DOCOMO, INC.), R1-2201517, Summary on UE features for NR coverage enhancement, Mar. 2022.

- Moderator(China Telecom), R1-2203029, Reply LS on Length of Maximum duration, Mar. 2022