3GPP TSG RAN WG1 #109-e			     R1-2204365

e-Meeting, May 9 – 20, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on Rel.17 UE features for NR MBS

Agenda Item:	8.16.12

Document for: 	Discussion and Decision

- Introduction

Updated Rel.17 UE feature list after RAN1#108-e including NR MBS has been agreed [1]. In this contribution, we present our views and updates regarding the UE features for NR MBS based on the list.

- Discussion

- FG 33-1: Broadcast

The only difference between dynamic slot-level repetition for multicast and dynamic slot-level repetition for broadcast is the difference in the TDRA list including the parameter repetitionNumber. There would be no need for separate FGs.

Proposal 1: Support of dynamic slot-level repetition for broadcast is merged with FG 33-3-1.

- FG 33-2-x

  - FG 33-2: Dynamic scheduling for multicast for PCell

It was agreed at the last meeting that only a single CC can be configured for multicast reception. There would be no need to make the type of FG 33-2 more granular than per band. The type should be per band.

Proposal 2: The type of FG 33-2 is per band.

  - FG 33-2a: Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

There would be little increase in UE complexity to support separate PUCCH resource configurations. We don’t see the need to separate FGs.

Proposal 3: Include support of separate PUCCH resource configurations from unicast in FG 33-2a.

  - FG 33-2b: DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

  - FG 33-2f: Dynamic multicast with DCI format 4_2

UE processing of these functions would not be band dependent. The type of these FGs would be sufficient for per UE.

Proposal 4: The type of FG 33-2b and 33-2f is per UE.

  - FG 33-2d: PTP retransmission for multicast

A UE supports PTP retransmission of unicast PDSCH. There would be no need to subdivide the type of FG 33-2d to support PTP retransmission of multicast PDSCH. The type is sufficient for per UE.

Proposal 5: The type of FG 33-2d is per UE.

  - FG 33-2e: Multiple G-RNTIs for group-common PDSCHs

Since processing time constraints depend on SCS, the type of FG 33-2e would be better to be per band.

Proposal 6: The type of FG 33-2e is per band.

- FG 33-3-1: Dynamic Slot-level repetition for group-common PDSCH

Since the type of FG for dynamic slot-level repetition for unicast is per band, the type of dynamic slot-level repetition for multicast should also be per band.

Proposal 7: The type of FG 33-3-1 is per band.

- FG 33-3-2: FDM-ed unicast PDSCH and group-common PDSCH

- FG 33-3-3: Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

Since the type of FG for the number of TDM-ed unicast PDSCHs in a slot is per FS, the type of FG 33-3-2 and 33-3-3 should also be per FS.

Proposal 8: The type of FG 33-3-2 and 33-3-3 is per FS.

- FG 33-3-3a: FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

- FG 33-3-3b: Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

- FG 33-3-4: Mode 1 for type1 codebook generation

FG 33-3-2 should be a prerequisite FG of FG 33-3-3a. FG 33-3-3 should be a prerequisite FG of FG 33-3-3b. Since the type of FG for Type-1 and Type-2 HARQ-ACK codebook for unicast is per UE, the type of FG 33-3-3a, 33-3-3b and 33-3-4 should also be per UE.

Proposal 9: Add FG 33-3-2 as a prerequisite FG of FG 33-3-3a.

Proposal 10: Add FG 33-3-3 as a prerequisite FG of FG 33-3-3b.

Proposal 11: The type of FG 33-3-3a, 33-3-3b and 33-3-4 is per UE.

- FG 33-3-5: feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type

Since the type of FG for support of simultaneous construction of two HARQ-ACK codebooks for unicast is per FS, the type of FG 33-3-5 should also be per FS.

Proposal 12: The type of FG 33-3-5 is per FS.

- FG 33-4-x

  - FG 33-4: NACK-only based HARQ-ACK feedback for multicast

PUCCH resources for unicast HARQ-ACK feedback are orthogonal among UEs. On the other hand, NACK-only based feedback allows PUCCH resources to be shared among UEs. It is not appropriate to use unicast PUCCH resources for NACK-only based feedback because the allocation policies are opposite. Support for separate PUCCH resource configurations should be a basic feature of NACK-only based HARQ-ACK feedback.

Proposal 13: Support of separate PUCCH configurations from unicast is included in FG 33-4.

  - FG 33-4-1: DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

The type of FG 33-4-1 should be per UE as well as DCI-based enabling/enabling ACK/NACK-based feedback.

Proposal 14: The type of FG 33-4-1 is per UE.

- FG 33-5

  - FG 33-5-1: SPS group-common PDSCH for multicast

Since the support of DCI format 4_2 for dynamic scheduling is in a separate FG, it would be natural to make the support of DCI format 4_2 for SPS in a separate FG. Since the type of FG for support of unicast SPS is per UE, the type of FG 33-5-1 should also be per UE.

Proposal 15: Separate support of DCI format 4_2 with CRC scrambled with G-CS-RNTI from FG 33-5-1.

Proposal 16: The type of FG 33-5-1 is per UE.

  - FG 33-5-1b: DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast

  - FG 33-5-1g: DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

The type of FG 33-5-1b and 33-5-1g should be per UE as well as DCI-based enabling/enabling ACK/NACK-based feedback for dynamic scheduling.

Proposal 17: The type of FG 33-5-1b and 33-5-1g are per UE.

  - FG 33-5-1d: PTP retransmission for SPS group-common PDSCH for multicast

Since the FG for PTP retransmission for dynamic scheduling is separate from the FG for ACK/NACK based feedback, it would be better to have a separate FG for SPS as well.

Proposal 18: Not merge FG 33-5-1d with FG 33-5-1a.

  - FG 33-5-1e: Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

Since the type of FG for dynamic slot-level repetition for unicast is per band, the type of dynamic slot-level repetition for multicast should also be per band.

Proposal 19: The type of FG 33-5-1 is per band.

  - FG 33-5-1h: Multiple G-CS-RNTIs for SPS group-common PDSCHs

Since processing time constraints vary by SCS, the type of FG 33-5-1h would be better to be per band.

Proposal 20: The type of FG 33-5-1h is per band.

  - FG 33-5-2: Multiple SPS group-common PDSCH configuration

Since the type of the FG for multiple SPS configuration for unicast is per band, the type of FG 33-5-2 should also be per band.

Proposal 21: The type of FG 33-5-2 is per band.

- FG 33-6

  - FG 33-6-1: DL priority indication for multicast in DCI

  - FG 33-6-1a: DL priority configuration for SPS multicast

Since the type of FG for support of the priority indicator field for unicast is per UE, the type of FG 33-6-1 should also be per UE. Component 2 os FG 33-6-1 can be removed since support of construction of two HARQ-ACK codebooks is included in FG 33-6-2.

Proposal 22: The type of FG 33-6-1 and 33-6-1a is per UE.

Proposal 23: Update FG 33-6-1 as follows

| 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 |
| --- | --- | --- | --- |

  - FG 33-6-2: Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE

Since the type of FG for support of the construction of two HARQ-ACK codebooks for unicast is per FS, the type of FG 33-6-2 should also be per FS.

Proposal 24: The type of FG 33-6-2 is per FS.

  - FG 33-6-3: More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

Since the type of FG for support of transmission of two PUCCHs in the same slot is per UE, the type of FG 33-6-3 would also be sufficient per UE.

Proposal 25: The type of FG 33-6-3 is per UE.

- FG 33-9: Supporting unicast PDCCH to release SPS group-common PDSCH

UE processing of this feature would not be band dependent. The type is sufficient for per UE.

Proposal 26: The type of FG 33-9 is per UE.

- Conclusion

In this contribution, we have the following proposals:

Proposal 1: Support of dynamic slot-level repetition for broadcast is merged with FG 33-3-1.

Proposal 2: The type of FG 33-2 is per band.

Proposal 3: Include support of separate PUCCH resource configurations from unicast in FG 33-2a.

Proposal 4: The type of FG 33-2b and 33-2f is per UE.

Proposal 5: The type of FG 33-2d is per UE.

Proposal 6: The type of FG 33-2e is per band.

Proposal 7: The type of FG 33-3-1 is per band.

Proposal 8: The type of FG 33-3-2 and 33-3-3 is per FS.

Proposal 9: Add FG 33-3-2 as a prerequisite FG of FG 33-3-3a.

Proposal 10: Add FG 33-3-3 as a prerequisite FG of FG 33-3-3b.

Proposal 11: The type of FG 33-3-3a, 33-3-3b and 33-3-4 is per UE.

Proposal 12: The type of FG 33-3-5 is per FS.

Proposal 13: Support of separate PUCCH configurations from unicast is included in FG 33-4.

Proposal 14: The type of FG 33-4-1 is per UE.

Proposal 15: Separate support of DCI format 4_2 with CRC scrambled with G-CS-RNTI from FG 33-5-1.

Proposal 16: The type of FG 33-5-1 is per UE.

Proposal 17: The type of FG 33-5-1b and 33-5-1g are per UE.

Proposal 18: Not merge FG 33-5-1d with FG 33-5-1a.

Proposal 19: The type of FG 33-5-1 is per band.

Proposal 20: The type of FG 33-5-1h is per band.

Proposal 21: The type of FG 33-5-2 is per band.

Proposal 22: The type of FG 33-6-1 and 33-6-1a is per UE.

Proposal 23: Update FG 33-6-1 as follows

| 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 |
| --- | --- | --- | --- |

Proposal 24: The type of FG 33-6-2 is per FS.

Proposal 25: The type of FG 33-6-3 is per UE.

Proposal 26: The type of FG 33-9 is per UE.

References

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e

- Draft Report of 3GPP TSG RAN WG1 #108-e v0.1.0