3GPP TSG RAN WG1 #109-e			R1-2204366

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on other UE feature related discussions

Agenda Item:	8.16.17

Document for: 	Discussion and Decision

- Introduction

The updated RAN1 UE features list for Rel-17 NR after RAN1#108-e has been agreed in [1]. In this contribution, we present our views regarding other UE feature related discussions.

- Discussion

As summarized in [2], there has been extensive discussions on whether to introduce FG 37-1 as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”.

Based on the discussion, following compromised proposal was provided by the FL.

Final proposal

·         Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

Ø  UE supports this new FG needs to support Rel-16 CA Option 2

·         UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

·         UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

Ø  Indicating “option 2” via the new per-BC FG

Ø  Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

However, there was only one company who could not agree on the above proposal, and the company provided the following revised proposal.

Proposal-rev

- Specify a new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching UL CA when “both option 1 and option 2” is indicated via FG 22-1)

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 1 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]  and indicating “option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]  and indicating “both option 1 and option 2” via FG 22-1 and indicating the new per-BC FG supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well.

Due to the lack of the meeting time, at the end of the last RAN1 meeting, there was no consensus on whether to introduce FG 37-1, and it has not been captured in the UE features list yet.

As in the above proposals, it seems all companies are fine to specify a new per-BC FG to indicate the support of “option 2” for Rel-17 2Tx-2Tx UL Tx switching, and all companies are also fine to support following four types of UEs supporting Rel-17 2Tx-2Tx UL Tx switching.

- Type 1: UE supporting option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching

  - For this type 1, companies are fine that FG37-1 is not necessary.

- Type 2: UE supporting option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching

  - For this type 2, companies are fine that FG37-1 is not necessary.

- Type 3: UE supporting option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching

  - For this type 3, companies are fine that FG37-1 is necessary.

- Type 4: UE supporting option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching

  - For this type 4, majority of companies are fine that FG37-1 is necessary, while only one company argued that FG37-1 is not necessary as this type of UE indicates the support of option 2 only via FG22-1 and the support of 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

So, only one remaining discussion point is whether reporting FG37-1 in case of above Type 4 UE (supporting only option 2 for both Rel-16/17 UL Tx switching) is necessary or not. In our view, although the support of option 2 only for both Rel-16/17 UL Tx switching can be interpreted based on the support of option 2 only via FG22-1 and the support of 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx], using FG37-1 in addition to them is also fine and it may be better to indicate FG37-1 for Type 4 in terms of the consistent meaning of FG37-1, i.e., this FG is to indicate the support of option 2 for Rel-17 2Tx-2Tx UL Tx switching. If the majority is fine to not indicate the FG37-1 for Type 4, we are also fine. The most important point is to define the FG37-1 at least for Type 3 as it is the compromised outcome of extensive discussion so far and this meeting is the final opportunity to do it for Rel-17.

Proposal 1:

The new FG 37-1 is defined for “Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA”.

- If UE supports option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “switchedUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “dualUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

| 37. NR_RF_FR1_enh | 37-1 | Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA | - Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA | 22-1, the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or sup-portedBandPairListNR-v17xx] | Yes | N/A |  | Per BC | N/A | N/A (FR1 only) | N/A | -If UE supports option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “switchedUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “dualUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Conclusion

In this contribution, we presented our views regarding the UE features for Rel-17 UL Tx switching, and following proposal was made.

Proposal 1:

The new FG 37-1 is defined for “Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA”.

- If UE supports option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “switchedUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

- If UE supports option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “dualUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].

References

- Moderators (AT&T, NTT DOCOMO, INC.), R1-2202929, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues, Feb. 2022.

- Moderator (NTT DOCOMO, INC.), R1-2201522, Summary on other UE feature related discussions, Feb. 2022.