3GPP TSG RAN WG1 #109-e	R1-2204367

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC

Title:	Discussion on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for: 	Discussion and Decision

Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are three objectives for extension of unified TCI framework, as shown below.

Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. […] - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss unified TCI framework related enhancements.

Unified TCI framework extension for multi-TRP

  - Observation of Rel.17 TCI framework

Beam management is one of the key features to support high frequency in 5G NR. Beam management consists of beam measurement, beam reporting, and beam indication. For the beam indication, Rel.15 specifies TCI state and spatial relation framework, and the TCI-state and spatial relation are signaled per each channel/RS. However, in practical gNB operation, since UE reports the largest L1-RSRP beam, gNB configures the best L1-RSRP beam for all UL/DL channels/RSs. In such operation, the beam indication mechanism in Rel.15 is too flexible and not efficient from signaling overhead perspective. Rel.16 beam indication mechanism reduces signaling overhead and latency. We believe MAC CE based beam indication works well in Rel.16, one MAC CE can indicate “common beam (i.e. TCI state of PDCCH)”, which is also applied to PDSCH/PUCCH/SRS/PUSCH by using default TCI state and default spatial relation. In Rel.17, unified TCI framework is specified, and it enables one MAC CE/DCI can indicate “common beam (i.e. indicated TCI state)”, which can be applied to PDCCH/PDSCH/A-CSI-RS/PUSCH/PUCCH/SRS. However, due to the time limitation, Rel.17 unified TCI framework supports single-TRP operation only. To support multi-TRP operation, enhancement of unified TCI framework is necessary.

In Rel.17 unified TCI framework, it is not allowed to configure both Rel.15/16 TCI-state/spatial-relation (except for positioning) and Rel.17 TCI state in the same band. This means once Rel.17 TCI state is configured, other features using Rel.15/16 TCI-state/spatial-relation (e.g. Rel.16/17 Multi-TRP) cannot be configured in the same band. Hence, it is necessary to extend Rel.17 unified TCI framework to support all Rel.16/17 Multi-TRP schemes.

Proposal 2-1:

  - Specify joint TCI state and separate UL/DL TCI state to support all of following Rel.16/17 M-TRP schemes:

    - Single/multi-DCI based M-TRP PDSCH NCJT in Rel.16

    - Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16

    - M-TRP PDCCH/PUSCH/PUCCH repetition in Rel.17

    - M-TRP inter cell in Rel.17

    - SFN-PDCCH/PDSCH in Rel.17

    - M-TRP BFR in Rel.17

  - Extension of Rel.17 TCI framework

In Rel.16/17, single-DCI based M-TRP and multi-DCI based M-TRP are specified to support two different scenarios. One scenario is ideal backhaul between TRPs, and one DCI from one TRP can schedule two PDSCHs from two TRPs. The other scenario is ideal/non-ideal backhaul between TRPs, and one DCI from one TRP can schedule one PDSCH from one TRP.

For unified TCI framework in Rel.17, RRC/MAC CE/DCI indicates one “indicated TCI state” (i.e. up to 1 joint TCI states or up to 1 DL TCI states + 1 UL TCI states), which is applied to multiple UL/DL channels/RSs. To support M-TRP operation, unified TCI framework should be extended to indicate two “indicated TCI states” (i.e. up to 2 joint TCI states or up to 2 DL TCI states + 2 UL TCI states). Rel.18 extension of unified TCI framework should take into account the differences of the two scenarios of single-DCI based M-TRP and multi-DCI based M-TRP. For example, for single-DCI based M-TRP, since the target scenario is for ideal backhaul, one beam indication DCI can indicate one or two “indicated TCI state(s)”. Meanwhile, for multi-DCI based M-TRP, since the target scenario is for ideal/non-ideal backhaul, one beam indication DCI can indicate one “indicated TCI state”.

For the number of indicated TCI states (i.e. the number of TRPs), we believe up to two TCI states are enough, because Rel.16/17 M-TRP schemes supports up to 2 TCI states. If Rel.18 feature (e.g. coherent-JT) will require more than two TCI states, Rel.18 unified TCI framework should be also extended to support larger number of TCI states.

Proposal 2-2:

  - The number of “indicated TCI states” can be up to 2 to support Rel.16/17 M-TRP features.

    - If more than 2 TCI states are required in Rel.18, the number of “indicated TCI states” is also increased to support Rel.18 schemes.

  - Specify different signaling mechanism to support unified TCI framework for M-TRP for the following schemes:

    - Single-DCI based M-TRP (for ideal backhaul scenario):

      - One beam indication DCI can indicate up to two “indicated TCI states”.

    - Multi-DCI based M-TRP (for ideal/non-ideal backhaul scenario):

      - One beam indication DCI can indicate one “indicated TCI states” for a TRP.

a) Single DCI based M-TRP                                   b) Multi DCI based M-TRP

Figure 2-1. Indication of two “indicated TCI states”.

In Rel.17 TCI framework, TCI state field of DCI format 1_1/1_2 (with/without DL assignment) can indicate “indicated TCI state”. In Rel.18 TCI state, similar mechanism can be considered. One discussion point is whether to add second TCI state field in DCI format 1_1/1_2. From our perspective, we don’t see the necessity to add new TCI state field. Similar to Rel.16 single DCI based M-TRP, one TCI codepoint can indicate one or two TCI states.

In Rel.15/16, the size of TCI state field is fixed 3bit in DCI format 1_1 and configurable 0~3bit in DCI format 1_2. To support multiple “indicated TCI state”, 3bit may not be enough for some cases. Hence, whether to increase the size of TCI state field can be discussed. Also, fixed 3bit TCI state field in DCI format 1_1 is not flexible, it would be more preferred to consider RRC configurable size of TCI state field in DCI format 1_1.

Proposal 2-3:

  - Strive to use one TCI state field (i.e. not adding second TCI state field).

  - Discuss whether to increase the max. number of bits for TCI state field in DCI format 1_1/1_2.

  - Consider RRC configurable size of TCI state field in DCI format 1_1.

Alt.1) Reuse existing TCI state field                    Alt.2) Add new TCI state field

Figure 2-2. TCI state field in DCI format 1_1/1_2.

  - Unified TCI extension for single DCI based multi-TRP

For single DCI based multi-TRP in Rel.17, one TCI codepoint can indicate up to two TCI states. There is no explicit higher layer index corresponding to TRP index, and 1st TCI state implies TCI state of PDSCH from 1st TRP and 2nd TCI state implies TCI state of PDSCH from 2nd TRP. We can reuse this mechanism to unified TCI extension for single DCI based multi-TRP.

For joint TCI state, figure 2-3 a) illustrates concept of unified TCI extension for single DCI based multi-TRP. For joint TCI state, after RRC configures TCI state pool/list, MAC CE activates multiple active joint TCI states. One DCI can indicate one or two indicated joint TCI states. As shown in figure 2-3 b), RRC/MAC CE can configure/activate one or two TCI states per each TCI codepoint. TCI state field of DCI format 1_1/1_2 (with/without DL assignment) can indicate one TCI codepoint, which corresponds to 1st joint TCI state and/or 2nd joint TCI state.

a) Concept for joint TCI state

b) TCI codepoint

Figure 2-3. Unified TCI extension for joint TCI state for single DCI based multi-TRP.

For separate DL/UL TCI state, figure 2-4 a) illustrates concept of unified TCI extension for single DCI based multi-TRP. For separate DL/UL TCI state, after RRC configures TCI state pool/list, MAC CE activates multiple active DL TCI states and UL TCI states. One DCI can indicate up to 2 DL TCI states and up to 2 UL TCI states simultaneously. As shown in figure 2-4 b), RRC/MAC CE can configure/activate up to 2 DL TCI states and up to 2 UL TCI states per each TCI codepoint. TCI state field of DCI format 1_1/1_2 (with/without DL assignment) can indicate one TCI codepoint, which can correspond to 1st DL TCI state, 1st UL TCI state, 2nd DL TCI state, and/or 2nd UL TCI state.

a) Concept for separate DL/UL TCI state

b) TCI codepoint

Figure 2-4. Unified TCI extension for separate DL/UL TCI state for single DCI based multi-TRP.

Proposal 2-4:

  - For unified TCI extension for single-DCI based multi-TRP,

    - For joint TCI state, one TCI codepoint corresponds to up to two joint TCI states.

    - For separate DL/UL TCI state, one TCI codepoint corresponds to up to two DL TCI states and/or up to two UL TCI states.

  - Unified TCI extension for multi-DCI based multi-TRP

For multi-DCI based multi-TRP in Rel.17, higher layer parameter CORESETPoolIndex = {0, 1}, which corresponds to TRP index, can be configured per each CORESET. The current specification of multi-DCI based multi-TRP is specified using CORESETPoolIndex. One DCI associated with CORESETPoolIndex = 0 indicates TCI state of 1st PDSCH from 1st TRP. The other DCI associated with CORESETPoolIndex = 1 indicates TCI state of 2nd PDSCH from 2nd TRP. We can reuse this mechanism to unified TCI extension for multi-DCI based multi-TRP. One potential discussion point is whether to reuse CORESETPoolIndex for unified TCI extension for multi-DCI based multi-TRP. In our view, CORESETPoolIndex should be reused, because we can reuse the existing specification for multi-DCI based multi-TRP in Rel.16/17.

Proposal 2-5:

  - For unified TCI extension for multi-DCI based multi-TRP, definition of CORESETPoolIndex = {0, 1} is reused.

For joint TCI state, figure 2-5 illustrates concept of unified TCI extension for multi-DCI based multi-TRP. After RRC configures TCI state pool/list, MAC CE activates multiple active joint TCI states for CORESETPoolIndex = 0 and multiple active joint TCI states for CORESETPoolIndex = 1, respectively. As shown in figure 2-5, RRC/MAC CE can configure/activate up to one TCI states per each TCI codepoint for each of CORESETPoolIndex. When DCI associated with CORESETPoolIndex = 0 indicates one TCI codepoint, one indicated joint TCI state associated with CORESETPoolIndex = 0 is determined. When DCI associated with CORESETPoolIndex = 1 indicates one TCI codepoint, one indicated joint TCI state associated with CORESETPoolIndex = 1 is determined.

For separate DL/UL TCI state, figure 2-6 illustrates concept of unified TCI extension for multi-DCI based multi-TRP. After RRC configures TCI state pool/list, MAC CE activates multiple active DL/UL TCI states for CORESETPoolIndex = 0 and for CORESETPoolIndex = 1, respectively. One DCI can indicate one DL TCI state and/or one UL TCI state. As shown in figure 2-6, RRC/MAC CE can configure/activate one DL TCI state and/or one UL TCI state per TCI codepoint. When DCI associated with CORESETPoolIndex = 0 indicates one TCI codepoint, one DL TCI state and/or one UL TCI state associated with CORESETPoolIndex = 0 can be indicated. When DCI associated with CORESETPoolIndex = 1 indicates one TCI codepoint, one DL TCI state and/or one UL TCI state associated with CORESETPoolIndex = 1 can be indicated.

Proposal 2-6:

  - For unified TCI extension for multi-DCI based multi-TRP,

    - For joint TCI state, one DCI associated with CORESETPoolIndex = 0 indicates one joint TCI state (1st indicated TCI state), and the other DCI associated with CORESETPoolIndex = 1 indicates one joint TCI state (2nd indicated TCI state).

    - For separate TCI state, one DCI associated with CORESETPoolIndex = 0 indicates one DL TCI state and/or one UL TCI state (1st indicated TCI state), and the other DCI associated with CORESETPoolIndex = 1 indicates one DL TCI state and/or one UL TCI state (2nd indicated TCI state).

Figure 2-5. Unified TCI extension for joint TCI state for multi-DCI based multi-TRP.

Figure 2-6. Unified TCI extension for separate DL/UL TCI state for multi-DCI based multi-TRP.

In the figure 2-5 and figure 2-6, RRC configures two separate TCI state pool/list. Another option is that RRC configures one TCI state pool/list for both CORESETPoolIndexes. We believe whether the separate TCI state pool/list is necessary depends on whether we will increase the max. number of RRC configured TCI states in Rel.18. If the max. number of RRC configured TCI states is the same as Rel.15/16 (i.e. 128), current 7-bit is enough in MAC CE to indicate TCI state ID. Hence, there is no need to configure different RRC configured TCI state pool/list. On the other hand, if we increase the max. number of TCI states to 256 as an example, 8-bit is needed to indicate TCI state ID for the common TCI state pool/list, which will increase MAC CE overhead. Even in this case, if we have different TCI state pool/list for each TRP of 128 TCI states for CORESETPoolIndex = 0 and another 128 TCI states for CORESETPoolIndex = 1, 7-bit is enough to indicate TCI state ID for CORESETPoolIndex = 0 and CORESETPoolIndex = 1, respectively.

Proposal 2-7:

  - For unified TCI extension for multi-DCI based multi-TRP, discuss whether to increase the max. number of RRC configured TCI states from Rel.17.

- 3. Simultaneous multi-panel UL transmission with unified TCI framework

In our companion paper [2], for simultaneous multi-panel PUSCH/PUCCH transmission, we discuss following transmission schemes. Details can be found in [2].

- S-DCI Multi-panel/M-TRP FDM PUSCH repetition: two PUSCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively.

- S-DCI Multi-panel/M-TRP SFN PUSCH repetition: all REs of a PUSCH are associated with two beams/panels.

- S-DCI Multi-panel/M-TRP SDM PUSCH scheme: different layer/CW of a PUSCH is associated with different beam/panel.

- M-DCI M-TRP PUSCH: two PDCCHs with different CORESETPoolID scheduling fully/partially overlapped PUSCHs.

- Multi-panel/M-TRP FDM PUCCH repetition: two PUCCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively.

- Multi-panel/M-TRP SFN PUCCH repetition: all REs of a PUCCH are associated with two beams/panels.

- Transmission of two overlapped PUCCHs associated with different panel/TRP.

As discussed in section 2, the extension of unified TCI framework should support Rel.16/17 M-TRP schemes. In addition to Rel.16/17 M-TRP schemes, the extension of unified TCI framework can also be applied to the above simultaneous multi-panel PUSCH/PUCCH Tx schemes (if supported). The two “indicated TCI states” can be used for two panels/TRPs, respectively. Details can be discussed after the support of simultaneous multi-panel PUSCH/PUCCH transmission schemes is discussed and decided in agenda item 9.1.4.1.

Proposal 3-1:

  - The extension of unified TCI framework can be applied to simultaneous multi-panel PUSCH/PUCCH Tx schemes (if supported). Details can be discussed after the support of simultaneous multi-panel PUSCH/PUCCH transmission schemes is discussed and decided in agenda item 9.1.4.1.

- Power control for M-TRP operation with unified TCI framework

For M-TRP operation, considering different channel condition between UE and different TRPs, per TRP power control should be supported. In Rel-17, PUSCH transmission power is determined as follows.

[dBm]

PUCCH transmission power is determined as follows.

[dBm]

For power control parameters (P0, alpha, PL-RS, closed-loop index), in Rel.17, these power control parameters are associated with each TCI state. With extension of unified TCI framework, with two “indicated TCI states” for two TRPs, per TRP power control parameters can be naturally derived.

Proposal 4-1:

  - For M-TRP operation, with extension of unified TCI framework, per TRP power control can be supported by Rel-17 unified TCI power control framework, i.e., power control parameters are associated with each TCI state.

On the other hand, for simultaneous multi-panel PUCCH/PUSCH transmission (if supported), PCMAX can be further studied. In Rel-17, PCMAX is determined per serving cell. For simultaneous multi-panel PUCCH/PUSCH transmission, how to determine PCMAX for transmission from each panel needs to be studied.

Proposal 4-2:

  - For simultaneous multi-panel PUSCH/PUCCH Tx scheme (if supported), how to determine PCMAX for transmission from each panel can be further studied.

- Conclusion

In this contribution, we discussed unified TCI framework related enhancements. Based on the discussion, we made the following proposals.

Proposal 2-1:

  - Specify joint TCI state and separate UL/DL TCI state to support all of following Rel.16/17 M-TRP schemes:

    - Single/multi-DCI based M-TRP PDSCH NCJT in Rel.16

    - Single-DCI based M-TRP PDSCH repetition with SDM/FDM/TDM in Rel.16

    - M-TRP PDCCH/PUSCH/PUCCH repetition in Rel.17

    - M-TRP inter cell in Rel.17

    - SFN-PDCCH/PDSCH in Rel.17

    - M-TRP BFR in Rel.17

Proposal 2-2:

  - The number of “indicated TCI states” can be up to 2 to support Rel.16/17 M-TRP features.

    - If more than 2 TCI states are required in Rel.18, the number of “indicated TCI states” is also increased to support Rel.18 schemes.

  - Specify different signaling mechanism to support unified TCI framework for M-TRP for the following schemes:

    - Single-DCI based M-TRP (for ideal backhaul scenario):

      - One beam indication DCI can indicate up to two “indicated TCI states”.

    - Multi-DCI based M-TRP (for ideal/non-ideal backhaul scenario):

      - One beam indication DCI can indicate one “indicated TCI states” for a TRP.

Proposal 2-3:

  - Strive to use one TCI state field (i.e. not adding second TCI state field).

  - Discuss whether to increase the max. number of bits for TCI state field in DCI format 1_1/1_2.

  - Consider RRC configurable size of TCI state field in DCI format 1_1.

Proposal 2-4:

  - For unified TCI extension for single-DCI based multi-TRP,

    - For joint TCI state, one TCI codepoint corresponds to up to two joint TCI states.

    - For separate DL/UL TCI state, one TCI codepoint corresponds to up to two DL TCI states and/or up to two UL TCI states.

Proposal 2-5:

  - For unified TCI extension for multi-DCI based multi-TRP, definition of CORESETPoolIndex = {0, 1} is reused.

Proposal 2-6:

  - For unified TCI extension for multi-DCI based multi-TRP,

    - For joint TCI state, one DCI associated with CORESETPoolIndex = 0 indicates one joint TCI state (1st indicated TCI state), and the other DCI associated with CORESETPoolIndex = 1 indicates one joint TCI state (2nd indicated TCI state).

    - For separate TCI state, one DCI associated with CORESETPoolIndex = 0 indicates one DL TCI state and/or one UL TCI state (1st indicated TCI state), and the other DCI associated with CORESETPoolIndex = 1 indicates one DL TCI state and/or one UL TCI state (2nd indicated TCI state).

Proposal 2-7:

  - For unified TCI extension for multi-DCI based multi-TRP, discuss whether to increase the max. number of RRC configured TCI states from Rel.17.

Proposal 3-1:

  - The extension of unified TCI framework can be applied to simultaneous multi-panel PUSCH/PUCCH Tx schemes (if supported). Details can be discussed after the support of simultaneous multi-panel PUSCH/PUCCH transmission schemes is discussed and decided in agenda item 9.1.4.1.

Proposal 4-1:

  - For M-TRP operation, with extension of unified TCI framework, per TRP power control can be supported by Rel-17 unified TCI power control framework, i.e., power control parameters are associated with each TCI state.

Proposal 4-2:

  - For simultaneous multi-panel PUSCH/PUCCH Tx scheme (if supported), how to determine PCMAX for transmission from each panel can be further studied.

- Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

[2] 3GPP R1-2204372, “Discussion on multi-panel transmission”, NTT DOCOMO, INC., RAN1#109e, May 2022