3GPP TSG RAN WG1 #109-e			R1-2204368

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on two TAs for multi-DCI

Agenda Item:	9.1.1.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94e meeting, a new WID [1] “MIMO Evolution for Downlink and Uplink” was approved. One of the objectives is as follows.

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we discuss enhancements on two TAs for UL transmission for M-DCI M-TRP operation.

- Discussion

In M-TRP deployment, propagation delay between UE and two TRPs can be different. If a common TA is used for UL transmission to two TRPs, the difference of propagation delay will lead to UL timing misalignment at TRP. In case the UL timing misalignment is larger than CP length, it will degrade UL performance. Thus, it is beneficial to support per TRP TA in unsynchronized M-TRP operation.

Proposal 2.1:

- Support per TRP TA in unsynchronized M-TRP operation.

In Rel-17, UL frame number i for transmission from UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE, where NTA is indicated by MAC RAR and TAC MAC CE. TAC in MAC RAR indicates initial TA used for the first uplink transmission after the reception of RAR. TAC in MAC RAR indicate a value of TA = 0, 1, 2, ..., 3846 and NTA=TA∙16∙642μ. TAC in TAC MAC CE indicates TA adjustment from current TA to new TA. TAC in TAC MAC CE indicates a value of TA = 0, 1, 2, ..., 63 and NTA_new=NTA_old+TA-31∙16∙642μ. To support per TRP TA, TAC in TAC MAC CE and MAC RAR should be considered separately.

For TAC in TAC MAC CE, since it indicates TA adjustment from current TA to new TA, the indication should per TRP. In Rel-17, TA command in TAC MAC CE is indicated per TAG where each TAG may contain multiple serving cells and TA command indicated for a TAG applies to all serving cells in the TAG. To indicate per TRP TA adjustment, following options can be considered.

- Option1: Configure different TRPs of a serving cell in different TAG. TAC in TAC MAC CE is indicated per TAG, same as in Rel-17.

- Option2: Different TRPs of a serving cell are within one TAG. TAC in TAC MAC CE s indicated per TRP, e.g., TRP ID is indicated in TAC MAC CE.

Proposal 2.2:

- Support per TRP indication of TA adjustment via TAC MAC CE. Following options can be considered.

  - Option1: Configure different TRP of a serving cell in different TAG. TAC in TAC MAC CE is indicated per TAG same as in Rel-17.

  - Option2: Different TRPs of a serving cell are within one TAG. TAC in TAC MAC CE is indicated per TRP, e.g., TRP ID is indicated in TAC MAC CE.

Regarding TAC in MAC RAR, whether per TRP indication is supported needs to be studied first. Since TAC in MAC RAR indicates initial TA for first uplink transmission after the reception of RAR, the other possible option is that initial TA indicated via MAC RAR is common for two TRPs, and per TRP TA is adjusted by TAC in TAC MAC CE as discussed above.

Proposal 2.3:

- For TAC in MAC RAR, following options can be considered.

  - Option1: initial TA indicated via MAC RAR is common for two TRPs.

  - Option2: support per TRP indication of initial TA via MAC RAR

Furthermore, TA is applied to PUSCH/PUCCH/SRS transmission. With per TRP TA, how to associate a PUSCH/PUCCH/SRS transmission with a TRP needs to be studied.

Proposal 2.4:

- Study association between PUSCH/PUCCH/SRS transmission and TRP.

In Rel-17, timeAlignmentTimer is maintained per TAG and UE behavior is defined for the cases when timeAlignmentTimer for PTAG is expired and when timeAlignmentTimer for STAG is expired. PTAG refers a TAG containing SpCell, whereas STAG refers to other TAGs. With per TRP TA, impact on timeAlignmentTimer needs to be studied. For example, with above option1 in Proposal 2.2 for TAC MAC CE (i.e., different TRP of a serving cell is contained in different TAG), one serving cell may be contained in two TAGs, then how to determine PTAG and STAG and UE behavior when timeAlignmentTimer expires needs to be studied. And with above option2 for TAC MAC CE, considering TAC is indicated per TRP instead of per TAG, whether to maintain timeAlignmentTimer per TRP needs to be considered.

Proposal 2.5:

- Study the impact of per TRP TA on timeAlignmentTimer.

- Conclusion

In this contribution, we discussed enhancements on two TAs for UL for M-DCI M-TRP operation. Based on the discussion, we made following proposals.

Xxx

Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94e, Dec. 2021