3GPP TSG RAN WG1 #109-e		R1-2204370

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on increased number of orthogonal DMRS ports

Agenda Item:	9.1.3.1

Document for: 	Discussion and Decision

- Introduction

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM,

- Striving for a common design between DL and UL DMRS

- Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

[…]

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

- Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for DMRS enhancements, as shown below.

In this contribution, we discuss extension for increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO for Rel-18 MIMO.

- Increasing orthogonal DMRS ports

2.1 Scope

In Rel.15, up to 12 orthogonal DMRS ports are specified. For SU-MIMO/MU-MIMO, different DMRS ports are allocated to different MIMO layers. The max. number of DMRS ports limits the total number of MIMO layers for SU-MIMO/MU-MIMO. For example, for UEs with 4-layer SU-MIMO, up to 3 UEs are multiplexed by MU-MIMO (total MIMO layers are 12). Increasing the max. number of DMRS ports allows to multiplex with a larger number of UEs, and it improves resource efficiency.

We believe increasing the max. number of DMRS ports is beneficial, and we should increase the max. number of DMRS ports for both DMRS type 1 and DMRS type 2 and for both single symbol and double symbol DMRS. Table 2-1 summarizes the max. number of DMRS ports in Rel.15 and proposed max. number of DMRS ports in Rel.18. We believe this enhancement is beneficial not only for MU-MIMO but also for SU-MIMO. For example, to support more than 4-layer SU-MIMO in DMRS type 1, double symbol DMRS is required in Rel.15. However, DMRS overhead of double symbol DMRS is twice than single symbol DMRS. If we increase the max. number of DMRS ports for single symbol DMRS of DMRS type 1 to 8-port, single symbol DMRS can be used for up to 8-layer SU-MIMO, which we believe beneficial for SU-MIMO case as well as MU-MIMO.

Table 2-1. The max. number of DMRS ports in Rel.15 and proposal in Rel.18.

Proposal 2-1

- Specify to increase the max. number of DMRS ports for PDSCH/PUSCH by 2 times larger than Rel.15.

  - For DMRS type 1, the max. number of DMRS ports for PDSCH/PUSCH is

    - Single symbol DMRS: 8 DMRS ports.

    - Double symbol DMRS: 16 DMRS ports.

  - For DMRS type 2, the max. number of DMRS ports for PDSCH/PUSCH is

    - Single symbol DMRS: 12 DMRS ports.

    - Double symbol DMRS: 24 DMRS ports.

2.2 Possible options to increase DMRS ports

Regarding how to increase the max. number of DMRS ports without increasing DMRS overhead, table 2-2 summarizes possible options:

Table 2-2. Possible options to increase DMRS ports in Rel.18.

| Possible options | Pros. | Cons. |
| --- | --- | --- |
| - Increase FD-OCC length (increase the number of DMRS ports within a CDM group) - FD-OCC length 2 4 or 6 | - Applicable to all cases - No impact by high UE velocity | - Performance degradation would be observed for large delay spread. |
| - Sparser frequency allocation (increase the number of CDM groups) | - Applicable to all cases - No impact by high UE velocity | - Performance degradation would be observed due to sparser frequency allocation. |
| - Utilize TD-OCC over non-contiguous DMRS symbols (e.g. across front/additional DMRS symbols) | - No impact by large delay spread | - Not applicable without additional DMRS - Restriction of freq. hopping |

For the comparison between the options, we would need to see the evaluation results. However, we can predict the tendency of performance that Opt.1 and 2 would be better than Opt.3 in case of high UE velocity. On the other hand, the performance of Opt.3 would be better than Opt.1 or Opt.2 in case of large delay spread. Between Opt.1 and Opt.2, Opt.1 would be better than Opt.2 especially in small delay spread, due to denser DMRS frequency allocation. Another important aspect is that frequency hopping is usually applied to PUSCH for coverage enhancement, and in that case, it would be difficult to apply TD-OCC across non-contiguous DMRS symbols in Opt.3. Hence, our preference is Opt.1.

Proposal 2-2

- Consider the following options to increase the max. number of DMRS ports for PDSCH/PUSCH (Opt.1 is preferred).

  - Opt.1) Increase FD-OCC length (increase the number of DMRS ports within a CDM group). FD-OCC length is increased from 2 to 4 or 6.

  - Opt.2) Sparser frequency allocation (increase the number of CDM groups).

  - Opt.3) Utilize TD-OCC over non-contiguous DMRS symbols (e.g. across front/additional DMRS symbols)

2.3 Details of FD-OCC (Opt.1)

We discuss more details on how to enhance FD-OCC in Opt.1. In Rel.15, length 2 FD-OCC is applied. For DMRS type 1, length 2 FD-OCC is applied to nearby two REs. For DMRS type 2, length 2 FD-OCC is applied to consecutive two REs. As illustrated in figure 2-1 and 2-2, following options can be considered to enhance FD-OCC length:

- For DMRS type 1:

  - Opt.1-1: length 6 is applied to 6 REs within a PRB

  - Opt.1-2: length 4 is applied to nearby 4 REs within a PRB or across consecutive PRBs

- For DMRS type 2:

  - length 4 is applied to 4 REs within a PRB

For Opt.1-1 of DMRS type 1, length 6 FD-OCC is applied to 6 REs in a PRB. This option can be applied to any number of PRBs of PDSCH/PUSCH. For Opt.1-2, when length 4 FD-OCC is applied to 6 REs in a PRB, another length 4 FD-OCC should be applied to 4 REs across consecutive two PRBs. Hence, the number of PRBs for PDSCH/PUSCH should be even. From performance perspective, Opt.1-2 (FD-OCC length 4) has better performance than Opt.1-1 (FD-OCC length 6), especially in case of large delay spread. From system perspective, we believe the constraint that gNB should schedule even number of PRBs for PDSCH/PUSCH is not a big issue. If we consider the use-case of this feature of larger DMRS ports in DMRS type 1, we believe this feature will be used for more than 4-layer of SU-MIMO or more than 8 total DMRS ports of MU-MIMO. In such scenario, we don’t think gNB will need to schedule one or odd number of PRBs for PDSCH/PUSCH. Hence, we believe Opt.1-2 is preferable for DMRS type 1.

For DMRS type 2, length 4 OCC can be applied within a PRB. This option can be applied to any number of PRBs for PDSCH/PUSCH.

Proposal 2-3

- For enhancement of FD-OCC length for DMRS of PDSCH/PUSCH, following options can be considered:

  - For DMRS type 1 (Opt.1-2 is preferred):

    - Opt.1-1: length 6 is applied to 6 REs within a PRB

    - Opt.1-2: length 4 is applied to nearby 4 REs within a PRB or across consecutive PRBs

      - The number of PRBs for PDSCH/PUSCH should be even.

  - For DMRS type 2:

    - length 4 is applied to 4 REs within a PRB

Figure 2-1. Extension of FD-OCC for DMRS type 1 (CDM group 0).

Figure 2-2. Extension of FD-OCC for DMRS type 2 (CDM group 0).

For length 4 FD-OCC (Opt.1-2), simple solution is to use Walsh matrix. Length 4 FD-OCC made by Walsh matrix is shown in Table 2-3. This matrix is also used for TD-OCC of CSI-RS.

Table 2-3. Length 4 FD-OCC made by Walsh matrix.

| OCC index | wf(0) | wf(1) | wf(2) | wf(3) |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | -1 |
| 2 | +1 | +1 | -1 | -1 |
| 3 | +1 | -1 | -1 | +1 |

Proposal 2-4

- For FD-OCC length 4, following OCC can be considered:

  - OCC index #0 = [+1  +1  +1  +1]

  - OCC index #1 = [+1   -1  +1   -1]

  - OCC index #2 = [+1  +1   -1   -1]

  - OCC index #3 = [+1   -1   -1  +1]

2.4 Multiplexing with Rel.15/16 UEs for MU-MIMO

Another discussion point is whether and how to multiplex between Rel.15-17 UEs and Rel.18 UEs for MU-MIMO. From system perspective, it is preferable to allow MU-MIMO across Rel.15-17 UEs and Rel.18 UEs. One simple solution is to multiplex Rel.15 DMRS and new Rel.18 DMRS by different CDM group (i.e. by FDM). However, this solution is not flexible enough, because the number of CDM group is limited (up to 2 for DMRS type 1 and up to 3 for DMRS type 2).

Another solution is to multiplex Rel.15 DMRS and new Rel.18 DMRS within the same CDM group. In this case, orthogonal DMRS sequence between Rel.15 DMRS and new Rel.18 DMRS is required. Table 2-4 shows length 2 FD-OCC in Rel.15. If we increase the max. number of DMRS ports by extending FD-OCC (Opt.1), as the above proposals, it is possible to have orthogonal FD-OCC between Rel.15 DMRS and Rel.18 DMRS within the same CDM group. For example, we can observe that length 4 OCC index #0 = [+1  +1  +1  +1] and length 4 OCC index #1 = [+1   -1  +1   -1] of Table 2-3 are the same as length 2 OCC index #0 =  [+1  +1] and length 2 OCC index #1 = [+1   -1] of Table 2-4. Hence, if gNB allocates length 2 OCC index #0 and/or #1 to Rel.15-17 UEs and length 4 OCC index #2 and/or #3 to Rel.18 UEs, these OCCs are orthogonal. However, whether the receiver can correctly decode the FD-OCC depends on how many numbers of REs are used by the receiver to decode FD-OCC. For example, if receiver uses received signal on one or multiple PRB(s) (i.e. multiple number of 4 REs), the receiver can decode FD-OCC even if DMRS with different FD-OCC length 2 or 4 is multiplexed. However, if the receiver uses received signal on multiple number of 2 REs to decode FD-OCC, the receiver cannot decode FD-OCC when DMRS with length 4 FD-OCC is multiplexed. For PUSCH, since the receiver is gNB, and gNB understands different length of FD-OCC is multiplexed, it is possible to decode Rel.15 DMRS and Rel.18 DMRS correctly. On the other hand, for PDSCH, since the receiver is UE, it may be difficult for Rel.15 UE to decode DMRS when Rel.18 DMRS is multiplexed.

Table 2-4. Length 2 FD-OCC in Rel.15.

| OCC index | wf(0) | wf(1) |
| --- | --- | --- |
| 0 | +1 | +1 |
| 1 | +1 | -1 |

Proposal 2-5

- Study whether/how to multiplex Rel.15 DMRS and Rel.18 DMRS with different DMRS port indexes.

- DMRS enhancement for 8 Tx UL SU-MIMO

In existing spec., antenna port indication tables for PUSCH are defined for UL rank from 1 to 4. As discussed in our companion contribution [2], 8TX UL transmission with up to 8 layers per UE should be supported. For PUSCH transmission with larger than 4 layers, each UE can be indicated with up to 8 DMRS antenna ports for a PUSCH transmission. Hence, new antenna port indication tables should be introduced for UL rank from 5 to 8.

In Rel-15, for the 4 cases of DMRS configurations in Table 2-1, antenna port indication tables are defined for rank 1, 2, 3, and 4, respectively. The antenna port indication for higher rank should be also discussed for the 4 cases respectively.

- Case1: DMRS type 1, single symbol, 4 ports

  - It can only support up to rank 4 per UE, so it cannot be configured for larger than 4 layers.

- Case2: DMRS type 1, double symbol, 8 ports

  - It can support up to rank 8 per UE. Hence, new antenna port indication tables can be defined for rank 5, 6, 7, and 8, respectively.

- Case3: DMRS type 2, single symbol, 6 ports

  - It can support up to rank 6 per UE. But it cannot be configured for larger than 6 layers. Hence, we can discuss whether to define new antenna port indication tables for rank 5 and 6, respectively.

- Case4: DMRS type 2, double symbol, 12 ports

  - It can support up to rank 8 per UE. Hence, new antenna port indication tables can be defined for rank 5, 6, 7, and 8, respectively.

Proposal 3-1

- Discuss to define new antenna point indication tables for following cases of DMRS configurations.

  - Case1: DMRS type 1, single symbol, 4 ports: no enhancement.

  - Case2: DMRS type 1, double symbol, 8 ports: define new antenna indication tables for rank 5, 6, 7, and 8, respectively.

  - Case3: DMRS type 2, single symbol, 6 ports: define new antenna indication tables for rank 5 and 6, respectively.

  - Case4: DMRS type 2, double symbol, 12 ports: define new antenna indication tables for rank 5, 6, 7, and 8, respectively.

In Rel-18, if the max. number of DMRS ports is increased by two times as discussed in Section 2, even the Case1 and Case3 above can support up to rank 8 per UE, which is also beneficial from resource efficiency perspective, as analysed in Section 2. Hence, we should first discuss and determine whether to support the increased max. number of DMRS ports for 8TX UL transmission for larger than 4 layers, and whether to support it for all cases or partial cases of DMRS configurations. Then we can discuss how to define the new antenna port indication tables for the cases with increased max. number of DMRS ports, and we need to consider its compatibility with the antenna port indications for the cases with existing max. number of DMRS ports.

Proposal 3-2

- Support the increased max. number of DMRS ports for 8TX UL transmission for larger than 4 layers.

- Conclusion

In this contribution, we discussed extension for increasing orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO for Rel-18 MIMO. Based on the discussion, we made following proposals.

For increasing orthogonal DMRS ports

Proposal 2-1

- Specify to increase the max. number of DMRS ports for PDSCH/PUSCH by 2 times larger than Rel.15.

  - For DMRS type 1, the max. number of DMRS ports for PDSCH/PUSCH is

    - Single symbol DMRS: 8 DMRS ports.

    - Double symbol DMRS: 16 DMRS ports.

  - For DMRS type 2, the max. number of DMRS ports for PDSCH/PUSCH is

    - Single symbol DMRS: 12 DMRS ports.

    - Double symbol DMRS: 24 DMRS ports.

Proposal 2-2

- Consider the following options to increase the max. number of DMRS ports for PDSCH/PUSCH (Opt.1 is preferred).

  - Opt.1) Increase FD-OCC length (increase the number of DMRS ports within a CDM group). FD-OCC length is increased from 2 to 4 or 6.

  - Opt.2) Sparser frequency allocation (increase the number of CDM groups).

  - Opt.3) Utilize TD-OCC over non-contiguous DMRS symbols (e.g. across front/additional DMRS symbols)

Proposal 2-3

- For enhancement of FD-OCC length for DMRS of PDSCH/PUSCH, following options can be considered:

  - For DMRS type 1 (Opt.1-2 is preferred):

    - Opt.1-1: length 6 is applied to 6 REs within a PRB

    - Opt.1-2: length 4 is applied to nearby 4 REs within a PRB or across consecutive PRBs

      - The number of PRBs for PDSCH/PUSCH should be even.

  - For DMRS type 2:

    - length 4 is applied to 4 REs within a PRB

Proposal 2-4

- For FD-OCC length 4, following OCC can be considered:

  - OCC index #0 = [+1  +1  +1  +1]

  - OCC index #1 = [+1   -1  +1   -1]

  - OCC index #2 = [+1  +1   -1   -1]

  - OCC index #3 = [+1   -1   -1  +1]

Proposal 2-5

- Study whether/how to multiplex Rel.15 DMRS and Rel.18 DMRS with different DMRS port indexes.

For 8 Tx UL SU-MIMO

Proposal 3-1

- Discuss to define new antenna point indication tables for following cases of DMRS configurations.

  - Case1: DMRS type 1, single symbol, 4 ports: no enhancement.

  - Case2: DMRS type 1, double symbol, 8 ports: define new antenna indication tables for rank 5, 6, 7, and 8, respectively.

  - Case3: DMRS type 2, single symbol, 6 ports: define new antenna indication tables for rank 5 and 6, respectively.

  - Case4: DMRS type 2, double symbol, 12 ports: define new antenna indication tables for rank 5, 6, 7, and 8, respectively.

Proposal 3-2

- Support the increased max. number of DMRS ports for 8TX UL transmission for larger than 4 layers.

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

- 3GPP R1-2204373, “Discussion on 8TX UL transmission”, RAN1#109-e, May 2022.