3GPP TSG RAN WG1 #109-e			R1-2204371

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on SRS enhancement

Agenda Item:	9.1.3.2

Document for: 	Discussion and Decision

- Introduction

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives requiring SRS enhancement, M-TRP CJT and 8TX UL transmission.

In this contribution, we discuss SRS enhancement for 8TX UL transmission and M-TRP CJT in Rel-18 MIMO.

- SRS enhancement for 8 TX UL transmission

As discussed in our companion contribution [2] for 8TX UL transmission, 8-port SRS resource with usage of ‘codebook’ is needed to support up to 8 layers for codebook-based PUSCH transmission. In addition, SRS enhancement for antenna switching for 8T8R should be also discussed.

2.1 Support of 8-port SRS

In order to support up to 8 layers for codebook-based PUSCH transmission, we think one of the most straightforward ways is to introduce 8-port SRS, i.e., extend the maximum number of SRS ports.

In NR, so far, up to four ports are available for SRS transmission per UE, which follows the following extraction from 38.211 [3]:

##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift αi for antenna port pi is given as αi=2πnSRScs,inSRScs,max nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise, where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1. […] Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC. KTC; nSRScs,max. 2; 8. 4; 12. 8; 6. ##### 6.4.1.4.3 Mapping to physical resources […] The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0.

In summary:

- For 2-port SRS, the same comb index is applied for both ports. Port multiplexing is achieved by applying different cyclic shifts (with the most discrete phase rotation values).

- For 4-port SRS,

  - For comb index, when the maximum number of cyclic shifts is 6 (i.e., comb8 is configured), or the configured cyclic shift is larger than half of the maximum number of cyclic shifts, two different comb indexes are applied: one for port 1000 and 1002, and the other for port 1001 and 1003. Otherwise, the same comb index is applied across all the ports, similar to 2-port SRS.

  - For cyclic shift values, when the maximum number of cyclic shifts is 6 (i.e., comb8 is configured), two different cyclic shift values are applied: one for port 1000 and 1001, and the other for 1002 and 1003. Otherwise, different comb indexes are used for each port.

Observation 2-1

- Legacy SRS supports up to 4 ports by using multiple cyclic shifts and/or multiple comb indexes depending on the maximum number of cyclic shifts

We believe the above should be a starting point for the discussion on 8-port SRS. On top of that, we see several potential directions to extend the number of SRS ports as described below:

- Use of more cyclic shift values

So far, up to four cyclic shifts are available for SRS transmissions, where different SRS port is associated with different cyclic shift values. Similarly, more than four cyclic shifts (e.g., eight cyclic shifts) can be considered to support 8-port SRS transmissions. In this case, the amount of resources required for the SRS transmissions can be minimized. However, orthogonality among ports can be degraded since the difference of phase rotation is smaller.

- Use of more frequency domain resources (e.g., more comb indexes)

As summarized above, except for some cases, a single comb index is shared by SRS ports transmitted by a UE. For 8-port SRS, more comb indexes (i.e., two indexes always, or four comb indexes) can be considered. In this case, cyclic shift values used for multiplexing in a RE can be kept as in Rel-17, thus, the same orthogonality among ports would be achieved. On the other hand, more comb indexes a UE uses, more frequency domain resources it consumes. It may degrade the system wise performance especially for UL. Also, per-symbol multiplexing capacity for different UEs may be degraded.

- Use of more time-domain resources

The third direction is to use more SRS symbols to express more than 4 SRS ports. Generally this direction has similar pros and cons to the use of more frequency-domain resources, i.e., orthogonality between different ports can be kept as in Rel-17, while more resources are needed for SRS transmission from UE perspective. Meanwhile, this direction could be a bit better than the use of more frequency-domain resources since the per-symbol multiplexing capacity may not be degraded, considering the amount of frequency-domain resources may not need to be increased.

Several approaches can be considered for this direction. One is, similar to DM-RS, TD-OCC across multiple SRS symbols. Another is, for example, to define the relationship between SRS port number(s) and the symbol number(s) configured for the SRS transmission. The former approach may have more significant specification/implementation impact since TD-OCC for SRS transmission will be a totally new functionality. The latter approach can be a bit more implementation-friendly, but may still require a certain amount of specification impact for e.g., specifying the relationship between SRS port and SRS symbol, which is also a new aspect for SRS. One note for the latter approach is that it may no longer be defined as 8-port SRS, rather a combination of the legacy SRS across symbols.

Table 2.1-1: Summary of the potential directions for supporting 8-port SRS

| Direction # | Pros | Cons |
| --- | --- | --- |
| - More cyclic shift values | The same amount of resources as in Rel-17 can be achieved | Orthogonality between different SRS ports can be degraded |
| - More frequency-domain resources | Orthogonality between different SRS ports can be kept as in Rel-17 | More resources in frequency-domain are required |
| - More time-domain resources | Orthogonality between different SRS ports can be kept as in Rel-17 | More resources in time-domain are required |

At this very beginning stage of the WI, we in general believe any direction can be studied further in RAN1. Meanwhile, we understand that the amount of workload for each agenda should be assessed and decided carefully. If the discussion needs to be narrowed down more, our suggestion is to focus on the direction#2 and/or #3, i.e., extension of cyclic shift values can be precluded, since its cons of the orthogonality degradation may be more significant issue. The other two directions, although it may consume more resources, can avoid such issue.

Another aspect could be whether to support 8-port SRS by using only one SRS resource configuration or not. So far, in NR, the supported 4-port SRS can be configured by using only one SRS resource configuration.

Given above, we draw the following proposals

Proposal 2-1

- Study how to support 8-port SRS transmission, based on Rel-17 SRS, considering at least the following directions

  - Use of more cyclic shifts (lower priority)

  - Use of more frequency domain resources

    - E.g., use of more comb indexes

  - Use of more time-domain resources

    - E.g., use of more SRS symbols for multi-port SRS transmission

2.2 SRS for 8T8R antenna switching

In Rel-15, SRS antenna switching configuration for DL CSI acquisition was supported, where less than 4 Rx ports with 1/2/4 Tx port(s) are considered. For T=R (1T=1R, 2T=2R, or 4T=4R), up to two SRS resource sets each with one SRS resource can be configured with usage of ‘antennaSwitching’. In Rel-17, this functionality was extended to cover antenna architectures with 6/8 Rx ports, where the maximum number of Tx ports is still 4. With a new Rel-17 UE capability reporting, up to two SRS resources configured with ‘Semi-persistent’ and up to one SRS resource set configured with ‘periodic’ can be configured.

In Rel-18, since we consider more than 4 layers for UL transmission, additional antenna architecture(s), e.g., 6 or 8 Tx ports, can be considered as well. Therefore, depending on the exact approach(s) to support more than 4 layers for UL transmission, whether/how to support SRS antenna switching configuration for such antenna architectures may need to be discussed in Rel-18. We see this aspect much dependent on whether to support 8-port SRS, as described in section 2.1. If we support 8-port SRS for antenna switching, full use of 8 Tx ports can be considered for the purpose of SRS antenna switching. On the other hand, if we do not support 8-port SRS for antenna switching, assuming the number of Tx ports for SRS transmission is kept as in Rel-17, we do not see the need to discuss a specification impact on this functionality since all cases can be covered by Rel-17 enhanced antenna switching configurations.

Proposal 2-2

- Discuss SRS configuration with usage of  ‘antennaSwitching’ for 8T8R.

  - Study whether to configure 8-port SRS for antenna switching.

- SRS enhancement for TDD CJT

As discussed in our companion contribution [4] for CJT, there are two deployment scenarios, intra-site M-TRP CJT and inter-site M-TRP CJT. For both scenarios, in TDD operation, UE can be configured with SRS transmission and multiple TRPs can measure the SRS to derive DL CSI.

3.1 SRS capacity enhancement

Since all the CJT UEs from multiple TRPs should be configured with SRS transmission for measurement by multiple TRPs, it is important to guarantee the orthogonality of those SRS transmissions in multiple TRPs. As more SRS resources may be required due to the increased number of TRPs, the current capacity of orthogonal SRS resources may not be sufficient. Thus, it could be necessary to improve SRS capacity. Otherwise, NW may have difficulty to accommodate all the SRS resources within limited orthogonal resources, which results in inefficient usage of UL resources for SRS.

In Rel-17, partial sounding has improved the SRS multiplexing capacity in frequency domain. Considering following constraints on SRS enhancement for TDD CJT in Rel-18 MIMO WID: 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences, further consideration on time domain (e.g., TD-OCC), frequency domain (e.g., partial sounding with larger partial frequency sounding factor), or sequence domain (e.g., larger max. cyclic shift) enhancement can be discussed. With above methods, SRS transmissions from more UEs can be multiplexed on the same UL resources.

Proposal 3-1

- Discuss to improve SRS capacity via time domain (e.g., TD-OCC), frequency domain (e.g., larger partial frequency sounding factor), or sequence domain (e.g., larger max. cyclic shift) enhancement.

3.2 Interference randomization for SRS

If there are many CJT UEs from multiple TRPs configured with SRS transmission for DL CSI acquisition, the orthogonality of those SRS transmissions cannot be guaranteed. Then the collision of two SRS resources from different TRPs is un-avoided and inter-TRP cross-SRS interference exists. As shown in Fig. 3-1, if SRS resources from UEs in different TRPs collide, the SRS resource from a CJT UE will suffer strong interference from a cell center UE. In addition, due to semi-static configuration of SRS parameters, the strong inter-TRP interference is continuous and will largely impact the accuracy of DL CSI acquisition.

Fig 3-1: An example of inter-TRP cross-SRS interference

To avoid continuous serious inter-TRP interference on SRS measurement, how to achieve interference randomization for SRS transmission should be studied. The interference randomization can be considered in terms of time, frequency or sequence domain. For example, dynamic update of SRS resource parameters, such as time/frequency resource allocation, hopping, sequence group number, sequency number, comb, CS, etc., can be beneficial to randomize the interference in time, frequency, or sequence domain. In addition, making some SRS parameters’ generation related to slot/symbol index as well as configurable index(es) (e.g., TRP index) could also be considered to randomize the SRS sequence/resource and avoid inter-TRP interference.

Proposal 3-2

- Discuss following enhancements for inter-TRP cross-SRS interference randomization.

  - Dynamic update of some SRS resource parameters, e.g., time/frequency resource allocation, hopping, sequence group number, sequency number, comb, CS, etc.

  - Enhanced determination of some SRS resource parameters, which could be related to slot/symbol index or TRP index.

- Conclusion

In this contribution, we discussed SRS enhancement for 8TX UL transmission and M-TRP CJT in Rel-18 MIMO. Based on the discussion, we made following proposals.

SRS enhancement for 8TX UL transmission

Observation 2-1

- Legacy SRS supports up to 4 ports by using multiple cyclic shifts and/or multiple comb indexes depending on the maximum number of cyclic shifts

Proposal 2-1

- Study how to support 8-port SRS transmission, based on Rel-17 SRS, considering at least the following directions

  - Use of more cyclic shifts (lower priority)

  - Use of more frequency domain resources

    - E.g., use of more comb indexes

  - Use of more time-domain resources

    - E.g., use of more SRS symbols for multi-port SRS transmission

Proposal 2-2

- Discuss SRS configuration with usage of  ‘antennaSwitching’ for 8T8R.

  - Study whether to configure 8-port SRS for antenna switching.

SRS enhancement for TDD CJT

Proposal 3-1

- Discuss to improve SRS capacity via time domain (e.g., TD-OCC), frequency domain (e.g., larger partial frequency sounding factor), or sequence domain (e.g., larger max. cyclic shift) enhancement.

Proposal 3-2

- Discuss following enhancements for inter-TRP cross-SRS interference randomization.

  - Dynamic update of some SRS resource parameters, e.g., time/frequency resource allocation, hopping, sequence group number, sequency number, comb, CS, etc.

  - Enhanced determination of some SRS resource parameters, which could be related to slot/symbol index or TRP index.

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

- 3GPP R1-2204373, “Discussion on 8 TX UL transmission”, RAN1#109-e, May 2022.

- 3GPP TSG RAN 38.211 V17.1.0, “Physical channels and modulation”, March 2022

- 3GPP R1-2204369, “Discussion on CSI enhancement”, RAN1#109-e, May 2022.