3GPP TSG RAN WG1 #109-e			R1-2204372

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on multi-panel transmission

Agenda Item:	9.1.4.1

Document for: 	Discussion and Decision

- Introduction

In RAN#94e meeting, a new WID [1] “MIMO Evolution for Downlink and Uplink” was approved. One of the objectives is as follows.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this contribution, we discuss enhancements on simultaneous multi-panel UL transmission.

- Discussion on multi-panel PUSCH transmission

2.1 Multi-panel PUSCH transmission in S-DCI M-TRP

With S-DCI M-TRP operation, simultaneous multi-panel transmission can be beneficial for reliability improvement and throughput improvement of PUSCH.

For PUSCH reliability, in Rel-17, TDM M-TRP PUSCH repetition was supported. In addition to Rel-17 M-TRP PUSCH repetition, with simultaneous multi-panel transmission, following schemes can be further considered for reliability enhancement.

- Multi-panel/M-TRP FDM PUSCH repetition: two PUSCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively.

- Multi-panel/M-TRP SFN PUSCH repetition: all REs of a PUSCH are associated with two beams/panels.

Figure1a. FDM repetition                       Figure1b. SFN repetition

Link-level simulation is performed to evaluate the performance of FDM repetition and SFN repetition. The simulation assumptions are given in Table I in appendix.  In the simulation, repetition number is 2. For TDM and FDM repetition, we assume two repetitions are associated with different beam and different precoding. For SFN repetition, we assume two repetitions are associated with different beam but the same precoding. From the simulation results in Fig. 2, it can be observed that multi-panel/M-TRP FDM repetition achieves almost the same BLER performance as M-TRP TDM repetition, while multi-panel/M-TRP SFN repetition has worse performance that TDM and FDM repetition. On other hand, with simultaneous multi-panel transmission, FDM repetition and SFN repetition can achieve lower latency, compared to Rel-17 TDM M-TRP PUSCH repetition.

Figure2. BLER performance of TDM/FDM/SFN repetition

Observation 2.1:

- Multi-panel/M-TRP FDM repetition achieves almost the same BLER performance as M-TRP TDM repetition, while multi-panel/M-TRP SFN repetition has worse BLER performance that TDM and FDM repetition.

Proposal 2.1:

- For S-DCI M-TRP, for reliability improvement, consider following schemes of simultaneous multi-panel PUSCH transmission.

  - Multi-panel/M-TRP FDM PUSCH repetition: two PUSCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively,

  - Multi-panel/M-TRP SFN PUSCH repetition: all REs of a PUSCH are associated with two beams/panels

For throughput improvement, following schemes of simultaneous multi-panel transmission can be considered.

- Multi-panel/M-TRP SDM scheme with 1 TB/CW: 1 TB/CW is transmitted in a PUSCH. Different layer of a PUSCH is associated with different beam/panel.

- Multi-panel/M-TRP SDM scheme with 2 TBs/CWs: two TBs/CWs are transmitted in a PUSCH. Different TB/CW is associated with different beam/panel and mapped to different layer.

Figure3a. SDM with 1TB/CW                     Figure3b. SDM with 2TBs/CWs

System level simulation is performed to evaluate the distribution of rank of single panel Tx. The simulation assumptions are given in Table II in appendix. From the simulation results in Fig. 3, it can be observed that in single panel Tx, the probability of high rank (rank=3 or 4) is quite low. Probability of rank=4 is almost 0%, and probability of rank=3 is about 1%. With multi-panel SDM transmission, the probability of high rank could be improved.

Figure3. Rank distribution in case of single panel Tx

Observation 2.2:

- With single panel Tx, the probability of high rank (rank=3 or 4) is very low. Probability of rank=4 is almost 0%, and probability of rank=3 is about 1%.

System level simulation is performed to evaluate SINR of simultaneous multi-panel Tx. The simulation assumptions are given in Table III  in appendix.  From the simulation results in Fig. 4, it can be observed that simultaneous multi-panel Tx can provide 3dB gain of 50%-ile UE SINR compared to single-panel Tx. In Fig.4, “1st panel” and “2nd panel” correspond to single-panle Tx. “1st panel” means single panel Tx using a panel with the higher SINR among 2 panels and  “2nd panel” means single panel Tx using a panel with the lower SINR among 2 panels. In multi-panel Tx, both “1st panel” and “2nd panel” are used.

Figure4. CDF of SINR of multi-panel Tx

Considering whether 1 TB/CW or 2 TBs/CWs is scheduled with multi-panel SDM transmission, one benefit of scheduling 2 TBs/CWs is that MCS can be indicated per TB/CW, i.e., indicated per panel. Figure 5 shows BLER-SNR curves of different MCS. It can be observed that the gap of SNR between different MCS is about 1dB. Meanwhile, system level simulation is performed to evaluate RSRP of two panels. The simulation assumptions are given in Table III in appendix. From the simulation results in Fig. 6, it can be observed that the probability is about 65% that the RSRP gap between two panels is larger than 1 dB. Thus, it can be inferred that supporting two TBs/CWs transmitted with two panels and with per TB/CW (per panel) MCS is beneficial. As discussed in our companion contribution [2], if 2 codewords PUSCH is supported for simultaneous multiple-panel transmission for up to 4 layers, the design on 2 codewords PUSCH is common and can be discussed jointly with that for 8TX UL transmission.

Figure5. BLER-SNR curves of different MCSs

Figure6. CDF of RSRP difference between two panels

Observation 2.3:

- The gap of SNR between two different MCS is about 1dB. The probability is about 65% that the RSRP gap between two panels is above 1 dB.

- It can be beneficial that two TBs/CWs are transmitted with two panels and with per TB/CW (per panel) MCS

Proposal 2.2:

- For S-DCI M-TRP, for throughput improvement, consider following schemes of simultaneous multi-panel PUSCH transmission.

  - Multi-panel/M-TRP SDM scheme with 1 TB/CW: 1 TB/CW is transmitted in a PUSCH. Different layer of a PUSCH is associated with different beam/panel.

  - Multi-panel/M-TRP SDM scheme with 2 TBs/CWs: two TBs/CWs are transmitted in a PUSCH. Different TB/CW is associated with different beam/panel and mapped to different layer.

For the above S-DCI M-TRP Multi-panel FDM, SFN, SDM PUSCH transmission schemes, per TRP beam indication can be supported with extension of unified TCI framework. Extension of unified TCI framework is discussed in our companion paper [3]. Furthermore, for per TRP indication of SRI/TPMI/TPC/PTRS-DMRS association, Rel-17 M-TRP PUSCH can be taken as baseline. In addition, for S-DCI M-TRP FDM PUSCH repetition, frequency domain resource allocation of two repetitions needs to be further studied. For S-DCI M-TRP SDM PUSCH transmission, mapping between layers and TRPs needs to be further studied.

2.2 Multi-panel PUSCH transmission in M-DCI M-TRP

With M-DCI M-TRP operation, simultaneous multi-panel transmission can be beneficial for throughput improvement of PUSCH.

For M-DCI M-TRP operation, in Rel-16, out-of-order scheduling of PDSCH and PUSCH is supported when two PDCCHs that schedule two PDSCHs/PUSCHs are associated with different CORESETPoolID. And In Rel-16, two PDCCHs with different CORESETPoolID scheduling full/partially overlapped PDSCHs is supported. With simultaneous multi-panel UL Tx, two PDCCHs with different CORESETPoolID scheduling full/partially overlapped PUSCHs can be supported.

Proposal 2.3:

- For M-DCI M-TRP, support two PDCCHs with different CORESETPoolID scheduling full/partially overlapped PUSCHs.

- Discussion on multi-panel PUCCH transmission

For PUCCH reliability, in Rel-17, TDM M-TRP PUCCH repetition was supported. In addition to Rel-17 M-TRP PUCCH repetition, with simultaneous multi-panel transmission, following schemes can be considered for reliability enhancement of PUCCH, which is similar as PUSCH.

- Multi-panel/M-TRP FDM PUCCH repetition: two PUCCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively,

- Multi-panel/M-TRP SFN PUCCH repetition: all REs of a PUCCH are associated with two beams/panels

Compared to TDM PUCCH repetition, FDM/SFN repetition can be beneficial for latency reduction. For M-TRP Multi-panel FDM, SFN PUCCH repetition schemes, per TRP beam indication can be supported with extension of unified TCI framework. Extension of unified TCI framework is discussed in our companion paper [3]. In addition to per TRP beam indication, for M-TRP FDM PUCCH repetition, frequency domain resource allocation of two repetitions needs to be considered. In Rel-17, frequency domain resources are configured for each PUCCH resource. To support M-TRP FDM PUCCH repetition, configuring two sets of frequency domain resources for a PUCCH resource or indicating two PUCCH resources for a PUCCH transmission needs to be considered, which may bring more spec. impact. Thus, from our perspective, FDM PUCCH repetition has lower priority than SFN PUCCH repetition.

Figure1a. FDM repetition                       Figure1b. SFN repetition

Proposal 3.1:

- For reliability improvement, consider following schemes of simultaneous multi-panel PUCCH transmission.

  - Multi-panel/M-TRP FDM PUCCH repetition (lower priority): two PUCCH repetitions are transmitted in non-overlapping frequency domain resources and associated with two beams/panels, respectively.

  - Multi-panel/M-TRP SFN PUCCH repetition: all REs of a PUCCH are associated with two beams/panels.

In addition to PUCCH repetition, simultaneous multi-panel transmission can be considered for transmission of two overlapped PUCCHs. In Rel-17, when two PUCCH transmissions overlap, UE resolves the overlapping by multiplexing the UCIs in one PUCCH or dropping one of the PUCCHs following certain rules. With simultaneous multi-panel Tx, allowing transmission of two overlapped PUCCHs associated different panel/TRP can be considered. To support transmission of two overlapped PUCCHs associated with different panel/TRP, association between PUCCH and panel/TRP needs to be studied, and new rules of UCI multiplexing/dropping need to be studied.

Proposal 3.2:

- For simultaneous multi-panel PUCCH transmission, consider transmission of two overlapped PUCCHs associated different panel/TRP.

- Conclusion

In this contribution, we discussed enhancements on simultaneous multi-panel transmission. Based on the discussion, we made following proposals.

Xxx

Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94e, Dec. 2021

[2] 3GPP R1-2204373, “Discussion on 8TX UL transmission”, NTT DOCOMO, INC., RAN1#109e, May 2022

[3] 3GPP R1-2204637, “Discussion on unified TCI framework extension for multi-TRP”, NTT DOCOMO, INC., RAN1#109e, May 2022

Appendix

Table I: LLS assumptions for TDM/FDM/SFN PUSCH repetition

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 28 GHz |
| SCS | 30 kHz |
| Bandwidth | 40 MHz (106 RBs) |
| Waveform | CP-OFDM |
| PUSCH allocation | 31 RBs |
|  | 4 OS |
| Channel Model | TDL-C, DS 100 ns |
| UE velocity | 3 km/h |
| gNB antenna | 4 Rx |
| UE antenna | 2 Tx |
| UE rank | 1 |
| DMRS configuration | Type 1 Single symbol |
| Precoding | SVD granularity: 1 RB |
| MCS | Modulation: QPSK Code rate: 120/1024 |
| RV | RV=0 for no repetition RV =[0, 2] with soft combining for repetition |
| Channel estimation | Realistic (2D-MMSE) |
| Receiver | MMSE |
| EVM | no EVM |

Table II: SLS assumptions for rank probability of single panel Tx

| Parameter | Values |
| --- | --- |
| Carrier frequency | 30GHz |
| Bandwidth | 80MHz |
| SCS | 120KHz |
| Network layout | Dense urban, 1 layer macro |
| Channel model | ITU Channel model B |
| BS antenna structure and TXRU | 2 x 2: 64Tx/Rx = (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 2TXRU=(Mp,Np,P,Mg,Ng) =(1,1,2,1,1) 4 x 4: 64Tx/Rx = (M,N,P,Mg,Ng) = (4,8,2,1,1), (dH,dV) = (0.5, 0.5)λ. TXRU: 4TXRU=(Mp,Np,P,Mg,Ng) =(1,2,2,1,1) (Panel selection is used) |
| UE antenna structure and TXRU | 2 x 2: 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ TXRU: 4TXRU=(Mp,Np,P,Mg,Ng)=(1,1,2,1,2) 4 x 4: 32Tx/Rx = (M,N,P,Mg,Ng) = (2,4 ,2,1,2), (dH,dV) = (0.5, 0.5)λ TXRU: 8TXRU=(Mp,Np,P,Mg,Ng)=(1,2,2,1,2) (Panel selection is used) |
| Number of UE TXRU | Beam sweeping: 4 (1 TXRU per panel per polarization), pannel selection is used then only 2 TxRU for Tx. |
| CSI-T | DFT based analog beam sweeping + CB |
| Channel estimation | real |
| Scheduling | Subband PF |
| MIMO receiver (CSI/data) | BS side analog beam selection + MMSE-IRC |
| Traffic model | Full buffer |
| ISD | 200m |
| Number of average UEs per macro sector | 10 |
| UE power | 23dBm |
| UE mobility | 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UL power control | Open Loop TPC |
| Modulation | 256QAM |
| TDD frame structure | DSUUD |

Table III: SLS assumptions for multi-panel Tx

| Parameter | Values |
| --- | --- |
| Carrier frequency | 30GHz |
| Bandwidth | 80 MHz for UL |
| SCS | 120KHz |
| Network layout | Dense urban, 1 layer macro |
| Channel model | 38.900 Channel model B |
| BS antenna structure and TXRU | 256Tx/Rx = (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ. (dgH,dgV) = (4.0, 2.0)λ TXRU: 8TXRU=(Mp,Np,P,Mg,Ng) =(1,1,2,2,2) |
| UE antenna structure and TXRU | 16Tx/Rx = (M,N,P,Mg,Ng) = (1,4,2,1,2), (dH,dV) = (0.5, 0.5)λ TXRU: 4TXRU=(Mp,Np,P,Mg,Ng)=(1,1,2,1,2) |
| Number of UE TXRU | Beam sweeping: 4 (1 TXRU per panel per polarization), pannel selection is used then only 2 TxRU for Tx. |
| CSI-T | DFT based analog beam sweeping + non CB |
| Number of layers | 2 |
| Modulation | 64QAM without EVM, support NR Real-channel estimation 64QAM |
| Scheduling | Subband PF |
| Azimuth beam width | 11deg |
| Elevation beam width | 11deg |
| Azimuth beam steering range | ±45deg |
| Elevation beam steering range | ±45deg |
| MIMO receiver (CSI/data) | BS side analog beam selection + MMSE-IRC |
| Traffic model | Full buffer, FTP model 1, FTP model 3 |
| BS receiver noise figure | 5 dB |
| Number of average UEs per macro sector | 10 |
| UE power | 23dBm |
| UE mobility | 100% outdoor (30 km/h) |
| UL power control | Open Loop TPC |
| UE distribution | Users randomly and uniformly dropped within the cluster. |
| UT attachment | Based on RSRP (formula (8.1-1) in TR36.873) from port 0 The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| TDD config | DSUUD |
| Layer mapping | NR method |