3GPP TSG RAN WG1 #109-e			R1-2204373

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on 8TX UL transmission

Agenda Item:	9.1.4.2

Document for: 	Discussion and Decision

- Introduction

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. One from the seven objectives is to study 8 TX UL operation.

In this contribution, we discuss the performance evaluations of 8TX UL transmission, and potential enhancements to support 8TX UL transmission.

- Performance evaluation for 8TX UL transmission

As justified in Rel-18 MIMO WID [1], advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) become more relevant, introducing necessary enhancements to support for 8 antenna ports as well as 4 and more layers for UL transmission can offer the needed improvement for UL coverage and average throughput.

To observe the performance gain of 8TX UL operation over 4TX UL operation, system-level-simulation (SLS) is performed. Since high SINR is required for 8TX UL operation, 8TX UL operation mainly has benefits in certain scenarios, such as indoor scenario or dense urban scenario. The simulation parameters are shown in appendix.

In the simulations, 4 cases are evaluated, including 4Tx-rank4, 8Tx-rank4, 8Tx-rank6, and 8Tx-rank8, where ‘m’Tx-rank‘n’ means ‘m’ Tx antenna is used with max rank limitation of ‘n’. For each case, 4 different assumptions are used.

- SVD, P=23: In this assumption, non-codebook based PUSCH transmission is assumed, and UE power is 23dBm.

- SVD, P=26: In this assumption, non-codebook based PUSCH transmission is assumed, and UE power is 26dBm considering advanced UEs could have higher power.

- NR Codebook, P=26: In this assumption, codebook based PUSCH transmission is assumed, and NR DL Type I single-panel codebook (N1 x N2=4 x 1) is reused for UL 8TX codebook.

- LTE Codebook, P=26: In this assumption, codebook based PUSCH transmission is assumed, and LTE DL 8TX codebook is reused for UL.

Fig. 2-1 (a), (b), (c) show the throughput performance in dense urban scenario for average UE, 95%-ile UE, and 5%-ile UE, respectively. Note that for baseline of 4Tx-rank4, only the upbound performance of SVD assuming ideal CSI is provided. In Fig. 2-1, it is obvious that 8TX UL transmission with either codebook based or non-codebook based PUSCH transmission provides significant performance gain over 4TX UL transmission in terms of average UE, 95%-ile UE, and 5%-ile UE throughput. Taking 8Tx-rank8 as an example, the performance gain over 4Tx-rank4 is shown in Table I.

- Average UE performance

- 95%-ile UE performance

- 5%-ile UE performance

Fig. 2-1: Throughput performance of 8TX UL transmission in dense urban scenario

Table I: Performance gain of 8Tx-rank8 over 4Tx-rank4

| 8Tx-rank8 over 4Tx-rank4 | Average UE performance gain | 95%-ile UE performance gain | 5%-ile UE performance gain |
| --- | --- | --- | --- |
| SVD, P=23 | 33.1% | 43.9% | 4.8% |
| SVD, P=26 | 34.0% | 42.1% | 13.1% |
| NR Codebook, P=26 | 24.9% | 35.3% | 18.7% |
| LTE Codebook, P=26 | 22.0% | 26.7% | 16.6% |

Observation 1

- 8TX UL transmission with either codebook based or non-codebook based PUSCH transmission provides significant performance gain in terms of average UE (+22.0%~34.0%), 95%-ile UE (+26.7%~43.9%) and 5%-ile UE (+4.8%~18.7%) throughput.

Comparing the 4 cases with different assumptions, we have some further observations.

- 8Tx-rank4 vs. 8Tx-rank6 vs. 8Tx-rank8: 8Tx-rank6 and 8Tx-rank8 achieve the highest average UE and 95%-ile UE throughput and the gap between the two cases is small.

- P=23 vs. P=26: The increased max. UE power largely improves 5%-ile UE throughput but has small impact on average UE and 95%-ile UE throughput. The reason is that due to UL TPC, cell centre UEs will not achieve max. power, and only cell edge UEs could achieve max. power.

- SVD vs. NR/LTE codebook: Non-codebook based PUSCH transmission has higher performance than codebook-based PUSCH transmission due to ideal CSI acquisition. But the performance gap between SVD and NR codebook is not so big due to high quantization granularity of NR codebook.

- NR codebook vs. LTE codebook: NR codebook provides higher performance than LTE codebook. But even with LTE codebook, the performance gain of 8Tx over 4Tx SVD is still significant. DL 8Tx Type I single-panel codebook has higher quantization granularity than LTE DL 8Tx codebook, hence, the performance is better. But the cost is that larger DCI indication overhead is required to indicate TPMI for UL transmission.

Observation 2

- Non-codebook based PUSCH transmission has higher performance than codebook-based PUSCH transmission for 8TX UL transmission. The performance gain between non-codebook based transmission and NR codebook is not so big.

- NR codebook provides higher performance than LTE codebook for 8TX UL transmission. 8TX with LTE codebook still has significant performance gain over 4TX UL transmission.

For the 4 cases, the rank distribution is provided in Fig 2-2. For 4Tx-rank4 in dense urban scenario, it is observed that there is up to ~70% probability for 4-layer scheduling. For 8Tx-rank4, even the max. rank is kept as 4Tx-rank4, the scheduling probability for rank4 is increased by ~20%. For 8Tx-rank6, the scheduling probability for rank6 is still high, and larger than 50% for non-codebook based transmission and codebook-based transmission with NR codebook. For 8Tx-rank8, the scheduling probability for larger than 4 layer exceeds 50% and the scheduling probability from 4-layer to 8-layer is high. Even for 8-layer transmission, the scheduling probability is higher than 10%. It can also be observed that the probability for rank smaller than 4 is smaller for all 8Tx cases, which implies that a UE with 8Tx will suffer from the lack of ranks less frequently compared to a UE with 4Tx. This also explains why even the cell edge UE throughput can be improved with support of 8Tx.

Fig. 2-2: Rank distribution of 4 cases with different assumptions in dense urban scenario

Observation 3

- 8TX UL transmission has high scheduling probability (>50%) for larger than 4 layers. For 8-layer transmission in 8Tx-rank8, the scheduling probability is higher than 10%.

- 8TX UL transmission has lower scheduling probability for smaller than 4 layers than that of 4TX UL transmission.

Based on the simulation results above, 8TX UL with either codebook-based or non-codebook based PUSCH transmission could provide significant throughput gain. Hence, it is beneficial to support 8TX UL transmission with up to 8 layers. And both codebook-based and non-codebook based PUSCH transmissions can be considered.

Proposal 1

- Support 8TX UL transmission with up to 8 layers per UE.

- Discussion on 8TX UL transmission

3.1 UE 8TX layout

For 8TX UEs, there could be different types of UE antenna layout for different scenarios (e.g. CPE, FWA, vehicle, industrial devices), including 4 x 1, 2 x 2, or other possible 8 antenna layout by implementation in commercial NW, as shown in Fig. 3-1. From specification perspective, the Rel-18 design should be able to applicable to any 8TX UE antenna layout. On the other hand, it is also important to identify the main UE antenna layout in commercial NW so that the Rel-18 design could fit the high priority scenarios better, based on UE capability reporting of 8TX UE antenna layout.

Fig 3-1: 8TX UE antenna layout

Proposal 2

- Discuss 8TX UE antenna layout and identify the important UE antenna layout in commercial NW.

- Support UE capability reporting of 8TX UE antenna layout.

- Rel-18 design should be able to applicable to any 8TX UE antenna layout.

3.2 PUSCH for >4 layers

In Rel-17, for PUSCH transmission, one codeword with up to 4 layers is supported and layer mapping is defined for one codeword. In Rel-18, for PUSCH transmission with up to 8 layers, two codewords can be introduced for more than 4 layers, with up to 4 layers per codeword. In that case, codeword-to-layer mapping for more than 4 layers for spatial multiplexing should be introduced for PUSCH. In Rel-17, the codeword-to-layer mapping for more than 4 layers for spatial multiplexing has been defined for PDSCH, which could be extended to PUSCH easily.

When two codewords are transmitted for a PUSCH for more than 4 layers, codeword-specific indications of MCS, NDI, and RV in DCI should be supported, like two codewords transmission for a PDSCH. In Rel-17, UCI can be multiplexed on PUSCH in some conditions. If two codewords are transmitted for a PUSCH, some enhancements on UCI multiplexing for two codewords PUSCH should be studied.

Above mainly discusses the potential enhancements on two codewords PUSCH for more than 4 layers. For simultaneous multi-panel UL transmission for a multi-panel UE, it is also beneficial to consider two codewords PUSCH transmission from two panels, for less than 4 layers [2]. In that case, the design on support of two codewords PUSCH is common and can be discussed jointly for the two features of 8TX UL transmission and multi-panel UE transmission.

Proposal 3

- Support two codewords for PUSCH transmission for more than 4 layers. Following enhancements can be further discussed.

  - codeword-to-layer mapping for more than 4 layers for spatial multiplexing

  - DCI enhancement with codeword-specific indications of MCS, NDI, and RV

  - UCI multiplexing on two codewords PUSCH

- Support two codewords for simultaneous multi-panel UL transmission for less than 4 layers. The design on support of two codewords PUSCH is common and discussed jointly for 8TX UL transmission and multi-panel UE transmission.

In existing spec., antenna port indication tables for PUSCH are defined for UL rank from 1 to 4. For PUSCH transmission with larger than 4 layers, each UE can be indicated with up to 8 DMRS antenna ports for a PUSCH transmission. Hence, new antenna port indication tables should be introduced for UL rank from 5 to 8. In addition, enhanced PTRS-DMRS association for more than 4 scheduled DMRS ports should be also discussed.

Proposal 4

- Support antenna port indication enhancement for UL rank from 5 to 8.

- Support enhanced PTRS-DMRS association for more than 4 scheduled DMRS ports.

3.3 Codebook based PUSCH

For PUSCH transmission with up to 8 layers, both codebook-based and non-codebook-based PUSCH transmissions should be discussed. But codebook-based PUSCH transmission has higher priority for us considering our NW operation.

For codebook-based PUSCH transmission, the UL 8TX codebook design is essential. In existing spec., three types of coherence are considered for UE, including non-coherent UE, partial-coherent UE, and full-coherent UE. And existing 2/4TX codebook design considers above UE coherence assumptions and defines different coherent codebook subsets. For UL 8TX codebook, similarly, three types of UE coherence assumptions should be considered, and different coherent codebook subsets should be introduced. Hence, UL precoders for non-coherent, partial-coherent, and full-coherent UEs for 1 to 8 layers should be defined. In addition, as discussed above, there could be different types of UE antenna layout for different scenarios, so that the 8TX UL codebook can be applicable to any 8TX UE antenna layout.

For non-coherent type, 1-port port-selection precoders can be considered. For partial-coherent type, the precoders are related to UE coherent assumption, e.g., if a 8TX UE has non-coherency between two groups of 4 ports but full-coherency within each group, the partial-coherent precoders can consider full-coherent transmission within a 4-port group. In that case, UE capability reporting of coherent ports for partial-coherent UE is beneficial for partial-coherent precoder design. For full-coherent precoders, existing NR DL Type I single-panel codebook can be a starting point. The simulation results in Section 2 have shown significant performance gain by just reusing NR DL Type I single-panel codebook. But there are many issues to be further discussed on how to reuse this DL codebook for UL. For example, both 4 x 1 and 2 x 2 antenna layout are supported in DL as well as mode 1 and mode 2, whether/how to support them in UL should be studied. In addition, with current oversampling factors, the number of precoders could be large, which requires large DCI indication overhead. Based on simulation results in Section 2, it is observed that after reducing the codebook granularity, the performance gain of 8TX over 4TX is also large. Hence, further study on how to reduce the codebook granularity and DCI indication overhead is needed.

In Rel-16, UL full power transmission modes are introduced to enable full power transmission of non-coherent and partial-coherent codebook subsets. Similarly for 8TX UL codebook, full power transmission enhancements should be also considered for non-coherent and partial-coherent codebook subsets.

Proposal 5

- Support codebook-based PUSCH transmission for more than 4 layers.

- Support to define new UL 8TX codebook, which should consider non-coherent, partial-coherent, and full-coherent UEs, and applicable to any 8TX UE antenna layout.

  - Support UE capability reporting of coherent ports for partial-coherent UE.

  - Support NR DL Type 1 single-panel codebook as starting point for UL full-coherent precoders.

  - The number of supported non-coherent, partial-coherent, and full-coherent precoders should be carefully considered to reduce the DCI indication overhead.

- Support full power transmission enhancements for non-coherent and partial-coherent precoders.

For codebook-based PUSCH transmission, in existing spec., multiple multi-port SRS resources can be transmitted with usage of ‘codebook’, and NW indicates a selected SRS resource in UL grant. For PUSCH transmission with up to 8 layers, 8-port SRS resource with usage of ‘codebook’ is needed. And the detailed enhancement is discussed in our company contribution [3].

Proposal 6

- Support 8-port SRS resource with usage of ‘codebook’.

3.4 Non-codebook based PUSCH

For non-codebook-based PUSCH transmission, in existing spec., multiple 1-port SRS resources can be transmitted with usage of ‘noncodebook’, and NW indicates the selected SRS resource(s) in DCI to indicate the UL rank and selected beam(s) for the scheduled PUSCH. For PUSCH transmission with up to 8 layers, similarly, eight 1-port SRS resources in an SRS resource set can be transmitted with usage of ‘noncodebook’, and NW indicates the selected SRS resource(s). In that case, new tables of SRI indication for non-codebook based PUSCH for 8 SRS resources should be defined for maximum UL rank from 1 to 8. And the SRI field bit size in DCI is increased.

Proposal 7

- Support non-codebook-based PUSCH transmission for more than 4 layers.

- Support configuration of eight 1-port SRS resources in an SRS resource set with usage of ‘noncodebook’.

- Support SRI indication enhancement for non-codebook-based PUSCH for 8 SRS resources configured with usage of ‘noncodebook’ for maximum UL rank from 1 to 8.

3.5 SRS for antenna switching

For UL transmission, SRS can be configured with usage of ‘antenna switching’ for DL CSI acquisition when the number of TX ports at UE is equal to or smaller than the number of RX ports. When 8TX UE is introduced, SRS transmission and antenna switching for 8T8R can be considered, as discussed in our company contribution [3].

Proposal 8

- Support to discuss SRS enhancement for antenna switching for 8T8R.

- Conclusion

In this contribution, we discussed the performance evaluations of 8TX UL transmission, and potential enhancements to support 8TX UL transmission. We have following observations and proposals.

Observation 1

- 8TX UL transmission with either codebook based or non-codebook based PUSCH transmission provides significant performance gain in terms of average UE (+22.0%~34.0%), 95%-ile UE (+26.7%~43.9%) and 5%-ile UE (+4.8%~18.7%) throughput.

Observation 2

- Non-codebook based PUSCH transmission has higher performance than codebook-based PUSCH transmission for 8TX UL transmission. The performance gain between non-codebook based transmission and NR codebook is not so big.

- NR codebook provides higher performance than LTE codebook for 8TX UL transmission. 8TX with LTE codebook still has significant performance gain over 4TX UL transmission.

Observation 3

- 8TX UL transmission has high scheduling probability (>50%) for larger than 4 layers. For 8-layer transmission in 8Tx-rank8, the scheduling probability is higher than 10%.

- 8TX UL transmission has lower scheduling probability for smaller than 4 layers than that of 4TX UL transmission.

Proposal 1

- Support 8TX UL transmission with up to 8 layers per UE.

Proposal 2

- Discuss 8TX UE antenna layout and identify the important UE antenna layout in commercial NW.

- Support UE capability reporting of 8TX UE antenna layout.

- Rel-18 design should be able to applicable to any 8TX UE antenna layout.

Proposal 3

- Support two codewords for PUSCH transmission for more than 4 layers. Following enhancements can be further discussed.

  - codeword-to-layer mapping for more than 4 layers for spatial multiplexing

  - DCI enhancement with codeword-specific indications of MCS, NDI, and RV

  - UCI multiplexing on two codewords PUSCH

- Support two codewords for simultaneous multi-panel UL transmission for less than 4 layers. The design on support of two codewords PUSCH is common and discussed jointly for 8TX UL transmission and multi-panel UE transmission.

Proposal 4

- Support antenna port indication enhancement for UL rank from 5 to 8.

- Support enhanced PTRS-DMRS association for more than 4 scheduled DMRS ports.

Proposal 5

- Support codebook-based PUSCH transmission for more than 4 layers.

- Support to define new UL 8TX codebook, which should consider non-coherent, partial-coherent, and full-coherent UEs, and applicable to any 8TX UE antenna layout.

  - Support UE capability reporting of coherent ports for partial-coherent UE.

  - Support NR DL Type 1 single-panel codebook as starting point for UL full-coherent precoders.

  - The number of supported non-coherent, partial-coherent, and full-coherent precoders should be carefully considered to reduce the DCI indication overhead.

- Support full power transmission enhancements for non-coherent and partial-coherent precoders.

Proposal 6

- Support 8-port SRS resource with usage of ‘codebook’.

Proposal 7

- Support non-codebook-based PUSCH transmission for more than 4 layers.

- Support configuration of eight 1-port SRS resources in an SRS resource set with usage of ‘noncodebook’.

- Support SRI indication enhancement for non-codebook-based PUSCH for 8 SRS resources configured with usage of ‘noncodebook’ for maximum UL rank from 1 to 8.

Proposal 8

- Support to discuss SRS enhancement for antenna switching for 8T8R.

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

- 3GPP R1-2204372, “Discussion on multi-panel transmission”, RAN1#109-e, May 2022.

- 3GPP, R1-2204371, “Discussion on SRS enhancement”, RAN1#109-e, May 2022.

Appendix

Table A-1: Simulation parameters

| Scenario | Parameters |
| --- | --- |
| Carrier frequency | 4GHz |
| Bandwidth | 20MHz |
| SCS | 15KHz |
| Network layout | Dense urban, 1 layer macro |
| Channel model | ITU Channel model B |
| BS antenna structure and TXRU | 128Tx/Rx = (M,N,P,Mg,Ng) = (8,8,2,1,1), (dH,dV) = (0.5, 0.8)λ. TXRU: 32TXRU=(Mp,Np,P,Mg,Ng) =(2,8,2,1,1) |
| UE antenna structure and TXRU | 4Tx/Rx = (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, 0.5)λ TXRU: 4TXRU=(Mp,Np,P,Mg,Ng)=(1,2,2,1,1) 8Tx/Rx = (M,N,P,Mg,Ng) = (1,4,2,1,1) TXRU: 8TXRU=(Mp,Np,P,Mg,Ng)=(1,4,2,1,1) |
| Number of UE TXRU | 4 / 8 |
| CSI-T | One analog beam at both BS and UE side + SVD or codebook-based precoding |
| Channel estimation | Real |
| Scheduling | Subband PF |
| MIMO receiver | MMSE-IRC |
| Traffic model | Full buffer |
| ISD | 200 |
| Number of average UEs per macro sector | 10 |
| Subband number | 20 |
| Max. UE power | 23dBm, 26dBm |
| UE mobility | 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UL power control | Open Loop TPC |
| Modulation | Up to 64QAM |
| Layer mapping | NR |
| TDD frame structure | DSUUD |