3GPP TSG RAN WG1 #109-e			R1-2204376

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on other aspects on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for: 	Discussion

- Introduction

At the RAN1#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of potential specification impacts as following

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements.

- Discussion on sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements

  - Potential sub use-cases

SID on AI/ML for NR air interface (NRAI) lists the examples of CSI feedback enhancement use-cases: overhead reduction, improved accuracy, and prediction.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

AI/ML has attained a lot of attention in reducing overhead with high accuracy in many research fields. In CSI feedback, this AI/ML technique can be integrated as well. One of the promising sub-use cases for these purposes is AI/ML-based compression and reconstruction of CSI feedback, known as auto-encoders of CSI feedback.

Fig. 1 illustrates the framework of auto-encoders of CSI feedback. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In auto-encoders of CSI feedback, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder.

Figure 1. The framework of auto-encoders of CSI feedback.

With this AI/ML-based compression technique, accuracy improvements under a certain overhead of CSI reports and overhead reduction for CSI reports achieving a certain performance can be expected [2]. Hence, Auto-encoders of CSI feedback should be studied as a sub use-case of CSI feedback enhancements in Rel-18 AI/ML for NRAI.

Proposal 1: Auto-encoders of CSI feedback should be studied as a sub use-case of CSI feedback enhancements in Rel-18 AI/ML for NRAI.

  - Potential specification impacts

If auto-encoder of CSI feedback is supported, UE is expected to compress the CSI and report encoded bits as UCI. There are several CSI candidates as the inputs and outputs for auto-encoder, e.g., channel matrix information or precoding matrix information. The related information PMI is already specified in the specification. However, PMI is not qualified to be the inputs and outputs of auto-encoder for CSI feedback, because PMI was designed to reduce the overhead of CSI report and some channel information is lost to represent PMI with bit sequences. Therefore, it is necessary to study the format of inputs and outputs of CSI for auto-encoder and whether the format of inputs and outputs for auto-encoders needs to be specified in the specification or not.

Observation 1: The format of inputs and outputs for auto-encoders needs to be studied.

Also, the mechanisms to report encoded bits as UCI needs to be specified in the specification. In the current specification, some CSI is reported only on PUSCH according to the codebook type and band granularity. The similar discussion is necessary for reporting encoded bits as well. On top of it, when UE compiles multiple AI/ML models achieving different compression ratios, static/semi-static/dynamic ML model selection according to channel quality can bring the gain. Furthermore, it could be beneficial to switch applying AI/ML approach or conventional CSI reports according to the performance of AI/ML models. In that case, the determination mechanisms on what UCI bits UE shall report, as depicted in Fig.2, needs to be defined too.

Figure 2. Determination mechanisms on what UCI bits UE shall report.

Observation 2: The mechanisms in CSI reports, such as determination mechanisms on reported UCI bits and how to report encoded bits, could be specification impacts.

In addition, the auto-encoder requires coordinated ML models on UE and gNB. Depending on the entity training AI/ML models, AI/ML models are required to be transferred to deploy on UE and gNB. Fig. 3 illustrates three scenarios: model training on NW, model training on UE, and model training on both UE and NW. If the signalling of trained AI/ML model information is specified to some extent in 3GPP, it could be also potential specification impacts for this sub-use case.

Observation 3: Methods to deploy coordinated AI/ML models on UE and gNB are potential specification impacts for auto-encoder of CSI feedback.

Figure 3. Model deployment from the model training entity, (a) model training on NW, (b) model training on UE, and (c) model training on UE and NW.

- Conclusion

In this contribution, we discussed the sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements. Based on the discussion we made following proposals.

Proposal 1: Auto-encoders of CSI feedback should be studied as a sub use-case of CSI feedback enhancements in Rel-18 AI/ML for NRAI.

Observation 1: The format of inputs and outputs for auto-encoders needs to be studied.

Observation 2: The mechanisms in CSI reports, such as determination mechanisms on reported UCI bits and how report encoded bits, could be specification impacts.

Observation 3: Methods to deploy coordinated AI/ML models on UE and gNB are potential specification impacts.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- NTT DOCOMO, R1-2204375, “Discussion on evaluation on AI/ML for CSI feedback enhancement”, Apr. 2021.