3GPP TSG RAN WG1 #109-e			R1-2204379

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for: 	Discussion

- Introduction

At RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss on the evaluation of NR duplex evolution.

- Scenario and frequency configuration

NR duplex evolution is beneficial for flexible UL/DL resource configuration, and it can be applied for various scenarios as shown in Fig.1. For example, one of the typical use cases of duplex evolution is for UL heavy traffic. With increasing UL time resource using UL subband, higher UL data transmission rate is expected at UL heavy traffic area, e.g. Indoor or Urban even when TDD UL/DL configuration is restricted due to alignment with neighbor cells/carriers to avoid CLI. Another typical use case is for enhancement of coverage performance. With increased UL time resource available for repetitions, enhancement of coverage performance is expected at cell edge of e.g. sub-urban, and Rural areas. Therefore, typical scenarios of Indoor, urban, sub-urban, Rural can be considered for the evaluation and study. To reduce the workload for evaluation, the scenarios for evaluation can be down-selected for each frequency range, e.g. Rural and Urban for FR1, and Indoor and Urban for FR2.

Proposal 1: Rural and Urban scenarios for FR1, and Indoor and Urban scenarios for FR2 is considered for evaluation.

As described above, typical use cases for NR duplex evolution are UL heavy traffic and enhancement of coverage performance. Therefore, for UL heavy traffic, eMBB with wider frequency bandwidth for UL subband with higher MCS can be assumed, and for coverage performance, VoIP with narrower frequency bandwidth for UL subband with lower MCS can be assumed as a baseline of the evaluation.

Proposal 2: Evaluation assumptions is derived by both UL heavy traffic scenario (e.g. eMBB), and coverage enhancement scenario (e.g. VoIP).

Figure 1. Example scenarios of subband non-overlapping full duplex

Possible frequency allocation for subband non-overlapping full duplex is shown in Fig.2. Frequency allocation with subband at middle PRBs (1-2/2-2) should be prioritized for the study and evaluation, since PRBs at higher and lower edge of frequency band can be considered as guard bands for adjacent carriers e.g., in inter-operator scenarios. Comparing UL subband (1-2) and DL subband (2-2), UL subband (1-2) should be prioritized, since increasing UL time resource is more beneficial considering widely used DL heavy TDD patterns.

Proposal 3: Frequency allocation of UL subband at middle PRBs is prioritized for the study and evaluation.

Figure 2. Possible frequency allocation for subband non-overlapping full duplex

- Evaluation assumptions for LLS

Evaluation of link level performance is necessary to study on the bandwidth of subband and the bandwidth of guard band for DL and UL subbands. Therefore, evaluation assumptions for link level simulation should be discussed and defined. For the LLS, following additional parameters need to be considered as shown in Fig.3. Especially, power difference between evaluation channel and interference channel is one of key parameters, and hence how to derive the power difference is important.

- Power difference between evaluation channel/subband and interference channel/subband

  - As shown in Fig.3 for UL channel evaluation, the PSD difference between UL channel transmitted by UE and DL interference signals transmitted by gNBs at gNB reception point needs to be studied and defined. For the DL interference signals, both self-interference (intra-gNB interference) and CLI interference (inter-gNB interference) need to be considered.

- Channel bandwidth of interference channels/subbands

  - The bandwidth of interference channels/subbands may affect the interference level at adjacent victim channels/subbands. For example, if the bandwidth of interference channel/subband is narrower, the interference level at the adjacent victim channel/subband would be smaller given fixed guard band since the power level of side-lobe increases with the bandwidth.

- Guard band between victim channel/subband and interference channel/subband

Once the appropriate power difference and channel bandwidth of interference channels/subbands are defined, we may perform LLS with the defined parameters to study necessity of guard band for the configuration.

Proposal 4: Evaluate link level performance with LLS for study on the bandwidth of subband and the bandwidth of guard band for DL and UL subbands

- Parameters of “power difference”, “bandwidth of interference channels/subbands”, and “bandwidth of guard band” need to be studied and defined for the evaluation

Figure 3. Example of potential simulation assumptions for LLS.

For the LLS and SLS, modeling of emissions of interference signal is one of essential points of the study, since guard band between DL and UL resource for subband non-overlapping full duplex is much narrower than that for FDD bands, and whether analog filter can be used or not is unclear. Therefore, we need to study what kind of components can be used to suppress the emissions, and how to model the emissions of interference signals at gNB and UE, respectively. As an initial evaluation, we use Welch filtering for time domain signal for emissions as shown in Fig.4.

Proposal 5: Study and define modeling of emissions of interference signal at gNB and UE, respectively for LLS and SLS evaluations.

Figure 4. Example of emissions of interference signal.

- Initial LLS evaluation results

As an initial LLS evaluation, PUSCH performance is evaluated while changing PSD difference between PUSCH channel and interference channels for both eMBB and VoIP scenarios, and Fig.5 shows the evaluation results. Simulation assumptions are listed in Table I in Appendex, and ones of the main difference between eMBB and VoIP scnarios is number of allocated PRBs for PUSCH channel, e.g. 160 PRBs for FR1 eMBB and 4 PRBs for FR1 VoIP, and selected MCS, e.g. MCS index of 8 for eMBB and index 0 for VoIP, respectivly.

As shown in Fig.5 (A) for FR1 eMBB, degradation of 0.1 dB is observed when guard band is 5 PRBs and PSD difference is 30 dB, and no degradation is observed when guard band is 25 PRBs with 10,20, 30 dB PSD diference. As shown in Fig.5 (B) for FR2 eMBB, degradation of 2.7 dB, 0.3 dB are observed for 1 PRB and 6 PRBs guard band with 30 dB PSD difference, respectively. And no remarkable degradation is obeserved when the PSD difference is smalle. Since required SNR for the target BLER is higher than that for FR1, wider guard band or smaller PSD difference is required for the target BLER of FR2. Therefore, PSD difference and guard bandwidth are key parameters for the evaluation and it should be studied. As shown in Fig.5 (C) for FR1 VoIP, degradation of 3.0 dB, 0.6 dB are observed for 5 PRBs and 25 PRBs guard band with 40 dB PSD difference, respectively. Since required SNR for VoIP is 9 dB lower than that for eMBB, small degradation was expected up to 40 dB PSD difference with considering the eMBB results as in Fig.5 (A). On the other hands, we can find larger degradation than we expected. In this evaluaton whole channel bandwidth are occupied by the PUSCH, interference channels, and gurad bands, so that the bandwidth of interference channels are increased while decreasing the bandwidth of PUSCH. Therefore, the larger bandwith of interference signals, which increase the emissions of interference signals, is one of the reasons for the larger degradation, and the bandwidth of interference singals is one of the important factors for the evaluation. As shown in Fig.5 (D) for FR 2 VoIP, degradation of 3.4 dB, 0.3 dB are observed for 1RBs and 6 RBs guard band with 30 dB PSD difference, respectively. As well as FR1 results, although the required SNR for the target BLER is 12.5 dB lower than that for eMBB, we can find the degradation of PUSCH performance. Based on the results, we conclude following observations.

Observations :

- eMBB FR1 : Degradation of 0.1 dB is observed when guard band is 5 RBs and PSD difference is 30 dB, and no degradation is observed  when guard band is 25 RBs with 10, 20, 30 dB PSD difference.

  - PSD difference and guard bandwidth are key parameters for the evaluation and it should be studied and defined

- eMBB FR2 : Degradation of 2.7 dB, 0.3 dB are observed for 1 PRB and 6 PRBs guard band with 30 dB PSD difference, respectively.

  - Since required SNR for the target BLER is higher, wider guard band or small PSD difference is required

- VoIP : Degradation of 3.0 dB, 0.6 dB are observed for 5 PRBs and 25 PRBs guard band with 40 dB PSD difference for FR1.

  - Larger bandwith of interference signals is one of the reasons for the degradation

(A) eMBB for FR1

(B) eMBB for FR2

(C) VoIP for FR1

(D) VoIP for FR2

Figure 5. LLS evaluation results for subband non-overlapping full duplex.

- Conclusion

In this contribution, we discussed on the evaluation of NR duplex operation. Based on the discussion we made the following proposals and observations.

Proposal 1: Rural and Urban scenarios for FR1, and Indoor and Urban scenarios for FR2 is considered for evaluation.

Proposal 2: Evaluation assumptions is derived by both UL heavy traffic scenario (e.g. eMBB), and coverage enhancement scenario (e.g. VoIP).

Proposal 3: Frequency allocation of UL subband at middle PRBs is prioritized for the study and evaluation.

Proposal 4: Evaluate link level performance with LLS for study on the bandwidth of subband and the bandwidth of guard band for DL and UL subbands

- Parameters of “power difference”, “bandwidth of interference channels/subbands”, and “bandwidth of guard band” need to be studied and defined for the evaluation

Proposal 5: Study and define modeling of emissions of interference signal at gNB and UE, respectively for LLS and SLS evaluations.

Observations :

- eMBB FR1 : Degradation of 0.1 dB is observed when guard band is 5 RBs and PSD difference is 30 dB, and no degradation is observed  when guard band is 25 RBs with 10, 20, 30 dB PSD difference.

  - PSD difference and guard bandwidth are key parameters for the evaluation and it should be studied and defined

- eMBB FR2 : Degradation of 2.7 dB, 0.3 dB are observed for 1 PRB and 6 PRBs guard band with 30 dB PSD difference, respectively.

  - Since required SNR for the target BLER is higher, wider guard band or small PSD difference is required

- VoIP : Degradation of 3.0 dB, 0.6 dB are observed for 5 PRBs and 25 PRBs guard band with 40 dB PSD difference for FR1.

  - Larger bandwith of interference signals is one of the reasons for the degradation

References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- 3GPP, TS38.830 v17.0.0, Dec. 2020.

# Appendix

Simulation assumptions for LLS is listed in Table.1. We follow simulation assumptions for evaluation of Rel-17 coverage enhancement [2].

Table I. Simulation assumptions for LLS.

|  | FR1 | FR1 | FR2 | FR2 |
| --- | --- | --- | --- | --- |
| Service | eMBB | VoIP | eMBB | VoIP |
| Channel | OtoI | OtoI | Indoor | Indoor |
| Target data rate | 5 Mbps | - | 5 Mbps | - |
| Target metric | 10% iBLER | 2% rBLER | 10% iBLER | 2% rBLER |
| Center frequency | 4 GHz | 4 GHz | 28 GHz | 28 GHz |
| Frequency bandwidth | 100 MHz | 100 MHz | 100 MHz | 100 MHz |
| Maximum number of PRBs | 270 | 270 | 66 | 66 |
| SCS | 30 kHz | 30 kHz | 120 kHz | 120 kHz |
| LoS/NLoS | NLoS | NLoS | - | - |
| Channel model | TDL-C | TDL-C | TDL-A | TDL-A |
| Delay spread | 300 | 300 | 30 | 30 |
| Mobile speed | 3 km/h | 3 km/h | 3 km/h | 3 km/h |
| Symbol allocation (S, L) | S = 0, L = 14 | S = 0, L = 14 | S = 0, L = 14 | S = 0, L = 14 |
| HARQ | Disabled | Enabled | Disabled | Enabled |
| Number of HARQ process | - | 8 | - | 8 |
| MCS (MCS table) | 8 | 0 | 11 | 0 |
| Channel coding | LDPC | LDPC | LDPC | LDPC |
| Waveform | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM | DFT-S-OFDM |