3GPP TSG RAN WG1 #109-e			R1-2204381

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for: 	Discussion

- Introduction

At RAN#94e meeting, a new SID [1] on “Study on evolution of NR duplex operation” was approved. The detailed objectives are as follows.

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss potential enhancements on dynamic/flexible TDD.

- Potential enhancements on dynamic/flexible TDD

Concerning the interference of duplex enhancement, following three cases are considered as shown in Fig.1.

1. Self-interference at gNB (intra-gNB interference)

Aggressor signal is DL signal transmitted by gNB, and victim signal is UL signal transmitted by UE at gNB reception. The self-interference is handled by gNB configuration / implementation, and how to handle, e.g. using antenna isolation, digital/analog filters and how much the self-interference can be suppressed should be studied and clarified by evaluation.

2. Cross link interference at UE (inter-UE interference)

Aggressor signal is UL signal transmitted by other UE, and victim signal is DL signal transmitted by gNB at UE reception. UE-UE CLI measurement and reporting mechanisms, such as CLI-RSSI and SRS-RSRP measurements were introduced in Rel-16, and it can be a baseline for UE-UE CLI for subband non-overlapping full duplex, and further enhancement can be considered, e.g. introducing spatial domain information, configuring multiple frequency resources for the CLI measurement.

3. Cross link interference at gNB (inter-gNB interference)

Aggressor signal is DL signal transmitted by other gNB, and victim signal is UL signal transmitted by UE at gNB reception. Introducing UL subband into DL resource of TDD is one of the typical use cases for subband non-overlapping full duplex. Therefore, inter-gNB interference is expected for the typical use case, and it is necessary to study the impact of inter-gNB interference. Since the impact would be critical for the duplex operation, CLI handling at gNB can be studied.

Figure 1. Example of protentional interference for duplex enhancement.

Figure 2 shows an example of interference for non-overlapping full duplex. As shown in Fig.2, (2-3) is one of the potential interference scenarios where the frequency of aggressor signals (e.g. DL) and that of victim signals (e.g. UL) do not overlap, especially when DL/UL time and frequency domain resource partition pattern among multiple cells is unified as discussed in our contribution [2]. On the other hands, if the resource partition pattern among multiple cells is not unified, or some of surrounding cells don’t operate with the enhanced duplex, (2-2) and (2-1) are also considered for the CLI handling. For these three cases, since the frequency resource of aggressor signals and that of victim signals are not completely overlapped, RSSI measurement can be a baseline for the CLI measurement.

Observation 1: RSSI measurement is a baseline of CLI measurement for subband non-overlapping full duplex, since the frequency bandwidth of aggressor signals and that of victim signals are not fully overlapped.

Figure 2. Example of interference for non-overlapping full duplex.

- Cross link interference management at UE

UE-UE CLI management is already supported in Rel-16, and CLI measurement window can be flexibly configured to UEs. Therefore, the mechanism of Rel-16 CLI can be a baseline for the interference management for subband non-overlapping full duplex. One of potential enhancements is to introduce spatial domain information for CLI measurement and reporting. As shown in Fig.3, if the direction of antenna beams of aggressor UE1 and that of victim UE0 match, e.g. “beam b” is the best beam and selected for DL reception of UE0, we may expect UE1 UL signal may interfere UE0 DL signal. On the other hands, if UE0 uses “beam c”, although the beam is the 2nd best beam but sufficient DL received power is expected, we may expect that the interference has little impact on UE0 reception. Therefore, interference management with spatial domain can be considered as a potential enhancement of UE-UE CLI.

Another enhancement is to introduce multiple frequency resource configuration for the CLI measurement window. As shown (2-3) in Fig.2, the DL frequency resource can be separated into two DL subbands, so that the CLI measurement can be performed at two DL subbands, respectively. Therefore, configuring multiple frequency resources for the CLI measurement window can be considered. In addition, it is also necessary to study that whether UE has a capability of simultaneous RSSI measurement at two DL subbands or not.

Proposal 1: For the enhancement of UE-UE CLI for subband non-overlapping full duplex, following aspects needs to be studied.

- Introduction of spatial domain information for CLI measurement

- Introduction of multiple frequency resource configurations for CLI measurement

Figure 3. Example of UE-UE CLI.

- Cross link interference management at gNB

Figure 4 shows an example of gNB-gNB CLI, and expected received power of aggressor signal transmitted by gNB1 and victim signal transmitted by UE0 at gNB0 reception are briefly calculated. The Tx power and antenna gain of gNB and UE for Rel-17 coverage enhancement SI [3] are used for the calculation, and line of sight is considered. Because of Tx power and antenna gain difference between gNB and UE, we may expect 15 dB PSD difference between the aggressor and victim signals for FR1, although the distance between gNB0 (reception point) and gNB1 is two times longer than that of gNB0 and UE0. Based on our initial LLS results as in [4], 15 dB PSD difference may not seriously impact the link level performance. On the other hands, we may need to carefully study this aspect, since the LLS results are based on the specific assumption of subband non-overlapping full duplex, e.g., guard band between aggressor and victim signals is configured. As shown in Fig.4 (b) for FR2, we may expect 33 dB PSD difference when antenna beam directions of gNB1 and gNB0 match. This is the worst case for the CLI scenario, on the other hands, we also note that the 30 dB PSD difference may impact the link level performance and require larger guard bands to avoid the interference based on our initial LLS results.

Proposal 2: Study on how much PSD difference is expected at gNB for FR1 and FR2, and if the difference is critical for the duplex operation, CLI handling at gNB is considered.

- example for FR1

- example for FR2

Figure 4. Example of gNB-gNB CLI.

- Conclusion

In this contribution, we discussed potential enhancements on dynamic/flexible TDD. Based on the discussion we made the following proposals and observation.

Observation 1: RSSI measurement is a baseline of CLI measurement for subband non-overlapping full duplex, since the frequency bandwidth of aggressor signals and that of victim signals are not fully overlapped.

Proposal 1: For the enhancement of UE-UE CLI for subband non-overlapping full duplex, following aspects needs to be studied.

- Introduction of spatial domain information for CLI measurement

- Introduction of multiple frequency resource configurations for CLI measurement

Proposal 2: Study on how much PSD difference is expected at gNB for FR1 and FR2, and if the difference is critical for the duplex operation, CLI handling at gNB is considered.

References

- 3GPP, RP-213591, CMCC, “Study on evolution of NR duplex operation,” Dec. 2021.

- R1- R1-2204380, NTT DOCOMO, “Discussion on subband non-overlapping full duplex”, May 2022.

- 3GPP, TS38.830 v17.0.0, Dec. 2020.

- R1- R1-2204379, NTT DOCOMO, “Discussion on evaluation on NR duplex evolution”, May 2022.