3GPP TSG RAN WG1#109-e	 	 	R1-2204386

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on solutions for integrity of RAT dependent positioning techniques

Agenda Item:	9.5.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, revised SID on Rel-18 NR positioning was agreed [1]. The study item includes objectives related to improved positioning accuracy, integrity and power efficiency as follows:

- Improved accuracy, integrity, and power efficiency:

  - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]:

    - Identify the error sources, [RAN1, RAN2].

    - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2]

    - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

  - Study solutions for accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers considering e.g. timing errors, phase coherency, frequency errors, power imbalance, etc [RAN4]:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this contribution, we present our view on solutions for integrity of RAT dependent positioning techniques for Rel-18 NR positioning.

- Error sources of RAT dependent positioning

In Rel-17 NR, RAT independent positioning integrity was introduced. The positioning integrity can provide quantitative reliability information on the reported positioning metrics for location services that require higher reliability. Rel-17 RAT dependent positioning supports many features enabling highly precise location estimation. Given that RAT dependent positioning is expected to be deployed in various use cases including automotive and IIoT as well as RAT independent positioning, RAT dependent positioning integrity which provides quantitative reliability information on the reported positioning metrics is also important feature in NR.

The current RAT dependent positioning supports timing-based methods (e.g. DL/UL-TDOA, Multi-RTT) and angle-based methods (e.g. DL-AoD, UL-AoA), and the positioning accuracy of each method may be impacted by different factors. In case of DL/UL-TDOA, the positioning accuracy may be impacted by TRP synchronization error. For example, 1ns synchronization error equal to 30cm positioning error. On the other hand, positioning accuracy of angle-based methods may be impacted by multi-path environment (e.g. NLOS probability) since the UE’s location is calculated from departure or arrival beam angle information. Therefore, we think that it may be better to consider what kind of error sources for timing-based methods and angle-based methods separately. In RAN1 work, to identify which error source impacts on positioning accuracy performance, evaluation of how much the candidate error sources (e.g. TRP synchronization error) impact on positioning accuracy performance should be considered separately for timing based methods and angle based methods. Based on the evaluation results, we can discuss what RAN1 spec impacts to enable RAT dependent positioning integrity.

Observation 1:

- In order to identify which error source impacts on positioning accuracy performance, evaluation of how much the candidate error sources (e.g. TRP synchronization error) impact on positioning accuracy performance should be considered separately for timing based methods and angle based methods.

- Conclusion

In this contribution, we discussed on solutions for integrity of RAT dependent positioning techniques for Rel-18 NR positioning. Based on the discussion, we made following observation.

Observation 1:

- In order to identify which error source impacts on positioning accuracy performance, evaluation of how much the candidate error sources (e.g. TRP synchronization error) impact on positioning accuracy performance should be considered separately for timing based methods and angle based methods.

Reference

- RP-213588, 3GPP RAN#94-e, “Revised SID on Study on expanded and improved NR positioning”, Electronic Meeting, Dec. 2021.