3GPP TSG RAN WG1#109-e	 	 	R1- 2204388

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on positioning for RedCap UEs

Agenda Item:	9.5.3

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, revised SID on Rel-18 NR positioning enhancement was agreed [1]. The study item includes objectives related to positioning support for RedCap UEs as follows:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs [RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In this contribution, we present our view on potential enhancements of positioning support for RedCap UEs for Rel-18 NR positioning.

- Potential enhancements for positioning support for RedCap UEs

  - Performance evaluations

There are huge demands of connectivity for various kinds of devices. They are expected to be used not only in the factory use cases such as for large-scale sensor networks but also in the consumer use cases such as for wearable devices. As the IoT society advances in the future, low complexity, and inexpensive devices are expected to spread. Introduction of RedCap UEs in Rel-17 is one of the solutions, and they are optimized for low-end and mid-end UEs.  RedCap UEs may have several use cases that require NR positioning support in addition to GNSS positioning. For example, NR positioning has an advantage in indoor use cases where the received power of GNSS signal may be weak. The need for indoor positioning/navigation (e.g., factory or building) is significant even when the UE can’t receive GNSS signal or when the UE can receive it but is heavily affected by multipath. Furthermore, even if GNSS positioning is available (e.g., outdoor positioning),  it would be good if the NR positioning can be used instead of GNSS positioning because GNSS requires more power consumption and the power consumption is important for RedCap devices. NR positioning may have an advantage over GNSS positioning in terms of power consumption since long battery life is a critical requirement especially for RedCap UEs. Therefore, NR positioning may be a solution to such demands, and it is beneficial to discuss NR positioning support for RedCap UEs. On the other hand, the major different from non-RedCap UEs is the reduction of bandwidth and number of RX branches, that have large impact to positioning performance. Based on the background, positioning support for RedCap UEs was discussed at the RAN#94-e meeting and it was agreed to discuss and evaluate the potential enhancement for RedCap positioning in Rel-18.

Regarding the performance evaluation, bandwidth is an important factor especially for timing-based positioning methods such as DL-TDOA, UL-TDOA, and Multi-RTT. As an example, PRS/SRS bandwidth aggregation is to be discussed in RAN4 for accuracy improvement. In general, using a narrower bandwidth will degrade positioning accuracy. Maximum bandwidth of RedCap UEs is 20MHz for FR1 and 100MHz for FR2 in Rel-17, and will be further reduced to 5MHz for FR1 in Rel-18. As mentioned so far, applying existing positioning procedures and measurements to RedCap UEs will lead to degraded positioning performance compared with non-RedCap UEs. The angle-based methods such as UL-AoA and DL-AoD can be used especially with mmW beam in FR2 to avoid such positioning performance degradation due to reduced bandwidth. However, in FR1 and lower frequency, accuracy of the angle-based methods may be limited. Thus, RAN1 should consider the accuracy compensation for timing-based positioning methods.

Observation 1:

- Applying existing timing-based positioning procedures and measurements to RedCap UEs will lead to degraded positioning performance compared with non-RedCap UEs due to reduced bandwidth.

We expect to support all the positioning methods for RedCap positioning as well as for non-RedCap UEs. Besides, we don’t find any needs to introduce RedCap-specific positioning method at present since it requires a significant spec impact. In Rel-18, RAN1 is planning to discuss carrier phase measurement support for non-RedCap UEs in AI 9.5.2.2. Considering the characteristic of carrier phase measurement positioning using phase information, it may be suitable for narrowband positioning since it doesn’t depend on the bandwidth. If Rel-18 non-RedCap UEs support carrier phase measurement, it may be better to discuss whether RedCap UEs can also support carrier phase measurement. However, we should consider that low complexity methods are preferred for this type of device. Therefore, it may be better to evaluate each method and consider which methods are suitable for RedCap UEs with considering complexity of the method in addition to performance.

Observation 2:

- It may be better to evaluate each method and consider which methods are suitable for RedCap UEs with considering complexity of the method in addition to performance.

As discussed above, the accuracy of RedCap positioning with existing positioning methods may be expected to be lower than that of non-RedCap positioning due to the bandwidth reduction. If it is clarified through the performance evaluation results, RAN1 should work on the performance improvement and discuss how much we can improve for RedCap UEs. We prefer to aim the same performance level as non-RedCap UEs, if technically possible. This level may contribute to the enhancement of the RedCap use case. For example, industrial applications such as sensor network may require a strict level of accuracy. Therefore, once the evaluation results reveal performance degradation, RAN1 should work for the enhancement for positioning for RedCap UEs and determine the target performance level.

Proposal 1:

- Once the evaluation results reveal performance degradation, RAN1 should work for the enhancement for positioning for RedCap UEs and determine the target performance level.

  - Potential enhancements to cope with bandwidth reduction

This section describes potential enhancements to cope with bandwidth reduction issues. For the complexity reduction, RedCap UEs have narrower bandwidth. The introduction of PRS frequency hopping may be one possible solution to compensate for the effect. Reference signal frequency hopping can virtually enhance the bandwidth, which contributes to using the frequency diversity. We are expecting some discussion points to support frequency hopping procedure for PRS. RAN1 may need to define the hopping design such as hopping procedure, hopping bandwidth, or hopping pattern designed for low complexity. Definition of RF retuning gap for frequency hopping may be needed due to narrowband implementation of RedCap UEs. The introduction of such a gap may bring the need for the further clarifications of the relationship with the PRS measurement without measurement gap introduced in Rel-17 positioning.

Another possible solution is joint measurement with reference signals in addition to PRS, which can efficiently use the limited bandwidth resources. There may be several discussion points to support the function, some of which are the definition of available reference signals, combining rules, and necessary capabilities. Therefore, we propose to consider PRS frequency hopping and joint estimation as potential solutions to overcome the performance concern due to the bandwidth reduction.

Proposal 2:

- RAN1 should consider PRS frequency hopping and joint estimation as potential solutions to overcome the performance concern due to the bandwidth reduction.

  - Concerns for power consumption

In addition to accuracy performance evaluation, power consumption performance evaluation can also be considered for RedCap positioning. Although low complexity processing may reduce UE’s power consumption, further optimization for RedCap UEs would be required. In Rel-17, LMF doesn’t recognize DRX cycle when the LMF requests UE/gNB to execute positioning processing (e.g., SRS transmission or PRS measurement). Thus, positioning may be indicated at a timing that is not optimal for the UE in terms of power consumption, and in the worst case, UE may not be able to perform positioning in DRX state. If Rel-18 positioning include the performance evaluation and enhancement associated with power consumption, it may be beneficial to discuss the optimization feasibility of positioning in DRX state. DRX cycle reporting from UE/gNB to LMF is one possible solution to support positioning in DRX state. When a LMF requests SRS transmission or PRS measurement to a UE/gNB, DRX cycle information can be used to adjust the periodicity in periodic or semi-persistent positioning. The relationship between the DRX on-duration time and the measurement timing indicated from the NW can also be considered for optimization. It can be adjusted to receive PRS from both serving and non-serving TRPs certainly.

Proposal 3:

- For Rel-18 positioning for RedCap UEs with lower power consumption, it should be discussed the optimization feasibility of positioning in DRX state.

- Conclusion

In this contribution, we discussed potential enhancements for positioning for RedCap UEs. Based on the discussion, we made following observations and proposals.

Observation 1:

- Applying existing timing-based positioning procedures and measurements to RedCap UEs will lead to degraded positioning performance compared with non-RedCap UEs due to reduced bandwidth.

Observation 2:

- It may be better to evaluate each method and consider which methods are suitable for RedCap UEs with considering complexity of the method in addition to performance.

Proposal 1:

- Once the evaluation results reveal performance degradation, RAN1 should work for the enhancement for positioning for RedCap UEs and determine the target performance level.

Proposal 2:

- RAN1 should consider PRS frequency hopping and joint estimation as potential solutions to overcome the performance concern due to the bandwidth reduction.

Proposal 3:

- For Rel-18 positioning for RedCap UEs with lower power consumption, it should be discussed the optimization feasibility of positioning in DRX state.

Reference

- RP-213588, 3GPP RAN#94-e, “Revised SID on Study on expanded and improved NR positioning”, Electronic Meeting, Dec. 2021.