3GPP TSG RAN WG1 #109-e			R1-2204389

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.6.1

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on potential solutions for further UE complexity reduction

Document for:	Discussion and Decision

- Introduction

At the RAN#94e meeting, a new SID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved [1]. The objectives of the SI are shown below.

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we discuss on the analysis of UE complexity reduction, coexistence with legacy UEs, other performance impacts and specification impacts for further UE complexity reduction.

- Discussion

  - UE bandwidth reduction to 5MHz in FR1

In Rel-17, UE bandwidth reduction to 20MHz in FR1 was discussed and some new features, e.g., separate initial DL/UL BWP for Rel-17 RedCap etc., are specified. In Rel-18, now we discuss the further UE complexity reduction techniques and one potential solution would be UE bandwidth reduction to 5MHz as captured in the objectives of the SID. In the following sections, we discuss the possible options for bandwidth reduction, potential issues and its solutions.

    - Possible options for bandwidth reduction

At the previous RAN meetings, it was discussed whether the target of BW reduction is RF and base band or base band only, but not concluded and it is still opened.

In Rel-18 workshop, some companies evaluated the additional cost reduction gain of from 20MHz BW to max 5MHz BW. According to these contributions, we can expect up to 10% additional cost reduction gain with the average of evaluations in [2][3][4] based on TR38.875[5].

In addition to the scenario that UE RF BW is reduced to 5MHz as maximum, according to the evaluations/analysis in [6], other option, i.e., base band BW only reduction, is considered. From this contribution, it was observed that the cost reduction gain is mainly comes from BB part and additional gain from RF BW reduction is not so large.

Obviously, both RF and base band BW reduction can provide higher cost reduction gain than the base band only BW reduction, however, this option is possibly challenging from specification impacts/standardization effort perspective which are discussed in the following sections. Thus, if it is expected that BB only BW reduction can provide comparable cost reduction gain compared to RF and BB BW reduction, this option (i.e., BB only BW reduction) can be considered further.

Based on the above, we briefly summarize the trade-off between cost reduction gain and potential specification impacts on RF and/or BB BW reduction in the following table 1.

Table1.  Trade-off between cost reduction gain and potential specification impacts

|  | Cost reduction gain | Impacts on SSB/control CHs | Impacts on data CHs |
| --- | --- | --- | --- |
| RF+BB | High | Large | Large |
| BB for DL/UL data | Middle | - | Large |
| BB for DL data | Middle | - | Middle |

As described in the SID, we believe it is important to ensure the cost reduction gain from Rel-17 RedCap to decide which option of BW reduction to apply for Rel-18 RedCap. On top of that, we also should take potential issues and/or specification impacts into account considering the limited TU for this WI.

Proposal 1: The trade-off between cost reduction gain and potential specification impacts on RF and/or BB BW reduction needs to be discussed.

    - Potential issues and solutions

In section 2.1.1, we discussed the possible options for BW reduction to 5MHz in FR1. In the following sections, we discuss the potential issues and solutions for each option above.

      - RF bandwidth reduction to 5MHz in FR1

In this section, we discuss the potential issues and solutions for RF BW reduction to 5MHz in FR1. For this option, potential issues/specification impacts are expected to be large compared to the base band only bandwidth reduction. If RF bandwidth is reduced to 5MHz in FR1, it may have impacts even for the channels/signals which are not problematic for Rel-17 RedCap UEs, e.g., SSB and/or CORESET#0 etc.

Issue: PBCH with 30 kHz SCS exceeds 5MHz BW

In the current specification, the BW for PBCH is specified as 20 RB, i.e., 7.2 MHz for 30 kHz SCS. If RF BW for Rel-18 RedCap is restricted to 5MHz, then the UE is not able to receive whole PBCH at once. It should be noted that Rel-15 SSB should be reused as described in the objective notes of the SID, thus Rel-18 RedCap specific SSB design is out of scope in this SI.

The simplest solution can be restriction on SCS for SSB, i.e., only 15 kHz SCS SSB is supported and 30kHz SCS SSB is not supported for Rel-18 RedCap in FR1. This solution may have large impact on operation since the band configured with 30kHz SCS SSB cannot accommodate Rel-18 RedCap UEs and the operation is restricted to be on the cell with SSB configured with 15 kHz SCS. On the other hand, given that  Rel-18 RedCap does not require high data rate or wide BW while coverage is one of the most important aspect to provide assumed IoT services, FDD bands would be the main deployment scenario, where 15kHz SCS can be applied to SSB, this solution can be a candidate solution.

According to the discussion of the Rel-18 WI “NR support for dedicated spectrum less than 5MHz for FR1”, similar issue (how to receive PBCH configured with 15 kHz SCS within 3MHz) was raised and puncturing of PBCH was captured as a solution. This solution may enable Rel-18 RedCap UEs with 5MHz RF BW to receive SSB configured with 30 kHz SCS, thus the same solution can be applied for this issue. However, the performance degradation of punctured PBCH within 5MHz BW for 30 kHz SCS would not be exactly the same with that for punctured PBCH within 3MHz BW for 15 kHz SCS, and it should be carefully investigated.

In addition to the above, it can be also considered that a UE receives one PBCH across multiple times with RF retuning. This solution also enables Rel-18 RedCap UEs with 5MHz RF BW to receive SSB configured with 30 kHz SCS. However, it may take a long time to receive one SSB for Rel-18 RedCap and also not preferable from power consumption pint of view.

Fig. 1: Example for PBCH reception with RF retuning

Based on the discussion above, we made the following proposal;

Proposal 2: RAN1 should discuss how to receive PBCH with 30 kHz SCS in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: not support 30 kHz SCS for SSB

- Alt.2: punctured PBCH within 5MHz BW

- Alt.3: receive one PBCH across multiple times with RF retuning

Issue: MIB-configured CORESET#0 for 15/30 kHz SCS (except 24 RB with 15 kHz SCS) exceeds 5MHz

Similar to PBCH reception, MIB-configured CORESER#0 BW seems also problematic if Rel-18 RedCap RF BW is reduced to 5MHz. According to the current specification, the CORESET#0 BW can be configured with 24, 48 or 96 RB for 15 kHz SCS and 24 or 48 RB for 30 kHz SCS. Therefore, except the case that configuration is 24 RB with 15 kHz SCS, the CORESET#0 BW exceeds 5MHz BW.

Same as the discussion for PBCH above, the most straightforward solution can be the restriction on the MIB-configured CORESET#0 configuration, i.e., only 24 RB with 15 kHz SCS can be configured for Rel-18 RedCap UEs. However, if this CORESET#0 configuration is shared among Rel-18 RedCap and non-Rel-18 RedCap UEs, it is not preferable in terms of configuration flexibility. Furthermore, the aggregation level can be also restricted, i.e., AL16 cannot be supported, and may have impact on the coverage. For this concern on coverage degradation, PDCCH repetition may be able to compensate the loss which is evaluated in our contribution for AI9.6.2 [7]. On the other hand, if the CORESET#0 configuration is not shared among Rel-18 RedCap and non-Rel-18 RedCap UEs, it needs to be discussed how to configure/indicate separate MIB-configured CORESET#0 for Rel-18 RedCap UEs while this operation is not preferable from NW overhead perspective but may be beneficial in terms of Rel-18 RedCap UE offloading.

To support CORESET#0 configured with 30 kHz SCS, it can be also considered that smaller number of RB (e.g., 12 RB) and lower AL (e.g., AL2) is introduced, which are not supported in the current specification since ALs for type0-PDCCH CSS is specified as AL4, 8 or 16. For this solution, the reliability/coverage degradation would be the issue but can be compensated by the PDCCH repetition same as the above solution. However, if smaller number of RB is newly introduced for Rel-18 RedCap, it means a gNB configures separate CORESET#0 resource for Rel-18 RedCap which is not preferable from NW overhead point of view and also specification impact would be large since the CORESET#0 resource tables specified in TS38.213 needs to be updated/modified.

In addition to the above, same as the discussion for PBCH above, it can be also considered that a UE receives one PDCCH across multiple times with RF retuning. This solution also enables Rel-18 RedCap UEs with 5MHz RF BW to receive PDCCH larger BW than 5MHz, thus MIB-configured CORESET#0 would be shared among Rel-18 RedCap and non-Rel-18 RedCap UEs. However, it may take a long time to receive one PDCCH for Rel-18 RedCap and also not preferable from power consumption point of view. If the delay for PDCCH reception could be a problem, we can consider non-interleaved mapping for PDCCH reception with MIB-configured CORESET#0 which is possibly reduce the number of receptions for a whole PDCCH.

Fig.2: Example for PDCCH reception in CORESET#0 with RF retuning

Based on the discussion above, we made the following proposal;

Proposal 3: RAN1 should discuss the configuration of MIB-configured CORESET#0 for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: support only 24 RB for 15 kHz SCS

  - Option: PDCCH repetition can compensate coverage degradation

- Alt.2: support smaller number of RBs and/or lower AL

  - Option: PDCCH repetition can compensate coverage degradation

- Alt.3: receive one PDCCH across multiple times (with RF retuning)

  - Option: non-interleaved mapping can be supported to reduce the number of receptions for a whole PDCCH

Issue: SIB1 resource may not be sufficient with 5MHz

It may also need a discussion whether SIB1 resource is sufficient if the BW of PDSCH for SIB1 reception is restricted to be within 5MHz. If a cell allow Rel-18 RedCap UEs to access, the BW SIB1 resource may need to be restricted as Rel-18 RedCap maximum BW even for non-Rel-18 RedCap UEs considering co-existence.

To avoid such restriction for non-Rel-18 RedCap UEs, Rel-18 specific SIB1 resource can be configured and also the contents of SIB1 can be optimized for Rel-18 RedCap. This may also require Rel-18 RedCap dedicated CORESET#0 configuration, but may be beneficial in terms of Rel-18 RedCap UE offloading.

Another possibility is PDSCH receptions across multiple times with RF retuning. This solution enables Rel-18 RedCap UEs with 5MHz RF and/or BB BW to receive PDSCH larger BW than 5MHz, thus SIB1 resource would be shared among Rel-18 RedCap and non-Rel-18 RedCap UEs. However, as discussed above, it may take a long time to receive one PDSCH for Rel-18 RedCap and also not preferable from power consumption pint of view.

Based on the discussion above, we made the following proposal;

Proposal 4: RAN1 should discuss the potential shortage of SIB1 resource for Rel-18 RedCap in FR1 including the following alternatives as possible solutions if RF and BB BW or BB BW for DL data channel for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: Rel-18 RedCap dedicated SIB1

- Alt.2: receive one PDSCH across multiple times (with RF retuning)

Issue: initial DL/UL BWP for non-Rel18 RedCap may exceed 5MHz

As discussed in Rel-17 RedCap WI, it may also need a discussion for separate initial DL/UL BWP configuration for Rel-18 RedCap UEs if RF BW is reduced to 5MHz.

At this point, there may be two possible solutions, 1) Rel-18 RedCap UE specific separate initial DL/UL BWP is supported, 2) initial DL/UL BWP is shared with non-eRedCap UE and/or Rel-17 RedCap UE, but channels/signals for Rel-18 RedCap UEs which are received/transmitted in the BWP are confined within 5MHz.

For the 1st solution, a Rel-18 RedCap UE does not expect that channels/signals which are received/transmitted in the BWP span larger BW than 5MHz and the resources for channels related to random access can be configured Rel-18 RedCap dedicatedly. In addition, especially for initial DL BWP, if separate initial DL BWP for Rel-18 RedCap UEs is supported, the same aspects as Rel-17 RedCap may need to be discussed, e.g., whether SSB/CORESET#0 are included in the initial DL BWP, whether CORESET#0 for eRedCap can be used as initial DL BWP and/or center frequency alignment with initial UL BWP for eRedCap.

For the 2nd solution, BWP itself can be configured with larger BW than 5MHz, however, all the resources for channel/signals which a Rel-18 RedCap UE expects to receive/transmit are confined within 5MHz BW, thus it can be required for the Rel-18 RedCap UE to perform RF retuning within the BWP. This operation is also applicable if only BB BW is reduced to 5MHz.

Based on the discussion above, we made the following proposal;

Proposal 5: RAN1 should discuss the configuration of initial DL/UL BWP for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: separate initial DL/UL BWP for Rel-18 RedCap

- Alt.2: initial DL/UL BWP is shared with non-eRedCap UE and/or Rel-17 RedCap UE, but CH/signals are confined within 5MHz

Issue: all the RACH occasions may not be confined within initial UL BWP for eRedCap

Associated with the discussion of initial UL BWP operation, it may need discussion whether all the ROs are not confined within 5MHz BW.

One possible solution is Rel-18 RedCap UE dedicated RO configurations to ensure all the ROs associated with each SSB index are confined within 5MHz which is supported for Rel-17 RedCap.

Another possible solution is that the NW indicates multiple initial UL BWP configurations and a Rel-18 RedCap UE selects one applicable initial UL BWP from them which includes the RO associated with the best SSB for the UE. By the way, PUCCH frequency hopping disabling was supported for PUCCH before dedicated configuration for Rel-17 RedCap to avoid PUSCH resource fragmentation issue if separate initial UL BWP is configured. However, if this solution, i.e., multiple initial UL BWP configurations, is supported for Rel-18 RedCap, PUSCH fragmentation no longer avoided even if the frequency hopping is disabled. Therefore, this solution is not preferable from the PUSCH resource fragmentation perspective.

Based on the discussion above, we made the following proposal;

Proposal 6: RAN1 should discuss the configuration of RACH occasions for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: Rel-18 RedCap UE dedicated RO configuration

- Alt.2: multiple initial UL BWP configurations

Issue: early indication of Rel-18 RedCap may be necessary for Rel-18 RedCap specific operation during random access

As discussed in Rel-17 RedCap WI, if Rel-18 RedCap UE specific operation is specified during initial access/random access, it would be necessary for the Rel-18 RedCap UE to report its capability as early indication during random access. It would depend on the framework of Rel-18 RedCap UE operation for initial access/random access. More specifically, if a Rel-18 RedCap UE can support the same operation as Rel-17 RedCap, the early indication mechanism supported in Rel-17 may be sufficient. However, considering that if it is necessary to ensure that PDSCH/PUSCH received/transmitted in initial DL/UL BWP is not span larger BW than 5MHz especially for Rel-18 RedCap UE, early indication of Rel-18 RedCap would be necessary.

Proposal 7: RAN1 should discuss how to report that a UE is Rel-18 RedCap if Rel-18 RedCap UE specific operation is specified during random access.

Issue: Whether SSB is included in initial/RRC-configured DL BWP for eRedCap

As discussed in Rel-17 RedCap WI, it is necessary to discuss whether SSB is included in the separate initial DL BWP and/or RRC-configured DL BWP for Rel-18 RedCap UEs. In Rel-17, the operation with NCD-SSB is mandatorily supported in RRC connected mode if CD-SSB is not included in the DL BWP. However, for Rel-18 RedCap UEs, it is not preferable to mandate gNB to transmit NCD-SSB since the NW overhead of NCD-SSB would not be the same with Rel-17 RedCap. For example, NCD-SSB can be configured for Rel-17 RedCap UEs and additional NCD-SSB can be configured especially for Rel-18 RedCap, which is FDMed with Rel-17 RedCap. Therefore, it should be also considered that operation with NCD-SSB reception is not mandatorily supported but Rel-18 RedCap supports SSB-less BWP operation, e.g., CD-SSB reception with measurement gap as basic operation.

Based on the discussion above, we made the following proposal;

Proposal 8: RAN1 should discuss whether SSB is included in initial/RRC-configured DL BWP for Rel-18 RedCap and how to operate in the CD-SSB-less DL BWP if it does not include CD-SSB including following alternatives;

- Alt.1: Rel-18 RedCap UEs expect NCD-SSB reception in the DL BWP

- Alt.2: Rel-18 RedCap UEs support SSB-less BWP operation, e.g., CD-SSB reception with measurement gap and/or CSI-RS based operation

Issue: TRS minimum BW for Rel-18 RedCap

In the current specification, minimum BW for TRS is specified as 52 or 48 especially for shared spectrum operation, which cannot be supported with 5MHz BW regardless of SCS, and hence it should be discussed how to handle this issue. For example, it can be considered that the TRS minimum BW for Rel-18 RedCap can be specified separately from that for legacy UEs.

Based on the discussion above, we made the following proposal;

Proposal 9: RAN1 should discuss the minimum BW for TRS for Rel-18 RedCap in FR1.

Issue: PF2/3 for 30/60 kHz SCS may exceed 5MHz depending on number of RB

In the current specification, the maximum number of RBs (16 is the maximum value in the spec) for PUCCH format 2 and 3 is configured by a higher layer parameter, and the actual number of RBs for PUCCH transmission is decided depending on the payload size of UCI. For Rel-18 RedCap UE, it can be expected that the maximum number of RBs for PF2/3 would be restricted, e.g., 10 RBs for 30 kHz SCS and 6 RBs for 60 kHz SCS if the BW is reduced to 5MHz. In that sense, if the UCI size is large such that the legacy UE transmits with 16 RBs, it may not be able to transmit with one PUCCH resource even if the highest target code rate is configured currently supported. By the way, in the current specification, the maximum UCI payload size for PF4 which is configured with single PRB is limited up to 115 bits. Given that the maximum number of RBs for PF2/3 is restricted for Rel-18 RedCap, the restriction on the maximum UCI payload size can be specified, which is also beneficial to ensure the reliability/coverage of PF2/3.

Based on the discussion above, we made the following proposal;

Proposal 10: RAN1 should discuss UCI payload size limitation is required for Rel-18 RedCap if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Issue: FH gain for PUCCH/PUSCH within 5MHz BW may affect coverage degradation

In the current specification, frequency hopping is supported for UL channels which can provide frequency diversity gain. However, if the maximum BW for UL BWP is restricted to 5MHz, it is expected that coverage would be degraded compared to the legacy UEs since the distance between 1st hop and 2nd hop in frequency domain is limited and the frequency diversity gain is lost. Regarding this issue, the frequency diversity loss can be evaluated first in AI9.6.2[x], and then it can be identified whether this issue should be handled in WI phase or not depending on the evaluation results of the coverage degradation with 5MHz BW.

Based on the discussion above, we made the following proposal;

Proposal 11: RAN1 should evaluate the effect on frequency diversity gain loss for Rel-18 RedCap if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Issue: DL BWP for SSB/SIB1/paging reception in RRC IDLE/INACTIVE mode

For Rel-17 RedCap, SIB1 and/or paging are received only in the BWP which includes CD-SSB in RRC idle/inactive mode. Similarly, it should be also discussed for Rel-18 RedCap on which DL BWP the UE receives SSB/SIB1/paging in RRC idle/inactive mode, depending on the BWP operation for Rel-18 RedCap. For example, SIB1 and/or paging reception can be only on the shared initial DL BWP with legacy UE which includes CD-SSB, which is the same operation with Rel-17 RedCap, can be considered. For another example, SIB1 and/or paging can be received on a separate initial DL BWP for Rel-18 RedCap which may or may not include CD-SSB. This option may be beneficial in terms of Rel-18 RedCap UE offloading, however, may not preferable in terms of NW overhead since Rel-18 RedCap dedicated SIB1/paging resources would be configured.

Based on the discussion above, we made the following proposal;

Proposal 12: RAN1 should discuss the DL BWP for SSB/SIB/paging reception for Rel-18 RedCap in RRC idle/inactive mode if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

      - Base band bandwidth reduction to 5MHz in FR1 for DL/UL or DL data channels

In this section, we discuss the potential issues and solutions for BB BW reduction for both DL and UL or only DL data channels to 5MHz in FR1.

For this option, BB BW for channels/signals other than data channels are not restricted at all, so potential issues/specification impacts are expected to be less/small compared to RF BW reduction, e.g., SSB and CORESET#0 can be received as it is in the current specification. More specifically, the BW for BWP is not restricted within 5MHz and a UE can transmit/receive control channels which spans larger BW than 5MHz. However, still some points are problematic even for this option especially initial access aspects. For example, it may need discussion whether SIB1 resource can be shared among Rel-18 RedCap and non-Rel-18 RedCap UEs, i.e., whether 5MHz BW is sufficient as SIB1 resource. In addition, PDSCH for Msg.2/4/B and PUSCH for Msg.3/A need to be configured within 5MHz BW for Rel-18 RedCap, and hence early indication to report its capability of Rel-18 RedCap.

Based on the discussion above, we made the following proposal;

Proposal 13: RAN1 should discuss at least the following aspects if BB BW for DL/UL data channels for Rel-18 RedCap is reduced to 5MHz.

- Whether Rel-18 RedCap dedicated SIB1 resource should be configured

- Whether or how to report its capability of Rel-18 RedCap as early indication during random access

  - Reduced UE peak data rate in FR1

In addition to the bandwidth reduction to 5MHz in FR1, another potential solution, UE peak data rate reduction in FR1, is captured in SID.

Scaling factor

According to TS38.306[8], the peak data rate is calculated by the following equation;

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of layers

is the maximum modulation order

is the scaling factor

is the numerology

is the average OFDM symbol duration in a subframe for numerology

is the maximum RB allocation in bandwidth  with numerology

is the overhead and takes the following values

[0.14], for frequency range FR1 for DL

If the BW is reduced to 5MHz, the target data rate for Rel-18 RedCap (i.e., 10Mbps in DL) is achieved by the current scaling factor (i.e., 0.75) as shown in the following table. Furthermore, as shown in the table, smaller value of scaling factor (e.g., 0.5) can be supported which can simplify the BB part and provide further cost reduction gain.

Table 2: Peak data rate [Mbps] with 5MH BW

|  | Scaling factor |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0.5 | 0.75 | 0.8 | 1 |
| FR1 DL, 16QAM, 1 layer, 25 RB, 15 kHz SCS | 6.69 | 10.03 | 10.70 | 13.38 |
| FR1 DL, 64QAM, 1 layer, 25 RB, 15 kHz SCS | 10.03 | 15.05 | 16.05 | 20.06 |

Proposal 14: RAN1 should discuss whether to introduce a new scaling factor value (e.g., 0.5) for data rate computation, with relaxation of the constraint that the multiple of is smaller than 4.

Relaxed UE processing time

To reduce UE peak data rate, it is captured as potential solution in SID objective that UE processing timeline, i.e., PDSCH processing time, PUSCH preparation time and CSI computation time, can be relaxed. According to TR38.875, the cost reduction gain of the doubled values from capability 1 for PDSCH processing time and PUSCH preparation time specified in the current specification, i.e., N1 = 16, 20 and 34 symbols for 15, 30 and 60 kHz SCS and N2 = 20, 24 and 46 symbols for 15, 30 and 60 kHz SCS, was evaluated, and it was observed that the estimated total cost reduction gain is up to 6% in FR1. Similarly, the total cost reduction gain for relaxed CSI computation time is evaluated and it was observed that the estimated total cost reduction gain is up to 5% in FR1.

For Rel-17 RedCap, the relaxed UE processing time is not adopted since the cost reduction gain is “relatively” smaller than other adopted complexity reduction techniques, and Rel-17 RedCap does not target low-end use cases. However, for Rel-18 RedCap, the main goal is to further embrace new use cases, especially requiring low-cost devices (and low energy consumption). In that sense, any complexity reduction techniques which are beneficial for low-end use cases should be supported as long as the cost reduction gain is justified with reasonable NW impact. Thus, we propose to adopt the relaxed UE processing time for Rel-18 RedCap.

Proposal 15: RAN1 should support the relaxed UE processing time for Rel-18 RedCap.

- For PDSCH processing time, PUSCH preparation time and CSI computation time, the doubled values from those in the current specification can be the candidate values.

Whether to support HARQ process number reduction

In addition to the relaxed UE processing time, HARQ process number reduction can be a solution to reduce UE peak data rate and can provide additional cost reduction gain, however, it was also not adopted for Rel-17 RedCap with the same reason for the relaxed UE processing time. As discussed the above, we believe that any complexity reduction techniques which can provide further cost reduction gain compared to Rel-17 RedCap should be considered, and also we don’t see a large specification impact on this technique. Therefore, we propose to support HARQ process number reduction for Rel-18 RedCap.

Proposal 16: RAN1 should support the HARQ process number reduction for Rel-18 RedCap.

- Conclusion

In this contribution, we discussed and its specification impacts. Based on the discussion, we made following proposals.

Proposal 1: The trade-off between cost reduction gain and potential specification impacts on RF and/or BB BW reduction needs to be discussed.

Proposal 2: RAN1 should discuss how to receive PBCH with 30 kHz SCS in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: not support 30 kHz SCS for SSB

- Alt.2: punctured PBCH within 5MHz BW

- Alt.3: receive one PBCH across multiple times with RF retuning

Proposal 3: RAN1 should discuss the configuration of MIB-configured CORESET#0 for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: support only 24 RB for 15 kHz SCS

  - Option: PDCCH repetition can compensate coverage degradation

- Alt.2: support smaller number of RBs and/or lower AL

  - Option: PDCCH repetition can compensate coverage degradation

- Alt.3: receive one PDCCH across multiple times (with RF retuning)

  - Option: non-interleaved mapping can be supported to reduce the number of receptions for a whole PDCCH

Proposal 4: RAN1 should discuss the potential shortage of SIB1 resource for Rel-18 RedCap in FR1 including the following alternatives as possible solutions if RF and BB BW or BB BW for DL data channel for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: Rel-18 RedCap dedicated SIB1

- Alt.2: receive one PDSCH across multiple times (with RF retuning)

Proposal 5: RAN1 should discuss the configuration of initial DL/UL BWP for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: separate initial DL/UL BWP for Rel-18 RedCap

- Alt.2: initial DL/UL BWP is shared with non-eRedCap UE and/or Rel-17 RedCap UE, but CH/signals are confined within 5MHz

Proposal 6: RAN1 should discuss the configuration of RACH occasions for Rel-18 RedCap in FR1 including the following alternatives if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

- Alt.1: Rel-18 RedCap UE dedicated RO configuration

- Alt.2: multiple initial UL BWP configurations

Proposal 7: RAN1 should discuss how to report that a UE is Rel-18 RedCap if Rel-18 RedCap UE specific operation is specified during random access.

Proposal 8: RAN1 should discuss whether SSB is included in initial/RRC-configured DL BWP for Rel-18 RedCap and how to operate in the CD-SSB-less DL BWP if it does not include CD-SSB including following alternatives;

- Alt.1: Rel-18 RedCap UEs expect NCD-SSB reception in the DL BWP

- Alt.2: Rel-18 RedCap UEs support SSB-less BWP operation, e.g., CD-SSB reception with measurement gap and/or CSI-RS based operation

Proposal 9: RAN1 should discuss the minimum BW for TRS for Rel-18 RedCap in FR1.

Proposal 10: RAN1 should discuss UCI payload size limitation is required for Rel-18 RedCap if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Proposal 11: RAN1 should evaluate the effect on frequency diversity gain loss for Rel-18 RedCap if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Proposal 12: RAN1 should discuss the DL BWP for SSB/SIB/paging reception for Rel-18 RedCap in RRC idle/inactive mode if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Proposal 13: RAN1 should discuss at least the following aspects if BB BW for DL/UL data channels for Rel-18 RedCap is reduced to 5MHz.

- Whether Rel-18 RedCap dedicated SIB1 resource should be configured

- Whether or how to report its capability of Rel-18 RedCap as early indication during random access

Proposal 14: RAN1 should discuss whether to introduce a new scaling factor value (e.g., 0.5) for data rate computation, with relaxation of the constraint that the multiple of is smaller than 4.

Proposal 15: RAN1 should support the relaxed UE processing time for Rel-18 RedCap.

- For PDSCH processing time, PUSCH preparation time and CSI computation time, the doubled values from those in the current specification can be the candidate values.

Proposal 16: RAN1 should support the HARQ process number reduction for Rel-18 RedCap.

References

[1]	RP-213661, Study on further NR RedCap UE complexity reduction, Ericsson.

[2]	RWS-210313, Motivation for Rel-18 WI on Enhanced RedCap, Ericsson.

[3]	RWS-210409, On RedCap enhancements in Rel-18, CATT.

[4]	RWS-210476, Rel-18 NR RedCap evolution, ZTE.

[5]	TR38.875, Study on support of reduced capability NR devices (Release 17).

[6]	R1-2201671, On further NR RedCap UE complexity reduction, Ericsson.

[7]	R1-2204390, Discussion on simulations and assumptions for further UE complexity reduction, NTT DOCOMO, INC.

[8]	TS38.306, User Equipment (UE) radio access capabilities.