3GPP TSG RAN WG1 #109-e			R1-2204390

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.6.2

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on simulations and assumptions for further UE complexity reduction

Document for:	Discussion and Decision

- Introduction

At the RAN#94e meeting, a new SID on further NR RedCap UE complexity reduction (FS_NR_redcap_enh) was approved [1]. The objectives of the SI are shown below.

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx: - Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - Notes: - Rel-15 SSB should be reused and L1 changes minimized. - Operation in BWP with/without SSB and without/with RF retuning should be considered. - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage. - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

In this contribution, we discuss on the potential evaluations of the impacts to coverage.

- Discussion

  - Potential evaluation scenarios

    - PBCH reception with 5MHz BW for 30 kHz SCS

In our contribution for AI9.6.1[2], we point out that the PBCH whose BW is 7.2 MHz for 30 kHz SCS cannot be received whole at once if the RF BW for Rel-18 RedCap is restricted to 5MHz, and this issue needs to be discussed.

As we discuss in [2], some potential solutions can be considered, e.g., puncturing of PBCH. This solution is applied to a similar issue (how to receive PBCH configured with 15 kHz SCS within 3MHz) on the discussion for Rel-18 WI “NR support for dedicated spectrum less than 5MHz for FR1”. According to the evaluation in [3], it was observed that the PBCH link budget degradation for punctured PBCH within 3MHz BW for 15 kHz SCS is less than 1dB for 2 PRB puncturing, and approximately 2.25dB for 4 PRB puncturing. However, for punctured PBCH within 5MHz BW for 30 kHz SCS, the coverage degradation is expected to be larger since more frequency resources need to be punctured, and it may be no longer practical. Hence, the coverage degradation by the puncturing PBCH can be one possible simulation scenario to evaluate whether it is practical solution or not.

In addition, it can be also considered that a UE receives one PBCH across multiple times with RF retuning. This solution also enables Rel-18 RedCap UEs with 5MHz RF BW to receive SSB configured with 30 kHz SCS, and the specification impact is expected to be small. However, it is unclear whether/how the coverage is degraded e.g., whether the performance can be affected by Doppler shift etc., compared to the legacy UE’s operation. Hence, it can be also evaluated to observe whether/how the coverage would be degraded if a PBCH is received across multiple times.

Proposal 1: RAN1 should discuss how Rel-18 RedCap UE can receive PBCH with 30 kHz SCS if the RF BW is reduced to 5MHz and evaluate the performance of possible solutions, for example

- Puncturing of PBCH

- PBCH reception across multiple times

    - PDCCH reception with 5MHz BW

In our contribution for AI9.6.2[2], some potential issues are identified related to PDCCH reception with 5MHz BW.

The potential issue which needs discussion is that MIB-configured CORESER#0 BW exceeds 5MHz except the case that configuration is 24 RB with 15 kHz SCS, and hence the aggregation level would be restricted, i.e., AL16 cannot be supported, and may have impact on the coverage for PDCCH. Furthermore, to support 30 kHz SCS for CORESET#0, it can be considered that smaller number of RBs (e.g., 12 RBs) and lower AL (e.g., AL2) for type0-PDCCH CSS are introduced. However, the reliability/coverage would be degraded for such low ALs, and hence it can be evaluated to observe the coverage degradation. We also provide some initial evaluation results in section 2.2. To address the concern on the coverage degradation, one possible solution is PDCCH repetition which may be able to compensate the loss. We also provide some initial evaluation results for PDCCH repetition gain in section 2.2. In addition, same as the discussion for PBCH above, another possible solution which can enable supporting high AL as legacy UEs is that a UE receives one PDCCH across multiple times with RF retuning. We also provide some initial evaluation results for PDCCH repetition across multiple times in section 2.2.

Proposal 2: RAN1 should discuss whether/how to address the coverage degradation for PDCCH if the RF BW is reduced to 5MHz and BW for CORESET and corresponding aggregation level are restricted, and evaluate the performance of possible solutions, for example

- PDCCH repetition

- PDCCH reception across multiple times

    - PUCCH/PUSCH FH gain with 5MHz BW

In our contribution for AI9.6.2 [2], it is also pointed out that if the maximum BW for UL BWP is restricted to 5MHz, the UL coverage for Rel-18 RedCap would be degraded compared to the legacy UEs since the distance between 1st hop and 2nd hop in frequency domain is limited and the frequency diversity gain is lost. However, it is unclear how the coverage loss is large and whether it can be acceptable. Hence, to identify whether it should be addressed in the WI phase, it would be worth evaluating whether/how the UL coverage would be degraded in this study phase before we investigate the solution further.

Proposal 3: RAN1 should evaluate the effect on frequency diversity gain loss for UL channels if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

  - Evaluation assumptions and simulation results

As we discussed in section 2.1.2, we evaluate some potential issue and possible solutions related to PDCCH reception for Rel-18 RedCap UE with 5MHz BW.

In this section, we provide our evaluation assumptions and link-level simulation results for the following three evaluations;

- The performances of PDCCH reception for reference UE

- The performance for PDCCH repetition

- The performance for PDCCH reception across multiple times

The simulation assumptions are summarized in the table 1. Basically, it follows the assumptions shown in TR 38.875 [4] and TR38.830 [5], but we assume the frequency bandwidth for reference UE as 20 MHz and the parameters related to the CCE-to-REG mapping are configured assuming MIB-configured CORESET#0.

Table 1: Simulation assumptions

| Center frequency | 700 MHz |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  | 30 kHz |  |
| Frequency bandwidth | 20 MHz (reference UE) | 5 MHz | 20 MHz (reference UE) | 5 MHz |
| Maximum number of PRBs | 106 | 25 | 51 | 11 |
| Channel model | TDL-C |  |  |  |
| Delay spread | 300 ns |  |  |  |
| Mobile speed | 3 km/h |  |  |  |
| PRB allocation | 24 / 48 / 96 | 24 | 24 / 48 | 11 |
| Symbol duration | 1 / 3 |  | 3 |  |
| Number of Tx antennas | 2 |  |  |  |
| Number of Rx antennas | 1 |  |  |  |
| PDCCH modulation | QPSK |  |  |  |
| Channdel coding | Polar code |  |  |  |
| Aggregation level | 1 / 2 / 4 / 8 / 16 |  |  |  |
| CCE-to-REG mapping | Interleaved |  |  |  |
| Interleaver size (R) | 2 |  |  |  |
| REG-bundle size (L) | 6 |  |  |  |
| Precoder granularity | 1 RB |  |  |  |
| Channel estimation | 2D-MMSE |  |  |  |
| Pay load size | 40 bit |  |  |  |

    - Evaluation results for PDCCH reception for the reference UE

In this section, we evaluate the PDCCH reception performances for each AL to evaluate how the reliability can be degraded if the AL is restricted to be low, and hence the target UE is the reference UE which can support PDCCH reception with 48/96 RB CORESET and AL16. In this evaluation, the evaluation metric is BLER and the BLER performance target is set as 1%.

We summarize the simulation results of required SINR of BLER 1% especially for 15 kHz SCS with different CORESET BW in Table 2.1, and also results with other parameters and BLER-SINR characteristics are shown in appendix. As shown in Table 2.1, we can observe that AL16 can provide up to 3.3 dB gain compared to AL8 when the CORESET BW is 48 or 96 RB, i.e., if the CORESET#0 BW is restricted to 24RB (at most AL8 can be used), we can estimate the BLER-SINR characteristics degrade approximately 3 dB compared to the legacy UEs performance.

Table 2.1: Required SINR [dB] for BLER 1%

|  | AL 1 | AL 2 | AL 4 | AL 8 | AL 16 |
| --- | --- | --- | --- | --- | --- |
| Reference UE: 24RB/3symbol | 18.11 | 8.88 | 3.27 | -0.44 | - |
| Reference UE: 48RB/3symbol | 18.02 | 8.79 | 3.05 | -0.68 | -3.79 |
| Reference UE: 96RB/3symbol | 18.03 | 9.09 | 3.55 | -0.29 | -3.63 |

    - Evaluation results for PDCCH repetition

In this section, we evaluate the PDCCH repetition performances for each ALs to evaluate whether it can compensate the coverage degradation as shown in section 2.2.1 if the AL is restricted to be low, and hence the target UE is Rel-18 RedCap UE which can support PDCCH reception with only 24 RB CORESET and the highest AL as AL8. In this evaluation, same as the evaluation above, the evaluation metric is BLER and the BLER performance target is set as 1%. Regarding the details of PDCCH repetition, we evaluate the case that PDCCH is repeatedly received twice/four times assuming time separation between each CORESETs as 20/80 ms.

Fig. 2.1: PDCCH repetition with 24 RB CORESET.

We summarize the simulation results of required SINR of BLER 1% for 15 kHz SCS in Tables 2.2a/b, and also BLER-SINR characteristics are shown in appendix.

As shown in the Table 2.2a, compared to the evaluation results shown in section 2.2.1, the performance gap between the required SINR for 2 PDCCH repetitions with AL4/AL8 and no repetition with AL8/AL16 respectively are no larger than 1dB, i.e., 2 PDCCH repetitions for AL4/AL8 can provide comparable performance with AL8/AL16 without repetition, respectively. Furthermore, if PDCCHs are received in the non-continuous slots, the time domain diversity gain can be observed and the performances are improved further.

Observation 1: AL4/AL8 with 2 PDCCH repetitions can provide comparable performance with AL8/AL16 without repetition, respectively.

In addition, as shown in Table2.2b, compared to the evaluation results shown in section 2.2.1, 4 PDCCH repetitions with AL4/AL8 provide better performance, i.e., up to 3.4 dB gain in the performance of the required SINR, than AL8/AL16, respectively. Furthermore, same as the 2 PDCCH repetitions, if PDCCHs are received in the non-continuous slots, the time domain diversity gain can be observed and the performances are improved further.

Observation 2: AL4/AL8 with 4 PDCCH repetition can provide better performance than AL8/AL16 without repetition, respectively.

Table 2.2a: Required SINR [dB] for BLER 1%

|  | AL 1 | AL 2 | AL 4 | AL 8 | AL 16 |
| --- | --- | --- | --- | --- | --- |
| Reference UE: 24RB/3symbol | 18.11 | 8.88 | 3.27 | -0.44 | - |
| Reference UE: 48RB/3symbol | 18.02 | 8.79 | 3.05 | -0.68 | -3.79 |
| 2 PDCCH repetition with 20ms gap: 24RB/3symbol | 11.60 | 4.14 | 0.07 | -3.09 |  |
| 2 PDCCH repetition with 80ms gap: 24RB/3symbol | 9.55 | 2.99 | -0.86 | -3.84 |  |

Table 2.2b: Required SINR [dB] for BLER 1%

|  | AL 1 | AL 2 | AL 4 | AL 8 | AL 16 |
| --- | --- | --- | --- | --- | --- |
| Reference UE: 24RB/3symbol | 18.11 | 8.88 | 3.27 | -0.44 | - |
| Reference UE: 48RB/3symbol | 18.02 | 8.79 | 3.05 | -0.68 | -3.79 |
| 4 PDCCH repetition with 20ms gap: 24RB/3symbol | 6.60 | 0.61 | -2.78 | -5.52 |  |
| 4 PDCCH repetition with 80ms gap: 24RB/3symbol | 3.80 | -1.36 | -4.46 | -6.95 |  |

    - Evaluation results for PDCCH reception across multiple times

In this section, we evaluate the PDCCH reception across multiple times with RF retuning, i.e., a UE receives one PDCCH in 48/96 RB CORESET across multiple times with 5MHz BW, for each ALs to evaluate whether the coverage degradation is observed compared to the legacy UE operation, and hence the target UE is Rel-18 RedCap UE which can support PDCCH reception with only 24 RB CORESET at once, but the highest AL as AL16 with 48/96 RB. In this evaluation, same as the evaluation above, the evaluation metric is BLER and the BLER performance target is set as 1%. Regarding the details of PDCCH reception, we evaluate the case that one PDCCH is received across two times/four times assuming time separation between each CORESETs as 20/80 ms.

Fig.2.3: PDCCH reception in 48RB CORESET across 2 monitoring occasions with 5MH BW each

We summarize the simulation results of required SINR of BLER 1% for 15 kHz SCS in Table 2.3, and also results with other parameters and BLER-SINR characteristics are shown in appendix.

As shown in the figure 3.2 and 3.3, compared to the evaluation results shown in section 2.2.1, the performance gap between the required SINR for PDCCH reception across multiple times and PDCCH reception with the legacy UE operation are no larger than 1dB, i.e., the operation of PDCCH reception across multiple times can provide comparable performance with the legacy UE operation. Furthermore, if PDCCH is received in the non-continuous slots, the time domain diversity gain can be observed and the performances are improved further.

Observation 3: The operation of PDCCH reception across multiple times with RF retuning can provide comparable performance with the operation of PDCCH reception for legacy UE.

Table 2.3: Required SINR [dB] for BLER 1%

|  | AL 2 | AL 4 | AL 8 | AL 16 |
| --- | --- | --- | --- | --- |
| Reference UE: 48RB/3symbol | 8.79 | 3.05 | -0.68 | -3.79 |
| PDCCH reception across 2 MOs with 20ms gap: 48RB/3symbol | 8.70 | 3.06 | -0.72 | -3.86 |
| PDCCH reception across 2 MOs with 80ms gap: 48RB/3symbol | 8.75 | 3.03 | -0.77 | -4.04 |

- Conclusion

In this contribution, we evaluated and discussed. Based on the evaluation results and discussion above, we derived following observations and proposals.

Proposal 1: RAN1 should discuss how Rel-18 RedCap UE can receive PBCH with 30 kHz SCS if the RF BW is reduced to 5MHz and evaluate the performance of possible solutions, for example

- Puncturing of PBCH

- PBCH reception across multiple times

Proposal 2: RAN1 should discuss whether/how to address the coverage degradation for PDCCH if the RF BW is reduced to 5MHz and BW for CORESET and corresponding aggregation level are restricted, and evaluate the performance of possible solutions, for example

- PDCCH repetition

- PDCCH reception across multiple times

Proposal 3: RAN1 should evaluate the effect on frequency diversity gain loss for UL channels if RF and BB BW for Rel-18 RedCap is reduced to 5MHz.

Observation 1: AL4/AL8 with 2 PDCCH repetitions can provide comparable performance with AL8/AL16 without repetition, respectively.

Observation 2: AL4/AL8 with 4 PDCCH repetition can provide better performance than AL8/AL16 without repetition, respectively.

Observation 3: The operation of PDCCH reception across multiple times with RF retuning can provide comparable performance with the operation of PDCCH reception for legacy UE.

References

- RP-213661, Study on further NR RedCap UE complexity reduction, Ericsson..

- R1-2204389, Discussion on potential solutions for further UE complexity reduction, NTT DOCOMO, Inc.

- RWS-210121, Railway, Smart Grid and PPDR support in Rel-18, Nokia, Nokia Shanghai Bell.

- TR38.875, Study on support of reduced capability NR devices (Release 17)

- TR38.830, Study on NR coverage enhancements (Release 17)

Appendix: Simulation results

- Summary of required SINR for BLER 1%

|  | AL 1 | AL 2 | AL 4 | AL 8 | AL 16 |
| --- | --- | --- | --- | --- | --- |
| Reference UE: 15 kHz SCS/24RB/3symbol | 18.11 | 8.88 | 3.27 | -0.44 | - |
| Reference UE: 15 kHz SCS/48RB/3symbol | 18.02 | 8.79 | 3.05 | -0.68 | -3.79 |
| Reference UE: 15 kHz SCS/96RB/3symbol | 18.03 | 9.09 | 3.55 | -0.29 | -3.63 |
| Reference UE: 15 kHz SCS/24RB/1symbol | 15.68 | 7.62 | 3.37 | - | - |
| Reference UE: 15 kHz SCS/48RB/1symbol | 15.60 | 7.50 | 3.08 | -0.16 | - |
| Reference UE: 15 kHz SCS/96RB/1symbol | 15.64 | 7.82 | 3.32 | -0.18 | -2.78 |
| Reference UE: 30 kHz SCS/24RB/3symbol | 16.84 | 8.27 | 2.81 | -1.12 | - |
| Reference UE: 30 kHz SCS/48RB/3symbol | 16.73 | 8.46 | 3.16 | -0.87 | -4.16 |
| 2 PDCCH repetition in consecutive slots: 24RB/3symbol | 12.24 | 4.29 | 0.11 | -3.07 | 12.24 |
| 2 PDCCH repetition with 20ms gap: 15 kHz SCS/24RB/3symbol | 11.60 | 4.14 | 0.07 | -3.09 | - |
| 2 PDCCH repetition with 80ms gap: 15 kHz SCS/24RB/3symbol | 9.55 | 2.99 | -0.86 | -3.84 | - |
| 4 PDCCH repetition in consecutive slots: 15 kHz SCS/24RB/3symbol | 7.64 | 1.17 | -2.28 | -5.13 | - |
| 4 PDCCH repetition with 20ms gap: 15 kHz SCS/24RB/3symbol | 6.60 | 0.61 | -2.78 | -5.52 | - |
| 4 PDCCH repetition with 80ms gap: 15 kHz SCS/24RB/3symbol | 3.80 | -1.36 | -4.46 | -6.95 | - |
| PDCCH reception across 2 MOs in consecutive slots: 15 kHz SCS/48RB/3symbol | 18.03 | 8.70 | 3.11 | -0.70 | -3.83 |
| PDCCH reception across 2 MOs with 20ms gap: 15 kHz SCS/48RB/3symbol | 18.11 | 8.70 | 3.06 | -0.72 | -3.86 |
| PDCCH reception across 2 MOs with 80ms gap: 15 kHz SCS/48RB/3symbol | 18.04 | 8.75 | 3.03 | -0.77 | -4.04 |
| PDCCH reception across 2 MOs in consecutive slots: 15 kHz SCS/96RB/3symbol | 18.08 | 9.07 | 3.56 | -0.30 | -3.63 |
| PDCCH reception across 2 MOs with 20ms gap: 15 kHz SCS/96RB/3symbol | 17.91 | 9.10 | 3.52 | -0.36 | -3.72 |
| PDCCH reception across 2 MOs with 80ms gap: 15 kHz SCS/96RB/3symbol | 18.21 | 8.86 | 3.01 | -0.84 | -4.23 |

- Simulation results regarding BLER-SINR characteristics

- Simulation results related to section 2.2.1

- Simulation results related to section 2.2.1

- Simulation results related to section 2.2.3