3GPP TSG RAN WG1 #109-e			R1-2204393

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on side control information to enable NR network-controlled repeaters

Agenda Item:	9.8.1

Document for: 	Discussion

- Introduction

At RAN#94e meeting, a new SID [1] on “Study on NR Network-controlled Repeaters” was approved. The detailed objectives are as follows.

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we discuss on side control information to enable NR network-controlled repeaters.

- Structure and capability of NW-controlled repeater

To study and identify which side control information is necessary for NW-controlled repeater, we should discuss and clarify the structure and capability of NW-controlled repeater, since the necessity of side control information for each feature would depend on the structure and capability. Figure 1 shows our assumed structure of NW-controlled repeater. NW-controlled repeater may have “UE function” to receive and decode signal of side control information from gNB in addition to “Repeater function” for amplify-and-forward the DL and UL signals of UEs. Figure 2 shows example of Tx and Rx timing of gNB, NW-controlled repeater and UE. Based on Fig.1 and Fig.2, we list up following six points for the discussion of NW-controlled repeater capability.

Figure 1. Example of structure of NW-controlled repeater

Figure 2. Example of Tx/Rx timing of gNB, NW-controlled repeater, and UE

- Capability of “UE function”

It should be studied and clarified whether “UE function” has full UE capability or limited UE capability. For example, if “UE function” has full UE capability, “UE function” can perform time synchronization with gNB using UE procedure, so that the timing information as side control information may not be necessary. If “UE function” does not have this capability, additional timing information may be necessary.

- Simultaneous “UE function” and “Repeater function” operations

If NW-controlled repeater cannot operate “UE function” and “Repeater function” simultaneously, “UE function” may not be able to receive/transmit cell specific signals/channels for access UEs (e.g. SSB, RACH occasion, common PDCCH), since they needs to be amplified-and-forwarded by “Repeater function” to access UEs. Therefore, in this case, separate time/frequency resource for “UE function” and “Repeater function” should be considered. It can also be considered that DL and UL may have different capability, e.g. NW-controlled repeater can operate “UE function” and “Repeater function” simultaneously for DL (receiving side control information while amplify-and-forward signals/channels for UEs), but not for UL (transmission from UE function can be performed only when repeater function does not perform amplify-and-forward).

- Simultaneous “DL” and “UL” operation

It should be studied and clarified whether NW-controlled repeater can operate amplify-and-forward for DL (towards UE(s)) and UL (towards gNB) simultaneously or not. If NW-controlled repeater does not have this capability, it needs to switch DL and UL operation, and TDD pattern information is necessary for the switching.

- Sharing information between “UE function” and “Repeater function”

If “UE function” has full UE capability, “UE function” has information, e.g. TDD pattern as a normal UE, and if the information can be used to control “Repeater function”, additional side control information is not necessary.

- Switching gap between DL Rx and DL Tx, UL Rx and UL Tx

As in SID, NW-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously. Therefore, we can assume that switching gap is not necessary to be considered for the NW-controlled repeater operation.

- Control of Tx timing of “Repeater function”

Since “Repeater function” consists of only analog components, Tx timing for forwarding cannot be controlled. We think it is natural assumption and it may not bring any issue for NW-controlled repeater operation.

Proposal 1: Study and clarity the structure of NW-controlled repeater, e.g. it may have “UE function” to receive and decode signal of side control information from gNB, and “Repeater function” for amplify-and-forward the DL and UL signals

Proposal 2: Study following capabilities for NW-controlled repeater.

1. Capability of “UE function”

2. Simultaneous “UE function” and “Repeater function” operations

3. Simultaneous “DL” and “UL” operation

4. Sharing information of “UE function” and “Repeater function”

5. Switching gap between DL Rx and DL Tx, UL Rx and UL Tx

6. Control of Tx timing of “Repeater function”

- Side control information

  - Beamforming information

Beamforming is beneficial to improve capacity, coverage performances and mitigate interference, and hence it is assumed that the repeater has beamforming capability (at least for FR2). For the repeater beam management, side control information for beamforming information would be necessary. Based on our assumed structure of NW-controlled repeater, following three functionalities should be considered as shown in Fig.3.

1. Beam management of “UE function”

Existing beam indication mechanism for normal UE can be used for the beam management for UE function e.g., to receive side control information.

2. Beam management of “Repeater function” to gNB (DL Rx and UL Tx beams)

Existing beam indication mechanism for normal UE would be only applicable to “UE function” or time resource for “UE function” even if antenna panel is shared by “UE function” and “Repeater function”, and hence the beam management of “Repeater function” to gNB should be studied. As in SID, since NW-controlled repeater is assumed to be stationary, the best beam between NW-controlled repeater and gNB may be static. For example, once a beam is indicated, the beam can be applicable for a period until a beam update is indicated. For the beam management, following two options can be considered.

Option 1 : “Repeater function” to gNB applies the beam indicated for “UE function”

Option 2 : beam of “Repeater function” to gNB is indicated by side control information.

Proposal 3: Study beam management of “Repeater function” to gNB, and following two options can be considered.

Option 1 : “Repeater function” to gNB applies the beam indicated for “UE function”

Option 2 : beam of “Repeater function” to gNB is indicated by side control information.

3. Beam management of “Repeater function” to UEs (DL Tx and UL Rx beams)

We can assume that NW-controlled repeater would need to sweep its DL Tx/UL Rx beams for forwarding e.g. cell-specific channels/signals, while it is expected that NW-controlled repeater can use the appropriate beam for a UE to forward UE-specific channels/signals. Therefore, beam management of “Repeater function” to UEs are necessary, and we need to study how to indicate DL Tx/UL Rx beams for “Repeater function”.

Proposal 4: Study beam management of “Repeater function” to UE. Beam of “repeater function” to UE can be indicated by side control information.

Figure 3. Example of beam management of NW-controlled repeater

  - Timing information

In our assumption, “UE function” should have at least a capability of time synchronization to gNB as a normal UE e.g., to identify frame/subframe/slot boundaries, and “Repeater function” may be able to follow the time synchronization information e.g., to switch between DL forwarding and UL forwarding as well as some corresponding characteristics for forwarding such as beam, power, etc. Thus, necessity of timing information as side control information is up to capability of NW-controlled repeater, and at first we may need to clarify the capability.

Proposal 5: Necessity of timing information can be studied after the study of NW-controlled repeater capability.

  - Information on UL-DL TDD configuration

If NW-controlled repeater needs to switch some characteristics for forwarding such as beam, power, etc. between DL forwarding and UL forwarding, information on TDD UL/DL configuration is necessary in addition to the time synchronization information. “UE function” of NW-controlled repeater may have the information, such as TDD-config-common, TDD-config-dedicated, DCI format 2_0, as a normal UE, and this information can be used for control of “Repeater function” if NW-controlled repeater has such capabilities. Therefore, necessity of TDD UL/DL configuration information as side control information is up to capability of NW-controlled repeater as well as timing information.

Proposal 6: Necessity of TDD UL/DL configuration information can be studied after the study of NW-controlled repeater capability.

  - ON-OFF information

ON/OFF control is beneficial to mitigate interference and improve energy efficiency. For example, if no time resource is allocated for “Repeater function” toward to access UEs, NW-controlled repeater does not need to operate the time resource, and it can be indicated by NW with ON/OFF information.

Proposal 7: Study ON/OFF information for NW-controlled repeater operation for mitigate interference and improve energy efficiency.

  - Power control information

Power control information is one of the discussion points for mitigating interference and improving energy efficiency. For the DL power control, it may be left to implementation, since NW may not have sufficient information of how to control the DL power, e.g. whether interference happens or not, and DL power control is not typical use case for current NW deployment. On the other hands for the UL power control, if NW-controlled repeater always forwards UL signals with fixed e.g., maximum output power, the signals may interfere UL signals directly from UEs to gNB whose Tx power are controlled by gNB. Therefore, UL Tx power control is beneficial to mitigate interference and UL Tx power control mechanism for NW-controlled repeater needs to be studied.

Proposal 8: Study UL Tx power control for NW-controlled repeater for mitigate interference.

- Conclusion

In this contribution, we discussed side control information to enable NR network-controlled repeaters. Based on the discussion we made the following proposals.

Proposal 1: Study and clarity the structure of NW-controlled repeater, e.g. it may have “UE function” to receive and decode signal of side control information from gNB, and “Repeater function” for amplify-and-forward the DL and UL signals

Proposal 2: Study following capabilities for NW-controlled repeater.

1. Capability of “UE function”

2. Simultaneous “UE function” and “Repeater function” operations

3. Simultaneous “DL” and “UL” operation

4. Sharing information of “UE function” and “Repeater function”

5. Switching gap between DL Rx and DL Tx, UL Rx and UL Tx

6. Control of Tx timing of “Repeater function”

Proposal 3: Study beam management of “Repeater function” to gNB, and following two options can be considered.

Option 1 : “Repeater function” to gNB applies the beam indicated for “UE function”

Option 2 : beam of “Repeater function” to gNB is indicated by side control information.

Proposal 4: Study beam management of “Repeater function” to UE. Beam of “repeater function” to UE can be indicated by side control information.

Proposal 5: Necessity of timing information can be studied after the study of NW-controlled repeater capability.

Proposal 6: Necessity of TDD UL/DL configuration information can be studied after the study of NW-controlled repeater capability.

Proposal 7: Study ON/OFF information for NW-controlled repeater operation for mitigate interference and improve energy efficiency.

Proposal 8: Study UL Tx power control for NW-controlled repeater for mitigate interference.

References

- 3GPP, RP-213700, ZTE, “Study on NR Network-controlled Repeaters,” Dec. 2021.