3GPP TSG RAN WG1 #109-e			R1-2204394

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on L1/L2 signaling for side control information

Agenda Item:	9.8.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94e meeting, a new SID [1] “Study on NR Network-controlled Repeaters” was approved. Detailed RAN1 objectives are as follows.

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

In this contribution, we discuss L1/L2 signaling to carry the side control information for NW controlled repeater.

- Discussion

As discussed in our companion paper [2], a NW controlled repeater can have a “UE function” for decoding side control information and a “repeater function” for amplify-and-forwarding signal from gNB/UE to UE/gNB. In the following discussion, we use the terms of “UE function” and “repeater function” to distinguish the two functions of repeater. And we use the term “repeater function to gNB” for discussion of repeater’s Tx/Rx to/from gNB, and “repeater function to UE” for discussion of repeater’s Tx/Rx to/from UE.

In the following sections, we discuss side control information for beamforming, UL-DL, timing, ON-OFF, power, respectively.

Figure1. Structure of a network-controlled repeater

2.1 Beamforming information

As discussed in our companion paper [2], beamforming is beneficial to improve capacity, coverage and mitigate interference. For a network-controlled repeater, beamforming information needs to be considered for UE function, repeater function to gNB, and repeater function to UE, respectively.

For UE function, beamforming information can be derived via Rel-17 mechanism for legacy UEs. For repeater function to gNB, considering that the best beam for repeater function to gNB and the best beam for UE function to gNB are likely to be the same, one possible way is that the beam indicated for UE function with Rel-17 mechanism can also be applied to repeater function. Another possible solution is to introduce new side control information for repeater function. Considering that a repeater is assumed to be stationary, the best beam between a repeater and gNB is likely to be static, hence, the indication of beam for repeater function to gNB can be static, that is, an indicated beam can be applicable for a long time duration. In summary, for beamforming of repeater function to gNB, following options can be considered.

- Option1: repeater function to gNB applies the beam indicated for UE function. New signaling is not needed.

- Option2: beam of repeater function to gNB is indicated by side control information where an indicated beam can be applicable for a time duration.

Proposal 1: For beamforming of repeater function to gNB, following options can be considered.

- Option1: repeater function to gNB applies beam indicated for UE function. New signaling is not needed.

- Option2: beam of repeater function to gNB is indicated by side control information where an indicated beam can be applicable for a time duration.

For repeater function to UE, beam control of repeater function to UE is a new feature and new signaling to carry the side control information will probably be needed. Considering the best beam of repeater function towards different UE will be different, the beam used for repeater function to UE may change frequently. Therefore, indicating beam of repeater function per time unit (e.g., per slot) can be considered. Regarding the signaling to carry the side control information, semi-static and dynamic indication can be considered. From our perspective, beam indication in the following cases should be taken into account as shown in Fig.2.

- Case1: when a cell-specific/semi-static signal is forwarded. These signals occur periodically, hence, periodic beam pattern can be indicated for repeater function to UE. Semi-static signaling can be suitable to indicate beam information for repeater function to UE for forwarding these signals.

- Case2: when a UE-specific/dynamic signal is forwarded. To achieve flexibility, dynamic signaling can be considered.

Proposal 2: For beamforming of repeater function to UE, a beam can be indicated per time unit (e.g., per slot). Whether semi-static or dynamic indication is supported can be further studied.

Figure2. Example of beamforming of repeater function to UE

Furthermore, how to indicate the beam for repeater function to UE needs to be studied. In Rel-17 beam indication for legacy UEs, a beam is usually indicated as a TCI state/spatial relation info that contains a reference RS, and once a TCI state/spatial relation is indicated, same spatial domain filter as the reference RS is used for a DL Rx or UL Tx. Since repeater function itself will not generate and transmit a RS, legacy beam indication mechanism referring to a reference RS is not suitable for repeater function. To indicate a repeater beam, one possible option is to introduce a new ID that directly refers to a spatial domain filter of repeater function to UE. Another possible option is to rely on an association between gNB beam ID (e.g., gNB RS ID) and repeater beam. More specifically, a gNB beam ID is indicated and a repeater beam is derived based on the association between gNB beam ID and repeater beam. In this case, different gNB beam ID is indicated when different repeater beam is used. In summary, for beamforming of repeater function to UE, a repeater beam can be indicated with following options.

- Option1: a new ID that directly refers to a spatial domain filter of repeater

- Option2: a gNB beam ID that is associated with a spatial domain filter of repeater

Proposal 3: For beamforming of repeater function to UE, a repeater beam can be indicated with following options.

- Option1: a new ID that directly refers to a spatial domain filter of repeater

- Option2: a gNB beam ID that is associated with a spatial domain filter of repeater

2.2 UL-DL information

Regarding UL-DL information, TDD configuration of UE function can reuse Rel-17 mechanism for legacy UEs, i.e., indicated by TDD-config-common, TDD-config-dedicated, and DCI format 2_0. For TDD configuration of repeater function to gNB/UE, following options can be considered.

- Option1: Repeater function follows TDD configuration of UE function.

- Option2: TDD configuration of repeater is indicated by side control information.

From our perspective, considering TDD configuration of a cell is usually aligned, TDD configuration of repeater function is probably the same as UE function. Thus, option1 is preferred and new signaling for UL/DL information of repeater function is not needed.

Proposal 4: For UL/DL information, repeater function follows TDD configuration of UE function. New signaling for UL/DL information of repeater function is not needed.

2.3 Timing information

As discussed in section 2.2, DL/UL of repeater function to gNB/UE can be controlled by network. For repeater function switching between DL and UL operation, the time of switching (or to say the boundary of DL/UL operation) needs to be determined. DL/UL frame boundary of UE function can be derived with Rel-17 mechanism same as legacy UEs. To derive DL/UL boundary of repeater function, following options can be considered.

- Option 1: Repeater function follows DL/UL boundary of UE function.

- Option 2: Timing information to derive DL/UL boundary of repeater function is indicated by side control information

- Option 3: Timing information of repeater function is up to implementation

From our perspective, for DL timing, DL operation of a repeater function should start at the time when DL signal from gNB is received. Thus, it is reasonable that DL timing of repeater function follows DL Rx time of UE function. For UL timing, to avoid interference, reception time of UL signal from different UE including UE function of repeater should be aligned at gNB. Similarly, reception time of UL signal from repeater function should also be aligned with other UL signals at gNB. Thus, it is reasonable that UL timing of repeater function to gNB follows UL Tx time of UE function. Meanwhile, for UL timing, Rx time of UL signal from UE at repeater function also needs to be considered because UL operation of repeater function should start at the time when UL signal from UE is received. In our view, one possible way is that by implementation gNB can control UL Tx timing of a scheduled UE to make sure that the Rx time of UL signal from UE at repeater function is aligned with the UL timing of repeater function to gNB (i.e., aligned with UL Tx time of UE function which is also controlled by gNB).

Thus, option1 is preferred, and new signaling for timing information of repeater function is not needed.

Proposal 5: For timing information, repeater function follows DL/UL boundary of UE function. New signaling for timing information of repeater function is not needed.

2.4 ON-OFF information

As discussed in our companion paper [2], ON-OFF control of repeater is beneficial for interference mitigation and energy efficiency. From our perspective, whether a repeater should be turned ON or OFF will depend on the condition of a scheduled UE. If gNB would schedule a UE that can directly connect to gNB, the repeater can be turned OFF; otherwise, if gNB would schedule a UE that needs a repeater’s amplify-and-forwarding, the repeater can be turned ON. Thus, ON/OFF status of a repeater could change frequently. Therefore, indicating ON/OFF per time unit (e.g., per slot) can be considered. Regarding the signaling to carry the side control information, semi-static and dynamic indication can be considered. With semi-static signaling, overhead can be reduced, on the other hand, with dynamic signaling, more flexibility of ON/OFF control can be achieved. Meanwhile, similar as discussed for beam information, forwarding of cell-specific/semi-static signal and UE-specific/dynamic signal should be taken into account. For forwarding of cell-specific/semi-static signal, semi-static indication could be suitable. While for forwarding of UE-specific/dynamic signal, dynamic indication can be considered.

Proposal 6: ON/OFF of repeater function can be indicated per time unit (e.g., per slot). Whether semi-static or dynamic indication is supported can be further studied.

2.5 Power information

As discussed in our companion paper [2], for DL Tx power of repeater function to UE, it may be left to implementation. For UL Tx power control of repeater function to gNB, it could be beneficial for interference mitigation. In this section, we mainly discuss UL Tx power control of repeater.

For UL Tx power control of UE function, Rel-17 mechanism for legacy UEs can be reused. For repeater function to gNB, following options can be considered.

- Option1: repeater function applies same UL Tx power as UE function.

- Option2: introduce new formular for repeater function UL Tx power control.

From our perspective, option 1 is a simper solution, however, in Rel-17 UL power control scheme for legacy UEs, Tx power of PUSCH, PUCCH of a UE is related to transmission bandwidth, BPRE, PUCCH format, etc. Thus, when a repeater function forwards an UL signal from UE, it may not be suitable that the same UL Tx power of UE function is directly reused.

If a new formular for repeater function UL Tx power control is considered, whether open loop or closed loop power control is supported needs to be studied first. For open loop power control, power control parameters can be determined semi-statically. For closed loop power control, UL Tx power of repeater function can be adjusted by dynamic indication.

Proposal 7: for UL power control of repeater function to gNB, following options can be considered.

- Option1: repeater function applies same UL Tx power as UE function.

- Option2: introduce new formular for repeater function UL Tx power control. Whether open loop or closed loop power control is supported can be further studied.

In more details, for open loop power control, following options can be considered to determine power control parameters semi-statically.

- Option1: open loop power control parameters of repeater function are up to implementation.

- Option2: repeater function applies power control parameters configured for UE function.

- Option3: power control parameters of repeater function are semi-statically configured by side control information.

For closed loop power control, following options can be considered to dynamically adjust the Tx power.

- Option1: repeater function applies closed loop power adjustment and TPC indicated for UE function.

- Option2: closed loop power adjustment for repeater function is dynamically indicated by side control information, e.g., similar as TPC indication for legacy UEs.

- Conclusion

In this contribution, we discussed L1/L2 signaling to carry the side control information for NW controlled repeater. Based on the discussion, we made following proposals.

Proposal 1: For beamforming of repeater function to gNB, following options can be considered.

- Option1: repeater function to gNB applies beam indicated for UE function. New signaling is not needed.

- Option2: beam of repeater function to gNB is indicated by side control information where an indicated beam can be applicable for a time duration.

Proposal 2: For beamforming of repeater function to UE, a beam can be indicated per time unit (e.g., per slot). Whether semi-static or dynamic indication is supported can be further studied.

Proposal 3: For beamforming of repeater function to UE, a repeater beam can be indicated with following options.

- Option1: a new ID that directly refers to a spatial domain filter of repeater

- Option2: a gNB beam ID that is associated with a spatial domain filter of repeater

Proposal 4: For UL/DL information, repeater function follows TDD configuration of UE function. New signaling for UL/DL information of repeater function is not needed.

Proposal 5: For timing information, repeater function follows DL/UL boundary of UE function. New signaling for timing information of repeater function is not needed.

Proposal 6: ON/OFF of repeater function can be indicated per time unit (e.g., per slot). Whether semi-static or dynamic indication is supported can be further studied.

Proposal 7: for UL power control of repeater function to gNB, following options can be considered.

- Option1: repeater function applies same UL Tx power as UE function.

- Option2: introduce new formular for repeater function UL Tx power control. Whether open loop or closed loop power control is supported can be further studied.

Reference

[1] 3GPP RP-213700, “New SI: Study on NR Network-controlled Repeaters”, RAN#94e, Dec. 2021

[2] 3GPP R1-2204393, “Discussion on side control information to enable NR network-controlled repeaters”, NTT DOCOMO INC., RAN1#109e, May 2022.