3GPP TSG-RAN WG1 #109-e			R1-2204396

e-Meeting, May 9 – 20, 2022

Agenda item:	9.9.2

Title:	Discussion on LTE CRS rate matching pattern

Source:	NTT DOCOMO, INC.

Document for:	Discussion/Decision

- Introduction

In RAN#94-e a new work item on enhancement of NR dynamic spectrum sharing (DSS) was approved [1][2]. One of the objectives is to allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP. In this contribution, we present our views on LTE CRS rate matching pattern.

- Discussion

LTE CRS is transmitted in symbols 0, 4, 7 and 11 when the number of CRS antenna ports is one or two. When the number of CRS antenna ports is four, CRS is transmitted in symbols 1 and 8 in addition to them. Figure 1 shows LTE CRS location when the number of CRS antenna ports is four.

Figure 1 Location of LTE CRS (4ports)

CRS location depends on the cell ID of the LTE cell. So, LTE CRS location can be different across neighbouring cells. If interferences from LTE CRS of multiple neighbouring LTE cells are significant, performing the rate matching around LTE CRS of multiple LTE cells may be useful to mitigate interference. However, in the current specification, two overlapping LTE CRS rate matching patterns can only be applied when multi-TRP transmission is configured. Therefore, it would be useful for interference mitigation to be able to apply multiple LTE CRS rate matching patterns regardless of the number of TRPs. As for the number of patterns, it would be sufficient to address the LTE CRS of a total of two cells: the neighbouring LTE cells with the strongest and the second strongest interferences.

Proposal 1: Two overlapping LTE CRS rate matching patterns can be applied to a PDSCH regardless of the number of TRPs.

- Conclusion

In this contribution, we have the following proposal:

Proposal 1: Two overlapping LTE CRS rate matching patterns can be applied to a PDSCH regardless of the number of TRPs.

- References

- RP-220842, Meeting Report for TSG RAN meeting #94e

- RP-213575, New WI: Enhancement of NR Dynamic spectrum sharing (DSS)