3GPP TSG RAN WG1 #109-e			R1-2204398

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.10.1

Source: 	NTT DOCOMO, INC.

Title: 	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and Decision

- Introduction

At the RAN#94e meeting, a new WID on Multi-carrier enhancements (NR_MC_enh) was approved and it was revised at the RAN#95e meeting [1]. At the RAN#95e meeting, additional note for this WID was agreed in [2] which will be added in the next WID revision. The objectives of the core part WI are shown below.

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling 2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Note: Extension of TX switching for 2 bands to multiple TAG configurations is included in the scope. The work is limited to RAN4. - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this contribution, we discuss on the objective 1, i.e., multi-cell PUSCH/PDSCH scheduling with a single DCI.

- Discussion

  - Target scenarios and assumptions

In Rel-15, for multi-cell operation, the functionality of cross-carrier PDSCH/PUSCH scheduling (Fig.1b) which enables a DCI on a cell to schedule PDSCH/PUSCH on another cell was supported in addition to self-carrier PDSCH/PUSCH scheduling (Fig.1a). Furthermore, in Rel-16/17, the functionality of multi-PDSCHs/PUSCHs scheduling with a single DCI for self and cross-carrier scheduling (Fig.1c/1d) was supported.

Fig.1a: self-carrier scheduling for single cell.

Fig.1b: cross-carrier scheduling for single cell.

Fig.1c: multi-PDSCH/PUSCH scheduling for single cell (self-carrier).

Fig.1d: multi-PDSCH/PUSCH scheduling for single cell (cross-carrier).

At the discussion for DSS in Rel-17 WI, it was evaluated and discussed whether to introduce a new functionality of multi-carrier scheduling with a single DCI, however, no consensus was achieved, and it was not supported since the scenario/assumption was restricted as only for FR1, up to 2 scheduled cells and only for PDSCH.  In Rel-18, the solutions for multi-carrier PDSCH/PUSCH scheduling with single DCI (Fig.2a/2b) is to be discussed and specified in this WI.

Fig.2a: cross-carrier scheduling for multiple cells.       Fig.2b: self and cross-carrier scheduling for multiple cells.

As described in the work plan [3], the allocated TUs and meetings for this WI are limited, and hence it is important for the timely completion to clarify the common understandings and to narrow-down the scope of the target scenarios/assumptions in the early phase of the WI discussion.

One possible point that can be clarified at this point is whether the fallback DCI (i.e., DCI format 0_0/1_0) can be a scheduling DCI of multi-carrier PDSCH/PUSCH scheduling. Obviously, multi-carrier scheduling requires high configurability for some DCI fields, thus we can preclude the fallback DCI from the candidate DCI formats supported for multi-carrier scheduling.

Proposal 1: Multi-carrier PDSCH/PUSCH scheduling with a single DCI is not supported by DCI format 0_0/ and DCI format 1_0.

We believe that the target type of SS set configuration for DCI for multi-carrier scheduling can also be clarified. According to the WID, it is described that the motivation for supporting multi-carrier scheduling is to increase the flexibility and spectral/power efficiency for DL/UL data scheduling by reducing the control overhead. Therefore, we propose that only USS set should be the applicable search space type for multi-carrier PDSCH/PUSCH scheduling DCI.

Proposal 2: The DCI for multi-carrier PDSCH/PUSCH scheduling is applicable to USS set only and not to any type of CSS set.

Another point we would like to clarify is a TB mapping for multiple cells, which is not clearly explained in the WID. Previously, some companies proposed that one TB can be mapped across multiple cells. However, this feature is not supported in the current specification, thus the specification impact/standardization workload would be large and may also have some higher layer impacts for RAN2. Hence, we propose focussing on the scenario that a separate TB is scheduled for each cell in this WI considering the limited TU allocation.

Proposal 3: Separate TB is scheduled for each cell by the single DCI scheduling multi-cell PDSCH/PUSCH.

We think that it is also good to clarify whether the single DCI can schedule PDSCH and PUSCH in different cell simultaneously. In the current specification, the simultaneous scheduling of DL and UL with a single DCI is not supported, and hence it is a completely new functionality and the specification impacts/standardization efforts are expected to be quite large. Accordingly, we propose focussing on the scenario that either PDSCHs or PUSCHs are scheduled with a single DCI in this WI.

Proposal 4: Either PDSCHs or PUSCHs on multiple cells are scheduled with a single DCI.

The next point we would like to clarify is the relation of SCSs among multiple cells for multi-carrier scheduling. According to the objectives captured in the WID, both intra-band and inter-band CA are clearly included in the scope of the discussion. Regarding the SCSs among multiple cells for multi-carrier scheduling, it is obvious that the case where the scheduling cell and all the scheduled cells are configured with the same SCS is the simplest scenario and should be included as a target scenario for both intra-band and inter-band CA. In addition, we think there are possibly two scenarios for supporting different numerologies among scheduling cell and scheduled cells, 1) different numerology between scheduling cell and scheduled cells (same numerology across scheduled cells) and 2) different numerology across scheduled cells. For example, one possible scenario for multi-carrier scheduling is that a FR1 cell schedules FR2 cells to efficiently utilize the wideband carriers in high frequency range with reliable and power-efficient PDCCH monitoring. To support such scenario, we believe that at least the case 1, i.e., different numerology between scheduling cell and scheduled cells (same numerology across scheduled cells), should be included as a scope of the discussion, and also case 2 can be considered if there is any clear motivation.

In addition, if the case where scheduling cell has lower SCS and scheduled cells have higher SCS is to be supported, we think that the combination between multi-carrier scheduling and multi-PDSCHs/PUSCHs per cell is beneficial and should be considered. Otherwise, in order to fully utilize the time domain resources in scheduled cells with higher SCS i.e., multiple slots within a time duration of a slot with SCS of scheduling cell, multiple DCIs need to be used in the scheduling cell within a slot. However, the current WID seems precluding such combination since “one PDSCH/PUSCH per cell” is described in the WID, and hence it may be necessary to update the WID if the case with different numerologies between scheduling cell and scheduled cells is to be supported.

Proposal 5: Multi-cell PDSCH/PUSCH scheduling targets to support at least following scenarios;

- Scheduling cell and scheduled cells are within a same band or in different bands with same numerology

- FFS: different numerologies, with potentially updating WID to allow “multiple PDSCHs/PUSCHs per cell”

  - case 1: between scheduling and scheduled cells (same numerology across scheduled cells)

  - case 2: across scheduled cells

  - Determination of target maximum number of scheduled cells

As captured in the objectives of this WI, the maximum number of cells which can be scheduled simultaneously by a single DCI needs to be specified. According to the current specification, the maximum number of cells that can be supported for multi-cell operation is 16 for both DL and UL, thus we can say that the maximum number of cells for multi-carrier scheduling is no larger than 16. Furthermore, as captured in the objectives of the WID, the maximum number should be equal to or larger than 3 since it was observed in the Rel-17 DSS discussion that the multi-carrier scheduling gain is not sufficiently provided if the number of scheduled cell is 2.

Observation 1: The maximum number of cells which can be scheduled simultaneously by a single DCI should be from 3 to 16.

In Rel-16 and Rel-17, multi-PDSCHs/PUSCHs scheduling with a single DCI was supported and the maximum number of PDSCHs/PUSCHs that can be scheduled simultaneously is specified as 8. If the framework of this functionality can be followed or expanded to support multi-carrier scheduling, we can consider 8 cells as one candidate value for the maximum number of scheduled cells.

From other perspectives, one important point to be considered for the determination of the maximum number of scheduled cells is DCI compression gain by multi-carrier scheduling with a single DCI, thus we believe the maximum number should not be too small. However, as there should be some DCI fields that need to have separate indication for each scheduled cell, larger number of scheduled cells will require larger DCI size and too large DCI size will be problematic in terms of PDCCH coverage performance and DCI size limitation based on the current specification. Therefore, we should consider the trade-off and a certain reasonable value for the maximum number of scheduled cells that can be simultaneously scheduled by single DCI. In addition, practical usage scenarios of the multi-cell scheduling should also be considered to decide the maximum number of scheduled cells. Considering the limited TUs/meetings for this WI, it would be beneficial to identify some candidate values for the maximum number of scheduled cells at this meeting so that companies can perform focused investigations on the candidate values with considering DCI design aspects. In that sense, we think that 8, 6 and 4 can be the candidate values for further discussion.

By the way, it should also be clarified that UE can be configured with a certain number of potential scheduled cells that is equal to or less than the maximum number of scheduled cells in the specification. In addition, maximum number of scheduled cells for single DCI multi-cell scheduling may be a part of UE capability reporting, and such possibility can be discussed when Rel-18 UE capabilities are discussed.

Based on the discussion above, we propose to agree on the following rough indication for the maximum number of scheduled cells at this point.

Proposal 6: Discuss following alternatives for the target maximum number of scheduled cells;

- Alt.1: 8

- Alt.2: 6

- Alt.3: 4

Note1: Following aspects should be considered to specify the maximum number of scheduled cells: DCI size limitation, DCI compression gain by multi-carrier scheduling by single DCI, PDCCH blocking rate, practical use-case.

Note2: UE is configured with a certain number of potential scheduled cells (equal to or less than above value), and UE capability for maximum number of scheduled cells may be considered.

From DCI design perspective, the maximum number of scheduled cells for single DCI multi-cell scheduling can be common between DL PDSCH scheduling case and UL PUSCH scheduling case. However, in terms of practical use-cases, number of scheduled cells for UL would be smaller than that for DL, considering UE’s DL CA and UL CA capabilities. Therefore, it may be possible to have different values for the maximum number of scheduled cells for DL and for UL.

  - DCI design

    - DCI design principle/framework

For the DCI which schedules PDSCH/PUSCH on multiple cells, it needs to be discussed how to achieve the DCI compression gain, i.e., which field(s) can be single value indication in DCI scheduling multiple scheduled cells.

For example, there should be some DCI fields e.g., DCI format identifier and carrier indication fields that are not necessary to be indicated separately for each scheduled cell. On the other hand, there should be some other DCI fields that require separate indication for each scheduled cell, e.g., new data indication, redundancy version and HARQ process number assuming that separate TB is scheduled for each cell.

In addition, there would be some DCI field(s) that can be common or separate for scheduled cells depending on scenarios and operation policy, e.g., relationship among scheduled cells. For example, some field(s) can be common across scheduled cells and a single value indication is enough in case that scheduled cells are within a same band, using same SCS and co-located, while it may be beneficial to have separate value indication of the field(s) for each scheduled cell in case that scheduled cells are in different bands, using different SCSs (if such case is supported) and/or non-co-located.

Furthermore, some field(s)/operation(s) may lead huge DCI overhead or complexity in case of multi-carrier scheduling and such operation may not be supported at all. For example, for Rel-17 multi-PDSCH/PUSCH scheduling, it was agreed that CBGTI and CBGFI fields are not present in the DCI for multi-PDSCHs scheduling. In addition, there are some DCI fields for specific features e.g., for multi-TRP operation, power saving enhancement, URLLC, etc., and it also needs to be discussed whether combinations between such specific features and multi-carrier scheduling are to be supported or not with considering DCI size impact and practical use case.

Based on the above, we propose to discuss based on the following four alternatives for each field of the DCI as DCI design discussion principle/framework.

Proposal 7: Discuss following alternatives for each field of the DCI scheduling multi-carrier PDSCH/PUSCH;

- Alt.1: indicate single value (applicable to all scheduled cells or single cell).

- Alt.2: indicate multiple values (each for each scheduled cell).

- Alt.3: configurable between Alt.1 and Alt.2.

- Alt.4: not support in the DCI scheduling multi-cell PDSCH/PUSCH.

Furthermore, as explained above, we also propose to agree that the following fields are indicated jointly or separately for each scheduled cell(s).

Proposal 8: The following DCI fields of a multi-carrier scheduling DCI should indicate single value;

- DCI format identifier

- Carrier indicator

Proposal 9: The following DCI fields of a multi-carrier scheduling DCI should indicate multiple values for each scheduled cell separately;

- New data indication

- Redundancy version

- HARQ process number

    - DCI design enhancements

It should be also discussed that which DCI format can be used for multi-carrier scheduling. One possible candidate is expansion of non-fallback DCI format (DCI format 0_1/1_1) for multi-carrier scheduling similar to the multi-PDSCHs/PUSCHs scheduling per cell introduced in Rel-16/17. This seems the most straightforward solution, however, it is also possible to consider the new DCI whose fields are optimized for multi-carrier scheduling. Thus, it should be discussed either the non-fallback DCI format or new DCI format can be applicable for multi-carrier scheduling. In addition to the DCI formats selected as a baseline for multi-carrier scheduling, applicability to the compact DCI format (DCI format 0_2/1_2) or configurability of the fields in the DCI can be further considered as it may be beneficial to achieve the DCI size compression.

Associated with the discussion of DCI fields and applicable DCI formats, it should also be discussed whether the current “3+1” DCI size budget should be maintained or can be enhanced. According to the current specification, a UE can monitor up to three different sizes of DCIs scrambled by C-RNTI and one DCI scrambled by RNTI other than C-RNTI. However, considering that the non-fallback DCI can be supported for single cell scheduling and the DCI size for single carrier scheduling may be different from that for multi-carrier scheduling, it may also need to be considered together whether “3+1” DCI size budget should be maintained and/or whether multi-cell scheduling DCI can also schedule single cell, to decide which DCI format (non-fallback or new format) is supported for multi-carrier scheduling.

Proposal 10: RAN1 should discuss the following aspects related to DCI design for multi-carrier PDSCH/PUSCH scheduling with a single DCI;

- whether multi-cell scheduling DCI is supported by non-fallback DCI (0_1/1_1) or by new DCI

- whether multi-cell scheduling DCI can also schedule single cell

- whether DCI size budget (3+1) is maintained

In our view, DCI field(s) such as carrier indicator field can be designed so that the multi-cell scheduling DCI can also schedule a single cell, and in such case the multi-cell scheduling DCI can be the expansion of non-fallback DCI format and the DCI size budget can be maintained.

  - HARQ related enhancements

Regarding the HARQ-ACK codebook construction for multi-carrier PDSCH scheduling, generally both type-1 and type-2 HARQ codebook can be applicable and may or may not require some clarifications/enhancements for each type of codebook. In addition to those codebooks, enhanced type-2, type-3 and enhanced type-3 codebook were introduced in Rel-16 and Rel-17. Especially for these codebooks, it may need to be discussed whether they are applicable or not for multi-carrier scheduling.

HARQ feedback timing also needs to be discussed for the multi-carrier scheduling. More specifically, it is unclear how to determine HARQ feedback timing if multiple PDSCHs in scheduled cells are not aligned in time domain. For example, assuming single PUCCH cell, single value indication for the HARQ feedback timing indicator field would be sufficient, but how to interpret the indicated value e.g., indicated k1 is an offset from which PDSCH to PUCCH, needs to be specified.

In addition, the relation between PUCCH cell group(s) and scheduled cells should be clarified, i.e., whether scheduled cells need to be within a same PUCCH cell group or can be across different PUCCH cell groups if UE supports two PUCCH cell groups.

Proposal 11: RAN1 should discuss the following aspects related to HARQ feedback for multi-carrier PDSCH scheduling with a single DCI;

- applicable HARQ-ACK codebook and required enhancements for each type of codebook if any

- HARQ feedback timing determination

- PUCCH cell group limitation

In our view,  regarding the applicable HARQ-ACK codebook types, enhanced type2 and 3 codebooks are also applicable for multi-cell PDSCH scheduling in general, while it may be necessary to discuss whether there is any issue in terms of DCI size increase if these codebook types are applicable to multi-cell PDSCH scheduling. Regarding HARQ feedback timing, we think the most straightforward way is that indicated k1 is interpreted as an offset between PUCCH and the PDSCH which ends at last in time domain (e.g., if different TDRAs for different PDSCHs can be indicated/configured), and it may be complicated if the case with different SCSs among different PDSCHs scheduled by the single DCI is to be supported. In addition, regarding the PUCCH cell group limitation, if scheduled cells are configured across multiple PUCCH cell group, the HARQ feedback indication would be more complicated, and hence it would be preferable to have a limitation that single DCI can schedule multiple cells that are in the same PUCCH cell group.

  - SS configuration/monitoring capability related enhancements

Regarding the search space configuration and monitoring capability, we think that there would be some discussion points. For example, it can be discussed whether a scheduled cell has only single scheduling cell with multi-cell scheduling DCI or can have multiple scheduling cells such as simultaneous configuration of multi-cell scheduling and self-carrier single cell scheduling, as relying only on multi-cell scheduling DCI for a scheduled cell may have some issues in terms of scheduling flexibility and performance.

Regarding the BD/CCE budget for each cell, at least the case that all the scheduled cells are configured with BD/CCE budget defined per slot should be supported. In addition, in Rel-16, it was supported that BD/CCE budget can be defined per span. Furthermore, if FR2-2 is included, BD/CCE budget must be defined per multi-slots for the cell. Accordingly, it can be discussed whether the cell which is configured with span or multi-slot PDCCH monitoring capability can be included in the scheduled cells.

Proposal 12: RAN1 should discuss the following aspects related to SS set configuration/monitoring capability and BD/CCE budget for multi-carrier PDSCH scheduling with a single DCI;

- applicable PDCCH monitoring capability

- BD/CCE budget for each cell

- limitation on scheduling cell(s) for a scheduled cell

  - Potential limitation on applicable scenario/operation

In section 2.1, we discuss the basic scenarios and assumptions which should be targeted in this WI. In addition to these scenarios, it can be discussed whether to support some specific scenarios and whether additional standardization effort is required for it. For example, whether FR2-2 can be supported for a scheduling cell and for scheduled cell(s). As discussed in 2.5, multi-slot based monitoring capability and higher SCSs for FR2-2 may require some additional standardization effort.

Another scenario which may need additional discussion is whether scheduling cell or scheduled cell can be a cell with shared spectrum operation. For this point, while it would be possible to support the cells with shared spectrum operation as scheduled cells for the multi-carrier scheduling, we can discuss two scenarios that may or may not require some additional standardization effort, 1) the cell with shared spectrum operation is scheduling cell and 2) scheduled cells include both cells with and without shared spectrum operation simultaneously.

In addition, it can be discussed whether SCell can be a scheduling cell for multi-carrier scheduling of multiple scheduled cells including P(S)Cell. In Rel-17, cross-carrier scheduling from SCell to P(S)Cell was supported, and it can be discussed whether this feature can be expanded for multi-carrier scheduling, e.g., PDCCH on SCell schedules PDSCH/PUSCH on P(S)Cell and SCell(s).

Based on the discussion above, we made the following proposal;

Proposal 13: For multi-carrier scheduling with a single DCI, the following scenarios need further investigation on additional specification impacts/standardization effort.

- whether FR2-2 can be supported for a scheduling cell and for scheduled cell(s)

- whether the cell with shared spectrum operation can be scheduling cell for the multi-carrier scheduling

- whether scheduled cells can include both cells with and without shared spectrum operation simultaneously

- whether SCell can be a scheduling cell for multi-carrier scheduling of multiple scheduled cells including P(S)Cell

- Conclusion

In this contribution, we discussed and its specification impacts. Based on the discussion, we made following observation and proposals.

Proposal 1: Multi-carrier PDSCH/PUSCH scheduling with a single DCI is not supported by DCI format 0_0/ and DCI format 1_0.

Proposal 2: The DCI for multi-carrier PDSCH/PUSCH scheduling is applicable to USS set only and not to any type of CSS set.

Proposal 3: Separate TB is scheduled for each cell by the single DCI scheduling multi-cell PDSCH/PUSCH.

Proposal 4: Either PDSCHs or PUSCHs on multiple cells are scheduled with a single DCI.

Proposal 5: Multi-cell PDSCH/PUSCH scheduling targets to support at least following scenarios;

- Scheduling cell and scheduled cells are within a same band or in different bands with same numerology

- FFS: different numerologies, with potentially updating WID to allow “multiple PDSCHs/PUSCHs per cell”

  - case 1: between scheduling and scheduled cells (same numerology across scheduled cells)

  - case 2: across scheduled cells

Observation 1: The maximum number of cells which can be scheduled simultaneously by a single DCI should be from 3 to 16.

Proposal 6: Discuss following alternatives for the target maximum number of scheduled cells;

- Alt.1: 8

- Alt.2: 6

- Alt.3: 4

Note1: Following aspects should be considered to specify the maximum number of scheduled cells: DCI size limitation, DCI compression gain by multi-carrier scheduling by single DCI, PDCCH blocking rate, practical use-case.

Note2: UE is configured with a certain number of potential scheduled cells (equal to or less than above value), and UE capability for maximum number of scheduled cells may be considered.

Proposal 7: Discuss following alternatives for each field of the DCI scheduling multi-carrier PDSCH/PUSCH;

- Alt.1: indicate single value (applicable to all scheduled cells or single cell).

- Alt.2: indicate multiple values (each for each scheduled cell).

- Alt.3: configurable between Alt.1 and Alt.2.

- Alt.4: not support in the DCI scheduling multi-cell PDSCH/PUSCH.

Proposal 8: The following DCI fields of a multi-carrier scheduling DCI should indicate single value;

- DCI format identifier

- Carrier indicator

Proposal 9: The following DCI fields of a multi-carrier scheduling DCI should indicate multiple values for each scheduled cell separately;

- New data indication

- Redundancy version

- HARQ process number

Proposal 10: RAN1 should discuss the following aspects related to DCI design for multi-carrier PDSCH/PUSCH scheduling with a single DCI;

- whether multi-cell scheduling DCI is supported by non-fallback DCI (0_1/1_1) or by new DCI

- whether multi-cell scheduling DCI can also schedule single cell

- whether DCI size budget (3+1) is maintained

Proposal 11: RAN1 should discuss the following aspects related to HARQ feedback for multi-carrier PDSCH scheduling with a single DCI;

- applicable HARQ-ACK codebook and required enhancements for each type of codebook if any

- HARQ feedback timing determination

- PUCCH cell group limitation

Proposal 12: RAN1 should discuss the following aspects related to SS set configuration/monitoring capability and BD/CCE budget for multi-carrier PDSCH scheduling with a single DCI;

- applicable PDCCH monitoring capability

- BD/CCE budget for each cell

- limitation on scheduling cell(s) for a scheduled cell

Proposal 13: For multi-carrier scheduling with a single DCI, the following scenarios need further investigation on additional specification impacts/standardization effort.

- whether FR2-2 can be supported for a scheduling cell and for scheduled cell(s)

- whether the cell with shared spectrum operation can be scheduling cell for the multi-carrier scheduling

- whether scheduled cells can include both cells with and without shared spectrum operation simultaneously

- whether SCell can be a scheduling cell for multi-carrier scheduling of multiple scheduled cells including P(S)Cell

References

[1]	RP-220834	Revised WID on Multi-carrier enhancements	NTT DOCOMO, INC.

[2]	RP-221008	Moderator’s summary of discussion [95e-39-R17-TEIs]	3GPP RAN WG4 Chair

[3]	R1-2204397	Work plan for Multi-carrier enhancements	NTT DOCOMO, INC.