3GPP TSG RAN WG1 #109-e			R1-2204400

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on XR specific power saving techniques

Agenda Item:	9.11.1

Document for: 	Discussion and Decision

- Introduction

At RAN#95-e meeting, revised SID on XR enhancements for NR was approved with the objective as follows [1].

Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements

In this contribution, we discuss XR specific power saving techniques.

- Discussion

  - CDRX enhancement

Since one of important factor for XR device is low power consumption, it should be beneficial to use CDRX for monitoring XR traffic. However, there are some issues on existing CDRX scheme with XR traffic considering latency requirement for XR service.

One issue is misalignment between XR traffic arrival interval and CDRX cycle as shown in Fig.1. For XR traffic arrival interval, non-integer value, e.g., 16.67ms, 8.33ms, is assumed. On the other hand, for existing CDRX cycle, limited integer values are defined, e.g., 10ms, 20ms. Thus, it would be difficult to ensure XR traffic arrives within DRX on duration.

Figure 1: Misalignment between XR traffic arrival interval and CDRX cycle

Another issue is that jitter on XR traffic has impact on UE power consumption or latency. If CDRX on duration is large in order to cover jitter, UE power consumption would be large. Alternatively, if CDRX on duration is small and XR traffic arrives before/after on duration, latency would be large.

Also, uplink transmission for XR might be potential issue. Uplink transmission for XR, e.g., pose/control information, might need to be transmitted frequently compared with downlink transmission for XR, and the active period for UL would be different from the active period for DL.

Proposal 1: Following potential issues should be considered for CDRX enhancement for XR:

- misalignment between XR traffic arrival interval and CDRX cycle

- jitter impact on XR traffic

- different periodicity between uplink transmission and downlink transmission for XR

In order to resolve these issues, one potential solution is semi-static/dynamic adaptation of CDRX on duration. For example, the start timing of CDRX cycle or on duration timer can be adapted so that XR traffic arrives within on duration. In addition, each CDRX cycle can have different on duration timer based on XR traffic amount or traffic type, e.g., uplink or downlink, for appropriate power consumption and latency.

In addition to above solution, it should be considered the necessity of new candidate values of CDRX cycle. The candidate values of existing long CDRX cycle are 10ms, 20ms…, and even 8ms, 9ms, 16ms and 17ms of CDRX cycle cannot be configured considering 8.33ms or 16.67ms of the XR traffic periodicity. Thus, it should be considered whether or not only semi-static/dynamic adaptation of CDRX on duration can sufficiently cover XR traffic based on the existing candidate values of CDRX cycle. Although non-integer values of CDRX cycle might have potential spec impact, integer value of CDRX cycle close with XR traffic periodicity can be considered with minimum spec impact.

Proposal 2: Study following potential CDRX enhancement schemes for XR:

- semi-static/dynamic adaptation of CDRX on duration

- new candidate values of CDRX cycle

  - PDCCH monitoring enhancement

In Rel-16 and Rel-17, the several power saving schemes for PDCCH monitoring enhancement were introduced. Basically, the existing schemes can cover wide use cases, e.g. SSSG switching, PDCCH skipping, WUS. However, XR-specific study for each scheme might not be sufficient, and any room for the enhancements should be studied. In addition, one important aspect is the relationship between PDCCH monitoring enhancement scheme and CDRX enhancement scheme. For example, it should be further studied whether PDCCH monitoring enhancement scheme can work independently or PDCCH monitoring enhancement scheme can work jointly with CDRX enhancement scheme.

Proposal 3: Study the necessity of enhancements on the existing schemes, e.g. SSSG switching, PDCCH skipping, WUS, taking into account the relationship between PDCCH monitoring enhancement scheme and CDRX enhancement scheme.

- Conclusion

In this contribution, XR specific power saving techniques was discussed. Based on the discussion, the following proposals were made:

Proposal 1: Following potential issues should be considered for CDRX enhancement for XR:

- misalignment between XR traffic arrival interval and CDRX cycle

- jitter impact on XR traffic

- different periodicity between uplink transmission and downlink transmission for XR

Proposal 2: Study following potential CDRX enhancement schemes for XR:

- semi-static/dynamic adaptation of CDRX on duration

- new candidate values of CDRX cycle

Proposal 3: Study the necessity of enhancements on the existing schemes, e.g. SSSG switching, PDCCH skipping, WUS, taking into account the relationship between PDCCH monitoring enhancement scheme and CDRX enhancement scheme.

References

- RP-220285, Revised SID: Study on XR Enhancements for NR, March 2022, Nokia