3GPP TSG RAN WG1 #109-e			       R1-2204401

e-Meeting, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on XR specific capacity improvement enhancements

Agenda Item:	9.11.2

Document for: 	Discussion and Decision

- Introduction

At RAN#95-e meeting, revised SID on XR enhancements for NR was approved with the objective as follows [1]:

Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements

In this contribution, possible enhancements for XR-specific capacity improvement are discussed.

- Discussions

Latency and data rate requirements are desired for XR services. Considering XR traffic is quasi-predictable, SPS/CG may be potential technique for XR. SPS and CG are beneficial for control overhead reduction, which can save more resources for data transmission. Moreover, CG is also beneficial for UL service latency reduction. In order to use SPS/CG more efficiently for XR services and improve XR capacity, following solutions can be considered.

Dynamic update of SPS/CG parameters

In R16, the granularity of SPS periodicity can be one slot, which means 1ms for 15kHz SCS, 0.5ms for 30kHz SCS, 0.25ms for 60kHz SCS, etc. The CG periodicity can be symbol level (e.g. 2 symbols, 4 symbols, 7 symbols) or slot level (e.g. 1 slot, 2 slots, 4 slots, etc). However, the SPS/CG periodicity may still mismatch XR traffic arrival periodicity, e.g. typical XR frame rates are 60, 120 frames per seconds (fps), of which frame periodicities are 16.67ms, 8.33ms. In such case, if a very small periodicity value is used, e.g. 1ms or 1slot SPS/CG periodicity, it may lead to resource wasting and/or heavy HARQ-ACK feedback overhead. On the other hand, if periodicity value around the XR service periodicity is used, e.g. 16ms or 17ms, large gap between XR traffic arrival and SPS/CG occasion may occur.

Fig 1: Example of mismatch of SPS/CG periodicity and XR traffic

Dynamic SPS/CG periodicity adjustment may be helpful for this case. For example, when the gap between XR traffic arrival and SPS/CG occasion is going to be larger than a certain value, the SPS/CG periodicity can be adjusted to align XR traffic arrival and SPS/CG occasion.

Fig 2: Example of SPS periodicity adjustment

Observation 1: Dynamic SPS/CG periodicity adjustment may be helpful for handling mismatch of XR traffic arrival and SPS/CG periodicity.

Another example of parameter adjustment is resource allocation of SPS/CG. For some XR services, e.g. AR UL traffic model with multiple streams, payload size in different time instances may be different. For such case, time and/or frequency resource allocation adjustment can be helpful.

Fig 3: Example of SPS/CG time/frequency domain resource allocation adjustment

Observation 2: Dynamic SPS/CG resource allocation adjustment may be helpful for handling variable packet size for XR.

Moreover, in some XR use cases, UE may move within an area, e.g. in a predefined/uncertain manner. It may lead to channel condition change or best Tx/Rx beam change. Update of MCS index, or TCI state/spatial relation information may be helpful.

Fig 4: Example of UE movement for XR service

Fig 5: Example of SPS/CG beam adjustment

Observation 3: Dynamic update of SPS/CG MCS/TCI state/spatial relation information may be helpful for UE movement for XR.

Proposal 1: Study dynamic update of SPS/CG parameters for XR, e.g., periodicity, resource allocation, MCS, or TCI state/spatial relation.

Multi-PDSCH/PUSCH in one SPS/CG periodicity

Compared to IIoT/URLLC service studied in Rel-15/16/17, XR service has larger packet size. It is possible that one PDSCH/PUSCH in one SPS/CG periodicity may be not enough to transmit one packet. With Rel-16 features, multiple SPS/CG configurations can be used for one packet. However, the supported number of SPS/CG configurations is limited. Moreover, multiple activation (or de-activation) DCIs are needed for multiple SPS/CG configurations. Therefore, multiple SPS PDSCHs / CG PUSCHs in one periodicity is useful for XR service with large payload size. On the other hand, one DCI scheduling multiple PDSCHs/PUSCHs has already been supported in Rel-17 for FR2-2. It can be a starting point for studying multiple SPS PDSCHs / CG PUSCHs in one periodicity, and the workload is expected to be not large.

Observation 4: It is possible that one PDSCH/PUSCH in one SPS/CG periodicity may be not enough to transmit one packet for XR. Using multiple SPS/CG configurations for one packet is not efficient.

Proposal 2: Study multiple SPS PDSCHs or CG PUSCHs in one SPS/CG periodicity for XR.

- Conclusion

In this contribution, we discussed possible solutions for XR capacity improvement. We have following observations:

Observation 1: Dynamic SPS/CG periodicity adjustment may be helpful for handling mismatch of XR traffic arrival and SPS/CG periodicity.

Observation 2: Dynamic SPS/CG resource allocation adjustment may be helpful for handling variable packet size for XR.

Observation 3: Dynamic update of SPS/CG MCS/TCI state/spatial relation information may be helpful for UE movement for XR.

Observation 4: It is possible that one PDSCH/PUSCH in one SPS/CG periodicity may be not enough to transmit one packet for XR. Using multiple SPS/CG configurations for one packet is not efficient.

Based on the observations, we have following proposals:

Proposal 1: Study dynamic update of SPS/CG parameters for XR, e.g., periodicity, resource allocation, MCS, or TCI state/spatial relation.

Proposal 2: Study multiple SPS PDSCHs or CG PUSCHs in one SPS/CG periodicity for XR.

References

- RP-220285, Revised SID: Study on XR Enhancements for NR, March 2022, Nokia