3GPP TSG-RAN WG1 #109-e			R1-2204402

e-Meeting, May 9 – 20, 2022

Agenda item:	9.12.1

Title:	Discussions on coverage enhancement for NR NTN

Source:	NTT DOCOMO, INC.

Document for:	Discussion/Decision

- Introduction

In RAN#94-e a new work item on NR NTN enhancements [1][2]. One of the objectives is to evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN. In this contribution, we present our views on coverage enhancement for NTN.

- Discussion

  - Preparation for simulation evaluation on NR NTN coverage enhancement

In the objectives of the WI the following assumptions are included.

- GSO and NGSO (LEO and MEO) with transparent payload.

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS capabilities

- Both “VSAT” devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1

- Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands.

- Scenario

The reference scenarios in TR 38.821 are listed in the following table.

Table 1 Reference scenarios in TR 38.821

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

Since transparent satellites are assumed in this WI, scenario A, C1 and C2 can be used as a baseline. Increasing scenarios will lead to an increase in evaluation workload. The number of scenarios should not be unnecessarily increased. Scenarios should be selected in consideration of actual deployments.

Proposal 1: Use scenario A, C1 and C2 in TR 38.821 as a baseline.

Proposal 2: Evaluation scenarios should be selected in consideration of actual deployments.

- Simulation assumption for SLS

For system level simulations, simulation assumptions in TR 38.821 can be used as a baseline. In the simulation assumption, the antenna gain of handheld UE is 0dBi. In Rel-18 preparation phase, some companies claimed that the actual smartphone’s antenna gain was around -5dBi. At least the antenna gain of handheld UE would need to be updated to a more realistic value.

Proposal 3: Use simulation assumptions in TR 38.821 with necessary updates (e.g. antenna gain of handheld UE).

- Evaluation methodology

It would be better to obtain the required SINR for each physical channel under the target scenario and compare with the SINR obtained from the link budget calculation to evaluate the coverage that can meet required performance. The list of physical channels in TR 38.830 can be used as a baseline for the physical channels to be evaluated. In other words, the following channels should be used as a baseline. Considering the difference in transmit power between UE and satellite, uplink channels would be a potential bottleneck. Uplink channels should be prioritized.

- PUSCH for eMBB (FDD)

- PUSCH for VoIP (FDD)

- PUCCH Format 1 with 2bits

- PUCCH Format 3 with 11bits

- PUCCH Format 3 with 22bits

- SSB

- PRACH format 0

- PRACH format B4

- Broadcast PDCCH (PDCCH of Msg.2)

- PDSCH for Msg.2

- PUSCH of Msg.3

- PDSCH of Msg.4

- Unicast PDCCH

- PDSCH for eMBB (FDD)

- PUCCH with HARQ-ACK for Msg.4

- PUSCH with SIP invite

- PUSCH for CSI 11bit

- PUSCH for CSI 22bit

For the simulation of physical channels, simulation assumptions for FDD in TR 38.830 can be used with necessary updates. At least the channel model should be replaced by the channel model specified in TR 38.811. Also, features for coverage enhancement introduced in Rel-17 (e.g., TBoMS, Joint channel estimation) should be fully utilized.

Proposal 4: For coverage evaluation, obtain the required SINR for each physical channel under the target scenario and compare with the SINR obtained from the link budget calculation.

- Use the following physical channels as a baseline for coverage evaluation.

  - PUSCH for eMBB (FDD)

  - PUSCH for VoIP (FDD)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (FDD)

  - PUCCH with HARQ-ACK for Msg.4

  - PUSCH with SIP invite

  - PUSCH for CSI 11bit

  - PUSCH for CSI 22bit

- Use the simulation assumptions in TR 38.830 with necessary updates.

Proposal 5: For NR NTN coverage evaluation, utilize the features introduced in Rel-17 coverage enhancement.

  - Preliminary simulation results

We performed preliminary simulations to evaluate the uplink coverage in NTN in S-band when the antenna gain of UE is -5dBi.

    - PUSCH performance

We simulated the geometry UL SINR in GEO and LEO-1200 scenarios. For the simulation, we set the antenna gain of UE to -5dBi. The parameters other than the antenna gain were the same as the simulation assumptions (i.e., Table 6.1.1.1-1 to 6.1.1.1-5) in TR 38.821. Set-1 satellite parameters were used and frequency re-use factor was set to 1. The channel bandwidth was set to 1.44 MHz. Figure 1 shows the simulation results of PUSCH SINR.

Figure 1 CDF of PUSCH SINR (UE antenna gain is -5dBi)

We performed link level simulations of PUSCH for VoIP. As simulation parameters, we used the parameters for TDD in the simulation assumptions for PUSCH for FR1 (i.e., Table A.1-1 and A.1-2) in TR 38.830. dmrs-AdditionalPosition was set to ‘pos3’. The following five cases were evaluated to see performance with the features introduced in Rel-17 coverage enhancement.

Table 2 Cases for PUSCH performance evaluation

|  | Number of repetitions | Number of slots for a TBoMS | Joint channel estimation |
| --- | --- | --- | --- |
| Case 1 | 8 | N/A | Disabled |
| Case 2 | 32 | N/A | Disabled |
| Case 3 | 4 | 8 | Disabled |
| Case 4 | 32 | N/A | Enabled |
| Case 5 | 4 | 8 | Enabled |

Figure 2 shows the simulation results of PUSCH BLER.

Figure 2 Simulation results of PUSCH BLER

The SINR required to achieve 2% BLER is as follows:

- Rep.8: -15.3 dB

- Rep.32: -18.7 dB

- TBoMS 8, Rep. 4: -18.7 dB

- Rep. 32, JCE: -19.4 dB

- TBoMS 8, Rep. 4, JCE: -19.4 dB

Combined with the SINR simulation results, the following observations were obtained:

- 25% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.8

- 91% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.32

- 91% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with TBoMS 8, Rep.4

- 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rep.32, JCE

- 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with TBoMS 8, Rep.4, JCE

Observation 1: Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rel-17 features.

    - PUCCH performance

We simulated PUCCH SINR as well as the PUSCH case. The channel bandwidth was set to 0.36 MHz. Figure 3 shows the simulation results of PUCCH SINR.

Figure 3 CDF of PUCCH SINR (UE antenna gain is -5dBi)

We performed link level simulations of PUCCH format 1. As simulation parameters, we used the parameters in the simulation assumptions for PUCCH for FR1 (i.e., Table A.1-1 and A.1-3) in TR 38.830. The following three cases were evaluated to see performance with the features introduced in Rel-17 coverage enhancement.

Table 3 Cases for PUCCH performance evaluation

|  | Number of repetitions | Joint channel estimation |
| --- | --- | --- |
| Case 1 | 4 | Disabled |
| Case 2 | 8 | Disabled |
| Case 3 | 8 | Enabled |

Figure 4 shows the simulation results of PUCCH error probability.

Figure 4 Simulation results of PUCCH error probability

The SINR required to achieve NACK to ACK probability of 0.1% is as follows:

- Rep. 4: -11.0dB

- Rep. 8: -13.5dB

- Rep. 8 & JCE 4: -14.4dB

Also, the SINR required to achieve ACK missed detection probability of 1% is as follows:

- Rep. 4: -13.3dB

- Rep. 8: -15.5dB

- Rep. 8 & JCE 4: -16.2dB

Combined with the SINR simulation results, the following observations were obtained:

- 55% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep4

- 95% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep8

- 97% GEO/ 100% LEO-1200 users can meet the required NACK to ACK probability of PUCCH format 1 with Rep8 & JCE4

- 93% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep4

- 99% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep8

- 99% GEO/ 100% LEO-1200 users can meet the required ACK missed detection probability of PUCCH format 1 with Rep8 & JCE4

Observation 2: Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 97% GEO/ 100% LEO-1200 users can meet the required error probability of PUCCH format 1 with Rel-17 features.

- Conclusion

In this contribution, we have the following observations and proposals:

Proposal 1: Use scenario A, C1 and C2 in TR 38.821 as a baseline.

Proposal 2: Evaluation scenarios should be selected in consideration of actual deployments.

Proposal 3: Use simulation assumptions in TR 38.821 with necessary updates (e.g. antenna gain of handheld UE).

Proposal 4: For coverage evaluation, obtain the required SINR for each physical channel under the target scenario and compare with the SINR obtained from the link budget calculation.

- Use the following physical channels as a baseline for coverage evaluation.

  - PUSCH for eMBB (FDD)

  - PUSCH for VoIP (FDD)

  - PUCCH Format 1 with 2bits

  - PUCCH Format 3 with 11bits

  - PUCCH Format 3 with 22bits

  - SSB

  - PRACH format 0

  - PRACH format B4

  - Broadcast PDCCH (PDCCH of Msg.2)

  - PDSCH for Msg.2

  - PUSCH of Msg.3

  - PDSCH of Msg.4

  - Unicast PDCCH

  - PDSCH for eMBB (FDD)

  - PUCCH with HARQ-ACK for Msg.4

  - PUSCH with SIP invite

  - PUSCH for CSI 11bit

  - PUSCH for CSI 22bit

- Use the simulation assumptions in TR 38.830 with necessary updates.

Proposal 5: For NR NTN coverage evaluation, utilize the features introduced in Rel-17 coverage enhancement.

Observation 1: Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 96% GEO/ 100% LEO-1200 users can meet the required BLER of PUSCH-VoIP with Rel-17 features.

Observation 2: Even when the antenna gain of UE is -5 dBi, in NTN in S-band, 97% GEO/ 100% LEO-1200 users can meet the required error probability of PUCCH format 1 with Rel-17 features.

- References

- RP-220842, Meeting Report for TSG RAN meeting #94e

- RP-213690, New WI: NR NTN (Non-Terrestrial Networks) enhancements

- TR 38.821 V16.1.0, Solutions for NR to support non-terrestrial networks (NTN)

- TR 38.830 V17.0.0, Study on NR coverage enhancements

- TR 38.811 V15.4.0, Study on New Radio (NR) to support non-terrestrial networks