3GPP TSG RAN WG1 #109-e								R1-2204404

e-Meeting, May 9th –20th, 2022

Agenda item:	8.16.6

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for REDCAP

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.6 regarding UE features for RedCap and captures the following email discussion.

[109-e-R17-UE-features-RedCap-01] Email discussion on UE features for RedCap – Shinya (DOCOMO) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e [1], there are following feature groups for RedCap.

- 28-1	RedCap UE

- 28-3	Half-duplex FDD operation type A for RedCap UE

The issues to be discussed are tagged and colour coded with High priority or Low priority based on ASN.1 impact.

- 28-1: RedCap UE

In [1], FG 28-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included FFS whether to add any other basic features for RedCap UE |  | Yes |  | [Network assumes the UE is not a RedCap UE] | [Per UE] | [No] | [No] |  | [RedCap UEs do not support carrier aggregation or dual connectivity.] It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A | Optional with capability signaling RedCap UE must indicate this FG is supported |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | ## 2.1 CORESET number For NR Rel-15/16 UE, the basic FG 3-1 for DL control channel is support of “one configured CORESET per BWP per cell in addition to CORESET0” [1]. So gNB can only configure one CORESET with ControlResourceSetId other than 0 in a BWP (initial/non-initial BWP), no matter if the BWP contains the bandwidth of CORESET0 or not. If RedCap UEs follow the same rule, only one CORESET can be used for both common and dedicated search spaces, when the BWP does not contain the bandwidth of CORESET0, which could significantly limit the flexibility of network scheduling, or even cause congestion problem compared with two separate CORESETs in legacy operation. Thus, we propose Proposal 1: A RedCap UE should support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0. ## 2.2 NCD-SSB In TR 38.822, FG 6-1 and FG 6-1a are described as following: 6-1; Basic BWP operation with restriction; 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. 6-1a; BWP operation without restriction on BW of BWP(s); BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell. FL6 Since the basic FG 6-1 also addresses CORESET#0, it is not preferable to use an existing FG with different interpretation and there are other related aspects associated with NCD-SSB, including number of SSBs, time domain periodicity and location, clarification of measurements etc.. Therefore for RedCap UEs, we suggest to introduce a new FG to report whether the UE expect NCD-SSB in the active BWP, e.g. to perform RRM/RLM/beam measurement of the serving cell. It is mandatory capability with signalling bit for proper test purpose. Proposal 2: Define a new FG for a RedCap UE to report whether it expects NCD-SSB or not in the active BWP, and no more than one SSB within one active BWP, the periodicity of NCD-SSB is not expected to be shorter than CD-SSB, support of a configurable time-domain offset from CD-SSB, and support of measurements based on NCD-SSB including RRM, RLM and BM. |
| --- | --- | --- |
| [3] | Ericsson | Some additional FG 28-1 components that were discussed in RAN1#108-e were considered to be pending further discussion under another agenda item (8.6.1.1). We propose that these components are added in order to clarify that these components are needed as mandatory features to ensure the intended operation for all RedCap UEs. Note the following relevant RAN1#108-e agreement [7]: Agreement: - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer. Add FG 28-1 (‘RedCap UE’) components according to the table below. 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included 6. RRC-configured DL BWP with CD-SSB or NCD-SSB 7. NCD-SSB based measurements in RRC-configured DL BWP. There are square brackets in some of the fields for FG 28-1. After the RAN#95-e decision that FG 28-1 should be reported per UE, we think that all these square brackets can be removed and text within them kept as is. Remove all square brackets (but keep the text within them) on the FG 28-1 row. For FG 28-1, the “Note” field indicates that “It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A”. Considering the following RAN2#117-e agreements, early indication in Msg3 and MsgA could also be added as components. - Dedicated LCID for RedCap is always indicated when CCCH is sent in MsgA by a RedCap UE (i.e. no other precondition). […] - The working assumption that Msg3 early identification is mandatorily supported by RedCap UE is confirmed. Add ‘Early indication of RedCap UE in Msg3 for 4-step RACH’ as an FG 28-1 component. Clarify in the feature list that ‘Early indication of RedCap UE in MsgA for 2-step RACH’ is supported for UEs that support FG 28-1 and 2-step RACH. |
| [4] | vivo, Guangdong Genius | For the 1st issue, based on the progress achieved so for AI 8.6.1, for the RedCap UE in the RRC_idle/inactive mode, when the separate initial DL BWP is used for paging, CD-SSB should be included. However, for the RedCap UE in the RRC_connected mode, CD-SSB is not needed if the RedCap UE indicating supporting the FG similar as FG 6-1a; or NCD-SSB can be included instead of CD-SSB in the separate initial DL BWP if the RedCap UE does not support the FG6-1a like feature. In addition, if BWP#0 configuration option 1 is agreed to be supported to configure RedCap UE with the separate initial DL BWP, then for the RedCap UE, regardless of its mode i.e., idle/inactive or connected, when the separate initial DL BWP is used for paging, CD-SSB should be included. From our understanding, the component of FG28-1 should be the basic and mandatory feature for RedCap UE, hence for paging monitoring in the separate initial DL BWP, SSB is always required. Therefore, this component can be described as for separate initial DL BWP used for paging, SSB is included. Alternatively, it can also be decided on how to capture it once the decision related to BWP#0 configuration option 1 is made in AI 8.6.1. Proposal 1: Following component can be added into FG28-1. - For separate initial DL BWP used for paging, SSB is included For the FG related to UE-specific BWP for RedCap UEs, there were some discussions, but no agreement was made yet. Based on the agreements made in AI 8.6.1, see below: Agreement - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer Considering forward compatibility, following is proposed: Proposal 2: Following “Basic BWP operation for RedCap UE” should be captured as a separate FG (one example is given in FG 28-x): - Basic BWP operation with restriction for RedCap UE - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - BW of a UE-specific RRC configured BWP includes SSB for PCell - BW of a UE-specific RRC configured BWP may not include BW of MIB-configured CORESET#0 - Support NCD-SSB and support existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including measurements based on NCD-SSB) in an RRC-configured DL BWP that does not include CD-SSB - NCD-SSB is configured by higher layer - NCD-SSB is QCL-ed with CD-SSB when the NCD-SSB and CD-SSB have the same SSB index Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 28-x; BWP operation with restriction; - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - BW of a UE-specific RRC configured BWP includes SSB for PCell - BW of a UE-specific RRC configured BWP may not include BW of MIB-configured CORESET#0 - Support existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including measurements based on NCD-SSB) in an RRC-configured DL BWP that does not include CD-SSB - NCD-SSB is configured by higher layer - NCD-SSB is QCL-ed with CD-SSB when the NCD-SSB and CD-SSB have the same SSB index; FG28-1; Yes; Impact on UE complexity; Per UE; No; No; Optional with capability signaling. In addition, in the last RAN1 meeting, there was discussion on for RedCap UE operating in the BWP without SSB, and following conclusion was made. Related LS [3] was also sent to RAN4. Therefore, there is no need for RAN1 to continue the discussion on how to capture such optional FG based on FG6-1a like FG and/or combination of FG6-1a like FG and CSI-RS. Conclusion: - From RAN1 perspective, whether and under what conditions a RedCap UE requires to be configured with existing measurement gaps to support operation without SSB in an RRC-configured active BWP, and its related UE feature discussion (including measurement gaps) is up to RAN4. - Send an LS to RAN4 to inform them about the conclusion.. It was agreed that FG 28-1 is defined per UE. At least for FR1/FR2 differentiation, it may be desirable to have a differentiation. So, we are open to consider such differentiation. Proposal 3: For FG28-1, there is no need for FDD/TDD differentiation. - FFS on the need of FR1/FR2 differentiation |
| [5] | ZTE, Sanechips | For 28-1, the following yellow highlighted are still in the discussion. Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note. Per UE; No; No; RedCap UEs do not support carrier aggregation or dual connectivity.. Both TDD band and FDD band are supported for RedCap, the necessity to differentiate FDD/TDD is not foreseen. If only FDD is supported, the TDD bands would be wasted for RedCap. Moreover, the reporting bits for FDD/TDD differentiation would be increased. Therefore, it is nature to assume ‘NO’ in that column in the table. As for the need of FR1/FR2 differentiation, according to the WID description, Generic requirements: - Device complexity: Main motivation for the new device type is to lower the device cost and complexity as compared to high-end eMBB and URLLC devices of Rel-15/Rel-16. This is especially the case for industrial sensors. - Device size: Requirement for most use cases is that the standard enables a device design with compact form factor. - Deployment scenarios: System should support all FR1/FR2 bands for FDD and TDD.. RedCap is expected to support all the FR1/FR2 bands. Therefore, it is not needed to differentiate FR1/FR2. Moreover, for that ‘Note’ column, the current text is appropriate to describe the characters of RedCap UE. Therefore, the current table for the yellow highlight part is fine. Proposal 2: For FG 28-1, - FDD/TDD differentiation can be set as ‘NO’ - FR1/FR2 differentiation can be set as ‘NO’ - The Note column can be kept as above table. Regarding the usage of NCD-SSB, it has been discussed whether a new FG should be defined or the existing 6-1/6-1a should be reused. It is noted that, FG 6-2, 6-3 and 6-4 are based on FG6-1, and FG 6-1a is based on FG6-1, 6-2, 6-3, or 6-4. If a new FG is defined, the support of FG 6-2, 6-3 and 6-4 for RedCap also should be modified for RedCap correspondingly. If we consider to reuse the existing features with modification, ‘Mandatory without capability signalling’ is also not appropriate for RedCap UE since FG 28-1 is still optional. And the description for SCell is also not needed since RedCap UE does not support SCell. Moreover, if a RedCap UE support FG 6-1, 6-1a, 6-2, 6-3 and 6-4, the FG-28-1 should be included in the prerequisite feature groups. If we consider to add the component to complete the NCD-SSB, however, it is not appropriate to support 6-1a, 6-2, 6-3 and 6-4 in the component column, since they are optional. Therefore, for the usage of NCD-SSB, defining a new FG is more straight and simple. Proposal 4: Regarding the usage of NCD-SSB, new FGs corresponding to FG 6-1, 6-1a, 6-2, 6-3 and 6-4 can be defined. |
| [6] | Samsung | The following agreements were made in RAN 1 #108: Agreement A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer We suggest to add “UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB in an RRC-configured DL BWP that does not include CD-SSB” as description for Separate initial DL BWP for RedCap UEs. Moreover, for the FFS for paging, we think the FFS can be removed. The following was captured in TS 38.306 v12.0.0 supportOfRedCap-r17 Indicates that the UE is a RedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Maximum FR2 RedCap UE bandwidth is 100 MHz; - Support of RedCap early indication based on Msg1, MsgA and Msg3 for random access; A RedCap UE shall always set the capability to “1”. We suggest to align with RAN 2 spec on the description of early indication for Redcap. Proposal #1: Update the component of FG 28-1 as following: 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH Msg1, MsgA and Msg3 for random access. 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB in an RRC-configured DL BWP that does not include CD-SSB FFS whether to add any other basic features for RedCap UE |
| [7] | OPPO | In our view, the necessary RedCap capabilities are all included. For the additional DL BWP, it may require SSB for paging purpose as agreed in RedCap session. We consider to include the bullet that CD-SSB is included for separated initial DL BWP for RedCap Ues. It was also agreed that RedCap UEs do not support carrier aggregation or dual connectivity. However, we see no need to indicate “No CA” in the FG 28-1 description or in the note for specification. - We proposed that the RedCap UE is always assumed to be Half duplex capability in the initial access. This is to avoid the misunderstanding of usable slot between gNB and UE. However, the updated specifications is to make all UE including HD RedCap UE access all slots and may skip other DL channels overlapped. We are fine to capture this in 38.213. Thus, 28-1 don’t need any HD-FDD assumption. - For the need of FDD/TDD differentiation, it seems the basic RedCap capability should not be further forked for duplex mode. The RF front end would be similar except for the duplexer part. Need of FR1/FR2 differentiation can be further discussed. The bandwidth of 20MHz and 100MHz are different. We can consider if the UE can indicate support them separately. - Proposal 1: For the need of FDD/TDD differentiation, 28-1 set as “no”. 28-1 set for FR1/FR2 separately. - For separate initial DL BWP used for paging, CD-SSB is included. |
| [8] | CMCC | According to the following agreement of RAN1#108e, there is one FFS for separate initial DL BWP, which is highlighted below. Agreement Following capabilities are added as components in FG 28-1 - Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included There are three cases for paging reception on separate initial DL BWP, - Case 1: Separate initial DL BWP in idle/inactive mode. For this case, if it is configured for paging, CD-SSB is included. - Case 2: Separate initial DL BWP with BWP#0 configuration option 1 in connected mode, paging can only be configured if it contains CD-SSB (and CORESET#0 for FR1). - Case 3: Separate initial DL BWP with BWP#0 configuration option 2 in connected mode, if is configured for paging, CD-SSB/NCD-SSB is included for UE supporting mandatory FG 6-1 but not optional FG 6-1a. The reference agreements is copied as following. For components added to FG28-1 separate initial DL BWP, we understand it is for case 1 and case 2, since there will be dedicated UE feature group for UE-specific RRC configured BWP, either by reuse FG6-1/6-1a or define new features. According to TS38.331 B.2, BWP#0 configuration option 2 is considered as RRC-configured BWP, so it is reasonable to consider case 3 separately. Agreement - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB So we prefer to modify the FFS to the following, - Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included. - For separate initial DL BWP only used for RACH, SSB may or may not be included. Proposal 1: Include the following part to separate initial DL BWP component of FG 28-1， - For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included. - SSB related UE capability During the RAN1#108e, the following agreements are made, Agreement Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement: For FR1, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a]without supporting CSI-RS. For FR2, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. Note: The cases that CSI-RS in this agreement can support are left to RAN4 According to the agreements, there are two kinds of UE capabilities for RedCap UEs when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. One is that RedCap UEs doesn’t support optional FG6-1a, this kind of RedCap UE will expect the RRC configured active DL BWP to contain NCD-SSB. The other one is that RedCap support capability that NCD-SSB is not needed on the RRC configured active DL BWP. For the first kind of capability, there are two options to define the UE feature, - Alt.1: No separate new FG is introduced, while add a note in FG6-1(Basic BWP operation with restriction) that “For RedCap UEs, BW of a UE-specific RRC configured BWP may or may not include BW of CD-SSB and the entire CORESET#0, if not, BW of the UE-specific RRC configured BWP includes NCD-SSB. - Alt.2: A new FG 28-x1 is introduced, with the following component, - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. Alt.1 introduces one case that BW of a UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0 into the traditional FG6-1, this is different from what legacy FG6-1 mentions. And there is a potential risk for Alt.1 that when R18 RedCap with 5MHz is introduced, will the mandatory feature be modified again? If so, the legacy UE feature 6-1 is better to keep as it is since R17. Alt.2 has no impact on legacy FG, and defining a separate FG keeps the possibility that the feature can be used by non-RedCap UEs in the future. Comparing these two options, we prefer Alt.2. The prerequisite feature group of this new feature can be FG 6-1, and since this is the default capability, RedCap UEs have to report this capability. Proposal 2: Define a new FG28-x1 for the capability of UE expect NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 3: The new FG28-x1 includes the following component, and the prerequisite feature group of this is FG 6-1, 6-2,6-3 or 6-4 - BW of UE-specific RRC configured BWP may not include BW of CD-SSB and the entire CORESET#0, in this case, BW of the UE-specific RRC configured BWP includes NCD-SSB. For the second kind of capability, RAN4 has made the following conclusion during RAN4#101b [5], For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0): - A RedCap UE that supports FG 6-1a but NOT support CSI-RS based L3 measurement operates in the BWP - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - the UE can support SSB based L3 measurement, but cannot support CSI-RS based L3 measurement. - A RedCap UE that supports FG 6-1a and CSI-RS based L3 measurement operates in the BWP - the UE can support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - the UE can support both SSB based L3 measurement and CSI-RS based L3 measurement with associated SSB. - RAN4 will not define CSI-RS L3 based measurement requirements for Redcap 1RX UE in Rel-17. - For serving cell timing related requirements, RAN4 will not define requirements based on CSI-RS in Rel-17. Based on the above conclusion, RedCap UE can perform RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS and L3 measurement based on CSI-RS with associated SSB if UE supports the corresponding capabilities. If the corresponding capabilities are not supported, UE may rely on re-tuning to CD-SSB for such measurement,which may be already supported as FG6-1a. So we propose to introduce a new FG 28-x2, to support this optional capability, which include the following components, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - support measurement operation based on FG6-1a. - support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. And the prerequisite feature group of this new feature can be FG 6-1a. Proposal 4: Define a new feature FG28-x2 for the capability of supporting operation without NCD-SSB when an RRC-configured active DL BWP in connected mode does not include CD-SSB and the entire CORESET#0. Proposal 5: The new FG28-x2 includes the following components, and the prerequisite feature group of this is FG 6-1a, - BW of UE-specific RRC configured BWP may not include BW of SSB(including CD-SSB or NCD-SSB) and the entire CORESET#0. - support measurement operation based on FG6-1a. - support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. |
| [9] | DOCOMO | - FG 28-1: RedCap UE - Based on the agreement in RAN#95e, no further discussion is necessary in RAN1 regarding the reporting type, i.e., FG 28-1 is reported per UE w/o FDD/TDD or FR1/FR2 differentiation. - The column of components can be updated based on the following agreement in RAN1#108-e: - 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging in connected mode, - BWP#0 configuration option 1: CD-SSB is included - BWP#0 configuration option 2: SSB may or may not be included depending on the support of FG 6-1a - For separate initial DL BWP only used for RACH, SSB may or may not be included - FFS whether to add any other basic features for RedCap UE Agreement - The following working assumptions from RAN1#107-e are NOT confirmed for idle/inactive mode and furthermore they are replaced by the agreements further down for connected mode. - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB. - The column of “Consequence if the feature is not supported by the UE” can be confirmed. - The Note “RedCap UEs do not support carrier aggregation or dual connectivity.” can be kept. - New FG for RRC-configured active DL BWP without CD-SSB or NCD-SSB - Based on the following agreements in RAN1#108-e, RRC-configured active DL BWP without CD-SSB or NCD-SSB (with or without supporting CSI-RS) is optional capability. We are fine with either defining a new FG for RedCap or reusing existing FG 6-1a (BWP operation without restriction on BW of BWP(s)). Agreement Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement: For FR1, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. For FR2, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. Note: The cases that CSI-RS in this agreement can support are left to RAN4 Agreement A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer Conclusion - From RAN1 perspective, whether and under what conditions a RedCap UE requires to be configured with existing measurement gaps to support operation without SSB in an RRC-configured active BWP, and its related UE feature discussion (including measurement gaps) is up to RAN4. - Send an LS to RAN4 to inform them about the conclusion.. - Report type of this FG should be per UE with FR1/FR2 differentiation or per band for more flexibility to indicate the support of this BWP operation for different FR. |
| [10] | NEC | The first part of the proposal is to capture the following RAN1 agreement in RAN1#108-e into FG28-1. Agreement: - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer It will be fine to include the main bullet point of this agreement into FG28-1. Though the main bullet point might be vague, including a complete list of the mandatory feature(s) based-on NCD-SSB would not be agreeable. The two sub-bullets would not need to be captured as such details of NCD-SSB would be captured by appropriate specifications. Regarding the second part, removing FFS from the second sub-bullet was suggested by moderator of agenda 8.6.1.1.  Component 5 in FG 28-1 is updated as follows  5. Separate initial DL BWP for RedCap UEs  It includes CSS/CORESET for random access  FFS: For separate initial DL BWP used for paging, CD-SSB is included  For separate initial DL BWP only used for RACH, SSB may or may not be included RAN1 made the following agreements in RAN1#108-e, CD-SSB needs to be included in a separate initial DL BWP in connected for BWP#0 configuration option 1, while CD- or NCD-SSB needs to be included for BWP#0 configuration option 2. Therefore, the statement “CD-SSB is included” is incorrect and need update. The following agreement could be described, for example, “For separate initial DL BWP used for paging, CD-SSB or CD-/NCD-SSB is included for BWP#0 configuration option 1 or BWP#0 configuration option2, respectively” but it looks a bit lengthy. Maybe, “proper SSB is included” or “CD-/NCD-SSB is included” would be enough as details would be described in specifications. Agreement: - The following working assumptions from RAN1#107-e are NOT confirmed for idle/inactive mode and furthermore they are replaced by the agreements further down for connected mode. - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB According to the discussion so far, the following proposal is made: Proposal: - Following updates highlighted in blue are proposed to the proposal in RAN1#108-e Proposal  The following capabilities are added as components in FG 28-1: - Support existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including measurements based on NCD-SSB) in an RRC-configured DL BWP that does not include CD-SSB - NCD-SSB is configured by higher layer - NCD-SSB is QCL-ed with CD-SSB when the NCD-SSB and CD-SSB have the same SSB index  Component 5 in FG 28-1 is updated as follows  5. Separate initial DL BWP for RedCap UEs  It includes CSS/CORESET for random access  FFS: For separate initial DL BWP used for paging, proper SSB CD-SSB is included  For separate initial DL BWP only used for RACH, SSB may or may not be included |
| [11] | Nokia/NSB | - Regarding discussion on listing other basic features for RedCap UE, we do not have a strong opinion on extra components to be added, but it should be clear that no further FGs should be added for Rel-17 RedCap. - Confirm consequence if feature is not supported. |
| [12] | ModiaTek | FG 28-1: For the FFS point under component 5, we suggest change it as follows. By saying only SSB, it can include the case when paging is configured in connected mode on a BWP configured with NCD-SSB. 5. Separate initial DL BWP for RedCap Ues - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included Proposal 1: For FG28-1, change “CD-SSB” to “SSB” for the second sub-bullet under component 5. Then confirm the rest as it is. |
| [13] | Intel | On the potential addition of the detail regarding UE expectation on CD-SSB presence for paging monitoring in separate initial DL BWP, while not strictly necessary, it can be seen as a salient characteristic of Rel-17 RedCap UEs, at the same level as the already-agreed component “For separate initial DL BWP only used for RACH, SSB may or may not be included”. Thus, it would be reasonable to also capture this detail for consistent handling vis-à-vis separate initial DL BWP for RACH-related DL reception. For the remaining outstanding aspects, we think that all the brackets can be removed, the contents inside the brackets confirmed, and the FFS on whether to add any other basic feature for RedCap UE removed. Proposal 1: - Confirm the following as a component for FG #28-1: - For separate initial DL BWP used for paging, CD-SSB is included - Remove the brackets and confirm the current descriptions for remaining details for FG #28-1. - Remove the following FFS component: - FFS whether to add any other basic features for RedCap UE. It is necessary to update FG #6-1 for Rel-17 RedCap UEs. In particular, for RedCap UEs, operation without CORESET #0 within the active DL BWP should be mandatory and only expectation on SSB in RRC-configured DL BWP may be considered as a modification to FG #6-1. Alternatively, a new conditionally mandatory FG for RedCap UEs such that the RedCap UE expects SSB in RRC-configured DL BWP should be introduced with FG #28-1 as pre-requisite. Proposal 3: - Adapt FG # 6-1 for RedCap UEs such that operation without CORESET #0 within the active DL BWP is mandated for RedCap UEs. - To implement this, add a note to FG 6-1: “For a UE supporting FG 28-1, component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)”” - Alternatively, introduce a new conditionally mandatory FG for RedCap UEs such that a RedCap UE may expect an SSB in RRC-configured DL BWP, with FG #28-1 as pre-requisite. On FG #6-1a, considering that the need for measurement gaps or other adjustments for support of FG #6-1a is currently under discussions in RAN4, it would be prudent to wait for further feedback from RAN4 before deciding on whether FG #6-1a may apply to RedCap UE as is or require any updates, including possibly a new separate FG for RedCap UEs. Proposal 4: - For FG #6-1a, RAN1 to wait for further inputs from RAN4 to determine whether any updates, including possibly defining a new separate FG for RedCap UEs, may be necessary. |
| [14] | Qualcomm | Based on the replies of RAN2 and RAN4 to the RAN1 LS [1], any RRC_CONNECTED RedCap UE can perform L1/L3 measurements based on NCD-SSB transmitted in its DL BWP configured by RRC [2-3]. Besides, it was agreed in RAN1#108e meeting that in an RRC-configured DL BWP without CD-SSB, a RedCap UE should support existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s). Therefore, the basic FG definition for RedCap UE should include SSB-based mandatory features using NCD-SSB. Proposal 1: FG 28-1 should include a support for existing SSB-based mandatory feature(s) using NCD-SSB transmitted in the RRC-configured DL BWP of RedCap UE. To clarify the pre-Release-17 non-RedCap UE behaviour associated with FG 6-1a (see Annex), RAN2 has sent an LS [4] to RAN1 and RAN4. Based on Clause 9.2 of TS 38.300 and the definition of “NeedForGapsInfoNR” in TS 38.331, a non-RedCap UE can indicate no-gap for SSB-based infra-frequency RRM measurements when operating in an un-restricted DL BWP without SSB and CORESET#0. Besides, a non-RedCap UE supporting FG 6-1a does not need the network to configure a L1 measurement gap for SSB outside the un-restricted DL BWP, since the non-RedCap UE can expand the RF BW to include the CD-SSB. However, FG 6-1a is not applicable to RedCap UE when the SSB suitable for L1 measurement is outside the max RF bandwidth supported by the RedCap UE (20 MHz in FR1, 100 MHz in FR2). Therefore, SSB-based BM/BFD/RLM outside the SSB-less active DL BWP can be supported as an optional FG for RedCap UE in R17, and a L1 measurement gap should be specified for SSB-based BM/BFD/RLM outside the SSB-less active DL BWP. Moreover, a RedCap UE can indicate the need for L3 measurement gap using “NeedForGapsInfoNR”, if the RedCap UE supports the optional FG for operating in SSB-less DL BWP configured by RRC. Therefore, we have the following proposals: Proposal 2: If a R17 RedCap UE in connected state optionally supports SSB-less DL BWP, the RedCap UE can measure suitable SSB outside the SSB-less active DL BWP for BM/BFD/RLM/RRM. Moreover, - a L1 measurement gap should be specified for SSB-based BM/BFD/RLM outside the SSB-less active DL BWP - the RedCap UE can request a measurement gap for SSB-based infra-frequency measurements by re-using “NeedForGapsInfoNR” IE - when both L1 and L3 measurement gaps for SSB are configured for the RedCap UE, the RedCap UE expects the measurement gaps are separated by a guard time, which is no less than the minimum retuning time required by the RedCap UE to measure a suitable SSB outside the supportable RF bandwidth |

Discussion

[FL1] High priority proposal 2-1:

- No FDD/TDD and FR1/FR2 differentiation are necessary for FG 28-1

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Not necessary: Ericsson, ZTE, DOCOMO, Nokia/NSB, MTK, Intel - FFS for FR1/FR2 differentiation: vivo - FR1/FR2 differentiation is necessary: OPPO |
| MediaTek | Support FL’s proposal |
| ZTE, Sanechips | Support FL proposal |
| Qualcomm | OK with the FL proposal |
| NTT DOCOMO | Support FL proposal based on the discussion in the last RAN plenary. Throughout the discussion, “per UE” was used as “pe UE without FDD/TDD and FR1/FR2 differentiation”. No need to reopen the discussion. |
| vivo | We can live with the proposal. |
| Intel | Support FL proposal. |
| Panasonic | We support the proposal. |
| CMCC | Support FL’s proposal |
| LGE | Support FL’s proposal |
| Nokia, NSB | Support |
| NEC | Support FL’s proposal |
| Ericsson | Support FL’s proposal. (Note that RAN#95e agreed that the UE capability reporting for FG 28-1 is per UE.) |
| Vodafone | Support FL’s proposal |
| Huawei, HiSilicon | Y. Perhaps also good to add a note that ‘per UE’ is already agreed then we can send update to RAN2 without highlighted ‘per UE’ - which may confuse Ran2 whether RAN1 is to against RANP conclusion. |
| FUTUREWEI | Ok with the FL proposal |
| Moderator | All companies are fine with the proposal. The same proposal is set for email endorsement [FL2] High priority proposal 2-1: - No FDD/TDD and FR1/FR2 differentiation are necessary for FG 28-1 |
| Moderator | Following was agreed by email endorsement Agreement - No FDD/TDD and FR1/FR2 differentiation are necessary for FG 28-1 |

[FL1] High priority question 2-2:

- Companies are encouraged to provide views on whether to add an FG for RRC-configured DL BWP with CD-SSB or NCD-SSB

  - New FG for RedCap: vivo, ZTE, CMCC, Intel, Huawei/HiSi

  - As a component of FG 28-1: Ericsson

  - Update FG 6-1: Intel

    - component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)”

| Company | Comment |
| --- | --- |
| Moderator | Following agreement was made in RAN1#108-e Agreement Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement: For FR1, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. For FR2, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. Note: The cases that CSI-RS in this agreement can support are left to RAN4 |
| MediaTek | We support Ericsson’s Proposal 1 that following two components should be added to FG 28-1 to reflect the following RAN1 agreement: - 6. RRC-configured DL BWP with CD-SSB or NCD-SSB - 7. NCD-SSB based measurements in RRC-configured DL BWP Agreement: - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer |
| ZTE, Sanechips | We are supportive to have corresponding description for BWP with restriction and BWP without restriction. As listed by FL, there are 3 options and one of them should be selected at least. If we consider it as a component in FG 28-1, for RedCap, we still need to consider how to interpret 6-1/6-1a/6-2/6-3/6-4, since the current description is not correct for RedCap. If we consider to reinterpret the 6-1/6-1a/6-2/6-3/6-4, it also would be complicated. For example, current 6-1 is Mandatory without capability signalling. For RedCap, it should be based on the optional FG28-1. Therefore, we slightly prefer to define a new FG to replace the legacy features, which looks more clearer. |
| NTT DOCOMO | We agree to add the descriptions regarding the operation in RRC-configured DL BWP with CD-SSB or NCD-SSB. We prefer to add the corresponding features as components of FG28-1. |
| vivo | We prefer to define separate FG for RRC-configured DL BWP with CD-SSB or NCD-SSB and we could clarify that RedCap UEs must indicate the support of such FG. |
| Intel | A separate FG or clarification for FG 6-1a for RedCap UEs would be necessary in our view. Otherwise, we run into issues/conflicts when considering FG 6-1 for RedCap UEs. FG 6-1, as currently described since Rel-15, should NOT apply to RedCap UEs due to the different expectation on presence of CORESET #0. |
| Panasonic | Our preference is to have new FG. If/when something new FG are further introduced in future release of RedCap UE, to have new FG are cleaner. |
| CMCC | We support a new FG for RedCap. The reason is that a new feature is more forward compatible than reusing the current FG 6-1, considering R18 further enhancement. |
| LGE | Either as a new FG or as a component of FG 28-1 is fine for us. |
| Nokia, NSB | There is not point is adding a new FG at this stage while at the same time saying it is mandatory for RedCap UEs. It is OK to add it as component to 28-1, if needed. |
| NEC | Our preference is to include it as a components of FG28-1. |
| Ericsson | We prefer to include “RRC-configured DL BWP with CD-SSB with CORESET#0 or NCD-SSB without CORESET#0” and “NCD-SSB based measurements in RRC-configured DL BWP” as components of FG 28-1. If they are included as part of some other FG, it needs to be ensured that they are anyway mandatory for RedCap UEs. |
| Vodafone | Our preference is to update FG 6-1 |
| Huawei, HiSilicon | New FG is more preferred. Our feeling is that NCD-SSB may have something more to be added for completion and it would be better to have a separate FG, e,g. rate matching, whether RO validation/reselection is separately performed, interaction with CD-SSB. Considering in future there may be other RedCap UEs (e.g.5 Mhz) also use it, or normal UEs to take use of it, it would be good to make it spelt out. |
| FUTUREWEI | We are fine for either as a component of FG 28-1 or as a new FG. |
| Moderator | Summary of companies view - New FG for RedCap: vivo, ZTE, CMCC, Intel, Huawei/HiSi, Pana, CMCC, LGE, [E///] - As a component of FG 28-1: Ericsson, MTK, DCM, LGE - Update FG 6-1: Intel, Vodafone - component 4 is modified as: “BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for Pcell/PScell (if configured)” Given new FG for RedCap has most support, following proposal is made. Further discuss directly over the reflector. [FL2] High priority proposal 2-2: - Add an FG for RRC-configured DL BWP with CD-SSB or NCD-SSB for RedCap as follows 28. NR_redcap; 28-1a; RRC-configured DL BWP with CD-SSB or NCD-SSB; 1) 1 UE-specific RRC configured DL BWP per carrier 2) RRC reconfiguration of any parameters related to BWP 3) RRC-configured DL BWP with CD-SSB with CORESET#0 or NCD-SSB without CORESET#0 4) NCD-SSB based measurements in RRC-configured DL BWP; 28-1; Yes; Per UE; No; No; A UE supporting this FG is not required to support FG 6-1; Optional with capability signaling RedCap UE must indicate this FG is supported. |
| Moderator | Following was agreed by email endorsement Agreement - Following components are added in FG 28-1 - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - [RRC-configured DL BWP with CD-SSB or NCD-SSB] - NCD-SSB based measurements in RRC-configured DL BWP - Add a note in FG 28-1: A UE supporting this FG is not required to support FG 6-1 - Add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB for RedCap 28. NR_redcap; 28-1a; RRC-configured DL BWP without CD-SSB or NCD-SSB; RRC-configured DL BWP without CD-SSB or NCD-SSB FFS whether to add additional components; 28-1; Yes; Per band; N/A; N/A; Optional with capability signaling. |
| FL3 | Companies are encouraged to provide view whether/how to update the component of [RRC-configured DL BWP with CD-SSB or NCD-SSB] in FG 28-1 |
| vivo | To be more accurate, we can revise the component as the following. [RRC-configured DL BWP with CD-SSB and CORESET#0 for FR1, or CD-SSB for FR2, or NCD-SSB for both FR1 and FR2] We would be also fine to keep the current formulation without change. |
| CMCC | Current sentence seems OK. It doesn’t mention CORESET#0, as RedCap support both cases with or without CORESET#0, depending on the multiplexing pattern of CD-SSB and CORESET#0. |
| NTT DOCOMO | We are fine with the current description and also fine with vivo’s update. |
| NEC | We are fine with the component as it is. CORESET#0 may or may not be included in FR1. |
| Samsung | No need to mention CORESET #0 |
| Ericsson | Current sentence seems OK. Perhaps we should say “UE-specific RRC configured” rather than just “RRC-configured” to be consistent with FG 6-1. |
| LGE | We are okay with the current wording. The suggestion from Ericsson is also fine for us. |
| Qualcomm | Fine with the wording in FL3 proposal |
| OPPO | We are fine with the adding “UE-specific” on FL proposal. |
| FL4 | Most companies are fine with the original wording with the update from Ericsson. Therefore, following proposal is made Low priority proposal 2-2a: - Component 9 in FG 28-1 is updated as: [UE-specific RRC- configured DL BWP with CD-SSB or NCD-SSB] |
| Intel | Support. |
| NTT DOCOMO | Fine with the proposal. |
| LGE | Okay. |
| Ericsson | Agree with FL proposal |
| ZTE, Sanechips | Support |
| CMCC | Support. |
| FL5 | This proposal is stable and hence, set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - Component 9 in FG 28-1 is updated as: [UE-specific RRC- configured DL BWP with CD-SSB or NCD-SSB] |

[FL1] High priority question 2-3:

- Companies are encouraged to provide views on whether to add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB

  - New FG for RedCap: HW/HiSi, ZTE, CMCC, DOCOMO, QC

  - Reuse FG 6-1a: DOCOMO

  - Wait for RAN4 input: Intel

| Company | Comment |
| --- | --- |
| Moderator | Following agreement was made in RAN1#108-e Agreement Replace the working assumption from RAN1#107e “Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities” with the following agreement: For FR1, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. For FR2, - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on [FG 6-1a] with supporting CSI-RS, or [FG 6-1a] without supporting CSI-RS. Note: The cases that CSI-RS in this agreement can support are left to RAN4 |
| ZTE, Sanechips | Similar comments as in question 2-2 |
| Qualcomm | It is necessary to introduce a new FG for RRC-configured DL BWP without CD-SSB or NCD-SSB, which can be optionally supported by RedCap UE |
| NTT DOCOMO | We agree to add the descriptions regarding the operation in RRC-configured DL BWP without SSB. We are fine with either option of defining new FG or reusing FG6-1a. |
| vivo | It’s better to define a new FG and make it optional for RedCap UEs, since the expected UE behavior of existing FG6-1a is not crystal clear (discussion ongoing [109-e-AI5-LSs-01]) |
| Intel | Our first preference would still be to wait for RAN4 – one of the main reasons why we had brackets around FG 6-1a in the RAN1 agreement. However, if we are to decide now, we prefer reusing FG 6-1a as it seems to satisfy the capability (it says SSB and can include both CD- and NCD-SSB automatically). |
| Panasonic | Our preference is new FG for cleaner design for possible future extension/modification. |
| CMCC | We support a new FG for RedCap. The reason is that a new feature is more forward compatible than reusing the current FG 6-1, considering R18 further enhancement. |
| LGE | Adding a new FG is preferred. |
| Ericsson | It can either be captured in a new FG or by reusing FG 6-1a. |
| Vodafone | Reuse FG 6-1a is our preference |
| Huawei, HiSilicon | Similar comments as for 2-2. |
| FUTUREWEI | We are fine either way |
| Moderator | Summary of companies view - New FG for RedCap: HW/HiSi, ZTE, CMCC, DOCOMO, QC, vivo, Pana, LGE, E///, FW - Reuse FG 6-1a: DOCOMO, Intel, E///, Vodafone, FW - Wait for RAN4 input: Intel Given new FG for RedCap has most support, following proposal is made. Note that reporting type is aligned with FG 6-1a. Further discuss directly over the reflector. [FL2] High priority proposal 2-3: - Add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB for RedCap as follows 28. NR_redcap; 28-1b; RRC-configured DL BWP without CD-SSB or NCD-SSB; RRC-configured DL BWP without CD-SSB or NCD-SSB and without CORESET#0; 28-1a; Yes; Per band; N/A; N/A; Optional with capability signaling. |
| Moderator | Following was agreed by email endorsement Agreement - Following components are added in FG 28-1 - 1 UE-specific RRC configured DL BWP per carrier - 1 UE-specific RRC configured UL BWP per carrier - RRC reconfiguration of any parameters related to BWP - [RRC-configured DL BWP with CD-SSB or NCD-SSB] - NCD-SSB based measurements in RRC-configured DL BWP - Add a note in FG 28-1: A UE supporting this FG is not required to support FG 6-1 - Add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB for RedCap 28. NR_redcap; 28-1a; RRC-configured DL BWP without CD-SSB or NCD-SSB; RRC-configured DL BWP without CD-SSB or NCD-SSB FFS whether to add additional components; 28-1; Yes; Per band; N/A; N/A; Optional with capability signaling. |
| FL3 | Companies are encouraged to provide view whether to add additional components in FG 28-1a |
| vivo | Revisit when decision is made for email thread [109-e-AI5-LSs-01] |
| CMCC | Similar to FG28-1, where “NCD-SSB based measurements in RRC-configured DL BWP” is included, we also think the following can be component of FG28-1a, - support measurement operation based on FG6-1a. - support RLM, BFD, CBD and L1 RSRP measurement based on CSI-RS if UE reports the corresponding capabilities. - support CSI-RS based L3 measurement with associated SSB if UE reports the corresponding capability. |
| NTT DOCOMO | Whether a RedCap UE can measure SSB outside the RRC-configured DL BWP for BM/BFD/RLM/RRM can be the additional components. It may depend on the discussion results of [109-e-AI5-LSs-01]. |
| Samsung | Depends on LSs-01, on the other hand, we are open for CMCC’s suggestion |
| Ericsson | CMCC’s suggestion seems fine to us. Note that the following conclusion made in RAN1 is perhaps relevant to the discussion on FG 28-1a. Conclusion: - From RAN1 perspective, whether and under what conditions a RedCap UE requires to be configured with existing measurement gaps to support operation without SSB in an RRC-configured active BWP, and its related UE feature discussion (including measurement gaps) is up to RAN4. - Send an LS to RAN4 to inform them about the conclusion. |
| LGE | We can wait for the potential updates from [109-e-AI5-LSs-01] |
| Qualcomm | FG 28-1a is related to the discussion in [109-e-AI5-LSs-01], we can wait for the decision made for non-RedCap UE as well as the MG discussion in RAN4 for R17 RedCap UE. |
| OPPO | Wait for the outcome of LS 01. |
| FL4 | As suggested by a number of companies, we can wait for the outcome in [109-e-AI5-LSs-01] |
| Intel | Agree |
| Huawei, HiSilicon | Agree with CMCC. The suggestion from CMCC is already agreed and should be considered. We are open for refinement but generally they seem to be reasonable. Other relevant issues can be added once there are conclusion from e.g. RAN4 – this is the same handling as FG28-1 where we agreed on something first and are adding more now. About the LS01, we are confused. Firstly, Mr. Chair said that there may not be any conclusion for LS01 next meeting; secondly, it is for R16 UEs as the LS targeted and now we already made a new FG just because we want to avoid the potential impact of 6-1a; thirdly, both the FL and some other vendors in LS01 claimed that RedCap is a separate discussion that is irrelevant. We can course add more if there is useful outcome from LS01 and agreed by RedCap folks. However shall not refuse to add other relevant descriptions that are already agreed. |
| Ericsson | Agree with FL proposal |
| ZTE, Sanechips | Similar as Component 9 in FG 28-1, ‘UE specific’ can be used. For 28-1a, OK to wait for the outcome in [109-e-AI5-LSs-01] if there is no other potential changes for discussion. |
| CMCC | As Huawei says, discussion of [109-e-AI5-LSs-01] is about legacy UE behavior, and the new UE capability without measurement gap is not applicable to RedCap UE. So we think the outcome of [109-e-AI5-LSs-01] will provide no help for this UE feature. |
| FL5 | Companies may need more time to check how to add additional components in FG 28-1a. Comeback next meeting. |

[FL1] High priority question 2-4:

- Companies are encouraged to provide views on whether to add FGs corresponding to FG 6-2/6-3/6-4 for RedCap

  - Support: ZTE

| Company | Comment |
| --- | --- |
| ZTE, Sanechips | Similar comments as in question 2-2 |
| DOCOMO | This can be discussed after some progress is made in question 2-2 and question 2-3 |
| vivo | For 6-2/6-3/6-4, we prefer to reuse existing FGs unless significant issue is identified. |
| CMCC | We are OK to define separate FGs corresponding to FG 6-2/6-3/6-4 for RedCap if necessary. |
| LGE | We prefer to comeback to this later as suggested by DOCOMO |
| Nokia, NSB | In principle not needed, but OK to discuss it later as well. |
| Ericsson | They can either be captured in new FGs or by reusing FG 6-2/6-3/6-4. |
| Huawei, HiSilicon | Seems Ok to reuse. |
| Moderator | Most companies prefer to reuse existing FGs or discuss later. Comeback after some progress is made in question 2-2 and question 2-3 |
| FL3 | Since question 2-2 and question 2-3 are resolved, let’s resume the discussion. Given most companies prefer to reuse existing FGs, following proposal is made [FL3] High priority proposal 2-4: - FG 28-1 is added as a prerequisite FGs of FG 6-2/6-3/6-4 - Component 5 in FG 6-2/6-3 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes [CD-SSB or NCD-SSB] - Component 6 in FG 6-4 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes [CD-SSB or NCD-SSB] |
| vivo | Support in principle. The highlighted part may be updated according to the decision of 28-1. |
| NTT DOCOMO | Support FL proposal. |
| FUTUREWEI | A question about the first bullet “FG 28-1 is added as a prerequisite FGs of FG 6-2/6-3/6-4” – the wording seems unclear. With this wording: an implication is the legacy UE can no longer support 6-2/6.3/6.4 unless it supports 28-1. I do not think that is the intention of the dependency because a legacy UE does not support 28-1 |
| NEC | We support the proposal. |
| Samsung | Fine. |
| Ericsson | Vivo’s suggestion seems fine to us. FUTUREWEI also has a good point. The prerequisite for FGs 6-2/6-3/6-4 should be “6-1 or 28-1”. |
| LGE | We support this proposal in principle. We agree with FUTUREWEI and Ericsson. |
| Qualcomm | Support FL3 proposal |
| OPPO | Agree with the proposal. |
| FL4 | Regarding the question from FW, to make it clear, the prerequisite FGs of FG 6-2/3/4 are “6-1 or 28-1”, as answered by E///. Also, since proposal 2-2a seems agreeable, square brackets in cyan highlighted part are removed. The proposal is updated accordingly. High priority proposal 2-4: - FG 28-1 is added as a The prerequisite FGs of FG 6-2/6-3/6-4 are revised as “6-1 or 28-1” - Component 5 in FG 6-2/6-3 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB - Component 6 in FG 6-4 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB |
| FUTUREWEI4 | Thank you for clarifying the pre-requisites |
| Intel | OK. |
| NTT DOCOMO | Support the proposal. |
| LGE | Okay. |
| Ericsson | Support |
| ZTE, Sanechips | For RedCap UE, the prerequisites“6-1 or 28-1”would cause ambiguity, since it conflicts with the following Note: - Add a note in FG 28-1: A UE supporting this FG is not required to support FG 6-1 |
| CMCC | Support |
| FL5 | @ZTE: I don’t think the prerequisite conflicts with the note since UE supporting FG 28-1 can support FGs 6-2/6-3/6-4 without supporting FG 6-1. Could you provide your view on how to revise the proposal? Companies are encouraged to provide view on whether ZTE’s comment is valid, and how to update the proposal. |
| Ericsson | We support the FL proposal. We do not see any inconsistency. |
| Nordic | We agree with view from FL. Proposal 2-4 seems to be an elegant solution to the problem |
| LGE | For non-RedCap the prerequisite is 6-1, but for RedCap the prerequisite is 28-1. That is our understanding. Perhaps some clarification wording may help, but we don’t see any problem with the FL proposal as it is. |
| ZTE, Sanechips | Then we are assuming, for NR UE, if it is reported as FG28-1 RedCap UE, FG6-1 is also not supported. Based on this understanding(seems a little bit different with LGE), we are fine. |
| NTT DOCOMO | We agree with FL and we are fine with Proposal 2-4 as it is. |
| FL6 | All companies are fine with the proposal and hence, it is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
| Moderator | Following was agreed via email endorsement Agreement - The prerequisite FGs of FG 6-2/6-3/6-4 are revised as “6-1 or 28-1” - Component 5 in FG 6-2/6-3 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB - Component 6 in FG 6-4 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB |

[FL3] Low priority question 2-5:

- Companies are encouraged to provide views on whether to add any other basic features for RedCap UE

  - Revise component 5 as

    - 5. Separate initial DL BWP for RedCap UEs

      - FFS: For separate initial DL BWP used for paging, CD-SSB is included

        - Ericsson, vivo, Samsung, OPPO, Intel

      - FFS: For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included.

        - CMCC

      - FFS: For separate initial DL BWP used for paging in connected mode,

        - BWP#0 configuration option 1: CD-SSB is included

        - BWP#0 configuration option 2: SSB may or may not be included depending on the support of FG 6-1a

        - DOCOMO

      - FFS: For separate initial DL BWP used for paging, proper SSB CD-SSB is included

        - NEC

      - FFS: For separate initial DL BWP used for paging, CD-SSB is included

        - MTK

  - Add a component for NCD-SSB based measurements in RRC-configured DL BWP

    - Ericsson, Samsung, NEC, QC

  - Add a component for Early indication of RedCap UE in Msg3 for 4-step RACH

    - Ericsson, Samsung

  - Add a note that Early indication of RedCap UE in MsgA for 2-step RACH’ is supported for UEs that support FG 28-1 and 2-step RACH

    - Ericsson, Samsung

  - A RedCap UE should support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0.

    - HW/HiSi

| Company | Comment |
| --- | --- |
| CMCC | For the FFS about including CD-SSB for paging, it is not correct for separation initial DL BWP with BWP#0 configuration option#2 according to the following agreement, that’s why we propose the above modification . Agreement - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB |
| Huawei, HiSilicon | The description about BWP#0 configuration option might need a bit more time as being discussed in other agenda that a possibility of not supporting one of the options. No strong view to further refine early indication as they are anyway not known by gNB (except that MsgA is known, but for a similar reason it does not matter) The CORESET related configuration should be captured as it is related to the update of 28-1 w.r.t. 6-1 in terms of CORESET#0 presence. |
| vivo | Our comments are as below - 5. Separate initial DL BWP for RedCap UEs - FFS: For separate initial DL BWP used for paging, CD-SSB is included - [vivo] Support - FFS: For separate initial DL BWP used for paging in idle/inactive mode or in connected mode as BWP#0 configuration option 1, CD-SSB is included. - [vivo] need to wait the decision from [109-e-R17_RedCap-01] about whether to support separate initial DL BWP configured by BWP#0 configuration option 1. - FFS: For separate initial DL BWP used for paging in connected mode, - BWP#0 configuration option 1: CD-SSB is included - BWP#0 configuration option 2: SSB may or may not be included depending on the support of FG 6-1a - [vivo] 1st sub-bullet: need to wait the decision from [109-e-R17_RedCap-01] about whether to support separate initial DL BWP configured by BWP#0 configuration option 1. 2nd sub-bullet is not needed as it is covered by existing component “[RRC-configured DL BWP with CD-SSB or NCD-SSB]” of 28-1 - FFS: For separate initial DL BWP used for paging, proper SSB CD-SSB is included - [vivo]The statement may be too general - FFS: For separate initial DL BWP used for paging, CD-SSB is included - [vivo]The statement may be too general |
| NTT DOCOMO | We agree with vivo for component 5 that we should wait the discussion progress of whether BWP#0 configuration option 1 is supported for separate initial DL BWP which is discussed in [109-e-R17_RedCap-01]. |
| NEC | CD-SSB or NCD-SSB may be included in a separate initial DL BWP when it is active BWP. |
| Samsung | For RACH related description, we think it needs to align with RAN 2 spec. For paging, this should be enough. No need to highlight in which mode. “For separate initial DL BWP used for paging, CD-SSB is included” |
| Ericsson | Wait for agreements in [109-e-R17_RedCap-01]. |
| LGE | Wait for agreements in [109-e-R17_RedCap-01]. |
| OPPO | We are fine for waiting for the agreement [109-e-R17_RedCap-01]. |
| FL4 | As a number of companies suggested, component 5 can be further discussed after some progress is made in [109-e-R17_RedCap-01]. For RACH related components, since we have already agreed that “It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A”, the update is not mandatory. For the last one, since companies haven’t provided their view, companies are invited to provide view whether the update is necessary or not. |
| Intel | We think the proposal from HW-HiSi is reasonable and we are open to adding such a component. |
| NTT DOCOMO | Regarding the number of CORESET(s) per BWP, it seems reasonable to support two configured CORESETs per BWP in addition to CORESET#0 if CORESET#0 is not included in the BWP. We are also fine to add this component as basic feature of RedCap considering that legacy UEs supports FG3-1 (one CORESET can be configured in addition to CORESET#0) without capability signaling. |
| Huawei, HiSilicon | Agree as proponent of last one. It is a very natural result. |
| Ericsson | We are fine with the last proposal (“A RedCap UE should support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0”). According to the legacy specification, FR2 mandatorily supports two CORESETs in addition to CORESET#0. Should a RedCap FR2 UE support two or three CORESETs when CORESET#0 is not in the BWP? From 38.306: multipleCORESET Indicates whether the UE supports configuration of up to two PDCCH CORESETs per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. If this is not supported, the UE supports one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signaling for FR2 and optional for FR1.. |
| ZTE, Sanechips | - RAN2 is also discussing the paging for BWP#0 configuration option 2. We need more time to check component 5. - For the component for Early indication of RedCap UE in Msg3 for 4-step RACH, we are not sure whether it is a RAN2 issue. But we are OK if other companies think it should be solved in RAN1. - For the last proposal, it means to revise FG3-1 or add Note or component in FG28-1 ? [FL] My take is FG 3-1 is mandatorily supported by RedCap UE as legacy, and add a component in FG 28-1. |
| CMCC | For the last proposal, we are OK for Huawei’s suggestion. |
| FL5 | Given most companies are fine with the update, following proposal is made with reflecting the comment from Ericsson Low priority proposal 2-5a: - Add following components in FG 28-1: - For FR1, support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0. - For FR2, support two configured CORESETs per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support three configured CORESETs per BWP per cell in addition to CORESET0. |
| Ericsson | Perhaps the proposal should be updated as follows: Low priority proposal 2-5a: - Add following components in FG 28-1: - For FR1, support one configured PDCCH CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured PDCCH CORESETs per BWP per cell in addition to CORESET0. - For FR2, support two configured PDCCH CORESETs per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support three configured PDCCH CORESETs per BWP per cell in addition to CORESET0. |
| ZTE, Sanechips | OK with Ericsson’s update. |
| NTT DOCOMO | We support proposal 2-5a and also fine with Ericsson’s update. |
| FL6 | All companies are fine with the proposal with Ericson’s revision and hence, it is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
| Moderator | No conclusion was made in this meeting. Comeback in next meeting. |

[FL3] Low priority question 2-6:

- Confirm the contents in square brackets in the columns of “Consequence if the feature is not supported by the UE” and “Note” in FG 28-1

  - Support: Ericsson, ZTE, DOCOMO, Nokia/NSB, MTK, Intel

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |
| vivo | For the Note column, we suppose the following statement should be deleted eventually. “It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A” |
| CMCC | OK |
| NTT DOCOMO | Support. |
| NEC | Support. Exact wording could be up to RAN2. |
| Samsung | Fine. But we think this may also capture in RAN 1 TR on UE feature |
| Ericsson | Ok |
| LGE | Support. |
| Qualcomm | Support |
| OPPO | We still suggest remove “RedCap UEs do not support carrier aggregation or dual connectivity.” Would there some better place in spec to capture that? |
| FL4 | This UE feature list will be capture in TR38.822 and also in TS38.306. Therefore, if the note is confirmed, RAN2 may implement the note in TS38.306. To make it clear, the proposal is updated as follows Low priority proposal 2-6: - Confirm the contents in square brackets in the columns of “Consequence if the feature is not supported by the UE” in FG 28-1 - Confirm the note “RedCap UEs do not support carrier aggregation or dual connectivity.” in FG 28-1 |
| Intel | Support. |
| NTT DOCOMO | Support. |
| LGE | Support. |
| Huawei, HiSi | Support |
| Ericsson | Support |
| ZTE, Sanechips | Support |
| CMCC | Support. |
| FL5 | This proposal is stable and hence, set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - Confirm the contents in square brackets in the columns of “Consequence if the feature is not supported by the UE” in FG 28-1 - Confirm the note “RedCap UEs do not support carrier aggregation or dual connectivity.” in FG 28-1 |

- 28-3: Half-duplex FDD operation for RedCap UE

In [1], FG 28-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | [UE is assumed to support FD-FDD in FDD bands] | [Per band] | [FDD only] | [FR1 only] |  |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#109e meeting.

| [3] | Ericsson | There are square brackets in some of the fields for FG 28-3. After the RAN#95-e decision that FG 28-3 should be reported per band, we think that all these square brackets can be removed and text within them kept as is, except that it needs to be clarified that the “Consequence if the feature is not supported by the UE” is that the “UE is assumed to support FD-FDD in that band” rather than in all FDD bands. For FG 28-3 (‘Half-duplex FDD operation for RedCap UE’), clarify that the “Consequence if the feature is not supported by the UE” is that the “UE is assumed to support FD-FDD in that band” rather than in all FDD bands. Remove all other square brackets (but keep the text within them) on the FG 28-3 row. Consider removing “type A” from all fields on the FG 28-3 row (since NR only has a single HD-FDD operation type). |
| --- | --- | --- |
| [4] | vivo, Guangdong Genius | For 28-3 of Half-duplex FDD operation type A for RedCap UE, since it was already agreed that the type is per band, there is no need of FDD/TDD differentiation and FR1/FR2 differentiation. Proposal 4: For FG28-3, no need of FDD/TDD differentiation and FR1/FR2 differentiation. |
| [5] | ZTE, Sanechips | For FG 28-3, similarly, the following yellow highlighted need further decision. Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation. Per band; FDD only; FR1 only. Obviously, HD-FDD is only supported in FDD bands. Moreover, as we know, FR2 bands do not support FDD operation. Therefore, FG 28-3 is only used in FR1. Proposal 3: FG 28-3, - FDD/TDD differentiation can be set as ‘FDD only’ - FR1/FR2 differentiation can be set as ‘FR1 only’ |
| [7] | OPPO | - In the updated UE capability, this capability FG28-3 is per band reported. It is allowed to let device to be a RedCap HD-FDD UE in one band, but not in another band. - The 28-3 can be looked as additionally set. If not indicate, UE should be FD in FDD band. Further, we should confirm it is FDD only and FR1 only. For FR2, there is not FDD band. - Further in the current specification of 38.214, it only defines the case that UE support enhanced Rel-17 type A PUSCH repetition. It seems we should not bond 28-3 to one specific capability out side of redcap feature. - Proposal 2: For FG 28-3, it is FDD only and FR1 only. No additional restriction of 28-3 with other UE features. |
| [9] | DOCOMO | - FG 28-3: Half-duplex FDD operation type A for RedCap UE - Based on the agreement in RAN#95e, no further discussion is necessary in RAN1 regarding the reporting type, i.e., FG 28-3 is reported per band, which is applicable only to FR1 FDD bands. - The column of “Consequence if the feature is not supported by the UE” can be confirmed. |
| [11] | Nokia/NSB | - Confirm consequence if feature is not supported. |
| [12] | MediaTek | Proposal 2: Confirm FG 28-3 as it is. |
| [13] | Intel | Similarly, for FG #28-3, all the outstanding details can be confirmed by removing the current brackets. Proposal 2: - Remove the brackets and confirm the current descriptions for remaining details for FG #28-3. |

Discussion

[FL3] Low priority question 4-1:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 28-3 which do not have capability signaling impacts

  - Confirm the contents in square brackets

    - Ericsson, ZTE, OPPO, DOCOMO, Nokia/NSB, MTK

  - The column of Consequence if the feature is not supported by the UE is revised as: UE is assumed to support FD-FDD in FDD that bands

    - Ericsson

  - Remove “type A” from FG28-3

    - Ericsson

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Y |
| vivo | We are fine with the proposals above by companies. |
| NTT DOCOMO | Support above proposals. |
| Ericsson | Fine with all three proposals. |
| LGE | We are okay with the three proposals. |
| Qualcomm | OK |
| FL4 | All companies are fine with the update. Following proposal is made Low priority proposal 4-1: - The columns of “Consequence if the feature is not supported by the UE” in FG 28-3 is updated as: UE is assumed to support FD-FDD in that FDD bands - Remove “type A” from FG28-3 |
| Intel | Support. |
| NTT DOCOMO | Support. |
| LGE | Support. |
| Ericsson | Support |
| ZTE, Sanechips | Before type B HD-FDD is precluded in Rel-18 RedCap WI, we think keeping type A is safer. The first bullet is fine with us. |
| CMCC | Support. |
| FL5 | @ZTE: There is no “Type A HD-FDD” in NR spec. It is safer to use the same terminology as spec. Given first bullet is stable, the proposal is separated into two. Proposal 4-1a is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. Low priority proposal 4-1a: - The columns of “Consequence if the feature is not supported by the UE” in FG 28-3 is updated as: UE is assumed to support FD-FDD in that FDD bands Further discuss proposal 4-1b. Since this is editorial issue, if it cannot be converged, no further discussion is necessary. Companies are encouraged to check whether you are fine with the proposal. Low priority proposal 4-1b: - Remove “type A” from FG28-3 |
| Ericsson | We support the FL proposals. |
| LGE | We are generally fine with the FL proposals. For proposal 4-1b, whether to introduce the type B HD-FDD will be determined next meeting at the latest. Then we can make editorial changes, if needed, without any concern. |
| ZTE, Sanechips | If there is no confusion for all of us, we are fine. |
| NTT DOCOMO | We support the FL proposals and also agree with LGE we can consider editorial change if needed in the future meeting. |
| FL6 | All companies are fine with the proposal and hence, it is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - The columns of “Consequence if the feature is not supported by the UE” in FG 28-3 is updated as: UE is assumed to support FD-FDD in that FDD bands |

- Other FGs

This section discusses other FGs which are not included in [1].

Following feedbacks are provided in contributions for the RAN1#109e meeting.

| [5] | ZTE, Sanechips | According to the WID [2] and agreement, UE capabilities related to CA, DC and wider max UE bandwidth are not applicable to RedCap UEs. However, it seems that whether to support the UE capabilities exceeding 1Tx/2Rx capability, are controversial in the discussion of UE feature agenda. Given the Rel-17 RedCap WID description, the maximum UL peak data rate for werables is 50Mbps. Obviously, support of more than 1Tx is not necessary, since the peak data rate can be up to 90Mbps for 64QAM and 1 layer with SCS=15KHz according to the calculation. More Tx antennas bring more complexity and cost, which is not aligned with the motivation of the WI. Moreover, for 1Rx RedCap UE, supporting more than 1 UL port seems to be impossible due to the physical circuit. Additionally, according to the RAN4 discussion [3] as following, actually 2Tx is excluded: Issue 1-1: Power class and TX architecture in FR1 - Proposals: - Option 1: Agree below agreement from RAN4#101-bis-e Issue 1-1-1: 1 PC3 UL TX architecture assumption - WF - For TX architecture of 23 dBm PA Issue 1-1-2: PC2 UL TX architecture assumption - WF - 1 TX of 26 dBm PA in Rel-17 and 2 TX architecture is excluded in Rel-17 Issue 1-1-3: PC2 support for HD-FDD mode - WF - PC2 support based on operator request - Recommended WF - Option 1 Discussion: Agreement: agree on Option 1.. Therefore, based on above, the FGs exceeding 1 Rx UE capability including 2-3, 2-13, 2-14, 2-16b should not be supported. Therefore, we have the following proposal: Proposal 1: UE capabilities exceeding 1Tx capability is not supported. Last, for those Rel-17 features, we also need to determine whether RedCap UE can support or not. At least eIAB and NR DC/CA further enhancements should not be supported according to the Rel-17 RedCap WID and agreement. Regarding above-52GHz, from our perspective, it is premature to support this feature, since some of the FGs actually exceeds the RedCap UE capability, e.g., SCS for DL control channel. For Rel-17 feMIMO, at least the FGs exceeding RedCap UE capability should be excluded, e.g., 23-1-1c, 23-1-1e, 23-1-1f, 23-1-1g, 23-8-3, 23-8-9, 23-9-3. As for the MBS, it seems be problematic to use separate initial DL BWP without SSB for RedCap UE. And this feature should be discussed with high priority since a LS [4] is sent to RAN1 from SA2. Proposal 5: for the Rel-17 features, - eIAB and NR DC/CA further enhancements should be excluded. - Discuss and decide above-52GHz and feMIMO are not supported or some of FGs in the feature are not supported. - Discuss and decide with high priority whether MBS should be supported for RedCap UE. |
| --- | --- | --- |

Discussion

[FL3] Low priority question 5-1:

- Companies are encouraged to provide views on whether RedCap UE can support UE capabilities exceeding 1Tx capability (FGs 2-3, 2-13, 2-14, 2-16b)

| Company | Comment |
| --- | --- |
| Moderator | This issue has been discussed in previous RAN1 meetings but no agreements were made. FL suggestion is no more discussion is necessary for this issue. If companies have different view, please provide the motivation why such restriction is necessary. |
| vivo | If there is no more discussion, what is the common understanding? |
| Ericsson | Fine with FL suggestion. RedCap UEs can indicate support. |
| LGE | Fine with FL suggestion. |
| OPPO | Don’t know if we should sweep every combination of capabilities. |
| FL4 | To vivo: no further agreement/conclusion means FGs 2-3, 2-13, 2-14, 2-16b are applicable to RedCap UEs, as legacy. |
| Huawei, HiSilicon | I think then we have problem with some company’s response, as our colleagues from RAN4 consider it is common understanding that more than 1Tx is not supported for RedCap. Seems still controversial and unclear… |
| Ericsson | We have already had this discussion in RAN1 and see no reason to repeat it now in RAN1. |
| ZTE, Sanechips | According to RAN4 agreement, actually, 2Tx is not supported in Rel-17. Therefore, the corresponding capabilities also should be excluded in RAN1. Issue 1-1: Power class and TX architecture in FR1 - Proposals: - Option 1: Agree below agreement from RAN4#101-bis-e Issue 1-1-1: 1 PC3 UL TX architecture assumption - WF - For TX architecture of 23 dBm PA Issue 1-1-2: PC2 UL TX architecture assumption - WF - 1 TX of 26 dBm PA in Rel-17 and 2 TX architecture is excluded in Rel-17 Issue 1-1-3: PC2 support for HD-FDD mode - WF - PC2 support based on operator request - Recommended WF - Option 1 Discussion: Agreement: agree on Option 1.. Based on this, UL MIMO is not supported and corresponding capabilities, e.g., FG2-14, are also not supported. |
| FL5 | Thank you HW and ZTE for pointing out the RAN4 decision. Companies are encouraged to provide view whether to preclude FGs 2-3, 2-13, 2-14, 2-16b for RedCap UE by, e.g. adding a note that this FG is not applicable to UE supporting FG 28-1. |
| Ericsson | The assumptions used in RAN4 for the Rel-17 RedCap RAN4 requirements do not mean that RAN1 needs to limit the UE implementation. RAN4 can add further requirements later in a release independent way, and then it would be unfortunate if the UE implementation is limited by the RAN1 UE feature list. Furthermore, the RAN4 assumptions may have been agreed with some special RedCap UEs in mind, specifically RedCap UEs with reduced antenna configurations and/or RedCap UEs with HD-FDD operation. For RedCap UEs that support the same antenna configuration and same FD-FDD operation as non-RedCap UEs, it may be relatively straightforward to add RAN4 requirements for RedCap UEs with 2 Tx, if additional RAN4 requirements are even needed. So, we do not think that RAN1 should impose additional restrictions regarding 2 Tx. |
| ZTE, Sanechips | It is strange that RAN1 need to support 2Tx when RAN4 make decision that 2Tx is excluded. How to implement 2Tx without related RAN4 requirement definition? If RAN4 add further requirements later in a release, RAN1 also can make corresponding change. At least in Rel-17, 2Tx and corresponding capabilities should be precluded. |
| NTT DOCOMO | We are fine to explicitly preclude the support of FG2-3/2-13/2-14/2-16b for UE supporting FG28-1. |
| FL6 | Based on the comment, following proposal is made. The intention is not close the door for future releases. Further discuss directly over the reflector. Low priority proposal 5-1: - Add a note in FGs 2-3, 2-13, 2-14, and 2-16b: This FG is not applicable to UE supporting FG 28-1 in Rel-17 |
| ZTE, Sanechips | Sorry for our confusion in our tdocs for FGs 2-3, 2-13, 2-14, and 2-16b. If we only consider the Tx related capabilities, then FG 2-13, 2-14, and 2-16b may be not supported. Given the limited time, we are not sure we have time to check all the capabilities exceeding the 1Tx. Therefore, it is expected to have the general description, like: Proposal: UE capabilities exceeding 1Tx capability is not supported for RedCap in Rel-17. |
| Ericsson | We do not feel comfortable making this decision now. What we could potentially agree to at this point is that RedCap UEs supporting a single Rx branch do not support UE capabilities exceeding a single Tx branch. But it might be better to postpone the whole issue till the next meeting. |
| Moderato | No conclusion was made in this meeting. If necessary, let’s comeback in next meeting. |

[FL3] Low priority question 5-2:

- Companies are encouraged to provide views on which Rel-17 UE features (FGs 23-x to 36-x) are not applicable to RedCap UEs

| Company | Comment |
| --- | --- |
| Moderator | This issue was discussed in the last RAN1 meeting but companies have different view on which Rel-17 UE features (FGs 23-x to 36-x) are not applicable to RedCap UEs. FL suggestion is no more discussion is necessary for this issue. If companies see strong concern not having further decision, please explain why such FGs should be excluded from RedCap UE. Note that the LS from SA2 on MBS UE feature will be discussed under AI5 as suggested by RAN1 chair. |
| vivo | If there is no more discussion, is it the common understanding that RedCap UE can indicate support of FGs 23-x to 36-x? |
| Ericsson | Fine with FL suggestion. RedCap UEs can indicate support. |
| LGE | Fine with FL suggestion. |
| Qualcomm | Agree with Moderator |
| FL4 | To vivo: At least no artificial limitation is introduced for FGs 23-x to 36-x. It is companies choice whether to support those FGs. |
| ZTE, Sanechips | For Rel-17 MBS, since similar issue is raised in [109-e-AI5-LSs-02], if we can converge fast, then it also would help facilitate the discussion in [109-e-AI5-LSs-02]. For RedCap, there are some issues to support MBS - Scheduling limitation. Since RedCap UE can only support maximum UE bandwidth 20M in FR1 and 100M in FR2, which is different with NR UE. When the PDCCH and PDSCH is scheduled for NR UE and RedCap UE, the bandwidth also should be limited to 20M in FR1 and 100M in FR2. Additionally, the modulation 64QAM for RedCap is assumed for DL in FR1, instead of 256QAM. And RedCap UE only supports 1Rx or 2Rx in FR1. Therefore, conservative scheduling is also needed because of the different Rx branches and modulation order assumption. - Configuration limitation of CFR. Broadcast support that the CFR can be configured with a size larger than CORESET#0, this kind of configuration should not be supported for RedCap. - Limitation on separate initial DL BWP. Since the CFR for MBS should contains CORESET#0, then for RedCap, separate initial DL BWP for RedCap should not be used, if this separate initial DL BWP may not contain CORESET#0. In this case, the introduction of separate initial DL BWP is actually kind of meaningless. So, if MBS is supported by for RedCap, at least the above aspects should be addressed. Otherwise, it would cause the serious impacts on the NR UE and NW scheduling/configuration. |
| FL5 | @ZTE: If we need to discuss whether current spec supports the combination of RedCap UE and MBS, it would be better to discuss in maintenance agenda, since UE feature discussion is not appropriate place to discuss such aspect. |
| ZTE, Sanechips | [109-e-AI5-LSs-02] is going to the conclusion and the time is limited in this meeting. We are OK to consider it in the future. However, it seems that the discussion about R17 features is stagnant now. |

- Conclusions

Following agreements were made in this meeting.

Agreement

- No FDD/TDD and FR1/FR2 differentiation are necessary for FG 28-1

Agreement

- Following components are added in FG 28-1

  - 1 UE-specific RRC configured DL BWP per carrier

  - 1 UE-specific RRC configured UL BWP per carrier

  - RRC reconfiguration of any parameters related to BWP

  - [RRC-configured DL BWP with CD-SSB or NCD-SSB]

  - NCD-SSB based measurements in RRC-configured DL BWP

- Add a note in FG 28-1: A UE supporting this FG is not required to support FG 6-1

- Add an FG for RRC-configured DL BWP without CD-SSB or NCD-SSB for RedCap

| 28. NR_redcap | 28-1a | RRC-configured DL BWP without CD-SSB or NCD-SSB | RRC-configured DL BWP without CD-SSB or NCD-SSB FFS whether to add additional components | 28-1 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Component 9 in FG 28-1 is updated as: [UE-specific RRC- configured DL BWP with CD-SSB or NCD-SSB]

Agreement

- Confirm the contents in square brackets in the columns of “Consequence if the feature is not supported by the UE” in FG 28-1

- Confirm the note “RedCap UEs do not support carrier aggregation or dual connectivity.” in FG 28-1

Agreement

- The columns of “Consequence if the feature is not supported by the UE” in FG 28-3 is updated as: UE is assumed to support FD-FDD in that FDD bands

Agreement

- The prerequisite FGs of FG 6-2/6-3/6-4 are revised as “6-1 or 28-1”

- Component 5 in FG 6-2/6-3 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB

- Component 6 in FG 6-4 is revised as: BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell. For RedCap UE, BW of a UE-specific RRC configured BWP includes CD-SSB or NCD-SSB

References

[1]	R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2203110	Rel-17 UE features for RedCap	Huawei, HiSilicon

[3]	R1-2203116	UE features for RedCap	Ericsson

[4]	R1-2203534	Remaining issues on UE feature for NR REDCAP	vivo, Guangdong Genius

[5]	R1-2203598	Discussion on RedCap UE features	ZTE, Sanechips

[6]	R1-2203881	Discussion on UE features for RedCap	Samsung

[7]	R1-2203999	Rel-17 RedCap UE features	OPPO

[8]	R1-2204284	Discussion on UE features for RedCap	CMCC

[9]	R1-2204361	Discussion on UE features for RedCap	NTT DOCOMO, INC.

[10]	R1-2204436	Discussion on UE feature for RedCap	NEC

[11]	R1-2204591	On UE features for REDCAP	Nokia, Nokia Shanghai Bell

[12]	R1-2204712	UE features for RedCap	MediaTek Inc.

[13]	R1-2204781	On Rel-17 UE features for RedCap	Intel Corporation

[14]	R1-2205003	UE features for RedCap	Qualcomm Incorporated