3GPP TSG RAN WG1 #109-e								R1-2204405

e-Meeting, May 9th –20th, 2022

Agenda item:	8.16.7

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for UE power saving enhancements

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.7 regarding UE features for UE power saving enhancements and captures the following email discussion.

[109-e-R17-UE-features-PowSav-01] Email discussion on UE features for UE power savings enhancements – Shinya (DOCOMO) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e [1], there are following feature groups for UE power saving enhancements.

- 29-1	Paging enhancement

- 29-2	TRS resources for idle/inactive UEs

- 29-3a	PDCCH skipping

- 29-3b	2 search space sets group switching

- 29-3c	3 search space sets group switching

- 29-3d	2 search space sets group switching with PDCCH skipping

The issues to be discussed are tagged and colour coded with High priority or Low priority based on ASN.1 impact.

- 29-1: Paging enhancement

In [1], FG 29-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | There is no description about the new agreed Type2A CSS for PEI PDCCH in the FG3-1. To keep no touch on FG 3-1, which is stable from Rel-15, it is proposed to have a similar sentence to FG 29-1 to capture the above UE mandatory capability to support 29-1, i.e. the component 3 currently in the square brackets: “- For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot” Proposal 1: Remove the square brackets of component 3 in UE feature 29-1. |
| --- | --- | --- |
| [3] | CATT | The SearchSpace configured for PEI is generally a normal search space configuration without any specific restriction if SearchSpaceID is greater than 0. Thus, the UE capability of paging enhancement FG29-1 does not need to capture the specific configuration for Type 2 PDCCH-CSS. Proposal 1: UE capability of paging enhancement FG29-1 does not need to capture the specific configuration for Type 2 PDCCH-CSS NR_UE_ pow_sav_enh; 29-1; Paging enhancement; 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot]; N; UE does not support paging enhancement; Per band; N; N; N; Optional with capability signaling. |
| [4] | vivo | For UE feature 3-1 provides Basic DL control channel, and with regards to the monitoring occasion, the following is described, - For type 1 with dedicated RRC configuration, type 3, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 PDCCH CSS without dedicated RRC configuration and for type 0, 0A, and 2 PDCCH CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1-PDCCH CSS without dedicated RRC configuration, or Types 0, 0A, or 2 PDCCH-CSS configurations within a single span of three consecutive OFDM symbols within a slot However, in Rel-17, a new DCI format 2_7 is specified as paging early indication which is monitored in Type2A-PDCCH CSS. Descriptions of the monitoring occasion of Type2A-PDCCH CSS is not provided in the current specification. Monitoring Type2A-PDCCH CSS can be similar to Type2-PDCCH CSS (for Paging), and hence the following proposal is made. ### Proposal 1: - Update the descriptions of 29-1 as follows, 29. NR_UE_pow_sav_enh; 29-1; Paging enhancement; 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot; Yes; UE does not support paging enhancement; Per band; N/A; N/A; N/A; Optional with capability signalling. |
| [6] | Samsung | The purpose of component 3 is to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1. However, we don’t see the need to repeat the legacy paging reception criterion for FG 29-1. FG 29-1 reuses the legacy paging PDCCH to provide the paging early indication, and there is no any new design of paging PDCCH reception from FG 29-1. Conclusion 1: No need to add component 3 to capture the PDCCH monitoring of type2A-CSS based on legacy UE capability FG3-1 |
| [7] | OPPO | Proposal 1: Prefer not including component 3 in 29-1. |
| [9] | NTT DOCOMO, INC. | - It’s not clear for us yet that whether “Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2” is needed in the component 3. |
| [10] | Nokia | - For component 3, remove references to other types and confirm the component: - 3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot |
| [12] | Intel Corporation | Proposal 1: Component 3 in FG 29-1 is not anything new with respect to FG 29-1 and can be removed. If anything needs to be kept in component 3 at all, it can be only for Type 2A CSS. |
| [14] | Qualcomm Incorporated | In RAN1 #108-e, a new UE feature FG [22-x] was agreed as an optional feature with capability signaling for PDCCH monitoring. This FG relaxes the UE PDCCH monitoring capability required by FG 3-1 by allowing UE to only monitor PDCCH in the first four OFDM symbols in a slot. With the FG [22-x], the UE is not always required to monitor Type 2 CSS PDCCHs in a single span of any three consecutive OFDM symbols within the entire slot. Based on this, the current FFS of component 3 of FG 29-1 should take the FG [22-x] into account and does not require the UE to monitor the PEI PDCCH in Type 2A CSS in the entire slot. It is still reasonable to assume the set of symbols where PEI PDCCH is monitored in Type 2A CSS is the same as the set of symbols where paging PDCCH is monitored in Type 2 CSS. 22. NR Others; [22-x]; PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot; Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols that are within the first four OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing.; Yes; N/A; Per UE; No; FR1 only; No; Optional with capability signalling. Observation 1: A UE that supports FG [22-x] is not required to monitor paging PDCCH in Type 2 CSS within a single span of any three consecutive OFDM symbols within a slot. FG 29-1 is about UE capability on paging enhancements. It should be only relevant to PEI PDCCH monitoring in Type 2A CSS and paging PDCCH monitoring in Type 2 CSS. It is unclear why PDCCH monitoring in Type 0, 0A and 1 CSSs is relevant. The text for Type 0, 0A and 1 CSSs should be removed from component 3 of FG 29-1. Observation 2: PDCCH monitoring in Type 0, 0A or 1 CSS is irrelevant to the paging enhancement feature FG 29-1. Hence, text related to Type 0, 0A or 1 CSS should be removed from the component 3 of FG 29-1. Proposal 1: For FG 29-1, the following update is proposed for component 3: - Symbols where UE monitors PEI PDCCH for Type 2A CSS follow symbols where UE monitors paging PDCCH for Type 2 CSS. - Adopt the following text for component 3 for FG 29-1 29. NR_UE_pow_sav_enh; 29-1; Paging enhancement; 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the set of OFDM symbols within a slot where UE can monitor the paging PDCCH in Type 2 CSS]; Yes; UE does not support paging enhancement; Per band; N/A; N/A; N/A; For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No; Optional with capability signalling. |

Discussion

[FL1] Low priority question 2-1:

- Companies are encouraged to provide views on whether/how to revise component 3 in FG 29-1

  - Confirm component 3

    - Huawei, HiSilicon, vivo

    - Remove Type 0/0A/1/2 CSS

      - Nokia/NSB, DOCOMO

  - component 3 is not necessary

    - CATT, Samsung, OPPO, Intel

  - Revise as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the set of OFDM symbols within a slot where UE can monitor the paging PDCCH in Type 2 CSS

    - Qualcomm

| Company | Comment |
| --- | --- |
| Qualcomm | We fine to either remove component 3 or let monitoring of Type 2A CSS follow the monitoring of Type 2 CSS. |
| Apple | We originally support the confirmation on component 3, which means that all the mentioned CSSs should fall into the same set of 3 symbols within a slot. Given the new FG mentioned by QC, we think QC’s proposal is fine. |
| CATT | Component 3 is not necessary since this is search space configuration issues not UE capability issues. |
| Ericsson1 | For component 3, at least the reference to Type 0/0A/1/2 is not needed. |
| ZTE,Sanechips | If the component 3 is needed, the Type 0/0A/1/2 CSS can be removed. |
| DOCOMO | Same view as Ericsson and ZTE. Also, we are OK to remove component 3. |
| Nokia, NSB | We are fine with revising component 3 to simplify the description. We think component 3 is useful there. |
| Huawei, HiSilicon | We think the component 3 should be kept: For the comments about the “Remove Type 0/0A/1/2 CSS”: The point made by component 3 is the monitoring occasions of type2A-CSS is in the single span of three consecutive OFDM symbols within a slot where the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 CSS configurations coincide. If we remove “Type 0/0A/1/2 CSS”, maybe the monitoring occasion of type2A-CSS can be in different span from the one of the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 CSS configurations. The meaning is different from that of FG3-1, if we remove Type 0/0A/1/2 CSS. The comments on QC’s proposal: the feature 22-x was just agreed in the last RAN1 meeting as an optional Rel-16 UE feature. PEI design in Rel-17 has been claimed as completed before the agreement of 22-x, and PEI should never have a chance to assume this optional feature as baseline during the Rel-17 discussion for PEI design. The same assumption in FG 3-1 is always assumed for PEI, which is the mandatory UE capability on this from Rel-15. |
| MTK | We prefer the keep component 3 since it is just the same description as the text for Type 2 CSS in FG 3-1 (R15 mandatory): - FG 3-1: “For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot” Another possibility is to say the requirement of Type 2 CSS in FG 3-1 also apply to Type 2A CSS: (Following text revised from QC’s proposal) - The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement set of OFDM symbols within a slot where UE can monitor the for paging PDCCH in Type 2 CSS as specified in FG 3-1 Besides, to our understanding, the 22-x feature agreed in last RAN1 meeting is resulted from DSS application to extend NR’s PDCCH reception from the first 3 symbols to the first 4 symbols, which seems not related to Type 2/2A CSS monitoring restriction (figure below courtesy of QC from R1-2202113). |
| Intel | We don’t think component 3 is needed, similar view as CATT. |
| Vivo | The description of the monitoring capability is missing, so we think it should be kept. DoCoMo or Qualcomm’s version is also fine. |
| FL3 | Summary of companies view - Confirm component 3 - Huawei, HiSilicon, vivo, MTK - Remove Type 0/0A/1/2 CSS - Nokia/NSB, DOCOMO, E///, ZTE - component 3 is not necessary - CATT, Samsung, OPPO, Intel, QC, DCM - Revise as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the set of OFDM symbols within a slot where UE can monitor the paging PDCCH in Type 2 CSS - Qualcomm, Apple, vivo - Revise as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS as specified in FG 3-1 - MTK Companies are encouraged to provide view whether you can live with the revision from QC or MTK |
| Qualcomm | QC revision has covered MTK’s revision for both FG 3-1 and FG [22-x]. So we prefer QC version. |
| MTK | We still prefer to confirm component 3 (exactly the same wording as the text for Type 2 CSS in FG 3-1) or take MTK’s revision. We tend to think “the same requirement …” is different from “the same set …” in QC’s version. Using “the same set …” may give a wrong impression that PEI needs to be put in the same symbol as paging. |
| Huawei, HiSilicon | We prefer to confirm the component 3 or MTK’s revision. Again, we don’t think FG [22-x] is relevant with what we discussed here. FG [22-x] is optional feature for connected mode UE for 15kHz SCS only. We have provided our comment/reasons that component 3 should be kept: For the comments about the “Remove Type 0/0A/1/2 CSS”: The point made by component 3 is the monitoring occasions of type2A-CSS is in the single span of three consecutive OFDM symbols within a slot where the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 CSS configurations coincide. If we remove “Type 0/0A/1/2 CSS”, maybe the monitoring occasion of type2A-CSS can be in different span from the one of the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 CSS configurations. The meaning is different from that of FG3-1, if we remove Type 0/0A/1/2 CSS. The comments on QC’s proposal: the feature 22-x was just agreed in the last RAN1 meeting as an optional Rel-16 UE feature. PEI design in Rel-17 has been claimed as completed before the agreement of 22-x, and PEI should never have a chance to assume this optional feature as baseline during the Rel-17 discussion for PEI design. The same assumption in FG 3-1 is always assumed for PEI, which is the mandatory UE capability on this from Rel-15. |
| ZTE,Sanechips | We are okay with the version suggested by MTK, i.e., same as  same as the requirement for paging PDCCH in Type 2 CSS as specified in FG 3-1. |
| Nokia, NSB | In principle Mediatek’s version is ok, but there is a problem with the text, as FG 3-1 is not really described in any specification. It is described only in TR38.822, and hence it is unclear if RAN2 would be able to make such reference in 38.306. |
| Ericsson3 | OK in principle with Mediatek version, but similar comment as Nokia about reference to FG 3-1 as it does appear in 38.306, and it is preferable to also avoid using “requirement” in the FG description. |
| Apple | Having read the comments, we also think FG 22-x is related to the PDCCH monitoring in connected mode in DSS operation, and should not affect how PEI operates for idle/inactive UEs. Therefore, we now support MTK’s version in principle. Fine with any fine-tuning on wording as commented by Nokia/NSB and Ericsson. |
| Intel | OK, in principle. Nokia’s point is also valid |
| MTK2 | To our understanding, as mentioned by Ericsson3, FG 3-1 also appears in 38.306 as shown below. As for Ericsson3’s comment on avoid using “requirement”, would it be better to use “restriction”? |
| Qualcomm2 | We do not agree with Apple’s comments that FG 22-x is only for connected mode. We also do not agree with Huawei that this FG should ignore the new FG 22-x. Moreover, our version does not preclude MTK’s proposal by removing FG 3-1 as shown in the following To address MTK’s concern, the following by modifying their proposal is also good for us: - The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS as specified in FG 3-1 |
| FL4 | Given most companies are fine with MTK’s version in principle, and QC proposed an update for MTK’s version, following proposal is set for further discussion. Low priority proposal 2-1: - Component 3 in FG 29-1 is revised as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS |
| Apple | OK with the updated proposal. To follow up on QC’s comments, I was not accurate in the statement that FG 22-x is only for connected mode. What I meant to say is that the CSS can already be supported in any 3 consecutive OFDM symbols in a slot based on FG 3-1, with or without FG 22-x. So what FG 22-x relaxes is the USS configurations. Therefore, it should not really have impact on PEI. |
| MTK | We are fine with the FL4 proposal. Also share similar comments with Apple on FG 22-x. |
| intel | We are fine with FL4 proposal |
| Huawei, HiSilicon | Agree with Apple that FG 22-x does not have impact on PEI. To avoid any confusion or debating in future, we would like to capture it clearly considering there seems the possibility that different companies may have different interpretation of the same sentence. We prefer the original description from MTK, however, if we need some revision for compromise, we prefer the following revision: - Component 3 in FG 29-1 is revised as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE mode UE; |
| OPPO | Fine with FL4 proposal. |
| Qualcomm | Regarding Apple and MTK’s comments on FG 22-x, we found the description of FG 22-x does not restrict it to USS. But we understand that there is no need to further debate on FG 22-x which is out of the scope of this feature. We are fine with Huawei’s suggestion and add “inactive” also as follows. - Component 3 in FG 29-1 is revised as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs; |
| ZTE, Sanechips | Okay with QC’s update |
| FL5 | Seems the QCs update would be acceptable to all, and hence, it is set for email endorsement. If you have concern on the proposal, please indicate it directly over the reflector. |
| Moderator | Following was agreed via email endorsement Agreement - Component 3 in FG 29-1 is revised as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs |

- 29-2: TRS resources for idle/inactive UEs

In [1], FG 29-2 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS occasions for idle/inactive Ues 1. Support reading TRS configuration from SIB [2. Support receiving L1 indication for TRS availability] |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | [Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7] | Optional without capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | Proposal 2: Remove the square brackets of component 2, and delete all the content in the note column for FG 29-2. |
| --- | --- | --- |
| [3] | CATT | For IDLE/Inactive UE power saving by the additional TRS/CSI-RS configuration, SIB-X is used to broadcast the configuration of TRS/CSI-RS resource with L1 signaling for the indication of TRS availability dynamically. The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI formats in Paging DCI and/or PDCCH-based PEI. Since TRS/CSI-RS availability would be indicated by L1 signaling dynamically, it is UE implementation whether to use TRS/CSI-RS for AGC or channel tracking even it is configured and available. It was agreed in RAN1#107b-e that TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI. UE could elect to retrieve L1 signaling of TRS availability indication from either PEI, paging DCI or both whenever CRC check passes at a DCI monitoring occasion. 29. NR_UE_pow_sav_enh; 29-2; TRS resources for idle/inactive Ues; TRS ccasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability; N; UE cannot receive TRS resources for idle/inactive mode; N/A; N; N; N; Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1; Optional without capability signalling. Proposal 3: The UE capability of TRS should be the UE obtaining the TRS configuration from the SIB and the L1 signaling from DCI format 2_7 for PEI if configured and DCI format 1_0 for Paging DCI. |
| [4] | vivo | RAN1 has agreed that the L1 indication for TRS availability can be either PEI (DCI 2_7) or paging DCI (DCI 1_0). However, receiving TRS availability by PEI (DCI 2_7) requires UE to support 29-1 first. Hence, it is better to separate it as a independent feature and add 29-1 as its prerequisite . ### Proposal 2: - Update the descriptions of 29-2 as follows, 29. NR_UE_pow_sav_enh; 29-2; TRS ccasion for idle/inactive Ues; TRS ccasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0; N; Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode; Optional without capability signalling. 29. NR_UE_pow_sav_enh; 29-2a; TRS ccasion for idle/inactive Ues; - Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7; 29-1, 29-2; N; Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode; Optional without capability signalling. |
| [5] | ZTE, Sanechips | Modify the note of FG 29-2 as “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1”. |
| [7] | OPPO | Proposal 2: Fine with the current components. Note in FG 29-2 is necessary. |
| [8] | Apple | Proposal: Adopt the following changes for FG 29-2: 29. NR_UE_pow_sav_enh; 29-2; TRS resources for idle/inactive Ues; TRS occasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via paging DCI 3. Support receiving L1 indication for TRS availability via DCI format 2_7; N; UE cannot receive TRS resources for idle/inactive mode; N/A; N/A; N/A; N/A; Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7; Optional without capability signalling. |
| [9] | NTT DOCOMO, INC. | - Regarding the note, it should be confirmed since it makes the description of this feature more clear. |
| [10] | Nokia | - 29-2: Confirm component 2 and note. |
| [13] | Ericsson | - FG 29-2 : - Add as component 2) “Support receiving L1 indication for TRS availability”. - Add in Note column “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7”. |
| [14] | Qualcomm Incorporated | Proposal 2: Confirm the FFS parts in FG 29-2. |
| [12] | Intel | Proposal 2: Support one of the following regarding FG 29-2: - Update component description as follows and Note column can be empty (preferred) TRS occasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI format 1_0 3. Support receiving L1 indication for TRS availability via DCI format 2_7 if the UE supports FG 29-1. - Confirm the content in yellow in both component 2 description and Note column 29. NR_UE_pow_sav_enh; 29-2; TRS resources for idle/inactive Ues; TRS ccasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability; FFS; N; Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE cannot receive TRS resources for idle/inactive mode; Per UE N/A; N/A; N/A; N/A; Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7; Optional without capability signalling. |

Discussion

[FL1] Low priority question 3-1:

- Companies are encouraged to provide views on whether/how to revise component 2 and the note column in FG 29-2

  - Component 2

    - Confirm:

      - Huawei, HiSilicon, CATT, OPPO, DOCOMO, Nokia, Ericsson, Qualcomm, Intel (only if Note is confirmed too)

    - Revise:

      - 2. Support receiving L1 indication for TRS availability via paging DCI

      - 3. Support receiving L1 indication for TRS availability via DCI format 2_7

      - Apple, Intel

    - Add another FG:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive Ues | TRS ccasions for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2a | TRS resources for idle/inactive Ues | - Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7 | 29-1, 29-2 | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode |  |  |  |  |  | Optional without capability signalling |

      - vivo

  - Note

    - No necessary

      - Huawei, HiSilicon

    - Confirm

      - OPPO, Apple, DOCOMO, Nokia, Qualcomm, Intel

    - Revise as: Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 FG 29-1

      - ZTE, intel

| Company | Comment |
| --- | --- |
| Qualcomm | We prefer to confirm the current FFS of component 2. For the note, we are fine to confirm the FFS or remove the note. |
| Apple | Our only intention is clarify that the UE supporting 29-2 may not support availability indication via DCI 2_7 if it does not support 29-1. We are flexible in terms of how to achieve it. If the proposal is to keep both component 2 and the note, we would also be fine. |
| CATT | We would like to confirm the WA of component2 and the note. |
| Ericsson1 | Confirm the component 2 and keep the note. |
| ZTE,Sanechips | We are okay without the note. However, if the note is needed, we think it should be revised as “Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 FG 29-1”, since there is capability signaling about “supporting receiving DCI format 2-7” |
| DOCOMO | We prefer to confirm the component 2 and the note. Also, we are Ok with Apple and ZTE’s suggestion since it’s clear that Ues supporting 29-2 may not support to receive the availability indication via DCI 2_7. |
| Nokia, NSB | Confirm the component 2 and keep the note. This text has been massaged a lot already in different meetings, it is unclear if there is any benefit in further editorial changes. |
| Huawei, HiSilicon | Confirm the component 2 and remove the note. We didn’t see any reason/benefit to have a note. |
| MTK | No strong view. We can accept to take majority view and move on. |
| Intel | Intel’s proposal was missing in the table and is now added. Note is useful and needed to clearly explain how the feature works. Otherwise, there is ambiguity in whether supporting L1 indication as in component 2 means UE also supports receiving DCI format 2-7. Note helps to resolve this. |
| Vivo | For progress, we are fine to confirm and remove the [] for component 2 and the note. |
| FL3 | - Component 2 - Confirm: - Huawei, HiSilicon, CATT, OPPO, DOCOMO, Nokia, Ericsson, Qualcomm, Intel (only if Note is confirmed too), CATT, vivo - Revise: - 2. Support receiving L1 indication for TRS availability via paging DCI - 3. Support receiving L1 indication for TRS availability via DCI format 2_7 - Apple, Intel - Add another FG: - vivo - Note - No necessary - Huawei, HiSilicon, QC - Confirm - OPPO, Apple, DOCOMO, Nokia, Qualcomm, Intel, CATT, E///, vivo - Revise as: Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 FG 29-1 - ZTE, intel Given the situation, following proposal is made [FL3] Low priority proposal 3-1: - Component 2 and the note column in FG 29-2 are confirmed. |
| Qualcomm | We support the FL3 proposal 3-1. |
| MTK | No strong view. We can accept to take majority view. |
| Huawei, HiSilicon | We don’t know why a note is needed. We prefer not to have the note. |
| ZTE, Sanechips | We are okay to go without the not, or revise it as below - Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7 FG 29-1 |
| Nokia, NSB | Support FL3 proposal |
| Ericsson3 | OK with FL3 proposal. |
| Apple | Support FL3 P3-1. |
| Intel | OK with FL3 proposal |
| FL4 | Similar to the last meeting, there is a dead-lock that some companies prefer one way and some others prefer another way. Given most companies are fine with the proposal, I would like to ask companies to live with it. If it is not acceptable, please provide another proposal which is acceptable to all. Low priority proposal 3-1: - Component 2 and the note column in FG 29-2 are confirmed. |
| Apple | Support the proposal |
| MTK | No strong view. |
| Intel | Support the FL4 proposal. Note that this is a compromised proposal. Original proposal was to update component 2 and add feature description clearly as part of component 2 or add new component 3. |
| Huawei, HiSilicon | The note is still not preferred; however, we can live with the FL4 proposal. |
| OPPO | Support the proposal. |
| ZTE, Sanechips | Okay with FL4 proposal |
| FL5 | Thank you HW for your flexibility. Now the proposal would be acceptable to all, and hence, it is set for email endorsement. If you have concern on the proposal, please indicate it directly over the reflector. |
| Moderator | Following was agreed via email endorsement Agreement - Component 2 and the note column in FG 29-2 are confirmed. |

- 29-3a to 29-3d: PDCCH monitoring adaptation within an active BWP

In [1], FGs 29-3a to 29-3d are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, if PDCCHSkippingDurationList is not configured |  | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, if PDCCHSkippingDurationList is not configured | 29-3b | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs, PDCCH skipping by scheduling DCI, and timer based SSSG switching | 29-3a, 29-3b | Y |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [3] | CATT | The objective of CONNECTED UE power saving with reducing PDCCH monitoring reduction is achieved by dynamic adaptation of PDCCH monitoring interval. It was agreed in RAN1#106-e that up to 2 bits are include in the scheduling DCI format 1_1, 1_2, 0_1, and 0_2 to indicate PDCCH monitoring adaptation. The UE feature of PDCCH adaptation discussion in RAN1#108-e had agreed to have UE feature groups of FG29-3a and FG29-3d operation. FG29-3c has the prerequisite of UE support of 2 SSSG switching in FG29-3b. FG29-3d has the prerequisite of both FG29-3a and FG29-3b. The UE feature for PDCCH monitoring adaptation would also include the indication from scheduling DCI for SSSG switching per UE. Proposal 4: The UE capability of PDCCH monitoring adaptation for CONNECTED mode UE FG29-3b and FG29-3c should be per UE. NR_UE_ pow_sav_enh; 29-3a; PDCCH skipping; Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured; Y; UE could not reduce PDCCH monitoring configured by the given search space.; Per band; N; N; N; Optional with capability signaling. NR_UE_ pow_sav_enh; 29-3b; 2 search space sets group switching; Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching if PDCCHSkippingDurationList is not configured; Y; UE could not reduce the PDCCH monitoring with the SSSG switching; Per UE; N; N; N; Optional with capability signaling. NR_UE_ pow_sav_enh; 29-3c; 3 search space sets group switching; Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching if PDCCHSkippingDurationList is not configured; 29-3b; Y; UE could not reduce the PDCCH monitoring with the SSSG switching; Per UE; N; N; N; Optional with capability signaling. NR_UE_ pow_sav_enh; 29-3d; 2 search space sets group switching with PDCCH skipping; Support of 2-bit indication of SSSG switching between 2 SSSGs PDCCH skipping by scheduling DCI and timer based SSSG switching; 29-3a, 29-3b; Y; UE could not reduce the PDCCH monitoring by PDCCH skipping and/or SSSG switching; Per band; N; N; N; Optional with capability signaling. |
| --- | --- | --- |
| [11] | MediaTek Inc. | In Rel-16, SSSG switching is also defined from the NR-U agenda, and the UE supported switch delay P is defined with the two following UE capabilities (cited from 38.306 V16.8.0): searchSpaceSwitchWithDCI-r16 Indicates whether the UE supports switching between two groups of search space sets with DCI 2_0 monitoring that comprises of the following functional components: - Monitor DCI 2_0 with a search space set switching field; - Support switching the search space set group with PDCCH decoding in group 1; - Support a timer to switch back to original search space set group; - Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group. The UE can switch search space set groups for different cells independently, unless the UE supports jointSearchSpaceSwitchAcrossCells-r16. The UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2, unless the UE supports searchSpaceSwitchCapability2-r16. The UE supports search space switching triggers to be configured for up to 4 cells or 4 cell groups.; Band; No; N/A; N/A. searchSpaceSwitchCapability2-r16 Indicates whether the UE supports search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS. If the UE supports this feature, the UE needs to report searchSpaceSwitchWithDCI-r16 or searchSpaceSwitchWithoutDCI-r16.; Band; No; N/A; N/A. Similar switch dealy P value should be reported for R17 SSSG switching features. Proposal 3: Include the following candidate values for SSSG switch delay in the “Components” column of FG 29-3 b/c/d: - Value 1: search space set group switching delay P=25/25/25/50 symbols for µ=0/1/2/3 - Value 2: search space set group switching delay P=10/12/22/44 symbols for µ=0/1/2/3 Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between Ues (Sidelink WI only)”.; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 29. NR_UE_pow_sav_enh; 29-3b; 2 search space sets group switching; Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, if PDCCHSkippingDurationList is not configured Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3}; Y; Per band; N/A; N/A; N/A; Optional with capability signaling. 29. NR_UE_pow_sav_enh; 29-3c; 3 search space sets group switching; Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, if PDCCHSkippingDurationList is not configured Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3}; 29-3b; Y; Per band; N/A; N/A; N/A; Optional with capability signaling. 29. NR_UE_pow_sav_enh; 29-3d; 2 search space sets group switching with PDCCH skipping; Support of 2-bit indication of SSSG switching between 2 SSSGs, PDCCH skipping by scheduling DCI, and timer based SSSG switching Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3}; 29-3a, 29-3b; Y; Per band; N/A; N/A; N/A; Optional with capability signaling. |
| [13] | Ericsson | - FG 29-3b : - Add in Note column “UE supports search space set group switching capability-1 according to Table 10.4-1 of 38.213.” to clarify that SSSGS is based on capability 1 switching time according to Table 10.4-1 of 38.213. (See Annex A for detailed agreement from RAN1#108-e). - (New) FG 29-3e - Introduce new FG (29-3e) with below description to indicate support of SSSG switching according to capability 2 switching time, analogous to 10-9d that was introduced for Rel-16 NR-U (see Annex B). 29. NR_UE_pow_sav_enh; 29-3e; Support Search space set group switching capability 2; Search space set group switching Capability-2 according to Table 10.4-1 of 38.213 for SSSG switching.; 29-3b; Yes; n/a; Per band; N/A; NA; Optional with capability signalling. |

Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether to revise the reporting type of FG 29-3b/3c as per UE

  - Support: CATT

| Company | Comment |
| --- | --- |
| Moderator | Following was agreed in RAN1#108-e Agreement - The type of FGs 29-3a to 29-3d is per band Proponent is invited to further explain why the above agreement should be reverted |
| Apple | We do not see the motivation for such changes. The agreement should be kept. |
| CATT | The SSSG switching function is the control function and independent of operation band. We believe that they should be per UE. |
| Huawei, HiSilicon | We didn’t see the reason to revert the agreement. |
| MTK | We do not see strong need to revert the agreement. |
| Intel | Reverting previous agreement is not strongly needed. |
| FL2 | Given most companies don’t think previous agreement should be reverted, no further discussion is necessary but proponent can further elaborate the motivation, if any. |

[FL1] High priority proposal 4-2:

- Add a new FG 29-3e to indicate the support of Search space set group switching capability 2

| 29. NR_UE_pow_sav_enh | 29-3e | Support Search space set group switching capability 2 | Search space set group switching Capability-2 according to Table 10.4-1 of 38.213 for SSSG switching. | 29-3b | Yes |  |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Add a note in FG 29-3b: UE supports search space set group switching capability-1 according to Table 10.4-1 of 38.213

  - Support: MTK, Ericsson

| Company | Comment |
| --- | --- |
| Apple | We are fine with the direction in general. But the switching time for FG 29-3d/3e should be clarified also. One possible way is to introduce the candidate value as a component of 29-3b, similar to what is suggested by MTK (the exact wording can be discussed). And for FG 29-3d/3ed, a note can be added that the switching values reported for 29-3b also apply (29-3b is the prerequisite for both). |
| CATT | We are OK with the addition. |
| Ericsson1 | Support. Our thinking is that the indicated capability (FG 29-3e) would apply to all the other SSSG switching cases also (29-3d/3e) indicated by the UE. |
| ZTE,Sanechips | Okay with the proposal in principle. |
| DOCOMO | We are OK with the proposal. |
| Huawei, HiSilicon | If the intention is to cover all 29-3b,c,d, may the way proposed by MTK can be considered, which reports the capability inside 29-3b,3c,and 3d. |
| MTK | We prefer to spell out the two switching delay value sets in 29-3b, c, d as suggested in our contribution. Also, R17 SSSG switching have different UE behaviors compared to R16 SSSG switching, so it seems better to write out the switch delay details as we suggested below: Candidate values for SSSG switching delay are: {Value 1: switching delay P=25/25/25/50 symbols for µ=0/1/2/3, Value 2: switching delay P=10/12/22/44 symbols for µ=0/1/2/3} |
| Intel | We are OK in principle |
| Vivo | Fine with the proposal. |
| FL2 | Based on the comments so far, the main discussion point is whether it is allowed to report different capabilities of switching delay among FG 29-3b/c/d. If current proposal is adopted, UE supporting switching capability-2 and FGs 29-3b/c/d, capability-2 is supported for all FGs 29-3b/c/d If MTK way is adopted (i,e, FGs 29-3b/c/d each report capability-1 or capability-2 individually), UE supporting FGs 29-3b/c/d can report either capability-1 or capability-2 individually. Further discuss directly over the reflector which option should be adopted. |
| Moderator | Following was agreed by email endorsement. Agreement - Add a new FG 29-3e to indicate the support of Search space set group switching capability 2 for FR1 29. NR_UE_pow_sav_enh; 29-3e; Support Search space set group switching capability 2 for FR1; Search space set group switching Capability-2 according to Table 10.4-1 of 38.213 for SSSG switching.; 29-3b; Yes; Per band; N/A; N/A (FR1 only); N/A; For UE supporting this FG and FG 29-3b, 29-3c, and/or 29-3d, search space set group switching Capability-2 is applied to the FGs; Optional with capability signalling. - Add a note in FGs 29-3b/3c/3d: UE supports search space set group switching capability-1 [according to Table 10.4-1 of 38.213] [FL3] Companies are encouraged to provide view whether capability-1 should follow the value in Table 10.4-1 of 38.213 or P=25/25/25/50 symbols for µ=0/1/2/3 |
| MTK | We prefer P=25/25/25/50 symbols for µ=0/1/2/3. For the switch delay values, we do not think we need to directly follow the values defined in R17 “above 52.6GHz” WI since it is targeting FR2-2. R17 UE power saving WI can be targeting general cell phone products including FR2-1, while R17 “above 52.6GHz” WI is targeting a new market and new type of product for FR2-2. Besides, for the previous RAN1 working assumption: - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), o the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP o Note: Pswitch is defined in Table 10.4-1 in TS38.213 The working assumption says “at least Pswitch symbols defined in Table 10.4-1 in TS38.213”, which means the exact switch delay can be larger, and not limited to the Pswitch values in TS38.213. |
| ZTE, Sanechips | We would like to clarify that the Pswitch defined in Table 10.4-1 in TS38.213 was originally introduced in Rel-16 NR-U (NOT Rel-17 B52.6G), which is also for FR1 (NOT for FR2-2). For the revision of the values, we think it should be discussed in main session, if needed. |
| Ericsson3 | The values in Table 10.4-1 should be followed. We do not need reverting of values already agreed and that are in the specification. |
| Apple | We think the values in Table 10.4-1 should be followed. We do not see any need to reopen the discussion. |
| MTK2 | To our understanding, the values mentioned by Ericsson3 are within a working assumption, which to my understanding reserves some room for further discussion. Now we are in the UE capability discussion session, and have a chance to examine the UE ability more comprehensively; hence, we think now is the time to have a final check on these values. We are open to discuss Pswitch values between 40 defined by B52.6G in 38.213 and 50 as proposed by us. ZTE’s suggestion to discuss this in main session is also fine to us, if companies prefer to do it this way. |
| CATT | We also believe the value in Table 10.4-1 should not be further discussed. |
| FL4 | If the values are in the spec as Table 10.4-1 of 38.213, we don’t need to discuss different value in UE feature discussion. If some companies prefer to change the spec, it should be discussed in maintenance agenda. Therefore, if following is not agreeable, no further discussion is necessary in this meeting. Interested companies are free to raise the issue in the maintenance agenda in next meeting. Low priority proposal 4-2a: - The note in FGs 29-3b/3c/3d is confirmed as: UE supports search space set group switching capability-1 according to Table 10.4-1 of 38.213 |
| Apple | Support the proposal |
| MTK | Thanks FL for the continuous efforts. As we mentioned before, we are not fine with Table 10.4-1 of 38.213 due to the reason that the value for μ=3 comes from B52.6 WI. We would raise this issue in the maintenance agenda in next meeting. |
| Intel | Ok with the proposal |
| OPPO | Support the proposal. |
| ZTE, Sanechips | Support FL proposal |
| FL5 | Seems MTK is not ready to go with the proposal. As mentioned in the last round, no further discussion is necessary in this meeting. Once the values are confirmed in maintenance, we can comeback. |

- Conclusions

Following agreements were made in this meeting.

Agreement

- Add a new FG 29-3e to indicate the support of Search space set group switching capability 2 for FR1

| 29. NR_UE_pow_sav_enh | 29-3e | Support Search space set group switching capability 2 for FR1 | Search space set group switching Capability-2 according to Table 10.4-1 of 38.213 for SSSG switching. | 29-3b | Yes |  |  | Per band | N/A | N/A (FR1 only) | N/A | For UE supporting this FG and FG 29-3b, 29-3c, and/or 29-3d, search space set group switching Capability-2 is applied to the FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Add a note in FGs 29-3b/3c/3d: UE supports search space set group switching capability-1 [according to Table 10.4-1 of 38.213]

Agreement

- Component 3 in FG 29-1 is revised as: The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs

Agreement

- Component 2 and the note column in FG 29-2 are confirmed.

References

[1]	R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2203087	Rel-17 UE features for UE power saving enhancements	Huawei, HiSilicon

[3]	R1-2203480	Discussion on UE feature of UE Power saving enhancements for NR	CATT

[4]	R1-2203535	Discussion on UE features for UE power saving enhancements	vivo

[5]	R1-2203599	Remaining issues of UE feature for UE power saving enhancements	ZTE, Sanechips

[6]	R1-2203882	UE features for UE power saving enhancements	Samsung

[7]	R1-2203997	Rel-17 UE Power Saving features	OPPO

[8]	R1-2204222	Views on UE features for Rel-17 UE power saving	Apple

[9]	R1-2204362	Discussion on Rel-17 UE features for UE power saving	NTT DOCOMO, INC.

[10]	R1-2204592	On UE features for UE power saving enhancements	Nokia, Nokia Shanghai Bell

[11]	R1-2204695	On UE features for UE power saving enhancements	MediaTek Inc.

[12]	R1-2204782	Discussion on UE features for UE power saving	Intel Corporation

[13]	R1-2204963	UE features for UE PS	Ericsson

[14]	R1-2205004	UE features for UE power saving enhancements	Qualcomm Incorporated