3GPP TSG RAN WG1 #109-e			R1-2204406

e-Meeting, May 9th –20th, 2022

Agenda item:	8.16.8

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR coverage enhancement

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.8 regarding UE features for NR coverage enhancement and captures the following email discussion.

[109-e-R17-UE-features-CovEnh-01] Email discussion on UE features for NR coverage enhancement – Shinya (DOCOMO) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

In the updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e [1], there are following feature groups for NR coverage enhancement.

- 30-1	Increased maximum number of PUSCH Type A repetitions

- 30-2	PUSCH Type A repetitions based on available slots

- 30-3	TB processing over multi-slot PUSCH

- 30-3a	Repetition of TB processing over multi-slot PUSCH

- 30-4	The maximum duration for DM-RS bundling

- 30-4a	DM-RS bundling for PUSCH repetition type A

- 30-4b	DM-RS bundling for PUSCH repetition type B

- 30-4c	DM-RS bundling for TB processing over multi-slot PUSCH

- 30-4d	DMRS bunding for PUCCH repetitions

- 30-4e	Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH

- 30-4f	Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling

- 30-4g	[Restart DM-RS bundling after the events that violate power consistency and phase continuity]

- 30-4h	DM-RS bundling for non-back-to-back transmission

- 30-5	Slot based dynamic PUCCH repetition indication

- 30-6	Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

The issues to be discussed are tagged and colour coded with High priority or Low priority based on ASN.1 impact.

- 30-1 to 30-2: Enhancements for PUSCH Type A repetitions

In [1], FGs 30-1 to 30-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | 5-14, 5-16, or 5-17 | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | Any features to support repetition type A can be one of the prerequisite feature groups for FG 30-2. Thus it should be one of {5-14, 5-16, 5-17, 11-6, 30-1}. Proposal 1: The prerequisite feature group for FG 30-2 is one of {5-14, 5-16, 5-17, 11-6, 30-1}. |
| --- | --- | --- |
| [3] | ZTE | As agreed for PUSCH repetition type A in maintenance session, counting based on available slot only applies to repetition number K>1. Therefore the components of FG 30-2 can be updated correspondingly. Regarding the prerequisite of FG 30-2, we see no much need to add FG 11-6 and 30-1 considering the fundamental FGs {5-14, 5-16, 5-17} have been included. If a UE needs to support available slot together with dynamic indication or increased maximum number of repetitions, it can always report related FGs together. With above, we have the following proposal. Proposal 1: For FG 30-2, adopt the following updates. Index; Feature group; Components; Prerequisite feature groups. 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for K>1[K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}. |
| [4] | Spreadtrum Communications | - FG 30-2 Component - According to [K], we think it can be change into K>1, and remove the [], based on the below agreement achieved in RAN1#108e. Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.. - Prerequisite FG 30-2: Regarding [11-6, 30-1], they can be removed. Because neither 11-6 nor 30-1 is the necessary requirement for available slot based repetition. |
| [6] | China Telecom | In our understanding, available slot counting can apply to Rel-15 PUSCH repetition (FG 5-14, 5-16, or 5-17), Rel-16 PUSCH repetition type A (FG 11-6) and Rel-17 enhanced PUSCH repetition type A (FG 30-1). We have the following proposal. Proposal 1: FGs for PUSCH repetition type A enhancements. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 30. NR_cov_enh; 30-1; Increased maximum number of PUSCH Type A repetitions; Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17.; 5-14, 5-16, or 5-17; UE does not support more than 16 repetitions PUSCH.; Per band; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}; UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots.; Per band; N/A; N/A; Optional with capability signalling. |
| [10] | Apple Inc. | Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported. 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}; Yes. For the prerequisite of FG30-2, FG30-2 is independent feature from FG30-1. FG30-2 and FG30-1 are two different repetition counting methods, UE can support FG30-2 without supporting FG30-1. FG11-6 is the enhancement for dynamic grant PUSCH and not related to configured grant PUSCH. If UE doesn’t support FG30-2 for dynamic grant PUSCH repetition type A based on available slot, then it’s not required to support FG11-6. Proposal 1: Prerequisite of FG30-2 doesn’t include FG11-6 and FG30-1. For the K values of FG30-2, there was no conclusion whether K=1 is supported under coverage enhancement session discussion. The same handling as FG30-1 can be adopted without waiting for the progress on coverage enhancement agenda. In this way, only listing maximum repetition number is enough. Proposal 2: Update FG30-2 components as: Transmission occasions for K repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. Maximum value of K (the number of repetitions) = 32. |
| [11] | CMCC | Proposal 1: The bracket around K could be removed for the component part of FG 30-2. Proposal 2: Either remove or keep the 11-6 and 30-1 as one of the prerequisite FG for FG 30-2 would be fine. Since if UE want to support Rel-16 repetitions based on available slot or maximum 32 repetitions based on available slots, it should claim to support the FG 11-6 or FG 30-1. |
| [12] | NTT DOCOMO, INC. | The relationship between FG 30-1 and 30-2 is the remaining discussion point, and following points are FFS points. 1. Number of repetitions (K) for FG 30-2 : 8 or 16 or 32 2. Prerequisite for FG 30-2 : including 30-1 or not Number of repetitions is independent from the FG 30-2, since it depends on whether UE supports repetitions of 16/32 or not, and FG 30-2 is for available slot counting. Therefore component of FG 30-2 does not need to include the number of repetition (K). And both available slot counting and increase number of repetitions can be configured simultaneously, FG 30-1 can be included in one of the prerequisite for FG 30-2. Proposal 1 : Remove number of repetition (K) from component of FG 30-2. Proposal 2 : FG 30-1 can be included in one of the prerequisite for FG 30-2. |
| [13] | Nokia, Nokia Shanghai Bell | - 30-1: - Add 11-6 as prerequisite since the increased maximum number of PUSCH repetitions is supported for numberOfRepetitions, which was introduced in 11-6, and remove 5-17 as the increased maximum number of PUSCH repetitions is not supported for pusch-AggregationFactor. - 30-2: - Replace list of pre-requisites with “one of {FG30-1 or 11-6}” as 30-1 already includes one of {5-14, 5-16, 5-17}. |
| [14] | Sharp | 30. NR_cov_enh; 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}; Yes; N/A; UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots.; Per band; N/A; N/A; N/A; Optional with capability signalling. In RAN1#108-e, FGs for PUSCH Type A repetitions were refined. However, there is still one remaining thing to be determined, which is the prerequisite FGs for FG 30-2. In our view, the UE supporting the available slot counting should be capable of some kind of PUSCH repetitions, i.e., at least one of Rel-15, 16 or 17 PUSCH repetitions, because it has been agreed that the available slot counting is applicable only for K>1. In this sense, “One of {5-14, 5-16, 5-17, 11-6, 30-1}” would be an appropriate prerequisite FGs for FG 30-2. At the same time, it is also true that UEs capable of FG 11-6 or 30-1 need to be capable of “one of {5-16, 5-17}” or “one of {5-14, 5-16, 5-17}”, respectively, anyway. Therefore, just putting “One of {5-14, 5-16, 5-17}” as prerequisite for FG 30-2 should also work. Proposal 1: For the prerequisite FGs for FG 30-2, “One of {5-14, 5-16, 5-17, 11-6, 30-1}” is slightly preferred. We can also live with “One of {5-14, 5-16, 5-17}”. |
| [15] | MediaTek Inc. | Considering the test cost and the applicability/support of the band (e.g., licensed band and unlicensed band), per band support is more preferred for all features. Proposal 1: All features are per band. All features of coverage enhancements are to improve performance rather than the basic features to build a new function. All features should be optional with capability signalling. Proposal 2: All UE features are optional with capability signalling |
| [16] | Intel Corporation | At the RAN1#108-e meeting, it was agreed to merge FGs 30-2 and 30-2a into an FG with per band and merge FGs 30-1 and 30-1a into an FG with per band with K = {20, 24, 28, 32} [1]. In addition, at the RAN1#106b-e meeting, it was agreed that maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32 [3]. In our view, for the UE feature groups for PUSCH repetition type A enhancement, all the K values should be supported for the case with counting based on available slots. Based on the discussions above, Table 1 illustrates suggested update for UE feature group for PUSCH repetition type A enhancement. Table 1. UE feature group for PUSCH repetition type A enhancement Index; Feature group; Components; Prerequisite feature groups. 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots., where K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}. Proposal 1 - For UE feature group of PUSCH repetition type A enhancement, - Consider Table 1 for UE feature group of PUSCH repetition type A enhancement. |
| [17] | Ericsson | There are two remaining open issues for FG 30-2, which were put in brackets in [1], including the value of K for available slot counting and the prerequisite feature groups. According to the following agreement made in RAN1#108 meeting, K=1 doesn’t apply to Available slot counting. The K>1 restriction for available slot counting has been captured in 38.214 v17.1.0, except for HD-FDD UE with DG-PUSCH, for which we provided a text proposal in our companion paper [5]. FG 30-2 is only about the available slot counting, so it would be clear without mentioning K in the column of components in UE feature list. Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.. Regarding the prerequisite feature groups, since the feature does not depend on 30-1 or 11-6 but can be combined with 30-1 and 11-6. The FGs {5-14, 5-16, or 5-17} would suffice. For Type A PUSCH repetition, the UE feature discussed so far is summarized and updated in Table 1. Table 1: Capabilities for PUSCH Repetition Type A Enhancement Index; Feature group; Components; Prerequisite feature groups; Comments. 30-2; PUSCH Type A repetitions based on available slots; Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.; One of {5-14, 5-16, 5-17, [11-6, 30-1]}. UE features for PUSCH Repetition Type A Enhancement are defined according to Table 1. |
| [18] | Qualcomm Incorporated | General remark applicable to all features for NR coverage enhancement Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. On PUSCH Type A repetition enhancements We make the following proposals to revise the UE capabilities for PUSCH Type A repetitions. Proposal 2: UE capabilities for enhanced PUSCH Type A repetitions are revised as follows: - The description of FG 30-2 is revised to clarify that PUSCH Type A repetitions based on available slot counting is only applicable when K > 1. - The prerequisites for FG 30-2 are one of {5-14. 5-16, 5-17}. |

Discussion

Low priority proposal 2-1:

- Companies are encouraged to provide views on whether to add FG 11-6 as a prerequisite FG for FG 30-1 and to remove FG 5-17 from the prerequisite FGs for FG 30-1

  - Nokia/NSB: Add 11-6 as prerequisite since the increased maximum number of PUSCH repetitions is supported for numberOfRepetitions, which was introduced in 11-6, and remove 5-17 as the increased maximum number of PUSCH repetitions is not supported for pusch-AggregationFactor.

| Company | Comment |
| --- | --- |
| Ericsson | Agree the prerequisite FGs for FG 30-1 can be 5-14, 5-16, or 11-6. DG-PUSCH with increased number of repetitions can only be configured via numberOfRepetitions in TDRA table. |

Low priority proposal 2-1:

- Companies are encouraged to provide views on which FG should be the prerequisite FGs for FG 30-2

  - Opt.1: one of {5-14, 5-16, 5-17, 11-6, 30-1}: Huawei, China Telecom, CMCC, DOCOMO, SHARP, Intel

    - Any features to support repetition type A can be one of the prerequisite feature groups for FG 30-2

  - Opt.2: one of {5-14, 5-16, 5-17} : ZTE, Spreadtrum, Apple, CMCC, SHARP, Ericsson, Qualcomm

    - Neither 11-6 nor 30-1 is the necessary requirement for available slot based repetition

    - If a UE needs to support available slot together with dynamic indication or increased maximum number of repetitions, it can always report related FGs together

  - Opt.3: one of {FG30-1 or 11-6} : NOKIA

    - 30-1 already includes one of {5-14, 5-16, 5-17}

| Company | Comment |
| --- | --- |
| Ericsson | Support Option 2. FG 30-2 does not depend on 30-1 or 11-6 but can be combined with 30-1 and 11-6. The FGs {5-14, 5-16, or 5-17} would suffice. |

Low priority proposal 2-4:

- Component of FG 30-2 is revised as: Transmission occasions for [K]>1 repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots.

| Company | Comment |
| --- | --- |
| Moderator | Following was agreed in RAN1#108-e Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.. |
| Ericsson | 30-2 is mainly about the available slot counting. It is different from 30-1, so we don’t see strong motivation to add K in the component. |

- 30-3 to 30-3a: TB processing over multi-slot PUSCH

In [1], FGs 30-3 to 30-3a are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | The features for TBoMS should not be associated with the feature of repetition type A. So it is sufficient that the prerequisite feature group for FG 30-3 is None and the prerequisite feature group for FG 30-3a is FG 30-3. Proposal 2: For the features for TBoMS, it is sufficient that - Prerequisite feature group for FG 30-3 is None. - Prerequisite feature group for FG 30-3a is FG 30-3. |
| --- | --- | --- |
| [3] | ZTE | For FG 30-3, we don’t think a prerequisite is needed since TBoMS is a totally new functionality and have different UE behavior compared to legacy PUSCH repetition type A. For FG 30-3a, we think it only depends on FG 30-3, and it doesn’t unnecessarily require the UE to support legacy PUSCH repetition. As a result, we have the following proposal. Proposal 2: For FGs for TBoMS, no prerequisite is needed for FG 30-3 and the prerequisite of FG 30-3a is FG 30-3. |
| [4] | Spreadtrum Communications | - Prerequisite - FG 30-2: Regarding [11-6, 30-1], they can be removed. Because neither 11-6 nor 30-1 is the necessary requirement for available slot based repetition. - FG 30-3: FG 11-6. Because TBoMS can be indicated by TDRA in DCI which TDRA table is configured with TBoMS resource allocation - FG 30-3a: FG 30-3. |
| [5] | vivo | In RAN1#106-e meeting, it has been agreed that the number of slots for TBoMS is counted based on available slots for UL transmission, and the available slots are determined based on the mechanisms determined for Type A PUSCH repetition counted on available slots. Hence, feature 30-2, i.e., type-A PUSCH repetitions counted on available slots, should be considered as prerequisite feature for feature 30-3. Proposal 1: For TBoMS, feature 30-2 should be considered as prerequisite feature for feature 30-3. |
| [6] | China Telecom | Proposal 2: FGs for TB processing over multi-slots. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 30. NR_cov_enh; 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode.; [11-6]; UE does not support TB processing over multi-slot PUSCH.; Per band; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-3a; Repetition of TB processing over multi-slot PUSCH; Support repetition of TB processing over multi-slot PUSCH in RRC connected mode.; TBD 30-3; UE does not support repetition of TB processing over multi-slot PUSCH.; Per band; N/A; N/A; Optional with capability signalling. |
| [7] | Xiaomi | For the prerequisite FG, although FG 30-3 utilize the similar time domain resource allocation mechanism with FG 11-6 which is the enhanced PUSCH repetition type A in Rel-16, there are still many differences between these two features, such as TBS calculation, RV determination for each allocated slot, rate matching mechanism and so on. Therefore, it is difficult to say that the FG 11-6 is the prerequisite feature group for FG 30-3. Besides, several companies proposed that FG 30-2 is the prerequisite feature group for FG 30-3 since available slots determination for TBoMS just follows the same design of enhanced PUSCH repetition type A in Rel-17. However, from our point of view, in unpaired spectrum or paired spectrum for HD-FDD UEs, it is mandatory to determine N slots allocated for TBoMS based on available slots but not consecutive slots. In addition, considering that FG 30-2 still works based on the feature of PUSCH repetition type A in Rel-15, we can’t agree with this proposal. Proposal 1: Determine N allocated slots for single TBoMS based on available slots is mandatory in unpaired spectrum or paired spectrum for HD-FDD UEs. Proposal 2: Don’t support taking FG 11-6 and FG 30-2 as the prerequisite feature groups for FG 30-3. In addition, TBoMS PUSCH is supported in both FR1 and FR2 bands, as well as in both TDD and FDD bands, and the general design is the same for these spectrum bands. Thus, we think there is no need to distinguish FG 30-3 between FDD and TDD bands, as well as between FR1 and FR2 bands. Proposal 3: No need to differentiate FG 30-3 between FDD and TDD bands, as well as between FR1 and FR2 bands. For the time domain resource allocation, TBS calculation and RV determination across multiple repetitions of TBoMS, it just follows the same rule as PUSCH repetition type A. While, as for time domain resource allocation, TBS calculation and RV determination within one repetition of TBoMS, it follows the same rule as single TBoMS PUSCH. Therefore, we can take the repetition of TBoMS as the combination of a single TBoMS PUSCH and PUSCH repetition type A. Thus, FG 11-6 in Rel-16 and FG 30-3 should be the prerequisite FGs for FG 30-3a. In addition, as for N*K slots counting based on available slots for repetition of TBoMS in unpaired spectrum or in paried spectrum for HD-FDD UEs, it follows the same principle as single TBoMS PUSCH. Proposal 4: FG 11-6 and FG 30-3 should be the prerequisite FGs for FG 30-3a. Proposal 5: Determine N*K slots based on available slots is mandatory for repetition of TBoMS in unpaired spectrum or paired spectrum for HD-FDD UEs. |
| [9] | OPPO | It is to be discussed whether to add an FG for the maximum concurrent TBoMS transmissions supported by a UE across all carriers when operating in UL-CA. The support of concurrent TBoMS transmissions across UL carriers when operating in UL-CA is still an open issue in AI 8.8.1.2. In our view, it should be first clarified whether UL-CA is a target scenario for coverage enhancement in AI 8.8.1.2. Then we can decide the support of concurrent TBoMS transmissions across UL carriers when operating in UL-CA. Finally, the corresponding UE feature for the maximum concurrent TBoMS transmissions can be discussed in UE feature session. Anyway, we prefer to defer this issue until some progress is made in AI 8.8.1.2. Proposal 1：Wait for the outcome of AI 8.8.1.2 before discussing whether to add an FG for the maximum concurrent TboMS transmissions supported by a UE across all carriers when operating in UL-CA. |
| [11] | CMCC | Proposal 3: The prerequisite of FG 30-3a contains FG 30-3. Proposal 4： It should not set up a dependency between FG 30-3 (TBOMS) and FG 30-2/2a (PUSCH Type A repetitions based on available slots) Proposal 5: No prerequisite for FG 30-3 is preferred. |
| [12] | NTT DOCOMO | Proposal 3: There is no prerequisite FG of FG 30-3. Proposal 4: The prerequisite FGs of FG 30-3a should be FG 11-6, FG 30-2, and FG 30-3. |
| [13] | Nokia, Nokia Shanghai Bell | - 30-3: - No need to have 11-6 as pre-requisite. |
| [14] | Sharp | One of them is the prerequisite FGs for FG 30-3. In RAN1#108-e, it was concluded that TBoMS with Type-1 CG would not be supported. As the number of allocated slot for a single TBoMS needs to be indicated by TDRA field value of the DCI format 0_1 or 0_2, TBoMS can be considered to require the support of the dynamic repetition factor indication, i.e., FG 11-6. Proposal 2: The prerequisite FGs for FG 30-3 should be “11-6”. Another remaining issue related to TBoMS is the prerequisite FGs for FG 30-3a. As FG 30-3a is about repetitions of TBoMS, it is straightforward that FG 30-3a requires the single-TBoMS feature, i.e. FG 30-3, as well as the dynamic repetition factor indication feature, i.e. FG 11-6. Proposal 3: The prerequisite FGs for FG 30-3a should be “11-6 and 30-3”. |
| [16] | Intel Corporation | Table 2. UE feature groups for TBoMS Index; Feature group; Components; Prerequisite feature groups. 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH for DG and Type-2 CG in RRC connected mode.; [11-6]. 30-3a; Repetition of TB processing over multi-slot PUSCH; Support repetition of TB processing over multi-slot PUSCH in RRC connected mode.; TBD One of {5-16, 11-6, 30-1}, 30-3. Proposal 2 - For UE feature groups of TBoMS, - Consider Table 2 for UE feature groups of TBoMS. |
| [17] | Ericsson | 30-3: With the addition of 30-3a, 30-3 is then about UE capability of a single TBoMS. Although TBoMS uses Type A-like resource allocation, it is a different type of UL transmission. FG 11-6 is the enhanced PUSCH repetition Type A in Rel-16 with the number of repetitions jointly coded with SLIV in TDRA table. We don’t see the need of a prerequisite feature beyond basic PUSCH transmission. It was agreed in RAN1#108e not to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG. FG 30-3 is applicable to DG-PUSCH and Type 2 CG-PUSCH, but a UE doesn’t have to support both DG-PUSCH and Type 2 CG-PUSCH in order to support 30-3. Therefore, we propose to change the word “and” to “or” in the Components column. Note that since 30-3a was agreed not to be merged with 30-3 in RAN1#107bis-e, we should clarify that 30-3 is without repetition. 30-3a: Its prerequisite FGs should be 30-3 and 11-6. Table 2: Capabilities for Transport Block over Multi-slot PUSCH Index; Feature group; Components; Prerequisite feature groups; Comments. 30-3; TB processing over multi-slot PUSCH; Support of TB processing over multi-slot PUSCH for DG and or CG without repetition in RRC connected mode.; [11-6]. 30-3a; Repetition of TB processing over multi-slot PUSCH; Support Repetition of TB processing over multi-slot PUSCH in RRC connected mode.; TBD30-3, 11-6. UE features for transport block over multi-slot PUSCH are defined according to Table 2. |
| [18] | Qualcomm Incorporated | We make the following proposals to revise the UE capabilities for TBOMS. Proposal 5: UE capabilities for TBOMS are revised as follows: - The prerequisites for FG 30-3 are 30-2. - The prerequisites for FG 30-3a are 30-3. |

Discussion

High priority question 3-1:

- Companies are encouraged to provide views on whether to add an FG for the maximum concurrent TboMS transmissions supported by a UE across all carriers when operating in UL-CA

  - Wait for the discussion in AI 8.8.1: OPPO

| Company | Comment |
| --- | --- |
| Moderator | As discussed in last RAN1 meeting, this issue can be discussed after some progress is made in AI8.8.1. |
| CMCC | UL-CA is focused on increasing the UL throughput, and the TBoMS mainly aim to enhance the coverage. In a coverage limited scenario, UE is assumed to transmit in the maximum power, , in which case UE does not have additional power for UL-CA. UL-CA and TBoMS should not perform together in a UE. We are open to wait for the discussion in AI 8.8.1 . |
| Intel | We share similar view as CMCC. This issue was discussed but it was concluded to deprioritize for TBoMS in case of CA. |
| QC | We are okay to discuss this separately under AI 8.8.1. |
| Sharp | We are ok to wait for the discussion under AI 8.8.1, |
| Panasonic | We support the view to wait the discussion under AI 8.8.1. For CA and TBoMS, we share similar view with CMCC. As this is questioned for DC, can we understand that DC and TBoMS are independent can be concluded? |
| Ericsson | Concurrent TBoMS transmission in CA is not being discussed at this meeting in my understanding. However, the need for such an FG should first be discussed in 8.8.1 in our view. |
| OPPO | We are ok to wait for the discussion on UL-CA and TBoMS under AI 8.8.1. Actually we have similar view as CMCC. |

Low priority question 3-2:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-3

  - No prerequisite FG: Huawei, HiSilicon, ZTE, China Telecom, Xiaomi, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Ericsson

    - The features for TBoMS should not be associated with the feature of repetition type A

    - TBoMS is a totally new functionality and have different UE behavior compared to legacy PUSCH repetition type A

  - FG 30-2: vivo, Qualcomm

    - the number of slots for TBoMS is counted based on available slots for UL transmission, and the available slots are determined based on the mechanisms determined for Type A PUSCH repetition counted on available slots

  - FG 11-6:  Spreadtrum, Intel, Sharp

    - TBoMS can be indicated by TDRA in DCI which TDRA table is configured with TBoMS resource allocation

| Company | Comment |
| --- | --- |
| Ericsson | No prerequisite FG is needed for FG 30-3. TboMS does not depend on Repetition Type A in the current specifications. |

Low priority question 3-3:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-3a

  - FG 30-3: Huawei, HiSilicon, ZTE, Spreadtrum, China Telecom, CMCC, Sharp, Qualcomm

    - it only depends on FG 30-3, and it doesn’t unnecessarily require the UE to support legacy PUSCH repetition

  - FG 11-6 and FG 30-3: Xiaomi, Sharp, Ericsson

    - the repetition of TBoMS as the combination of a single TBoMS PUSCH and PUSCH repetition type A

  - FG 11-6 and FG 30-2 and FG 30-3: NTT DOCOMO

  - FG 5-16 or 11-6 or 30-1 and FG 30-3: Intel

| Company | Comment |
| --- | --- |
| Ericsson | We support FG 11-6 and FG 30-3. The single TBoMS and Rel-16 PUSCH repetition Type A are the prerequisite FGs of TBoMS repetition. |

Low priority question 3-4:

- Companies are encouraged to provide views on whether to replace “DG and CG” with “DG and Type2 CG” in FG 30-3

  - Support: Intel

| Company | Comment |
| --- | --- |
| Moderator | Following conclusion was made in RAN1#108-e Conclusion There is no consensus in RAN1 to support TBoMS for CG-Type 1. |
| Ericsson | Support. As moderator points out Type 1 CG is not supported for TBoMS. |

- 30-4 to 30-4h: DM-RS bundling

In [1], FGs 30-4 to 30-4h are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [30-4] | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [30-4a, 30-4b, 30-4c, or 30-4d] | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | Considering the maximum number of repetition, 16 slots should be included in the set of maximum duration for TDD. And according to the above, a Note for FG 30-4 should include: - Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}. - Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}. Proposal 3: A Note for FG 30-4 should be added as: - Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}. - Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}. Table 1 Prerequisites Index; Feature group; Prerequisite feature groups. 30-4; The maximum duration for DM-RS bundling; FFS. 30-4a; DM-RS bundling for PUSCH repetition type A; [30-4], [30-1] or [30-2] one of {5-14, 5-16, 5-17, 11-6, 30-1}. 30-4b; DM-RS bundling for PUSCH repetition type B; [30-4], [11-5] [30-1]. 30-4c; DM-RS bundling for TB processing over multi-slot PUSCH; [30-4], [30-3] or 30-3a. 30-4d; DMRS bunding for PUCCH repetitions; [30-4], [4-23] or 25-2. 30-4e; Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c]. 30-4f; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]. 30-4g; Restart DM-RS bundling after the events that violate power consistency and phase continuity; [30-4]. 30-4h; DM-RS bundling for non-back-to-back transmission; [30-4a, or 30-4b, or 30-4c or 30-4d]. Proposal 4: Adopt the changes to prerequisites for FG 30-4a/4b/4c/4d/4e/4f/4g/4h as shown in the last column of Table 1. First of all, for a given tolerance, the CFOs of different operating frequency are different, which may cause the “compensation leftover for frequency error” be different. And UE’s implementation and RF characteristics on different band to maintain the phase continuity could be different. So it is reasonable to define that the UE features of DMRS bundling is set to per band. Actually, FG 30-4 that has been set per band, should be a direct or indirect prerequisite to all other FGs of DMRS bundling. Therefore, per UE for FG 30-4a/4b/4c/4d/4e/4f/4g/4h is sufficient. Proposal 5: The type of the granularities for FG 30-4a/4b/4c/4d/4e/4f/4g/4h are per UE. For FG 30-4a/4b/4c/4d/4e/4f/4g/4h, FR1/FR2 differentiation and TDD/FDD differentiation are not necessary. And the need for FR1/FR2 differentiation and TDD/FDD differentiation should be N/A. Proposal 6: For FG 30-4a/4b/4c/4d/4e/4f/4g/4h, - No for the need for FR1/FR2 differentiation and TDD/FDD differentiation. - N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2. |
| --- | --- | --- |
| [3] | ZTE | Regarding above set of values can be reported by UE, it should be added in the note of FG 30-4. For TDD, we can keep value 16 in the set of values similar as value 32 for FDD. As for the prerequisite, we don’t see any dependency on other FGs. Therefore, we have the following proposal. Proposal 3: For FG 30-4, no prerequisite is needed, and the following note should be added. - The set of values of maximum duration lengths for FDD are {4, 8, 16, 32} slots. - The set of values of maximum duration lengths for TDD should are {2, 4, 8, 16} slots. Regarding the UE reporting granularity, we think it’s sufficient for reporting by per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them. Proposal 4: For FG 30-4x, the prerequisites are setting as follows, and is reported per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them - FG 30-4a - FG 30-4 and FG 5-14 or 5-16 or 5-17 - FG 30-4b - FG 30-4 and FG 11-5 - FG 30-4c - FG 30-4 and FG 30-3 - FG 30-4d - FG 30-4 and FG 4-23 - FG 30-4e - FG 30-4a or 30-4b or 30-4c - FG 30-4f - FG 30-4d, 4-23 - FG 30-4g - FG 30-4 - FG 30-4h - FG 30-4a or 30-4b or 30-4c or 30-4d |
| [4] | Spreadtrum Communications | - Type: - FGs 30-4 is per band. Since DM-RS bundling cannot be implemented for some band combination. - FGs 30-4x: per band per BC. Agree with the explanation that DMRS bundling can be done in some bands, but not in these band combinations. - Prerequisite - FG 30-4: N/A - FG 30-4a: FG 30-4 and one of {5-14, 5-17, 5-16}. - Do not need FG 30-1 or 30-2. DM-RS bundling can be supported without larger PUSCH repetition number, or available slot. - FG 30-4b: FG 30-4 and 11-5. - Do not need 30-1. Because FG 30-1 is for PUSCH repetition type A, it is not related with PUSCH repetition type B. - FG 30-4c: confirm FG 30-4 and 30-3 - FG 30-4d: confirm FG 30-4 and 4-23 - FG 30-4e: confirm 30-4a or 30-4b, or 30-4c - FG 30-4f: confirm 30-4d |
| [5] | vivo | For DMRS bundling, FG 30-4 is per band, and UEs can have different capabilities of maintaining phase continuity and power consistency on different bands. Similarly, FG 30-4a to FG 30-4h should also be per band. Proposal 2: For DMRS bundling, FG 30-4a to FG 30-4h should be per band. |
| [6] | China Telecom | Proposal 3: FGs for DMRS bundling for PUSCH/PUCCH. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 30. NR_cov_enh; 30-4; The maximum duration for DM-RS bundling; The maximum duration during which UE is able to maintain power consisitency consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH. For paired spectrum and SUL, the set of values are {4, 8, 16, 32} For unpaired spectrum, the set of values are {2, 4, 8, 16}; FFS; UE does not support DM-RS bundling for PUSCH/PUCCH; Per band; N/A; N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4a; DM-RS bundling for PUSCH repetition type A; Support DM-RS bundling for PUSCH repetition type A; [30-4], One of {5-14, 5-16, 5-17, 11-6, [30-1]} or [30-2]; UE does not Support DM-RS bundling for PUSCH repetition type A; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4b; DM-RS bundling for PUSCH repetition type B; Support DM-RS bundling for PUSCH repetition type B; [30-4], [11-5] [30-1]; UE does not Support DM-RS bundling for PUSCH repetition type B; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4c; DM-RS bundling for TB processing over multi-slot PUSCH; Support DM-RS bundling for TB processing over multi-slot PUSCH; [30-4], [30-3]; UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH; [Per UE] Per band; FFS N/A; [No] N/A; Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS; Optional with capability signalling. 30. NR_cov_enh; 30-4d; DMRS bunding for PUCCH repetitions; Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4; [30-4], [4-23]; UE does not support DMRS bunding for PUCCH repetitions; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4e; Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH; Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c]; UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4f; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]; UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity]; [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.; [30-4]; [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity]; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. 30. NR_cov_enh; 30-4h; DM-RS bundling for non-back-to-back transmission; Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)]; [30-4a, or 30-4b, or 30-4c, or 30-4d]; UE does not Support DM-RS bundling for non-back-to-back transmission; [Per UE] Per band; FFS N/A; [No] N/A; Optional with capability signalling. |
| [7] | Xiaomi | For FG30-4g, it has been agreed in last RAN1 meeting that UE is mandatory to support restarting DM-RS bundling due to semi-static events. Therefore, UE capability of restarting DMRS bundling is applied only to dynamic events. FG30-4g component need to update accordingly. Proposal 6: Updating the FG30-4g to apply to dynamic event only. |
| [8] | Samsung | Proposal 1: Define candidate values for FG 30-4 for FDD and TDD as following: - FDD: {4, 8, 16, 32} slots - TDD: {2, 4, 8, 16} slots Proposal 2: Update 30-4g as following in alignment with the corresponding RRC parameter description. 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity]; [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.. |
| [9] | OPPO | It is FFS whether the type of FGs 30-4, 30-4a to 30-4g and 30-4h should be per UE, per band, or per FS. In our view, per band FG is preferred for DMRS bundling feature, unless RAN4 has some concerns on it. Proposal 2：Type of FGs 30-4, 30-4a to 30-4h is per band. |
| [10] | Apple | Regarding the type of FG30-4a to FG30-4g, as the basic function FG30-4 maximum duration is per band indication, thus the following DMRS bundling features could indicate per band as well. As for per feature set reporting, it’s not necessary to share the DMRS bundling capability among the bands, especially supporting CA is not the typical use case for UL coverage limited UE. Proposal 3: The type of FG30-4a to FG30-4g is per band. |
| [11] | CMCC | Proposal 6: The prerequisite FG of FG 30-4 should be vacant. The prerequisite FG of FG 30-4a should be FG 30-4 and one of the {5-14,5-16,5-17}. The prerequisite FG of FG 30-4b should be FG 30-4 and FG 11-6. The prerequisite FG of FG 30-4c should be FG 30-4 and FG 30-3. The prerequisite FG of FG 30-4d should be FG 30-4 and FG 4-23. The prerequisite FG of FG 30-4e should be one of FG {30-4a, 30-4b or 30-4c}. The prerequisite FG of FG 30-4f should be FG 30-4d. Proposal 7: FG 30-4a to 30-4h should be per band reported and differentiated between FR1 and FR2. Proposal 8 No need to differentiate TDD and FDD for FG 30-4a to FG 30-4h. Proposal 9: Remove the bracket for Feature group and Components in FG 30-4h. |
| [12] | NTT DOCOMO | In Rel-17 joint channel estimation, the length of nominal TDW that UE is configured with is up to the maximum duration. For those reasons, 16 slots should be included in the set of values for TDD in UE capability, to achieve the coverage enhancements in a variety of TDD patterns. Proposal 5: 16 slots should be included in the set of maximum duration values for TDD. Types of FGs 30-4a to 30-4h are still controversial [2]. As LS from RAN4 suggests the final decision on the granularity of UE capability to be made in RAN1, RAN1 should make the decision on it [3]. At the RAN1#108-e meeting, companies argued either per band or {per band and per BC}. In our view, the difference between per FS and {per band and per BC} is unclear. Also, we think per band is sufficient for FGs 30-4a to 30-4h, unless the explicit technical reasons why {per band and per BC} is necessary are clear. Proposal 6: FGs 30-4a to 30-4h can be supported per band. The prerequisite FGs for 30-4a, 30-4b, 30-4c, 30-4d, 30-4e, 30-4f, 30-4g, and 30-4h can be {FG 30-4 and FGs 5-14 or 5-16 or 5-17}, {FGs 30-4 and 11-5}, {FGs 30-4 and 30-3}, {FGs 30-4 and 4-23}, {FGs 30-4a or 30-4b or 30-4c}, {FG 30-4d}, {FG 30-4}, and {FGs 30-4a or 30-4b or 30-4c or 30-4d}, respectively. Proposal 7: The prerequisite FGs for 30-4a, 30-4b, 30-4c, 30-4d, 30-4e, 30-4f, 30-4g, and 30-4h can be {FG 30-4 and FGs 5-14 or 5-16 or 5-17}, {FGs 30-4 and 11-5}, {FGs 30-4 and 30-3}, {FGs 30-4 and 4-23}, {FGs 30-4a or 30-4b or 30-4c}, {FG 30-4d}, {FG 30-4}, and {FGs 30-4a or 30-4b or 30-4c or 30-4d}, respectively. |
| [13] | Nokia, Nokia Shanghai Bell | - 30-4: - The reporting of this feature should be so that either it includes all possible values supported in the specifications, or else UE should report a value of maximum duration from a set of candidate values. Further details can be found in our companion contributions in [3][4]. Therefore, the following should be added to the description of “Components”: - “ UE reports one value of maximum duration from the following set of candidate values: - For FDD, maximum duration = 4, 8, 16 or 32 slots. - For TDD, maximum duration = 2, 4, 8, [or 16] slots.” - 30-4a/b/c/d: - Per band seems appropriate, considering the UE needs to maintaing consistency among DM-RS transmissions, and that might be band-dependent. - 30-4e: - Confirm prerequisites one of {30-4a, 30-4b, 30-4c}. - Per UE is sufficient if prerequisites are agreed per band. No differentiation needed. - 30-4g: - Description of "Feature group" should be (changes are highlighted in red): - "Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity" - Description of "Components" should be (changes are highlighted in red): - "Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity, except events which are triggered by DCI or MAC CE but regarded as semi-static events" - Description of "Consequence if the feature is not supported by the UE" should be (changes are highlighted in red): - "UE does not support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity" - Per UE is sufficient if prerequisites are agreed per band. No differentiation needed. - 30-4h: - It is not consistent if 30-4h is the only place where it is indicated that 30-4a/b/c/d are limited to back-to-back cases. The simpler solution seems to be to update the description of 30-4/a/b/c/d to indicate this restriction explicitly, in which case the yellow highlighted text in 30-4h can be confirmed as is. - Confirm the prerequisites |
| [15] | MediaTek Inc. | Considering the test cost and the applicability/support of the band (e.g., licensed band and unlicensed band), per band support is more preferred for all features. Proposal 1: All features are per band. Proposal 3: By default, UE is not expected to support the combined feature A+B if feature A and feature B are indicated and supported separately. For 30-4h, the yellow part seems stable without any comments in the last meeting. So it can be confirmed if there is no more concern. 30. NR_cov_enh; 30-4h; DM-RS bundling for non-back-to-back transmission; Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d); 30-4a, 30-4b, 30-4c, or 30-4d. Proposal 4: Confirm the components and perquisite FGs for 30-4h. |
| [16] | Intel Corporation | Table 3. UE feature groups for DMRS bundling Index; Feature group; Components; Prerequisite feature groups. 30-4a; DM-RS bundling for PUSCH repetition type A; Support DM-RS bundling for PUSCH repetition type A; [30-4], [30-1], or [30-2], one of {5-14, 5-16, 5-17, 11-6}. 30-4b; DM-RS bundling for PUSCH repetition type B; Support DM-RS bundling for PUSCH repetition type B; [30-4], [11-5] [30-1]. 30-4c; DM-RS bundling for TB processing over multi-slot PUSCH; Support DM-RS bundling for TB processing over multi-slot PUSCH; [30-4], [30-3], 30-3a. 30-4d; DMRS bunding for PUCCH repetitions; Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4; [30-4], [4-23]. 30-4e; Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH; Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c]. 30-4f; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d]. 30-4g; [Restart DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity]; [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g., frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.; [30-4] 30-4a, 30-4b, 30-4c, or 30-4d. 30-4h; DM-RS bundling for non-back-to-back transmission; Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)], respectively,; [30-4a, 30-4b, 30-4c, or 30-4d]. Proposal 3 - For UE feature groups of DMRS bundling for PUSCH and PUCCH - UE features for DMRS bundling are defined per band. - FDD/TDD differentiation is not necessary. - Consider Table 3 for UE feature groups of DMRS bundling. |
| [17] | Ericsson | 30-4{a,b,c,d}: Since FG 30-4h was agreed to support the non-back-to-back case, the text ‘over consecutive symbols’ should be added to these feature groups to indicate that they only support back-to-back transmission. Otherwise, the description of features 30-4{a,b,c,d} do not stand on their own, and the reader has to infer from 30-4h what is not supported in 30-4{a,b,c,d}. 30-4: The typo ‘consisitency’ should be corrected. 30-4a: There does not seem to be a need for 30-1 or 30-2 as a prerequisite, since neither an extended number of repetitions nor available slot determination is needed to make DMRS bundling work (although these could be supported with bundling). However, DMRS bundling for PUSCH will be supported for Rel-15, -16, and -17 PUSCH repetition Type A. We note that 11-6, PUSCH Repetition Type A, depends on one of 5-16 and 5-17. Since prerequisites are what is required to configure a feature, and not what can be configured with a feature, this dependency on 5-16 and 5-17 means that 11-6 is not technically a prerequisite. We note that this has already been discussed for 30-1 and 30-2, and that 5-14, 5-16, or 5-17 are listed as prerequisites for 30-1 and 30-2. Therefore, the prerequisites are 5-14, 5-16, or 5-17 as well as 30-4 for 30-4a. 30-4b: The working assumption confirmed in RAN1#106 identifies support for PUSCH repetition type B in a UE configured joint channel estimation as an additional UE capability. Therefore, in addition to the basic support of DMRS bundling, the UE will also need to support PUSCH repetition Type B, i.e. Rel-16 feature 11-5. There does not seem to be a need to have 30-1 as a prerequisite, since the extended number of repetitions is not needed to make DMRS bundling work. 30-4c: Depending on only TBoMS and maximum duration is sufficient in our view, so the square brackets can be removed. 30-4d: As suggested (tentatively) by the moderator, the Rel-15 feature for PUCCH repetition (4-23, ‘Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8’), seems needed for PUCCH bundling for slots format 1/3/4. 30-4e: As discussed in [2] and [3], new inter-slot frequency hopping patterns provide notable gains for PUSCH joint channel estimation when DMRS bundling is not configured. PUSCH Type A scheduling (2-16) is assumed as a baseline and so should be a prerequisite. Especially since UEs in a cell not supporting JCE or TBoMS should be able to hop with UEs configured for JCE and/or TBoMS, and since enhanced frequency hopping patterns can have gains for UEs not configured for JCE and/or TBoMS, 2-16 should be a sufficient prerequisite for PUSCH inter-slot frequency hopping, and e.g. 30-4{a,b,c} should not be prerequisites. 30-4f: After simulating PUCCH frequency hopping according to a new pattern designed originally for joint channel estimation, we find that the pattern is beneficial for PUCCH as well as for PUSCH, as can be seen in our contribution [4]. PUCCH repetition 4-23 can be a starting point for a prerequisite for enhanced inter-slot frequency hopping for PUCCH. 30-4g: It was agreed in RAN1#107 that the UE capability of restarting DMRS bundling is applied only to dynamic events. Dynamic events are not defined in 38.214; instead events for which power consistency and phase continuity is maintained over an actual TDW that is created in response to specific events are identified. Therefore, we suggest the component should be ‘Support power consistency and phase continuity over an actual TDW created in response to events identified in 38.214 subclause 6.1.7, where such support of consistency/continuity for the event requires UE capability.’. 30-4h: The feature as added after RAN1#107bis seems clear and correct to us as written, and so the square brackets can be removed. Regarding the Type (per UE, per band, etc.) for 30-4a to 30-4h: the following proposal was made in RAN1#108, but required further discussion and was left for later meetings. Table 3b: Capabilities for PUSCH and PUCCH Joint Channel Estimation Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 30-4; The maximum duration for DM-RS bundling; The maximum duration during which UE is able to maintain power consisitency consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH; FFS; Per Band. 30-4a; DM-RS bundling for PUSCH repetition type A; Support DM-RS bundling for PUSCH repetition type A over consecutive symbols; [30-4], and [30-1] or [30-2] [{5-14, 5-16, or 5-17}]; [Per UE] Per Band and Per BC, according to the approach above.. 30-4b; DM-RS bundling for PUSCH repetition type B; Support DM-RS bundling for PUSCH repetition type B over consecutive symbols; [30-4], [11-5] [30-1]; [Per UE] Per Band and Per BC, according to the approach above.. 30-4c; DM-RS bundling for TB processing over multi-slot PUSCH; Support DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols; [30-4], [30-3]; [Per UE] Per Band and Per BC, according to the approach above.. 30-4d; DMRS bunding for PUCCH repetitions; Support DM-RS bundling for PUCCH repetitions over consecutive symbols for PUCCH formats 1/3/4; [30-4], [4-23]; [Per UE] Per Band and Per BC, according to the approach above.. 30-4e; Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH; Support enhanced inter-slot frequency hopping pattern with inter-slot bundling for PUSCH; [30-4a] or [30-4b] or [30-4c] 2-16; [Per UE]. 30-4f; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling; [30-4d[ 4-23; [Per UE]. 30-4g; [Restart DM-RS bundling after the events that violate power consistency and phase continuity]; [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] Support power consistency and phase continuity over an actual TDW created in response to events identified in 38.214 subclause 6.1.7, where such support of consistency/continuity for the event requires UE capability.; [30-4], and {30-4a, 30-4b, or 30-4c}; [Per UE] Per Band and Per BC, according to the approach above.. 30-4h; DM-RS bundling for non-back-to-back transmission; Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) ]; [30-4a, 30-4b, 30-4c, or 30-4d]; [Per UE] Per Band and Per BC, according to the approach above.. UE features for PUSCH and PUCCH joint channel estimation are defined according to Tables 3a and 3b |
| [18] | Qualcomm Incorporated | We have the following proposals on UE capability reporting for PUSCH/PUCCH DMRS bundling. Proposal 3: On UE features 30-4 to 30-4g: all features on DMRS Bundling (PUSCH and PUCCH) to be indicated at the per FS granularity. Proposal 4: On UE feature 30-4 for maximum duration for DMRS bundling, since a UE may report different values for different modulation orders, choose from the following two options: - restrict applicability of 30-4 to QPSK or lower modulation order, or - introduce new FGs to cover the remaining modulation orders. |

Discussion

[FL1] High priority proposal 4-1:

- Add the following note in FG 30-4

  - Candidate values for the maximum duration for FDD are {4, 8, 16, 32}

    - Support: Huawei, HiSilicon, ZTE, China Telecom, Samsung, Nokia, Nokia Shanghai Bell

  - Candidate values for the maximum duration for TDD are {2, 4, 8, 16}

    - Support: Huawei, HiSilicon, ZTE, China Telecom, Samsung, NTT DOCOMO, [Nokia/NSB]

      - 16: to achieve the coverage enhancements in a variety of TDD patterns

| Company | Comment |
| --- | --- |
| CMCC | Fine with this proposal. We also support the candidate value {16} in the maximum duration for TDD for potential usage. |
| ZTE | Support the proposal. |
| Intel | We are fine with the proposal. For 16 in TDD, we support this for coverage enhancement. |
| Nokia, NSB | OK |
| QC | Support |
| NTT DOCOMO | Support the proposal. |
| Sharp | Fine with the proposal. |
| Moderator | All companies are fine with the proposal. This proposal can be confirmed quickly in the GTW [GTW1] High priority proposal 4-1: - Add the following note in FG 30-4 - Candidate values for the maximum duration for FDD are {4, 8, 16, 32} - Candidate values for the maximum duration for TDD are {2, 4, 8, 16} |
| Panasonic | We support the proposal. |
| Ericsson | Support the proposal |
| OPPO | Support the proposal |
| Moderator | Following was agreed in the GTW on May 10 Agreement - Add the following note in FG 30-4 - Candidate values for the maximum duration for FDD are {4, 8, 16, 32} - Candidate values for the maximum duration for TDD are {2, 4, 8, 16} |

[FL1] High priority proposal 4-2:

- Reporting type of FGs 30-4a to 30-4h is per band

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Per UE: Huawei, HiSilicon, ZTE, Nokia/NSB (for FGs 30-4e/g) - Per band as FG 30-4 is sufficient, and FG 30-4 should be prerequisite of all other FGs of DMRS bundling - TDD/FDD differentiation - Not necessary: Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell - FR1/FR2 differentiation - Not necessary: Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell - Per band: vivo, China Telecom, OPPO, Apple, CMCC, NTT DOCOMO, MediaTek, Intel, Nokia/NSB (for FGs 30-4a/b/c/d) - CFOs of different operating frequency are different - Per band and per BC: SPRD(?), Ericsson - DMRS bundling can be done in some bands, but not in these band combinations - Per FS (per band per BC): SPRD(?), Qualcomm |
| CMCC | Per UE and per band are acceptable. Besides FG30-4, all the other FGs including 30-4a to 30-4g have less relevance with the frequency bands. If it is applicable, per band is more preferable to us. Finer granularity such as per FS or per BC is preferred. |
| ZTE | Per UE reporting is sufficient considering the UE already reports the maximum duration per band. Take FG 30-4a as an example. If a UE reported the maximum duration for some of the bands and the UE also supports PUSCH repetition A on these bands, we are not sure why the UE need to report FG 30-4a only on part of these bands as it seems no additional complexity here. |
| Intel | We are fine with the proposal. In order to meet the requirement for power consistency and phase continuity, different RF components for different bands or in FR1 and FR2 may need to be implemented/considered. |
| Nokia, NSB | There is no strict need to make all of them per band, given that the pre-requisites would already define the supported operating bands. But we can compromise to the proposal as is for the sake of progress. |
| QC | As outlined in the earlier meeting, if a UE supports DMRS bunding in Band A and Band B, it cannot be assumed that it supports DMRS bundling simultaneously in both bands. RF architecture for the two bands and how they interact determines a UE’s ability to do so. We suggest going with per band per BC. As a compromise, we are okay to go with per band and per BC. This then allows the UE to indicate its inability to support bundling in certain band combinations. |
| NTT DOCOMO | Support the proposal. Per band seems sufficient. We also think, before supporting per band and per BC, it is necessary to reach clear consensus among companies on understanding this type and the explicit reason why it needs to be supported. |
| Sharp | We are fine with the proposal. Per band reporting is sufficient. |
| Moderator | Summary of companies view - Per UE: Huawei, HiSilicon, ZTE, Nokia/NSB (for FGs 30-4e/g), CMCC - Per band as FG 30-4 is sufficient, and FG 30-4 should be prerequisite of all other FGs of DMRS bundling - TDD/FDD differentiation - Not necessary: Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell - FR1/FR2 differentiation - Not necessary: Huawei, HiSilicon, ZTE, Nokia, Nokia Shanghai Bell - Per band: vivo, China Telecom, OPPO, Apple, CMCC, NTT DOCOMO, MediaTek, Intel, Nokia/NSB (for FGs 30-4a/b/c/d), CMCC, Sharp - CFOs of different operating frequency are different - Per BC: CMCC - Per band and per BC: SPRD(?), Ericsson, QC - DMRS bundling can be done in some bands, but not in these band combinations - allows the UE to indicate its inability to support bundling in certain band combinations. - Per FS (per band per BC): SPRD(?), Qualcomm, CMCC - if a UE supports DMRS bunding in Band A and Band B, it cannot be assumed that it supports DMRS bundling simultaneously in both bands Given a number of companies showed their flexibility to accept per band, the same proposal is set for the discussion in the GTW. Supporting companies are encouraged to address the concern from companies supporting “per band and per BC” or “per FS”. [GTW1] High priority proposal 4-2: - Reporting type of FGs 30-4a to 30-4h is per band |
| Panasonic | Similar to the discussion in High priority question 3-1, we are not so sure the support of DMRS bundling together with CA/DC. If DMRS bundling is not together with CA/DC, it might be sufficient with per band. If it is together with CA/DC, per band and per BC would be more reasonable as depending on band combination, the same or different RF components may be used. |
| Ericsson | We understand that UE vendors may not be able to support CA and DMRS bundling. Can companies explain how such UEs can inform the network that CA is not supported with DMRS bundling? Then regarding how to report per band and per BC: A capability of per band and per BC could be defined in various ways. One way could be to indicate per band capability, and if the UE does not support a particular band combination, to indicate a lack of support for that band combination. This seems inconsistent with the guidance from RAN2 that says that defining “incapability” bits should be avoided (see the LS in R1-2001513). An alternative approach could be based on the definition of powerClass in 38.306, which has both per band and per band combination capabilities. Here we give an example using 30-4 for simplicity, but this should be applied to other DMRS bundling related UE capabilities (i.e. 30-4a,b,c,d and 30-4g,h) Example Per Band + Per Band Capability Definition Definitions for parameters; Per. maxDMRS-BundlingDurationBand-r17 Indicates the maximum number of consecutive slots during which the UE is able to maintain power consistency and phase continuity to support DM-RS bundling for a PUSCH or PUCCH when UE is configured with a single UL serving cell.; Band. DMRS-BundlingBC-r17 Indicates that the UE supports DM-RS bundling for a PUSCH or PUCCH also when UE is configured with more than one UL serving cell. The capability per band of the band combination is then given by maxDMRS-BundlingDurationBand-r17.; BC. The per band + BC capability seems appropriate for 30-4a,b,c,d and 30-4g,h since these features in our view are ‘DMRS bundling’ features. However, as discussed in R1-2204874, inter-slot frequency hopping should be independent of DMRS bundling. Furthermore, frequency hopping is not affected by CA configurations, and indeed Rel-15 inter-slot hopping for PUSCH (UE feature 5-10) has per UE granularity. Given that 5-10 is per UE, we think that is a reasonable granularity for Rel-17 for UE features 30-4e and 30-4f. Per-band capability can also be considered if sufficiently motivated, e.g. to address IoDT test issues. |
| Moderator | This proposal was discussed in the GTW but no consensus was achieved. The decision may depend on whether UL-CA/DC is supported for DM-RS bundling. Let’s further discuss directly over the RAN1 reflector to converge by the next GTW. High priority proposal 4-2: - Reporting type of FGs 30-4a/b/c/d/g/h is per band [and per BC] |
| Moderator | No consensus was achieved in this meeting. Comeback in next meeting. |

Low priority proposal 4-3:

- Revise the prerequisite feature groups for FGs 30-4 to 30-4h as follows

  - FG 30-4: None

  - FG 30-4a: FGs 30-4 and one of {5-14, 5-16, 5-17}

    - FFS whether to add FGs 11-6, 30-1, and 30-2

  - FG 30-4b: FGs 30-4 and 11-5

    - FFS whether to add FG 30-1

  - FG 30-4c: FGs 30-4 and 30-3

    - FFS whether to add FG 30-3a

  - FG 30-4d: FGs 30-4 and 4-23

    - FFS whether to add FG 25-2

  - FG 30-4e: FGs 30-4a, 30-4b, or 30-4c

  - FG 30-4f: FG 30-4d

    - FFS whether to add FG 4-23

  - FG 30-4g: FG 30-4

    - FFS whether to add FGs 30-4a, 30-4b, 30-4c, and 30-4d

  - FG 30-4h: FG 30-4a, 30-4b, 30-4c, or 30-4d

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - FG 30-4 - No prerequisite: Huawei, HiSilicon, Spreadtrum, China Telecom, CMCC - FG 30-4a - FG 30-4 and FG 5-14 or 5-16 or 5-17 or 11-6 or 30-1: Huawei, HiSilicon, China Telecom - FG 30-4 and FG 5-14 or 5-16 or 5-17: ZTE, Spreadtrum, CMCC, NTT DOCOMO, Ericsson - FG 30-4 and FG 30-1 or 30-2: Nokia, Nokia Shanghai Bell - FG 30-4 and FG 30-1 and FG 30-2 and FG 5-14 or 5-16 or 5-17 or 11-6: Intel - FG 30-4b - FG 30-4 and FG 11-5: Huawei, HiSilicon, ZTE, Spreadtrum, China Telecom, NTT DOCOMO, Intel, Ericsson - FG 30-4 and FG 11-6: CMCC - FG 30-4 and FG 11-5 and FG 30-1: Nokia, Nokia Shanghai Bell - FG 30-4c - FG 30-4 and FG 30-3 or 30-3a: Huawei, HiSilicon, Intel - FG 30-4 and FG 30-3: ZTE, Spreadtrum, China Telecom, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Ericsson - FG 30-4d - FG 30-4 and FG 4-23: ZTE, Spreadtrum, China Telecom, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Intel, Ericsson - FG 30-4 and FG 4-23 or 25-2: Huawei, HiSilicon - FG 30-4e - FG 30-4a or 30-4b or 30-4c: Huawei, HiSilicon, ZTE, Spreadtrum, China Telecom, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Intel - FG 2-16: Ericsson - To support enhanced frequency hopping without DMRS bundling - FG 30-4f - FG 30-4d: Huawei, HiSilicon, Spreadtrum, China Telecom, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, intel - FG 30-4d and FG 4-23: ZTE - FG 4-23: Ericsson - FG 30-4g - FG 30-4: Huawei, HiSilicon, ZTE, China Telecom, NTT DOCOMO, Nokia, Nokia Shanghai Bell - FG 30-4a or 30-4b or 30-4c or 30-4d: Intel - FG 30-4 and FG 30-4a or 30-4b or 30-4c: Ericsson - FG 30-4h - FG 30-4a or 30-4b or 30-4c or 30-4d: Huawei, HiSilicon, ZTE, China Telecom, NTT DOCOMO, Nokia, Nokia Shanghai Bell, MediaTek, Intel, Ericsson |

Low priority proposal 4-4:

- Revise components of FG 30-4 to 30-4h as:

| Index | Feature group | Components |
| --- | --- | --- |
| 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |
| 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A over consecutive symbols |
| 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B over consecutive symbols |
| 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols |
| 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions over consecutive symbols for PUCCH formats 1/3/4 |
| 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH |
| 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling |
| 30-4g | [Restart DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g., frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. |
| 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] |

- Revise the column “Consequence if the feature is not supported by the UE” of FG 30-3g as: [UE does not support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity]

| Company | Comment |
| --- | --- |
| Ericsson | Whether Rel-17 inter-slot frequency hopping can be configured without DMRS bundling is being discussed in AI 8.8.2. Therefore, we think that 30-4e and 30-4f should not be included in this proposal at present. Regarding 30-4g, the proposal seems a bit inconsistent, since it lists specific events but says others may be defined later. Also, restarting DMRS bundling for dynamic events is not actually described as such in 38.214; there it says that UEs maintain power consistency and phase continuity over an actual TDW subject to UE capability. So, our thinking is to refer to 38.214, which is then more clear. That is, we suggest the component is described as “Support power consistency and phase continuity over an actual TDW created in response to events identified in 38.214 subclause 6.1.7, where such support of consistency/continuity for the event requires UE capability.” |

- 30-5: Slot based dynamic PUCCH repetition indication

In [1], FG 30-5 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | Per UE | No | No | N/A |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | The prerequisite feature groups for FG 30-5 are FG 4-23 or 25-2. Proposal 7: The prerequisite feature group for FG 30-5 is FG 4-23 or 25-2. |
| --- | --- | --- |
| [3] | ZTE | The only remaining issue for FG 30-5 is the prerequisite FG. In our view, both legacy FG 4-23 for PUCCH repetition for PUCCH format 1/3/4 or new Rel-17 FG 25-2 for PUCCH repetition for PUCCH format 0/2 are the prerequisite FG for FG 30-5. Proposal 5: For FG 30-5, the prerequisite FG is 4-23 or 25-2. |
| [6] | China Telecom | For dynamic PUCCH repetition indication, we think the description “Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4” already implicitly indicates that slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 is supported. Proposal 4: FGs for dynamic PUCCH repetition indication. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 30. NR_cov_enh; 30-5; Slot based dynamic PUCCH repetition indication; Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4; [4-23 and/or 25-2]; UE does not support Dynamic PUCCH repetition indication; Per UE; No; No; Optional with capability signalling. |
| [11] | CMCC | Proposal 10: Remove the bracket for Prerequisite feature groups in FG 30-5. |
| [13] | Nokia, Nokia Shanghai Bell | - 30-5: - Per UE |
| [16] | Intel Corporation | At the RAN1#106b-e meeting, the following agreement was made for dynamic PUCCH repetition indication [3]. Agreement Support dynamic PUCCH repetition factor indication for all PUCCH formats including format 0, 1, 2, 3, 4 with a unified mechanism as agreed in RAN1#106e under agenda 8.8.2. Note: it does not impact the discussion of slot level or sub-slot level repetition. Based on this and considering that slot based PUCCH repetition is mainly targeted for coverage enhancement, Table 4 illustrates suggested update for UE feature groups for dynamic PUCCH repetition factor indication. Table 4. UE feature groups for dynamic PUCCH repetition factor indication Index; Feature group; Components; Prerequisite feature groups. 30-5; Slot based dynamic PUCCH repetition indication; Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4; [4-23 and/or 25-2]. Proposal 4 - For UE feature group of dynamic PUCCH repetition factor indication - Consider Table 4 for UE feature group of dynamic PUCCH repetition factor indication. |
| [17] | Ericsson | The following has been agreed: Agreement (from RAN1#107) - Revised the component in FG 30-5 as “Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4” - Add a component in FG 30-5 for support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 - FG 4-23 and/or FG 25-2 are the prerequisite feature groups for FG 30-5 - Add FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 Agreement (from RAN1#108) - FG 30-5 is not split to one for PUCCH formats 1/3/4 and the other for PUCCH formats 0/2 Since both format 1/3/4 and 0/2 are agreed, the prerequisite feature groups of FG 30-5 should be 4-23 and/or 25-2. Therefore, we suggest to drop the brackets. Table 4: Capabilities for PUCCH Repetition Enhancement Index; Feature group; Components; Prerequisite feature groups; Comments. 30-5; Slot based dynamic PUCCH repetition indication; Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4; [4-23 and/or 25-2]. UE features for PUCCH repetition enhancement are defined according to Table 4 |

Discussion

Low priority proposal 5-3:

- The prerequisite FGs of FG 30-5 are confirmed as 4-23 or 25-2

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - FG 4-23 or 25-2 : Huawei, ZTE, Intel - 4-23 and/or 25-2 (remove []) : China Telecom, CMCC, Ericsson |
| Ericsson | Not sure that companies actually disagree. In our view, if the UE supports at least one of 4-23 and 25-2, dynamic PUCCH repetition can be supported. Perhaps this phrasing is more clear, and could be used to check companies views? |

- 30-6: Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

In [1], FG 30-6 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | For the type of the granularity for FG 30-6, per UE is preferable. Besides, the need for FR1/FR2 differentiation and TDD/FDD differentiation are No, and the capability interpretation for mixture of FDD/TDD and/or FR1/FR2 is N/A. Proposal 8: For FG 30-6, - Per UE for the type of the granularity. - No for the need for FR1/FR2 differentiation and TDD/FDD differentiation. - N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2. Proposal 9: In summary, adopt the changes of the table in Appendix. |
| --- | --- | --- |
| [3] | ZTE | For FG 30-6, the only remaining issue is the UE capability reporting granularity. In our view, it’s sufficient for reporting by per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them. Proposal 6: For FG 30-6, support per UE reporting and no need of differentiation for FDD/TDD, FR1/FR2 or mixture of them. |
| [6] | China Telecom | Proposal 5: FGs Msg.3 repetition. Features; Index; Feature group; Components; Prerequisite feature groups; Consequence if the feature is not supported by the UE; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Note; Mandatory/Optional. 30. NR_cov_enh; 30-6; Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI; [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI]; UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI.; [Per UE]; [No]; [No]; Optional with capability signalling. |
| [7] | Xiaomi | For Msg.3 repetition, Considering in some bands, such as for NTN bands in case of the long communication distances, or for unlicensed bands in case of worse interference from other coexisting systems, 16 repetitions for msg.3 may be not enough, and further enhancements may be needed. Thus, we think the type of FG 30-6 can be supported per band. For the differentiation between FR1 and FR2 bands, and the differentiation between FDD and TDD bands, it can share the same design as FG 30-3 as discussed in Section 2.1. Proposal 7: The type of FG 30-6 is per band. Proposal 8: No need to differentiate FG 30-6 between FDD and TDD bands, as well as between FR1 and FR2 bands. |
| [8] | OPPO | It is FFS whether the type of FG 30-6 should be per UE or per band. We think the feature seems not related to band in most scenarios. We prefer type of FG 30-6 is per UE. Proposal 3：Type of FGs 30-6 is per UE. |
| [10] | Apple | According to the agreements, Msg3 PUSCH repetitions only support inter-slot hopping for initial and re-transmission. There is no consensus to support intra-slot hopping. Thus, it’s better to introduce a component in FG20-6 to make it clear. Proposal 4: Adding a component to FG30-6: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. |
| [11] | CMCC | Proposal 11: FG 30-6 should be reported as per UE level not per band. And it is not necessary to differentiate between TDD and FDD. Proposal 12: Remove the bracket for Components in FG 30-5. Proposal 13: No need to differentiate TDD and FDD for FG 30-6. |
| [12] | NTT DOCOMO | It was discussed that the FG 30-6 is supported per UE or per band. Since Msg3 repetition is not the band specific feature, the FG can be per UE. Also, the differentiation between TDD and FDD is not necessary, following FGs of repetition type A. Proposal 8: FG 30-6 can be supported per UE with no differentiation between TDD and FDD. |
| [13] | Nokia, Nokia Shanghai Bell | - 30-6: - Confirm FG description - Per UE |
| [14] | Sharp | For Type A PUSCH repetitions for Msg3, the only remaining issue is whether the FG 30-6 is per UE or per band/scenario. In our view, per-band granularity is not necessary for this feature, because we do not believe Msg 3 repetitions would lead to any significant difference in different bands. Per-UE with differentiation of NTN or unlicensed would be a reasonable option, considering the IODT for those scenarios. Proposal 4: FG 30-6 should be per UE with differentiation of NTN or unlicensed. |
| [15] | MediaTek | Considering the test cost and the applicability/support of the band (e.g., licensed band and unlicensed band), per band support is more preferred for all features. Proposal 1: All features are per band. |
| [16] | Intel Corporation | Table 5. UE feature group for Msg3 PUSCH repetition Index; Feature group; Components; Prerequisite feature groups. 30-6; Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI; [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI]. Proposal 5 - For UE feature group of Msg3 PUSCH repetition - UE feature for Msg3 PUSCH repetition is defined per UE. - FDD/TDD differentiation is not necessary. - Consider Table 5 for UE feature group of Msg3 PUSCH repetition. |
| [17] | Ericsson | Table 5: Capabilities for Type A PUSCH repetition for Msg3 Index; Feature group; Components; Prerequisite feature groups; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC). 30-6; Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI; [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI]; [Per UE]. UE features for Type A PUSCH repetition for Msg3 are defined according to Table 5 |
| [18] | Qualcomm Incorporated | Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |

Discussion

[FL1] High priority proposal 6-1:

- Reporting type of FG 30-6 is per UE without FDD/TDD and FR1/FR2 differentiation

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Per UE: Huawei, HiSilicon, ZTE, China Telecom, OPPO, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Intel, Ericsson - FDD/TDD differentiation - Not necessary: Huawei, HiSilicon, ZTE, China Telecom, CMCC, NTT DOCOMO, Nokia Shanghai Bell, Intel - FR1/FR2 differentiation - Not necessary: Huawei, HiSilicon, ZTE, China Telecom, Nokia Shanghai Bell - Per UE with differentiation of NTN or unlicensed: Sharp - Per band: Xiaomi, MediaTek, Qualcomm |
| CMCC | Fine with this proposal. This is similar as other repetition related features that it is band agnostic. Then it is not necessary to report per band. And it is also not necessary to differentiate between TDD and FDD. |
| ZTE | Support |
| Intel | We are fine with the proposal. |
| Nokia, NSB | Support |
| QC | Don’t support. Msg3 repetition is based on available slot counting. A UE may not support available slot counting in every band. |
| NTT DOCOMO | Support the proposal. However, we can compromise per band, because msg3 repetition is counted on the basis of available slot counting and FG of available slot determination is per band. |
| Sharp | Fine with the proposal. At the same time, we share the view from DOCOMO that this can be aligned with the granularity of FG30-2, so we can live with the per band. |
| Moderator | Summary of companies view - Per UE: Huawei, HiSilicon, ZTE, China Telecom, OPPO, CMCC, NTT DOCOMO, Nokia, Nokia Shanghai Bell, Intel, Ericsson, Sharp - FDD/TDD differentiation - Not necessary: Huawei, HiSilicon, ZTE, China Telecom, CMCC, NTT DOCOMO, Nokia Shanghai Bell, Intel - FR1/FR2 differentiation - Not necessary: Huawei, HiSilicon, ZTE, China Telecom, Nokia Shanghai Bell - Per UE with differentiation of NTN or unlicensed: Sharp - Per band: Xiaomi, MediaTek, Qualcomm, DOCOMO, Sharp As mentioned by some companies, Msg3 repetition is based on available slot counting, whose FG 30-2 is supported as per band. Further discuss in the GTW whether to go with Alt1 or Alt2 [GTW1] High priority proposal 6-1: - Alt1: Reporting type of FG 30-6 is per UE without FDD/TDD and FR1/FR2 differentiation - Alt2: Reporting type of FG 30-6 is per band |
| Ericsson | Alt 1 is our first preference. We can accept Alt 2 for the IoDT test concern. |
| OPPO | Fine with this proposal. |
| Moderator | Following was agreed by email endorsement Agreement - Reporting type of FG 30-6 is per band |

Low priority question 6-2:

- Companies are encouraged to provide views on whether to add component for inter-slot frequency hopping for Msg3 repetitions in FG 30-6

  - Support: Apple

| Company | Comment |
| --- | --- |
| Ericsson | We don’t see the need. A UE supports inter-slot frequency hopping for Msg3 repetition if it supports Msg3 repetition. Otherwise, gNB needs further PRACH partitioning for UEs supporting Msg3 repetition with or without inter-slot frequency hopping. |

- Conclusions

Following agreements were made in this meeting.

Agreement

- Add the following note in FG 30-4

  - Candidate values for the maximum duration for FDD are {4, 8, 16, 32}

  - Candidate values for the maximum duration for TDD are {2, 4, 8, 16}

Agreement

- Reporting type of FG 30-6 is per band

References

[1]	R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2203097	Rel-17 UE features for NR coverage enhancement	Huawei, HiSilicon

[3]	R1-2203198	Discussion on UE features for NR coverage enhancement	ZTE

[4]	R1-2203318	UE features for NR coverage enhancement	Spreadtrum Communications

[5]	R1-2203536	Remaining issues on UE features for NR coverage enhancement	vivo

[6]	R1-2203654	Remaining issues on UE features for Rel-17 NR coverage enhancements	China Telecom

[7]	R1-2203783	Discussion on UE features for NR coverage enhancement	xiaomi

[8]	R1-2203883	UE features for NR coverage enhancement	Samsung

[9]	R1-2203998	Rel-17 Coverage Enhancement UE features	OPPO

[10]	R1-2204223	Views on Rel-17 Coverage Enhancement UE Features	Apple

[11]	R1-2204285	Discussion on Rel.17 UE features for NR coverage enhancement	CMCC

[12]	R1-2204363	Discussion on Rel-17 UE features for NR coverage enhancement	NTT DOCOMO, INC.

[13]	R1-2204593	On UE features for NR coverage enhancement	Nokia, Nokia Shanghai Bell

[14]	R1-2204665	UE feature for Rel-17 coverage enhancement	Sharp

[15]	R1-2204729	Discussion on UE features for NR Coverage Enhancement	MediaTek Inc.

[16]	R1-2204783	Discussion on UE features for NR coverage enhancement	Intel Corporation

[17]	R1-2204874	UE Features for NR Coverage Enhancement	Ericsson

[18]	R1-2205005	UE Features for Coverage Enhancements	Qualcomm Incorporated