3GPP TSG RAN WG1 #109-e			R1-2204409

e-Meeting, May 9th –20th, 2022

Agenda item:	8.16.12

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on UE features for NR MBS

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.12 regarding UE features for NR MBS and captures the following email discussion.

[109-e-R17-UE-features-MBS-01] Email discussion on UE features for NR MBS – Shinya (DOCOMO) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

In the Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e [1], there are following feature groups for NR MBS.

- 33-1	Broadcast

- 33-2	Dynamic scheduling for multicast for PCell

- 33-2a	Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

- 33-2b	DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast

- 33-2d	PTP retransmission for multicast

- 33-2e	Multiple G-RNTIs for group-common PDSCHs

- 33-2f	Dynamic multicast with DCI format 4_2

- 33-2g	MIMO layers for multicast PDSCH

- 33-2h	Dynamic scheduling for multicast for SCell

- 33-3-1	Dynamic Slot-level repetition for group-common PDSCH

- 33-3-2	FDM-ed unicast PDSCH and group-common PDSCH

- 33-3-3	Intra-slot TDM-ed unicast PDSCH and group-common PDSCH

- 33-3-3a	FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

- 33-3-3b	Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast

- 33-3-4	Mode 1 for type1 codebook generation

- 33-3-5	Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type

- 33-4	NACK-only based HARQ-ACK feedback for multicast

- 33-4-1	DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast

- 33-5-1	SPS group-common PDSCH for multicast

- 33-5-1a	Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast

- 33-5-1b	DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast

- 33-5-1d	PTP retransmission for SPS group-common PDSCH for multicast

- 33-5-1e	Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

- 33-5-1f	NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

- 33-5-1g	DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

- 33-5-1h	Multiple G-CS-RNTIs for SPS group-common PDSCHs

- 33-5-2	Multiple SPS group-common PDSCH configuration

- 33-6-1	DL priority indication for multicast in DCI

- 33-6-a	DL priority configuration for SPS multicast

- 33-6-2	Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE

- 33-6-3	More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot

- 33-9	Supporting unicast PDCCH to release SPS group-common PDSCH

The issues to be discussed are tagged and colour coded with High priority or Low priority based on ASN.1 impact.

- 33-1: Broadcast

In [1], FG 33-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH - DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | Since DCI format 4_0 is defined in TS 38.212 for scheduling broadcast, DCI format 1_0 in the 3rd component needs to be updated to DCI format 4_0. MBS broadcast includes MCCH and MTCH, both of which will be scheduled by G-RNTI. The 6th component needs to clarify that group-common PDSCH including MCCH and MTCH will be TDM-ed or either one will be TDM-ed with unicast in different slots. It was proposed to have two additional components to be included in FG33-1 regarding rate matching - Support of semi-static rate-matching resource set configuration. - Support of rate-matching around LTE CRS. It was concerned to directly include such two bullets but instead was suggested to add “A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast” in the column of note to clarify the rate matching capabilities are the ones mandatory support in Rel-15. This suggestion can be taken for approval in this meeting. One more leftover issue for FG33-1 from the last meeting was whether merging DCI indicating slot-level repetition up to 16 for MTCH into FG 33-3-1. We think it could be merged and ends up a general capability with either FG33-1 or FG33-2 as prerequisite FG. The update FG33-3-1 refers to section 2.4 Overall, the FG33-1 for broadcast can be updated as in the following proposal. Proposal 1: Updating FG33-1 as follows in red: 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH.; Up to RAN2; Up to RAN2; Up to RA-N2; Up to RAN2; Note: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast.; Up to RAN2. |
| --- | --- | --- |
| [4] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 5 of FG 33-1, DCI format 1_0 is adjusted as DCI format 4_0; Last meeting, after discussion, we have are agreed to take dynamic slot-level repetition for MTCH as separated FG from FG33-1. Whether to merge the capability into capability of dynamic slot-level repetition for multicast (i.e., FG33-3-1) is still FFS. If UE has the capability of supporting dynamic slot-level repetition for multicast (or broadcast), it is natural for UE to have the capability of supporting dynamic slot-level repetition for broadcast (or multicast). From our side, we have no strong preference, and fine to merge the capability of dynamic slot-level repetition for MTCH into FG33-3-1. Proposal 2: Support to merge the capability of dynamic slot-level repetition for MTCH into FG33-3-1. |
| [5] | vivo | repetitions from 1 to 8 for MTCH, it is better to support several fixed values, i.e., {2, 4, 8} times repetitions as defined for unicast or multicast. Regarding DCI indicated slot-level repetition for MTCH, it has been agreed as another FG in last meeting, and it could be merged with FG 33-3-1. Furthermore, multiple G-RNTIs facilitate different broadcast services, and thus, support of G-RNTIs should be included in 33-1. Proposal 1 For higher layer configured slot-level repetition in FG 33-1, UE supports {2, 4, 8} times repetitions. Proposal 2 DCI indicated slot-level repletion is merged with FG33-3-1. Proposal 3 ‘Support of G-RNTIs’ is included in FG 33-1. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC); Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support {2, 4, 8} times semi-static slot-level repetition for MTCH - Support of N>=1 G-RNTIs for broadcast. FFS details of N; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |
| [6] | Xiaomi | In RAN1#108 e-meeting, the following agreement on how to process MBS broadcast DCI was agreed: Agreement Regarding the number of DCIs that a UE can process in a slot or span, MBS broadcast DCI monitored by the UE is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b for RRC_CONNECTED UEs.. The above agreement addresses MBS UE capability related to DCI processing. It should be captured in FG 33-1. We propose to add the following component for FG 33-1 in order to address the newly achieved agreement in main session. - Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b. Proposal 1: Add the following component for FG 33-1: - Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b. |
| [7] | OPPO | In RAN1 #108-e meeting, it was agreed that dynamic slot-level repetition for MTCH is separated from FG 33-1, and the maximum number of dynamic slot-level repetitions is 16. In FG 33-3-1, it is about dynamic slot-level repletion for group-common PDSCH in multicast which is focused on UEs in RRC_CONNECTED states, while this separated component above is for PDSCH (conveying MTCH) for broadcast. Both components are dynamic slot-level repetition, but they are related to different cast-type. To differentiate them associated with multicast and broadcast, respectively, DCI indication on slot-level repetition up to 16 for MTCH is not merged with FG 33-3-1. - “DCI indicated slot-level repetition up to 16 for MTCH” is defined as an independent FG which is not merged with FG 33-3-1. |
| [8] | Apple | In last RAN1 meeting, there were discussions on the broadcast PDSCH rate matching around LTE CRS. It was clarified the rate-matching around LTE CRS is Rel-15 FG 5-28. Both FG 5-26 and 5-28 are mandatory feature in Rel-15. And there were agreements that multicast PDSCH could be configured with rate matching pattern. So FG 5-26 and 5-28 can be prerequisite of FG33-1 or add a note to require supporting FG5-26 and FG5-28 for broadcast. Proposal 1: Add FG5-26 and 5-28 as prerequisite of FG33-1 or add a note in FG 33-1: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast. Regarding the type of FG 33-1, should it per UE, per band or per FSPC. As indicated by RAN2, RRC connected UE could receive the MBS broadcast on Scell or non-serving cell. In addition, RAN1 agreed the related UE capability is to be defined in RAN2. Thus, leaving it to RAN2 to make decision is enough. - Agreement - From RAN1 perspective, it is feasible for UE in RRC_CONNECTED state to receive MBS broadcast on an activated SCell as long as UE has capability of supporting MBS broadcast on SCell. From RAN1 perspective, if a UE is to receive MBS broadcast on SCell, - The capability of supporting MBS broadcast on SCell is separate capability from the one of CA for unicast. - The UE is not required to monitor DCI formats associated with SI-RNTI, P-RNTI, RA-RNTI in SCell. - Overbooking for SCell is not supported. - MBS broadcast reception on SCell can be supported only for RRC_CONNECTED UEs only with self-scheduling. - Type0-PDCCH CSS set is only configured on the primary cell of the MCG. - Configuring the search space on SCell for PDCCH monitoring of MBS DCI formats is via unicast RRC signaling. - The UE capability is expected to be defined by RAN2. - E.g. the total number of component carriers for receiving broadcast on SCell may be subject to UE capability - The UE is not required to receive broadcast on PCell and SCell simultaneously. Proposal 2: It’s up to RAN2 to determine FG33-1 is per UE, per band or per SFPC. |
| [9] | CMCC | The DCI indicated slot-level repetition for MTCH has been defined as another FG, one remaining issue is whether to merge it with FG 33-3-1 (DCI indicated slot-level repetition for multicast). Considering there is no technical difference between DCI indicated slot-level repetition for broadcast MTCH GC-PDSCH and multicast GC-PDSCH which both adding repetition number in TDRA table, the FG for DCI indicated slot-level repetition up to 16 for MTCH can be merged with FG 33-3-1. Proposal 1. DCI indicated slot-level repetition up to 16 for MTCH is merged with FG 33-3-1. |
| [10] | NTT DOCOMO | The only difference between dynamic slot-level repetition for multicast and dynamic slot-level repetition for broadcast is the difference in the TDRA list including the parameter repetitionNumber. There would be no need for separate FGs. Proposal 1: Support of dynamic slot-level repetition for broadcast is merged with FG 33-3-1. |
| [11] | Nokia, NSB | - 33-1: - Since this is a broadcast feature, it is preferred to have the DCI indicated slot-level repetition up to [8 or 16] for MTCH as a component to this FG, rather than a separate one. |
| [12] | MediaTek | In the latest approved TS 38.212 spec, the three DCI formats were defined for MBS broadcast and multicast that DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. In order to align the MBS DCI format naming, we suggest the following proposal: Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. Considering the concept of group-common PDCCH/PDSCH is used for broadcast and multicast and it only state “group-common PDCCH/PDSCH” in some FG, it may cause some reader confused, e.g., which means for broadcast or multicast? Therefore, we want to clarify the concept more clearly with some constrains, e.g., group-common PDCCH/PDSCH for multicast or broadcast, respectively. Proposal 2: Clarify the wording of group-common PDCCH/PDSCH more clearly for broadcast and multicast respectively, e.g., group-common PDCCH/PDSCH for multicast or broadcast. Regarding the discussion of which granularity type definition should be defined for MBS UE feature, from our view, the different carriers within the same band will have different characteristics or usage purpose, especially for considering the band n28 is a shared band by operators. What’s more, considering the IODT testing cost, we prefer the type definition for MBS UE features should be based on the granularity per FSPC for the remaining feature group. Proposal 3: The type definition for MBS UE features should be based on the granularity per FSPC for the remaining feature group. Regarding the number of MCCH-RNTI and G-RNTI for broadcast, there has not been discussed for now and need to make it clearer. For the MCCH-RNTI, we think only one MCCH-RNTI is sufficient for UE obtaining the MBS control information as similar with the legacy LTE SC-PTM mechanism. For the G-RNTI issue, RAN2 has achieved an agreement that “one-to-many mapping between G-RNTI and MBS sessions is supported” in previous meeting, which means the multiple services if existed can use only for G-RNTI for services reception. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multiple multicast G-RNTIs/G-CS-RNTIs, broadcast MCCH-RNTI) however, a total number RNTI supported by UE is limited. Considering the above reason and broadcast is a best-effort reception, we suggest only one G-RNTI is supported for broadcast reception. Proposal 4: For FG 33-1, adding a note that “For component 1, only one MCCH-RNTI is supported for broadcast”. Proposal 5: For FG 33-1, adding a note that “For component 2, only one G-RNTI is supported for broadcast”. Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception since we also have agreed that the CFR frequency for MCCH and MTCH can be configured by SIBx. Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;. Proposal 6: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”. Regarding slot-level repetition for MTCH, the following agreement was achieved in last meeting: Agreement: The capability for support of dynamic slot-level repetition for MTCH is separated from FG 33-1. - maximum number of dynamic slot-level repetitions is 16 - FFS whether to merge with FG 33-3-1. Regarding the FFS whether to merge with FG 33-3-1, from our understanding, broadcast and multicast are two different FGs, what’s more, some detailed information has not decided yet for now, e.g., the whether the same or different reporting type for broadcast and multicast. So, at current stage, we slightly prefer using a separate FG to describe the capability for supporting of dynamic slot-level repetitio for MTCH. Proposal 7: Adding a new separate FG 33-1-1 to capture the dynamic slot-level repetition for broadcast MTCH. 33-1; Broadcast; - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH DCI indicated slot-level repetition up to 16 for MTCH is defined as another FG, FFS whether to merge with FG 33-3-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN; For component 1, only one MCCH-RNTI is supported for broadcast For component 2, only one G-RNTI is supported for broadcast For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx; Up to RAN2. 33-1-1; Dynamic slot-level repetition for broadcast MTCH; - Support up to X times dynamic slot-level repetition for broadcast MTCH.; 33-1; Per FSPC; Candidate values for X is: {8, 16}; Optional with capability signalling. |
| [13] | Ericsson | FG 33-3-1 is a feature group that includes dynamic slot level repetition for G-PDSCH for multicast, as it is depended on FG 33-2, the basic FG for multicast. In order for broadcast UEs to be able to support dynamic repetition witout having to support multicast, we propose to separate the broadcast support from multicast support by either adding 33-1 as a prerequisite to 33-3-1, or creating a separate FG . Support Dynamic Slot-level repetition for group-common PDSCH for broadcast as part of 33-3-1. At least one of FG 33-1 or FG 33-2 should be supported by the UE as prerequisite. Proposed changes: 33. NR_MBS; 33-3-1; Dynamic Slot-level repetition for group-common PDSCH; - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast.; 33-2 Or 33-1 Note: 33-1 prerequisite to support the feature for broadcast 33-2 prerequisite to support the feature for multicast. |
| [14] | Qualcomm | Since broadcast and multicast are separate features, FG33-1a for DCI-indicated slot-level repetition for MTCH should be added, separate from FG 33-3-1. We suggest the changes on FG 33-1 as: 33. NR_MBS; 33-1; Broadcast; - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI(s) for MTCH. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for broadcast in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH - Support of FDMed MCCH and PBCH; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. 33. NR_MBS; 33-1a; Dynamic slot-level repetition for Broadcast; Support of DCI-indincated slot-level repetition up to 16 for MTCH; 33-1; Yes; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |

Discussion

High priority proposal 2-1:

- Merge the capability for DCI indicated slot-level repetition up to 16 for MTCH with FG 33-3-1.

  - The prerequisite FGs for FG 33-3-1 is revised as: 33-1 or 33-2

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Merge with FG 33-3-1: Huawei, HiSilicon, vivo, CMCC, NTT DOCOMO, Spreadtrum Communications, Ericsson - no technical difference between DCI indicated slot-level repetition for broadcast MTCH GC-PDSCH and multicast GC-PDSCH which both adding repetition number in TDRA table - Not merge: OPPO, MediaTek, Qualcomm - they are related to different cast-type - some detailed information has not decided yet for now, e.g., the whether the same or different reporting type for broadcast and multicast - As a component of FG 33-1: Nokia/NSB This proposal can be discussed after the reporting type of FG 33-2 is agreed. |
| Huawei, HiSilicon | I just wonder why FG33-3-1 cannot be a common FG even though reporting type is different as long as both are supported. |
| ZTE | We support to merge it with FG33-3-1 as there are basically the same functionality. |
| OPPO | When we discuss and define the feature groups for MBS, it is obvious that broadcast and multicast are split into independent FGs, even some of the components/functionalities are same between broadcast and multicast. If 33-3-1 is supposed to be defined as a common FG for both broadcast and multicast, how to treat other 33-3-x FGs should also clarified. At current situation, “DCI indication of slot-level repetition of MTCH” can be a independent FG under FG 33-1 series. |
| Qualcomm | We don’t support merging with FG33-3-1. The broadcast and multicast are independent FGs. The required TDRA table configuration and DCI format for the DCI-based slot-level repetition are also different for broadcast and multicast. |
| FL2 | It seems companies supporting separate FG explained more reasons than the reporting type. Therefore, following proposal is made. If you cannot live with the proposal, please provide another proposal which is acceptable to all [FL2] High priority proposal 2-1: - Introduce a separate FG for DCI indicated slot-level repetition up to 16 for MTCH from FG 33-3-1: 33. NR_MBS; 33-1-1; DCI indicated slot-level repetition up to 16 for broadcast MTCH; Support up to 16 times dynamic slot-level repetition for broadcast MTCH.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |
| vivo | We can go for a separate FG for progress. |
| LG Electronics | We are OK with the separate FG. |
| Huawei, HiSilicon | OK |
| MediaTek | Support the proposal. |
| Spreadtrum | Support |
| OPPO | Support. |
| Qualcomm | Support |
|  | All companies are fine with the proposal. [GTW3] High priority proposal 2-1: - Introduce a separate FG for DCI indicated slot-level repetition up to 16 for MTCH from FG 33-3-1: 33. NR_MBS; 33-1-1; DCI indicated slot-level repetition up to 16 for broadcast MTCH; Support up to 16 times dynamic slot-level repetition for broadcast MTCH.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |
|  | Following was agreed in the GTW on May 12. Agreement - Introduce a separate FG for DCI indicated slot-level repetition up to 16 for MTCH from FG 33-3-1: 33. NR_MBS; 33-1-1; DCI indicated slot-level repetition up to 16 for broadcast MTCH; Support up to 16 times dynamic slot-level repetition for broadcast MTCH.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |

[FL1] High priority proposal 2-2:

- Companies are encouraged to provide view whether to add a component for support of  G-RNTIs

  - Support: vivo

    - multiple G-RNTIs facilitate different broadcast services

| Company | Comment |
| --- | --- |
| LG Electronics | This seems not needed. The number of G-RNTIs that UE receive for broadcast could be up to UE implementation. |
| Huawei, HiSilicon | If N ends up multiple candidate values including N=1, I assume it is equivalent to UE reports the number of N separately from this FG. In addition, UE can also report the interest of multiple broadcast services (by multiple G-RNTIs) in MII, so not necessary to duplicate reporting. |
| MediaTek | Not support. As stated in our contribution, supporting more RNTIs will have larger impact to UE hardware design, which against the restriction of Rel-17 WID that “In order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided)”. Besides, considering the UE cannot report the UE capability in RRC IDLE/INACTIVE state, the same MBS broadcast is targeted for RRC IDLE/INACTIVE and RRC CONNECTED UEs, the same broadcast scheduling (e.g., the mapping relationship b/w multicast service and G-RNTI) is needs to be unified for RRC IDLE/IANCTIV and RRC CONNECTED UEs. What’s more, RAN2 has achieved the following agreements for the mapping rule, which means the multiple MBS sessions can be mapped into one G-RNTI. So, we think one G-NRIT for broadcast services is enough and prefer adding a note that “For component 2, only one G-RNTI is supported for broadcast”. RAN2’s agreement: - one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work. |
| Spreadtrum | Not needed. This is broadcast, and it can be up to UE’s implementation, and should not be the UE capability. |
| ZTE | We are supportive to have more than 1 G-RNTIs. If RAN1 cannot converge, this issue can also be left to RAN2. |
| OPPO | Not support more than 1 G-RNTIs for broadcast, since the motivation and benefit is not strong, and one G-RNTI can also fulfil the requirement and functionality. |
| Qualcomm | More than one G-RNTI is supported for broadcast MTCH corresponding to different services, which is also the RAN1/RAN2 design assumption. |
| FL2 | Given most companies do not see the necessity to introduce the capability, following proposal is made [FL2] High priority proposal 2-2: - It is up to RAN2 whether to introduce the capability for support of G-RNTIs for broadcast |
| vivo | From our understanding, there exist multiple broadcast services. Mapping all broadcast services to single G-RNTI causes higher UE complexity and power consumption, UE has to always decode all the broadcast PDSCHs because it has no idea whether its interested services are included before decoding. We are fine to leave it to RAN2. |
| Huawei, HiSilicon | OK |
| ZTE | OK |
| MediaTek | We slightly prefer the issue can be discussed in RAN1, and still think one G-RNTI for broadcast is sufficient since supporting multiple RNTIs will have the UE hardware impact as commented earlier. Regarding the power consumption for MBS UE, we think it is not a critical issue than hardware impact. If majority views support the proposal, we can live with it. |
| Spreadtrum | Ok |
| OPPO | OK |
| Nokia, NSB | We would prefer to have the decision in RAN1, but we can leave it to RAN2 for the sake of progress, as the issue involves both WGs in a way. |
| Qualcomm | Ok for progress |
|  | All companies are fine with the proposal [GTW3] High priority proposal 2-2: - It is up to RAN2 whether to introduce the capability for support of G-RNTIs for broadcast |
|  | Following was agreed in the GTW on May 12. Agreement - It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE |

Low priority proposal 2-3:

- Components of FG 33-1 are revised as

  - Component 1: Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI.

  - Component 2: Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH.

  - Component 5: Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast.

  - Component 6: Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots.

  - Component 8: Support {2, 4, 8} times of higher layer configured slot-level repetition up to 8 for MTCH

  - Add a component “Broadcast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.”

  - Add a component for support of FDMed MCCH and PBCH.

  - Add a note that “For component 1, only one MCCH-RNTI is supported for broadcast”.

  - Add a note that “For component 2, only one G-RNTI is supported for broadcast”.

  - Add a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”.

  - Add a note that “A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast.”

    - Support: Huawei, HiSilicon, Apple

| Company | Comment |
| --- | --- |

- 33-2 to 33-2h: Dynamic scheduling for multicast

In [1], FGs 33-2 to 33-2h are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the Gnb to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | [Per band or per FSPC] | [N/A] | [N/A] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | To align with TS 38.212 CR, the DCI format 1_0 in the 4th component of FG33-2 should be updated to 4_1 (based on DCI format 1_0). It was discussed in the last meeting that whether FG33-1 is the basic feature group for Rel-17 NR MBS and if it is then FG33-1 for broadcast will be the prerequisite FG for any other FGs defined. We clarified in the last meeting that broadcast and multicast are separate FGs demanded in different cases and there are still some differences for developing such two features from UE implementation aspect although the basic feature sets for them are strived to be common. Therefore, for early commercialization, it is preferred to keep FG33-1 and FG33-2 as separate independent FGs. When considering UE capability for supporting both multicast and broadcast, it would be preferred to describe it in FG for multicast instead of defining additional FG for supporting both. For example, for the 5th component of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots where the group-common PDSCH is intended for multicast, it could be updated to consider broadcast group-common PDSCH as well if UE supports FG33-1. In addition, the following was agreed in the last meeting: Agreement - FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case - Type of FG for PCell case is [per band or per FSPC] - Type of FG for SCell case is per FSPC - FG for PCell is added as a Prerequisite FG for FG for SCell As FG33-2 for SCell is reported per FSPC, FG33-2 for PCell can also be reported per FSPC. Alternatively, it can be reported per BC considering UE reporting larger CA bandwidth class for a BC then UE may not report the support of FG33-2. For FG33-2a, considering support of shared PUCCH resource configurations with unicast has been the component for this FG, support of separate PUCCH resource configurations from unicast can be separate FG from FG 33-2a if separate PUCCH resources configurations may take extra UE buffer. Proposal 2: Updating FG33-2, FG33-2a, FG33-2b, , and adding FG33-2c as follows in red: 33. NR_MBS; 33-2; Dynamic scheduling for multicast for PCell; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast.; FFS; Yes; Per BC or per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a; 33-2; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling; 33-2a; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-2c; Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast; Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast; 33-2a; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. RAN2 has agreed to define UE capability for the number of simultaneous G-RNTIs / G-CS-RNTIs reception for multicast [2] and companies were invited to provide the views on the actual maximum number of G-RNTIs/G-CS-RNTIs (or their combination) for UE to simultenously monitor, for which RAN2 discussions seem to leave it to RAN1 [3]. FG33-2e should be revised as the maximum number of G-RNTIs supported for multicast dynamic scheduling and the candidate values can be {2, 3, 4} or keep the max number is 4. In addition, FG33-2e and FG33-5-1h are currently worded for the number of supported G-RNTIs/G-CS-RNTIs for multicast. In light of avoiding device hardware impact to facilitate implementation and deployment of the feature, receiving multicast and broadcast may take the resources built for unicast from UE implementation perspective without changing the existing hardware, e.g., the maximum number of RNTIs UE supports that was dimensioned for unicast will be shared with multicast as well when to support MBS multicast. Therefore, in addition to the value of the maximum number of G-RNTIs/G-CS-RNTIs supported for multicast in FG33-2e/FG33-5-1h, UE reporting the total number of all RNTIs (e.g., as introduced in FG33-10) that UE can support can also help network schedule unicast properly. Additional reporting the total number of all RNTIs was discussed in the last meeting and the comment is mainly about the relation with other features besides NR MBS. As explained, the motivation was not changing UE hardware to support multicast/broadcast. Since the RNTIs UE supports as capability should be pre-dimensioned before UE implementation and when building multicast/broadcast on top of the current UE without changing the hardware, in addition to reporting the number of G-RNTIs or G-CS-RNTIs, reporting the total number of RNTIs (including all cases, e.g., PS-RNTI, IMT-RNTI, ….) can help network decide which feature can be configured to the UE. Without this reporting, UE may have to not report the capability of supporting some UE feature (but actually UE does have such capability) so as to control the features corresponding to the RNTIs not exceeding UE capability. It is per cell. The deficit of RNTs is originated from supporting multiple G-RNTI/G-CS-RNTI. If UE does not support MBS, this number may not be needed to be reported as legacy. From this sense, we can discuss it under MBS and even can have MBS FG33-2e/FG33-5-1h as the prerequisite FG. There are two alternative that can be considered as following proposal: Proposal 3: Consider the following two alternatives to modify the FG33-2e/FG33-5-1h: - Alt1: Updating FG33-2e/FG33-5-1h and adding FG33-10 as follows in red: 33. NR_MBS; 33-2e; Multiple G-RNTIs for dynamic group-common PDSCHs; The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. FFS details.; 33-2; Yes; Per UE; No; No; Candidate values for X is: {2, 3, 4}; Optional with capability signalling. 33. NR_MBS; 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; The maximum number of G-CS-RNTIs for SPS multicast per CC; 33-5-1; Yes; Per UE; No; No; Candidate values for X is: {2, 3, 4}; Optional with capability signalling. 33. NR_MBS; 33-10; Total number of RNTIs; The maximum number of all RNTIs UE can support per CC.; 33-2e and/or 33-5-1h; Yes; Per UE; No; No; Candidate values for X is: {16, 32}; Optional with capability signalling. - Alt2: Updating FG33-2e/FG33-5-1h as follows in red: 33. NR_MBS; 33-2e; Multiple G-RNTIs for dynamic group-common PDSCHs; - The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. - Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.; 33-2; Yes; Per UE; No; No; For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted}; Optional with capability signalling. 33. NR_MBS; 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; - The maximum number of G-CS-RNTIs for SPS multicast per CC. - Indication of whether the maximum number of G-CS-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.; 33-5-1; Yes; Per UE; No; No; For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted}; Optional with capability signalling. As discussed in section 2.9, support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling can be a separate FG in FG33-5-1 and can be merged into FG33-2f, which can be updated with G-CS-RNTI included. Proposal 4: Updating FG33-2f as follows in red: 33. NR_MBS; 33-2f; Dynamic multicast and SPS multicast with DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-RNTI/G-CS-RNTI for multicast; 33-2 or 33-5-1; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. |
| --- | --- | --- |
| [3] | ZTE | Currently, there are different views on the reporting granularity of FG33-2, i.e.., per band or per FSPC. Technically speaking, receiving multicast on which cell doesn’t have much difference in terms of implementation complexity. Meanwhile, in Rel-17, although there is only up to one carrier can be configured to receive multicast. However, in order to have a UE feature design that is future proof, it is more appropriate to make it as per band at least for FG 33-2. Proposal 1: Make FG33-2 as a per band FG. Proposal 2: Add FG6-5 (Basic DL NR-NR CA operation) as the prerequisite of FG33-2h. In FG 33-2a and FG33-4, we have the following two FFS on whether to have separate FG for separate PUCCH resource configuration. 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a. 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4. Regardless of shared PUCCH resource configuration or separate PUCCH resource configuration, the total number of PUCCH resource is kept unchanged compared with Rel-15. The only difference is where to pick the PUCCH resource. PUCCH resource configuration itself really doesn’t deserve to have a separate FG from our perspective. Proposal 3: Merge separate PUCCH resource configurations from unicast with FG33-2a and FG33-4. |
| [4] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 4 of FG 33-2, DCI format 1_0 is adjusted as DCI format 4_1; |
| [6] | Xiaomi | In RAN1#107bis e-meeting, the following agreement on how to processing multicast DCI was agreed: Agreement Regarding the number of DCIs that a UE can process in a slot or span, multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b.. The above agreement addresses MBS UE capability related to DCI processing. It should be captured in FG 33-2. We propose to add the following component for FG 33-2 in order to address the newly achieved agreement in main session. - Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b. Proposal 2: Add the following component for FG 33-2: - Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b. It is still open whether to define separate PUCCH resource configurations from unicast as separate FG from that of supporting shared PUCCH resources with unicast. Supporting separate PUCCH resource configurations for unicast and multicast respectively is an advanced UE capability. The default behavior for a UE should be transmitting UCI on shared PUCCH resources with unicast. Hence we have the following proposal: Proposal 3: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a. |
| [7] | OPPO | In FG 33-2a, one legacy issue for ACK/NACK-based feedback is whether to have a separate FG on PUCCH resource configurations from unicast. It is agreed that when PUCCH resources for multicast is not configured, the PUCCH resources configured for unicast can be used/shared by MBS multicast ACK/NACK feedback transmission. If a separate FG on PUCCH resource configuration is defined, some UEs may not support such FG while other UEs in the same multicast group can support it, the only result is that all UEs in the group have to share PUCCH resources with unicast, which may lead to increase UL feedback burden and complexity, and even impact on unicast UL feedback performance. It is suggested that separate PUCCH resource configurations from unicast should be merged as one of the components in FG 33-2a. - For FG 33-2a, define “separate PUCCH resource configurations from unicast” should be merged as one of the components in FG 33-2a. For FG 33-2e, firstly it should be clarified about the use of groupcast rather than multicast. Multicast is used for MBS reception by UEs in RRC_CONNECTED states, and broadcast is used for MBS reception by UEs in RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED states. FG 33-2 is FG related to multicast, while FG 33-1 is related to broadcast, so FG 33-2e should use multicast instead of groupcast. On the other hand, FG 33-2e is to define the capability on the number of G-RNTIs, meanwhile both broadcast and multicast transmission use G-RNTI(s). If FG 33-2e defines the number of G-RNTI(s) including both broadcast and multicast, the component description should be explicitly mentioned, i.e. “Capability on number of G-RNTI for broadcast and multicast”. Therefore, groupcast should be replaced by the exact intention terminology, e.g. “multicast” or “multicast and broadcast”. Secondly, about the details of this FG, the baseline is that UE implementation complexity and requirement should not be increased by additionally supporting G-RNTI(s) used for MBS. It was agreed that each G-RNTI is mapped with one MBS session, and Rel-17 NR MBS supports multiple sessions transmission TDM/FDM. In latest version of TS 38.331 (v17.0.0), the maximum number of G-RNTI is defined as 16. When a UE is interested in multiple MBS sessions, it can receive MBS sessions as much as it can as long as its capability allows. Furthermore, the reception of MBS cannot impact on the normal unicast reception or degrade the unicast reception performance. Only if gNB can properly schedule unicast and MBS, the system can work efficiently and UE can support up to the maximum number of G-RNTIs, i.e. 16. - For FG 33-2e, the terminology “groupcast” should be replaced by either “multicast” or “multicast and broadcast”. - For FG 33-2e, the maximum number of G-RNTI can be supported by a UE is 16. |
| [8] | Apple | Regarding ACK/NACK based feedback and NACK-only based feedback, one open issue is whether introduce a separate FG for separate PUCCH resource configuration. For ACK/NACK HARQ feedback, the separated PUCCH resource configuration can be another FG, the shared PUCCH resource configuration with unicast is the basic function of FG33-2a. If multicast HARQ-ACK bit multiplexing with HARQ bit of unicast, the unicast PUCCH resource will be applied according to the agreement that PRI indicates last DCI for unicast. If there is no separated PUCCH resource configuration for multicast, the HARQ feedback function works well. For NACK-only based HARQ feedback, the shared and separated PUCCH resource configurations can be merged into FG33-4, as one of benefits to introduce the NACK-only feedback is to save the PUCCH resource by sharing the resource among the users, and to reduce the gNB blind detection PUCCH complexity. Proposal 3: For ACK/NACK based HARQ feedback, define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a. |
| [9] | CMCC | In last RAN1 meeting, ACK/NACK based HARQ-ACK feedback for dynamic multicast scheduling is separated from the FG 33-2, and there were also some discussions about the component of PUCCH configuration in FG 33-2a with one remaining issue that whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a. During the main session discussion in Rel-17, both shared or separate PUCCH resource configuration are supported for multicast PUCCH configuration and both two options have pros and cons. As the component of PUCCH resource configuration in FG 33-2a, we prefer to keep both shared and separate PUCCH resource configuration as the components in FG 33-2a, which provides more flexibility for gNB to configure the PUCCH resource as well as the PRI indication. Proposal 2. Separate PUCCH resource configurations from unicast is defined as a component in FG 33-2a. |
| [10] | NTT DOCOMO | It was agreed at the last meeting that only a single CC can be configured for multicast reception. There would be no need to make the type of FG 33-2 more granular than per band. The type should be per band. Proposal 2: The type of FG 33-2 is per band. There would be little increase in UE complexity to support separate PUCCH resource configurations. We don’t see the need to separate FGs. Proposal 3: Include support of separate PUCCH resource configurations from unicast in FG 33-2a. UE processing of these functions would not be band dependent. The type of these FGs would be sufficient for per UE. Proposal 4: The type of FG 33-2b and 33-2f is per UE. A UE supports PTP retransmission of unicast PDSCH. There would be no need to subdivide the type of FG 33-2d to support PTP retransmission of multicast PDSCH. The type is sufficient for per UE. Proposal 5: The type of FG 33-2d is per UE. Since processing time constraints depend on SCS, the type of FG 33-2e would be better to be per band. Proposal 6: The type of FG 33-2e is per band. |
| [11] | Nokia, NSB | - 33-2: - Add 33-1 as pre-requisite, it is unclear why a UE would support multicast but not broadcast. - Merge 33-4 into this one - Per UE, as the support of this feature is not related to specific bandwidth or carrier frequency, for example. The discussion on potentially making if per FSPC has been based only on the desire to limit the number of carriers for which the feature would be supported. However, per FSPC goes way beyond that, and it makes the feature nearly impossible to deploy in practice, as network cannot know in advance which carriers will be reported as supported by different UEs. Moreover, the freedom allowed by FSPC would imply UEs can indicate support only in carriers that are not deployed at all in the network, effectively limiting the operator’s capability to plan their own network. - 33-2a/b/d/e/f: - Per UE |
| [12] | MediaTek | In the latest approved TS 38.212 spec, the three DCI formats were defined for MBS broadcast and multicast that DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. In order to align the MBS DCI format naming, we suggest the following proposal: Proposal 1: The MBS multicast broadcast DCI format naming in UE feature discussion should be aligned with that of latest approved TS 38.212 spec, i.e., MBS broadcast and multicast DCI format 4_0, DCI format 4_1, and DCI format 4_2 correspond to broadcast DCI format, multicast first DCI format and multicast second DCI format, respectively. Regarding the multicast service reception on SCell, the following agreement was achieved in last meeting: Agreement: If UE supports carrier aggregation for unicast, multicast reception on an activated SCell with self-scheduling is supported subject to UE capability in Rel-17. - UE is not expected to be configured simultaneously with more than one component carrier for multicast reception. - Cross-carrier scheduling for multicast reception is not supported in Rel-17. - The capability of supporting MBS multicast on SCell is a separate capability from the CA capability for unicast. - The granularity of UE reporting the capability of supporting MBS multicast reception is per FSPC. The part of the agreements has been reflected in the updated UE features after RAN1#108-e, however, some descriptions are not clear in [2]. For example, there is no any sentence to reflect this restriction that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception.” Thus, we suggest adding a note to reflect the agreement for both FG 33-2 and FG 33-2h. Proposal 8: For FG 33-2, adding a note that “for component 2, up to one CFR is supported for multicast reception”. Regarding the CFR number for multicast reception, the following agreement was achieved in previous RAN1 meeting: Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17.. Thus, we prefer to update the 2nd component based on the latest agreement. Proposal 9: For FG 33-2, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”. Proposal 10: For FG 33-2h, adding a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”. Since more than one component carrier for multicast reception is not supported, it implies that the UE will not support the FG 33-2 and FG 33-2h simultaneously, so, the current prerequisite FG for the FG 33-2h is not correct, which is needed to be removed. Proposal 11: For FG 33-2h, delating the prerequisite feature group description. Regarding the reporting type for the FG 33-2, the issue was discussed in last meeting and needed to be further decided between per band or per FSPC in this meeting. As we agreed before, only one CC is used for multicast reception and the reporting type for SCell CC is per FSPC, from our understanding, the similar reporting type can be used for the PCell CC. Besides, considering band 28 is shared band by operator, which means that the entire band may not only be used for MBS, and it is not very clear which carrier will be used the MBS scheduling for now, so, the reporting type of FSPC is more flexible for MBS fast commercial deployment in current stage. Proposal 12: The reporting type of FG 33-2 is per FSPC. 33-2; Dynamic scheduling for multicast for PCell; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast; FFS; Yes; [Per band or per FSPC] Per FSPC; [N/A]; [N/A]; Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception Note2: for component 2, up to one CFR is supported for multicast reception; Optional with capability signalling. 33-2h; Dynamic scheduling for multicast for SCell; Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell.; 33-2; Yes; Per FSPC; N/A; N/A; Note 1: UE is not expected to be configured simultaneously with more than one component carrier for multicast reception; Optional with capability signalling. In the RAN2#116-e meeting, one-to-many mapping between G-RNTI and MBS sessions/services was agreed as following. one-to-many mapping between G-RNTI and MBS sessions is supported and it is assumed that this does not introduce additional specification work.. Since the multiple MBS sessions associated with one G-RNTI is supported, the motivation for supporting multiple G-RNTIs for group-common PDSCH is not clear, and supporting multiple G-RNTIs will increase the probability of false alarm for PDCCH detection, which will degrade the system performance. In addition, MBS UE has introduced the multiple RNTIs for MBS feature (e.g., multicast G-RNTI/G-CS-RNTI, broadcast MCCH-RNTI/G-RNTI), however, a total number RNTI supported by UE is limited. Considering the reason above, we think up to 2 G-NRTIs are sufficient for Rel-17 UE supporting multicast services if FG 33-2e is supported. Proposal 13: For FG 33-2e, adding a note that “For component 1, the candidate value for M is: {0, 1, 2}”. 33-2e; Multiple G-RNTIs for group-common PDSCHs; - Capability on number of Support up to M G-RNTI for multicast groupcast FFS details.; 33-2; Yes; Per UE; No; No; For component 1, the candidate value for M is: {0, 1,2}; Optional with capability signalling. |
| [13] | Ericsson | In our view, it would be beneficial for feature uptake to have a separate FG for support of separate PUCCH configured for multicast. It is possible to support multicast and reuse unicast PUCCH, especially if NACK only is not supported/not in use. Therefore, a separate feature group can be signalled. Define support for a separate PUCCH resource configuration from unicast as separate FG from FG 33-2a |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs on FG 33-2 and FG33-2x as: 33. NR_MBS; 33-2; Dynamic scheduling for multicast for PCell; - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast; FFS; Yes; per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast define separate PUCCH resource configurations from unicast as separate FG (as FG33-8-1) from FG 33-2a; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2b; DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2; 33-2a, 33-2f; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2d; PTP retransmission for multicast; Support of PTP retransmission for multicast; 33-2a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2e; Multiple G-RNTIs for group-common PDSCHs; Capability on number of G-RNTI for groupcast; [33-2]; Yes; Per FSPC; N/A; N/A; Candidate values: {2, 3, ,…15}; Optional with capability signalling. 33. NR_MBS; 33-2f; Dynamic multicast with DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2g; MIMO layers for multicast PDSCH; Supported maximal number of MIMO layers for multicast PDSCH; 33-2; Yes; UE supports 1 MIMO layer only for multicast PDSCH; Per FSPC; N/A; N/A; Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2; Optional with capability signalling. 33. NR_MBS; 33-2h; Dynamic scheduling for multicast for SCell; Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell.; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-2l; Modulation order for multicast PDSCH; Supported maximal modulation order for multicast PDSCH; 33-2; Yes; - for FR1, the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled.; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a or 33-4; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-8-2; Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-ConfigurationList for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-8-1 or 33-8-3, 33-6-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[GTW1] High priority proposal 3-1:

- Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a [or 33-4] | Yes |  |  | [Per band or per FSPC] | [N/A] | [N/A] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Separate FG: Huawei, HiSilicon, Xiaomi, Apple, Ericsson, Qualcomm - if separate PUCCH resources configurations may take extra UE buffer - separate PUCCH resource configurations for unicast and multicast respectively is an advanced UE capability - Include in FG 33-2a: ZTE, OPPO, CMCC, NTT DOCOMO - the total number of PUCCH resource is kept unchanged compared with Rel-15 - If a separate FG on PUCCH resource configuration is defined, some UEs may not support such FG while other UEs in the same multicast group can support it, the only result is that all UEs in the group have to share PUCCH resources with unicast, which may lead to increase UL feedback burden and complexity, and even impact on unicast UL feedback performance - provides more flexibility for gNB to configure the PUCCH resource as well as the PRI indication |
| Moderator | This issue was discussed in the GTW session on May 9 but no consensus was achieved. Comeback in next GTW. |
| Moderator | Following combined proposal with 6-1 was discussed in the GTW on May 10 but no consensus was achieved. High priority proposal 3-1/6-1: - Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a; Yes; [Per band or per FSPC]; [N/A]; [N/A]; Optional with capability signalling. - Opt1: - Introduce new FG for support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource - FFS whether to separate FGs for Alt1 and Alt4 in the RAN1 agreement - Support of separate PUCCH resource configurations from unicast is added as a component of this new FG - Opt2: - Add the capability for support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource as a component of FG 33-4 - At least one of Alt1 and Alt4 is included in FG 33-4, FFS which one or both - Support of separate PUCCH resource configurations from unicast is added as a component of this new FG Also, some other options were proposed by companies. Let’s further discuss directly over the RAN1 reflector to converge by the next GTW. |
| Qualcomm | Opt1 is preferred. Alt4 together with separate PUCCH resource from unicast can be included in the new FG. Alt1 should be a separate FG with its prerequisite FG33-2a since NACK-only is transformed to ACK/NACK feedback. |
| Apple | If Opt1 is agreed, it implicitly means FG 33-4 only supports 1bit HARQ-ACK feedback, it seems too restrictive to FG33-4. For Opt2, it could be updated as - Opt2: - Add the capability for support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource as a component of FG 33-4 - At least one of Alt1 and Alt4 is included in FG 33-4, FFS which one or both - If Alt4 is supported, support of separate PUCCH resource configurations from unicast is added as a component of this new FG |
| Moderator | Following was agreed in the GTW on May 11. Agreement - Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a; Yes; [Per band or per FSPC]; [N/A]; [N/A]; Optional with capability signalling. - FG 33-4 includes Alt1 as a component, Add FG 33-4a including Alt4 as component with separate PUCCH resource configurations Let’s further discuss the detail of FG 33-4/4a in proposal 6-1a |

[FL1] High priority question 3-2:

- Companies are encouraged to provide views on the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e

  - 2: MediaTek

  - {2, 3, 4}: Huawei, HiSilicon

  - {2, 3, …, 15}: Qualcomm

  - 16: OPPO

| Company | Comment |
| --- | --- |
| LG Electronics | UE can indicate up to 16 services in MBS interest indication for both broadcast and multicast. Thus, the candidate value could be up to 16 in theory. However, we wonder if receiving 16 services is practical. The candidate values can be much smaller than 16. |
| Huawei, HiSilicon | Question to Qualcomm is 15 is only for G-RNTI? If it is the case, I wonder whether it is an UE specifically designed for MBS because I am assuming supporting up to 15 G-RNTI will affect unicast severely. |
| MediaTek | As we commented in proposal 2-2, supporting more RNTIs will have larger impact to the UE hardware design, and RAN2 has agreed that supporting one-to-many mapping b/w G-RNTI and MBS sessions(services). Thus, there does not need to support the multiple G-RNTIs. Considering supporting multiple G-RNTIs has been agreed to define a separate UE capability, we think it is sufficient to define up to 2 G-RNTIs for multicast. |
| Spreadtrum | Share the same view with other companies, the number of RNTIs would bring effect on UE’s hardware implementation. Practically, we think 2 may be enough where one is for high priority, and another is for low priority. |
| ZTE | We share similar view as LG. Maybe we can have {2,3,4,…16} |
| OPPO | As we mentioned in our contribution that maximum number of G-RNTI, i.e. 16, can be supported only if gNB can properly schedule unicast and MBS, which means that the reception of MBS cannot impact unicast reception or degrade unicast performance. Theoretically 16 can be supported when UE capability is allowed. For more practice reason, we also think smaller number, e.g. 2, can also work. For compromise, we can also accept other candidate max. number of G-RNTIs smaller than 16. |
| Qualcomm | To answer Huawei’s question: it is a value based on RAN2 decision of maxG-RNTI-r17. Anyway, it is a UE capability, and up to UE to report the supported max number. |
| FL2 | Summary of companies view - 2: MediaTek - {2, 3, 4}: Huawei, HiSilicon - {2, 3, …, 15}: Qualcomm - {2, 3, …, 16}: ZTE, OPPO - Less than 16: LGE As lower capable UE can report smaller values while at most the value which is allowed by spec should be included, following proposal is made [FL2] High priority proposal 3-2: - The candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e is {2, 3, …, 16} |
| Huawei, HiSilicon | we are really concerned about the impact to unicast and the potential impact to UE hardware implementation. We can be ok with a larger number as proposed by this one as long as we can agree the number is ”added” or “subtracted” as component reported in this FG. |
| ZTE | Ok with FL proposal 3-2. |
| Spreadtrum | Ok, considering 2 is included in the candidates and it is up to UE’s reporting. |
| MediaTek | Not support. From my understanding, RAN2 does not make any decision about the max number of G-RNTIs for group common PDSCH. They just use 16 as a placeholder for the running CR, and the final decision is up to RAN1’s discussion. • maxG-RNTI-r17 = 16, FFS the final value should be different based on the related RAN1 discussion on UE capabilities • maxG-CS-RNTI-r17 = 8, FFS the final value should be different based on the related RAN1 discussions on UE capabilities and G-CS-RNTI to MBS SPS mapping. As we commented in previous round, supporting larger RNTI number will have larger hardware impact, which against the Rel-17 MBS WID as copied following: Rel-17 MBS WID restriction: In order to facilitate implementation and deployment of the feature, the overall implementation impact should be limited, and the UE complexity should be minimized (e.g. device hardware impact should be avoided).. Please note that Rel-17 MBS has introduce so many RNTIs, e.g., 1 MCCH-RNTI, [1]G-RNTI for broadcast, at least 2 G-RNTIs for multicast, at least 2 G-CS-RNTIs for broadcast, so, at least 6 RNTIs value are introduced for MBS feature. If we want to further introduce more G-RNTIs, it really has larger impact to UE hardware implementation. Hopefully, we can further consider the impact for the UE side. To sum up, we think it is sufficient to define up to 2 G-RNTIs for multicast. |
| OPPO | OK with FL proposal 3-2. |
| Nokia, NSB | OK with FL proposal 3-2. We do not understand the concerns about the maximum number being large. As mentioned by other companies already, the UE has the chance to indicate what it can support, and the minimum value is actually 2 in the current proposal, not 16. This should address the concerns raised by Mediatek and Huawei, for example. |
| Qualcomm | ok |
|  | Most companies are fine with the proposal. An FFS is added to address the concern from HW, which can be discussed in GTW [GTW3] High priority proposal 3-2: - The candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e is {2, 3, …, 16} - FFS: add a component in FG 33-2e for indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation |
|  | Following was discussed in the GTW on May 12 but no consensus was achieved. Further discuss directly over the reflector. High priority proposal 3-2: - The candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e is {2, 3, …, 16} - Add a component to report the remaining number of RNTIs for the UE other than the G-RNTIs - FFS: Candidate values |
| GTW4 | Companies still have different view. Need further discussion on GTW High priority proposal 3-2: - The candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e is {2, 3, …, 16} - Alt1: Add a component to report the remaining number of RNTIs for the UE other than the G-RNTIs - FFS: Candidate values: {8, 16, 32} - Alt2: Add a component to report whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. - candidate value is {added, subtracted} - Alt3: Add a separate FG33-xx for total number of RNTIs as follows: 33-xx; Total number of RNTIs; The maximum number of all RNTIs UE can support per CC.; 33-2e and/or 33-5-1h; Yes; Per UE; No; No; Candidate values for X is: {16, 32}; Optional with capability signalling. - Alt4: Ask RAN2 to decide the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e |
| MediaTek | We don’t support to ask RAN2 since RAN2 think the final values should be decided by RAN1 UE feature discussion. As we commented earlier, we suggest to add a new alternative for the discussion: - Alt 5: The candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e is {2, 3} - Note: UE can report one of these candidate values |
|  | Following was agreed in the GTW on May 13. Further discuss FFS directly over the reflector. Agreement - There is no consensus in RAN1 for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. - FFS how to indicate RAN1 status/concern |
| Moderator | Following was agreed by email endorsement Agreement Update the previous agreement as follows: Agreement - There is no consensus in RAN1 for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. - FFS how to indicate RAN1 status/concern - Note: from RAN1 perspective, the larger number of G-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget. |

[FL1] High priority question 3-3:

- Companies are encouraged to provide views on whether to add a component in FG 33-2e for indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.

  - Support: Huawei, HiSilicon

| Company | Comment |
| --- | --- |
| Moderator | See HW/HiSi view for more detail --- In addition, FG33-2e and FG33-5-1h are currently worded for the number of supported G-RNTIs/G-CS-RNTIs for multicast. In light of avoiding device hardware impact to facilitate implementation and deployment of the feature, receiving multicast and broadcast may take the resources built for unicast from UE implementation perspective without changing the existing hardware, e.g., the maximum number of RNTIs UE supports that was dimensioned for unicast will be shared with multicast as well when to support MBS multicast. Therefore, in addition to the value of the maximum number of G-RNTIs/G-CS-RNTIs supported for multicast in FG33-2e/FG33-5-1h, UE reporting the total number of all RNTIs (e.g., as introduced in FG33-10) that UE can support can also help network schedule unicast properly. Additional reporting the total number of all RNTIs was discussed in the last meeting and the comment is mainly about the relation with other features besides NR MBS. As explained, the motivation was not changing UE hardware to support multicast/broadcast. Since the RNTIs UE supports as capability should be pre-dimensioned before UE implementation and when building multicast/broadcast on top of the current UE without changing the hardware, in addition to reporting the number of G-RNTIs or G-CS-RNTIs, reporting the total number of RNTIs (including all cases, e.g., PS-RNTI, IMT-RNTI, ….) can help network decide which feature can be configured to the UE. Without this reporting, UE may have to not report the capability of supporting some UE feature (but actually UE does have such capability) so as to control the features corresponding to the RNTIs not exceeding UE capability. It is per cell. The deficit of RNTs is originated from supporting multiple G-RNTI/G-CS-RNTI. If UE does not support MBS, this number may not be needed to be reported as legacy. From this sense, we can discuss it under MBS and even can have MBS FG33-2e/FG33-5-1h as the prerequisite FG. |
| Huawei, HiSilicon | It is real big concern from UE implementation based on not changing the hardware. If people have concern to define a separate FG reporting the total number of all RNTIs, the only way we can accept is to clarify by adding components that whether it is added or subtracted from the maximum number of supported RNTIs without multicast operation. |
| MediaTek | From our understanding, only in the MBS feature, it exists the one RNTI type will be configured multiple G-RNTI values if UE reporting the corresponding capability. For the other UE features, gNB can calculate the corresponding RNTI number if UE report that this feature is supported. Thus, we think gNB side can exactly know the total RNTIs even if UE does not this capability of max number of RNTIs. Considering the meeting progress, we are also fine to define separate FG that a maximum number of G-RNTIs supported for multicast for network double check if majority views agree this. |
| ZTE | We share similar view as MediaTek. Maybe we can directly define the maximum number of G-RNTIs. |
| vivo | Regarding adding components to clarify whether the maximum number of G-RNTIs. is added or subtracted from the maximum number of supported RNTIs without multicast operation, we have one question for clarification , how does the add components help to not change the UE’s hardware? |
| FL2 | Proponent is encouraged to address the comments/questions from companies. |
| Huawei, HiSilicon | I think MTK/ZTE are still commenting the earlier proposal from HW/HiSi that a separate FG for the max number of all RNTIs for which we still have good justification presented in the first row of this table but I doubt companies have seriously thought about. Given the current situation, we can also accept an alternative proposal as FL asked about, i.e., when UE reports FG 33-2e the number of G-RNTIs as components UE also indicates this number for G-RNTI is added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. The reason we can accept it was because the maximum number of supported RNTIs for legacy is not reported to NW. To answer vivo’s question, for example, if UE plans to build multicast on top of current UE implementation, UE can report the number of G-RNTI is “subtracted from the maximum number of supported RNTIs without multicast operation”, so it does not need to change UE hardware implementation. For advanced UE has already dimensioned to support multicast, then UE can report the number of G-RNTI is “added from the maximum number of supported RNTIs without multicast operation”. Hopefully it clarifies. |
| ZTE | @Huawei, when you say “the maximum number of supported RNTIs for legacy”, which UE feature are you referring to and what is the exact number of it? Based on our understanding, there is no such UE capability for UE to report “the maximum number of supported RNTIs for legacy” in Rel-15/16/17. |
| Moderator | This issue will be discussed together with proposal 3-2 |

[FL1] High priority question 3-4:

- Companies are encouraged to provide views on whether to introduce a new FG for supported maximal modulation order for multicast PDSCH

  - Support: Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | Proponent is encouraged to provide the motivation to introduce the new FG |
| Huawei, HiSilicon | We also wonder the reasoning of introducing this FG. |
| ZTE | We also wonder the reasoning of introducing this FG. |
| FL2 | Proponent is encouraged to provide the motivation to introduce the new FG |
| Qualcomm | From UE perspective, UE may be able to support 1024QAM for unicast traffic but not required to support it for multicast. It is same logic as FG33-2g, where different max number of MIMO layers is reported for multicast, and lower max modulation for multicast could be considered for initial multicast implementation, |
| FL3 FL4 | Companies are encouraged to check the motivation provided by the proponent |
| Huawei, HiSilicon | If the concern is about 1024QAM for multicast, we can consider making agreement that it is not supported for multicast. |
| MediaTek | Thanks for QC’s clarification. The motivation is clear for us, and we support the proposal. Huawei’s proposal is also fine for us. |
| Nokia, NSB | Adding an FG for this purpose is nearly equivalent to making 1024QAM unavailable for multicast in practice. Hence we are open for the proposal from Huawei above too, though it is unclear if we have the mandate to make that kind of explicit agreement in a UE feature session. |
| ZTE | Similar view as Nokia. |
| Spreadtrum | We are fine with Huawei’s proposal. 1024QAM is not practical for MBS. |
| vivo | We are ok with either current proposal or HW’s proposal to indicate 1024 QAM is not available for multicast. |
| GTW5 | Given most companies are fine not to support 1024 QAM for multicast, following proposal is made High priority proposal 3-4: - Add a note in FG 33-2: A UE supporting this FG and FG 36-1 is not required to support 1024 QAM for multicast in Rel-17. |
| FL5 | This proposal was discussed in the GTW but no consensus was achieved. As proponent (QC) mentioned, the motivation is not only for precluding 1024 QAM for multicast. As asked in the GTW, proponents in encouraged to elaborate the motivation for further discussion. Other companies are also encouraged to check the motivation and provide further comments, if any. |
| Qualcomm2 | The max modulation order is used for max data rate calculation. The legacy FGs are for unicast, i.e., supportedModulationOrderDL and separate FGs for 256QAM-FR2 and 1024QAM-FR1. A UE may be able to support 256QAM for FR2 and 1024QAM for FR1 for unicast, but not for multicast. In this case, we think to add a new FG for multicast is easier. Otherwise, we have to add restriction of precluding multicast in pdsch-256QAM-FR2 and FG36-1 respectively. Considering forward compatibility, multicast may be able to support same modulation as unicast, if we have a new FG for multicast then no need to introduce separate FGs of 256QAM-FR2 and 1024QAM-FR1 for multicast later. supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12].; FSPC; No; N/A; N/A. pdsch-256QAM-FR2 Indicates whether the UE supports 256QAM modulation scheme for PDSCH for FR2 as defined in 7.3.1.2 of TS 38.211 [6].; Band; No; N/A; FR2 only. |
| Huawei, HiSilicon | Thanks to Qualcomm for explanation. We think for UE receiving multicast, the modulation order UE supported for unicast can also be supported for multicast from UE perspective. In addition, this FG is used for defining the max data rate upper bound. If not talking about the scaling factor introduced for FDM-ed unicast and multicast, the max data rate defined for unicast should be the upper bound for multicast as well. Introducing another upper bound for just receiving multicast, we still failed to see the necessity. |
| GTW6 | No further comments have been provided to QC’s motivation. Proposal is made based on their preference. High priority proposal 3-4: - Introduce a new FG for supported maximal modulation order for multicast PDSCH - Candidate values for FR1: {256QAM, 1024QAM} - Candidate values for FR2: {64QAM, 256QAM} |
| FL6 | This proposal was discussed in the GTW on May 18 but no consensus was achieved. Companies are generally fine to go with this direction but need more time to check. Companies are encouraged to check the proposal and provide comments, if any. High priority proposal 3-4: - Introduce a new FG for supported maximal modulation order for multicast PDSCH - Component 1: For FR1, up to 1024QAM is supported - Component 2: For FR2, up to 256QAM is supported - Consequence if the feature is not supported by the UE: - Alt1: The UE supports up to 256QAM for FR1 and 64QAM for FR2 - Alt2: The UE supports the same modulation order as unicast - FFS additional components |
| Qualcomm | In general we support the proposal with Alt2 is selected. High priority proposal 3-4: - Introduce a new FG for supported maximal modulation order for multicast PDSCH - Component 1: For FR1, up to 1024QAM is supported - Component 2: For FR2, up to 256QAM is supported - Consequence if the feature is not supported by the UE: - Alt2: The UE supports the same modulation order as unicast - FFS additional components For the reporting type, we prefer per FSPC considering this new FG supportedModulationOrderDL-Multicast can be in a similar way as supportedModulationOrderDL for unicast to calculated the max data rate based on the supported modulation for multicast PDSCH. |
| Huawei, HiSilicon | If Qualcomm proposes this new FG is reported for FSPC and it is used for the max data rate calc, at least this is the common understanding between us how this FG is used for per the spec, although we are not so convinced yet why a different max data rate needs to be defined for multicast separately. Granted this new FG is defined, regarding the components, I am still worried whether the current description is correct or not because I am not sure even for legacy, whether 1024 can be included in this FG. Another question to Qualcomm is that: are you assuming when UE reports 1024 for FR1 unicast and supports 256 for FR2 unicast then UE also impliedly supports 1024 for FR1 multicast and 256 for FR2 multicast as well? To my understanding, I was thinking what Qualcomm probably cares the most is separating this 1024 for FR1 multicast and 256 for FR2 multicast from that for unicast, instead of separating this “supportedModulationOrderDL” from that for unicast which is just used for the max dare calc. |
| MediaTek | After further reviewing the FG 36-1 and QC’s motivation to introducing the 1024QAM feature, we have a little confused for now. To my understanding, the initial motivation to introduce the 1024QAM FG is that UE may not support the 1024QAM for multicast as QC clarification that “From UE perspective, UE may be able to support 1024QAM for unicast traffic but not required to support it for multicast.” However, the current motivation is to calculate the maximum data rate for multicast. From our understanding, the maximum data rate for multicast is smaller than or equal to the unicast data rate as we commented in main session. Considering the motivation is confused for us, we prefer the previous proposal as copied following, which also aligns with the QC’s initial motivation. High priority proposal 3-4: - Add a note in FG 33-2: A UE supporting this FG and FG 36-1 is not required to support 1024 QAM for multicast in Rel-17. Alternatively, considering the forward compatibility, maybe we can add a note in FG 33-2 that if UE reports to support the FG 33-2 and FG 36-1, the UE also can support the 1024QAM for multicast. Either is fine for us. |
| GTW7 | Further discuss in the GTW High priority proposal 3-4: - Introduce a new FG for supported maximal modulation order for multicast PDSCH - Component 1: For FR1, up to 1024QAM is supported - Component 2: For FR2, up to 256QAM is supported - Consequence if the feature is not supported by the UE: - Alt1: The UE supports up to 256QAM for FR1 and 64QAM for FR2 - Alt2: The UE supports the same modulation order as unicast - FFS additional components |
| Qualcomm | To Huawei: Even with same max modulation order, the max data rate/TBS LBRM of multicast can be different from unicast, since now we already have max MIMO layer for multicast in FG33-2g. For unicast, supportedModulationOrderDL itself does not include 1024QAM, as you said. But if FG36-1 is supported for unicast, 1024QAM will be used to calculate the max data rate for unicast for the band supporting 1024QAM in the reported FSPC. Similarly, the new FG supportedModulationOrderDL-Multicast just indicates the maximum supported modulation order to be applied for the max data rate calculation for multicast per FSPC. This new FG allows UE support different max modulation order for unicast and multicast. If UE supports 1024QAM for FR1 and/or 256QAM for FR2 for unicast only but not multicast, UE can report this new FG to explicit say 256QAM in FR1 and 64QAM in FR2 for multicast (e.g., if Alt2 is used). To MTK: As explained, not only 1024QAM for FR1, 256QAM for FR2 also may not be supported in the first implementation of multicast. For RedCap UE, 256QAM in FR1 is optional for unicast. For forward compatibility and flexibility, if RedCap UEs can support multicast, the new FG allows UE to support up to 256QAM in FR1 for unicast but up to 64QAM in FR1 for multicast. |
|  | Following was agreed in the GTW on May 20. Agreement - Introduce a new FG for supported maximal modulation order for multicast PDSCH - Component 1: For FR1, up to 1024QsAM is supported, candidate values {256QAM, 1024QAM} - Component 2: For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM} - Consequence if the feature is not supported by the UE: - The UE supports the same modulation order as unicast - FFS additional components - Add a note: A UE shall support the corresponding mandatory maximum modulation for unicast. |

[GTW1] High priority question 3-5:

- Companies are invited to provide view whether you can live with the reporting type of FG 33-2 being per band or per BC

| Company | Comment |
| --- | --- |
| Moderator | - FG 33-2 - Per UE: Nokia, NSB - support of this feature is not related to specific bandwidth or carrier frequency, for example. The discussion on potentially making if per FSPC has been based only on the desire to limit the number of carriers for which the feature would be supported. However, per FSPC goes way beyond that, and it makes the feature nearly impossible to deploy in practice, as network cannot know in advance which carriers will be reported as supported by different UEs. Moreover, the freedom allowed by FSPC would imply UEs can indicate support only in carriers that are not deployed at all in the network, effectively limiting the operator’s capability to plan their own network - Per band: ZTE, NTT DOCOMO - receiving multicast on which cell doesn’t have much difference in terms of implementation complexity - only a single CC can be configured for multicast reception - Per BC: Huawei, HiSilicon - UE reporting larger CA bandwidth class for a BC then UE may not report the support of FG33-2 - Per FSPC: Huawei, HiSilicon, MediaTek, Qualcomm - Same as FG 33-2h - band 28 is shared band by operator, which means that the entire band may not only be used for MBS, and it is not very clear which carrier will be used the MBS scheduling for now, so, the reporting type of FSPC is more flexible for MBS fast commercial deployment in current stage - FG 33-2a - Per UE: Huawei, HiSilicon, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2b - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2d - Per UE: NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2e - Per UE: Huawei, HiSilicon, Nokia, NSB - Per band: NTT DOCOMO - Per FSPC: MediaTek, Qualcomm - FG 33-2f - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2g - Per UE: - Per FSPC: MediaTek, Qualcomm - FG 33-2h - Per UE: - Per FSPC: MediaTek, Qualcomm For the 1st round, let’s focus on FG 33-2. If you cannot live with per band or per BC, please provide alternative proposal which can be agreeable for all companies. |
| Moderator | This issue was discussed in the GTW session on May 9 but no consensus was achieved. Comeback in next GTW with following alternatives as starting point. High priority question 3-5: - Down select the reporting type of FG 33-2 from the followings - Per band: Nokia, ZTE - Per BC: HW - Per FSPC: SPRD, HW, QC, MTK, Apple - Per band and per BC: QC - Per FS: [HW, ZTE], MTK |
| Moderator | Following was agreed in the GTW on May 10 Agreement - Reporting type of FG 33-2 is per FS |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update - FG 33-2a - Per UE: Huawei, HiSilicon, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2b - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2d - Per UE: NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2e - Per UE: Huawei, HiSilicon, Nokia, NSB - Per band: NTT DOCOMO - Per FSPC: MediaTek, Qualcomm - FG 33-2f - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB - Per FSPC: MediaTek, Qualcomm - FG 33-2g - Per UE: - Per FSPC: MediaTek, Qualcomm - FG 33-2h - Per UE: - Per FSPC: MediaTek, Qualcomm |
| Huawei, HiSilicon | Ok to have per FS as FG33-2. |
| MediaTek | From my understanding, the following FGs have been agreed in previous meeting: - FG 33-2g and FG 33-2h have been agreed in R1#108-e Agreement - FG 33-2g is updated as follows 33. NR_MBS; 33-2g; MIMO layers for multicast PDSCH; Supported maximal number of MIMO layers for multicast PDSCH; 33-2; Yes; UE supports 1 MIMO layer only for multicast PDSCH; Per FSPC; Optional with capability signalling. Agreement - FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case - Type of FG for SCell case is per FSPC And the corresponding modifications have been reflected in the updated RAN1 UE features list for Rel-17 NR after RAN1 108-e (R1-2202928 ) 33-2g; MIMO layers for multicast PDSCH; Per FSPC. 33-2h; Dynamic scheduling for multicast for SCell; Per FSPC. - FG 33-2e has been agreed by RAN2 that the reporting type is per UE in the approved TS 38.306 as copied following. maxNumberRNTIs-MBS-r17 Indicates the maximum number of simultaneous reception of PDCCH scrambled with G-RNTIs/G-CS-RNTIs for MBS multicast.; UE; No; No; No.. Regarding the other FGs, e.g., FG 33-2a, FG 33-2b, FG 33-2d, FG 33-2f, our preference is per FSPC. |
| Spreadtrum | For FG 33-2g, 33-2h, last meeting we have agreed and not marked in yellow in endorsed UE feature list last meeting. Further discussion is not needed. For other FGs, we are fine as per FS. |
| Nokia, NSB | As observed by Spreadtrum above, FG 33-2g, and 33-2h are not up to discussion since they have been agreed to already, even if we think 33-2h should be revised to per FS given the agreement on 33-2. For the other FGs, since they all have 33-2 as pre-requisite, it is sufficient to define the signalling as per UE. The corresponding features will be supported only where the pre-requisites are fully satisfied, and no need to repeat the signalling over the different band combinations. |
|  | Thank you for pointing out my mistake. Summary of companies view - FG 33-2a - Per UE: Nokia, NSB, ZTE - Per FS: Huawei, HiSilicon, SPRD, MTK, Apple - Per FSPC: MediaTek, Qualcomm - FG 33-2b - Per UE: NTT DOCOMO, Nokia, NSB - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-2d - Per UE: Nokia, NSB, - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-2e - Per UE: Nokia, NSB, MTK (agreeed?) - Per band: NTT DOCOMO - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: Qualcomm - FG 33-2f - Per UE: NTT DOCOMO, Nokia, NSB - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
|  | Reporting type of FG 33-2a was discussed in the GTW on May 12 but no consensus was achieved. Further discuss directly over the reflector. |
| GTW4 | Further discuss on the GTW High priority proposal 3-5a: - Down select the reporting type of FG 33-2a from the followings - Per UE: Nokia, NSB, ZTE - Per FS: Huawei, HiSilicon, SPRD, MTK, Apple, Qualcomm - Per FSPC: MediaTek, Qualcomm Further discuss over the reflector |
| Moderator | Followings were agreed in the GTW on May 16. Agreement - Reporting type of FG 33-2a is per BC Agreement - Reporting type of FG 33-2b is per band Agreement - Reporting type of FG 33-2d is per BC - FFS: Revise the component: Support of PTP retransmission for multicast on the cell same as multicast initial transmission Following proposal was discussed in the GTW on May 16 but no consensus was achieved. Further discuss directly over the reflector. Proposal - Reporting type of FG 33-2e is [per UE or band] - FFS whether it is conflicting with RAN2 agreement Remaining issue Reporting type of FG 33-2f is per UE/band/FS/FSPC |
| GTW5 | Followings were agreed in the GTW on May 17. Agreement - Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 Agreement - Reporting type of FG 33-2f is per band |

Low priority proposal 3-6:

- Components of FG 33-2 are revised as

  - Component 4: Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast.

  - Component 5: Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots.

  - Add a note that “Multicast DCI is treated as unicast DCI scheduling DL following the current feature group 3-1/3-5a/3-5b”.

  - Add a note that “for component 2, up to one CFR is supported for multicast reception”.

  - Add a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”

- Components of FG 33-2e is revised as

  - Component 1: The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC.

- Components of FG 33-2f is revised as

  - Component 1: Support of DCI format 4_2 with CRC scrambled with G-RNTI/G-CS-RNTI for multicast.

- Components of FG 33-2h are revised as

  - Add a note that “UE is not expected to be configured simultaneously with more than one component carrier for multicast reception”

| Company | Comment |
| --- | --- |

Low priority question 3-7:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 33-2 to 33-2h

  - FG 33-2

    - Add 33-1: Nokia, NSB

  - FG 33-2b

    - Add 33-2f: Qualcomm

  - FG 33-2h

    - Add FG6-5: ZTE

    - delete the description: MediaTek

| Company | Comment |
| --- | --- |

- 33-3-1: Dynamic Slot-level repetition for group-common PDSCH

In [1], FG 33-3-1 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the Gnb to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | As discussed in section 2.1, dynamic slot-level repetition is proposed to be a separate FG from FG33-1 for broadcast and it can be merged into FG33-3-1 with FG33-1 as prerequisite FG for broadcast. The FG33-3-1 can be updated as the following proposal. Proposal 5: Updating FG33-3-1 as follows in red: 33. NR_MBS; 33-3-1; Dynamic Slot-level repetition for group-common PDSCH; - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast or for broadcast MTCH.; 33-1 or 33-2; Yes; Per UE; No; No; Candidate values for X is: {8, 16}; Optional with capability signalling. |
| --- | --- | --- |
| [5] | vivo | DCI indicated slot-level repetition for MTCH could be merged with FG 33-3-1. 33. NR_MBS; 33-3-1; Dynamic Slot-level repetition for group-common PDSCH; - Support up to X times dynamic slot-level repetition for group-common PDSCH with CRC scrambled by G-RNTI.; 33-1 or 33-2; Yes; [Per UE]; [No]; [No]; Candidate values for X is: {8, 16}; Optional with capability signalling. |
| [7] | OPPO | For FG 33-3-1, the upper bound of dynamic slot-level repetition for group-common PDSCH for multicast has two candidates for further down selection, i.e. 8 or 16. In this contribution about FG 33-1, it is proposed not to merge the “DCI indicated slot-level repetition up to 16 for MTCH” with FG 33-3-1, but the number of repetitions can be aligned since they are both related to DCI-based indication on the repetition. Therefore, FG 33-3-1 should also support up to 16 times dynamic repetition for multicast. - For FG 33-3-1, support up to 16 times dynamic slot-level repetition for group-common PDSCH for multicast. |
| [10] | NTT DOCOMO | Since the type of FG for dynamic slot-level repetition for unicast is per band, the type of dynamic slot-level repetition for multicast should also be per band. Proposal 7: The type of FG 33-3-1 is per band. |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs of FG 33-3-x and add FG33-3-2a for additional scaling factor for max data rate to support FDMed unicast PDSCH and multicast PDSCH, where details of the additional scaling factor are discussed in [2]. 33. NR_MBS; 33-3-1; Dynamic Slot-level repetition for group-common PDSCH; - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast.; 33-2; Yes; Per FSPC; N/A; N/A; Candidate values for X is: {8, 16}; Optional with capability signalling. |

Discussion

High priority question 4-1:

- Companies are encouraged to provide views on the reporting type of FG 33-3-1.

  - Per UE: Huawei, HiSilicon

  - Per band: NTT DOCOMO

  - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update - Per UE: Huawei, HiSilicon - Per band: NTT DOCOMO - Per FSPC: MediaTek, Qualcomm |
| vivo | Our position is mis-captured, just fix it. |
| Huawei, HiSilicon | Per FS is also acceptable. |
| MediaTek | Per FSPC |
| Spreadtrum | Either of Per FS or per UE is fine to us. |
| Nokia, NSB | Per UE is fine, given that the pre-requisite is per FS already. |
|  | Summary of companies view - Per UE: Huawei, HiSilicon, vivo, SPRD, Nokia - Per band: NTT DOCOMO - Per FS: HW, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Apple | Per FS |
| Moderator | No consensus was achieved in this meeting. Comeback next meeting. |

Low priority question 4-2:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-3-1.

  - Update to “FG 33-1 or FG 33-2”: Huawei, HiSilicon, vivo

| Company | Comment |
| --- | --- |

Low priority question 4-3:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 33-3-1 which do not have capability signalling impacts.

| Company | Comment |
| --- | --- |

- 33-3-2 to 33-3-5: Multiplexing of unicast PDSCH and group-common PDSCH

In [1], FGs 33-3-2 to 33-3-5 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | [TBD], 33-3-3b | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | [33-2b] | Yes |  |  | [Per FSPC] | [No] | [No] |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination including supportedBandwidthDL, maxNumberMIMO-LayersPDSCH and supportedModulationOrderDL that can determine an maximum TB size in a slot. For UE supporting FDM-ed unicast PDSCH and one group-common PDSCH in a slot, the maximum TBS size is supposed to be the total maximum TB size for both unicast and group-common PDSCH, which should be clarified if it is not reported. It should be noted that two cases (i.e., FDM-ed unicast and multicast and FDM-ed unicast and broadcast) can be considered but separate UE capabilities indications should be set. MBS broadcast may be received by RRC_IDLE/INACTIVE UEs in addition to RRC_CONNECTED UEs, so even UE reports the support of FDM-ed unicast and broadcast, from network perspective, the scheduling for broadcast would take precedence over the scheduling for unicast because UEs in RRC_IDLE/INACTIVE UEs may not support FDM-ed unicast and broadcast. However, multicast is transmitted to UEs in RRC_CONNECTED state and then network has the full information of UEs regarding whether support FDM-ed unicast and multicast, so scheduling of unicast and multicast can be more flexibly adjustable based on the capability reporting. In addition, it should be reported per FSPC. Proposal 6: Updating FG33-3-2 and adding FG33-3-2a as follows in red: 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and one group-common PDSCH for broadcast in a slot; - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-1 or 33-2; Yes; Per FSPC; NA; NA; Optional with capability signalling. 33. NR_MBS; 33-3-2a; FDM-ed unicast PDSCH and one group-common PDSCH for multicast in a slot; - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-2; Yes; Per FSPC; NA; NA; Optional with capability signalling. Similar rationale for the update for FG33-3-2, the maximum TBS size supported in a slot should be clarified as the total maximum TB size for both unicast and group-common PDSCH when UE supports intra-slot TDM-ed unicast and group-common PDSCH. It should be noted that two cases (i.e., FDM-ed unicast and multicast and FDM-ed unicast and broadcast) are included depending on FG33-1 or FG33-2 is prerequisite FG for FG33-3-3. In addition, it should be reported per FSPC. Since FG33-3-3 is reported separately from FG33-1, MBS broadcast transmission should be based on the assumption that there are UEs supporting FG33-1 but not support FG33-3-3. Therefore, the component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC should be only applied to multicast with FG33-1 as prerequisite FG. Proposal 7: Updating FG33-3-3 as follows in red: 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; 33-1 or 33-2; Yes; Per FSPC; NA; NA; Note: component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC is only applied to multicast with FG33-1 as prerequisite FG.; Optional with capability signalling. |
| --- | --- | --- |
| [7] | OPPO | For FG 33-3-3a and 33-3-3b, there is a common legacy issue about the value of X G-RNTIs. One baseline is that here the value of X should not exceed the G-RNTI number defined in FG 33-2e which is the total number of G-RNTIs supported by a UE. Following this baseline, the value of X should be any integer number, e.g. 1, 2, 3 and etc. - For FG 33-3-3a and 33-3-3b, the value of X G-RNTIs should be no larger than the number of G-RNTI defined in FG 33-2e. |
| [9] | CMCC | In the main session discussion, the TDMed and FDMed between unicast and group-common PDSCH are both applied for dynamic and SPS transmission, but the current prerequisite FG for FG 33-3-2 and FG 33-3-3 only consider dynamic multicast/broadcast scheduling. Therefore, FG 33-5-1 should also be the prerequisite FG for FG 33-3-2 and FG 33-3-3. Proposal 6. FG 33-5-1 should be the prerequisite FG for FG 33-3-2 and FG 33-3-3. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot.; 33-1, 33-2，33-5-1. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; 33-1, 33-2,33-5-1. |
| [10] | NTT DOCOMO | Since the type of FG for the number of TDM-ed unicast PDSCHs in a slot is per FS, the type of FG 33-3-2 and 33-3-3 should also be per FS. Proposal 8: The type of FG 33-3-2 and 33-3-3 is per FS. FG 33-3-2 should be a prerequisite FG of FG 33-3-3a. FG 33-3-3 should be a prerequisite FG of FG 33-3-3b. Since the type of FG for Type-1 and Type-2 HARQ-ACK codebook for unicast is per UE, the type of FG 33-3-3a, 33-3-3b and 33-3-4 should also be per UE. Proposal 9: Add FG 33-3-2 as a prerequisite FG of FG 33-3-3a. Proposal 10: Add FG 33-3-3 as a prerequisite FG of FG 33-3-3b. Proposal 11: The type of FG 33-3-3a, 33-3-3b and 33-3-4 is per UE. Since the type of FG for support of simultaneous construction of two HARQ-ACK codebooks for unicast is per FS, the type of FG 33-3-5 should also be per FS. Proposal 12: The type of FG 33-3-5 is per FS. |
| [11] | Nokia, NSB | - 33-3-2: - Per UE - 33-3-3: - Per UE - 33-3-3a: - Per UE - 33-3-3b: - Per UE - 33-3-4: - Per UE - 33-3-5: - Per UE, motivation for finer granularity is not clear. If limitations on level of support is needed, then it is preferrable to add those restrictions explicitly in the component description. |
| [12] | MediaTek | Regarding the prerequisite feature groups 33-1, we think the broadcast reception is “best-effort” reception and the UE in RRC IDLE/INACTIVE cannot report capability. So, we think FDMed reception for MBS is supported for multicast and not broadcast. Proposal 14: For FG 33-3-2, the prerequisite feature groups 33-1 should be deleted. 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.; 33-1, 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. Regarding the FG 33-3-3, we share the similar view as that of FG 33-3-2 and has the following proposal: Proposal 15: For FG 33-3-3, the prerequisite feature groups 33-1 should be deleted. 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC - Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s).; 33-1, 33-2; Yes; Per UE Per FSPC; No; No; Optional with capability signalling. |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs of FG 33-3-x and add FG33-3-2a for additional scaling factor for max data rate to support FDMed unicast PDSCH and multicast PDSCH, where details of the additional scaling factor are discussed in [2]. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH; - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot.; [33-1, 33-2]; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-3-2a; Calculating maximum data rate of FDMed unicast PDSCH and group-common PDSCH; Scaling factor for max data rate to support FDMed unicast PDSCH and group-common PDSCH in a slot per CC.; 33-3-2; Yes; If not reported, same as the scaling factor for max data rate of unciast PDSCH; Per FSPC; N/A; N/A; Value of scaling factor: {1.6, 1.2, 1, and 0.75}; Optional with capability signalling. 33. NR_MBS; 33-3-3; Intra-slot TDM-ed unicast PDSCH and group-common PDSCH; - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).; [33-1, 33-2]; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-3-3a; FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast; - Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs; 33-2a, 33-3-2; Yes; Per FSPC; N/A; N/A; Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e; Optional with capability signalling. 33. NR_MBS; 33-3-3b; Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast; - Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast - Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs; 33-2a; Yes; Per FSPC; N/A; N/A; Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {2, 3, 4} with X no lareger than max number of G-RNTIs of FG33-2e; Optional with capability signalling. 33. NR_MBS; 33-3-4; Mode 1 for type1 codebook generation; Supports type1-Codebook-Generation-Mode configured as mode 1; 33-3-3b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-3-5; Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type; Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot; 33-2a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[FL1] High priority proposal 5-1:

- Candidate values for X in FGs 33-3-3a/33-3-3b is {2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e

- Components of FG 33-3-3a are revised as

  - Add a component for support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs.

- Components of FG 33-3-3b are revised as

  - Add a component for support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast with max number X of G-RNTIs.

  - Support: Qualcomm

| Company | Comment |
| --- | --- |
| LG Electronics | OK |
| Huawei, HiSilicon | We failed to see the necessity. |
| MediaTek | Since it has agreed to define a separate UE capability for supporting multiple G-RNTIs, i.g., FG33-2e, we think it will be better that not mentioned the G-RNTIs number in here, we suggest deleting the wording of “with max number X of G-RNTIs”. |
| Spreadtrum | The motivation is not clear to us |
| ZTE | Similar view as Huawei, MediaTek and Spreadtrum. |
| OPPO | Similar view with Huawei, MediaTek, Spreadtrum and ZTE. |
| Qualcomm | Here is for codebook generation, where the size of sub-codebook concatenation will be increased due to larger number of G-RNTIs. According to RAN2 maxG-RNTI-r17, which can be as large as 16, it is impossible for UE to support 16 subcodebook concatenation. |
| FL2 | Proponent provided the motivation. Companies are encouraged to check the view and provide further comments, if any. |
| Huawei, HiSilicon | If UE is not able to provide full size of codebook for the reported number of G-RNTIs, then UE does not need to report such high number of G-RNTIs. What does UE expect to happen if the number of G-RNTIs for codebook is less than the number of G-rNTIs UE reports? NW should not configure the max number of G-RNTIs to UE or NW should disable the HARQ-ACK feedback when the max number of G-RNTis UE reports are configured to UE? I am just worried about it would further complex NW scheduling unnecessarily and complex the discussion on how to generate HARQ-ACK codebook… |
| Qualcomm | To Huawei, for case of Type-2 codebook, only the sub-codebook for G-RNTI(s) with feedback enabled are to be concatenated. So it is clear that the number of G-RNTIs UE can support and the number of G-RNTIs included in the feedback codebook are different features. We already supported flexible GC-DCI enabling/disabling to dynamically adjust the codebook size, which should be no larger than the UE capability. |
|  | [GTW3] Need further discussion in GTW |
| FL3 FL4 | This proposal could not be discussed in the GTW on May 12. Proponents provide further clarification. Companies are encouraged to check the view and provide further comments, if any. |
| Huawei, HiSilicon | Thanks to Qualcomm for clarification. Indeed, the number of G-RNTIs for the codebook varies upon the G-RNTIs configured with HARQ-ACK enabled but it is up to NW to enable/disable HARQ-ACK. The UE capability whether can support the CB size with the number of G-RNTI as the same number of G-RNTIs in general UE supports will even complicate more NW scheduling and specification maintenance. |
| Nokia, NSB | We tend to agree with Huawei that such complication is not welcome and we do not support the proposal. |
| ZTE | Similar view as Nokia. |
| GTW5 | Given most companies are not convinced with the motivation from proponent, following proposal is made High priority proposal 5-1: - “FFS value of X G-RNTIs” is deleted from FGs 33-3-3a and 33-3-3b |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | ok |
| MediaTek | Support the proposal |
| GTW6 | High priority proposal 5-1: - “FFS value of X G-RNTIs” is deleted from FGs 33-3-3a and 33-3-3b |
| FL6 | No objection has been received in the last round. Proponent is encouraged to check whether you can live with it. If not, please provide another proposal which is acceptable to all |
| Qualcomm | We still have some concern for the codebook generation. If UE is configured with 8 G-RNTIs plus unicast with multiple CCs, the UE may have to concatenate subcodebooks of 8 multicast G-RNTIs in addition to all the CCs for unicast C-RNTI. This X is optionally reported. If the UE does not report X, by default the UE can support codebook generation for all the configured G-RNTIs. |
| Huawei, HiSilicon | To follow-up Qualcomm’s comment: in addition, the spec also needs to describe when UE reports this X, the codebook should generate per the reported number of G-RNTIs, which will even complicate more spec maintenance. In addition, we are still discussing how to count the HARQ-ACK bits for NAKC-only for Alt1/Alt4, including whether Alt4 is applied to more than G-RNTI or the codebook should also consider the G-RNTI even not configured for a given UE, etc, it has been complicated. This capability of X if defined will even more complicate that discussion. If not essentially necessary, we should avoid that situation. |
| GTW7 | Further discuss in the GTW High priority proposal 5-1: - “FFS value of X G-RNTIs” is deleted from FGs 33-3-3a and 33-3-3b |
| Qualcomm | To Huawei: This X only applies to Type-2 CB. The size of Type-2 CB is not fixed, which is dependent on the supported X G-RNTIs with HARQ-ACK feedback. gNB can still configure up to (8-X) G-RNTIs without HARQ-ACK feedback. Even if gNB wants to enable all configured G-RNTIs to support HARQ-ACK feedback, gNB can configure Type-1 CB with a fixed CB size independent from the number of configured G-RNTIs. The FG33-2a is the prerequisite of FGs 33-3-3a and 33-3-3b, which includes the CB type and enabling/disabling per G-RNTI for gNB flexible configuration. If no X, it means UE has to support all configured G-RNTIs for both Type-1 and Type-2 CBs, which we think it’s not reasonable. |
| Moderator | No consensus was achieved in this meeting. Comeback next meeting. |

[FL1] High priority question 5-2:

- Companies are encouraged to provide views on whether to introduce a new FG for scaling factor for max data rate to support FDMed unicast PDSCH and group-common PDSCH in a slot per CC.

  - Support: Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | Proponent is encouraged to provide the motivation to introduce the new FG |
| Huawei, HiSilicon | We share the sympathy of the motivation. However, some details needs to be fixed so I wonder whether would be more efficient to be discussed in the AI8.12.2 first? |
| ZTE | Similar view as Huawei, we prefer to have clear understanding on the intended UE behavior in main session first and then come back to this FG discussion. |
| vivo | We share similar view as other companies, this issue can be discussed in the main session first. |
| OPPO | Same view with other companies that it can be discussed in the main session. |
| FL2 | As suggested by some companies, this issue can be discussed at AI8.12.2 at first. |

[FL1] High priority question 5-3:

- Companies are encouraged to provide views on whether to split FG 33-3-2 into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH.

  - Support: Huawei, HiSilicon

| Company | Comment |
| --- | --- |
| Moderator | See HW/HiSi view for more detail: --- It should be noted that two cases (i.e., FDM-ed unicast and multicast and FDM-ed unicast and broadcast) can be considered but separate UE capabilities indications should be set. MBS broadcast may be received by RRC_IDLE/INACTIVE UEs in addition to RRC_CONNECTED UEs, so even UE reports the support of FDM-ed unicast and broadcast, from network perspective, the scheduling for broadcast would take precedence over the scheduling for unicast because UEs in RRC_IDLE/INACTIVE UEs may not support FDM-ed unicast and broadcast. However, multicast is transmitted to UEs in RRC_CONNECTED state and then network has the full information of UEs regarding whether support FDM-ed unicast and multicast, so scheduling of unicast and multicast can be more flexibly adjustable based on the capability reporting. |
| MediaTek | As Huawei mentioned, supporting FDMed b/w unicast and broadcast is not supported for RRC IDLE/INACTIVE UEs. Besides, considering the broadcast is best-effort reception and without strict latency and reliability requirement, we think there is no need to support the FDMed case for broadcast and suggest only keep the multicast PDSCH only in FG 33-3-2, e.g., delete the prerequisite FG 33-1 in the FG33-3-2. |
| vivo | We understand that supporting of FDM between one unicast PDSCH and one broadcast PDSCH is for RRC-connected UEs, as there is no unicast for RRC_IDLE/INACTIVE UEs. We think separated FG is not needed. |
| FL2 | Proponent is encouraged to address the comments from companies, if any. |
| Huawei, HiSilicon | The FG for FDM-ed broadcast and unicast is reported for assisting NW scheduling when UE is in RRC_connected state. When UE does not report this FG and when UE receives broadcast in RRC_CONNECTED state, NW will schedule unicast not to be partially or fully overlapped with broadcast in time domain. |
| MediaTek | As we commented in previous round, we prefer the FG 33-3-2(FDMed case) only used for multicast and delete the prerequisite FG 33-1(broadcast) in the FG33-3-2. |
| Qualcomm | Multicast and broadcast are separate features. If UE can support FDMed unicast and broadcast in RRC_CONNECTED mode, we prefer it is a new FG, separate from FG33-3-2. |
|  | [GTW3] Need further discussion in GTW |
| FL3 FL4 | This proposal could not be discussed in the GTW on May 12. Proponents provide further clarification. Companies are encouraged to check the view and provide further comments, if any. |
| Huawei, HiSilicon | Supporting separate FG33-3-2 for broadcast +unicast can address MTK’s concern I suppose. |
| MediaTek | If FDMed case is supported for broadcast, we prefer making it as a separate FG since broadcast and multicast are two separate FGs. |
| Spreadtrum | Slightly prefer separate UE capability |
| vivo | Ok for a separate FG for broadcast for progress |
| GTW5 | Given most companies are fine to separate the FG, following proposal is made High priority proposal 5-3: - FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.; [33-1, 33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in a slot.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | Ok. Note: 33-1-2, typo, it should be “FDM-ed unicast PDSCH and group-common PDSCH for broadcast” |
| MediaTek | As Huawei indicated, there is a typo for FG 33-1-2. Although we do not support FDMed case for the broadcast case since broadcast reception is a best effort reception, we are generally fine with the direction for the proposal as a compromise as commented in previous round. However, we do not prefer leaving all the other column to be decided by RAN2 since supporting FDMed case was discussed and agreed in previous RAN1 meeting and RAN2 does not have more background for the discussion. Also, we know that the other column for FG 33-1 has been agreed to be decided by RAN2. As a compromise, at least the last column needed to be decided in RAN1. Considering whether support the FDMed case for broadcast is based on the UE’s capability, we suggest the last column should be modified as “Optional with capability signalling” 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast broadcast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in a slot.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2 Optional with capability signalling. |
| GTW6 | Thank you very much for pointing out the typo. The proposal is updated accordingly. Further discuss whether to defer to RAN2 for the columns. High priority proposal 5-3: - FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.; [33-1, 33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for broadcast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in a slot.; 33-1; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2. |
| FL6 | This proposal was discussed in the GTW on May 18 but no consensus was achived. Companies are generally fine to have these two FGs. The discussion point is whether to leave some columns to RAN2 or decide in RAN1. Companies are encouraged to provide view on 1) Whether FG 33-1-2 is Optional with capability signalling or the columns is left to RAN2 2) Whether the reporting type of FG 33-1-2 is decided in RAN1 or left to RAN2 High priority proposal 5-3: - FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.; [33-1, 33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for broadcast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot.; 33-1; Yes; Up to RAN2; Up to RAN2; Up to RAN2; Optional with capability signalling. |
| OPPO | For the two questions from FL, we would say that Neither should be up to RAN2. For FG 33-1-2, we would like to ask question for clarification: whether the network know that a RRC_CONN UE is receiving a broadcast? If I remember it correctly, we had an agreement that RRC_CONN UEs can report broadcast reception interest to NW. With the interest reception indication from a UE, NW may assume this UE is receiving the MBS broadcast. By reporting FG 33-1-2 to gNB, NW can also know this UE is capable for FDM reception of unicast PDSCH and broadcast PDSCH in one slot. |
| Qualcomm | The reporting type of both 33-3-2 and 33-1-2 should decided by RAN1. We prefer per FSPC for both 33-3-2 and 33-1-2. The reason is that - Which CC of a CC group can support multicast/broadcast is per FSPC for SCell case. - UE reports IE FeatureSetDownlinkPerCC for the parameters of max data rate is per FSPC, whether the UE can support FDMed unicast and multicast/broadcast PDSCHs is different in the CC with multicast/broadcast of a CC group. |
| Huawei, HiSilicon | Firstly, FG33-3-2 can also be clarified in bullet 1 that it is for RRC_CONNECTED though it may have been obvious to everyone. Secondly, we actually agree with Qualcomm that both FG33-3-2 and FG33-1-a should be decided by RAN1 and should be per FSPC, and the reason is exactly the same as Qualcomm pointed out. |
| MediaTek | As we commented in previous round, it should be decided by RAN1 that “FG 33-1-2 is Optional with capability signalling”. I think it is clearly to RAN1 that support FDMed for broadcast is an optional UE capability. If the FG is optional without capability, it means that UE always needs to support the FDMed case for broadcast, which is not accepted for us since supporting FDMed needs more processing effort and hardware/software impact, which against the Rel-17 MBS WID restriction. Thus, we support the FG 33-1-2 if defined is Optional with capability signalling, otherwise, we do not support FDMed for broadcast. In addition, regarding the reporting type for FG 33-3-2 and FG 33-1-2 if supported, we agree with QC’s comments that these needs to be decided by RAN1 and FSPC is supported. |
| GTW7 | Proposal is updated based on the comments so far. High priority proposal 5-3: - FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot.; [33-1, 33-2]; Yes; [Per UE] Per FSPC; [No] N/A; [No] N/A; Optional with capability signalling. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for broadcast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot.; 33-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
|  | Following was agreed in the GTW on May 19 Agreement - FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and group-common PDSCH for multicast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot.; [33-1, 33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for broadcast; 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot.; 33-1; Yes; FFS; FFS; FFS; Optional with capability signalling. |

High priority question 5-4:

- Companies are encouraged to provide views on the reporting type of FGs 33-3-2 to 33-3-5.

  - FG 33-3-2

    - Per UE: Nokia, NSB

    - Per FS: NTT DOCOMO

    - Per FSPC: Huawei, HiSilicon, MediaTek, Qualcomm

  - FG 33-3-3

    - Per UE: Nokia, NSB

    - Per FS: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-3-3a

    - Per UE: NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-3-3b

    - Per UE: NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-3-4

    - Per UE: NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-3-5

    - Per UE: Nokia, NSB

    - Per FS: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update |
| Huawei, HiSilicon | At least FG33-3-2 and FG33-3-3 should stick to per FSPC because this FDM-ed and TDM-ed multiplexing |
| MediaTek | Per FSPC |
| Spreadtrum | Fine with all FGs as per FS. For FG 33-3-3b/33-4, also fine as per UE. |
| Nokia, NSB | Fine with per UE, given that all those depend on 33-2 already. |
| Qualcomm | We also prefer per FSPC. Especially for FG33-3-2 and 33-3-3, which impact the data rate and UE processing capability per slot in one CC of CA, it should be per FSPC. |
|  | Summary of companies view - FG 33-3-2 - Per UE: Nokia, NSB - Per FS: NTT DOCOMO, SPRD - Per FSPC: Huawei, HiSilicon, MediaTek, Qualcomm - FG 33-3-3 - Per UE: Nokia, NSB - Per FS: NTT DOCOMO, SPRD - Per FSPC: MediaTek, Qualcomm, Huawei, HiSilicon, - FG 33-3-3a - Per UE: NTT DOCOMO, Nokia, NSB - Per FS: SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-3-3b - Per UE: NTT DOCOMO, Nokia, NSB, SPRD - Per FS: SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-3-4 - Per UE: NTT DOCOMO, Nokia, NSB, SPRD - Per FS: SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-3-5 - Per UE: Nokia, NSB - Per FS: NTT DOCOMO, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Moderator | No consensus was achieved in this meeting. Comeback next meeting. |

Low priority proposal 5-5:

- Components of FG 33-3-2 are revised as

  - Component 1: Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.

- Components of FG 33-3-3 are revised as

  - Component 1: Support TDM between one unicast PDSCH and one group-common PDSCH for multicast in a slot.

  - Component 2: Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH for multicast in a slot per CC

  - Component 3: Support TDM among N (N>1) group-common PDSCHs for multicast in a slot per CC

  - Component 4: Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs for multicast in a slot per CC

  - Component 5: The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. Note:  Group-common PDSCH(s) for multicast are counted as unicast PDSCH(s).

  - Add a note that “component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC is only applied to multicast with FG33-1 as prerequisite FG.”.

Low priority question 5-7

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 33-3-2 to 33-3-5

  - FG 33-3-2

    - Delete FG 33-1: MediaTek

    - Add FG 33-5-1: CMCC

  - FG 33-3-3

    - Delete FG 33-1: MediaTek

    - Add FG 33-5-1: CMCC

  - FG 33-3-3a

    - Add FG 33-2a: Qualcomm

    - Add FG 33-3-2: NTT DOCOMO, Qualcomm

  - FG 33-3-3b

    - Add FG 33-2a: Qualcomm

    - Add FG 33-3-3: NTT DOCOMO

  - FG 33-3-5

    - Delete 33-2b: Qualcomm

    - Add 33-2a: Qualcomm

| Company | Comment |
| --- | --- |

- 33-4 to 33-4-1: NACK-only based HARQ-ACK feedback for multicast

In [1], FGs 33-4 to 33-4-1 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling | [33-4] | Yes |  |  | [Per UE] | FFS | FFS |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | Similar to the support of ACK/NACK-only based feedback for multicast as in FG33-2a, the support of NACK-only based feedback and RRC based enabling/disabling NACK-only based feedback as well as PTM retransmission can be merged into a single FG33-4. Since UE does not feedback ACK when the TB is decoded correctly and the PUCCH resources might be shared among UEs for reporting NACK if the TB is failed in decoding, network cannot differentiate which UE is failed in decoding so UE does not need to support PTP retransmission for NACK-only based feedback. Moreover, considering support of shared PUCCH resource configurations with unicast has been the component for this FG, support of separate PUCCH resource configurations from unicast can be separate FG from FG 33-4 if separate PUCCH resources configurations may take extra UE buffer. In addition, regarding supporting more than one NACK-only feedback in the same PUCCH transmission, the two Alternatives are supported taking into UE capability into consideration: Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability With the above analysed, the FG33-4 can be updated as follows: Proposal 8: Updating FG33-4 and adding FG33-4-1a as follows in red: 33. NR_MBS; 33-4; Support of NACK-only based HARQ-ACK feedback and RRC-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; - Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - Support of shared PUCCH resource configurations with unicast - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast. FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4; 33-2; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-4-1a; Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast; - Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast.; 33-4; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. |
| --- | --- | --- |
| [3] | ZTE | In FG 33-2a and FG33-4, we have the following two FFS on whether to have separate FG for separate PUCCH resource configuration. 33. NR_MBS; 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a. 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4. Regardless of shared PUCCH resource configuration or separate PUCCH resource configuration, the total number of PUCCH resource is kept unchanged compared with Rel-15. The only difference is where to pick the PUCCH resource. PUCCH resource configuration itself really doesn’t deserve to have a separate FG from our perspective. Proposal 3: Merge separate PUCCH resource configurations from unicast with FG33-2a and FG33-4. |
| [4] | Spreadtrum Communications | During previous RAN1 meetings, we have the common understanding on reusing the design for ACK/NACK based feedback for NACK-only as far as possibly. For the basic function of ACK/NACK based feedback, i.e., FG33-2a, supporting of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC ignalling is also included. Likewise, for the basic function of NACK-only feedback, we also think the function of NACK-only enabling/disabling by RRC also should be included. Thus, we have the following proposal: Proposal 3: Component 1 of FG33-4 can be revised as ‘Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling’. |
| [5] | vivo | Regarding to supporting more than one NACK-only feedback in the same PUCCH transmission. RAN 1 has agreed two modes: mode 1 is UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. Mode 2 is UE transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback. Therefore, two separate UE capabilities should be added correspondingly. Proposal 4 Add separate UE capabilities for supporting more than one NACK-only feedback in the same PUCCH transmission. Regarding to whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4, from our understanding, separate PUCCH resource configurations from unicast does not require separate UE capability, as the total number of PUCCH resources from all PUCCH-Config/PUCCH-ConfigurationList is not changed which was agreed in RAN1. Proposal 5 Include separate PUCCH resource configurations from unicast in 33-4. 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - [Support NACK-only based HARQ-ACK feedback.] - Support of shared/separate PUCCH resource configurations with unicast; [33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signaling; [33-4]; Yes; [Per UE]; FFS; FFS; Optional with capability signalling. 33. NR_MBS; 33-4-2; ACK/NACK based multiple NACK-only feedback in the same PUCCH; Support of multiplexing more than one NACK-only feedback bits by transforming NACK-only into ACK/NACK HARQ bits; 33-4; Optional with capability signalling. 33. NR_MBS; 33-4-3; Sequence based multiple NACK-only feedback in the same PUCCH; Support of transmitting a specific sequence or a PUCCH transmission corresponding the combination more than one NACK-only HARQ feedback; 33-4; Optional with capability signalling. |
| [6] | Xiaomi | It is still open whether to define separate PUCCH resource configurations from unicast as separate FG from that of supporting shared PUCCH resources with unicast. Supporting separate PUCCH resource configurations for unicast and multicast respectively is an advanced UE capability. The default behavior for a UE should be transmitting UCI on shared PUCCH resources with unicast. Hence we have the following proposal: Proposal 4: Define separate PUCCH resource configurations from unicast as separate FG from FG 33-4. |
| [7] | OPPO | For FG 33-4, similar with ACK/NACK based feedback in FG 33-2a, enabling/disabling of NACK-only based feedback by RRC ignalling can be within the same FG. Supporting NACK-only based HARQ-ACK feedback requires enabling/disabling mechanism, while DCI-based mechanism is not the only one. Furthermore, using RRC-based ignalling to enable/disable NACK-only based HARQ-ACK feedback is considered as a default method. Therefore, the bracket should be removed from the first component, and RRC ignalling configuration on enabling/disabling should also be added. - For FG 33-4, the bracket should be removed. - For FG 33-4, “and support enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling” should be added to the component. For FG 33-4, one legacy issue similar with FG 33-2a is that whether to define separate PUCCH resource configurations from unicast as separated FG. NACK-only based feedback has different characteristic from that of ACK/NACK based feedback. Always sharing PUCCH resource between multicast with NACK-only and unicast with ACK/NACK based feedback mechanisms may have impact or degradation on unicast reliability performance. A third component “Support of separated PUCCH resource configurations with unicast” should be added to FG 33-4. - For FG 33-4, adding another component that “Support of separated PUCCH resource configurations with unicast”. |
| [8] | Apple | Regarding ACK/NACK based feedback and NACK-only based feedback, one open issue is whether introduce a separate FG for separate PUCCH resource configuration. For ACK/NACK HARQ feedback, the separated PUCCH resource configuration can be another FG, the shared PUCCH resource configuration with unicast is the basic function of FG33-2a. If multicast HARQ-ACK bit multiplexing with HARQ bit of unicast, the unicast PUCCH resource will be applied according to the agreement that PRI indicates last DCI for unicast. If there is no separated PUCCH resource configuration for multicast, the HARQ feedback function works well. For NACK-only based HARQ feedback, the shared and separated PUCCH resource configurations can be merged into FG33-4, as one of benefits to introduce the NACK-only feedback is to save the PUCCH resource by sharing the resource among the users, and to reduce the gNB blind detection PUCCH complexity. Proposal 4: For NACK-only based feedback, support both shared and separate PUCCH resource configurations with unicast. In last RAN1 meeting, it was agreed to support more than one bit feedback for NACK-only based feedback. RRC ignalling can configure either Alt1 or Alt 4. Supporting two alternatives shall be independent UE capabilities to provide the UE implementation flexibility. Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability. Proposal 5: Define two separate NACK-only feedback modes. Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported. 33. NR_MBS; 33-4-2; NACK-only HARQ-ACK bits transforming into ACK/NACK HARQ bits; Supporting more than one NACK-only feedback in the same PUCCH transmission by transforming NACK-only HARQ-ACK bits to ACK/NACK HARQ bits.; 33-4; Yes. 33. NR_MBS; 33-4-3; PUCCH resource table for NACK-only feedback; Supporting more than one NACK-only feedback in the same PUCCH transmission by defining HARQ-ACK bit to PUCCH resource mapping table; 33-4. |
| [9] | CMCC | Similar discussion in FG 33-2a, for NACK-only based HARQ-ACK feedback, we also prefer both shared and separate PUCCH resource configuration as the components in FG 33-4. Proposal 3. Separate PUCCH resource configurations from unicast is defined as a component in FG 33-4. |
| [10] | NTT DOCOMO | PUCCH resources for unicast HARQ-ACK feedback are orthogonal among Ues. On the other hand, NACK-only based feedback allows PUCCH resources to be shared among Ues. It is not appropriate to use unicast PUCCH resources for NACK-only based feedback because the allocation policies are opposite. Support for separate PUCCH resource configurations should be a basic feature of NACK-only based HARQ-ACK feedback. Proposal 13: Support of separate PUCCH configurations from unicast is included in FG 33-4. The type of FG 33-4-1 should be per UE as well as DCI-based enabling/enabling ACK/NACK-based feedback. Proposal 14: The type of FG 33-4-1 is per UE. |
| [11] | Nokia, NSB | - 33-4: - To be merged in 33-2, or at least to be mandatorily indicated for UEs supporting 33-2, as it is an essencial functionality for groupcast. - Per UE |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs of FG33-4 and FG33-4-1. In addition, we suggest adding FG33-4a for supporting more than one NACK-only in a PUCCH resource based on the RAN1 agreement. Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast; - Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling - Support of PTM retransmission for multicast - Support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only - Support of shared PUCCH resource configurations with unicast - Support one NACK-only based HARQ-ACK feedback in a PUCCH resource define separate PUCCH resource configurations from unicast as separate FG (FG33-8-3) from FG 33-4; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-4a; More than one NACK-only based HARQ-ACK feedback for multicast in a PUCCH resource; Support of more than one NACK-only based HARQ-ACK feedback in a PUCCH resource; 33-4; Yes; Per FSPC; N/A; N/A; Note: up to 4 TBs with NACK-only feedback in a PUCCH resource; Optional with capability signalling. 33. NR_MBS; 33-4-1; DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC ignalling via DCI format 4_2; 33-4, 33-2b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[GTW1] High priority proposal 6-1:

- Support of separate PUCCH resource configurations from unicast is added as a component of FG 33-4

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Separate FG: Huawei, HiSilicon, Xiaomi, Qualcomm - separate PUCCH resources configurations may take extra UE buffer - separate PUCCH resource configurations for unicast and multicast respectively is an advanced UE capability - Include in FG 33-4: ZTE, vivo, OPPO, Apple, CMCC, NTT DOCOMO - the total number of PUCCH resource is kept unchanged compared with Rel-15 - Always sharing PUCCH resource between multicast with NACK-only and unicast with ACK/NACK based feedback mechanisms may have impact or degradation on unicast reliability performance - one of benefits to introduce the NACK-only feedback is to save the PUCCH resource by sharing the resource among the users, and to reduce the gNB blind detection PUCCH complexity - If a separate FG on PUCCH resource configuration is defined, some Ues may not support such FG while other Ues in the same multicast group can support it, the only result is that all Ues in the group have to share PUCCH resources with unicast, which may lead to increase UL feedback burden and complexity, and even impact on unicast UL feedback performance - It is not appropriate to use unicast PUCCH resources for NACK-only based feedback because the allocation policies are opposite |
| Moderator | This issue will be discussed together with proposal 3-1 |
| Moderator | Following was agreed in the GTW on May 11. Agreement - Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a; Yes; [Per band or per FSPC]; [N/A]; [N/A]; Optional with capability signalling. - FG 33-4 includes Alt1 as a component, Add FG 33-4a including Alt4 as component with separate PUCCH resource configurations [FL2] High priority proposal 6-1a: - Update FGs 33-4 and FG 33-4a as follows 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming; 1. [Support NACK-only based HARQ-ACK feedback.] for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) Multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4; [33-2]; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-4a; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission; 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of shared PUCCH resource configurations with unicast 3. Support of separate PUCCH resource configurations from unicast; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. |
| Huawei, HiSilicon | Ok to 6-1a. |
| ZTE | OK with 6-1a. |
| Spreadtrum | Generally we are fine. But we are wondering about the Prerequisite FG for the two FGs. From our side, we think the two FGs are parallel, FG33-2 can be as the Prerequisite FG for both. |
| OPPO | OK. |
| Qualcomm | For 33-4, we think NACK-only transforming to ACK/NCK can be applied to single or multiple TBs. Since UE support NACK transformed to ACK/NACK, UE is required to support ACK/NACK for multicast. If the prerequisite of FG33-4 is FG33-2a, which already includes shared PUCCH resource configurations with unicast, PTM retransmission, and codebook Type, we can delete “shared PUCCH resource configurations with unicast” in 33-4. For 33-4a, we suggest its prerequisite is FG33-4 and delete single TB case and shared PUCCH resource. The reason is if UE only support 33-4a but not 33-4, UE cannot deal with NACK-only colliding with other PUCCH/PUSCH, and Alt4 only cannot support the case of more than 4 TBs. Therefore, we suggest 33-2a; Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast; 33-2. 33-4; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming; 1. [Support NACK-only based HARQ-ACK feedback.] for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4; [33-2]33-2a. 33-4a; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission; 1. Support multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 3. Support of separate PUCCH resource configurations from unicast; 33-4. |
|  | Prerequisite FGs can be discussed later. Once it is decided, related components will be updated accordingly [GTW3] High priority proposal 6-1a: - Update FGs 33-4 and FG 33-4a as follows 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming; 1. [Support NACK-only based HARQ-ACK feedback.] for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) Multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4; [33-2]FFS; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-4a; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission; 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of shared PUCCH resource configurations with unicast 3. Support of separate PUCCH resource configurations from unicast; FFS; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. |
|  | Following was agreed in the GTW on May 12. Further discuss the reporting type in question 6-4 Agreement - Update FGs 33-4 and FG 33-4a as follows 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming; 1. [Support NACK-only based HARQ-ACK feedback.] for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits [2. Support of shared PUCCH resource configurations with unicast] FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4; 33-2a; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-4a; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission; 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH a) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of shared PUCCH resource configurations with unicast 2. Support of separate PUCCH resource configurations from unicast; 33-4; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. |
| Qualcomm | Regarding “[2. Support of shared PUCCH resource configurations with unicast]”, can we confirm to delete it in 33-4? |
| Moderator | Comeback the component 2 next meeting. |

[GTW1] High priority proposal 6-2:

- Introduce new FG for support of more than one NACK-only based HARQ-ACK feedback for dynamically scheduled multicast in a PUCCH resource

  - FFS whether to separate FGs for Alt1 and Alt4 in the RAN1 agreement

  - Support: vivo, Apple, Qualcomm

| Company | Comment |
| --- | --- |
| LG Electronics | Support |
| Moderator | This issue will be discussed together with proposal 3-1 |

[FL1] High priority question 6-3:

- Companies are encouraged to provide views on whether to merge FG 33-4 with FG 33-2.

  - Support: Nokia, NSB

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Based on the earlier discussion, I am impressed we sort of concluded already it should be separate. |
| MediaTek | Not support. We agree with Huawei’s view. As discussed in previous meeting, the reliability concern has been addressed by PTM retransmission in FG 33-2 and HARQ-ACK can be split as separate FG. Besides, considering the NACK only feedback is new UE behaviour, it is preferred to define as a separate FG for facilitating fast MBS commercial deployment. |
| Spreadtrum | In our understanding, we already have common understanding that HARQ-ACK feedback operation is optional UE capability. The discussion on this issue is not needed. |
| vivo | Not support, and that’s why we also don’t support including multiple NACK-only feedbacks in 33-4, it will require UE to support FG33-2 as a prerequisite of FG33-4 |
| OPPO | Not support to merge. |
| FL2 | As commented by companies, this issue has been discussed in previous meeting. No further discussion is necessary unless strong motivation is provided from proponent. |
| Nokia, NSB | OK not to pursue this further. |

High priority question 6-4:

- Companies are encouraged to provide views on the reporting type of FGs 33-4 and 33-4-1.

  - FG 33-4

    - Per UE: Huawei, HiSilicon, vivo, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-4-1

    - Per UE: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update Also, companies are encouraged to provide view on the reporting type of new FG 33-4a |
| Huawei, HiSilicon | Ok per FS. |
| MediaTek | Per FSPC |
| Spreadtrum | Ok as per FS. Per UE also fine to us. |
| Nokia, NSB | Per UE, as 33-2 is already per FS. No need to add overhead unnecessarily. |
|  | - FG 33-4 - Per UE: vivo, Nokia, NSB, SPRD - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-4ª - Per UE: SPRD, Nokia - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: - FG 33-4-1 - Per UE: NTT DOCOMO, SPRD, Nokia - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Apple | Per FS |
| FL5 | Since the reporting type of FGs for ACK/NACK-based HARQ-ACK feedback was agreed, similar proposal is made High priority proposal 6-4: - Reporting type of FGs 33-4 is per BC - Reporting type of FGs 33-4a is per BC - Reporting type of FGs 33-4-1 is per band |
| Huawei, HiSilicon | Seems ok |
| GTW6 | High priority proposal 6-4: - Reporting type of FGs 33-4 is per BC - Reporting type of FGs 33-4a is per BC - Reporting type of FGs 33-4-1 is per band |
| FL6 | No objection has been received in the last round. This proposal is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - Reporting type of FGs 33-4 is per BC - Reporting type of FGs 33-4a is per BC - Reporting type of FGs 33-4-1 is per band |

Low priority proposal 6-5:

- Components of FG 33-4 are revised as

  - Add a component for support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling.

  - Add a component for support of PTM retransmission for dynamic scheduling for multicast.

  - Add a component for support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only.

| Company | Comment |
| --- | --- |

Low priority question 6-6:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 33-4 and 33-4-1.

  - FG 33-4-1

    - Add 33-2b: Qualcomm

| Company | Comment |
| --- | --- |

Low priority question 6-7:

- Companies are encouraged to provide views on whether/how to revise any other contents in FGs 33-4 and 33-4-1 which do not have capability signalling impacts

| Company | Comment |
| --- | --- |

- 33-5-1 to 33-5-2: SPS group-common PDSCH for multicast

In [1], FGs 33-5-1 to 33-5-2 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between Ues (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1f | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | FFS max number | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | FFS the value range for K FFS: Candidate values for M | Optional with capability signalling |

Following feedbacks are provided in contributions for RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | FG33-5-1h should be revised as the maximum number of G-CS-RNTIs for SPS multicast per CC, because such number is from the configuration-wise instead of scheduling-wise. Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling can be a separate FG in FG33-5-1 and can be merged into FG33-2f. Support of PTM retransmission can be grouped into FG33-5-1a. Support of PTP retransmission can be separate in FG33-5-1d but is proposed to be separate from FG33-2d. Support of FG33-5-2 can be reported per UE. Proposal 9: Updating FG33-5-1/1a/1d/2 as follows in red: 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling; 33-2; Yes; [Per UE]; [No]; [No]; Optional with capability signalling. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH - Support of PTM retransmission associated with G-CS-RNTI for SPS multicast; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-5-1d; PTP retransmission for SPS group-common PDSCH for multicast; Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a; 33-5-1a; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration; - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per UE; No; No; FFS the value range for K FFS: Candidate values for M; Optional with capability signalling. Proposal 3: Consider the following two alternatives to modify the FG33-2e/FG33-5-1h: - Alt1: Updating FG33-2e/FG33-5-1h and adding FG33-10 as follows in red: 33. NR_MBS; 33-2e; Multiple G-RNTIs for dynamic group-common PDSCHs; The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. FFS details.; 33-2; Yes; Per UE; No; No; Candidate values for X is: {2, 3, 4}; Optional with capability signalling. 33. NR_MBS; 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; The maximum number of G-CS-RNTIs for SPS multicast per CC; 33-5-1; Yes; Per UE; No; No; Candidate values for X is: {2, 3, 4}; Optional with capability signalling. 33. NR_MBS; 33-10; Total number of RNTIs; The maximum number of all RNTIs UE can support per CC.; 33-2e and/or 33-5-1h; Yes; Per UE; No; No; Candidate values for X is: {16, 32}; Optional with capability signalling. - Alt2: Updating FG33-2e/FG33-5-1h as follows in red: 33. NR_MBS; 33-2e; Multiple G-RNTIs for dynamic group-common PDSCHs; - The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. - Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.; 33-2; Yes; Per UE; No; No; For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted}; Optional with capability signalling. 33. NR_MBS; 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; - The maximum number of G-CS-RNTIs for SPS multicast per CC. - Indication of whether the maximum number of G-CS-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.; 33-5-1; Yes; Per UE; No; No; For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted}; Optional with capability signalling. |
| --- | --- | --- |
| [3] | ZTE | One remaining issue of FG33-5-1 is whether to have a separate FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. 33. NR_MBS; 33-2f; Dynamic multicast with DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast. Since we already have a FG 33-2f to indicate dynamic multicast with DCI format 4_2, if UE indicates support of FG 33-2f and FG 33-5-a, then it means UE can support DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. Proposal 4: No need to have a separate FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. If UE indicates support of FG 33-2f and FG 33-5-a, then it means UE can support DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. Another left issue is whether to introduce the following FG 33-5-1d. 33. NR_MBS; 33-5-1d; PTP retransmission for SPS group-common PDSCH for multicast; Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a; 33-5-1a; Yes; Per UE; No; No; Optional with capability signalling. Since we already have FG 33-2d (PTP retransmission for multicast), from our perspective, FG 33-5-1d can be merged with FG 33-2 because the complexity is PTP retransmission itself instead of retransmission for DG PDSCH or SPS. Proposal 5: Merge FG 33-5-1d with FG 33-2d. Another left issue is whether to introduce the following FG 33-5-1a. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH; 33-5-1; Yes; Per UE; No; No; Optional with capability signalling. Since Rel-15, the FG of HARQ-ACK feedback includes both DG PDSCH and SPS. We can follow the same rule, i.e., merging FG 33-5-1a with FG 33-2a. Proposal 6: Merge FG 33-5-1a with FG 33-2a. |
| [6] | Xiaomi | Currently, the component of FG 33-5-1 is defined as below: - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH One open issue is whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. In Rel-16, whether the newly introduced DCI format 1_2 can be used for SPS release is defined as a separate UE capability. We prefer to follow the same logic as Rel-16 NR IioT here. Proposal 5: For FG 33-5-1, the support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling is a separate UE capability. The same logic as dynamic scheduling should be adopted here, i.e. PTP retransmission can be a separate FG compared to the basic retransmission capability supporting PTM retransmission. Proposal 6: FG 33-5-1d should be kept as a separate FG. |
| [7] | OPPO | For FG 33-5-1, one legacy issue is whether to separate the capability for support of DCI format 4_2 for multicast SPS scheduling. At least DCI format 4_1 can be used for multicast SPS scheduling, therefore, DCI format 4_2 can be a separated FG from FG 33-5-1. - For FG 33-5-1, separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. For FG 33-5-1d, support of PTP retransmission for SPS multicast can be kept in a separate FG, while PTM can be used as the default retransmission scheme in multicast SPS. - For FG 33-5-1d, “Support of PTP retransmission for SPS multicast” can be a separate FG and not merged with FG 33-5-1a. |
| [9] | CMCC | The first remaining issue is whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling from FG 33-5-1. Considering DCI format 4_1 and DCI format 4_2 for dynamic scheduling are two separate FGs, the separation of DCI format 4_1 and DCI format 4_2 with G-CS-RNTI can also be supported. Proposal 4. Separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling from FG 33-5-1. Another remaining issue is the merge of PTP retransmission in ACK/NACK based feedback for SPS multicast GC-PDSCH in FG 33-5-1a, since the PTP retransmission is a separate FG in multicast dynamic scheduling, the same rule can be applied for SPS multicast as well. Proposal 5. Don’t need to merge FG 33-5-1d with FG 33-5-1a. |
| [10] | NTT DOCOMO | Since the support of DCI format 4_2 for dynamic scheduling is in a separate FG, it would be natural to make the support of DCI format 4_2 for SPS in a separate FG. Since the type of FG for support of unicast SPS is per UE, the type of FG 33-5-1 should also be per UE. Proposal 15: Separate support of DCI format 4_2 with CRC scrambled with G-CS-RNTI from FG 33-5-1. Proposal 16: The type of FG 33-5-1 is per UE. The type of FG 33-5-1b and 33-5-1g should be per UE as well as DCI-based enabling/enabling ACK/NACK-based feedback for dynamic scheduling. Proposal 17: The type of FG 33-5-1b and 33-5-1g are per UE. Since the FG for PTP retransmission for dynamic scheduling is separate from the FG for ACK/NACK based feedback, it would be better to have a separate FG for SPS as well. Proposal 18: Not merge FG 33-5-1d with FG 33-5-1a. Since the type of FG for dynamic slot-level repetition for unicast is per band, the type of dynamic slot-level repetition for multicast should also be per band. Proposal 19: The type of FG 33-5-1e is per band. Since processing time constraints vary by SCS, the type of FG 33-5-1h would be better to be per band. Proposal 20: The type of FG 33-5-1h is per band. Since the type of the FG for multiple SPS configuration for unicast is per band, the type of FG 33-5-2 should also be per band. Proposal 21: The type of FG 33-5-2 is per band. |
| [11] | Nokia, NSB | - 33-5-1: - Per UE - 33-5-1b/e/f/g/h: - Per UE - 33-5-2: - Per UE |
| [12] | MediaTek | Regarding the multiple G-CS-RNTIs for SPS group-common PDSCHs, it is similar with FG 33-2e (i.g., multiple G-RNTIs for group-common PDSCHs). Based on the same reason, we suggest that up to 2 G-CS-RNTIs are supported for SPS scheduling if FG 33-5-1 is supported. Proposal 16: For FG 33-2e, adding a note that “the candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2}”. Another remaining issue is that whether the multiple G-CS-RNTI is per slot per CC, actually, RAN2 has agreed that the reporting type is per UE in the approved TS 38.306 [5] as copied following. RAN2’s agreements: maxNumberRNTIs-MBS-r17 Indicates the maximum number of simultaneous reception of PDCCH scrambled with G-RNTIs/G-CS-RNTIs for MBS multicast.; UE; No; No; No.. Thus, the wording of “per slot per CC” in the component should be deleted, and the reporting type should be updated based on the RAN2’s approved CR, e.g., per UE. Proposal 17: For FG 33-2e, the wording of “per slot per CC” in the component should be deleted, and the reporting type should be updated based on the RAN2’s approved CR, e.g., per UE. 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; Max number of G-CS-RNTIs for SPS multicast [per slot per CC]; 33-5-1; Yes; Per UE; [No]; [No]; Note 1: The candidate value for SPS multicast G-CS-RNTI is: {0, 1, 2}; Optional with capability signalling. |
| [13] | Ericsson | We do not think the clarification “per CC” is needed. The multicast main feature group 33-2 is already define as per FSPC. Similarly, we do not see the use of “per slot”. A single multicast PDSCH per slot is supported by rel17, with the possibility to also have TDM or FDM of unicast in the slot if supported. Remove “per slot per CC” from the description of FG 33-5-1h |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs of FG33-5-1 and FG33-5-1x as 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast; - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH - Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast - Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast - ACK/NABK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH 2) Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3) Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4) Support of shared SPS-PUCCH-AN-List with unicast; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1b; Dynamic multicast with DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1c; DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC ignalling via DCI format 4_2; 33-5-1a, 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1d; PTP retransmission for SPS group-common PDSCH for multicast; Support of PTP retransmission associated with CS-RNTI for SPS multicast; 33-5-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1e; Dynamic Slot-level repetition for SPS group-common PDSCH for multicast; Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Yes; Per FSPC; N/A; N/A; Candidate values for X is: {8, 16}; Optional with capability signalling. 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast 3) Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4) Support of shared SPS-PUCCH-AN-List with unicast; 33-5-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1g; DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC ignalling via DCI format 4_2; 33-5-1f, 33-5-1b; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1h; Multiple G-CS-RNTIs for SPS group-common PDSCHs; Max number of G-CS-RNTIs for SPS multicast; 33-5-1; Yes; Per FSPC; N/A; N/A; Candidate values: {2, 3,…8}; Optional with capability signalling. 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration; - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast; 33-2; Yes; Per FSPC; N/A; N/A; Candidate values of K and M: {2, 3, .. 8}; Optional with capability signalling. 33. NR_MBS; 33-8-3; PUCCH resource configuration for multicast feedback for SPS GC-PDSCH; - Support of a SPS-PUCCH-AN-List for multicast HARQ-ACK feedback of all multicast SPS configuration(s), separate from that of SPS unicast configurations; 33-5-1a or 33-5-1f; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

[GTW1] High priority proposal 7-1:

- Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Separate FG: Xiaomi, OPPO, CMCC, NTT DOCOMO, Qualcomm - Merge with FG 33-2f: Huawei, HiSilicon, ZTE |
| LG Electronics | We prefer separate FG. |
| ZTE | We support to merge it with FG 33-2f. If UE can already support DCI format 4_2 for G-RNTI, we are not sure about the additional difficulty to support DCI format 4_2 for G-CS-RNTI. |
| OPPO | We support separate FG. |
| FL2 | Summary of companies view - Separate FG: Xiaomi, OPPO, CMCC, NTT DOCOMO, Qualcomm, LGE - Merge with FG 33-2f: Huawei, HiSilicon, ZTE The same proposal is set for GTW [GTW2] High priority proposal 7-1: - Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling |
| Huawei, HiSilicon | Our view was not correctly summarized. We support DCI format 4_2 should be separate FG from the FG for SPS, i.e., FG33-5-1 but can be merged to the one for dynamic with DCI 4_2 scheduling, i.e., FG33-2f and with 33-2 or 33-5-1 as prerequisite FG. |
| ZTE | We share the same view as Huawei. |
| MediaTek | Support the proposal |
| Spreadtrum | Fine with proposal, also fine with Huawei’s suggestion. |
| OPPO | OK with this proposal 7-1. |
| Qualcomm | Support |
|  | Further discuss which alternative should be adopted [GTW3] High priority proposal 7-1: - Alt1: Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling - Alt2: The capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling is merged with FG 33-2f - The prerequisite FGs for FG 33-2f are “33-2 or 33-5-1” |
| FL3 FL4 | This proposal could not be discussed in the GTW on May 12. Companies are encouraged to provide view which alternative should be adopted with the reason. |
| Huawei, HiSilicon | We are ok with either. |
| MediaTek | Alt 1 is preferred since dynamic scheduling and SPS are two separate FGs |
| LG Electronics | We slightly prefer Alt1. |
| ZTE | Alt2. If UE already supports DCI format 4_2 and supports SPS, what’s the additional complexity to support DCI format 4_2 indicating activation/activation for MBS SPS? |
| Spreadtrum | Any one is fine to us. |
| Vivo | OK is either one |
| GTW5 | Given Alt1 has slightly more support, following proposal is made High priority proposal 7-1: - Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | ok |
| ZTE | Now we have one FG for “33-2f Dynamic multicast with DCI format 4_2” and another separate FG for “33-5-1, SPS group-common PDSCH for multicast”. If UE indicates support of FG 33-5-1 and not for FG 33-2f, then UE can only use DCI format 4_1 with G-CS-RNTI for SPS scheduling. If UE indicates support of FG 33-5-1 and FG 33-2f, then UE can use DCI format 4_1 or 4_2 with G-CS-RNTI for SPS scheduling. The above two FGs already provide much flexibility for UE implementation. We are still not convinced about the scenario that UE supports DCI format 4_2 for DG PDSCH but not for SPS. |
| GTW6 | Need further discussion in the GTW High priority proposal 7-1: - Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling |
| FL6 | ZTE still prefers to merge with FG 33-2f. Companies who prefer to add the FG are encouraged to provide view why separate FG from FG 33-2f is necessary to address the concern from ZTE. |
| Qualcomm | With separate FGs, the UE can support DCI format 4_1/4_2 for dynamic grant multicast but only support DCI format 4_1 for SPS multicast. We think whether to UE needs to support DCI format 4_2 for SPS can be different from that of DG, due to independent FG of 33-5-x and 33-2x. |
| Huawei, HiSilicon | Again, we have no strong view for this particular issue but we want progress. |
| GTW7 | Further discuss in the GTW High priority proposal 7-1: - Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling |
|  | Following was agreed in the GTW on May 19. Agreement - Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling - FFS whether to include retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI |

[FL1] High priority question 7-2:

- Companies are encouraged to provide view whether to merge FG 33-5-1a with FG 33-2a

  - Support: ZTE

    - Since Rel-15, the FG of HARQ-ACK feedback includes both DG PDSCH and SPS

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We prefer to have separate FGs. |
| MediaTek | Not support. Considering the dynamic scheduling and SPS are two separate FGs, it is better to define the separate HARQ-ACK FGs |
| vivo | Separate FGs is preferred |
| OPPO | We prefer to have it as a separate FG. |
| FL2 | Given most companies prefer separate FG, following proposal is set [FL2] High priority proposal 7-2: - FG 33-5-1a is confirmed as follows 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH; 33-5-1; Yes; FFS; FFS; FFS; Optional with capability signalling. |
| Huawei, HiSilicon | ok |
| ZTE | This is not our preference. But if we are the only company has concern on it, we can compromise to live with it for progress. |
| MediaTek | Support the proposal |
| Spreadtrum | Support |
| OPPO | Support. |
| Qualcomm | Support |
|  | All companies are fine with the proposal [GTW3] High priority proposal 7-2: - FG 33-5-1a is confirmed as follows 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH; 33-5-1; Yes; FFS; FFS; FFS; Optional with capability signalling. |
|  | Following was agreed in the GTW on May 12. Agreement - FG 33-5-1a is confirmed as follows 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH; 33-5-1; Yes; FFS; FFS; FFS; Optional with capability signalling. |

[GTW1] High priority question 7-3:

- Confirm FG 33-5-1d as a separate FG from FG 33-5-1a

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Merge with FG 33-5-1a: - Merge with FG 33-2d: ZTE - the complexity is PTP retransmission itself instead of retransmission for DG PDSCH or SPS - Not merge: Huawei, HiSilicon, Xiaomi, OPPO, CMCC, NTT DOCOMO, Qualcomm - same logic as dynamic scheduling |
|  | [GTW3] High priority question 7-3: - Confirm FG 33-5-1d as a separate FG from FG 33-5-1a |
|  | Following was agreed in the GTW on May 12. Agreement - Confirm FG 33-5-1d as a separate FG from FG 33-5-1a |

High priority question 7-4:

- Companies are invited to provide view whether to add a component in FG 33-5-1h for Indication of whether the maximum number of G-CS-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation.

  - Support: Huawei, HiSilicon

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after some progress is made in question 3-3 |
| FL4 | Since RAN1 agreed to ask RAN2 to decide the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e, similar proposal is made High priority proposal 7-4: - There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget. |
| MediaTek | In last RAN1 meeting, RAN1 received an LS from RAN2 with the following question about the maximal number of G-CS-RNTI for multicast (R2-2202016): Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?. RAN1 replied the question with the following agreements (R1-2202591): Agreement In the reply LS on MBS SPS to RAN2, capture the following for Q1: - RAN1 confirms that RAN2’s understanding is correct. - RAN1 thinks that the maximum number of G-CS-RNTI configured for UE should be subject to UE capability.. Thus, I am not sure whether it is a good direction for the discussion since RAN2 want RAN1 can make decision. Considering the note, maybe we can define a smaller value (e.g., 2 or 3) and make final decision in RAN1. |
| Nokia, NSB | Support the proposal. Regarding Mediatek’s concern, the proposal gives extra information to RAN2 about RAN1’s understanding on the maximum number. |
| Spreadtrum | Support FL’s proposal |
| GTW5 | Further discuss in the GTW High priority proposal 7-4: - There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget. |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | ok |
| GTW6 | High priority proposal 7-4: - There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget. |
| FL6 | No objection has been received in the last round. This proposal is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget. |

[FL1] High priority question 7-5:

- Companies are invited to provide view on the candidate values for the max number of G-CS-RNTIs for SPS multicast per CC in FG 33-5-1h

  - 2: MediaTek

  - {2, 3, 4}: Huawei, HiSilicon

  - {2, 3, …, 8}: Qualcomm

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | {2, 3, 4} seems sufficient |
| MediaTek | Share the similar view with question 3-2, and up to 2 G-CS-RNTIs are sufficient. |
| FL2 | This issue can be discussed after some progress is made in question 3-2 |

[FL1] High priority question 7-6:

- Companies are encouraged to provide views on the value range for N/M/K for FG 33-5-2

  - {2, 3, …, 8}: Qualcomm

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | We wonder whether such candidate value is for each of N, M, K? such value should be bounded somehow. |
| FL2 | More input is necessary for decision |
| FL3 FL4 | Companies are encouraged to provide the view |
| Apple | The parameter K maybe not needed, as we agreed ‘UE is not expected to be configured simultaneously with more than one component carrier for multicast reception’ Following the design principle of FG12-2 Multiple SPS configurations, N=8, value range of M could be {1,2,…,8}. In addition, the third component can be introduced. if MBS UE is configured with multiple SPS configuration for unicast, the total number of SPS configuration for both multicast and unicast should be no larger than candidate value. - Support [N>1] 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N 8 SPS group-common PDSCH configurations per CFR for multicast - The total number of SPS configurations for both multicast and unicast is no larger than N, and activated SPS group-common PDSCH configurations is no larger than M. Agreement If UE supports carrier aggregation for unicast, multicast reception on an activated SCell with self-scheduling is supported subject to UE capability in Rel-17. - UE is not expected to be configured simultaneously with more than one component carrier for multicast reception. - Cross-carrier scheduling for multicast reception is not supported in Rel-17. - The capability of supporting MBS multicast on SCell is a separate capability from the CA capability for unicast. - The granularity of UE reporting the capability of supporting MBS multicast reception is per FSPC |
| GTW5 | Further discuss based on the proposal from Apple. High priority proposal 7-6: - Update the components of FG 33-5-2 as follows: - 1. Support [N>1] up to 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - 2. Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast - 3. The total number of SPS configurations for both multicast and unicast is no larger than 8, and activated SPS group-common PDSCH configurations is no larger than M. - Candidate value set for M is {1, 2, …, 8} |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | For my understanding, when we say “up to 8” in the 1st bullet, does it actually mean it is still up to UE to report with candidate value, 1,2….8? |
| ZTE | Support the FL proposal. |
| GTW6 | To HW: The wording is similar to unicast one, i.e. FG12-2. My interpretation is the UE is required to support 2, 3, …, and 8 SPS group-common PDSCH configuration per CFR for multicast. If only “8” is there, the UE is not required to support 2, 3, …, 7 SPS group-common PDSCH configuration per CFR for multicast. High priority proposal 7-6: - Update the components of FG 33-5-2 as follows: - 1. Support [N>1] up to 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - 2. Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast - 3. The total number of SPS configurations for both multicast and unicast is no larger than 8, and activated SPS group-common PDSCH configurations is no larger than M. - Candidate value set for M is {1, 2, …, 8} |
| FL6 | Companies are encouraged to check whether you can live with the proposal. |
| Huawei, HiSilicon | Thanks Shinya for explanation. I missed “configuration” from the bullet 1. Now I understand it and has the same interpretation as you. With this said, we are ok with it. |
| GTW7 | Final check in the GTW High priority proposal 7-6: - Update the components of FG 33-5-2 as follows: - 1. Support [N>1] up to 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - 2. Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast - 3. The total number of SPS configurations for both multicast and unicast is no larger than 8, and activated SPS group-common PDSCH configurations is no larger than M. - Candidate value set for M is {1, 2, …, 8} |
|  | Following was agreed in the GTW on May 19. Agreement - Update the components of FG 33-5-2 as follows: - 1. Support [N>1] up to 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - 2. Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast - 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 [per cell], and activated SPS group-common PDSCH configurations is no larger than M. - Candidate value set for M is {1, 2, …, 8} |

High priority question 7-7:

- Companies are encouraged to provide views on the reporting type of FGs 33-5-1 to 33-5-2.

  - FG 33-5-1

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1a

    - Per UE: Huawei, HiSilicon, ZTE

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1b

    - Per UE: NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1d

    - Per UE: Huawei, HiSilicon, ZTE

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1e

    - Per UE: Nokia, NSB

    - Per band: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1f

    - Per UE: Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1g

    - Per UE: NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-5-1h

    - Per UE: Nokia, NSB, MediaTek

    - Per band: NTT DOCOMO

    - Per FSPC: Qualcomm

  - FG 33-5-2

    - Per UE: Huawei, HiSilicon, Nokia, NSB

    - Per band: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update |
| Huawei, HiSilicon | Per UE should be sufficient. |
| MediaTek | Align with the dynamic scheduling reporting type. |
| Spreadtrum | FG 33-5-1h: per FS. The number of RNTIs would impact UE’s implementation, thus we prefer per FS as FG 33-2. FG 33-5-2: per band. In unicast, supporting multiple SPS configurations is per band. Thus, per band can be considered for multicast SPS. Other FGs can be per UE. |
| Nokia, NSB | As mentioned before, given that FG 33-2 this can be per UE. No need to repeat the signalling unnecessarily. |
|  | - FG 33-5-1 - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-5-1a - Per UE: Huawei, HiSilicon, ZTE, SPRD, Nokia, NSB, - Per FSPC: MediaTek, Qualcomm - FG 33-5-1b - Per UE: NTT DOCOMO, Nokia, NSB, Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-5-1d - Per UE: Huawei, HiSilicon, ZTE, SPRD, Nokia, NSB, - Per FSPC: MediaTek, Qualcomm - FG 33-5-1e - Per UE: Nokia, NSB, Huawei, HiSilicon, SPRD - Per band: NTT DOCOMO - Per FSPC: MediaTek, Qualcomm - FG 33-5-1f - Per UE: Nokia, NSB, Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-5-1g - Per UE: NTT DOCOMO, Nokia, NSB, Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-5-1h - Per UE: Nokia, NSB, MediaTek, Huawei, HiSilicon, - Per FS: SPRD - Per band: NTT DOCOMO - Per FSPC: Qualcomm - FG 33-5-2 - Per UE: Huawei, HiSilicon, Nokia, NSB - Per band: NTT DOCOMO, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Apple | Per FS for all. |
| FL5 | Since the reporting type of FGs for dynamic multicast was agreed, similar proposal is made. High priority proposal 7-7: - Reporting type of FGs 33-5-1 is per FS - Reporting type of FGs 33-5-1a is per BC - Reporting type of FGs 33-5-1b is per band - Reporting type of FGs 33-5-1d is per BC - FFS: Revise the component: Support of PTP retransmission for SPS multicast on the cell same as multicast initial transmission - Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 |
| Huawei, HiSilicon | Thanks to FL. I guess it is aligned with all corresponding FGs for dynamic scheduling. We are fine. |
| GTW6 | High priority proposal 7-7: - Reporting type of FGs 33-5-1 is per FS - Reporting type of FGs 33-5-1a is per BC - Reporting type of FGs 33-5-1b is per band - Reporting type of FGs 33-5-1d is per BC - FFS: Revise the component: Support of PTP retransmission for SPS multicast on the cell same as multicast initial transmission - Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 |
| FL6 | No objection has been received in the last round. This proposal is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - Reporting type of FGs 33-5-1 is per FS - Reporting type of FGs 33-5-1a is per BC - Reporting type of FGs 33-5-1b is per band - Reporting type of FGs 33-5-1d is per BC - FFS: Revise the component: Support of PTP retransmission for SPS multicast on the cell same as multicast initial transmission - Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 |
| Apple | Checked spec 38.331, SPS configuration number is configured per BWP, the component 3 of FG33-5-2 can be updated accordingly. maxNrofSPS-Config-r16 INTEGER ::= 8 -- Maximum number of SPS configurations per BWP – SPS-ConfigIndex The IE SPS-ConfigIndex is used to indicate the index of one of multiple DL SPS configurations in one BWP. SPS-ConfigIndex information element SPS-ConfigIndex-r16 ::= INTEGER (0.. maxNrofSPS-Config-r16-1) |

Low priority proposal 7-8:

- Components of FG 33-5-1 are revised as

  - Add a component for support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast.

  - Add a component for support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast.

  - Add a component for ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI

- Components of FG 33-5-1a are revised as

  - Add a component for support of PTM retransmission associated with G-CS-RNTI for SPS multicast

  - Add a component for support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only

  - Add a component for support of shared SPS-PUCCH-AN-List with unicast

- Components of FG 33-5-1h are revised as

  - Component 1: Max number of G-CS-RNTIs for SPS multicast [per slot per CC]

| Company | Comment |
| --- | --- |

Low priority question 7-9:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-5-1 to 33-5-2.

| Company | Comment |
| --- | --- |

- 33-6-1 to 33-6-3: HARQ-ACK with different priorities for multicast

In [1], FGs 33-6-1 to 33-6-3 are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority configured for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [2] | Huawei, HiSilicon | For dynamic scheduling, the priority indication can be included in DCI format 4_2 for multicast. The priority index cannot be directly configured by RRC signalling, although it means low priority when the priority indication is not included in the DCI format. Therefore, separating UE capabilities of supporting priority configuration from supporting priority indication included in DCI format does not seem necessary. For multicast SPS scheduling, the priority index is configured together with the configuration for SPS. There could be an UE capability of support of priority configuration for multicast SPS but it is not needed to have a capability defined for support of priority indicator field in the DCI format just for multicast SPS scheduling. Proposal 10: Updating FG33-6-1 and adding FG33-6-1a as follows in red: 33. NR_MBS; 33-6-1; DL priority indication in DCI for multicast dynamic scheduling; - Support of priority indicator field configured in DCI format 4_2 with CRC scrambled with G-RNTI for multicast dynamic scheduling. - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE; 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-6-1a; DL priority configuration for SPS multicast; Support of priority configured for multicast HARQ-ACK feedback of SPS multicast; 33-6-1; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |
| [4] | Spreadtrum Communications | In latest 38.212 spec [2], DCI format for broadcast has been captured as DCI format 4_0, and DCI format for multicast has been captured as DCI format 4_1 and DCI format 4_2. In order to align with the current spec, we have the following proposal: Proposal 1: Revise DCI format to align with 38.212, - In component 1 of FG 33-6-1, DCI format 1_1 is adjusted as DCI format 4_2; |
| [10] | NTT DOCOMO | Since the type of FG for support of the priority indicator field for unicast is per UE, the type of FG 33-6-1 should also be per UE. Component 2 os FG 33-6-1 can be removed since support of construction of two HARQ-ACK codebooks is included in FG 33-6-2. Proposal 22: The type of FG 33-6-1 and 33-6-1a is per UE. Since the type of FG for support of the construction of two HARQ-ACK codebooks for unicast is per FS, the type of FG 33-6-2 should also be per FS. Proposal 24: The type of FG 33-6-2 is per FS. Since the type of FG for support of transmission of two PUCCHs in the same slot is per UE, the type of FG 33-6-3 would also be sufficient per UE. Proposal 25: The type of FG 33-6-3 is per UE. |
| [11] | Nokia, NSB | - 33-6-1: - Per UE - 33-6-2: - Per UE |
| [14] | Qualcomm | We suggest the following changes for the remaining FFSs of FG33-6-x as 33. NR_MBS; 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE; 33-2a, 33-2f; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1a; DL priority configuration for SPS multicast; Support of priority configured for multicast HARQ-ACK feedback of SPS multicast; 33-5-1a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-2; Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE.; 33-6-1; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-3; More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-1, 33-6-2; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |

Discussion

High priority question 8-1:

- Companies are encouraged to provide views on the reporting type of FGs 33-6-1 to 33-6-3.

  - FGs 33-6-1

    - Per UE: Huawei, HiSilicon, NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

  - FG 33-6-1a

    - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB

    - Per FSPC: MediaTek, Qualcomm

  - FGs 33-6-2

    - Per UE: Nokia, NSB

    - Per FS: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

  - FGs 33-6-3

    - Per UE: NTT DOCOMO

    - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update |
| Huawei, HiSilicon | Per UE seems sufficient but can also accept per FS. |
| MediaTek | Per FSPC |
| Spreadtrum | Fine with All as per FS. Also fine with per UE for FG33-6-1, 33-6-1a, 33-6-3. |
| Nokia, NSB | Same as above, per UE given that 33-2 is per FS already |
|  | - FGs 33-6-1 - Per UE: Huawei, HiSilicon, NTT DOCOMO, SPRD, Nokia, NSB, - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FG 33-6-1a - Per UE: Huawei, HiSilicon, NTT DOCOMO, Nokia, NSB, SPRD - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FGs 33-6-2 - Per UE: Nokia, NSB, Huawei, HiSilicon, - Per FS: NTT DOCOMO, Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm - FGs 33-6-3 - Per UE: NTT DOCOMO, Huawei, HiSilicon, SPRD, Nokia, NSB - Per FS: Huawei, HiSilicon, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Apple | Per FS for all. |
| Moderator | No consensus was achieved in this meeting. Comeback next meeting. |

Low priority proposal 8-2:

- Components of FG 33-6-1 are revised as

  - Component 1: “Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast dynamic scheduling”.

| Company | Comment |
| --- | --- |

Low priority question 8-3:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-6-1 to 33-6-3.

  - FG 33-6-1

    - Add FG 33-2f: Qualcomm

  - FG 33-6-1a

    - Add FG 33-5-1a: Qualcomm

| Company | Comment |
| --- | --- |

- 33-9: Supporting unicast PDCCH to release SPS group-common PDSCH

In [1], FG 33-9 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | [33-5-1] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [4] | Spreadtrum Communications | The Feature group refers to use unicast PDCCH to release SPS group-common PDSCH. In details, it means that one PDCCH scrambled with CS-RNTI is used to release SPS group-common PDSCH. In our mind, only UE supports unicast SPS, CS-RNTI would be configured. Thus, we suggest to add FG 5-18 (i.e., the capability of supporting DL SPS for unicast) as the prerequisite FG of FG33-9. Proposal 4: Support to add FG5-18 as the prerequisite FG of FG33-9. |
| --- | --- | --- |
| [10] | NTT DOCOMO | UE processing of this feature would not be band dependent. The type is sufficient for per UE. Proposal 26: The type of FG 33-9 is per UE. |
| [11] | Nokia, NSB | - 33-9: - Per UE |
| [14] | Qualcomm | We suggest the changes for the remaining FFSs of FG 33-9 as: 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1; Yes; Per FSPCPer UE; N/ANo; N/ANo; Optional with capability signalling. |

Discussion

High priority question 9-1:

- Companies are encouraged to provide views on the reporting type of FG 33-9.

  - Per UE: NTT DOCOMO, Nokia, NSB

  - Per FSPC: MediaTek, Qualcomm

| Company | Comment |
| --- | --- |
| Moderator | This issue can be discussed after the reporting type of FG 33-2 is decided |
| FL2 | Given the reporting type of FG 33-2 was agreed as per FS, companies are encouraged to provide further view if your preference/acceptable reporting type is update |
| Huawei, HiSilicon | Per UE or per FS |
| MediaTek | Per FSPC |
| Spreadtrum | Fine with per UE or per FS |
| Nokia, NSB | Same as above, per UE given that 33-2 is per FS already (assuming at least one pre-requisite will be added here which also has 33-2 as pre-requisite). |
|  | - Per UE: NTT DOCOMO, Nokia, NSB, HW, SPRD - Per FS: HW, SPRD - Per FSPC: MediaTek, Qualcomm [GTW3] Need further discussion in GTW |
| GTW4 | This proposal could not be discussed in the GTW on May 12. Comeback in the next GTW |
| Apple | Per FS |
| Moderator | No consensus was achieved in this meeting. Comeback next meeting. |

Low priority question 9-2:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 33-9.

  - Add FG 5-18: Spreadtrum Communication

| Company | Comment |
| --- | --- |

- Other FGs

This section discusses other FGs which are not included in [1].

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [4] | Spreadtrum Communications | Proposal 5: Add new FGs to reflect the capability of supporting p/sp ZP-CSI-RS for PDSCH RE mapping. 33. NR_MBS; 33-xx; Support group-common PDSCH RE-mapping patterns; 1) Supported sp ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Note 2: The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16; 2-33a, 33-2; Yes; Per UE; No; No; Optional with capability signalling. 33. NR_MBS; 33-yy; Support different p ZP-CSI-RS resource set configuration for multicast; Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; 33-xx; Yes; Per UE; No; No; Optional with capability signalling. |
| --- | --- | --- |

Discussion

[FL1] High priority question 10-1:

- Companies are encouraged to provide views on whether to introduce new FGs for the following capabilities.

  - Support of group-common PDSCH RE-mapping patterns

  - Support of different p-ZP-CSI-RS resource set configuration for multicast

    - Support: Spreadtrum Communications

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | ok |
| Spreadtrum | Support. This reflects the agreements we have achieved in RAN1#108. Agreement For multicast RRC_CONNECTED UEs, sp-ZP-CSI-RS-ResourceSetsToAddModList can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RE symbol level granularity, - the REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList configured in PDSCH-Config-Multicast are declared as not available for GC-PDSCH when their activation delivered by unicast PDSCH is applied. - sp-ZP-CSI-RS-ResourceSetsToAddModList configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - sp-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. - The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Also include this agreement in an LS to RAN2. Agreement Update the previous agreement for p-ZP-CSI-RS-ResourceSet as below: For multicast RRC_CONNECTED UEs, p-ZP-CSI-RS-ResourceSet can be configured in PDSCH-Config-Multicast for GC-PDSCH rate matching, subject to UE capability. For PDSCH resource mapping with RE symbol level granularity, - the REs indicated by p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast are declared as not available for GC-PDSCH. - p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config for unicast do not apply for GC-PDSCHs. - p-ZP-CSI-RS-ResourceSet in PDSCH-Config-Multicast for multicast do not apply for unicast PDSCHs. - The total number of periodic ZP-CSI-RS-Resources p-ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 - If p-ZP-CSI-RS-ResourceSet is configured in both PDSCH-Config and PDSCH-Config-Multicast, it is subject to UE capability whether the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast can be different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config. Also include this agreement in an LS to RAN2. |
| vivo | ok |
| FL2 | Given all companies are fine to introduce the FGs, following proposal is made [FL2] High priority proposal 10-1: - Introduce new FGs for the following capabilities. - Support of group-common PDSCH RE-mapping patterns - Support of different p-ZP-CSI-RS resource set configuration for multicast 33. NR_MBS; 33-10; Support group-common PDSCH RE-mapping patterns; 1) Supported SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Note 2: The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16; 2-33a, 33-2; Yes; FFS; FFS; FFS; Optional with capability signalling. 33. NR_MBS; 33-10a; Support different p-ZP-CSI-RS resource set configuration for multicast; Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; 33-10; Yes; FFS; FFS; FFS; Optional with capability signalling. |
| Huawei, HiSilicon | Looks ok but need a bit more time to check the granularity for reporting. |
|  | Reporting type can be discussed later [GTW3] High priority proposal 10-1: - Introduce new FGs for the following capabilities. - Support of group-common PDSCH RE-mapping patterns - Support of different p-ZP-CSI-RS resource set configuration for multicast 33. NR_MBS; 33-10; Support group-common PDSCH RE-mapping patterns; 1) Supported SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Note 2: The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16; 2-33a, 33-2; Yes; FFS; FFS; FFS; Optional with capability signalling. 33. NR_MBS; 33-10a; Support different p-ZP-CSI-RS resource set configuration for multicast; Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; 33-10; Yes; FFS; FFS; FFS; Optional with capability signalling. |
|  | Following was agreed in the GTW on May 12. Further discuss the reporting type Agreement - Introduce new FG for the support of group-common PDSCH RE-level rate matching for multicast 33. NR_MBS; 33-10; Support group-common PDSCH RE-level rate matching for multicast; 1) Supported SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Note 2: The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16; 2-33a, 33-2; Yes; FFS; FFS; FFS; Optional with capability signalling. [FL3][FL4] High priority question 10-1a: - Companies are encouraged to provide view on the reporting type of FG 33-10. |
| Huawei, HiSilicon | As 2-33a is included in phy-ParametersFRX-Diff, we think this reporting should be the same, i.e., per UE but with FRx differentiation, in this case, our RAN2 colleagues suggest it can be reported per band. |
| Nokia, NSB | OK to have it as per UE with FRx differentiation. |
| LG Electronics | OK with per UE with FRx differentiation |
| Spreadtrum | Ok to keep the same type as unicast |
|  | All companies are fine with per UE with FR1/FR2 differentiation. High priority proposal 10-1a: - Reporting type of FG 33-10 is per UE with FR1/FR2 differentiation |
| FL5 | This proposal could not be discussed in the GTW on May 17. If you cannot live with the proposal, please provide another proposal which is acceptable to all. |
| Huawei, HiSilicon | ok |
| ZTE | Support the FL proposal. |
| GTW6 | High priority proposal 10-1a: - Reporting type of FG 33-10 is per UE with FR1/FR2 differentiation |
| FL6 | No objection has been received in the last round. This proposal is set for email endorsement. If you have concern on this proposal, please indicate it directly over the reflector. |
|  | Following was agreed via email endorsement Agreement - Reporting type of FG 33-10 is per UE with FR1/FR2 differentiation [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 |

- Conclusions

Following agreements were made in this meeting.

Agreement

- Reporting type of FG 33-2 is per FS

Agreement

- Define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [N/A] | [N/A] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG 33-4 includes Alt1 as a component, Add FG 33-4a including Alt4 as component with separate PUCCH resource configurations

Agreement

- Introduce a separate FG for DCI indicated slot-level repetition up to 16 for MTCH from FG 33-3-1:

| 33. NR_MBS | 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE

Agreement

- Update FGs 33-4 and FG 33-4a as follows

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. [Support NACK-only based HARQ-ACK feedback.] for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits [2. Support of shared PUCCH resource configurations with unicast] FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast | 33-4 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

Agreement

- FG 33-5-1a is confirmed as follows

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- Confirm FG 33-5-1d as a separate FG from FG 33-5-1a

Agreement

- Introduce new FG for the support of group-common PDSCH RE-level rate matching for multicast

| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Supported SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 Note 2: The total number of periodic ZP-CSI-RS-Resources that a UE can be configured with is the same as for unicast in Rel-16 | 2-33a, 33-2 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- There is no consensus in RAN1 for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. Ask RAN2 to decide the candidate values

  - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e.

  - FFS how to indicate RAN1 status/concern

Agreement

Update the previous agreement as follows:

Agreement

- There is no consensus in RAN1 for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e. Ask RAN2 to decide the candidate values

  - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-RNTIs for group-common PDSCHs in FG 33-2e.

  - FFS how to indicate RAN1 status/concern

  - Note: from RAN1 perspective, the larger number of G-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget.

Agreement

- Reporting type of FG 33-2a is per BC

Agreement

- Reporting type of FG 33-2b is per band

Agreement

- Reporting type of FG 33-2d is per BC

  - FFS: Revise the component: Support of PTP retransmission for multicast on the cell same as multicast initial transmission

Agreement

- Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2

Agreement

- Reporting type of FG 33-2f is per band

Agreement

- Update the components of FG 33-5-2 as follows:

  - 1. Support [N>1] up to 8 SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group

  - 2.  Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast

  - 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 [per cell], and activated SPS group-common PDSCH configurations is no larger than M.

- Candidate value set for M is {1, 2, …, 8}

Agreement

- FG 33-3-2 is split into 2 FGs; One for support of FDM between one unicast PDSCH and one multicast PDSCH, the other for support of FDM between one unicast PDSCH and one broadcast PDSCH.

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH for multicast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |

Agreement

- Add an FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling

  - FFS whether to include retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI

Agreement

- Reporting type of FGs 33-4 is per BC

- Reporting type of FGs 33-4a is per BC

- Reporting type of FGs 33-4-1 is per band

Agreement

- There is no consensus in RAN1 for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h. Ask RAN2 to decide the candidate values

  - From RAN1 perspective, at least {2, 3, …, 8} should be supported for the candidate values for the max number of G-CS-RNTIs for SPS multicast in FG 33-5-1h.

  - Note: from RAN1 perspective, the larger number of G-CS-RNTIs UE supports, the more impact to whether UE can report the capability of supporting other UE features under a given UE implement with a certain number of RNTI budget.

Agreement

- Reporting type of FGs 33-5-1 is per FS

- Reporting type of FGs 33-5-1a is per BC

- Reporting type of FGs 33-5-1b is per band

- Reporting type of FGs 33-5-1d is per BC

  - FFS: Revise the component: Support of PTP retransmission for SPS multicast on the cell same as multicast initial transmission

- Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2

Agreement

- Reporting type of FG 33-10 is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2

Agreement

- Introduce a new FG for supported maximal modulation order for multicast PDSCH

  - Component 1: For FR1, up to 1024QsAM is supported, candidate values {256QAM, 1024QAM}

  - Component 2: For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM}

  - Consequence if the feature is not supported by the UE:

    - The UE supports the same modulation order as unicast

  - FFS additional components

  - Add a note: A UE shall support the corresponding mandatory maximum modulation for unicast.

References

[1]	R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2203071	Rel-17 UE features for NR MBS	Huawei, HiSilicon

[3]	R1-2203199	Discussion on Rel-17 UE features for NR MBS	ZTE

[4]	R1-2203319	UE features for R17 NR MBS	Spreadtrum Communications

[5]	R1-2203538	UE features for NR MBS	vivo

[6]	R1-2203779	Discussion on UE features for NR MBS	Xiaomi

[7]	R1-2203975	Discussion on UE features for NR MBS	OPPO

[8]	R1-2204225	Views on Rel-17 MBS UE Features	Apple

[9]	R1-2204286	Discussion on UE features for NR MBS	CMCC

[10]	R1-2204365	Discussion on Rel.17 UE features for NR MBS	NTT DOCOMO, INC.

[11]	R1-2204595	On UE features for NR MBS	Nokia, Nokia Shanghai Bell

[12]	R1-2204718	Views on UE features for NR MBS	MediaTek Inc.

[13]	R1-2204947	Views on NR MBS UE features	Ericsson

[14]	R1-2205008	UE features for MBS	Qualcomm Incorporated