3GPP TSG RAN WG1 #109-e			R1-2204411

e-Meeting, May 9th –20th, 2022

Agenda item:	8.16.17

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary on other UE feature related discussions

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.17 including any other UE feature related discussions not directly relevant to 8.16.1 ~ 8.16.16, i.e., not captured in [1] or [2], and captures the following email discussion.

[109-e-R17-UE-features-Others-01] Email discussion on other Rel-17 UE feature issues – Shinya (DOCOMO) - 1st check point for LS to RAN2: May 13 - Final check point for any remaining issues: May 20

Also, following issue in AI5 is discussed in this email discussion.

Release 17 - NR_CSIRS_L3meas R1-2203018 LS on the applicability of mixed numerology on UE capability maxNumberCSI-RS-RRM-RS-SINR RAN4, Apple To be taken into account as part of discussions on Rel-17 UE features in agenda item 8.16.

- UE features for UL Tx switching

In [2], FG 37-x is captured as placeholder for potential RAN1 UE features for Rel-17 UL Tx switching.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. [NR_RF_FR1_enh] | 37-x |  |  |  |  |  |  |  |  |  |  |  |  |

Following feedbacks are provided in contributions for the RAN1#109-e meeting.

| [3] | Huawei, HiSilicon | Based on RAN1 discussions on Rel-17 Tx switching, Rel-16 UE behaviors are fully reused to Rel-17. For example, for UL CA Option 1, the mechanism of Rel-16 uplink switching specified in S6.1.6.2 of TS 38.214 is reused, with the additional clarification that a switching between two carriers also covers the case of 2-port transmission to 2-port transmission in addition to the existing cases of 2-port to 1-port and 1-port to 2-port transmissions [2], as shown by the following agreement. Agreements: - For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.. - 6.1.6.2 Uplink switching for carrier aggregation For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.. Compared with the existing UE behaviors as highlighted above, the new add-on in the agreement is obviously a simple clarification for UL-CA Option 1, which causes no unrealistic UE implementation issue. Therefore, we don’t see a need to have new UE capability for UL-CA Option 1. For UL CA Option 2, based on the Rel-17 agreement [2] and Rel-16 agreement [3], the only new UE behavior for the additional switching state is a simple clarification on the state ambiguity issue [4]. It does not cause any unrealistic UE implementation, as shown by the following agreements. Rel-17 [2][4]: Agreements: - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control.. Agreement: - For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, introduce a new RRC parameter to configure between 1) and 2) - 1) The state of Tx chains supporting 2Tx transmission on the carrier is assumed. - 2) 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.. Rel-16 [3]: Agreements: - For inter-band UL CA, if uplink Tx switching is configured: - For option 1 of mapping between UL transmission ports and Tx chain, the switching period is only applicable when the UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2. - Note: 2Tx carrier 2 refers to an UL carrier capable of 2 Tx chains and both 1-port and 2-port UL transmissions. - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1 Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on carrier 2. - If the current state of Tx chains is 0 Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port transmission on carrier 1. - For other cases, the state of Tx chains of last UL transmission is assumed. - Note: No spec change to power configuration and power control.. Therefore, for UL-CA Option 2, the only difference of UE behavior is also very small, which obviously cause no unrealistic implementation issue. We don't see a need to have new UE capability for UL-CA Option 2. Because, in RAN2#116bis-e, the following agreed CR[5] has shown that the “uplinkTxSwitchingPeriod2T2T” is used to indicated whether 2Tx-2Tx switching is supported by UE supporting 1Tx-2Tx switching. There is no need to add signaling to indicate whether UE support R17 UL Tx switching. ULTxSwitchingBandPair-v17xx ::= SEQUENCE { uplinkTxSwitchingPeriod2T2T-r17 ENUMERATED {n35us, n140us, n210us} OPTIONAL }. supportedBandPairListNR-r16, supportedBandPairListNR-v17xx Indicates a list of band pair supporting UL Tx switching as defined in TS 38.101-1 [15] for a given band combination. A UE supporting 2Tx-2Tx switching should include both of supportedBandPairListNR-r16 and supportedBandPairListNR-v17xx. And the UE shall include the same number of entries listed in the same order as in supportedBandPairListNR-r16. If the UE does not support 2Tx-2Tx switching for a given band pair, the field of uplinkTxSwitchingPeriod2T2T in the corresponding entry is absent.. There is existing FG 22-1 to indicate Option 1/Option 2 and it is per BC capability since Rel-16, as shown below. Features; Index; Feature group; Components. 22. NR Others; 22-1; Indicating supported option for UL Tx switching for inter-band UL CA; Indicating supported option for UL Tx switching for inter-band UL CA Candidate values set is {option1, option2, both option 1 and option 2}. In RAN1#108-e, new UE capability FG 37-1 is proposed as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA” [1]. The reasoning seemed to be more flexibility with the potential reduction of workload for implementation and testing. However, such flexibility is not needed in case that a UE reports Rel-16 UL CA option1 and also reports the new Rel-17 UE capability defined in RAN2 for 2Tx-2Tx switching. 37. NR_RF_FR1_enh; 37-1; Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA; Indicating supported option for 2Tx-2Tx UL Tx switching for inter-band UL CA - Candidate values set is {option1, option2, both option 1 and option 2}; 22-1; Yes; N/A; Per BC; N/A; N/A (FR1 only); N/A; If UE supports Option1 for FG 37-1, the UE also needs to support Option1 for FG 22-1; If UE supports Option2 for FG 37-1, the UE also needs to support Option2 for FG 22-1; If UE supports both Option1 and Option2 for FG 37-1, the UE also needs to support both Option1 and Option2 for FG 22-1;; Optional with capability signalling. Another motivation for FG 37-1 seemed to grant a UE the flexibility to report Option 1 only (or Option 2 only) for 2Tx-2Tx even the UE has reported a support of {both Option 1 and Option 2} for 1Tx-2Tx. However, Rel-17 UL CA Option1 is exact the same as Rel-16 UL CA Option1, which both triggers UL Tx switching only when UL carrier is switched. It’s unreasonable that a UE capable of Rel-16 Option 1 and 2Tx-2Tx cannot support Rel-17 Option 1, which is allowed by the new capability FG 37-1. This would cause unnecessary troubles for gNB only capable of Rel-16 Option 1 and Rel-17 Option 1. Therefore, the new FG37-1 is not justified and unnecessary. Observation 1: Since Rel-16 UE behaviors of UL Tx switching is reused with small add-ons for Rel-17, the existing FG 22-1 is sufficient for Rel-17 UL Tx switching while new UE capability FG37-1 is unnecessary. From UE flexibility perspective, as analyzed above, the existing UE capabilities have provided all potential flexibility for UEs capable of Option 1 only or Option 2 only. Thus, no new UE capability is needed. As for UEs capable of both Option 1 and Option2, because the Rel-17 Option 2 has very small spec impact compared to Rel-16 Option 2, additional reporting flexibility is not justified and thus no new UE capability is needed either. Observation 2: Full UE capability flexibility has been supported by the existing agreed UE capabilities at least for UEs capable of UL-CA Option 1 only and UEs capable of UL-CA Option 2 only. However, for the sake of progress, we are OK for further discussion with the following proposal as a starting point, where changes are only about the UEs capable of both Option 1 and Option 2: - Specify a new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching UL CA when “both option 1 and option 2” is indicated via FG 22-1) - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “both option 1 and option 2” via FG 22-1 and indicating the new per-BC FG supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. Proposal: Take the following proposal as a starting point for any discussion on the new UE capability of Rel-17 UL Tx switching: - Specify a new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching UL CA when “both option 1 and option 2” is indicated via FG 22-1) - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “both option 1 and option 2” via FG 22-1 and indicating the new per-BC FG supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. |
| --- | --- | --- |
| [4] | ZTE | During RAN1#108-e meeting, companies discussed the following proposal and it was almost agreeable to companies. Final proposal - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2 - UE supports this new FG needs to support Rel-16 CA Option 2 - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following, - Indicating “option 2” via the new per-BC FG - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1) From our perspective, the above can be used as the baseline. Regarding whether to use “uplinkTxSwitchingPeriod2T2T-r17” or “supportedBandPairListNR-v17xx”, our understanding is that “uplinkTxSwitchingPeriod2T2T-r17” is the correct IE since RAN2 also uses this IE to indicate whether UE supports Rel-17 UL Tx switching. Thus the proposal can be updated as following. Proposal 1: Endorse the following - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2 - UE supports this new FG needs to support Rel-16 CA Option 2 - UE indicating uplinkTxSwitchingPeriod2T2T-r17 and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well. - UE indicating uplinkTxSwitchingPeriod2T2T-r17 supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following, - Indicating “option 2” via the new per-BC FG - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1) |
| [5] | DOCOMO | As summarized in [2], there has been extensive discussions on whether to introduce FG 37-1 as “Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA”. Based on the discussion, following compromised proposal was provided by the FL. Final proposal  Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2  UE supports this new FG needs to support Rel-16 CA Option 2  UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.  UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,  Indicating “option 2” via the new per-BC FG  Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1) However, there was only one company who could not agree on the above proposal, and the company provided the following revised proposal. Proposal-rev - Specify a new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching UL CA when “both option 1 and option 2” is indicated via FG 22-1) - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “both option 1 and option 2” via FG 22-1 and indicating the new per-BC FG supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well. Due to the lack of the meeting time, at the end of the last RAN1 meeting, there was no consensus on whether to introduce FG 37-1, and it has not been captured in the UE features list yet. As in the above proposals, it seems all companies are fine to specify a new per-BC FG to indicate the support of “option 2” for Rel-17 2Tx-2Tx UL Tx switching, and all companies are also fine to support following four types of UEs supporting Rel-17 2Tx-2Tx UL Tx switching. - Type 1: UE supporting option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching - For this type 1, companies are fine that FG37-1 is not necessary. - Type 2: UE supporting option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching - For this type 2, companies are fine that FG37-1 is not necessary. - Type 3: UE supporting option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching - For this type 3, companies are fine that FG37-1 is necessary. - Type 4: UE supporting option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching - For this type 4, majority of companies are fine that FG37-1 is necessary, while only one company argued that FG37-1 is not necessary as this type of UE indicates the support of option 2 only via FG22-1 and the support of 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. So, only one remaining discussion point is whether reporting FG37-1 in case of above Type 4 UE (supporting only option 2 for both Rel-16/17 UL Tx switching) is necessary or not. In our view, although the support of option 2 only for both Rel-16/17 UL Tx switching can be interpreted based on the support of option 2 only via FG22-1 and the support of 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx], using FG37-1 in addition to them is also fine and it may be better to indicate FG37-1 for Type 4 in terms of the consistent meaning of FG37-1, i.e., this FG is to indicate the support of option 2 for Rel-17 2Tx-2Tx UL Tx switching. If the majority is fine to not indicate the FG37-1 for Type 4, we are also fine. The most important point is to define the FG37-1 at least for Type 3 as it is the compromised outcome of extensive discussion so far and this meeting is the final opportunity to do it for Rel-17. Proposal 1: The new FG 37-1 is defined for “Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA”. - If UE supports option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “switchedUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. - If UE supports option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. - If UE supports option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. - If UE supports option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “dualUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. 37. NR_RF_FR1_enh; 37-1; Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA; - Indicating “option 2 support for Rel-17 2Tx-2Tx UL Tx switching for inter-band UL CA; 22-1, the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or sup-portedBandPairListNR-v17xx]; Yes; N/A; Per BC; N/A; N/A (FR1 only); N/A; -If UE supports option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “switchedUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching, the UE does not indicate this FG and the UE indicates “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “both” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx]. -If UE supports option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching, the UE indicates this FG, “dualUL” via FG22-1 (uplinkTxSwitching-OptionSupport-r16) and the support of Rel-17 2Tx-2Tx UL Tx switching via [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx].; Optional with capability signaling. |
| [6] | Nokia/NSB | Define the FG for support of UL TX switching as: 37. [NR_RF_FR1_enh]; 37-1; Indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA; Indicating support for 2Tx-2Tx UL Tx switching for inter-band UL CA; 22-1; Yes; N/A; Per BC; N/A; N/A (FR1 only); N/A; The UE supports the same options as signalled by FG 22-1.; FFS details. Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list. |
| [7] | Qualcomm | In the RAN1#108-emeeting, the proposals were discussed in email thread [108-e-R17-UE-features-Others-01]. Two proposals from different proponents were summarized by FL [5] as below and the difference was highlighted. Proposal #1; Proposal #2. RAN2 FG; FG 22-1 (Rel-16); Rel-17 2Tx-2Tx; FG 22-1 (Rel-16); Rel-17 2Tx-2Tx. Support; Option 1; Option 1; Option 1; Option 1. Support; Option 1 and 2; Option 1; Option 1 and 2; Option 1. Support; Option 2; Option 2 without new per-BC FG; Option 2; Option 2 with new per-BC FG. Support; Option 1 and 2; Option 1 and 2 with new per-BC FG; Option 1 and 2; Option 1 and 2 with new per-BC FG. The consensus is to define a new capability for inter band CA Option 2. The difference between Proposal #1 and #2 are how a Rel-17 Option 2 only UE to report its capability. Proposal #1 suggests to use Rel-16 Option 2 capability together with the Rel-17 2Tx-2Tx switching capability, while Proposal #2 suggests directly report the new capability. The proponent of Proposal #1 claims Proposal #1 would save the newly defined capability bit by report the legacy Rel-16 Option 2 capability for a Rel-17 Option 2 only UE. However, we can’t agree with this. Based on our understanding, as this new capability is to be defined as an optional capaiblity, this bit would be consumed regardless for a band combination where the UE supports 2Tx-2Tx. Therefore, we support Proposal # 2 above and propose to adopt the following proposal while the highlighted capability could be revisited by RAN2. - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2 - UE supports this new FG needs to support Rel-16 CA Option 2 - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following, - Indicating “option 2” via the new per-BC FG - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1) Proposal: Agree on the following proposal for Rel-17 UE capability. - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2 - UE supports this new FG needs to support Rel-16 CA Option 2 - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well. - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following, - Indicating “option 2” via the new per-BC FG - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1) |

Discussion

Proposal 2-1:

- Down select from one of the following alternatives for indicating supported option for UL Tx switching for 2Tx-2Tx inter-band UL CA

  - Alt1:

    - Specify a new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching UL CA when “both option 1 and option 2” is indicated via FG 22-1)

    - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 1 as well.

    - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 2” via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well.

    - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “both option 1 and option 2” via FG 22-1 and indicating the new per-BC FG supports Rel-17 2Tx-2Tx UL Tx switching UL CA Option 2 as well.

  - Alt2:

    - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

      - UE supports this new FG needs to support Rel-16 CA Option 2

    - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

    - UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

      - Indicating “option 2” via the new per-BC FG

      - Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

  - Alt3:

    - Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching

      - The UE supports the same options as signalled by FG 22-1.

| Company | Comment |
| --- | --- |
| Moderator | Summary of companies view - Alt1: HW/HiSi - Alt2: ZTE, DOCOMO, QC - Alt3: Nokia/NSB Comparison of the alternatives Alt1; Alt2; Alt3. RAN2 FG; FG 22-1 (Rel-16); Rel-17 2Tx-2Tx; FG 22-1 (Rel-16); Rel-17 2Tx-2Tx; FG 22-1 (Rel-16); Rel-17 2Tx-2Tx. Support; Option 1; Option 1; Option 1; Option 1; Option 1; Option 1 with new per-BC FG. Support; Option 1 and 2; Option 1; Option 1 and 2; Option 1; N/A; N/A. Support; Option 2; Option 2 without new per-BC FG; Option 2; Option 2 with new per-BC FG; Option 2; Option 2 with new per-BC FG. Support; Option 1 and 2; Option 1 and 2 with new per-BC FG; Option 1 and 2; Option 1 and 2 with new per-BC FG; Option 1 and 2; Option 1 and 2 with new per-BC FG. Note that Alt3 does not need to define the new FG since the RAN2 FG [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] can be used for the same purpose to indicate the support of Rel-17 2Tx-2Tx UL Tx switching This issue has been discussed in previous RAN1 meetings but companies still have different view. Moderator would at first ask companies whether following four types of UEs can be considered for Rel-17 2Tx-2Tx UL Tx switching, and the only remaining issue is whether the Type 3 UE should/can report the new FG. If you have different view on them, please provide alternative proposal which can be acceptable to all companies. If all companies share the same understanding on them, we should make decision on the remaining issue (i.e., Alt.1 vs Alt.2) so that we can define the new FG at least for differentiating Type 2 UE and Type 4 UE. Companies are encouraged to provide views/questions/suggestions if any. - Type 1: UE supporting option 1 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching - Type 2: UE supporting option 1+2 for Rel-16 1Tx-2Tx UL Tx switching and option 1 only for Rel-17 2Tx-2Tx UL Tx switching - Type 3: UE supporting option 2 only for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching - Type 4: UE supporting option 1+2 for both Rel-16 1Tx-2Tx and Rel-17 2Tx-2Tx UL Tx switching |
| NTT DOCOMO | As described in our contribution, we agree that Type 1/2/3/4 can be considered and the remaining issue is Alt.1 vs Alt.2 only, according to the discussion in previous RAN1 meetings. The most important point is to define the new FG to allow both Type 2 and Type 4 as it is the outcome of extensive discussion so far and this meeting is the final opportunity to do it for Rel-17. So we suggest to agree on either Alt.1 or Alt.2 within the first week of this meeting to ask RAN2 to define the capability signalling. |
| Qualcomm | We support Alt 2 and can’t agree Alt 1. During the discussion in RAN1 #108-e, the common consensus is to define a new capability for inter band CA Option 2. The difference between Alt 1 and Alt 2 are how a Rel-17 Option 2 only UE to report its capability. Alt 1 suggests using Rel-16 Option 2 capability together with the Rel-17 2Tx-2Tx switching capability, while Alt 2 suggests directly report the new capability. The proponent of Alt 1 claims it would save the newly defined capability bit by report the legacy Rel-16 Option 2 capability for a Rel-17 Option 2 only UE. However, we can’t agree with this. Based on our understanding, as this new capability is to be defined as an optional capaiblity, this bit would be consumed regardless for a band combination whether the UE supports 2Tx-2Tx. On the UE types, we think there should be more types of UE needs to be considered, at least e. g. UE supporting Option 1+2 for Rel-16 1Tx-2Tx switching and Option 2 for Rel-17 2Tx-2Tx switching. As we commented couple meetings ago, UE implementation would be restricted by network due to IoDT consideration. There might be other UE types during feature development and thus keep the flexibility of the newly defined signalling is needed. With all the respect, we don’t see any point to restrict a newly defined capability signalling only be used for a sub-set of cases and ask Network to speculate the UE’s capability. |
| ZTE | We share similar view as Qualcomm. This issue has been discussed for several times and it was almost agreed in last meeting, we hope we can finalize this issue in this meeting. |
| Huawei, HiSilicon | The key is that the existing UE capabilities, i.e. [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and FG 22-1, are sufficient and flexible enough to indicate the UE Type 1, Type3, Type 4 and UEs capable of 1Tx-2Tx only. The only remaining issue is whether UE Type 2 can be further indicated by UE capability reporting. As analysed in our tdoc R1-2203171, it is unnecessary because the spec change on 2Tx-2Tx Option 2 UL Tx switching is very marginal compared to 1Tx-2Tx UL switching only. With such marginal spec change, we feel that a testing environment that has been capable of 1Tx-2Tx Option2 and 2Tx-2Tx Option1 must be capable of the test for 2Tx-2Tx Option2. In other words, we don’t see any IoDT issue caused by incompetent testing environment that forces a UE type 4 to have a fallback report for UE Type 2. Therefore, a new UE capability is unnecessary. If company prefers clean UE capability reporting, the existing UE capabilities are the cleanest. On the contrary, the Alt 2 introduces unnecessary reporting overlaps with the existing capabilities and costs more spec impacts to clarify the overlaps. As a result, the Alt2 requires all Rel-18 gNBs capable of Option 1 only or Option 2 only to implement the new capability checking caused by Alt 2 which should only require gNBs capable of both Option1 and Option2 to implement it. For the reasons above, we cannot agree Alt 2. We sincerely hope we would not have to repeat our concerns again. |
| Moderator | Further discuss directly over the reflector. |
| Moderator | No conclusion is made in this meeting. |

- Applicability of mixed numerology on UE capability maxNumberCSI-RS-RRM-RS-SINR

As suggested by RAN1 chair, following issue in AI5 is discussed in this email discussion.

Release 17 - NR_CSIRS_L3meas R1-2203018 LS on the applicability of mixed numerology on UE capability maxNumberCSI-RS-RRM-RS-SINR RAN4, Apple To be taken into account as part of discussions on Rel-17 UE features in agenda item 8.16.

In R1-2203018, RAN4 informs following:

In RAN4#102-e, RAN4 has discussed the scenario of mixed numerologies across all measurement frequencies, where the corresponding slot duration is not clearly specified depending on the different numerologies. RAN4 concludes that the UE capability of maxNumberCSI-RS-RRM-RS-SINR and the related requirements should be specified based on the duration associated with the slot of minimum SCS in case of mixed numerologies. ACTION: RAN4 respectfully asks RAN1 and RAN2 to take RAN4 conclusions in consideration for their work.

Discussion

Question 3-1:

- Companies are encouraged to provide view on whether/what RAN1 action is necessary for R1-2203018.

| Company | Comment |
| --- | --- |
| Apple | We agree with RAN4 conclusion. We do not think RAN1 needs any further action, it can be captured in TS38.306 |
| NTT DOCOMO | We share the same view with Apple. RAN4 conclusion is reasonable and RAN2 can capture it in TS38.306. No RAN1 action is necessary. |
| Qualcomm | We agree with Apple. The RAN4 conclusion is agreeable and TS38.306 can be updated by RAN2, accordingly. |
| Huawei, HiSilicon | We share the view of above companies and it is already captured in RAN2 discussion agenda. No RAN1 following up discussion needed for now. |
| Moderator | Given all companies don’t think RAN1 action is necessary, no further discussion is necessary for this issue. |

- Conclusions

No conclusions are made in this meeting.

References

[1]	R1-2202926	Updated RAN1 UE features list for Rel-17 LTE after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

[3]	R1-2203171	Rel-17 UE features for UL Tx switching	Huawei, HiSilicon

[4]	R1-2203202	Discussion on other Rel-17 UE features	ZTE

[5]	R1-2204366	Discussion on other UE feature related discussions	NTT DOCOMO, INC.

[6]	R1-2204598	On Remaining Rel-17 UE features	Nokia, Nokia Shanghai Bell

[7]	R1-2205012	UE features for Rel-17 UL Tx switching	Qualcomm Incorporated