3GPP TSG RAN WG1 #109-e	                                       	  R1-2204414

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.1

Source:	Lenovo

Title:	XR-specific power saving techniques

Document for:	Discussion

# Introduction

The following objectives on XR-specific Power Saving (RAN1, RAN2) have been identified by the study item description (RP-220285)

- Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques:

  - C-DRX enhancement.

  - PDCCH monitoring enhancement.

This contribution expresses our initial views on the identified power saving enhancements.

# Discussion

## C-DRX enhancement

NR supports integer C-DRX cycles (e.g., 16 ms), however, XR traffic frame inter-arrival time is not integer (e.g., 16.67 ms). Such a mismatch between the C-DRX cycle and the frame inter-arrival time could lead to additional power consumption. In Rel-17 XR study item, it has been suggested to adjust some of the C-DRX cycle start offsets to reduce the additional power consumption. Such an adjustment could be done dynamically or semi-statically. In our view, any proposed scheme needs to solve the mismatch issue in the presence of frame arrival jitter, otherwise the benefit of such adjustment of C-DRX cycle start offset would be impacted.

Observation 1: C-DRX cycle start offset adjustment needs to solve the mismatch issue in the presence of frame arrival jitter, otherwise the benefit of such adjustment would be impacted.

### ADU dropping impact on C-DRX

According to SP-211166:

- Packets within a frame have dependency with each other since the application needs all of these packets for decoding the frame. Hence one packet loss will make other correlative packets useless even they are successfully transmitted. For example, XR applications impose requirements in terms of Media Units (Application Data Units), rather than in terms of single packets/PDUs.

Assuming RAN/scheduler is aware of such packet dependencies, the already scheduled packets that their reception might not be useful can be discarded leading to power saving. The active time of the corresponding DRX cycle can be reduced to potentially save more power. Existing tools such as DRX Command MAC CE and/or PDCCH skipping can be used. It would be good to further check if any additional enhancements needed.

Proposal 1: Study impact of ADU dropping on DRX timers (including drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL).

## PDCCH monitoring enhancement

PDCCH monitoring adaptation and search space set group switching techniques (e.g., via scheduling DCI or group-common DCI indication) can be used to reduce UE’s power consumption by skipping PDCCH monitoring in some slots such as the slots where the network does not expect to schedule an UL and/or DL transmission. To keep the DCI size limited for the DCI indicating the PDCCH skipping duration, only a limited number of PDCCH skipping durations (up to 3 values from a configured set of possible values) can be indicated.

For XR traffic, as shown in Figure 1, a potential range for PDCCH skipping is different from one video frame or application data unit (ADU) to another video frame/ADU due to the following:

- the arrival times are quasi-periodic instead of periodic due to traffic jitter

- the video frame/ADU sizes are different

Figure 1: potential range for PDCCH skipping can be different after each video frame or ADU.

To enable network to indicate a more relevant PDCCH skipping duration,

- PDCCH skipping duration indication field size can be increased (at the expense of DCI overhead) or

- given the limited DCI field size for PDCCH skipping duration indication (e.g., at most 2 bits for PDCCH skipping), UE can determine PDCCH skipping duration from a subset (e.g., containing at most 3 possible PDCCH skipping durations) of a larger set of RRC configured skipping durations; where the subset is determined based on a MAC-CE indication, or based on the XR traffic periodicity, jitter statistics and previous jitter realizations for a DL video transmission (assuming RAN/scheduler is aware of some XR traffic characteristics (e.g., traffic periodicity, jitter statistics for DL video transmissions)).

Proposal 2: PDCCH skipping duration is selected from a larger set of RRC configured skipping durations to accommodate XR traffic jitter or varying video frame size.

## HARQ-ACK enhancements for power saving

Concentrating UL and DL transmissions in nearby resources in time can save power. For instance, HARQ-ACK of a PDSCH can be postponed to a nearest UL resource that is close to a later scheduled DL transmission. For instance, in Figure 2, the 2nd UL resource is the earliest available UL resource that is at most ‘l1’ slots after the 2nd PDSCH and no later than ‘l2’ slots after the 1st PDSCH. Such postponing increases the delay by l2-k1 slots and can be useful in cases that such additional delay is tolerable.

Figure 2: HARQ-ACK resource determination for UL-DL alignment: 2nd PUCCH is used for HARQ-ACK for the 1st PDSCH as opposed to the 1st PUCCH indicated by k1 as the gap between the 2nd PUCCH and 2nd PDSCH is small.

Proposal 3: Study the power-saving vs. delay trade-off of postponing HARQ-ACK of a PDSCH to a nearest UL resource that is close to a later PDSCH.

# Conclusions

This contribution provided our views regarding XR-specific power saving enhancements as follows:

Proposal 1: Study impact of ADU dropping on DRX timers (including drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL).

Proposal 2: PDCCH skipping duration is selected from a larger set of RRC configured skipping durations to accommodate XR traffic jitter or varying video frame size.

Proposal 3: Study the power-saving vs. delay trade-off of postponing HARQ-ACK of a PDSCH to a nearest UL resource that is close to a later PDSCH.

Observation 1: C-DRX cycle start offset adjustment needs to solve the mismatch issue in the presence of frame arrival jitter, otherwise the benefit of such adjustment would be impacted.