3GPP TSG RAN WG1 #109-e	                                      	  R1-2204415

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.2

Source:	Lenovo

Title:	XR-specific Capacity Enhancement Techniques

Document for:	Discussion

# Introduction

The following objectives on XR-specific capacity improvements (RAN1, RAN2) have been identified by the study item description (RP-220285)

- Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms:

  - SPS and CG enhancements;

  - Dynamic scheduling/grant enhancements.

This contribution expresses our initial views on the identified capacity improvements.

# Discussion

## Dynamic scheduling/grant enhancements

If the scheduler, and/or the UE is aware of some of XR application layer information (e.g., (a) which packets belong to an ADU/PDU set, (b) importance information of a PDU set (c) dependency information between different PDU sets (d) delay budget of a packet/PDU set), such information can be exploited to optimize resource allocation as will be discussed in the following.

### Delay Awareness

The gNB can take knowledge of PDU set/packet delay into account in scheduling transmissions, e.g., by giving priority to transmissions close to their delay budget limit, and by not scheduling (e.g., UL) transmissions exceeding a packet/PDU set delay budget. The UE can also take advantage of such knowledge to save UE’s power by determining if an UL transmission (e.g., PUCCH in response to PDSCH, UL pose, or PUSCH) corresponding to a transmission that exceeds its delay budget can be dropped. Additionally, UE does not need to wait for re-transmission of a PDSCH that will never occur (e.g., DRX retransmission timers can be stopped). In case that the gNB does not have direct access to application layer information, e.g. due to privacy issues, etc. but if the UE can identify application layer information based on internal interaction with its application layer and can convert the application layer information to relevant radio interface Quality of Service (QoS) requirements, the UE can indicate Retransmission related Control Information (RCI) for both DL and UL packets to the gNB. For example, the RCI (e.g. information of a remaining valid duration for retransmission of a TB, a request of no further retransmission of a TB) can be transmitted along with HARQ-ACK feedback or other uplink control information (UCI).

Proposal 1: Study the benefits of indicating the remaining delay budget for a packet and/or a PDU set:

- gNB indicating such delay budget to UE for DL/UL packet and/or PDU set

- UE indicating such delay budget to gNB for UL/DL packet and/or PDU set

Note that a notion of the delay budget could be indicated: e.g., (a) the UE can indicate to the gNB that no further retransmission of an UL TB is required or (b) gNB can cancel the already scheduled transmissions corresponding to a packet/PDU set for which a delay budget is exceeded.

### Multi-PD(U)SCH Scheduling

The packet size of I-frames/I-slices is large, so there would be more TBs (e.g., over multiple slots) scheduled by a single DCI. In this case, UE transmits the HARQ-ACK after the last TB will cause large delay to the first several scheduled TBs, so how to decrease the latency should be studied. For example, multiple TBs could be divided into multiple TB groups and the HARQ-ACK for one TB group could be transmitted timely after that TB group.

Proposal 2: Study latency reduction for HARQ-ACK transmission for multi-PD(U)SCH scheduling.

Another solution to the delayed HARQ-ACK issue (for both single DCI multi-TB scheduling, and multiple single DCIs scheduling couple of TBs each) could be to send any NACK of the first few TBs in a resource that is earlier than a resource used for multiplexing all HARQ-ACK of the multiple TBs. Such a scheme can help gNB schedule the retransmissions in time and avoid PDB expiration. To avoid any ambiguity between gNB and UE, the HARQ-ACK of all TBs (including the ones that have been NACK’d in an earlier PUC(S)CH) are multiplexed in the later PUC(S)CH.

Proposal 3: Investigate HARQ-NACK prioritization benefits to avoid PDB expiration.

Large video frames of XR traffic can be transmitted using a carrier with high SCS to satisfy the latency and reliability requirements of the XR traffic. In XR, a video frame can be an I-frame, P-frame, or can be composed of I-slices, and/or P-slices. I-frames/I-slices are more important and larger than P-frames/P-slices. Assuming the scheduler is aware of I-slice and P-slice boundaries, the specified multiple TB scheduling (UL or DL) can be further enhanced considering the differences between I-slices and P-slices in a video frame that includes both an I-slice and a P-slice, scheduled by a single DCI. For instance, HARQ-ACK/PUSCH priority, and HARQ-ACK bunding can be different for an I-slice vs. a P-slice.

Proposal 4: Study if multi-PD(U)SCH scheduling should be further enhanced based on application awareness.

### PHR

If gNB receives upfront some power headroom information from the UE, the NW can provide efficient UL scheduling and consequently shorten the transmission time of the UE, which will ultimately also lead to increased system capacity. To ensure a timely PHR, an UL DCI can trigger a PHR, e.g., when a first TB of an ADU is scheduled.

Proposal 5: Study techniques providing timely PHR, e.g., UL DCI triggering a PHR.

To better match the PHR timers with the XR traffic arrival times, and hence provide a more accurate PHR, new timer values can be proposed for instance, derived based on the XR traffic arrival periodicity or UL pose periodicity.

Proposal 6: Study if PHR should be further enhanced based on XR traffic arrival periodicity or UL pose periodicity.

## SPS and CG enhancements

Considering XR traffic jitter and XR packet variable size, dynamic scheduling seems to be a good tool to be used to serve the XR traffic. Nonetheless, SPS/CG can be used to exploit quasi-periodic nature of video frame arrivals without the need for DCI and/or SR signalling. SPS/CG enhancements can help with addressing XR traffic jitter and variable size. For instance, within a SPS/CG period, multiple SPS/CG configurations having the same periodicity with SPS resources of different size and starting time can be used along with blind decoding to adapt to variable packet size and arrival time.

Further, in XR applications such as immersive online gaming and a smart helmet, latency-sensitive set of data collected from multiple cooperating sensors/devices (e.g. motion sensors, cameras, and audio devices) of a UE may need to be uploaded to an XR server within a certain time window for real-time rendering and/or virtual control of machines or other objects. Similarly, multiple packets of different QoS requirements for the multiple cooperating devices of the UE may need to be downloaded from an application server in a quasi-synchronous manner. Thus, SPS/CG enhancement such as joint activation of multiple SPS/CG configurations (potentially with different periodicities) for an indicated duration can address handling of multiple traffics of different QoS requirements in a quasi-synchronous manner with reduced DL and UL control singling overhead.

Proposal 7: Study SPS/CG enhancements to address XR traffic variable packet size and arrival time and quasi-synchronous communication of multiple flows. Enhancements may include:

- Enabling, within a SPS/CG period, multiple SPS/CG configurations having the same periodicity with SPS resources of different size and starting time

- Joint activation of multiple SPS/CG configurations for an indicated duration to handle multiple traffics of different QoS requirements in a quasi-synchronous manner with reduced control signaling overhead

## Enhancements applicable to both semi-static and dynamic scheduling

### Network Coding

Network coding (NC) can provide packet-based redundancy and better exploit link diversity beyond the current PDCP duplication procedure. As an outer coding mechanism applied before forward-error correction coding (FEC), NC offers therefore adaptive packet-based redundancy and can be further coupled with transmission mechanisms exploiting link diversity (e.g., carrier aggregation, multi-layer transmission, multi-TRP transmissions etc.) to support reliable and high throughput applications such as XR. However, the application and impact of NC needs to be considered across the L1/L2 stack. The NC error recovery effectiveness is influenced by the packet redundancy level which in turns depends on channel state and link quality. Lack of adequate HARQ feedback and too low redundancy for NC packets would lead to missing the delay budget of XR packets as their acknowledgements would be handled earliest by RLC status reports, thus incurring more than 20ms of feedback latency.

Proposal 8: Study the application of NC for XR traffic and its influence on L1/L2 stack.

Proposal 9: Study efficient and backwards-compatible feedback procedures for NC and investigate role of existent HARQ feedback loop.

### Video slice mapping to CBG and CBG enhancements

It is expected that TBs of XR traffic are generally carrying a high payload potentially fitting two or more video slices to accommodate the ADUs and XR traffic requirements in terms of peak data rate, average throughput, and packet delay budget. Given the bursty, quasi-periodic XR traffic characteristic, one of the challenges of XR traffic at L1 is the necessity to quickly retransmit and resolve any erroneously received TBs by a receiver. As errors may affect only part of the CBs of a TB, it is thus of interest to reduce the retransmission overhead to the CB payloads affected only. To this end, the CBG feature of NR may be used. However, the mapping of XR traffic logical channels to a TB, subsequent CBs and enclosing CBGs may benefit from application awareness to map and align video slices to CBGs. Beyond such slice-level awareness, slice importance awareness may be relevant to L1 for FEC with unequal error protection, providing lower coding rates for highly important slices and higher coding rates for lower importance slices. Leveraging the XR application awareness at L1/L2 may thus provide two-fold gains, on one hand reducing the overhead of retransmissions via CBGs, and on the other hand offering proportional error protection to importance of XR information carried by the CBGs.

Proposal 10: Investigate leveraging XR application awareness (e.g., video slice and stream awareness, video slice importance) to map video slices to TB CBGs for optimized transmissions and retransmissions of XR traffic.

However, for CBG-based retransmission, one drawback is the HARQ-ACK feedback overhead may be too much considering the maximum number of CBGs per TB is configured by RRC signaling. In that sense, for each TB, the number of corresponding CBG-level HARQ-ACK information bits is always equal to the RRC configured maximum number. Even though a TB is scheduled with a small TB size, UE has to generate the CBG-level HARQ-ACK information bits for the TB and align the CBG-level HARQ-ACK information bits in the HARQ-ACK codebook with the RRC configured maximum number, e.g., by appending NACK bits. Thus, unnecessary HARQ-ACK feedback overhead may be caused which impacts UE transmit power consumption and UL coverage especially when the maximum number of CBGs per TB is configured to 8. Therefore, it is necessary to investigate signaling overhead reduction for CBG-based HARQ-ACK feedback, especially for the XR traffic.

Proposal 11: Investigate signaling overhead reduction for CBG-based HARQ-ACK feedback for XR traffic.

### Application awareness and error concealment exploitation

XR application awareness at RAN level is beneficial for identification of ADUs boundaries and ADU importance level. These can be exploited jointly with decoder-based error concealment mechanisms which are embedded into modern video decoders of XR-ready terminals. The latter can reconstruct pictures or picture regions based on available video coded information despite erroneous or missing ADUs (e.g., video slices). With the appropriate information of the application encoding-decoding chain capabilities, the identification of ADUs and their importance, the RAN can support skipping of retransmissions of non-important ADUs (e.g., static/quasi-static P-frames, video coded slices encoding high resolution picture of acknowledged video coded slice containing the low resolution of the same picture) and corresponding packets that would exceed the packet delay budget of the assigned QoS.

Observation: The XR ADU/packet importance value can help the RAN skip retransmissions of non-important ADUs.

# Conclusions

This contribution provided our views regarding XR-specific capacity improvements as follows:

Proposal 1: Study the benefits of indicating the remaining delay budget for a packet and/or a PDU set:

- gNB indicating such delay budget to UE for DL/UL packet and/or PDU set

- UE indicating such delay budget to gNB for UL/DL packet and/or PDU set

Proposal 2: Study latency reduction for HARQ-ACK transmission for multi-PD(U)SCH scheduling.

Proposal 3: Investigate HARQ-NACK prioritization benefits to avoid PDB expiration.

Proposal 4: Study if multi-PD(U)SCH scheduling should be further enhanced based on application awareness.

Proposal 5: Study techniques providing timely PHR, e.g., UL DCI triggering a PHR.

Proposal 6: Study if PHR should be further enhanced based on XR traffic arrival periodicity or UL pose periodicity.

Proposal 7: Study SPS/CG enhancements to address XR traffic variable packet size and arrival time and quasi-synchronous communication of multiple flows. Enhancements may include:

- Enabling, within a SPS/CG period, multiple SPS/CG configurations having the same periodicity with SPS resources of different size and starting time

- Joint activation of multiple SPS/CG configurations for an indicated duration to handle multiple traffics of different QoS requirements in a quasi-synchronous manner with reduced control signaling overhead

Proposal 8: Study the application of NC for XR traffic and its influence on L1/L2 stack.

Proposal 9: Study efficient and backwards-compatible feedback procedures for NC and investigate role of existent HARQ feedback loop.

Proposal 10: Investigate leveraging XR application awareness (e.g., video slice and stream awareness, video slice importance) to map video slices to TB CBGs for optimized transmissions and retransmissions of XR traffic.

Proposal 11: Investigate signaling overhead reduction for CBG-based HARQ-ACK feedback for XR traffic.

Observation: The XR ADU/packet importance value can help the RAN skip retransmissions of non-important ADUs.