3GPP TSG RAN WG1 Meeting #109-e	R1-2204417

e-Meeting, May 9th – 20th 2022

Agenda item:	9.2.2.1

Source: 	Lenovo

Title: 	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, a SID [1] on AI/ML for air-interface was agreed to explore the benefits and potential gains of using AI/ML techniques compared with traditional methods at the air-interface level for a few carefully selected use cases and assess the potential specification impact to enable improved support of AI/ML based algorithms. In this contribution document, we elaborate our views on evaluation methodology and KPI for the use case of AI/ML for CSI feedback enhancement.

# Discussion

## Model Characteristics

Following the discussion in our contribution document on the general framework of AI/ML model [2], we provide a Model Categorization Card (MCC) for each AI/ML proposal that describes how the AI/ML model is categorized with respect to basic characteristics of the AI/ML model. The MCC does not suggest how the proposed AI method actually performs but gives a high-level view of the proposed scheme and its characteristics during the lifecycle.

Multiple schemes have been proposed in the literature for CSI feedback overhead reduction using AI/ML. As one example, we discuss the proposed scheme in [3], i.e., CSINet for reducing feedback overhead. Before going to the evaluation steps, we can complete the MCC to get a high-level understanding of the example CSINet AI model, as follows.

- Table 1 AI Model Characterization Card (MCC) for example CSINet.

AI/ML Proposal; Model Training/ Validation Phase; Model Deployment and Inference Phase; Model Monitoring/Update Phase. Model Training; Learning Paradigm; Data Set; Data Collection; Deployment; Inference Data collection; Model Monitoring; Model update paradigm; Update Data collection. The Name of the proposed scheme; Offline (simulation); Centralized; Simulated data; Only simulated data; In network; Legacy signaling - no spec impact; No monitoring needed; Centralized; No field data needed. Pre-trained (simulation) + fine-tuned (field); Federated; Offline field data; Legacy signaling - no spec impact; In UE; Collaboration b/w a UE & gNB; At corresponding nodes; Federated; Offline field data. Field; Distributed; Online field data; Collaboration b/w a UE & gNB; Split b/w network and UE; Across multiple nodes; At another node; Distributed; Online field data. Across multiple nodes; Initiated periodically.

We again note that CSINet here is just an example to better clarify what we mean on having an MCC for each model. There is also no model update procedure explained in CSINet [3], but as an example we can assume that the model gets updated periodically using some data collected from the environment.

- In order to have a high-level understanding of a CSI-feedback AI/ML model, it is suggested to complete the Model Categorization Card (MCC) for the proposed CSI-feedback scheme.

Below we discuss how the MCC can be completed for a CSI-feedback scheme:

- Model Training/Validation: How we train the model and what are the requirements

  - Model Training:

    - No training phase and the model learns on "live" data

    - Model trained in simulation environment

    - Model needs to be pre-trained in simulation environment, followed by a fine-tuning phase in the real environment (It is likely that most of the proposed schemes probably fall in this category)

    - Training happens in the real environment (e.g., no pre-training is possible in the lab.)

As collection of field data for training of CSI-feedback models is not straightforward, most of the recent studies in literature, e.g. [3] [4] [5], use mathematical models, e.g., UMa, UMi, InH, RMa channel models and train their CSI feedback scheme in simulation environment and argue that the trained model can work in practical environment (not actual implementation yet). These models can be categorized in group (ii) in this feature.

More comprehensive training procedure might first train the model using the simulated CSI and then fine-tune their model using field data after actual deployment in the environment. These models will be categorized in group (iii) in this feature. Note that the training data may be provided via conventional CSI feedback, e.g., Type-I/Type II codebook-based reporting with sparse reporting over time/frequency, or via new framework for training feedback. Further details can be found in our contribution on further issues on AI/ML-based CSI feedback enhancement [6].

  - Training paradigm:

    - Centralized Learning

    - Federated Learning

    - Distributed Learning

Current CSI-feedback schemes usually train a model in UE, in gNB or at a pair of UE/gNB (like autoencoder models [3]) but in all these cases the model is trained centrally, i.e., all training data and computation are at one location. So for most approaches group (i) should be selected.

  - Training Data Set (including validation data set)

    - No need for field data – Training data will be collected through simulation environment

    - Data needs to be collected from the field, but it can be collected offline – offline field data

    - Data needs to be collected from the field and it should be collected during training – online field data

Due to multiple challenges on obtaining/validating field data, most of the studies use mathematical models, e.g., UMa, UMi, InH, RMa channel models for construction of the training data, i.e., group (i). However, it would be beneficial to discuss how we can collect field data that provides good representation of the environment. Such field data can be used to train a model and we can compare the results of that with models trained using simulation data. Field data is also desirable for model validation and can be used to test the generalizability of a model trained using simulated data. The results of this comparison would be very insightful on whether we can rely on simulation-based training or adequate field data is needed.

- Evaluations should start based on data generated using mathematical models. FFS: whether/how to collect field data that provides good representation of the environment.

  - Collaboration during collection of Training/validation Data: Define which nodes are collaborating in preparation of the training/validation data.

    - No Field data needed. Data from simulation environment used.

    - The training/validation data can be collected without specification impact.

    - Collaboration between the UE and gNB is needed.

    - In addition to (i) and/or (ii), training/validation data also depends on information from other UE(s)/gNB(s)

As mentioned previously, most schemes in the literature belong to group (i) i.e., no data collection in the field is assumed and training data generated using simulations. Schemes that require collecting training data in field using legacy NR-based CSI feedback approaches would be categorized in group (ii), whereas schemes with AI-specific training data collection procedure would be categorized in group (iii).

- Model Deployment and Model Inference:

  - Deployment: The model will be deployed:

    - In network, e.g., gNB

    - In UE

    - A part of the model is deployed in UE and another part is deployed in the network

Most schemes in the literature consist of some modules in UE and some modules in gNB so they are categorized in group (iii) (e.g., an autoencoder scheme with an encoder at the UE and the decoder at the gNB). Of course, proposals can be discussed that can be categorized under group (i) or (ii).

  - Collaboration on Inference data preparation: Define what is the input of the AI model and which nodes are involved in construction of the input data.

    - The input data only depends on the data available (e.g., measured) at the node it has been deployed in (e.g., the corresponding gNB, UE, or gNB/UE pair)

    - The input data needs to be exchanged (e.g., signaled) between the UE and gNB.

    - In addition to (i) and/or (ii), the input data also depends on information from other UE(s)/gNB(s)

The inference input data in CSI-feedback is usually the current estimate of the CSI e.g., CSI-RS based channel measurements as currently defined in Rel-17. So, new collaboration is not needed, and we can categorize the scheme in group (i), (for inference data collection). Other schemes in which the model input requires exchanging further data across nodes should be categorized under group (ii) or (iii).

- Model Update: As some of the parameters of the environment changes (e.g., weather, traffic, etc.) we may need to update our model.

  - Model Monitoring: This procedure helps finding out when the model underperforms to initiate a model update procedure within the AI/ML framework

    - No need for model monitoring and update

    - Model monitoring is performed at the corresponding node

    - Model monitoring is performed at another node/entity in the network

    - Model updates are initiated periodically

In case of a change in the statistics of the channel and/or environment settings (compared to what have been used during the training) we might observe degradation in the performance of the CSI-feedback. Detection of such event is probably one of the most challenging parts in the lifecycle of AI/ML model for CSI-feedback. The main difficulty is due to non-availability of the ground-truth data during the inference time so verifying whether the model is working properly is not straightforward.

One possible solution for monitoring of a CSI-feedback model is to periodically use conventional schemes to determine and estimate the CSI and then compare it with the results of the AI/ML model and in case of large deviation the system is triggered to start the model update procedure. Clearly, this method induces some overhead in the network.

- Discuss how to determine the accuracy of the CSI-feedback AI model and criteria to initiate model update procedure, taking into consideration computation and any communication overhead needed.

  - Model update paradigm: Similar to the training phase, some nodes should cooperate for execution of the model update. This can be categorized as:

    - No model update is needed

    - Centralized Learning

    - Federated Learning

    - Distributed Learning

Similar to the discussion under section 1.d (Training paradigm), most of the model update procedures for CSI-feedback can be categorized under group (ii).

  - Data Collections for model update

    - No model update needed

    - Simulation data using updated environment characteristics can be used

    - Offline field data is needed for model update

    - Online field data is needed for model update

It is not likely that only simulated samples can be used for updating of the CSI-feedback model. We instead may need to have samples collected from the field. The data collection procedure for the model update should be well described for each proposal.

- Each proposed CSI-feedback AI/ML model should be accompanied with a data collection procedure which can be used during the model update. FFS: communication overhead and the complexity of the proposed procedure.

## Evaluation Methodology

It is always ideal to evaluate the performance of any AI/ML model proposal in a real environment. Such evaluations, by definition, test the model when the environment changes and provide means to check for the scalability of the proposed scheme when deployed in networks with different settings. Unfortunately, such evaluation is not always feasible and we should use simulation tools and define different scenarios to better model and evaluate the performance in a real environment.

Focusing on CSI-feedback overhead reduction use case, most of the studies in the literature focus on cases with one user equipped with a single antenna and a gNB having multiple antennas. Although this setup may suffice for initial investigation, it does not provide an accurate characterization of real networks with multiple users equipped with multiple antennas. We therefore propose to include at least the following two scenarios during evaluations:

- Setup 1: A UE with multiple antennas and multiple antennas at the gNB. Current known channel models like those in TR38.901 for generation of channel samples can be used with different UE mobility assumptions.

- Setup 2: Two (maybe stationary) UEs each with multiple antennas and a multiple antenna gNB. This can be used to evaluate the performance with MU-MIMO scheduling at the gNB

Clearly, this is a high level description of simulation setups and many details of each setup should be further discussed. For example, for each setup we need to define different antenna array structures, different channel models and speed. Events that result in a change in the channel and environment statistics should also be defined to aid evaluation of adaptability of the model pre/during/after the change events.

- Agree on specifying evaluation details of a few simulation setups including: SU-MIMO with multiple antennas at both UE and gNB; MU-MIMO with 2 UEs and multiple antennas at both UEs and gNB.

- Simulation assumptions should include definition of event(s) modelling changes in network settings or channel statistics during the simulation for evaluating AI Model adaptiveness to new conditions.

## KPIs and metrics for evaluation of 	`CSI-Feedback

In this part we follow the general KPI and evaluation discussion in [2], and provide additional views on KPI/metrics for the CSI-feedback use case.

- Model Characteristics

  - Inference Accuracy:

Several metrics can be used for evaluation of a proposed CSI-feedback scheme. It mainly depends on the objective of the proposed CSI feedback procedure.

- CSI Reconstruction error: many previous studies consider the MSE between the actual channel realization and the channel estimate that has been reconstructed at the gNB (after feedback). Although it is a valid metric and could be reported in the analysis, having the complete channel knowledge is not the primary goal of the CSI feedback.

- Similarity between the  selected precoder and the optimal precoder: Another possible metric is to evaluate the similarity between the precoder that maximizes the transmission rate/MCS and the precoder based on the CSI feedback (e.g., cosine similarity). This is potentially a good metric but we note that finding the "optimal" precoder for some scenarios (specially the multiuser cases) is not a straight forward task. Additionally, since there may not be a global optimum for the precoder design, two distinct precoders may provide a similar performance. Hence, this metric may be misleading in some special cases.

- Achievable rate/throughput: A more realistic metric is the achievable downlink rate/throughput that the gNB can achieve after getting the feedback from the UE. In other words, the feedback should be such that the gNB can find a precoder that maximizes the achievable downlink rate (regardless of the fact whether the full channel information is available at the gNB).

- It is preferred to select Achievable rate/throughput for evaluation of different CSI feedback proposals. In cases that the optimal precoder can be determined, similarity to optimal precoder can be used as the evaluation metric as well.

  - Average model update rate:

Some CSI feedback methods might be trained under certain conditions and then after changes in the environment the model may need to be retrained. Based on the training procedure and all details of the model, it is necessary to report how often the model needs to go through the model updated procedure. For example, if we train CSINet with UMa channel model, do we need to retrain it with other channel statistics and if yes, how often is retaining needed. Note that more frequent model update may result in large communication overhead and delays in having valid CSI-feedback, and on the other hand les frequent model update may lead to performance degradation due to the outdated training model. We acknowledge that agreeing on a value of the rate for the model update might be a bit complicated, but it is essential for evaluation between different cases.

- It is essential to quantify how often the model has to go through the model update procedure.

- Specification Impact

In our contribution on further aspects of AI/ML for CSI feedback [6], we discuss the potential sub use-cases under CSI feedback, including CSI feedback overhead reduction and CSI prediction. Steps of implementing the AI/ML model from an NR-based signaling perspective are explained. Additionally, a brief discussion on potential specification impact corresponding to different AI/ML model assumptions is provided. Further details can be found in [6].

- Time required for training/Convergence time

  - Model update:

If the proposed methods are not universal and need to be retrained based on some changes in the environment, discussion is needed on the approximate time required to update the model. For computation of the training time, a common measure (e.g., FLOPs, MACs) can be adopted so that the reported results are comparable.

- The amount of time required for the model to have valid CSI-feedback after initiation of the model update stage should be evaluated.

- Communication Overhead

  - Model transfer:

How many bits are needed to transfer the model and the weights for model-transfer based AI/ML models.

  - Inference:

How many feedback bits should be transmitted to the gNB to determine the CSI? This corresponds to the feedback overhead of the CSI-feedback method.

  - Model update:

How many extra bits should be transmitted between the UE and gNB to construct the required training data for the updating the CSI-feedback model. Note that the dataset for model update should also have the (almost) ground-truth version of the desired output.

- Communication Latency/Delay Requirement

  - Model update

How long should the system collect data that is needed for updating the CSI-feedback model.

- Computational Complexity

  - Inference

The inference computation complexity of the CSI-feedback model at both UE and gNB (if it exist at both ends). As it is a feedforward path of the model, it is usually not expected to be large.

  - Model update

The computation complexity of updating the CSI-feedback model. Updating process usually needs higher computational requirement. It might even need to be executed on a GPU for faster convergence in real-time environments. Such node capabilities (for UEs and CPEs) can be signaled to the gNB as part of capability signaling.

- Robustness

  - Sensitivity:

Sensitivity of the model to errors in the data (input data and, in case of online training, training data) and the latency in data transfer over the network, if applicable.

Accessibility to the exact channel realization in real environment is not practical. If the model uses such information as the grand truth of the CSI, it should mention the level of acceptable noise in the measured channel realization. Alternatively, one can report the accuracy of the model for a range of noisy training data. Also the model should report the inference accuracy considering the range of noisy input data.

- The inference accuracy should be reported for a range of noisy training CSI and also noisy input data for the proposed CSI-feedback mechanism.

  - Adaptability:

Whether the model adapts to shifts in data distribution which may occur due to dynamically changing network scenarios (channel statistics, traffic patterns, etc.).

To check how adaptable the model is, we assume a model is trained and then evaluate its performance on different channel statistics without updating the model for the new settings.

- The inference accuracy of the model (without model updates) should be reported for different channel models including UMa, UMi, CDL (with various parameters).

  - Scalability:

Does the model scale if the network parameters are changed?

Proposals should be clear on whether it is needed to train separate models for e.g., different number of antennas ports available at the UE/gNB, subcarrier spacing, or channel bandwidth.

- Evaluate the scalability of the model for different parameter settings, e.g., different number of antenna ports available at the UE or gNBs, subcarrier spacing, or channel bandwidth.

# Conclusions

This contribution addressed AI/ML-based CSI feedback enhancements. We have the following observations and proposals:

Proposal 1	In order to have a high-level understanding of a CSI-feedback AI/ML model, it is suggested to complete the Model Categorization Card (MCC) for the proposed CSI-feedback scheme.

Proposal 2	Evaluations should start based on data generated using mathematical models. FFS: whether/how to collect field data that provides good representation of the environment.

Proposal 3	Discuss how to determine the accuracy of the CSI-feedback AI model and criteria to initiate model update procedure, taking into consideration computation and any communication overhead needed.

Proposal 4	Each proposed CSI-feedback AI/ML model should be accompanied with a data collection procedure which can be used during the model update. FFS: communication overhead and the complexity of the proposed procedure.

Proposal 5	Agree on specifying evaluation details of a few simulation setups including: SU-MIMO with multiple antennas at both UE and gNB; MU-MIMO with 2 UEs and multiple antennas at both UEs and gNB.

Proposal 6	Simulation assumptions should include definition of event(s) modelling changes in network settings or channel statistics during the simulation for evaluating AI Model adaptiveness to new conditions.

Proposal 7	It is preferred to select Achievable rate/throughput for evaluation of different CSI feedback proposals. In cases that the optimal precoder can be determined, similarity to optimal precoder can be used as the evaluation metric as well.

Proposal 8	It is essential to quantify how often the model has to go through the model update procedure.

Proposal 9	The amount of time required for the model to have valid CSI-feedback after initiation of the model update stage should be evaluated.

Proposal 10	The inference accuracy should be reported for a range of noisy training CSI and also noisy input data for the proposed CSI-feedback mechanism.

Proposal 11	The inference accuracy of the model (without model updates) should be reported for different channel models including UMa, UMi, CDL (with various parameters).

Proposal 12	Evaluate the scalability of the model for different parameter settings, e.g., different number of antenna ports available at the UE or gNBs, subcarrier spacing, or channel bandwidth.

# References

| [1] | RP-213599, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," RAN#94e, Dec 2021. |
| --- | --- |
| [2] | R1-2204416, "General aspects of AI/ML framework," Lenovo, RAN1#109-e, May 2022. |
| [3] | Wen, C.K., Shih, W.T. and Jin, S.,, "Deep learning for massive MIMO CSI feedback," IEEE Wireless Communications Letters,, vol. 7, no. 5, pp. 748-751, 2018. |
| [4] | Lu, Z., Zhang, X., He, H., Wang, J. and Song, J., "Binarized aggregated network with quantization: Flexible deep learning deployment for CSI feedback in massive MIMO system," IEEE Transactions on Wireless Communications., 2022. |
| [5] | Madadi, P., Jeon, J., Cho, J., Lo, C., Lee, J. and Zhang, J.,, "PolarDenseNet: A Deep Learning Model for CSI Feedback in MIMO Systems.," arXiv preprint arXiv:2202.01246., 2022. |
| [6] | R1-2204418, "Further aspects of AI/ML for CSI feedback," Lenovo, RAN1#109-e, 2022. |
| [7] | R1-2204417, "Evaluation on AI/ML for CSI feedback enhancement," Lenovo, RAN1#109-e, May 2022. |