3GPP TSG RAN WG1 #109-e	R1-2204418

e-Meeting, May 9th – 20th, 2022

Source:	Lenovo

Title:	Further aspects of AI/ML for CSI feedback

Agenda Item:	9.2.2.2

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, a SID [1] was agreed that aims at having an in-depth study of supporting AI/ML-based frameworks for enhanced performance and/or reduced complexity/overhead for RAN. Three use cases were agreed to be studied, so as to provide a variety of case studies for methods of implementing AI/ML-based algorithms for NR air interface, as well as the potential gains and challenges accompanied with these algorithms.

In RAN#94-e [1], the following agreements were made

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

# Outline of AI-based CSI feedback framework

In our contribution on evaluation of AI/ML CSI on CSI feedback enhancement [2], an elaborate discussion on the different aspects of the AI/ML model characteristics is provided, including the model training/validation, model deployment/inference, model update, as well as KPIs and evaluation methodology. In this contribution, we focus on a few characteristics that are expected to have significant specification impact, including the network-UE collaboration levels, as well as the AI model training node.

## Network-UE collaboration levels

Taking the potential impacts on the specifications of air interface into account, it is necessary to consider different collaboration levels to support AI/ML operations between gNB and UE, that is, the type of information and processing required to be interacted to satisfy arbitrary AI/ML operations, which is also one of the key issues to select the sub use cases to be studied in this study.

We’d like to classify the categories as listed in Table 1 as described in [3], according to the collaboration operations either for model management or for model inference.

Table 1 Proposed categories of Network-UE collaboration levels

| Category | Collaboration for the model management | Collaboration for the model inference | Comments |
| --- | --- | --- | --- |
| Cat.1 | × | × | One node handles the CSI training and inference. No specification impact is expected |
| Cat.1a | × | × | One node handles the CSI training and inference, however signaling is needed across both nodes to communicate a few non-AI parameters |
| Cat.2 | ü | × | Signaling between both nodes to communicate AI model management related parameters |
| Cat.3 | ü | ü | Signaling between both nodes to communicate AI model management related parameters, as well as AI-based inference of the measured CSI, e.g., AI model transform for auto-encoder architectures |

Study AI models and associated network-UE collaboration levels for AI-based CSI feedback framework

## AI model training node

In the sequel, we focus on AI-based CSI frameworks with centralized AI model training, i.e., the model training is performed at one node. As described in our contribution for AI-based CSI evaluation methodology [2], different assumptions on the model training exist, including simulation-based model training, or real-time model training based on channel measurement/reporting over the air interface. In this section, we focus on real-time model training, since simulation-based model training can be largely handled with marginal specification impact and can be used during performance evaluation. In light of that, two alternatives are discussed/compared that differ in the node handling the AI-based model training, as follows

- The AI model is trained at the UE node. This alternative may appear reasonable since the UE would possess training data based on the received CSI-RS symbols for using legacy channel measurement framework. However, the memory and computational complexity requirements for this operation would be significant, since a new AI model should be re-trained whenever the environment changes, e.g., change of the UE location/orientation

- The AI model is trained at the network. One advantage of this approach is that the network is expected to possess significantly more power and computational capabilities compared with a UE, and hence can manage training the AI model, as well as store large amounts of training data. Moreover, since most network nodes, e.g., gNB, are assumed to be fixed, the environment with respect to the network node is expected to be static (from a perspective of a network node with a fixed location, orientation and coverage area), and hence a similar AI model can be applicable to UEs within a specific region/area of the cell for a reasonable period of time. The one challenge with this approach is related to obtaining the training data at the network node, especially for FDD systems in which the UL/DL channel reciprocity may not hold. Note that under this setup, the overhead corresponding to feeding back the training data from the UE to the network should be considered as one of the metrics when assessing the efficiency of an AI/ML model. Moreover, if a model-transfer based approach is adopted for CSI feedback, e.g., auto-encoder scheme, the signaling overhead of the model should also be considered.

Note that for AI-based auto-encoding/decoding schemes, although the auto-encoding and auto-decoding processes are pursued at different nodes, the AI model training is usually expected to be done centrally in either the network or UE, which communicates the auto-encoder (or auto-decoder) parameters to the other node. Hence, the categorization above for the AI model training node still applies

Study the advantages/disadvantages of network-based AI model training vs. UE-based AI model training

# Use Cases for AI/ML-based CSI feedback enhancements

As described in the SID [1], the potential objectives illustrated for CSI enhancements are overhead reduction, improved accuracy, and prediction. Since these objectives can be studied from many aspects, it is necessary to clarify and align the aspects that would facilitate the further discussion.

We start with a simplified CSI feedback procedure between a serving gNB and UE as illustrated in Figure 1, as follows.

Figure 1. A simplified CSI feedback procedure

- Step 1): The serving gNB transmits CSI-RSs based on a CSI resource configuration.

In this step, the CSI-RS overhead can be potentially reduced via AI/ML approaches, e.g., learning the propagation environment to configure the proper resources.

- Step 2): The UE measures the CSI based on the received CSI-RSs and generates a CSI report based on a received CSI reporting configuration.

In this step, the accuracy can be potentially increased via AI/ML approaches, e.g., learning the channel’s statistical values to enhance channel estimation.

- Step 3): The UE feeds back the CSI report in PUSCH/PUCCH according to the CSI reporting configuration.

In this step, the overhead corresponding to CSI reporting feedback, especially the PMI, can be potentially reduced via AI/ML approaches, e.g., using an autoencoder to compress the frequency/spatial parameters of the channel.

- Step 4): The serving gNB recovers the frequency/spatial parameters of the channel matrices from the received PMI.

In this step, the accuracy of the recovered channel matrices can be potentially increased via AI/ML approaches, e.g., using an NN to recover the precoding matrices from received PMI, via compressed sensing.

- Step 5): The serving gNB precodes the PDSCH data based on the recovered channel matrices.

In this step, the CSI for downlink transmission (e.g., precoding generation) can be predicted via AI/ML approaches, e.g., learning the precoder for future slots based on CSI feedback in prior slots.

## CSI feedback overhead reduction/resolution enhancement

In NR, CSI reporting (as illustrated in Step 2) includes multiple quantities, e.g., CRI, RI, PMI, LI, CQI, L1-RSRP and L1-SINR. Different methods of reporting each indicator exist based on the configured report format, or the configured codebook type corresponding to PMI. Up to Rel. 17, two codebook types are supported, Type-I codebook (including 2 sub-types), wherein the precoder is based on compression of the spatial-domain dimensions of the precoder based on a 2D DFT compression, and Type-II codebook (including 5 sub-types), wherein the precoder is based on compression of both the spatial-domain dimensions and the frequency-domain dimensions of the precoder based on a second DFT compression. For each precoding matrix types, fixed amplitude and phase quantization schemes are assumed.

Based on work in the previous NR releases to specify new PMI codebooks and CSI feedback parameters, the performance of different approaches has been assessed based on the tradeoff between throughput and CSI feedback overhead. Adopting such joint objective (Increasing throughput and reducing CSI feedback overhead) provides more insight compared with comparing the throughput and overhead of each scheme separately, since the throughput-overhead tradeoff curve reveals the throughput gain (loss) corresponding to adding (removing) CSI feedback bits of a given scheme. In light of that, we prefer to merge CSI feedback overhead reduction and CSI throughput enhancement into one sub use-case.

CSI feedback overhead reduction and CSI throughput enhancement objectives are not to be treated in isolation, but into one sub use-case of CSI feedback enhancement

For the CSI feedback overhead reduction/resolution enhancement sub use-case, the AI model may correspond to one or both of the following

- Exploiting frequency-domain channel correlation: AI model is trained to infer CSI corresponding to M sub-bands based on CSI feedback corresponding to M’ frequency-domain indices, where M’< M. Note that both PMI and CQI report quantities can be considered for such frequency-domain inference.

- Exploiting spatial-domain channel correlation: AI model is trained to infer CSI, e.g., PMI, corresponding to K antenna ports based on CSI feedback corresponding to K’ spatial-domain indices, where K’< K.

For this sub use-case, Rel. 16 Type-II codebook should be considered as a baseline for comparison with AI-based CSI feedback reduction/resolution enhancement approach, since Rel. 16 Type-II codebook supports both spatial and frequency domain compression via DFT process.

Rel. 16 Type-II codebook is the baseline for AI-based CSI feedback overhead reduction and/or resolution enhancement in terms of throughput-overhead tradeoff

## CSI prediction

One sub use-case in which AI can be useful is in terms of CSI prediction (as illustrated in Step 5), e.g., estimating a subset of CSI parameters corresponding to time slot t, based on CSI measured at time slot t+t0. Such use case would be beneficial for scenarios where UEs are moving with moderate/high speed, in which conventional CSI measurement/feedback approaches may be inefficient, e.g., in highways, high-speed trains. Note that PMI, RI, and CQI report quantities can be considered as candidates for CSI report quantities that can be predicted using AI. We would also like to note that an agenda item that aims at studying CSI reporting enhancements for moderate/high speeds is being discussed as part of the MIMO agenda for Rel. 18 [4]. In our opinion, this agenda item can be very helpful for the AI-based CSI prediction study, as multiple non-AI schemes are expected to be proposed and compared in the study phase of that agenda item, and hence can provide a good baseline for this use case. In light of that, our preference is to defer the discussion on this sub use-case to RAN1#110bis-e meeting, until more insight from the study phase of CSI enhancement for high speed users is available.

Defer the discussion on AI-based CSI prediction sub use-case to RAN1#110bis-e

## CSI resource configuration corresponding to AI-based CSI feedback

One fundamental aspect of CSI reporting framework is the CSI resource configuration and transmission therein (as illustrated in Step 1). In RAN#94-e, no consensus was reached on supporting AI-based CSI-RS transmission enhancement to reduce CSI-RS transmission complexity/overhead and/or performance improvement. However, considering the CSI-RS overhead when comparing AI-based CSI frameworks is needed for a better characterization of the overall gain/cost of supporting a given AI model for CSI feedback. For instance, an AI model for CSI feedback that achieves a given throughput-overhead tradeoff point with a smaller frequency density and/or larger periodicity of the transmitted CSI-RSs should be favored over another AI model for CSI feedback that achieves a similar throughput-overhead tradeoff point with a larger frequency density and/or smaller periodicity of the transmitted CSI-RSs. The means of introducing the CSI-RS overhead to the performance metric should be considered for further study.

Include CSI-RS overhead as a performance metric upon comparing different CSI feedback frameworks under the AI-based CSI feedback framework study

# Specification impact for AI-based CSI feedback enhancements

In this section, we discuss potential specification impact corresponding to AI/ML-based CSI feedback enhancement. Mainly, we discuss potential specification impact on capability signaling, AI-based CSI reporting configuration, and AI-based CSI feedback. Note that the specification impact would heavily depend on the network-UE collaboration level, as described in Section 2, as well as the use case, as described in Section 3. In the sequel, we provide a high-level overview of potential specification impact, as follows.

## Capability signaling corresponding to AI-based CSI framework

In general, for AI operation over the air interface, the UE may be involved in AI model training, inference, or any signaling corresponding to AI model, based on the collaboration level and the use case. Obviously, not all UEs are expected to be capable of handling such signaling/computation, and hence AI-related signaling and/or computation is expected to be optional. Hence, UE capability signaling is needed for Cat.1a, Cat.2, Cat.3 collaboration levels, to indicate whether the UE can handle such features. A few examples are provided below for signaling corresponding to UE capability of supporting the following:

- AI model training at UE (if UE-based AI model training is specified), or more generally indication of the complexity/computational capability of the AI model that can be trained at the UE

- AI model inference (if UE-based AI model inference is specified), or more generally indication of the complexity/computational capability of the AI inference process at the UE

- Training data collection and transmission to network for AI model training (if real-time training data corresponding to AI model is configured to be fed back to the network side)

Study capability signaling enhancement corresponding to AI-based CSI feedback under different network-UE collaboration levels

## CSI reporting configuration

In NR, a UE configured with feeding back a CSI report to the network is expected to receive a CSI reporting configuration (as illustrated in Step 1 of Figure 1) that identifies the CSI-RS resource(s) corresponding to channel/interference measurement, the report quantities that are expected to be measured by the UE based on the configured CSI-RS resource(s), the format(s)/codebook type corresponding to each of the configured reporting quantities, as well as the time-domain behavior of the CSI report(s) that are expected to be fed back by the UE. For AI/ML-based CSI reporting, enhancements to the CSI reporting configuration are needed for Cat.1a, Cat.2, Cat.3 collaboration levels. Examples of such enhancements are as follows,

- Whether a distinct reporting configuration is specified for AI/ML-based CSI feedback, e.g., AI-based CSI reporting configuration. This would depend on the collaboration level as well as the use case. For instance, the more explicit the network-UE collaboration is, e.g., Cat.3 with AI-based auto-encoder framework supported, the more likely that a standalone AI-based CSI reporting configuration is needed.

- Introducing CSI reporting configuration parameters corresponding to training data feedback by the UE (in case real training data feedback of the CSI is supported). Note that such data may not be equivalent to PMI reported via legacy codebook types, i.e., the network may configure the UE with feeding back the training data to the network in a format that is distinct from that of PMI, e.g., a new report quantity may be introduced for training data. This may be applicable under Cat.2, Cat.3

- Introducing CSI reporting configuration parameters corresponding to AI model parameters in case of AI model transfer, e.g., details of the AI-based auto-encoder in case the model training is applied at the UE, parameters corresponding to an NN model, e.g., number of nodes, hidden layers, activation function, etc. This may be applicable under Cat.3

- Introducing CSI reporting configuration parameters corresponding to performance metric measurement and feedback for AI model parameters, e.g., reporting CQI corresponding to CSI-RS resources that are beamformed via AI model at the network, compared with the CQI corresponding to CSI-RS resources that are non-beamformed, or beamformed via conventional/legacy approaches. This may be applicable under Cat.1a, Cat.2, or Cat.3

Study CSI reporting configuration enhancement for AI-based CSI feedback under different network-UE collaboration levels

## CSI report

For AI/ML-based CSI feedback, especially under Cat.2, Cat.3, the fields of a CSI feedback report are expected to be different compared with a conventional NR-based CSI report. Examples of such potential discrepancies are as follows,

- Whether feedback corresponding to AI-based CSI feedback would be classified as a CSI report, or a different report type, e.g., AI-based CSI report. The latter alternative may be considered in case a standalone AI-based CSI reporting configuration is specified, e.g., under Cat.3

- Introducing a new codebook type corresponding to the AI-based CSI feedback report comprising PMI information, e.g., a Type-III codebook

- For a case wherein the UE is configured to feed back real training data of the CSI to the network, whether a CSI report includes CSI parameters corresponds to both training data and legacy PMI information

- Introducing new CSI fields in the CSI report, as configured in the CSI reporting configuration, e.g., AI-based auto-encoder/NN parameters. This may be applicable under Cat.3

- Number of CSI report parts corresponding to AI-based CSI feedback, and the mapping order of CSI fields therein

- Signaling a computational complexity metric, e.g., number of CPUs, that quantifies measurements and/or computations corresponding to an AI-based CSI report, as well as the number of AI-based CSI reports that can be computed by the UE simultaneously across one (or all) CCs

- Signaling for sharing/acquiring information needed for the training phase, e.g., an indication of the measured CSI parameter values to enable training/updating the weights of CSI Auto-encoder.

Study CSI reporting content enhancement for AI-based CSI feedback under different network-UE collaboration levels

# Conclusion

This contribution addressed AI/ML-based CSI feedback enhancements. We have the following proposals:

Proposal 1	Study AI models and associated network-UE collaboration levels for AI-based CSI feedback framework

Proposal 2	Study the advantages/disadvantages of network-based AI model training vs. UE-based AI model training

Proposal 3	CSI feedback overhead reduction and CSI throughput enhancement objectives are not to be treated in isolation, but into one sub use-case of CSI feedback enhancement

Proposal 4	Rel. 16 Type-II codebook is the baseline for AI-based CSI feedback overhead reduction and/or resolution enhancement in terms of throughput-overhead tradeoff

Proposal 5	Defer the discussion on AI-based CSI prediction sub use-case to RAN1#110bis-e

Proposal 6	Include CSI-RS overhead as a performance metric upon comparing different CSI feedback frameworks under the AI-based CSI feedback framework study

Proposal 7	Study capability signaling enhancement corresponding to AI-based CSI feedback under different network-UE collaboration levels

Proposal 8	Study CSI reporting configuration enhancement for AI-based CSI feedback under different network-UE collaboration levels

Proposal 9	Study CSI reporting content enhancement for AI-based CSI feedback under different network-UE collaboration levels

# References

| [1] | RP-213599, Qualcomm, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface," RAN#94-e, Dec. 6-17, 2021. |
| --- | --- |
| [2] | R1-2204417, Lenovo, "Evaluation on AI/ML for CSI feedback enhancement," RAN1#109-e, May 9-20, 2022. |
| [3] | R1-2204416, Lenovo, "General aspects of AI/ML framework," RAN1#109-e, May 9-20, 2022. |