3GPP TSG RAN WG1 Meeting #109-e	R1-2204419

e-Meeting, May 9th – 20th 2022

Agenda item:	9.2.3.1

Source: 	Lenovo

Title: 	Evaluation on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In [1], a SI on AI/ML for air-interface was agreed to explore the benefits and potential gains of using AI/ML techniques compared with traditional methods at the air-interface level for a few carefully selected use cases and assess the potential specification impact to enable improved support of AI/ML based algorithms. As pointed in the approved SID [1], beam management is one of the three important use cases to exploit AI/ML for PHY:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we provide our views on evaluation AI/ML for beam management, focusing on the evaluation methodology and KPIs.

# Model Categorization

Before proceeding onto the evaluation methodology and the KPIs, it would be informative to have high-level details on the key aspects of a proposed AI/ML method. As discussed in the document on “General framework of AI/ML model” [2], we recommend providing such details in the form of a table, which we refer to as the “Model Characetrization Card” (MCC). Note that MCC would not provide details about how the AI/ML method works, but mentions different aspects of it over the entire life cycle of any AI/ML model.

A multitude of schemes based on AI/ML techniques have been proposed in the recent past for the beam management. For the purpose of illustration, we hereby include the MCC for a recent method “Deep Active Learning for Initial Alignment”, proposed in [3]. Note that, as this method does not involve any model update phase, we have shaded the columns in gray corresponding to Model Update.

Table 1 AI/ML Model Characterization Card for “Deep Active Learning for Initial Alignment” [3]

(Added here only as an illustrative example)

| AI/ML Proposal | Model Training/ Validation Phase |  |  |  | Model Deployment and Inference Phase |  | Model Monitoring/Update Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Model Training | Learning Paradigm | Data Set | Data Collection | Deployment | Inference Data collection | Model Monitoring | Model update paradigm | Update Data collection |
| The Name of the proposed scheme | Offline (simulation) | Centralized | Simulated data | Only simulated data | In network | Legacy signaling - no spec impact | No monitoring needed | Centralized | No field data needed |
|  | Pre-trained (simulation) + fine-tuned (field) | Federated | Offline field data | Legacy signaling - no spec impact | In UE | Collaboration b/w a UE & gNB | At corresponding nodes | Federated | Offline field data |
|  | Field | Distributed | Online field data | Collaboration b/w a UE & gNB | Split b/w network and UE | Across multiple nodes | At another node | Distributed | Online field data |
|  |  |  |  | Across multiple nodes |  |  | Initiated periodically |  |  |

# Evaluation Methodology

## Key modeling considerations

Some of the key points to be considered/discussed while deciding the evaluation methodology for proposed AI/ML Models for Beam Management.

- UE Mobility and Rotation Models

  - Models for UE mobility and rotation/orientation (i.e., to capture the changes in UE orientation and rotations) that are to be used

  - Range of values for each parameter in the UE mobility and UE rotation/orientation models

Note: The discussion in additional modeling components section of TR38.901 could serve as a starting point with regard to the UE mobility and rotation models.

- Channel Models: Spatially consistent channel models that provide channel evolution (or, gradual changes in the channel) as the UE moves along a trajectory need to be considered. It is to be discussed and decided whether the details on ‘Spatial Consistency’ available in TR38.901 can provide the necessary guidance here. Further, alternative methods that are simple to employ but effectively mimic the spatialy consistency may also be discussed.

- Different UE Speeds: Beam management should be evaluated for different UE speeds. For example, in case UE travelling by high speed trains, or at a high speed on a highway, predictive beam management could be more effective. Thus, it would be good to consider different UE speeds separately in evaluating an AI/ML method.

- Single-beam and multi-beam Scenarios: A gNB can employ either single beam or multiple beams based on whether it has a single antenna panel or multiple antenna panels with an RF chain associated with each panel. In our view, evaluations of an AI/ML method for beam management should consider both single- and multi-beam/antenna-panels scenarios. The proposed AI/ML method should be evaluated in both scenarios to evaluate its robustness and adaptability to different deployment scenarios.

The beam management AI/ML model evaluation methodology should support evalutations with realistic modeling of UE mobility and rotation, spatially consistent channel models, different UE speeds, and single-beam/panel and multi-beam/panel scenarios for evaluating a model's robustness and adaptability.

## Dataset for Evaluation

In general, for the system level simulation, the drop-based approach randomly generates the parameters for a channel realization for each UE placement in an assumed scenario based on a statistical model in TR38.901. To support and reflect the real propagation environment, e.g., similar channel realizations at nearby locations,  several options to maintain spatial consistency throughout the simulation area can be adopted, which can in some degree simulate the smooth transition of channels during UE motion. We suggest using TR38.901 as a starting point for the spatially consistent mobility simulations.

Generate the dataset for the model training and evaluation for beam management sub use cases based on the statistical models in TR 38.901 taking in to account spatially consistency.

# KPIs and Performance Metrics

In this section, we list the metrics and KPIs/Performance Metrics that an AI/ML approach should be evaluated against as per the agreed/accepted evaluation methodology in the agreed scenario. We propose that the the set of KPIs/Performance Metrics that need to considered in evaluating an AI/ML model for beam management should include the following.

- Accuracy and Effectiveness

  - Average error probability/success rate in identifying the correct beam alignment (based on L1-RSRP or L1-SINR, depending on the particular scenario)

    - This can be computed as follows: Out of “N” instances of beam alignment, how many times the proposed AI/ML method failed to choose the correct beam. For a reliable estimate of this error probability, this need to be computed for a large value of “N” (~1000 or above). The correct/genie-determined beam, for comparison, could be obtained from the ground truth (which is possible in simulated scenarios). Or, the beam identified by the state-of-the-art Rel-15/16 based schemes may be used as the correct beam for comparison.

  - Effectiveness of the beam alignment in terms of L1-RSRP or L1-SINR.

    - The difference between the L1-RSRP/L1-SINR value of the beam-pair determined by the AI/ML model and that of the genie-determined beam-pair can be used as a measure of effectiveness.

    - Effectiveness is considered as a metric due to the following reason: While probability of identifying the correct beam-pair is an important metric, it’s affect on the link performance is equally important. This is because, sometimes determining the beam adjacent to the correct beam as the best beam may not cause any considerable loss in the SNR/SINR of the link or may not degrade the link throughput (Ex: when the UE Rx beam is wide, multiple gNB Tx beams may result in almost similar performance). On the other hand, in some situations, even a small error in beam alignment may cause significant loss in the link quality (Ex: when both the Tx and Rx beams are very narrow).

  - Beam Failure Rate: To evaluate the effectiveness of the algorithm's beam tracking ability.

    - This is a useful metric especially for beam prediction sub use case. If the AI/ML model's beam tracking is not good, there would be more instances of beam failure.

    - This can be computed as the number of beam failure events in a time period and compared with state-of-the-art Rel-15/16 based schemes with the same configuration, e.g., the same radio link quality threshold for beam failure detection.

All the above KPIs need to be reported across a range of SNR/SINR values covering low, moderate and high SNR/SINR regimes. Further, the KPIs may be chosen appropriately depending on the scenario being considered. For example, if we are evaluating the multi-beam scenario (for example, with simultaneously active multiple panels at the gNB), SINR and sum-rate/network throughput are more appropriate measures. If we are focusing on single UE scenario, then signal strength and single UE throughput are appropriate metrics.

KPIs for the AI/ML beam management performance evaluation should include beam alignment success rate, effectiveness (in terms of improvement in L1-RSRP or L1-SINR relative to legacy schemes), and beam failure rate.

- Beam management Signaling overhead

Signaling overhead incurred by the AI/ML method, which may include the following, need to be considered:

- Number of reference signals (e.g., CSI-RS, SRS, SS blocks etc.) configured for a beam management procedure.

- Number of measurement reports and the corresponding report content.

- Any other signals that need to be exchanged between UE and gNB to support the AI/ML model, such as signaling in another carrier (e.g., FR1), UE location information, spatial features of the environment etc.

Further, the signaling overhead incurred during every phase in the lifecycle of the AI/ML model (training, validation, inference, model update) should be reported.

We suggest using the state-of-the-art Rel-17 based schemes as the baseline.

Beam management signaling (e.g., RS, measurement reports) overhead required for a proposed AI/ML model (at every phase in the model lifecycle) should be evaluated. This can be reported relative to state-of-the-art Rel-17 based baseline schemes.

- Latency

Time required for each phase of the beam management, as applicable to the method being considered. It should include the time required for selecting the Tx-Rx beam pair and include the following.

- Tx beam training: The average time required for the UE to acquire/select a gNB Tx beam with a measured L1-RSRP/L1-SINR larger than a threshold among a set of configured Tx beams.

- Rx beam training: The average time required for the UE to acquire/select a Rx beam with measurements on the same gNB Tx beam and measured L1-RSRP/L1-SINR larger than a threshold among a set of Rx beams.

Beam acquisition/selection latency including Tx/Rx beam training should be evaluated for a proposed AI/ML model.

- Computational complexity

Computational complexity and/or the power consumption of the beam managment AI/ML model is a metric that captures the cost incurred by the method. The computational complexity and power consumption are tightly coupled and an important consideration for a UE. (In some cases, power consumption could be a more useful/appealing metric; for example, in the case of UE).

The complexity/power consumption should be considered for

- Model inference

- Model update/retraining, if applicable. In case of model update/retraining, the frequency of update/retraining also need to be accounted for.

- Model training complexity should be considered only if the training happens on-site in an online fashion, at the expense of network resources

It would be useful/insightful to consider both the average and the worst case values for the complexity/power consumption.

Computation complexity of the proposed AI/ML model should be evaluated at every phase in the model lifecycle - training, inference, update.

- Time required for training/Convergence time

  - Model Training

    - For example, in case of beam selection/prediction AI/ML model based on reinforcement learning (RL with Q-learning) with online training, we may have to wait for some time before the training converges to a certain state before using the AI/ML model for inference. We may not use the model for inference during this period and we have to fall back to non-AI/ML methods during this period. Note that training time need not be considered if the model is trained offline (e.g., in the lab) before it is deployed in the field.

  - Model update/re-training

    - If the model is not adaptable to changing network scenarios/environment and/or a change in the statistical properties of the data (e.g., shift in the data distribution), it need to be updated or re-trained from time to time and we may not be able to use the model for inference during such update sessions.

Note:  The training time required in the lab before model deployment does not need to be accounted. Some of the AI/ML methods for beam management may not require dedicated training time and this metric need not be reported for such methods.

A proposed AI/ML model that is based on online learning (e.g., online training, model update/re-training) should evaluate the convergence time for the model to reach a valid inference state.

- Measurements from Sensor(s) and Hardware support

- Does the AI/ML method requires side information (e.g., measurements from non-3GPP technologies). For example, does the proposed beam management AI/ML method require measurements from additional sensors like GPS, LIDAR, Camera, or additional hardware like memory storage, GPU execution?

  - UE

  - gNB

- For example, an AI/ML method may require the gNB to have powerful cameras (or equipping a vehicular UE with a LIDAR) so that the BM can be done based on the signals from these sensors. Such an AI/ML method can result in an additional cost and support for sensors, which need to be taken into account while comparing it with other methods.

Any additional side information (e.g., measurements from non-3GPP technologies) needed for a proposed beam management AI/ML model should be stated.

- Robustness

- Sensitivity of the beam management AI/ML model to

  - Errors in the data (e.g., erroneous measurements exchanged between UE and gNB)

  - Latency (e.g., latency in generating and reporting the measurement reports)

- Importance of this KPI may be described by considering the following scenario: Two methods might perform equally well under favorable conditions (such as, highly reliable and low latency reporting of L1-RSRP/L1-SINR measurements over the network). However, performance of one of the methods may degrade sustantially with errors in the RSRP/SINR measurements or with an increased latency in the availability of measurement reports at the inference node. It is recommended to evaluate robustness by considering a range of signaling error probabilities and latency values in the low, moderate and high SNR/SINR regimes.

Inference accuracy sensitivity of a proposed beam management AI/ML model should be evaluated for a range of noisy input data and delay constraints.

- Scalability

Does the model scales well if the network parameters change? Some of the relevant parameters to be considered here for beam management use case are the following (Please note that the list below is only indicative but not exhaustive):

  - Number of beams at gNB and UE

  - Number of active UEs in a multi-beam scenario (when the gNB can simultaneously form more than one beam)

  - UE mobility

- Note: While discussing the scalability, it is recommended to clearly state how the performance (accuracy/effectiveness of inference, latency, Beam failure rate etc.) and the cost (computational complexity, signaling overhead etc.) scales with the network parameters.

- Scalability needs to be included as a KPI due to the following reasons: The inference complexity of a method may increase linearly or exponentially with the cardinality of the set of the beams to be searched over (obviously, linear complexity is preferred). A method may maintain same complexity but its accuracy may degrade significantly with the increase in the search space. Some methods might work well under only stationary environments and may significantly underperform with high UE mobility.

Scalability of a proposed beam management AI/ML model for different parameter settings, e.g., number of beams at gNB/UE, should be evaluated.

- Adaptability/Generalizability

- Does the model adapts to dynamically changing network environment (and corresponding changes in the statistical characteristics of the data)? Some of the different network conditions are indicated below (please note that this is not an exhaustive list).

  - Different channel conditions

    - Channels with single dominant path vs. multipath channels

  - Different UE speeds and rotation patterns

    - Different possible orientations of the UE, UEs with different speeds etc.

  - Does the AI/ML model works with any generic beam design?

    - Ex: Some of the methods may assume a particular beam codebook design and may not work well with other beam designs.

Inference accuracy of a proposed beam management AI/ML model (without model updates) should be reported for different channel models including UMa, UMi (with various parameters), UE speed, UE orientations, and beam codebook design.

The following table summarises the KPIs/Performance metrics discussed above.

Table 1: Proposed KPIs for AI/ML based Beam Management

| No. | KPI | Description |
| --- | --- | --- |
| 1 | Accuracy and Effectiveness | - Accuracy can be measured in the average error probability in finding the correct beam pair. It can be presented as the average probability of mis-detection of correct beam pair at different values of SNR/SINR (preferably as a plot) - The resulting L1-RSRP/L1-SINR value in dB/dBm with the selected beam-pair (depending on whether we are considering single-beam/no-interference or multi-beam/inter-beam interference scenario). |
| 2 | Signaling Overhead | - Overhead, in terms of resources/reports/signaling needed for - The reference signals (CSI-RS, SRS, SSB) that need to be configured - The measurement reports that need to be sent over the network - Any other signaling needed for the functioning of AI/ML model |
| 3 | Latency | - The time required for Tx-Rx beam pair selection (in seconds) |
| 4 | Computational Complexity | - Complexity/power consumption of model inference plus the complexity of the model update/retraining if applicable, along with the frequency of update/retraining. - Complexity/power consumption of training to be considered only if the training happens onsite in an online fashion. - In some cases, the power consumption incurred by the method might be a more useful/insightful metric (Ex: power consumption in the UE). |
| 5 | Time required for Training | - Considered only in case of online training after deployment of the model (e.g., An RL method trained onsite). - Time required for model update/retraining, if applicable, along with the frequency of update/retraining. |
| 6 | Measurements from Sensor(s) and Hardware support | - Whether the nodes need to be equipped with additional resources such as sensors/actuators (e.g., Camera, LIDAR), memory, hardware (e.g., GPU) for providing any additional side information (UE orientation/location, visual/spatial details of the environment etc.) required by the AI/ML model. |
| 7 | Robustness | - Sensitivity of the method to - Errors in measurement reports/signals - Latency in generating the measurement reports and transferring them over the network. |
| 8 | Scalability | - How the performance and the cost of the method scales with the network parameters (such as number of beams at gNB/UE, UE speed etc.) - Can be determined by evaluating KPIs with different values of the key system parmeters. (Please refer to the detailed discussion of KPIs.) |
| 9 | Adatability/Generalizability | - Captures whether the model adopts to the dynamics of the network, such as different channel conditions, different UE speeds and rotation/orientation patterns, multiple types of beam designs etc. |

Finally, the following points is noted with regard to the KPIs listed and discussed above.

- The proposed AI/ML method for beam management may address the entire beam management problem (i.e., Initial Access, Beam tracking/refinement and Beam recovery) or it may cater only to certain phases/parts of the beam management. For example, the AI/ML model might address only the beam acquisition/section phase; Or, the model might only be concerned with an efficient way of beam tracking and/or beam prediction. Any such details about the proposed AI/ML model should be stated with the evaluation results.

- For the KPIs related to the cost of the proposed AI/ML model, the cost of every phase in the lifecycle of the AI/ML model (Training, validation, inference, model update) should be considered, whenever network/UE resources are involved. For example, the signaling overhead may involve not only the signaling required for inference, but also the signaling required for model update, training (in case of online training) etc.

- For some of the KPIs, the exact definition and how/under what settings to measure the particular KPI may need to be discussed and agreed. Further, it is recommended to state average as well as the worst case values of the KPIs, wherever it is applicable. (Ex: Average and worst case latency, average and worst case complexity etc.)

KPIs for the AI/ML beam management evaluation should at least include accuracy and effectiveness, beam measurement overhead, latency, training convergence time, model robustness, scalability, adaptability and complexity/power consumption.

# Conclusions

In this contribution, we discussed the evaluation methodology and KPIs/Performance metrics for AI/ML based beam management and made the following proposals:

Proposal 1:	The beam management AI/ML model evaluation methodology should support evalutations with realistic modeling of UE mobility and rotation, spatially consistent channel models, different UE speeds, and single-beam/panel and multi-beam/panel scenarios for evaluating a model's robustness and adaptability.

Proposal 2:	Generate the dataset for the model training and evaluation for beam management sub use cases based on the statistical models in TR 38.901 taking in to account spatially consistency.

Proposal 3:	KPIs for the AI/ML beam management performance evaluation should include beam alignment success rate, effectiveness (in terms of improvement in L1-RSRP or L1-SINR relative to legacy schemes), and beam failure rate.

Proposal 4:	Beam management signaling (e.g., RS, measurement reports) overhead required for a proposed AI/ML model (at every phase in the model lifecycle) should be evaluated. This can be reported relative to state-of-the-art Rel-17 based baseline schemes.

Proposal 5:	Beam acquisition/selection latency including Tx/Rx beam training should be evaluated for a proposed AI/ML model.

Proposal 6:	Computation complexity of the proposed AI/ML model should be evaluated at every phase in the model lifecycle - training, inference, update.

Proposal 7:	A proposed AI/ML model that is based on online learning (e.g., online training, model update/re-training) should evaluate the convergence time for the model to reach a valid inference state.

Proposal 8:	Any additional side information (e.g., measurements from non-3GPP technologies) needed for a proposed beam management AI/ML model should be stated.

Proposal 9:	Inference accuracy sensitivity of a proposed beam management AI/ML model should be evaluated for a range of noisy input data and delay constraints.

Proposal 10:	Scalability of a proposed beam management AI/ML model for different parameter settings, e.g., number of beams at gNB/UE, should be evaluated.

Proposal 11:	Inference accuracy of a proposed beam management AI/ML model (without model updates) should be reported for different channel models including UMa, UMi (with various parameters), UE speed, UE orientations, and beam codebook design.

Proposal 12:	KPIs for the AI/ML beam management evaluation should at least include accuracy and effectiveness, beam measurement overhead, latency, training convergence time, model robustness, scalability, adaptability and complexity/power consumption.

# References

- RP-213599, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," RAN#94e.

- R1-2204416, "General aspects of AI/ML framework," Lenovo, RAN1#109-e, May 2022.

- F. Sohrabi, Z. Chen and W. Yu, "Deep Active Learning Approach to Adaptive Beamforming for mmWave Initial Alignment," ICASSP 2021 - 2021 IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2021, pp. 4940-4944. (Extended journal version available at arXiv:2012.13607).