3GPP TSG RAN WG1 Meeting #109-e	R1-2204423

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.3.2

Source: 	Lenovo

Title: 	Sub-band non-overlapping full duplex

Document for:	Discussion

# Introduction

During RAN#94-e meeting, the following objectives were agreed to study enhanced NR duplex operation  [1]:

- Study the sub-band non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-sub-band CLI and inter-sub-band CLI in case of the sub-band non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-sub-band CLI, and the inter-operator CLI at gNB and the inter-sub-band CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss methods to enable sub-band non-overlapping full duplex operation.

# Framework for sub-band full duplex operation

Sub-band full duplex operation (SBFD) aims at extending a duration over which uplink transmissions can occur without causing significant interference issues, in order to improve uplink coverage and capacity and to reduce latency in unpaired spectrum. Simultaneous usage of DL and UL is only at gNB and not at UE side. The examples of duplexing scheme are illustrated in Figure 1(a) and 1(b).

Figure 1. Examples for sub-band full duplex operation.

For sub-band full duplex operation, the first question to be discussed is whether and how to determine a sub-band structure in time and frequency domains. Two approaches can be considered as follows:

- Scheme 1:  Using TDD configuration in Rel-15 (such as a cell-specific TDD UL/DL configuration, a UE-specific TDD UL/DL configuration, and additionally a dynamic SFI indication) to indicate more flexible symbols, in which UEs can be indicated to receive in a DL sub-band or transmit in an UL sub-band based on gNB scheduling. The UE just follows the scheduling for data transmission in the flexible symbols and does not need to determine a sub-band region explicitly.

- Scheme 2:  UE is indicated a sub-band region explicitly based on specific gNB configurations. The sub-band region is not necessarily within flexible symbols configured by the legacy TDD configuration.

The advantage of scheme 1 may be more scheduling flexibility and less specification effort. However, scheduling complexity might be higher, and interference coordination between gNBs may be challenging considering that a UL/DL slot/frame structure is not fixed. Also, performance for configured transmission and reception would be impacted since the flexible symbols may not be used for such transmissions (e.g., overlapping with sub-band(s) with variable communication direction). Considering that UL/DL directions are mostly fixed in currently deployed commercial TDD networks, scheme 1 (configuring more flexible symbols) would incur more impacts on legacy UE operations.

While scheme 2 may have more standard impacts, scheme 2 is expected to make much less impact on legacy UE operation. Furthermore, it may be easier to control inter-UE and inter-gNB interference in scheme 2 with more structured coordination among gNBs. Thus, we think scheme 2 should be studied for Rel-18 NR duplex evolution.

Proposal 1:  Explicit sub-band configuration for sub-band full duplex operation in a cell should be studied in Rel-18.

# Potential areas for enhancement

With legacy TDD UL/DL configurations, a UE can be configured with only one communication direction (i.e. DL, flexible, or UL) for a given symbol in a given cell, and cannot be configured with multiple communication directions for different sub-bands for the given symbol in the given cell. Thus, how to configure a sub-band region to realize sub-band full duplex operation in a cell needs to be discussed.

One way to enable sub-band full duplex operation in a cell is to utilize a bandwidth part (BWP) based operation framework of NR. A BWP configuration can naturally configure a sub-band within a carrier. Further, an active BWP can be dynamically switched, if dynamic switching of the active BWP is supported by UE capability. If gNB can configure a UE with a BWP-specific communication direction for a given BWP and/or configure the UE with a BWP dedicated for sub-band full duplex operation in a cell, sub-band full duplex operation by gNB can be realized in a given carrier.

Proposal 2: Consider enhancing BWP operation to support sub-band full duplex operation in a cell.

Assuming that UEs supporting sub-band full duplex operation in a cell and UEs not supporting SBFD operation in the cell co-exists in the cell, an impact on legacy operation should be studied.

For PDCCH transmission, considering that configuration of control resource sets (CORESETs) in Rel-15 could be discontinuous in the frequency domain by bitmap, gNB can guarantee the PDCCH transmission not impacted when an UL sub-band is configured within a bandwidth of a DL BWP. Thus, legacy PDCCH operation can be reused.

For PDSCH transmission, PDSCH resource mapping with RB symbol level granularity in Rel-15 could be reused to avoid using an UL sub-band by including resources for the UL sub-band into the rateMatchPatternToAddModList. Thus, legacy PDSCH operation can also be reused.

Observation 1: Legacy PDCCH and PDSCH operation can be reused to support sub-band full duplex operation in a cell.

For PUSCH transmission in an UL sub-band configured for sub-band full duplex operation in a cell, one issue is how a PUSCH configuration is determined. One option is to use a PUSCH configuration of an active UL BWP, where the UL sub-band is included. Alternatively, a separate UL BWP (for sub-band full duplex operation) and a corresponding separate PUSCH configuration for the separate UL BWP can be configured, or a separate PUSCH configuration for PUSCH transmission in the UL sub-band for SBFD within the active UL BWP can be configured.

As shown in Figure 2, to use a Type 1 CG PUSCH configuration both for an UL sub-band and for an active UL BWP, additional restriction to the Type1 CG PUSCH configuration is necessary to guarantee a PUSCH transmission confined within a bandwidth of the UL sub-band. Therefore, a separate PUSCH configuration for PUSCH transmission in the UL sub-band for SBFD may be desirable.

Figure 2. CG PUSCH transmission in sub-band full duplex operation.

For PUCCH transmission, one issue is whether to allow a PUCCH transmission in an UL sub-band configured for sub-band full duplex operation in a cell, and if allowed, how a PUCCH configuration is determined in the UL sub-band. Allowing PUCCH transmissions in the UL sub-band is clearly beneficial for reducing HARQ-ACK feedback latency. However, more standard impact is expected to support PUCCH transmission in the UL sub-band, e.g., a separate PUCCH configuration may be needed considering different bandwidths of the UL sub-band and an UL BWP.

Proposal 3: Consider separate PUSCH/PUCCH configurations for PUSCH/PUCCH transmissions in an UL sub-band configured for sub-band full duplex operation in a cell.

For a UE supporting sub-band full duplex operation in a cell, it should be studied how the UE can determine a communication direction for a given symbol depending on a frequency resource, e.g. overriding rules for legacy TDD configuration.

Proposal 4: UE behaviours related to determining a communication direction need to be studied for a UE supporting sub-band full duplex operation in a cell.

# Conclusion

In summary, we propose the followings for Rel-18 NR sub-band non-overlapping full duplex operation:

- Proposal 1:  Explicit sub-band configuration for sub-band full duplex operation in a cell should be studied in Rel-18.

- Proposal 2: Consider enhancing BWP operation to support sub-band full duplex operation in a cell.

- Proposal 3: Consider separate PUSCH/PUCCH configurations for PUSCH/PUCCH transmissions in an UL sub-band configured for sub-band full duplex operation in a cell.

- Proposal 4: UE behaviours related to determining a communication direction need to be studied for a UE supporting sub-band full duplex operation in a cell.

# References

- RP-213591, “Study on Evolution of NR Duplex Operation”.