3GPP TSG RAN WG1 Meeting #109-e	R1-2204424

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.7.2

Source: 	Lenovo

Title: 	Network energy saving techniques

Document for:	Discussion

# Introduction

In RAN #94-e meeting, 3GPP agreed a new study item “Study on network energy savings for NR” [1]. One of objectives is to study and identify techniques on gNB and UE side to improve network energy savings in terms of both base station (BS) transmission and reception, which may include, without precluding other techniques:

- How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information

- Information exchange/coordination over network interfaces

In this contribution, we discuss potential techniques for network energy savings in New Radio (NR).

# Considerations for flexible transmission by gNB

3GPP New Radio (NR) features a configurable synchronization signal and broadcast channel (i.e. SS/PBCH block (SSB)) periodicity ranging from 5ms to 160ms and on-demand system information (SI) delivery, which make NR more energy efficient compared to LTE in terms of required energy per bit. However, densely deployed network nodes and relatively high power consumption per node from massive MIMO and/or high frequency band operations would lead to overall increased power consumption by 5G infrastructure. For example, gNB may transmit the maximum 64 SSBs and the maximum 64 corresponding copies of PDCCH/PDSCH for SIB1 delivery in every 20ms in FR2. Thus, enhancements may be necessary to provide greater flexibility and efficiency in terms of turning on and off a network node, e.g. partially turning off for a particular spatial coverage based on traffic loads and user distributions.

For example, it may be beneficial to flexibly change a set of actually transmitted SSBs out of a set of predefined SSB candidate positions based on UEs’ spatial distribution and predicted (or estimated) UE locations (e.g., knowledge of UE spatial distribution for different times of a day). gNB may estimate current UE locations using 3GPP/non-3GPP positioning technologies, various sensors, e.g. radar and camera and/or predict future UE locations based on estimated UEs’ movement trajectories/directions and speeds (additionally using artificial intelligence and/or machine learning), and may adjust a SSB transmission pattern within a half frame accordingly.

In another example, it may be desirable to transmit SSBs within a half frame with different periodicities. In response to gNB’s determining that there is no or few RRC connected UEs served by a subset of SSBs (i.e. a subset of downlink (DL) Tx beams) and/or there are very few UEs in locations related to the subset of SSBs in a cell, the gNB can configure a longer periodicity (e.g. 20ms or longer) for the subset of SSBs. Further, when gNB provides one or more TRS occasions, which are configured for RRC connected UEs and quasi-co-located with a subset of SSBs, to RRC idle/inactive UEs in a cell, the gNB can configure a longer periodicity for the subset of SSBs.

Similarly, NR paging can be enhanced to reduce network energy consumption, based on UE location or other information. In legacy NR paging design, paging monitoring occasions and corresponding paging message delivery are designed to reach UEs in an entire cell range, especially including UE(s) in poor radio conditions (e.g. UEs located in a cell boundary, UEs with high speed, etc), with using low modulation and coding schemes (MCS) and high control channel element (CCE) aggregation level. If the gNB can acquire the UE location or other information, paging DCI and paging PDSCH can be transmitted flexibly only for necessary spatial directions.

Proposal 1: Study how to provide more flexibility in transmission of common signals/channels such as SSBs, SI, and paging.

Similar to LTE supporting an interference-aware small cell on/off mechanism in a heterogeneous network, it may be beneficial to allow gNB to flexibly turn on/off transmission in the time domain via semi-persistent or dynamic signalling. For example, gNB is not allowed to transmit data or signal in DL slots/symbols that are indicated as transmission OFF, except for essential signals that must be transmitted in these DL slots/symbols. Multiple gNBs can coordinate transmission ON/OFF operations for interference management (e.g. mute some subframes/slots/symbols) and align their transmissions to limited subframes/symbols to reduce the effective transmission time.

Proposal 2: Study how to support dynamic gNB transmission ON/OFF mechanism.

# Considerations for efficient offloading

Multiple low-transmit-power network nodes such as multiple transmission and reception points (TRPs) and/or smart repeaters deployed within a cell may be beneficial, e.g., to overcome channel blockage in high frequency bands, can increase spectral efficiency based on spectral reuse, and allow UEs to perform layer-1 (L1) and/or layer-2 (L2) based mobility instead of layer-3 (L3) based mobility (i.e. a serving cell change). Both a smart repeater and a remote TRP can create additional spatial coverage potentially different from a base station by employing a set of beams different from a set of beams that the base station uses.

In NR, a cell can have only one SSB burst pattern, and all SSBs in a SSB burst have the same Tx power. A SSB burst pattern (i.e. SSB positions in burst) makes an impact on SSB-random access channel (RACH) occasion association and SIB1/paging PDCCH monitoring occasions. Thus, if one network node within a cell and corresponding SSBs are turned off, all UEs in the cell may have to re-determine SSB-RACH occasion association and SIB1/paging PDCCH monitoring occasions. To minimize impact on UE power consumption, it is desirable that impact of network energy saving operation is limited to UEs served by a turned-off network node.

Proposal 3: Study efficient L1/L2 based mobility mechanisms to support flexible on/off of a network node within a cell.

When a cell provides an overlapped coverage with another cell and/or traffic load is light, it may be beneficial to turn off or put the cell in a dormant state for network power saving. Before turning off the cell, a number of UEs served by the cell may need to be moved by handover to another cell in short time. Similarly, several UEs on the cell may need to be redirected to a neighbouring cell of another frequency layer (or another radio access technology (RAT)) to turn off the cell.

To efficiently offload UEs to another cell while flexibly turning off or turning on a cell, group-based mobility such as changing a primary cell of a group of UEs simultaneously and/or redirecting a group of UEs to a different frequency layer simultaneously may be beneficial.

Proposal 4: Study group-based mobility mechanism to support flexible on/off of a cell with efficient offloading.

# Conclusion

In summary, we propose the followings for potential enhancements for network energy savings in Rel-18 NR:

- Proposal 1: Study how to provide more flexibility in transmission of common signals/channels such as SSBs, SI, and paging.

- Proposal 2: Study how to support dynamic gNB transmission ON/OFF mechanism.

- Proposal 3: Study efficient L1/L2 based mobility mechanisms to support flexible on/off of a network node within a cell.

- Proposal 4: Study group-based mobility to support flexible on/off of a network node with efficient offloading.

# References

- RP-213554, “Study on network energy savings for NR”.