-3GPP TSG RAN WG1 #109	R1-2204430

e-Meeting, May 9th – 20th, 2022

Agenda item:		9.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On the evaluation methodology for NR duplexing enhancements

Document for:		Discussion and Decision

# Introduction

RAN has agreed in RP-220633 a new Study Item on evolution of NR duplex operation with following objectives:

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

Specifically, this contribution provides recommendations for the evaluation methodology, including assumptions for system-level simulations in line with the highlighted objective.

# Evaluation methodology for Dynamic TDD

Dynamic TDD was studied during the Rel-16 cross-link interference (CLI) study (TR 38.828) with special focus on adjacent channel coexistence. As also discussed in our companion contribution (R1-2204432), the Rel-16 study concluded that dynamic TDD on Macro FR1 scenarios is challenging from adjacent channel perspective, especially due to the gNB-to-gNB adjacent channel interference where one base station transmits in the DL direction, while another base station from another operator deployed on an adjacent channel receives an UL transmission.

For this reason, our preference is that dynamic TDD should only be applicable for small-cells or low-power base stations. If the assumptions for macro deployment coexistence studies change from Rel-16 evaluations, new conclusions might be derived and FR1 macro deployments adopting dynamic TDD might be seen as feasible.

Individually selecting the optimal TDD radio frame configuration in an uncoordinated manner generates co-channel CLI as gNBs might choose different link directions for a given slot. Standardization work to measure and report the CLI experienced by UEs (UE-to-UE CLI) was addressed as part of the Rel-16 CLI study (TR 38.828). In Rel-18 the focus should be on the gNB-to-gNB CLI. Here, our main objective is to study solutions that allow gNBs to adopt dynamic TDD while combating the generated cross-link interference.

Observation 1: Based on Rel-16 studies, dynamic TDD operation is mainly feasible for small cells due to low transmit power and antenna gain resulting in improved coexistence with e.g. other-operator deployments in adjacent channels.

Observation 2: Unless the underlying assumptions change as compared to the Rel-16 Cross-link Interference Study Item, adjacent channel coexistence studies should not be repeated for dynamic TDD.

Proposal 1: The dynamic TDD study should focus on identifying enhancements and solutions that allow gNBs to more freely adjust their TDD radio frame configuration in line with their traffic demands by combating cross-link interference.

## Simulation Scenarios

For evaluating the benefits of enhancements for dynamic TDD, system-level simulations are known to be a powerful tool as it realistically captures the dynamics of the system e.g. from realistic (bursty) traffic behaviour and interference. Focusing on small-cell scenarios, dynamic TDD on the so-called indoor hotspot (InH) scenario was already evaluated during Rel-14 flexible duplexing studies as summarized in TR 38.802 (together with traditional Urban Macro and Dense Urban scenarios), while HetNet scenarios consisting of both a traditional macro layer and a small-cell layer belonging to the same operator have not been sufficiently studied to the best of our knowledge. This scenario might also be interesting for operators that lease part of their spectrum for private network operation. In such case, operators are allowed to use the leased bands as long as they can guarantee certain performance in the private networks.

Based on the conclusions from the Rel-16 study on adjacent channel coexistence, macro cells are configured with fully-static/synchronized TDD. As mandated by spectrum regulations and due to the high demands of downlink traffic, macro cells typically adopt a DL-heavy TDD configuration. On the other hand, small cells are configured with dynamic TDD. Each small cell individually chooses the optimal radio frame configuration that best fits the instantaneous traffic needs. The candidates radio frame configuration ranges from the most UL-heavy (e.g. DUUUU, where ‘D’ and ‘U’ are downlink and uplink radio slots, respecitvely) to the most DL-heavy options, where the most DL-heavy radio frame matches the TDD pattern used at the macro cells, e.g. DDDDU. All the candidates radio frame are assumed to have a single link direction switching point within the periodic TDD radio frame pattern. In our modelling, prior to the start of each radio frame, the dynamic TDD cell computes the ratio between the DL buffer size and the DL+UL buffer size. This ratio is then used for the selection of the next radio frame. If needed in certain coordination mechanisms, signalling exchange between cells via Xn/F1 interface is transmitted via the backhaul with ideal conditions and negligible latency.

Observation 3: Dynamic TDD operation has not been studied for a HetNet (two-layer) co-channel scenario, where the Macro layer uses static and fully aligned/synchronized DL-heavy TDD radio frame configuration (to ensure inter-operator coexistence) while the small cell layer operates with dynamic TDD in line with their UL and DL traffic demands.

Motivated by this, we think a good starting point for defining the evaluation scenarios for dynamic TDD are those defined for the LTE small cell study in TR 36.872 and illustrated in Figure 1:

Figure 1: Small cell scenarios defined in TR 36.872

From these scenarios in Figure 1, our main preference is Scenario 1 consisting of outdoor Macro cells and outdoor small cells with cochannel deployment. This scenario is ideal to evaluate the benefits related to inter-gNB coordination and cross-link interference mitigation techniques. Note also that this is the most suitable scenario for operators with limited amount of NR spectrum available. Scenario 2a, 2b or 3 does not consider any interference between the macro and small cell layers as these are deployed on separate carrier frequencies. From cross-link interference point, the challenges in Scenario 2a, 2b and 3 are therefore not very different than those in a small-cell only scenario (evaluated during the Rel-14 study in TR 38.802). Moreover, note that due to lower gNB transmit power, it is expected that the gNB-to-gNB CLI is weaker than in macro cells + small cells scenarios. Therefore, these scenarios are considered as low priority.

Proposal 2: For evaluating the performance and identifying enhancements for dynamic TDD operation, focus on a two-layer HetNet scenarios with outdoor macro cells + outdoor or indoor small cells with cochannel deployment (e.g. based on Scenario 1 from TR 36.872).

- Macro layer uses static DL-heavy TDD radio frame configuration (to ensure inter-operator coexistence) while the small cell layer operates with dynamic TDD in line with their UL and DL traffic demands.

# Evaluation methodology for Sub-band non-overlapping Full duplex

Contrary to dynamic TDD, the performance of sub-band non-overlapping full duplex (SBFD) has not been studied before by the 3GPP from neither co-channel nor adjacent-channel perspective. Table 1 lists different evaluation scenarios in terms of UL/DL subband and spectrum allocations, and the types of interference that are present for each of them. Figure 2 also depicts the types of interference found in a SBFD deployment. Please note that each interference type is identified with a number in both Table 1 and Figure 2.

Table 1: Types of interference in SBFD deployment for different UL/DL subband configuration assumptions.

|  | Case A: Single operator. Same SBFD UL-DL PRB partitioning in all the cells | Case B: Single operator. Different SBFD UL-DL PRB partitioning in each cell | Case C: Single operator. Coexistence of SBFD with legacy TDD cells in co-channel | Case D: Two or more operators. Coexistence of SBFD (with same UL-DL partitioning) with legacy TDD in adjacent-channel |
| --- | --- | --- | --- | --- |
| - gNB self-interference | X | X | X | X |
| - intra-cell UE-to-UE co-channel inter-subband CLI | X | X | X | X |
| - inter-cell UE-to-UE co-channel inter-subband CLI | X | X | X | X |
| - gNB-to-gNB co-channel inter-subband CLI | X | X | X | X |
| - gNB-to-gNB inter-cell co-channel intra-subband CLI |  | X | X |  |
| - UE-to-UE inter-cell co-channel intra-subband CLI |  | X | X |  |
| - gNB-to-gNB inter-cell adjacent channel CLI |  |  |  | X |
| - UE-to-UE inter-cell adjacent channel CLI |  |  |  | X |

Figure 2: Types of interference in SBFD deployment

First of all, we would like to note that the interference types (5) and (6) related to inter-cell co-channel CLI are similar to the ones present in dynamic TDD. Therefore, we think that the evaluation scenarios in Case B and Case C should be regarded as low priority as the corresponding challenges can be handled as part of the dynamic TDD evaluation. Instead, the focus for the SBFD study should be on the impacts of the full-duplex-specific sources of interference, namely (1)-(4). Though, adjacent channel CLI towards neighbor operators is not specific to SBFD, interference types (7) and (8) may also need to be considered as SBFD operation may have slightly different impacts on adjacent channel CLI than e.g. dynamic TDD. Therefore, we suggest focussing on the evaluation scenarios in Case A and Case D as per Table 1 above. The following observations and proposals are made:

Observation 4: On the evaluation methodology for full duplex on non-overlapping PRBs, the selected scenarios should focus on the impacts of the full-duplex-specific sources of interference, namely:

- (intra-cell) gNB self-interference from in-band emissions

- (inter-cell) gNB-to-gNB interference from in-band emissions

- (intra-cell and inter-cell) UE-to-UE interference from in-band emissions

- (inter-cell) gNB-to-gNB interference from adjacent-channel emissions

- (inter-cell) UE-to-UE interference from adjacent-channel emissions

Proposal 3: For evaluating the performance and identifying enhancements for SBFD, focus on co-channel scenarios with same UL-DL PRB partitioning in all the cells and on adjacent channel scenario with legacy TDD.

On the deployment type, as also noted in our companion contributions R1-2204431 and R1-2204433, we think SBFD on macro cells is very challenging from both co-channel and adjacent-channel interference. Nevertheless, we think Urban Macro or Dense Urban scenarios could still be evaluated as a way to reach a conclusion among 3GPP partners on their feasibility. To potentially reduce simulation effort, one possibility is that RAN4 first discusses whether SBFD is significantly different than dynamic TDD from adjacent channel coexistence perspective, before running actual simulations. In addition to Urban Macro or Dense Macro deployments, small-cell deployments could also be evaluated but with particular focus on co-channel aspects.

Proposal 4: For evaluating the performance and identifying enhancements for full duplex on non-overlapping PRBs, focus on the following scenarios for system-level simulations:

- Urban Macro or Dense urban macro, with focus on co-channel and adjacent channel coexistence.

- Indoor small cell deployments, e.g. open office scenario from TR 38.901 or Indoor scenario from TR 38.828, or outdoor small cell deployment, e.g. by only considering the small-cell layer of HetNet Scenario 2A from TR 36.872. Small cell deployment evaluations shall focus on co-channel interference aspects.

## Modeling of Adjacent Channel Interference for Full duplex on non-overlapping PRBs

Modeling of unwanted emissions (essentially resulting in increased interference or noise) outside an UL or DL PRB allocation is of uttermost importance to correctly assess the benefits of full-duplex on non-overlapping PRBs. In the Rel-16 cross-link interference study (TR 38.828), assumptions were made on the UE and the gNB Adjacent Channel Selectivity (ACS) and Adjacent Channel Leakage Ratio (ACLR), which are used to determine the corresponding Adjacent Channel Interference Ratio (ACIR) for BS-BS, BS-UE and UE-UE links for inter-operator coexistence studies. However, for full duplex on non-overlapping PRBs, we see need to adopt additional models as the problem of interest here is not only related to “adjacent channel” interference but also “in-band” interference since both the aggressor and victim PRBs are placed on the same radio carrier.

In this regard, 3GPP TS 38.101-1, Section 6.4.2.3 already specifies requirements for UE in-band emissions (IBE) where the generated interference on non-allocated PRBs is specified as a sum of a General, IQ Image and a Carrier leakage component, which value depends on the MCS, transmit power and separation between the allocated and non-allocated PRBs. We think this existing UE IBE model should be adopted for SBFD evaluations.

In addition to UE IBE, assumptions on gNB and UE in-channel selectivity (ICS) and gNB IBE are also needed. In our understanding, RAN4 specifications do not have requirements for UE ICS and gNB IBE and therefore some discussions in RAN1 and RAN4 would be needed to agree on a set of values and modeling assumptions. For gNB ICS, there are some requirements specified in TS 38.104, Section 7.8; however, as we also elaborated in R1-2204433, we think the specified requirements are overly conservative and thus we think RAN4 should discuss new values taking realistic gNB capabilities into account.

Proposal 5: For the modelling of in-band emissions (IBE) for SBFD evaluations, adopt the UE IBE model as specified 3GPP TS 38.101-1, Section 6.4.2.3 including General, IQ Image and Carrier leakage components.

Proposal 6: RAN1 and RAN4 to discuss and agree on models for gNB and UE ICS and gNB IBE.

Based on the agreed gNB and UE IBE and ICS models and quantities, an inter-subband interference ratio (ISIR) (in dB) can be calculated which determines the interference generated by an allocated (i.e. ‘aggressor’) PRB or resource element (RE) to a non-allocated (‘victim’) PRB/RE, relative to the aggressor's transmit power (normalized per PRB or RE). This is used for co-channel inter-cell and intra-cell interference calculations. For the gNB self-interference, an additional self-interference suppression parameter α (in dB) can be considered expressing the gNB’s capability to suppress in-band interference using a combination of Tx-Rx isolation and analog or digital cancellation. Simulation can be run with different values of α, e.g. 50 dB (conservative), 80 dB (optimistic) and 110 dB (highly-optimistic) to understand what level of interference suppression is needed to guarantee acceptable performance.

For gNB-gNB or UE-UE links (interference types (2)-(4) in Figure 2), we don’t see as realistic to consider any additional interference suppression, so the resulting co-channel inter-subband interference is only a function of the aggresor’s transmit power and coupling gain between the aggressor and victim gNB or UE. In this regard, it needs to be discussed whether the fast fading effects should be considered between these links.

Proposal 7: For the modelling of inter-subband co-channel interference in SBFD:

- IBE and ICS models are used to determine an inter-subband interference ratio (ISIR) in dB which determines the interference generated by an allocated (i.e. ‘aggressor’) PRB or resource element (RE) to a non-allocated (‘victim’) PRB/RE, relative to the normalized aggressor’s transmit power.

- For gNB self-interference calculations, assume a self-interference suppression value α (in dB) expressing the gNB’s capability to suppress inter-subband interference (determined from ISIR) using a combination of Tx-Rx isolation and analog or digital cancellation.

  - Simulation can be run with different values of α, e.g. from 50 dB (conservative) to 110 dB (highly-optimistic).

- For gNB-gNB or UE-UE links, the resulting co-channel inter-subband interference is only a function of the aggressor’s transmit power, ISIR and coupling gain between the aggressor and victim gNB or UE. No additional interference suppression is considered.

# Other evaluation assumptions (traffic and propagation models, etc.)

To show the benefits of dynamic TDD and SBFD, mixed DL and UL traffic should be considered. Different traffic assumptions and KPIs are proposed depending on the target use case.

Dynamic TDD evaluations:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - The offered load can be selected to meet a certain PRB resource utilization (RU) target, e.g. 10% and 40% for low and medium load, respectively.

  - For the proposed HetNet scenario with dynamic TDD, it may be further considered to have DL-heavy traffic for the UEs connected to the macro-cells, while the small-cell UEs have UL-heavy or balanced traffic.

SBFD evaluations:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - The offered load can be selected to meet a certain PRB resource utilization (RU) target, e.g. 10% and 40% for low and medium load, respectively.

  - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered.

- FTP3 traffic model with small payload size, e.g. 32-1500 Bytes or other similar traffic models as discussed during Rel-16 URLLC/IIoT Study Item in TR 38.824.

  - Arrival rate per UE can be fixed, e.g. to 100 packet/second in both UL and DL.

  - Main KPIs: CDF of the latency successfully delivered FTP3 packets and/or percentage of UEs meeting certain latency and reliability requirement.

  - Note: Companies to explicitly account for half-duplex UE limitations preventing the UE to receive and transmit at the same time.

Proposal 8: For the traffic models and KPIs for dynamic TDD evaluations, assume the following:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load).

  - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered.

    - For the proposed HetNet scenario with dynamic TDD, it may be further considered to have DL-heavy traffic for the UEs connected to the macro-cells, while the small-cell UEs have balanced or UL-heavy traffic.

Proposal 9: For the traffic models and KPIs for SBFD evaluations, assume the following:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load).

  - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered.

- FTP3 traffic model with small payload size, e.g. 32-1500 Bytes or other similar traffic models as discussed during Rel-16 URLLC/IIoT Study Item in TR 38.824.

  - Main KPIs: CDF of the latency successfully delivered FTP3 packets and/or percentage of UEs meeting certain latency and reliability requirement.

  - Arrival rate per UE can be fixed, e.g. to 100 packet/second in both UL and DL.

  - Note: Companies to explicitly account for half-duplex UE limitations preventing the UE to receive and transmit at the same time.

Finally, with regards to channel models, our preference is to adopt as much possible those specified in TR 38.901 for both gNB-UE links as well as gNB-gNB and UE-UE links. Table 2 below shows our channel model preference depending on the deployment scenario. We would like to point out that existing InH and UMi models take only into account the 2D distance but not the UE height for calculating the LOS probability. In other words, the LOS probability for a gNB-UE link and UE-UE link and gNB-gNB link are the same for a certain 2D distance between the transmitter and receiver. We think RAN1 should discuss the validity of these models and potentially agree on minor modifications, e.g. assume LOS for all gNB-gNB links with a 2D distance below a certain threshold.

Proposal 10: For the channel models for dynamic TDD and SBFD system-level evaluations, re-use the existing channel models in TR 38.901 as much as possible for gNB-to-UE, gNB-to-gNB and UE-to-UE links.

- See Table 2 in this contribution for considerations on channel model and UE and BS height for Urban Macro (UMa), Dense Urban and Indoor Hotspot (InH) scenarios.

- RAN1 to discuss whether changes to the LOS probability calculations in UMi and InH channel models are needed to explicitly account for the transmitter-receiver height (in both absolute and relative terms).

Table 2: Channel models for gNB-UE, gNB-gNB and UE-UE links.

|  | UMa (500 m ISD) | Dense Urban (200 m ISD) | Indoor Hotspot (20 m ISD) |
| --- | --- | --- | --- |
| gNB-UE | TR 38.901 UMa | TR 38.901 UMi | TR 38.901 InH |
| gNB-gNB | TR 38.901 UMa with hUE = hBS = 25m | TR 38.901 UMi with hUE = hBS = 10m | TR 38.901 InH with hUE = hBS = 3m |
| UE-UE | TR 38.901 UMi with hUE = hBS | TR 38.901 UMi with hUE = hBS | TR 38.901 InH with hUE = hBS = 1.5m |

|  | HetNet (500 m ISD macro layer + outdoor small cells) | HetNet (500 m ISD macro layer + indoor small cells) |
| --- | --- | --- |
| Macro gNB-UE | TR 38.901 UMa | TR 38.901 UMa |
| Small cell gNB-UE | TR 38.901 UMi | TR 38.901 InH |
| Macro-to-Macro | TR 38.901 UMa with hUE = hBS = 25m | TR 38.901 UMa with hUE = hBS = 25m |
| Macro-to-small cell | TR 38.901 UMa with hUE =10m and hBS = 25m | TR 38.901 UMa with hUE = 3m and hBS = 25m, and O2I penetration loss |
| Small cell-to-small cell | TR 38.901 UMi with hUE = hBS = 10m | TR 38.901 InH with hUE = hBS = 3m |
| UE-UE | TR 38.901 UMi with hUE = hBS | TR 38.901 InH if UEs are in the same office, otherwise UMi. |

# Conclusion

In this contribution, we have provided our view on the evaluation assumptions for dynamic TDD and sub-band full duplex (SBFD) Rel-18 studies.

For dynamic TDD we have the following observations and proposals:

Observation 1: Based on Rel-16 studies, dynamic TDD operation is mainly feasible for small cells due to low transmit power and antenna gain resulting in improved coexistence with e.g. other-operator deployments in adjacent channels.

Observation 2: Unless the underlying assumptions change as compared to the Rel-16 Cross-link Interference Study Item, adjacent channel coexistence studies should not be repeated for dynamic TDD.

Proposal 1: The dynamic TDD study should focus on identifying enhancements and solutions that allow gNBs to more freely adjust their TDD radio frame configuration in line with their traffic demands by combating cross-link interference.

Observation 3: Dynamic TDD operation has not been studied for a HetNet (two-layer) co-channel scenario, where the Macro layer uses static and fully aligned/synchronized DL-heavy TDD radio frame configuration (to ensure inter-operator coexistence) while the small cell layer operates with dynamic TDD in line with their UL and DL traffic demands.

Proposal 2: For evaluating the performance and identifying enhancements for dynamic TDD operation, focus on a two-layer HetNet scenarios with outdoor macro cells + outdoor or indoor small cells with cochannel deployment (e.g. based on Scenario 1 from TR 36.872).

- Macro layer uses static DL-heavy TDD radio frame configuration (to ensure inter-operator coexistence) while the small cell layer operates with dynamic TDD in line with their UL and DL traffic demands.

For SBFD, we have the following observations and proposals:

Observation 4: On the evaluation methodology for full duplex on non-overlapping PRBs, the selected scenarios should focus on the impacts of the full-duplex-specific sources of interference, namely:

- (intra-cell) gNB self-interference from in-band emissions

- (inter-cell) gNB-to-gNB interference from in-band emissions

- (intra-cell and inter-cell) UE-to-UE interference from in-band emissions

- (inter-cell) gNB-to-gNB interference from adjacent-channel emissions

- (inter-cell) UE-to-UE interference from adjacent-channel emissions

Proposal 3: For evaluating the performance and identifying enhancements for SBFD, focus on co-channel scenarios with same UL-DL PRB partitioning in all the cells and on adjacent channel scenario with legacy TDD.

Proposal 4: For evaluating the performance and identifying enhancements for full duplex on non-overlapping PRBs, focus on the following scenarios for system-level simulations:

- Urban Macro or Dense urban macro, with focus on co-channel and adjacent channel coexistence.

- Indoor small cell deployments, e.g. open office scenario from TR 38.901 or Indoor scenario from TR 38.828, or outdoor small cell deployment, e.g. by only considering the small-cell layer of HetNet Scenario 2A from TR 36.872. Small cell deployment evaluations shall focus on co-channel interference aspects.

Proposal 5: For the modelling of in-band emissions (IBE) for SBFD evaluations, adopt the UE IBE model as specified 3GPP TS 38.101-1, Section 6.4.2.3 including General, IQ Image and Carrier leakage components.

Proposal 6: RAN1 and RAN4 to discuss and agree on models for gNB and UE ICS and gNB IBE.

Proposal 7: For the modelling of inter-subband co-channel interference in SBFD:

- IBE and ICS models are used to determine an inter-subband interference ratio (ISIR) in dB which determines the interference generated by an allocated (i.e. ‘aggressor’) PRB or resource element (RE) to a non-allocated (‘victim’) PRB/RE, relative to the normalized aggressor’s transmit power.

- For gNB self-interference calculations, assume a self-interference suppression value α (in dB) expressing the gNB’s capability to suppress inter-subband interference (determined from ISIR) using a combination of Tx-Rx isolation and analog or digital cancellation.

  - Simulation can be run with different values of α, e.g. from 50 dB (conservative) to 110 dB (highly-optimistic).

- For gNB-gNB or UE-UE links, the resulting co-channel inter-subband interference is only a function of the aggressor’s transmit power, ISIR and coupling gain between the aggressor and victim gNB or UE. No additional interference suppression is considered.

In addition, the following proposals have been made with regards to propagation and traffic models:

Proposal 8: For the traffic models and KPIs for dynamic TDD evaluations, assume the following:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load).

  - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered.

    - For the proposed HetNet scenario with dynamic TDD, it may be further considered to have DL-heavy traffic for the UEs connected to the macro-cells, while the small-cell UEs have balanced or UL-heavy traffic.

Proposal 9: For the traffic models and KPIs for SBFD evaluations, assume the following:

- FTP3 traffic model with large payload size, e.g. 0.5 MBytes.

  - Main KPI: User perceived throughput (UPT).

  - Offered load selected to meet a certain resource utilization (RU) target, e.g. 10% (low load) and 40% (medium load).

  - Both DL-heavy traffic, 4:1, and symmetric 1:1 DL:UL traffic can be considered.

- FTP3 traffic model with small payload size, e.g. 32-1500 Bytes or other similar traffic models as discussed during Rel-16 URLLC/IIoT Study Item in TR 38.824.

  - Main KPIs: CDF of the latency successfully delivered FTP3 packets and/or percentage of UEs meeting certain latency and reliability requirement.

  - Arrival rate per UE can be fixed, e.g. to 100 packet/second in both UL and DL.

  - Note: Companies to explicitly account for half-duplex UE limitations preventing the UE to receive and transmit at the same time.

Proposal 10: For the channel models for dynamic TDD and SBFD system-level evaluations, re-use the existing channel models in TR 38.901 as much as possible for gNB-to-UE, gNB-to-gNB and UE-to-UE links.

- See Table 2 in this contribution for considerations on channel model and UE and BS height for Urban Macro (UMa), Dense Urban and Indoor Hotspot (InH) scenarios.

- RAN1 to discuss whether changes to the LOS probability calculations in UMi and InH channel models are needed to explicitly account for the transmitter-receiver height (in both absolute and relative terms).