3GPP TSG RAN WG1 #109	R1-2204433

e-Meeting, May 9th – 20th , 2022

Agenda item:		9.3.4

Source:	Nokia, Nokia Shanghai Bell

Title:	RF considerations of dynamic TDD and SBFD

Document for:		Discussion and Decision

# Introduction

RAN has agreed in RP-213591 a new Study Item on evolution of NR duplex operation with following objectives [1]:

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

This contribution presents aspects that need to be considered in the study from the RF and implementation perspective. Many of the topics may be evaluated by RAN4, but the results need to be taken into consideration in the RAN1 studies in order for the dynamic TDD and sub-band non-overlapping full duplex (SBFD) features to be successful.

# Discussion

## Adjacent channel

### General

The operation with dynamic TDD and SBFD should be possible without significant constraints on the adjacent channel deployments. Many of the NR TDD bands e.g. around 3.5 to 4 GHz are allocated to multiple operators, with each having spectrum assignments in the order of 100 MHz or less. Operators should be able to deploy their networks without excess restrictions coming from the other operators, or without having to undue protect the other operators networks. Traditionally this coexistence has been guaranteed by synchronization of the TDD UL/DL configurations and UL/DL switching times, and the reasonable RAN4 RF requirements that are derived from this kind of deployment.

The dynamic TDD studies in Rel-16 considered adjacent channel coexistence, but SBFD warrants that some of the aspects are reviewed again. Since the main goals of the SI are related to improving uplink performance, the assumption is that some of the downlink slots are used for uplink. This introduces interference scenarios for base station to base station, and UE to UE.

### Base station to base station interference (co-location)

When one base station is transmitting downlink while an adjacent one is receiving uplink, the interference mechanisms are due to transmitter unwanted emissions, and receiver selectivity.

Transmitter unwanted emissions on adjacent channel

Firstly, the transmitter unwanted emissions generated due to transmitter non-idealities may couple to a receiver, and cause a rise in the receiver’s noise floor. The unwanted emissions are characterized by unwanted emission mask and adjacent channel leakage ratio (ACLR). For simplicity in the case of gNB, the ACLR provides a decent measure of unwanted emissions on the adjacent channel. As a starting point for FR1, 45 dB ACLR as defined in RAN4 may be used (from 38.104 clause 6.6.3):

Table 6.6.3.2-1: Base station ACLR limit BS channel bandwidth of lowest/highest carrier transmitted BWChannel (MHz); BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted; Assumed adjacent channel carrier (informative); Filter on the adjacent channel frequency and corresponding filter bandwidth; ACLR limit. 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90,100; BWChannel; NR of same BW (Note 2); Square (BWConfig); 45 dB. 2 x BWChannel; NR of same BW (Note 2); Square (BWConfig); 45 dB. BWChannel /2 + 2.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 45 dB (Note 3). BWChannel /2 + 7.5 MHz; 5 MHz E-UTRA; Square (4.5 MHz); 45 dB (Note 3). NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA..

Note: the 45 dB assumption does not consider base station ACLR absolute basic limits, which may be more relaxed in some cases.

There is some physical separation between co-located base station antennas, which provides isolation from the transmitter to the receiver. The assumption used in RAN4 base station RF specifications is 30 dB, as listed in 38.104 clauses 6.6.5.2.4 and 7.5.3, among others:

##### 6.6.5.2.4 Co-location with other base stations These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA and/or NR BS are co-located with a BS. The requirements assume a 30 dB coupling loss between transmitter and receiver and are based on co-location with base stations of the same class. ### 7.5.3 Co-location minimum requirements for BS type 1-C and BS type 1-H This additional blocking requirement may be applied for the protection of NR BS receivers when GSM, CDMA, UTRA, E-UTRA or NR BS operating in a different frequency band are co-located with a NR BS. The requirement is applicable to all BS channel bandwidths supported by the NR BS. The requirements in this clause assume a 30 dB coupling loss between interfering transmitter and NR BS receiver and are based on co-location with base stations of the same class.

As the transmitter interfering signal couples to the receiver antenna, the power will be split into the different branches of the receiver. Most of the NR TDD macro networks use active antenna systems (AAS), with multiple transceivers to enable digital or hybrid beamforming. This may help in dealing with the interference.

Figure 1: Adjacent channel unwanted emissions for 100 MHz channel bandwidth 
(Dense Urban: 44 dBm TRP transmit power and 5 dB receiver NF).

Figure 1 illustrates the adjacent channel interference impact at the victim gNB receiver. The total unwanted emissions suppression requirement is based on the receiver noise figure (e.g. 5 dB for macro base station), and the acceptable performance degradation. This total suppression value can be achieved through 1) transmitter ACLR, 2) antenna isolation, and 3) power splitting between victim gNB receivers. Note that SBFD (i.e. zeroing the UL part of the channel) will not in general improve the unwanted emissions on the adjacent channel, as those are heavily dominated by the transmitter linearization process and do not depend on the transmit configuration.

Figure 2 shows the victim gNB receiver performance degradation as a function of ACLR + antenna isolation. For a typical 64TRX device, the adjacent channel suppression should be in the order of 115 dB or better to achieve small penalty, indicating about 70 dB of minimum isolation between the base station antennas in the Dense Urban scenario. Using the RAN4 assumptions of 30 dB isolation and 45 dB ACLR, the receiver performance degradation will be in the order of 40 dB.

Figure 2: gNB receiver desense due to adjacent channel transmitter 
(Dense Urban: 44 dBm TRP transmit power and 5 dB receiver NF).

Note that from the base station unwanted emissions perspective, dynamic TDD and SBFD are similar, and no great difference exists. In dynamic TDD, the adjacent channel interference is integrated across the whole channel, and the 45 dB ACLR provides a good assumption of the leakage power. In SBFD, as stated above, the adjacent channel emissions will be similar as in dynamic TDD, but the victim operator uplink part may be located e.g. in the middle of the adjacent channel, or at the far edge, hence providing some additional offset. However the shape of typical gNB transmitter emissions are relatively flat across the adjacent channel, and only about 1-5 dB improvement could be reasonably expected, if any.

Observation 1: For a typical 64TRX device, the adjacent channel suppression should be in the order of 115 dB or better to achieve small penalty, corresponding to about 70 dB of minimum isolation between the base station antennas in Dense Urban scenario. Using the RAN4 assumptions of 30 dB isolation and 45 dB ACLR, the receiver performance degradation will be in the order of 40 dB.

Observation 2: Due to the FR1 gNB unwanted emissions spectrum shape, which is mostly flat across the adjacent channel, SBFD provides little or no benefit in adjacent channel coexistence compared to dynamic TDD.

Receiver adjacent channel selectivity

Secondly, receiver selectivity must be considered together with the transmitter unwanted emissions. From the receiver perspective in a dynamic TDD or flexible duplexing scenario, the adjacent channel contains strong interference power, that need to be suppressed sufficiently to avoid receiver overload.

Using the above RAN4 assumptions and 64TRX victim in a Dense Urban scenario (44 dBm TRP aggressor gNB transmit power), the adjacent channel interference power per victim gNB receiver is -4 dBm. Considering the baseline 46 dB ACS performance requirement, this leads to residual interference of -50 dBm which is about 40 dB higher than the thermal noise level. Higher isolation and better ACS performance is clearly needed.

Table 1 shows ACLR/ACS calculations for the co-located Dense Urban scenario, with AAS macro and non-AAS (passive antenna) micro base stations. In the table the Baseline-column uses RAN4 minimum requirements while Advanced and Ideal assume better performance. The feasibility of Advanced or Ideal performance has not been evaluated.

Table 1: ACLR and ACS for Dense Urban scenario (co-located gNBs).

As RAN4 only assumes 30 dB antenna isolation between co-located base stations, co-location using current minimum requirements is not possible either for dynamic TDD or flexible duplexing, even for small base station classes. Significantly higher isolation and/or transceiver performance is clearly needed. Since co-location in a macro network is a typical assumption, these constraints should be carefully evaluated in RAN4 to ensure feasibility in actual deployments.

Observation 3: Existing minimum RAN4 RF requirements and assumptions do not enable co-location of base stations using either dynamic TDD or SBFD.

Proposal 1: Send LS to RAN4 for feedback on achievable gNB-to-gNB isolation and on adjacent channel RF performance in co-located deployments.

### Base station to base station interference (coexistence)

This section considers the possibility of adjacent channel non-collocated deployments in the case that RAN4 confirms that co-location of base stations proves difficult. Also, the coexistence scenario should be met regardless of co-location feasibility.

The total gNB-to-gNB isolation requirement would be the same as shown in Table 1. For example, in the ideal case where around 1 dB of receiver desense is accepted and the RF performance assumptions are improved over RAN4 specifications, approximately 70 dB of isolation is required. Note that in co-location, the antenna radiation patterns are typically not relevant as the coupling happens in the near field, but in the coexistence scenario in which separation distances are larger, the antenna gains of the transmitter and receiver are included in the isolation value.

Calculating the peak array gain of a 8x8 element array with 8 dBi element gain can be in the order of 25 dBi when practical losses are assumed. For two base stations facing one other in line of sight, the worst case path loss therefore has to be greater than 70 dB + 25 dB + 25 dB = 120 dB, which indicates 6 km of separation in free space at 3.8 GHz frequency. With UMa LOS path loss model, the separation distance is 4500 meters, and with NLOS path loss 610 meters (using hBS = 25 m at both nodes). These kind of separation distances seem impractical.

On the other hand, it could be assumed that the DL transmit and UL receive beams would not be usually pointing at one another. In such cases, the sidelobe gain might be better applied. The gNB antenna panels will typically have some downtilt, and if it is not expected that UEs above horizon are to be served, directivity towards the other macro base stations could be minimized. Example macro base station antenna radiation patterns as per TR 38.802 Table A.2.1-4 are shown in Figure 3.

Figure 3: Azimuth and Elevation for 4 GHz 8x8 array, 0.5λ horizontal and 0.8λ vertical spacing, 8 dBi element gain, simulated at 3.8 GHz carrier frequency. Azimuth Cut from the array horizontal axis; downtilt applied in Elevation Cut. Blue curve: beam steered at [0°, 0°]. Red curve: beam steered at [-60°, 0°].

It can be observed that if the aggressor gNB beam azimuth is pointing towards the victim gNB, but there is sufficient downtilt so that the beamwidth does not cover the horizon, sidelobe suppression of about 13 dB or more could be expected (blue curve). Further, if the aggressor beam is steered away from the victim gNB, the resulting directivity may be even in the order of 0 dBi or less. Note that the nulls of the radiation pattern cannot be relied upon, as the wider the relative bandwidth (i.e. transmit bandwidth relative to carrier frequency), and the more mismatches there are in the phase shifters, the shallower the nulls are. Hence an envelope of the sidelobes could be a better assumption.

Using the above macro gNB panel configuration as an example, Table 2 shows calculations on the minimum separation distances with the UMa channel model. For micro gNB, the example uses a 4 TRX directional panel of 2 elements per polarization, with 65° horizontal and 35° vertical 3 dB beamwidth. 70 dB of gNB-to-gNB isolation is assumed in all RF performance cases, which could be sufficient for a relatively low receiver desense.

Table 2: gNB separation distances for Dense Urban scenario (adjacent channel coexistence).

From Table 2, it can be observed that careful antenna design is needed to minimize unwanted emissions towards other base stations. It is likely that passive antennas will not have sufficient spatial rejection to mitigate interference, and for dynamic TDD or SBFD to be feasible in the adjacent channel coexistence scenario, AAS are needed in all gNBs. Even then interference might occur.

The micro gNBs without significant beamforming capability, do not seem to reach sufficient separation distances in the example calculation. With AAS and careful design, LOS deployments could become feasible.

Observation 4: Passive antenna systems are not sufficient for dynamic TDD or SBFD adjacent channel coexistence in a non-collocated deployment in Dense Urban scenario (outdoor macro and micro gNBs). AAS is practically required at all outdoor gNBs.

Proposal 2: Send LS to RAN4 for feedback on feasibility of adjacent channel coexistence in outdoor non-collocated deployments with AAS and non-AAS gNBs.

### UE to UE interference

The dynamic TDD studies evaluated UE-to-UE adjacent channel interference, and no great issues were found. If two UEs would be indoor, with one connected to an outdoor macro cell, and the other to an indoor cell with a different TDD configuration, some interference could be observed.

In SBFD, the UE UL allocation during flexible slots could be moved away from the channel edge, e.g. to the center of the base station channel bandwidth, or to the opposite edge. Figure 4 shows simulated emissions for NR100 with full and 20 MHz PRB allocations in different positions.

Figure 4: UE emissions spectrum for different sub-band allocations.

As can be seen from Figure 4, the unwanted emissions on the adjacent channel are suppressed greatly when the UL PRB allocation is narrow, and when moved away from the channel edge. Center (inner) resource allocations are the best from adjacent channel unwanted emissions perspective.

The UE adjacent channel selectivity however does not really improve in SBFD, unless the UE can configure its analog and digital filters to its allocated DL sub-band. Since this would be highly implementation and capability dependent, it is proposed not to assume anything in system level evaluations, and adjacent channel selectivity as is should be used. This means that UE-to-UE ACIR in SBFD would be dominated by ACS, which would result in only slightly improved ACIR over the assumptions used in dynamic TDD studies.

However, the deployment scenarios in SBFD are likely easier from UE adjacent channel coexistence point of view, than in dynamic TDD. Still it may be difficult to rule out deployment or device geometries, which might have coexistence problems.

Observation 5: Adjacent channel UE-to-UE coexistence in SBFD is better than in dynamic TDD.

## Co-channel

### General

SBFD operation introduces various new RF interference scenarios that need to be addressed if the feature is going to be specified. This section discusses these aspects.

### gNB in-channel emissions

The evaluation of SBFD requires modelling of gNB emissions within the own channel, as part of the channel is allocated to uplink transmissions.

As described in clause 2.1.2, the specification 38.104 contains requirements that apply outside of the channel bandwidth, namely ACLR and ACLR absolute basic limits (38.104 clause 6.6.3). In addition to the ACLR requirements, 38.104 specifies requirements on out-of-band emissions within the operating band (38.104 clause 6.6.4).

There are no legacy requirements on emissions within the base station channel bandwidth. Figure 5 illustrates one option how to define such emission model for SBFD system level evaluations.

Figure 5: gNB in-channel emission example.

The unwanted emissions result mainly from non-linear power amplification, and digital pre-distortion (linearization) and crest factor reduction processes. Quantization, thermal, and phase noise may also contribute. The transmitter digital baseband may include channel filters to reduce the noise outside the channel bandwidth.

When a part of the channel is allocated for uplink, it may be possible to estimate the downlink transmitter emissions from ACLR and the emission masks that apply outside the channel. This may require additional sub-band filters in the gNB, but such filters can be reasonably expected as the gNB transceiver architecture needs re-design anyway.

Note that the absolute level of the unwanted emissions do not depend on the number of transmitted DL PRBs. If part of the channel is not transmitted, the total transmit power is lower (assuming the power spectral density of the DL signal is maintained), but the noise introduced in the linearization and crest factor reduction processes will maintain the level of the unwanted emissions constant.

Proposal 3: For SBFD system level evaluations, the gNB emissions on the uplink sub-band can assume the same performance as for adjacent channel, i.e. use ACLR as the starting point.

### Guardband between DL and UL allocations

The specification 38.104 lists the following minimum guardbands from the outermost DL subcarrier to the channel edge:

### 5.3.3 Minimum guardband and transmission bandwidth configuration The minimum guardband for each BS channel bandwidth and SCS is specified in table 5.3.3-1 for FR1 and in table 5.3.3-2 for FR2. Table 5.3.3-1: Minimum guardband (kHz) (FR1) SCS (kHz); 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 40 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. 15; 242.5; 312.5; 382.5; 452.5; 522.5; 592.5; 552.5; 692.5; N/A; N/A; N/A; N/A; N/A. 30; 505; 665; 645; 805; 785; 945; 905; 1045; 825; 965; 925; 885; 845. 60; N/A; 1010; 990; 1330; 1310; 1290; 1610; 1570; 1530; 1490; 1450; 1410; 1370.

The same guardband values could be used as a best case estimate between DL and UL allocations. For out-of-band emissions, the limits may not be as strict as what is needed in FDU operation for in-channel, but nevertheless these may be used as a starting point. In the case of NR100 for example, the minimum guardband would be 845 kHz which equates to 3 PRBs at 30 kHz subcarrier spacing.

It should be noted, that the specified minimum guardband assumes some form of digital filtering to suppress the OFDM modulation noise outside the gNB channel bandwidth. For FDU operation, similar filtering will need to be implemented to suppress the modulation noise within the UL allocation of the band.

Proposal 4: Use the minimum guardband values from 3GPP TS 38.104 clause 5.3.3 between SBFD DL and UL allocations as a starting point in system level evaluations.

### gNB in-channel selectivity

The specification 38.104 clause 7.8 includes requirements for base station receiver to receive a wanted signal on part of the UL PRBs, while having interfering signal on adjacent PRBs at a higher power spectral density. The power difference in the test is in the order of 20 dB, but this requirement is based on legacy coexistence simulations of power levels that might occur in the field. In other words, any of the new interference scenarios of SBFD are not considered in the in-channel selectivity requirement.

Typical base station receivers would not rely on analog channel filters in the adjacent channel selectivity tests, and neither, by definition, in the in-channel selectivity tests. As the receiver digital filters are likely re-designed for any SBFD gNB, the legacy in-channel selectivity requirement is very conservative. As a starting point in system level evaluations, ACS performance could be used for in-channel selectivity.

Observation 6: Legacy gNB in-channel selectivity requirements are very conservative and not based on interference scenarios observed in SBFD deployments.

Proposal 5: As a starting point in SBFD system level evaluations, use the adjacent channel selectivity performance for in-channel selectivity.

### gNB self-interference cancellation

In SBFD with part of the channel allocated for DL transmissions, and part for UL reception in the same slot, the gNB self-interference results from 1) in-channel unwanted emissions, and 2) in-channel selectivity. Both have been discussed above in the previous sections.

The self-interference cancellation requirement is quite significant. Cancellation architectures for gNB are illustrated in Figure 5. In a legacy TDD transceiver, the main leakage paths are through the circulator, and reflections from the antenna path. Typical circulator port-to-port isolation may be in the order of 20 dB, which is similar to the antenna return loss with very good matcing. Hence for a 64TRX device with 44 dBm total transmit power (Dense Urban scenario), each receiver path would see worst case blocking signal of about 6 dBm, which cannot be handled.

Figure 6: Different gNB architecture options for SBFD self-interference cancellation.

Literature shows designs for different kinds of RF cancel paths, that cancel the transmitted signal in the RF domain, before it enters the sensitive receiver circuitry. One option (shown in Figure 5) is a direct path from the PA output to the LNA input, phase and amplitude matched to the transmit signal coupled from the antenna. Performance figures in the order of 20 dB have been demonstrated, which might be feasible to suppress the blocking signal to a more tolerable level. The problems associated with such RF cancellation are the complexity of the circuitry (not scaling well to multi-TRX solutions), and the leakage across transceiver antenna elements (each receiver would require cancellation of multiple transmitters). In addition, the added passive components in the receiver chain increase the noise figure of the base station, even if cancellation is not needed. We do not see such RF cancellation techniques are mature enough for commercial use at the present time.

If separate TX and RX antennas are used, it may be possible to isolate the transmitters from the receivers. Some physical shielding may be incorporated into the antenna construction, further adding to isolation. The drawbacks are that for a given size and weight (of an AAS macro antenna, for example), only half of the elements or less are available for TX and RX, and also results in the loss of reciprocity between TX and RX. However, isolation in the order of 70 dB could be envisioned at about 4 GHz frequency, which in turn allows SBFD with only small penalty to the gNB receiver. No further analog cancellation might be necessary.

Observation 7: Using separate TX and RX antennas seems the most promising SBFD gNB RF architecture.

Some calculations with the earlier RF performance assumptions and 20…70 dB TX-to-RX isolation are presented in Table 3. While there are many different implementation aspects to the receiver linearity, similar approach as for ACS has been adopted for the in-channel selectivity.

Table 3: Self-interference in Dense Urban scenario.

Using the performance assumptions for gNB in-channel emissions and in-channel selectivity, it is possible to create a model for the resulting self-interference in SBFD usage. The proposed model is discussed in R1-2204430.

Proposal 6: Send LS to RAN4 and ask feedback on gNB in-channel emissions (including guard band between DL and UL), in-channel selectivity performance, and achievable self-interference cancellation performance, to be used in system level evaluations.

### UE to UE interference

In SBFD deployments, intra-cell inter-subband interference may occur between UEs that are connected to the same cell, and one is receiving its DL allocation at the same time that another one is transmitting in UL. This is contrary to legacy TDD or dynamic TDD. In dynamic TDD, inter-cell interference may occur from UL to DL, but studies showed that this may be relatively infrequent.

RAN4 specifies the UE in-band emissions in 38.101-1 clause 6.4.2.3. These requirements may be used as the starting point in system level evaluations for SBFD.

RAN4 however does not specify in-channel selectivity for UEs, as this interference scenario is a new one. UE implementations may use analog channel filtering to cope with adjacent channel selectivity requirements, but in SBFD, analog filtering might not be applicable. Whether or not a UE could adjust its receive filtering based on the allocated sub-band, will depend various issues (e.g. UE hardware implementation, and physical layer design e.g. allowed sub-band switching times). In system level evaluations, no sub-band based analog filtering should be assumed.

Observation 8: UE in-band emission requirements are specified by RAN4, but no in-band selectivity requirements exist. For in-band selectivity in system level evaluations, no analog filtering should be assumed.

Proposal 7: Send LS to RAN4 to ask for assumptions for UE in-channel selectivity performance to be used in SBFD system level evaluations.

# Conclusion

The document discussed adjacent channel and co-channel RF interference issues in dynamic TDD and SBFD. The following observations and proposals were made:

Observation 1: For a typical 64TRX device, the adjacent channel suppression should be in the order of 115 dB or better to achieve small penalty, corresponding to about 70 dB of minimum isolation between the base station antennas in Dense Urban scenario. Using the RAN4 assumptions of 30 dB isolation and 45 dB ACLR, the receiver performance degradation will be in the order of 40 dB.

Observation 2: Due to the FR1 gNB unwanted emissions spectrum shape, which is mostly flat across the adjacent channel, SBFD provides little or no benefit in adjacent channel coexistence compared to dynamic TDD.

Observation 3: Existing minimum RAN4 RF requirements and assumptions do not enable co-location of base stations using either dynamic TDD or SBFD.

Observation 4: Passive antenna systems are not sufficient for dynamic TDD or SBFD adjacent channel coexistence in a non-collocated deployment in Dense Urban scenario (outdoor macro and micro gNBs). AAS is practically required at all outdoor gNBs.

Observation 5: Adjacent channel UE-to-UE coexistence in SBFD is better than in dynamic TDD.

Observation 6: Legacy gNB in-channel selectivity requirements are very conservative and not based on interference scenarios observed in SBFD deployments.

Observation 7: Using separate TX and RX antennas seems the most promising SBFD gNB RF architecture.

Observation 8: UE in-band emission requirements are specified by RAN4, but no in-band selectivity requirements exist. For in-band selectivity in system level evaluations, no analog filtering should be assumed.

Proposal 1: Send LS to RAN4 for feedback on achievable gNB-to-gNB isolation and on adjacent channel RF performance in co-located deployments.

Proposal 2: Send LS to RAN4 for feedback on feasibility of adjacent channel coexistence in outdoor non-collocated deployments with AAS and non-AAS gNBs.

Proposal 3: For SBFD system level evaluations, the gNB emissions on the uplink sub-band can assume the same performance as for adjacent channel, i.e. use ACLR as the starting point.

Proposal 4: Use the minimum guardband values from 3GPP TS 38.104 clause 5.3.3 between SBFD DL and UL allocations as a starting point in system level evaluations.

Proposal 5: As a starting point in SBFD system level evaluations, use the adjacent channel selectivity performance for in-channel selectivity.

Proposal 6: Send LS to RAN4 and ask feedback on gNB in-channel emissions (including guard band between DL and UL), in-channel selectivity performance, and achievable self-interference cancellation performance, to be used in system level evaluations.

Proposal 7: Send LS to RAN4 to ask for assumptions for UE in-channel selectivity performance to be used in SBFD system level evaluations.

RAN1 is asked to discuss and agree the proposals.

# References

- RP-220633, Study on Evolution of NR Duplex Operation, CMCC