3GPP TSG RAN WG1 #109-e	R1-2204437

e-Meeting, May 9 – 20, 2022

Agenda item:	9.6.1

Source:	NEC

Title: 	Discussion on potential solutions to further reduce UE complexity

Document for:	Discussion and Decision

Introduction

In RAN#94, study on further NR RedCap UE complexity reduction has been approved [1]. Objective of the study is as follows:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1]

  - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact

  - Potential solutions, which may complement each other, for reducing device complexity are focusing on:

    - UE bandwidth reduction to 5MHz in FR1,

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

    - reduced UE peak data rate in FR1,

      - Possibly including restricted bandwidth for PDSCH and/or PUSCH

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

- Notes:

  - Rel-15 SSB should be reused and L1 changes minimized.

  - Operation in BWP with/without SSB and without/with RF retuning should be considered.

  - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage.

  - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.

Discussion

2.1 Reduction of maximum UE supported bandwidth down to 5MHz

Maximum UE supported bandwidth of 5MHz would overlap with LTE-MTC because LTE-MTC supports 5 MHz channel bandwidth in Rel-14. Features overlapping with LTE-MTC would be understood out of scope of NR. Therefore, support of maximum UE bandwidth of 5MHz can be out of scope of NR.

Observation:

- Reduction of maximum UE supported bandwidth down to 5MHz can be out of scope of NR as it overlaps with LTE-MTC which supports 5MHz channel bandwidth in Rel-14

If reduction of maximum UE supported bandwidth down to 5MHz would be supported, there would be impact on the specification and the network.

In SID, it is stated that Rel-15 SSB should be reused. Bandwidths of SSB in FR1 are summarized in Table 1.

|  | SCS 15kHz | SCS 30kHz |
| --- | --- | --- |
| PBCH | 3.6 MHz | 7.2 MHz |
| PSS, SSS | 1.905 MHz | 3.81 MHz |

Table 1. SS/PBCH block bandwidth in FR1

For SCS=15 kHz, 5 MHz channel bandwidth is enough to detect PSS/SSS and receive PBCH. For SCS=30 kHz, 5 MHz channel bandwidth is enough for PSS/SSS detection but is not enough for PBCH reception.

Observation:

- For detection of SSB, there would be no issue to detect PSS/SSS of either SCS=15 kHz or 30 kHz

Observation:

- There would be no issue to receive PBCH of SCS = 15 kHz

- 5MHz channel bandwidth is not enough to receive full bandwidth of PBCH of SCS = 30 kHz

To support Rel-17 RedCap by a network would increase overhead on radio resources. If maximum 5 MHz channel bandwidth UE would introduce further overhead and/or resource fragmentation, spectral efficiency of the cell would be degraded. So, if maximum 5 MHz channel bandwidth UE would be supported in Rel-18, it will be worth to consider that further increase of overhead on radio resources on top of overhead for Rel-17 RedCap UE should be avoided.

To avoid increase of overhead on radio resources and impact to specifications, it would be preferable that MIB-configured CORESET#0 is reused in idle/inactive as well as SSB. Table 2 captures bandwidth of MIB-configured CORESET#0 for SCS=15 kHz and SCS=30 kHz in FR1. 5MHz CBW is enough to access CORESET#0 of SCS=15 kHz and NRBCORESET=24 while not enough for all other configurations.

| NRBCORESET | SCS 15kHz | SCS 30kHz |
| --- | --- | --- |
| 24 | 4.32 MHz | 8.64 MHz |
| 48 | 8.64 MHz | 17.28 MHz |
| 96 | 17.28 MHz |  |

Table 2. Bandwidth of CORESET#0 in FR1

It would also be preferable to share resources on separate initial UL/DL BWP between Rel-17 RedCap UE and maximum 5 MHz channel bandwidth UE. As bandwidth of common CORESET on a separate initial DL BWP can be flexible, it may be shared between Rel-17 RedCap UE and maximum 5MHz channel bandwidth UE, though sharing common CORESET would not be efficient for Rel-17 RedCap UE.

Observation:

- Reusing MIB-configured CORESET#0 would be preferable from overhead and specification impact point of view, while only a single CORESET#0 configuration could be supported for maximum 5 MHz channel bandwidth UE

- Maximum 5MHz channel bandwidth UE can support CORESET#0 configuration with SCS=15 kHz and NRBCORESET=24

Proposal:

- It should be avoided further increase of overhead on radio resources on top overhead for Rel-17 RedCap UE. It should be considered to share resources of separate initial UL/DL BWP to avoid further overhead on top of overhead for Rel-17 RedCap, if maximum 5MHz bandwidth is supported

2.2. Relaxed UE processing time

The evaluation results in [2] show, relaxed UE processing time (double N1 and N2) also provides cost reduction and low down power consumption. Currently for a normal UE that support capability 2 on a given cell, there are 2 processing capabilities for PDSCH/PUSCH data timing: processing capability 1 is default and processing capability 2 can be enabled by high layer parameter.

If relaxed UE processing time for Rel-18 RedCap UE is imported, then a third timing capability needs to be defined, and should discuss whether the Rel-18 RedCap UE only support the third timing capability, or can support 2 processing capabilities as normal UE, and choose one of them as the default capability.

There are some scheduling timing related parameters in current specification which are directly associated with the PDSCH/PUSCH processing time, for example, default TDRA table for PUSCH preparation, the range of K1 and K2, default HARQ-ACK feedback timing set, HARQ-ACK information in response to a SPS PDSCH release etc., so they may need to be updated accordingly.

There are also some hardcode timing related parameters which are associated with the processing capability implicitly, for example, timing for PDSCH corresponding to SI-RNTI, scheduling timeline condition between a dynamic PDSCH and a SPS PDSCH in Clause 5.1 of TS38.214 etc., suggest to take care and re-estimate these hardcode parameters.

Proposal:

- If UE with relaxed UE processing time would be supported

  - Update all timing related parameters which are directly associated with the PDSCH/PUSCH processing time

  - Take care and re-estimate the hardcode timing related parameters which may be associated with the processing capability implicitly

Summary

In this contribution, we discussed potential solutions to further reduce UE complexity and provide the following observation and proposals.

2.1 Reduction of maximum UE supported bandwidth down to 5MHz

Observation:

- Reduction of maximum UE supported bandwidth down to 5MHz can be out of scope of NR as it overlaps with LTE-MTC which supports 5MHz channel bandwidth in Rel-14

Observation:

- There would be no issue to detect PSS/SSS of either SCS=15 kHz or 30 kHz

Observation:

- There would be no issue to receive PBCH of SCS = 15 kHz

- 5MHz channel bandwidth is not enough to receive full bandwidth of PBCH of SCS = 30 kHz

Observation:

- Though reusing MIB-configured CORESET#0 would be preferable from overhead and specification impact point of view, only a single CORESET#0 configuration could be supported for maximum 5 MHz channel bandwidth UE

- Maximum 5MHz channel bandwidth UE can support CORESET#0 configuration with SCS=15 kHz and NRBCORESET=24

Proposal:

- It should be avoided further increase of overhead on radio resources on top overhead for Rel-17 RedCap UE, if maximum 5MHz bandwidth is supported

- It should be considered to share resources of separate initial UL/DL BWP to avoid further overhead on top of overhead for Rel-17 RedCap, if maximum 5MHz bandwidth is supported

2.2. Relaxed UE processing time

Proposal:

- If UE with relaxed UE processing time would be supported

  - Update all timing related parameters which are directly associated with the PDSCH/PUSCH processing time

  - Take care and re-estimate the hardcode timing related parameters which may be associated with the processing capability implicitly

Reference

[1] RP-213661	New SID on Study on further NR RedCap UE complexity reduction	SID

[2] TR 38.875 V17.0.0