3GPP TSG RAN WG1 #109-e				 	         		R1-2204440

e-Meeting, May 9th – 20th, 2022

Agenda Item:  9.1.1.1

- Source:	ITRI

- Title:	Discussion on unified TCI framework extension for multi-TRP

Document for:	Discussion and Decision

Introduction

In RAN meeting #94e, the work item on MIMO evolution for downlink and uplink was approved for Release 18. One of the key objectives in WID [1] is:

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

According to above description, the unified TCI framework enhancement focusing on M-TRP will be one of the major issues in the following meetings in Rel-18. Thus, in this contribution, we provide our views for extended unified TCI framework.

Unified TCI framework

In high mobility scenario, channels and reference signals within intra-band CCs may use same beam for DL reception and use same beam for UL transmission. In this case, signalling overhead could be reduced by indicated one or two TCI states for those channels and reference signals within intra-band CCs. Therefore, in Rel-17, two types of unified TCI framework on single-TRP operation are introduced. One type is joint TCI state and another one is separate TCI state. In the joint TCI state type as shown in Fig.1 (a), if one TCI state indicated by a codepoint of TCI field in a DCI, the indicated TCI state is used for determining both the DL QCL information and the UL TX spatial filter. In the separate TCI state type as shown in Fig.1 (b), a codepoint of TCI field indicates a UL TCI state and/or a DL TCI state, where the UL TCI state is used for determining the UL TX spatial filter and the DL TCI state is used for determining the DL QCL information.

Fig.1 TCI field for unified TCI framework

Besides, L1-based beam indication with Rel-17 unified TCI is supported to indicate the TCI state for common beam updating through DCI format 1_1/1_2 with or without DL assignment, as shown in Fig. 2, which could reduce the latency delay of beam updating for those channels or reference signals of which TCI state could be configured by RRC or activated by MAC CE.

Fig.2 L1-based beam indication if joint TCI state type is configured

The benefit of unified TCI framework and L1-based beam indication is not only for single TRP operation, but also could be extended to multiple TRP operation. Since the number of TRPs is up to two to serve UE, two common DL and two common UL beams is sufficient for unified TCI framework. Hence, if the unified TCI framework is extended to multi-TRP use case, up to two DL TCI states and up to two UL TCI states can be considered.

Proposal 1: Support up to two DL TCI states and up to two UL TCI states for unified TCI framework on multi-TRP operation.

If separate TCI state type is configured for multi-TRP operation, the number of codepoints of TCI field may be not sufficient for all the possibility of indicated TCI states because there may be up to two DL TCI states and up to two UL TCI states indicated by a codepoint. In this case, we can consider to increase the number of codepoints of TCI filed, or to introduce the second TCI field where the original TCI field is associated with one TRP and the second TCI filed is associated with another TRP.

When UE has the capability of beam correspondence, joint TCI state is suitable for multi-TRP operation. However, if one of UE panel is close to human body as shown is Fig. 3, the DL beam and UL may not be the same for considering maximum power emission (MPE). In this case, two type of unified framework can be configured, where joint TCI state type is associated with a TRP without considering MPE, and separate TCI state type is associated with another TRP with MPE issue.

Fig.3 Joint common beam and separate common beam

Proposal 2: UE cab be configured both separate TCI state and joint TCI state simultaneously for considering MPE

For high mobility scenario, the UE would not only be moving within a coverage of TRP, but also would moving ahead to the coverage of another TRP. In this case, UE may be from one TRP operation into two TRP operation, and then back to one TRP operation. Therefore, dynamic switch between single TRP operation and multiple TRP operation on unified TCI framework is needed for smoothly communication among TRPs.

Proposal 3: Support dynamic switch between single TRP operation and multiple TRP operation on unified TCI framework

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Support up to two DL TCI states and up to two UL TCI states for unified TCI framework on multi-TRP operation

Proposal 2: UE cab be configured both separate TCI state and joint TCI state simultaneously for considering MPE

Proposal 3: Support dynamic switch between single TRP operation and multiple TRP operation on unified TCI framework

References

[1]    RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”