3GPP TSG RAN WG1 #109-e		R1-2204498

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.2.1

Source:	Spreadtrum Communications

Title:	             Discussion on general aspects of AI/ML framework

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. The objective of the study item is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact, and set up a common AI/ML framework, including functional requirements of AI/ML architecture, which could be used in future projects.

For the study, in our mind, the first thing is to have one clear/common picture on general aspects of AI/ML work, e.g., the term, AI/ML model, the lifecycle of AI/ML model and others, then more detailed evaluation/study work per use case can be effectively carried out.

In this paper, we would share our opinions on the general aspects of AI/ML framework.

Discussion

Notation/terminology

It is the first time to introduce AI/ML technology being hot in IT industry, into wireless air interface field. Thus, it may be completely new to some people in the group. But fortunately, in R17 3GPP has one study on enhancement for data collection for NR and ENDC  (FS_NR_ENDC_data_collect) , and has formed one document TR37.817 [2]. The document includes the terminology, general framework, and use cases and solutions for Artificial Intelligence in RAN. It can help us better understand how AI to work in 3GPP, even if the use cases are totally different.

The following terms on AI/ML are listed in TR37.817, and can be borrowed for the SID as the starting point. New terminologies on AI/ML can be added, if needed. It is natural that conventional terminology in wireless air interface can be inherited.

-	Data collection: Data collected from the network nodes, management entity or UE, as a basis for AI/ML model training, data analytics and inference.

-	AI/ML Model: A data driven algorithm by applying machine learning techniques that generates a set of outputs consisting of predicted information and/or decision parameters, based on a set of inputs

-	AI/ML Training: An online or offline process to train an AI/ML model by learning features and patterns that best present data and get the trained AI/ML model for inference.

-	AI/ML Inference: A process of using a trained AI/ML model to make a prediction or guide the decision based on collected data and AI/ML model.

Proposal 1: The terms of TR37.817 (i.e., Section 3 of TR37.817) can be as the starting point of the terminology on AI/ML for the study.

Functional Framework of AI/ML

TR37.817 also provides one functional framework for AI/ML in RAN level, illustrated in below Figure 1.

Figure 1. Functional Framework for RAN Intelligence

The detailed explanations of all terminologies in the framework or Fig.1 can be found in TR37.817, and would not be repeated here. In our opinion, the functional framework also could be referenced for AI/ML enabled in air interface, while some enhancement can be considered if needed.

Proposal 2: The general framework of TR37.817 (i.e., Section 4 of TR37.817) also can be as the starting point of AI functional framework for the study. Further enhancements also can be considered if needed.

AI/ML model

The SID has clearly stated that specific AI/ML models are not expected to be specified and are left to implementation. But for purpose of evaluation, reference AI/ML models for training and inference are needed to be defined for evaluation and cross checking. The details on AI/ML model needs to be provided, including the type of natural network, layer structure, parameters, loss function, complexity, size and so on. In addition, since different use cases may correspond to different characterizes which AI/ML can learn, the reference AI/ML model can be considered separately for each use case.

Proposal 3: For evaluation and cross checking, reference AI/ML model for each use case can be considered separately.

- The type of natural network, layer structure, parameters, loss function, complexity of the AI/ML model should be provided.

Regarding the complexity of AI/ML model/algorithm, in general, it would at least cover the baseband processing capability requirement, the storage requirement, and power cost. Flops (floating point operations) can be used to denote the computation amount, but it is not enough to demonstrate the complexity. Like PDSCH/PUSCH processing capability 1 and 2 defined in R15, when we define the baseband processing capability requirement for AI/ML operation, the computation time requirement and computation amount should be combined to characterize the baseband processing requirement. The layer structure, parameters and others influence the requirement of buffer size. In addition, the storage requirement also depends on the generalization of AI/ML model. For example, if the generalization is poor, more AI/ML model should be storage. Power cost is another factor we should consider, especially at UE side. How to define the power model for AI/ML can be discussed.

Proposal 4: The computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm.

Collaborative level

During the process of approving the SID, the moderator has ever proposed the following collaborative level between UE and gNB.

- 0a) No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes. E.g. MIMO detection/channel estimation

- 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms purely implementation based. E.g., RS reduction

- 1) Inter-node assistance to improve the respective nodes AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes. E.g. CSI compression and recovery

- 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes. E.g., Federated learning

From above, we can see that there is no standard impact for collaborative level 0a), and it is totally up to implementation. Given limited TU budget and many use cases to be evaluated, we suggest only to consider collaboration level 0b), level 1) and level 2) for the study.

Proposal 5: Suggest to consider the following collaborative level for study

- 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms purely implementation based.

- 1) Inter-node assistance to improve the respective nodes AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes.

- 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes.

Lifecycle management of AI/ML model

The lifecycle of AI/ML model typically includes these stages of model training, model validation, model test, model deployment, model inference, model monitoring, and model update. In general, model generation includes model training, model validation, and model test. Next, we would provide our views on these stages of AI/ML model.

- AI/ML model generation

For online training of AI/ML model, additional complexity is needed, e.g., a large amount of data needed to be storage, and tackled. It can not be beared at UE side, e.g, for smartphone. Thus, we suggest to focus offline AI/ML model training at the first stage. Advanced UE, e.g., laptop, can be considered in future.

Proposal 6: Offline AI/ML model training is the first priority.

In general, AI/ML model generation can be located at UE side or gNB side, or at both sides, which can be use case specific. F	or example, when the function of CSI predication is considered at UE side, AI/ML model generation can be located at UE side; when the function of CSI predication is considered at gNB side, AI/ML model generation can be located at gNB side.

In reality, the channel status of wireless communication is dynamic and complex. The generality of one AI/ML model should be enough good, otherwise, much more expense would be brought about, e.g., the storage requirement would be sharply increasing. Thus, how to enhance the generality of AI/ML model should be specially considered during AI/ML model generation. For example, mixed data sets can be considered.

Proposal 7: The generalization of AI/ML model should be specifically considered during AI/ML model generation.

- AI/ML model deployment

AI/ML model deployment depends on collaborative level, which is use case specific. For example, for collaboration level 0b, since the AI/ML model generation and inference are located at the same side, no model transfer is needed, and model deployment can be up to implementation. Meanwhile, for collaboration level 2, model exchange is needed. On the flip side, whether one AI/ML model can be deployed at one side also depends on whether the side has the capability of bearing the complexity brought about by AI/ML algorithm. The model complexity and the capability of UE/gNB should be matched.

Proposal 8: The AI/ML model can be transferred, or not, which is use case specific.

- AI/ML model inference

In general, AI/ML model inference can be located at UE side or gNB side, or at both sides, which can be use case specific. For example, for the sub use case – CSI prediction, the model inference can be located at UE side or gNB side. While for sub use case – CSI compression and recovery, the model inference are located at both UE side and gNB side.

Proposal 9: AI/ML model inference can be located at UE side or gNB side or both UE and gNB side, which is use case specific.

For evaluation, one reference AI/ML model for each use case should be considered to align among companies. Other than legacy typical KPIs in 3GPP, e.g., throughput, BLER, new metrics also should be considered to reflect the performance and complexity of AI/ML model, e.g., GCS [3].

From the perspective of standard, how to enable and configure the function enabled by AI/ML should be discussed. On the other hand, the coexistence with legacy operations should also be considered. For example, since the high complexity and power cost issue, in some scenarios（e.g., no high performance requirement）, the function of AI/ML can be turned off to save power.

- AI/ML model monitoring and update

In our understanding, all of use cases still can be embedded into current communication procedure. In details, CSI feedback and beam management still can reuse the current CSI framework, and Positioning also can reuse the current positioning mechanism. Thus, legacy monitoring mechanism and update can be reused. Besides, higher performance requirement should be set for AI/ML enabled feature when monitoring AI/ML model, since the better performance is the reason why we support AI/ML model with higher complexity.

Proposal 10: Higher performance requirement should be considered when monitoring AI/ML model.

The generalization of AI/ML model is one factor influencing AI/ML model updating frequency. For example, if AI/ML model is scenario specific, when UE moves from one place to another place, the AI/ML model should be updated. Frequent AL/ML model updating would bring much overhead, and deteriorate performance.

Proposal 11: The better generalization of AI/ML model should be strived, to avoid frequent AI/ML model updating.

Dataset

According to the SID, the dataset can be derived based on the statistical model from TR38.901 and TR 38.857. Although the dataset can be dependent on use case, the common assumptions in dataset construction for training, validation and test for the selected use cases are needed for cross-checking. In addition, for AI/ML model generalization, mixed data from different statistical models can be considered, training, validation, test and inference stage.

As for field data, since there is no consensus among people during the stage of SID approve, it is still open for the study item. In our understanding, field data is very important to ensure the trained AI/ML model can work well in real word. After all, the final dream of the study is to make the AI/ML work in real product. Field data can be considered in training, validation, test stage. Field data should also be public, to align the evaluations among companies.

Proposal 12: Field data should be considered.

Conclusion

In this contribution, we provide our opinions on general aspects of AI/ML:

Proposal 1: The terms of TR37.817 (i.e., Section 3 of TR37.817) can be as the starting point of the terminology on AI/ML for the study.

Proposal 2: The general framework of TR37.817 (i.e., Section 4 of TR37.817) also can be as the starting point of AI functional framework for the study. Further enhancements also can be considered if needed.

Proposal 3: For evaluation and cross checking, reference AI/ML model for each use case can be considered separately.

- The type of natural network, layer structure, parameters, loss function, complexity of the AI/ML model should be provided.

Proposal 4: The computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm.

Proposal 5: Suggest to consider the following collaborative level for study

- 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms purely implementation based.

- 1) Inter-node assistance to improve the respective nodes AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes.

- 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes.

Proposal 6: Offline AI/ML model training is the first priority.

Proposal 7: The generalization of AI/ML model should be specifically considered during AI/ML model generation.

Proposal 8: The AI/ML model can be transferred, or not, which is use case specific.

Proposal 9: AI/ML model inference can be located at UE side or gNB side or both UE and gNB side, which is use case specific.

Proposal 10: Higher performance requirement should be considered when monitoring AI/ML model.

Proposal 11: The better generalization of AI/ML model should be strived, to avoid frequent AI/ML model updating.

Proposal 12: Field data should be considered.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- 3GPP TR 37.817: " Study on enhancement for Data Collection for NR and EN-DC "

- R1-2204499, “Discussion on evaluation on AIML for CSI feedback enhancement” ,  Spreadtrum Communications