3GPP TSG RAN WG1 #109-e		 R1-2204499

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.2.2.1

Source:	Spreadtrum Communications, BUPT

Title:	             Discussion on evaluation on AIML for CSI feedback enhancement

Document for:	Discussion and decision

Introduction

In RAN#94e meeting, one SID on AI/ML for Air interface is approved [1]. Three use cases are identified as initial set of use case shown below, and representative sub use cases should be recognized by RAN#98.

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this paper, we would present our opinions on sub use cases, evaluation methodology, and KPI for the use case of CSI feedback enhancement.

Discussion

In current system, CSI feedback is used for DL link adaption. Based the measurement on CSI-RS, UE could achieve CSI results, including RI, CQI and PMI. Then, in light of the CSI report, gNB could adopt reasonable scheduling strategy to improve DL performance. One typical flow of CSI acquisition and related procedure is illustrated below in Figure 1.

Figure 1 one illustration of CSI acquisition and related procedure

Next, we will present our views on the sub use cases of CSI feedback, taking the examples in Fig.1.

- For sub use case - RS overhead reduction,

  - By reducing the density or port number of CSI-RS , e.g., in t1, or t4, the overhead can reduced;

- For sub use case - CSI compression and recovery,

  - By reducing the payload size of CSI report, e.g., in t2 or t5, the overhead also can be reduced;

  - In addition, the CSI accuracy can be improved, if the CSI report includes high resolution channel information. This is because that in legacy method the CSI report includes codebook information, which is the coarse quantization of channel information.

- For sub use case - CSI predication,

  - By reducing the number/occasions of CSI measurement and reporting, e.g., no CSI-RS transmission in t4, and no CSI reporting in t5, the overhead also can be reduced, and UE power cost also can be saved;

  - On the other hand, the CSI accuracy can be improved, if the CSI information is based on the predicated channel information of DL transmission occasion. This is because that in legacy CSI measurement and reporting, the transmission occasion of NZP CSI-RS for the measurement is earlier than its target DL transmission occasion, the CSI information may not match with the DL transmission.

From the above initial analysis, the sub use case - RS overhead reduction could achieve relatively less gain. In addition, the gNB-UE collaboration level for sub use case - RS overhead reduction and sub use case - CSI predication seems to be the same. Given the limited TU budget, we think the sub use case - RS overhead reduction can be depriorized.

Furthermore, when referring to the predication functionality of AI/ML operation, the sub use case - CSI predication is overlapping with the sub use case of beam predication in time domain. Either one can be selected as the representative use case. With what we have said, we have no strong opinion on this. If people think time is enough, we are fine to have evaluation on both.

At our side, we think the sub use case - CSI compression and recovery is worthy of spending time on evaluation. It is one most typical use case where both UE and gNB participate in AI/ML inference, and the performance gain can be expected at least based on the initial analysis.

Proposal 1: Suggest to take sub use case - CSI compression and recovery as the first priority for evaluation for CSI feedback.

Sub use case - CSI compression and recovery

- Evaluation assumption

In Rel-15, Type II codebook is introduced to support high-resolution CSI feedback. Further enhancement on TypeII codebook is supported in Rel-16 where frequency codebook is introduced, to improve system performance and reduce the overhead. Since the benefits of CSI compression and recovery is reducing overhead and improving the accuracy, we prefer to take the performance of R16 Type II codebook as one baseline for comparison.

Proposal 2: R16 TypeII codebook can be as one performance baseline on behalf of the conventional CSI feedback.

Since Rel-16 Type II would be as the performance baseline, it is reasonable to take the SLS evaluation assumption for CSI enhancement in Rel-16 (seen in Annex 1) [2] as the starting point.

From our side, for CSI compression and recovery, some information of the evaluation assumption of R16 CSI enhancement should be updated. For example,

- To reflect the generalization, scenario should be diverse as far as possibly, i.e., Urban Macro, UMi, and indoor should be included.

- Baseline for performance evaluation:

  - On behalf of conventional CSI feedback, R16 Type-II codebook is one baseline for performance evaluation

  - Reference AI/ML model is another baseline for calibration.

Proposal 3: Suggest to take the SLS evaluation assumption for CSI enhancement in Rel-16 as the starting point. Additional updates can be considered.

- KPI

For CSI compression and recovery, since it is one mechanism of CSI feedback, all of conventional performance requirement for CSI feedback naturally should be considered, e.g., throughput and CSI feedback overhead from system level simulation. Besides, new metrics should be considered to reflect the AI/ML model.

- GCS

At present, both in industry and academic, Generalization Cosine Similarity (GCS) has been widely used as one tool to evaluate the similarity between the original eigenvector and recovery eigenvector as the below equation shows that:

ρ=E[vHvv2v2]									(1)

where v denotes the recovery eigenvector, and v denotes the original eigenvector. GCS can characterize the learning capability of AI/ML.

- Complexity

In conventional CSI feedback, complexity has never been taken into consideration, since the complexity is relatively small and accepted. With what we have said, Type II codebook has not been successfully commercialized for its higher complexity than TypeI codebook.  For data driven AI/ML algorithm, it is well-known for the huge computation capability requirement. Thus, the complexity should be considered as one KPI, to ensure it can be deployed in reality. Otherwise it is just paper work.

Regarding the complexity of AI/ML model/algorithm, in general, it would at least cover the baseband processing capability requirement, the storage requirement, and power cost. Flops (floating point operations) can be used to denote the computation amount, but it is not enough to demonstrate the complexity. Like PDSCH/PUSCH processing capability 1 and 2 defined in R15, when we define the baseband processing capability requirement for AI/ML operation, the computation time requirement and computation amount should be combined to characterize the baseband processing requirement. The layer structure, parameters and others influence the requirement of buffer size. In addition, the storage requirement also depends on the generalization of AI/ML model. For example, if the generalization is poor, more AI/ML model should be storage. Power cost is another factor we should consider, especially at UE side.

Based on above, we have the following proposal:

Proposal 4: All of conventional performance metric for CSI feedback should be continued to be considered. Some new metrics should be additionally considered, e.g.,

- GCS

- The computation complexity

  - The computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm.

Conclusion

In this contribution, we provide our opinions on the sub use cases of CSI feedback, and evaluation methodology on AI/ML for CSI feedback enhancement. We have the following proposals:

Proposal 1: Suggest to take sub use case - CSI compression and recovery as the first priority for evaluation for CSI feedback.

Proposal 2: R16 TypeII codebook can be as one performance baseline on behalf of the conventional CSI feedback.

Proposal 3: Suggest to take the SLS evaluation assumption for CSI enhancement in Rel-16 as the starting point. Additional updates can be considered.

Proposal 4: All of conventional performance metric for CSI feedback should be continued to be considered. Some new metrics should be additionally considered, e.g.,

- GCS

- The computation complexity

  - The computation time requirement, the amount of computation, buffer size requirement, and power cost should be jointly considered to characterize the complexity of AI/ML model/algorithm.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Draft Report of 3GPP TSG RAN WG1 #94b-e

- Annex 1: Evaluation assumption for CSI enhancement in Rel-16

Table 2-1. SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-15 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. |