3GPP TSG RAN WG1 #109-e		R1-2204501

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.2.3.2

Source:	Spreadtrum Communications

Title:	             Discussion on other aspects on AI/ML for beam management

Document for:	Discussion and decision

Introduction

According to R18 AI/ML for Air interface SID [1], the following objective is included in the scope:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we will discuss some representative sub use cases and potential specification impact on AI/ML for beam management.

Discussion

Representative sub use cases

According to current NR specification, the indicated beam should be selected only from the beams measured and reported by UE. In other words, UE has to measure all candidate beams to find the best one. Therefore, the current beam selection procedure will cause large resource overhead as well as latency. The problem will get worse when the number of gNB beams and UE beams increases.

For a downlink channel between a UE and its serving gNB, different beams can be treated as different spatial filters. Thus, there will be some correlation information among the effective channels received by UE. It is possible that each received beam contains some information about the channel property, and hence helps to select the best beam corresponding to the current channel. However, it is not easy to get an analytical expression to estimate the best beam based on measurement result. With the help of AI/ML, it is possible to generate a model where the input is the measurement result of each beam and the output is the selected best beam.

Figure 2.1-1. AI/ML based beam selection

Proposal 1: AI/ML based beam selection can be considered as one of the representative sub use cases.

According to current NR specification, for DL beam management, the reported beams corresponds to the slot where the beams have been measured. For beam indication, beam application time is required for UE to prepare the beam before transmission/reception. If a UE is changing its location over time, the best beam may also change. Therefore, based on current beam management procedure, the indicated beam may not be the best beam for the target slot.

In practice, a UE usually moves with a constant speed or angular speed within a time interval. At the same time, the best beam changes from one to another with a fixed pattern. Without the information of UE location and the 3D model of the deployment scenario, it is not easy to predict the best beam since the UE’s trajectory projected onto the sphere centered on the base station is unknown. AI/ML can be considered as a potential solution to predict UE’s trajectory, and hence helps to find the best beam for the slot when UE transmission/reception happens.

Figure 2.1-2. AI/ML based beam prediction

Proposal 2: AI/ML based beam prediction can be considered as one of the representative sub use cases.

Potential specification impact

Regarding AI/ML based beam selection, since different gNB may have different antenna array structures, the correlation information between the beams can be different for each gNB. In other words, the dataset could be different for different gNBs. Therefore, in order to save the overhead of information exchange, AI/ML related training should be conducted at gNB side. Besides, since the dataset doesn’t require update frequently, offline training should be enough. Regarding inference operation, it is possible to be conducted at UE side as long as the overhead of the assistance information related to AI model management is acceptable.

Proposal 3: For AI/ML based beam selection, training could be conducted by gNB, while inference could be conducted by UE for better performance.

With the assumption that training is conducted by gNB while inference is conducted by UE, there are some potential specification impacts. The first potential enhancement is to support AI model related information configuration from gNB to UE by higher layer parameters. The configuration can be delivered by RRC for each serving cell or each TRP. MAC CE and/or DCI can also be considered to bring some configuration flexibility, if necessary.

Proposal 4: For AL/ML based beam selection, support to configure AI model related information to UE.

From physical layer point of view, AI/ML based beam selection includes beam measurement and reporting, which is the same as current beam management procedure. Therefore, the current CSI framework can be reused. UE can be configured with one or more resources for AI/ML based beam measurement in a resource setting. The 1/2-port CSI-RS resource and SSB can be reused. Regarding beam reporting, different from the current reporting rule, a new reporting quantity should be defined to allow UE reporting an index of a beam/resource that was not directly measured. What’s more, since UE and gNB need to implement the same AI model, UE is required to report its capability related to the AI model. For example, whether UE support AI/ML based beam selection, which type of AI model is supported, etc.

Some other aspects related to UE processing capability can be further studied. For example, whether to define CPU for AI/ML based beam reporting, how to define new CSI processing time requirement, and the relationship between AIML based CSI report and legacy CSI report.

Proposal 5: For AL/ML based beam selection,

- The current CSI framework can be reused as starting point

- The 1/2-port CSI-RS resource and SSB can be reused as measurement resource

- Define new reporting quantity for beam that was not directly measured

- Define new UE processing capability for AI/ML based beam reporting

Regarding AI/ML based beam prediction, since the purpose is to select the indicated beam, it’s reasonable that the inference operation can be conducted by gNB. Also, gNB has absolute control of when and which beams should be measured and reported, the dataset can be maintained by gNB. Therefore, the training operation can also be conducted by gNB.

Proposal 6: For AI/ML based beam prediction, both training and inference could be conducted by gNB.

Based on the assumption that both training and inference could be conducted by gNB, AI model is implemented by gNB and it could be transparent to UE. Therefore, we don’t see any signaling is needed to be defined.

Proposal 7: For AL/ML based beam prediction, AI model is implemented by gNB and transparent to UE.

Regarding the specification impact, in our views, Rel-17 TCI framework can be reused. Based on current specification, up to 128 DL/joint TCI states and up to 64 UL TCI states can be configured per BWP per CC by RRC, which are assumed to cover all possible beams in this serving cell. After RRC (re-)configuration, MAC CE is used to activate some of the TCI states, and DCI can be further used to indicated a DL/joint TCI and/or UL TCI state. Based on the analysis, in our views,  no more enhancement is required.

Proposal 8: For AL/ML based beam prediction, the Rel-17 TCI framework can be reused, no more enhancement is required.

Conclusion

In this contribution, we provided our opinions on representative sub use cases and potential specification impact on AI/ML for beam management. The following proposals have been achieved:

Proposal 1: AI/ML based beam selection can be considered as one of the representative sub use cases.

Proposal 2: AI/ML based beam prediction can be considered as one of the representative sub use cases.

Proposal 3: For AI/ML based beam selection, training could be conducted by gNB, while inference could be conducted by UE for better performance.

Proposal 4: For AL/ML based beam selection, support to configure AI model related information to UE.

Proposal 5: For AL/ML based beam selection,

- The current CSI framework can be reused as starting point

- The 1/2-port CSI-RS resource and SSB can be reused as measurement resource

- Define new reporting quantity for beam that was not directly measured

- Define new UE processing capability for AI/ML based beam reporting

Proposal 7: For AL/ML based beam prediction, AI model is implemented by gNB and transparent to UE.

Proposal 8: For AL/ML based beam prediction, the Rel-17 TCI framework can be reused, no more enhancement is required.

Reference

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface