# 3GPP TSG-RAN-WG1 #109-e	R1-2204502

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.12.1

Source:	Google Inc.

Title:	Remaining reliability issues of MBS for RRC_CONNECTED UEs

Document for:	Discussion and decision

# 1. Introduction

In this document, we provide our views on multiplexing type-2 HARQ codebook in PUSCH for multiple MBS.

# 2. Discussion

## 2.1 Multiplexing type-2 HARQ codebook in PUSCH

Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded Agreement When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. - The UL-DAI for multicast is 1-bit for Type-1 codebook - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs. - The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded - FFS: details of the UL DAI field

In the last meeting, RAN1 has agreed to introduce 2 additional UL DAI bits to support type-2 HARQ codebook for all configured G-RNTIs, but how to apply these 2 bits to multiple separated DAI for multiple G-RNTI still need further study. There are 3 alternatives listed in the agreement. However, the details about how to implement these alternatives are not captured. Thus in this document, we provide our views on how these alternatives may work or may not work.

In legacy DAI mechanism to report a type-2 HARQ codebook in PUCCH, if only C-DAI is available, the UE may miscalculate the HARQ codebook size if the last DCI is missing. In MBS, RAN1 has agreed that HARQ feedback of each G-RNTI associates to a separate DAI and a sub-codebook, then all sub-codebooks will be concatenated together to form the HARQ codebook. In this regard, if a last DL DCI of at least one of the G-RNTIs is missing, the UE will miscalculate the associated sub-codebook size, thereby generates a codebook that the base station cannot read.

Observation 1: The probability that UE miscalculates the HARQ codebook size is positive correlated to the number of configured G-RNTIs.

In unicast transmission, when a base station schedules a UE to multiplex a type-2 HARQ codebook in PUSCH, the base station indicates a 2 bits UL-DAI in the UL DCI, which protects an error case that UE miscalculate the codebook size due to more than 4 contiguous last DL DCIs missing. However, based on the agreement above, 2 bits UL-DAI will be introduced for all configured G-RNTIs. Then a question to us is whether a base station can always find a perfect UL-DAI for multiple separated DAI processes?

Considering the example in Figure 1, a UE is configured with 4 G-RNTIs in a single cell, and a base station respectively sends 2, 3, 4 and 1 TBs with these G-RNTI 1, 2, 3, and 4. Assuming the base station can indicate arbitrary UL-DAI value. If the indicated VT-DAI=1, and the UE misses the scheduling DCI of TB_2, the UE will assume that there is only 1 TB has been transmitted with G-RNTI 1, thereby size of the 1st sub-codebook of multicast will be incorrect. Likewise, if the base station indicates VT-DAI=2 and the UE misses the scheduling DCI of TB_c, the UE will assume that there is only 2 TBs have been transmitted with G-RNTI 2, which leads to incorrect 2nd sub-codebook size. Similarly, VT-DAI equals to 3 and 4 may also lead to incorrect sub-codebook size determination in some cases that a last DCI of a G-RNTI is missing.

Observation 2: The miscalculation probability of one DCI missing depends on how many last DCIs that indicates a VC-DAI equal to (VT-DAI+1)mod4.

| PDCCH occasion index | m=0 | m=1 | m=2 | m=3 | m=4 |
| --- | --- | --- | --- | --- | --- |
| CC #0 |  |  | G-RNTI 1 DCI 1_1 TB_1 VC-DAI=1 | G-RNTI 1 DCI 1_1 TB_2 VC-DAI=2 | C-RNTI DCI 0_1 VT-DAI=[1, 2, 3, 4] |
|  | G-RNTI 2 DCI 1_1 TB_a VC-DAI=1 |  | G-RNTI 2 DCI 1_1 TB_b VC-DAI=2 | G-RNTI 2 DCI 1_1 TB_c VC-DAI=3 |  |
|  | G-RNTI 3 DCI 1_1 TB_w VC-DAI=1 | G-RNTI 3 DCI 1_1 TB_x VC-DAI=2 | G-RNTI 3 DCI 1_1 TB_y VC-DAI=3 | G-RNTI 3 DCI 1_1 TB_z VC-DAI=4 |  |
|  |  | G-RNTI 4 DCI 1_1 TB_i VC-DAI=1 |  |  |  |

Figure 1. Example for no change of DAI procedures

Regarding Alt-1, “UL-DAI indicates the sum of the DL-DAIs.” The UL-DAI seems can help UE to determine the total codebook size among multiple G-RNTIs. However, if the size indicated by UL-DAI doesn’t match to the sum of received DL-DAIs, how does the UE know the missed DCI(s) belongs to which sub-codebook(s)? Considering the example in Figure 2, where a UE is configured with 2 G-RNTIs in a single cell. The base station sends 3 TBs with G-RNTI 1 and 2 TBs with G-RNTI-2. The UL-DAI is set to VT-DAI =1 due to 5 TBs have been transmitted. The gNB expects the UE to generate the 1st and 2nd sub-codebooks as {{A/N, A/N, A/N}, {A/N, A/N}}. If the scheduling DCI of TB_3 is missing, although the UE knows there is one DCI missing according to VT-DAI, but the UE cannot determine whether to append a NACK in the 1st or 2nd sub-codebook. If the UE selects the wrong sub-codebook, the base station will not be able to parse the HARQ codebook.

| PDCCH occasion index | m=0 | m=1 | m=2 | m=3 | m=4 |
| --- | --- | --- | --- | --- | --- |
| CC #0 | G-RNTI 1 DCI 1_1 TB_1 VC-DAI=1 | G-RNTI 1 DCI 1_1 TB_2 VC-DAI=2 |  | G-RNTI 1 DCI 1_1 TB_3 VC-DAI=3 | C-RNTI DCI 0_1 VT-DAI=1 |
|  | G-RNTI 2 DCI 1_1 TB_a VC-DAI=1 |  | G-RNTI 2 DCI 1_1 TB_b VC-DAI=2 |  |  |

Figure 2. UL-DAI indicates the sum of DL-DAIs

To make this alternative work, UE should able to tell the gNB which DL assignment is missing. One way to achieve that is to include the last received VC-DAI  of a G-RNTI to the associated sub-codebook. Take Figure 2 as an example, if the scheduling DCI of TB_3 and TB_b are missing, the UE generates {0, 1, A/N, A/N} for the 1st sub-codebook, where the first 2 bit {0, 1} refer to last received VC-DAI=2. Likewise, the UE generates {0, 0, A/N} to the 2nd sub-codebook. Next, in order to correct the codebook size, the UE appends 2 NACKs at the end of the codebook according to the VT-DAI, thereby the codebook become {{0, 1, A/N, A/N}, {0, 0, A/N}, N, N}. On the other side, the gNB determines the size of a sub-codebook according to the first 2 bits, then parses remaining ACK/NACKs in the sub-codebook.

Proposal 1: Update Alt-1 with following. UE includes 2 bits of the last received C-DAI value (VC-DAI) of a G-RNTI at the beginning of the associated sub-codebook.

Observation 2: If adopting Alt-1 without other details (the UL-DAI indicates the sum of C-DAIs), the last C-DAI missing of a G-RNTI may cause an incorrect sub-codebook. With the update of proposal 1, the Alt-1 can work properly and can tolerate up to 3 last C-DAI missing among G-RNTIs. Regarding the codebook size, 2 bits will be added to each sub-codebook.

Regarding Alt-2 that “DL-DAIs have the same value.” The description is not clear to us. Assuming it refers that all VC-DAI indicated in the same PDCCH occasion have the same value, and VC-DAI of all configured G-RNTIs maintains one C-DAI increment. The indicated UL-DAI can protect the C-DAI as legacy mechanism. However, because the VC-DAI of different G-RNTI updates depending on each other, it would not work if a UE only some of G-RNTIs that sharing the same C-DAI increment.

Observation 3: The mechanism of Alt-2 is unclear to us, more details are needed to justify the applicability.

Regarding Alt-3 that “largest DL DAI value among the configured G-RNTIs,” the detail has been introduced in Lenovo’s contribution [2] that all sub-codebook should have the same size according to the largest DL-DAI (i.e. the Y value calculated from (Y-1)modTd+1=VC-DAI). Considering the transmission pattern in Figure 3, if the scheduling DCI of TB_2, TB_3, and TB_4 are missing, the UE will generate the 1st, 2nd, and 3rd sub-codebooks as {{A/N, N, N, N}, {A/N, A/N, A/N, N}, {A/N, A/N, N, N}}, respectively. The UL-DAI is used to protect the C-DAI with the largest value and determine the size of all sub-codebooks. In this case, there is no miscalculation error unless more than 3 last DL DCI with the largest C-DAI are missing.

| PDCCH occasion index | m=0 | m=1 | m=2 | m=3 | m=4 | m=5 |
| --- | --- | --- | --- | --- | --- | --- |
| CC #0 | G-RNTI 1 DCI 1_1 TB_1 VC-DAI=1 | G-RNTI 1 DCI 1_1 TB_2 VC-DAI=2 |  | G-RNTI 1 DCI 1_1 TB_3 VC-DAI=3 | G-RNTI 1 DCI 1_1 TB_4 VC-DAI=4 | C-RNTI DCI 0_1 VT-DAI=4 |
|  | G-RNTI 2 DCI 1_1 TB_a VC-DAI=1 |  | G-RNTI 2 DCI 1_1 TB_b VC-DAI=2 | G-RNTI 2 DCI 1_1 TB_c VC-DAI=3 |  |  |
|  | G-RNTI 3 DCI 1_1 TB_x VC-DAI=1 |  |  |  | G-RNTI 3 DCI 1_1 TB_y VC-DAI=2 |  |

Figure 3. VT-DAI associate with the largest C-DAI

Observation 4: With sub-codebook size aligns to the largest C-DAI, Alt-3 can tolerate up to 3 contiguous last C-DAI missing. Regarding the codebook size, the number of extra NACKs to be added depends on the difference between the transmitted TBs of each G-RNTI and the largest C-DAI value.

# 3. Conclusion

Observation 1: If there is no change on C-DAI and UL-DAI mechanism, 2 bits UL-DAI is insufficient to cover all sub-codebook size determination error cases when the last C-DAI of a G-RNTI is missing. However, if the error case caused by the last C-DAI missing issue is not a concern to the group, there is no need to adopt any other alternatives.

Proposal 1: Update Alt-1 with following. UE includes 2 bits of the last received C-DAI value (VC-DAI) of a G-RNTI at the beginning of the associated sub-codebook.

Observation 2: If adopting Alt-1 without other details (the UL-DAI indicates the sum of C-DAIs), the last C-DAI missing of a G-RNTI may cause an incorrect sub-codebook. With the update of proposal 1, the Alt-1 can work properly and can tolerate up to 3 last C-DAI missing among G-RNTIs. Regarding the codebook size, 2 bits will be added to each sub-codebook.

Observation 3: The mechanism of Alt-2 is unclear to us, more details are needed to justify the applicability.

Observation 4: With sub-codebook size aligns to the largest C-DAI, Alt-3 can tolerate up to 3 contiguous last C-DAI missing. Regarding the codebook size, the number of extra NACKs to be added depends on the difference between the transmitted TBs of each G-RNTI and the largest C-DAI value.

Summary: If the last C-DAI missing issue is not a concern to the group, there is no change needed to the C-DAI and UL-DAI mechanism. Otherwise, from reliability perspective, Alt-3 is better than Alt-1, and from codebook size perspective, the size of Alt-1 is more static than Alt-3. From our perspective, we do have concern to the increased error probability caused by codebook size miscalculation. In this regard, we prefer to adopt a method that addresses this issue. The error tolerance of Alt-1 is acceptable to us, therefore we support Alt-1 (with proposal 1). However, we are also OK to other alternatives as long as the last C-DAI missing issue is addressed.

# 4. Reference

[1]	Chairman’s note, RAN1 meeting #108e, Feb. 2022.

[2]	R1-2202228, Lenovo, RAN1 meeting #108e, Feb. 2022.